3GPP TSG-RAN5 Meeting #98	R5-230623r1

# Athens,Greece, 27th Feb 2023- 3rd Mar 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 3519 | rev |  | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of MBS Multicast TC 14.2.1.1.8-Multiplex_Multicast_and_Unicast_HARQ |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS_5MBS_5MBUSA-UEConTest |  |  |  |  |  | Date: |  |  | 2023-02-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | MBS TC 14.2.1.1.8 needs to be added to test UE feedback multicast HARQ information and unicast HARQ information in same slot. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of MBS TC 14.2.1.1.8 MBS Multicast/ MAC / DL Data Transfer/ Multiplex multicast HARQ-ACK information with unicast HARQ-ACK information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR_MBS_5MBS_5MBUSA-UEConTest WP will be incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 14.2.1.1.8 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.523-2 CR 0317 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | 1st revision: Update coversheet |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

14.2.1.1.8	MBS Multicast/ MAC / DL Data Transfer/ Multiplex multicast HARQ-ACK information with unicast HARQ-ACK information

14.2.1.1.8.1	Test Purpose (TP)

(1)

with { UE in RRC_Connected state and Multicast MRB established with one RLC-UM entity for PTM transmission and one RLC-AM entity for PTP transimission and HARQ feedback for Multicast with ACK-NACK mode is enabled by RRC and unicast is configured to use dynamic harq-ack codebook and multicast is configured to use semi-static harq-ack codebook }

ensure that {

when { UE needs to multiplex the unicast and multicast HARQ-ACK information in a same PUCCH based on SS scheduling }

then { UE uses the last unicast DCI format to determine the PUCCH resource }

}

(2)

with { UE in RRC_Connected state and Multicast MRB established with one RLC-UM entity for PTM transmission and one RLC-AM entity for PTP transimission and HARQ feedback for Multicast with ACK-NACK mode is enabled by RRC and unicast is configured to use dynamic harq-ack codebook and multicast is configured to use semi-static harq-ack codebook }

ensure that {

when { UE needs to multiplex the unicast and multicast HARQ-ACK information in a same PUCCH based on SS scheduling }

then { UE appends the HARQ-ACK codebooks for the multicast HARQ-ACK information to the HARQ-ACK codebooks for the unicast HARQ-ACK information }

}

14.2.1.1.8.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.213, clause 18 and 9.1.2.1. Unless otherwise stated these are Rel-17 requirements.

[TS 38.213, clause 18]

If a UE would report unicast HARQ-ACK information and multicast HARQ-ACK information with same priority index in a slot, the UE multiplexes the unicast HARQ-ACK information and the multicast HARQ-ACK information following the procedures in this clause and in clauses 9.1.2 and 9.1.3.

If, for unicast and multicast HARQ-ACK information of same priority value, a UE

-	is provided

-	either pdsch-HARQ-ACK-Codebook = dynamic or pdsch-HARQ-ACK-Codebook-r16 for unicast HARQ-ACK information and pdsch-HARQ-ACK-Codebook = semi-static for multicast HARQ-ACK information,

-	or pdsch-HARQ-ACK-Codebook = semi-static for unicast HARQ-ACK information and pdsch-HARQ-ACK-Codebook = dynamic for multicast HARQ-ACK information, and

-	would multiplex the unicast and multicast HARQ-ACK information in a same PUCCH or PUSCH

the UE

-	appends the HARQ-ACK codebooks for the multicast HARQ-ACK information to the HARQ-ACK codebooks for the unicast HARQ-ACK information

-	if OACK+OSR+OCSI≤11, the UE determines nHARQ-ACK for obtaining a power of a PUCCH transmission with the HARQ-ACK information, as described in clause 7.2.1, as a sum of the nHARQ-ACK value from clause 9.1.2.1 or clause 9.1.3.3 and the nHARQ-ACK value from clause 9.1.3.1.

A UE determines a PUCCH resource for a PUCCH transmission with HARQ-ACK information as described in clauses 9.2 and 9.2.1 through 9.2.5.

If a UE multiplexes in a PUCCH HARQ-ACK information of same priority associated with unicast DCI formats and with multicast DCI formats in a same PUCCH, the last DCI format that the UE uses to determine the PUCCH resource, as described in clause 9.2.3, is a last unicast DCI format.

[TS 38.213, clause 9.1.2.1]

For a serving cell c, an active DL BWP, and an active UL BWP, as described in clause 12, the UE determines a set of MA,c occasions for candidate PDSCH receptions for which the UE can transmit corresponding HARQ-ACK information in a PUCCH in slot nU. If serving cell c is deactivated, the UE uses as the active DL BWP for determining the set of MA,c occasions for candidate PDSCH receptions a DL BWP provided by firstActiveDownlinkBWP-Id. The determination is based:

a)	on a set of slot timing values K1 associated with the active UL BWP on the primary cell or, if the PUCCH transmission is indicated by a DCI format to be on the PUCCH-sSCell as described in clause 9A, on a set of slot timing values K1 associated with the active UL BWP on the PUCCH-sSCell

…

-	If the UE is configured to monitor PDCCH for multicast DCI formats for serving cell c

-	if the UE is not provided type1-Codebook-GenerationMode = 'mode1', K1 is additionally provided by the union of dl-DataToUL-ACK from pucch-ConfigMulticast1/pucch-ConfigurationListMulticast1 or pucch-ConfigMulticast2/pucch-ConfigurationListMulticast2 and dl-DataToUL-ACK-ForDCI Format4-1

-	if the UE is not provided dl-DataToUL-ACK-ForDCI Format4-1, K1 is provided by the union of dl-DataToUL-ACK from pucch-ConfigurationListMulticast1 or pucch-ConfigurationListMulticast2 and the slot timing values {1, 2, 3, 4, 5, 6, 7, 8}

14.2.1.1.8.3	Test description

14.2.1.1.8.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1.

-	The SS configures the NR Cell 1 as the "Serving cell".

-	System information combination NR-1 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used in NR cell 1.

UE:

-	None.

Preamble:

-	The UE is in state 1N-A on NR Cell 1(serving cell) according to TS 38.508-1 [4] Table 4.4A.2-3 with Test Mode = on to activate UE TEST MODE C and Test Loop Function = off.

-	The UE is made interested in receiving MBS Multicast service with MBS service ID '000101'H.

14.2.1.1.8.3.2	Test procedure sequence

Table 14.2.1.1.8.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1a1-1b12a1 | Steps 1a1 to 1b12a1 of the generic procedures described in TS 38.508-1 subclause 4.9.34 are performed on NR Cell 1 to establish an associated PDU Session to the MBS DNN and join in MBS Multicast session. | - | - | - | - |
| 2a1-2a2 | Steps 9a1 to 9a2 of the generic procedures described in TS 38.508-1 subclause 4.5.4.2-3 are performed on NR Cell 1 with condition UE TEST LOOP MODE C and Multicast MRB. | - | - | - | - |
| 3 | The SS indicates a new transmission addressed to the G-RNTI assigned to the UE in SFN x, slot 2 and the "PDSCH-to-HARQ_feedback timing indicator" is configured to require UE to feedback HARQ-ACK information in SFN x, slot 9. | <-- | (PDCCH (G-RNTI)) | - | - |
| 4 | The SS transmits a MBS Packet on the MTCH with LCID matched with the LCID configured for receving PTM transmission. The CRC is calculated in such a way, it will result in CRC pass on UE side. | <-- | MBS Packet | - | - |
| 5 | The SS indicates a new transmission addressed to the C-RNTI assigned to the UE in SFN x, slot 5 and the "PDSCH-to-HARQ_feedback timing indicator" is configured to require UE to feedback HARQ-ACK information in SFN x, slot 9. | <-- | (PDCCH (C-RNTI)) | - | - |
| 6 | The SS transmits a MBS Packet on the DTCH with LCID matched with the LCID configured for receving PTP transmission. The CRC is calculated in such a way, it will result in CRC pass on UE side. | <-- | MBS Packet | - | - |
| 7 | Check: Does the UE transmit HARQ-ACK codebooks according to Table 14.2.1.1.8.3.2-2 using PUCCH resource indicated by the unicast DCI in step 5? | --> | HARQ ACK/NACK | 1,2 | P |
| 8 | The SS transmits an UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST message. | <-- | NR RRC: DLInformationTransfer TC: UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST | - | - |
| 9 | UE responds with UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE. | --> | NR RRC: ULInformationTransfer TC: UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE | - | - |
| 10 | Check: Is the number of reported MBS Packets received on the MRB in step 9 equal to 2? | - | - | 1,2 | P |
| 11 | The SS indicates a new transmission addressed to the C-RNTI assigned to the UE in SFN y, slot 2 and the "PDSCH-to-HARQ_feedback timing indicator" is configured to require UE to feedback HARQ-ACK information in SFN y, slot 9. | <-- | (PDCCH (C-RNTI)) | - | - |
| 12 | The SS transmits a MBS Packet on the DTCH with LCID matched with the LCID configured for receving PTP transmission. The CRC is calculated in such a way, it will result in CRC pass on UE side. | <-- | MBS Packet | - | - |
| 13 | The SS indicates a new transmission addressed to the G-RNTI assigned to the UE in SFN y, slot 5 and the "PDSCH-to-HARQ_feedback timing indicator" is configured to require UE to feedback HARQ-ACK information in SFN y, slot 9. | <-- | (PDCCH (G-RNTI)) | - | - |
| 14 | The SS transmits a MBS Packet on the MTCH with LCID matched with the LCID configured for receving PTM transmission. The CRC is calculated in such a way, it will result in CRC pass on UE side. | <-- | MBS Packet | - | - |
| 15 | Check: Does the UE transmit HARQ-ACK codebooks according to Table 14.2.1.1.8.3.2-2 using PUCCH resource indicated by the unicast DCI in step 11? | --> | HARQ ACK/NACK | 1,2 | P |
| 16 | The SS transmits an UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST message. | <-- | NR RRC: DLInformationTransfer TC: UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST | - | - |
| 17 | UE responds with UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE. | --> | NR RRC: ULInformationTransfer TC: UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE | - | - |
| 18 | Check: Is the number of reported MBS Packets received on the MRB in step 17 equal to 4? | - | - | 1,2 | P |

Table 14.2.1.1.8.3.2-2: HARQ-ACK codebooks feedback

| HARQ-ACK codebooks | Comment | Condition |
| --- | --- | --- |
| {ACK,NACK,ACK,NACK,NACK,NACK,NACK,NACK } |  | Step7 AND FDD AND SCS 15KHz |
| {ACK,NACK,ACK,NACK,NACK,NACK,NACK } |  | Step7 AND TDD AND SCS 15KHz |
| {ACK,NACK,ACK,NACK,NACK,NACK } |  | Step7 AND SCS 30KHz |
| {ACK,NACK,ACK,NACK,NACK } |  | Step7 AND SCS 120KHz |
| {ACK,NACK,NACK,NACK,ACK,NACK,NACK,NACK } |  | Step15 AND FDD AND SCS 15KHz |
| {ACK,NACK,NACK,ACK,NACK,NACK,NACK } |  | Step15 AND TDD AND SCS 15KHz |
| {ACK,NACK,NACK,NACK,ACK,NACK } |  | Step15 AND SCS 30KHz |
| {ACK,NACK,ACK,NACK,NACK } |  | Step15 AND SCS 120KHz |

14.2.1.1.8.3.3	Specific message contents

Table 14.2.1.1.8.3.3-1: ACTIVATE TEST MODE (preamble, Table 14.2.1.1.8.3.2-1)

Derivation Path: TS 36.508 [6], Table 4.7A-1, condition UE TEST LOOP MODE C

Table 14.2.1.1.8.3.3-2: RRCReconfiguration (step 1a15, Table 14.2.1.1.8.3.2-1)

| Derivation Path: TS 38.508-1 [4],Table 4.6.1-13 and condition NR |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |  |
| radioBearerConfig |  | RadioBearerConfig with condition MRBm and AMPTP_UMPTM | m=1 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| masterCellGroup |  | CellGroupConfig | Table 14.2.1.1.8.3.3-4 |  |
| dedicatedNAS-MessageList SEQUENCE (SIZE(1..maxDRB)) OF DedicatedNAS-Message {} |  | DedicatedNAS-Message |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 14.2.1.1.8.3.3-3: RRCReconfiguration (step 1b10, Table 14.2.1.1.8.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 and condition NR |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |  |
| radioBearerConfig |  | RadioBearerConfig with condition DRBn and MRBm and AMPTP_UMPTM | n is chosen as the next available number higher or equal to 2 m=1 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| masterCellGroup |  | CellGroupConfig | Table 14.2.1.1.8.3.3-5 |  |
| dedicatedNAS-MessageList SEQUENCE (SIZE(1..maxDRB)) OF DedicatedNAS-Message {} |  | DedicatedNAS-Message |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 14.2.1.1.8.3.3-4: CellGroupConfig (Table 14.2.1.1.8.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19, condition MRBm (m=1) and AMPTP_UMPTM |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| mac-CellGroupConfig | MAC-CellGroupConfig with condition MBS_Multicast and RRC_Enable_HARQFeedback and ACK_NACK |  |  |
| physicalCellGroupConfig | physicalCellGroupConfig | Table 14.2.1.1.8.3.3-6 |  |
| spCellConfig SEQUENCE { |  |  |  |
| spCellConfigDedicated | ServingCellConfig | Table 14.2.1.1.8.3.3-7 |  |
| } |  |  |  |
| } |  |  |  |

Table 14.2.1.1.8.3.3-5: CellGroupConfig (Table 14.2.1.1.8.3.3-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19, condition MRBm_DRBn and AMPTP_UMPTM (Note 1) |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| mac-CellGroupConfig | MAC-CellGroupConfig with condition MBS_Multicast and RRC_Enable_HARQFeedback and ACK_NACK |  |  |
| physicalCellGroupConfig | physicalCellGroupConfig | Table 14.2.1.1.8.3.3-6 |  |
| spCellConfig SEQUENCE { |  |  |  |
| spCellConfigDedicated | ServingCellConfig | Table 14.2.1.1.8.3.3-7 |  |
| } |  |  |  |
| } |  |  |  |
| Note 1: n is set to the same value as for the radioBearerConfig IE in Table 14.2.1.1.8.3.3-3 and m=1 |  |  |  |

Table 14.2.1.1.8.3.3-6: PhysicalCellGroupConfig (Table 14.2.1.1.8.3.3-4, Table 14.2.1.1.8.3.3-5)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-106 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalCellGroupConfig ::= SEQUENCE { |  |  |  |
| MulticastConfig-r17 SEQUENCE { |  |  |  |
| pdsch-HARQ-ACK-CodebookListMulticast-r17 CHOICE { |  |  |  |
| setup SEQUENCE (SIZE (1..2)) OF ENUMERATED { | 1 entry |  |  |
| ENUMERATED [1] | semiStatic | entry 1 |  |
| } |  |  |  |
| } |  |  |  |
| type1-Codebook-GenerationMode-r17 | mode2 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.2.1.1.8.3.3-7: ServingCellConfig (Table 14.2.1.1.8.3.3-4, Table 14.2.1.1.8.3.3-5)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-167, condition MBS_Multicast |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| uplinkConfig SEQUENCE { |  |  |  |
| initialUplinkBWP | BWP-UplinkDedicated | Table 14.2.1.1.8.3.3-8 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.2.1.1.8.3.3-8: BWP-UplinkDedicated (Table 14.2.1.1.8.3.3-7)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-UplinkDedicated ::= SEQUENCE { |  |  |  |
| pucch-ConfigMulticast1-r17 CHOICE { |  |  |  |
| setup | PUCCH-Config | Table 14.2.1.1.8.3.3-9 |  |
| } |  |  |  |
| } |  |  |  |

Table 14.2.1.1.8.3.3-9: PUCCH-Config (Table 14.2.1.1.8.3.3-8)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-15, condition RF (Note 1) |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUCCH-Config ::= SEQUENCE { |  |  |  |
| schedulingRequestResourceToAddModList | Not present |  |  |
| dl-DataToUL-ACK SEQUENCE (SIZE (1..8)) OF INTEGER { | 8 entries |  |  |
| INTEGER[1] | 1 | entry 1 |  |
| INTEGER[2] | 2 | entry 2 |  |
| INTEGER[3] | 3 | entry 3 |  |
| INTEGER[4] | 4 | entry 4 |  |
| INTEGER[5] | 5 | entry 5 |  |
| INTEGER[6] | 6 | entry 6 |  |
| INTEGER[7] | 7 | entry 7 |  |
| INTEGER[8] | 8 | entry 8 |  |
| } |  |  |  |
| } |  |  |  |
| Note 1: Set intraSlotFrequencyHopping = Not present to ensure that PUCCH Resource for Multicast is different from PUCCH Resource for Unicast. |  |  |  |

Table 14.2.1.1.8.3.3-10: CLOSE UE TEST LOOP (step 2a1, Table 14.2.1.1.8.3.2-1)

Derivation Path: 38.508-1 [4], Table 4.7A-3, condition UE TEST LOOP MODE C and Multicast MRB

Table 14.2.1.1.8.3.3-11: UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST (step 8 and step 16, Table 14.2.1.1.8.3.2-1)

Derivation Path: 36.508 [6], Table 4.7A-9

Table 14.2.1.1.8.3.3-12: Physical layer parameters for DCI format 4_1 (step 3 and step 13, Table 14.2.1.1.8.3.2-1)

| Parameter | Value | Value in binary | Condition |
| --- | --- | --- | --- |
| PDSCH-to-HARQ_feedback timing indicator | K1 = 7 | “110” | Step 3 |
|  | K1 = 4 | “011” | Step 13 |
| Note: K1 set for DCI 4_1 is { 1, 2, 3, 4, 5, 6, 7, 8 } because dl-DataToUL-ACK-MulticastDCI-Format4-1 is not provided |  |  |  |

Table 14.2.1.1.8.3.3-13: Physical layer parameters for DCI format 1_1 (step 5 and step 11, Table 14.2.1.1.8.3.2-1)

| Parameter | Value | Value in binary | Condition |
| --- | --- | --- | --- |
| PDSCH-to-HARQ_feedback timing indicator | K1 = 4 | “100” | Step 5 |
|  | K1 = 7 | “101” | Step 11 |
| Note: K1 set for DCI 1_1 is { 2, 3, 4, 5, 6, 7, 8, 9 } . |  |  |  |

<End of modified section 1>