3GPP TSG-RAN5 Meeting #98	R5-230624r1

# Athens, Greece, 27th Feb 2023- 3rd Mar 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 3520 | rev |  | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of MBS Multicast TC 14.2.1.2.1-DRX PTM and PTP transmission |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS_5MBS_5MBUSA-UEConTest |  |  |  |  |  | Date: |  |  | 2023-02-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | MBS TC 14.2.1.2.1 needs ot be added to test that UE could receive unicast and multicast MAC PDU when unicast DRX and multicast DRX configured with different parament and all the Timer related to the DRX could start and stop according to the requirment of the core spec 38.321 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of MBS TC 14.2.1.2.1 MBS Multicast/ MAC/ DRX operation/ PTM transmission / PTP transmission |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR_MBS_5MBS_5MBUSA-UEConTest WP will be incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 14.2.1.2.1 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.523-2 CR 0317 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | 1st revision: Update the coversheet. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

14.2.1.2	MBS Multicast/ MAC/ DRX operation

14.2.1.2.1	MBS Multicast/ MAC/ DRX operation/ PTM transmission / PTP transmission

14.2.1.2.1.1	Test Purpose (TP)

(1)

with { UE in RRC_Connected state and Multicast MRB established with one RLC-UM entity for PTM transmission and one RLC-UM entity for PTP transmission, and Multicast DRX and unicast DRX are configured }

ensure that {

when { Long DRX cycle for a G-RNTI is configured and [(SFN * 10) + subframe number] modulo (drx-LongCycle-PTM) = drx-StartOffset-PTM }

then { UE starts the drx-OnDurationTimerPTM and monitors the PDCCH for this G-RNTI during drx-OnDurationTimerPTM }

}

(2)

with { UE in RRC_Connected state and Multicast MRB established with one RLC-UM entity for PTM transmission and one RLC-UM entity for PTP transmission, and Multicast DRX and unicast DRX are configured }

ensure that {

when { Long DRX cycle for a G-RNTI is configured and a new DL transmission is indicated on the PDCCH during Active Time }

then { UE starts or restarts the drx-InactivityTimerPTM in the first symbol after the end of the PDCCH reception and monitors the PDCCH for this G-RNTI during drx-InactivityTimerPTM }

}

(3)

with { UE in RRC_Connected state and Multicast MRB established with one RLC-UM entity for PTM transmission and one RLC-UM entity for PTP transmission, and Multicast DRX and unicast DRX are configured }

ensure that {

when { Long DRX cycle for C-RNTI is configured and [(SFN * 10) + subframe number] modulo (drx-LongCycle) = drx-StartOffset }

then { UE monitors the PDCCH for C-RNTI during drx-OnDurationTimer }

}

(4)

with { UE in RRC_Connected state and Multicast MRB established with one RLC-UM entity for PTM transmission and one RLC-UM entity for PTP transmission, and Multicast DRX and unicast DRX are configured }

ensure that {

when { Long DRX cycle for C-RNTI is configured and a new DL transmission is indicated on the PDCCH during Active Time }

then { UE starts or restarts the drx-InactivityTimer in the first symbol after the end of the PDCCH reception and monitors the PDCCH for C-RNTI during drx-InactivityTimer }

}

(5)

with { UE in RRC_Connected state and Multicast MRB established with one RLC-UM entity for PTM transmission and one RLC-UM entity for PTP transmission, and Multicast DRX and unicast DRX are configured }

ensure that {

when { DRX Command MAC CE indicated by PDCCH addressed to with DCI scrambled with a G-RNTI is received }

then { UE stops drx-onDurationTimerPTM of the DRX for this G-RNTI }

}

(6)

with { UE in RRC_Connected state and Multicast MRB established with one RLC-UM entity for PTM transmission and one RLC-UM entity for PTP transmission, and only unicast DRX is configured }

ensure that {

when { PDCCH indicated a DL multicast transmission is received }

then { UE stops drx-RetransmissionTimerDL for the corresponding HARQ process }

}

(7)

with { UE in RRC_Connected state and Multicast MRB established with one RLC-UM entity for PTM transmission and one RLC-UM entity for PTP transmission, both multicast DRX and unicast DRX are configured, and allowCSI-SRS-Tx-MulticastDRX-Active is configured }

ensure that {

when { Both unicast DRX and multicast DRX are not in Active Time }

then { UE stop report CSI on PUCCH }

}

(8)

with { UE in RRC_Connected state and Multicast MRB established with one RLC-UM entity for PTM transmission and one RLC-UM entity for PTP transmission, only unicast DRX is configured, and allowCSI-SRS-Tx-MulticastDRX-Active is not configured }

ensure that {

when { Unicast DRX is not in Active Time }

then { UE stop report CSI on PUCCH }

}

(9)

with { UE in RRC_Connected state and Multicast MRB established with one RLC-UM entity for PTM transmission and one RLC-UM entity for PTP transmission, both multicast DRX and unicast DRX are configured, and allowCSI-SRS-Tx-MulticastDRX-Active is configured }

ensure that {

when { Unicast DRX or multicast DRX are in Active Time}

then { UE does not stop report CSI on PUCCH }

}

(10)

with { UE in RRC_Connected state and Multicast MRB established with one RLC-UM entity for PTM transmission and one RLC-UM entity for PTP transmission, only unicast DRX is configured, and allowCSI-SRS-Tx-MulticastDRX-Active is not configured }

ensure that {

when { Unicast DRX is in Active Time }

then { UE does not stop report CSI on PUCCH }

}

14.2.1.2.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.321, clause 5.7, 5.7b. Unless otherwise stated these are Rel-17 requirements.

[TS 38.321, clause 5.7]

1>	if the Long DRX cycle is used for a DRX group, and [(SFN × 10) + subframe number] modulo (drx-LongCycle) = drx-StartOffset:

2>	if DCP monitoring is configured for the active DL BWP as specified in TS 38.213 [6], clause 10.3:

….

2>	else:

3>	start drx-onDurationTimer for this DRX group after drx-SlotOffset from the beginning of the subframe.

…

1>	if a DRX group is in Active Time:

2>	monitor the PDCCH on the Serving Cells in this DRX group as specified in TS 38.213 [6];

2>	if the PDCCH indicates a DL transmission; or

2>	if the PDCCH indicates a one-shot HARQ feedback as specified in clause 9.1.4 of TS 38.213 [6]; or

2>	if the PDCCH indicates a retransmission of HARQ feedback as specified in clause 9.1.5 of TS 38.213 [6]:

3>	if this Serving Cell is configured with downlinkHARQ-FeedbackDisabled:

…

3>	else:

4>	start or restart the drx-HARQ-RTT-TimerDL for the corresponding HARQ process(es) whose HARQ feedback is reported in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback.

…

3>	stop the drx-RetransmissionTimerDL for the corresponding HARQ process(es) whose HARQ feedback is reported;

3>	stop the drx-RetransmissionTimerDL-PTM for the corresponding HARQ process;

…

2>	if the PDCCH indicates a new transmission (DL, UL or SL) on a Serving Cell in this DRX group:

3>	start or restart drx-InactivityTimer for this DRX group in the first symbol after the end of the PDCCH reception.

…

1>	if DCP monitoring is configured for the active DL BWP as specified in TS 38.213 [6], clause 10.3; and

1>	if the current symbol n occurs within drx-onDurationTimer duration; and

1>	if drx-onDurationTimer associated with the current DRX cycle is not started as specified in this clause:

…

1>	else:

2>	in current symbol n, if a DRX group would not be in Active Time considering grants/assignments scheduled on Serving Cell(s) in this DRX group and DRX Command MAC CE/Long DRX Command MAC CE received and Scheduling Request sent until 4 ms prior to symbol n when evaluating all DRX Active Time conditions as specified in this clause; and

2>	if allowCSI-SRS-Tx-MulticastDRX-Active is not configured or, in current symbol n, if all multicast DRXes corresponding to the DRX group would not be in Active Time considering multicast assignments/DRX Command MAC CE for MBS multicast received until 4 ms prior to symbol n when evaluating all DRX Active Time conditions as specified in Clause 5.7b and all multicast sessions corresponding to the DRX group are configured with multicast DRX:

3>	not transmit periodic SRS and semi-persistent SRS defined in TS 38.214 [7] in this DRX group;

3>	not report CSI on PUCCH and semi-persistent CSI configured on PUSCH in this DRX group.

…

[TS 38.321, clause 5.7b]

For MBS multicast, the MAC entity may be configured by RRC with a DRX functionality per G-RNTI or per G-CS-RNTI that controls the UE's PDCCH monitoring activity for the MAC entity's G-RNTI(s) and G-CS-RNTI(s) as specified in TS 38.331 [5]. When in RRC_CONNECTED, if multicast DRX is configured for a G-RNTI or G-CS-RNTI, the MAC entity is allowed to monitor the PDCCH for this G-RNTI or G-CS-RNTI discontinuously using the multicast DRX operation specified in this clause; otherwise the MAC entity monitors the PDCCH for this G-RNTI or G-CS-RNTI as specified in TS 38.213 [6]. The multicast DRX operation specified in this clause is performed independently for each G-RNTI or G-CS-RNTI and independently from the DRX operation specified in clauses 5.7 and 5.7a.

RRC controls multicast DRX operation per G-RNTI or per G-CS-RNTI by configuring the following parameters:

-	drx-onDurationTimerPTM: the duration at the beginning of a DRX cycle;

-	drx-SlotOffsetPTM: the delay before starting the drx-onDurationTimerPTM;

-	drx-InactivityTimerPTM: the duration after the PDCCH occasion in which a PDCCH indicates a new DL multicast transmission for the MAC entity;

-	drx-LongCycleStartOffsetPTM: the long DRX cycle drx-LongCycle-PTM and drx-StartOffset-PTM which defines the subframe where the long DRX cycle starts;

…

When multicast DRX is configured for a G-RNTI or G-CS-RNTI, the Active Time includes the time while:

-	drx-onDurationTimerPTM or drx-InactivityTimerPTM or drx-RetransmissionTimerDL-PTM for this G-RNTI or G-CS-RNTI is running.

When multicast DRX is not configured for a G-RNTI or G-CS-RNTI and unicast DRX is configured, the MAC entity shall for this G-RNTI or G-CS-RNTI:

1>	monitor the PDCCH as specified in TS 38.213 [6];

1>	if the PDCCH indicates a DL multicast transmission; or

1>	if a MAC PDU is received in a configured downlink multicast assignment and CS-RNTI is configured:

…

2>	stop the drx-RetransmissionTimerDL for the corresponding HARQ process.

When multicast DRX is configured for a G-RNTI or G-CS-RNTI, the MAC entity shall for this G-RNTI or G-CS-RNTI:

…

1>	if a drx-HARQ-RTT-TimerDL-PTM expires:

2>	if the data of the corresponding HARQ process was not successfully decoded:

3>	start the drx-RetransmissionTimerDL-PTM for the corresponding HARQ process in the first symbol after the expiry of drx-HARQ-RTT-TimerDL-PTM.

1>	if a DRX Command MAC CE indicated by PDCCH addressed to a G-RNTI or G-CS-RNTI, or by a configured downlink multicast assignment is received:

2>	stop drx-onDurationTimerPTM of the DRX for this G-RNTI or G-CS-RNTI;

2>	stop drx-InactivityTimerPTM of the DRX for this G-RNTI or G-CS-RNTI.

1>	if [(SFN × 10) + subframe number] modulo (drx-LongCycle-PTM) = drx-StartOffset-PTM:

2>	start drx-onDurationTimerPTM after drx-SlotOffsetPTM from the beginning of the subframe.

1>	if the MAC entity is in Active Time for this G-RNTI or G-CS-RNTI:

2>	monitor the PDCCH for this G-RNTI or G-CS-RNTI as specified in TS 38.213 [6];

2>	if the PDCCH indicates a DL multicast transmission:

…

3>	stop the drx-RetransmissionTimerDL-PTM for the corresponding HARQ process;

3>	stop the drx-RetransmissionTimerDL for the corresponding HARQ process.

2>	if the PDCCH indicates a new multicast transmission for this G-RNTI or G-CS-RNTI:

3>	start or restart drx-InactivityTimerPTM in the first symbol after the end of the PDCCH reception.

14.2.1.2.1.3	Test description

14.2.1.2.1.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1 is the serving cell.

-	System information combination NR-1 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used in NR Cell 1.

UE:

-	None.

Preamble:

-	The UE is in state 1N-A on NR Cell 1(serving cell) according to TS 38.508-1 [4] Table 4.4A.2-3 with Test Mode = on to activate UE TEST MODE C and Test Loop Function = off.

-	The UE is made interested in receiving MBS Multicast service with MBS service ID '000101'H.

14.2.1.2.1.3.2	Test procedure sequence

Table 14.2.1.2.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1a1-1b12a1 | Steps 1a1 to 1b12a1 of the generic procedures described in TS 38.508-1 subclause 4.9.34 are performed on NR Cell 1 to establish an associated PDU Session to the MBS DNN and join in MBS Multicast session. | - | - | - | - |
| 2a1-2a2 | Steps 9a1 to 9a2 of the NR RRC_CONNECTED procedure in TS 38.508-1 Table 4.5.4.2-3 are executed with condition UE TEST LOOP MODE C and Multicast MRB. | - | - | - | - |
| 3 | The SS transmits RRCReconfiguration to configure unicast DRX and multicast DRX parameters. | <-- | NR RRC: RRCReconfiguration | - | - |
| 4 | The UE transmits RRCReconfigurationComplete. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 5 | In the first PDCCH occasion when the drx-onDurationTimerPTM is running, the SS indicates the transmission of a MBS Packet via RLC-UM for PTM transmission on the PDCCH for G-RNTI. | <-- | MBS Packet. | - | - |
| 6 | In the last PDCCH occasion while the drx-onDurationTimerPTM is still running, the SS indicates the transmission of a MBS Packet via RLC-UM for PTM transmission on the PDCCH for G-RNTI. | <-- | MBS Packet. | - | - |
| 7 | drx-InactivityTimerPTM PDCCH-occasions after the transmission of the MBS Packet transmitted in step 6 was indicated on the PDCCH, the SS indicates the transmission of a MBS Packet via RLC-UM for PTM transmission on the PDCCH for G-RNTI. | <-- | MBS Packet. | - | - |
| 8 | The SS transmits an UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST message. | <-- | NR RRC: DLInformationTransfer TC: UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST | - | - |
| 9 | UE responds with UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE. | --> | NR RRC: ULInformationTransfer TC: UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE | - | - |
| 10 | Check: Is the number of reported MBS Packets received on in step 9 equal to 3? | - | - | 1,2 | P |
| 11 | In the first PDCCH occasion when the drx-onDurationTimer is running, the SS indicates the transmission of a MBS Packet via RLC-UM for PTP transmission on the PDCCH for C-RNTI. | <-- | MBS Packet. | - | - |
| 12 | Check: Does the UE transmit a HARQ ACK for the MBS Packet in Step 11? | --> | HARQ ACK | 3 | P |
| 13 | In the last PDCCH occasion while the drx-onDurationTimer is still running, the SS indicates the transmission of a MBS Packet via RLC-UM for PTP transmission on the PDCCH for C-RNTI. | <-- | MBS Packet. | - | - |
| 14 | Check: Does the UE transmit a HARQ ACK for the MBS Packet in Step 13? | --> | HARQ ACK | 3 | P |
| 15 | drx-InactivityTimer PDCCH-occasions after the transmission of the MBS Packet transmitted in step 13 was indicated on the PDCCH, the SS indicates the transmission of a MBS Packet via RLC-UM for PTP transmission on the PDCCH for C-RNTI. | <-- | MBS Packet. | - | - |
| 16 | Check: Does the UE transmit a HARQ ACK for the MBS Packet in Step 15? | --> | HARQ ACK | 4 | P |
| 17 | Check: Does the UE transmit a CSI Report during when the drx-onDurationTimerPTM is running? | --> | CSI Report | 9 | P |
| 18 | In the first PDCCH occasion when the drx-onDurationTimerPTM is running, the SS indicates the transmission of a DRX MAC Control element on the PDCCH for G-RNTI. UE successfully decodes the MAC PDU and stop the drx-onDurationTimerPTM. | <-- | DRX MAC Control element | - | - |
| 19 | Wait drx-InactivityTimerPTM expires after step18. | - | - | - | - |
| 20 | Check: Does the UE transmit a CSI Report after Step19? | --> | CSI Report | 5,7 | F |
| 21 | The SS transmits RRCReconfiguration to disable multicast DRX parameters. | <-- | NR RRC: RRCReconfiguration | - | - |
| 22 | The UE transmits RRCReconfigurationComplete. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 23 | In the last PDCCH occasion while the drx-onDurationTimer is still running, the SS indicates the transmission of a MBS Packet via RLC-UM for PTP transmission on the PDCCH for C-RNTI. The CRC is calculated in such a way, it will result in CRC error on UE side. | <-- | MBS Packet. | - | - |
| 24 | The UE transmit a HARQ ACK for the MBS Packet in Step 23? | --> | HARQ NACK | - | - |
| 25 | Check: Does the UE transmit a CSI Report during when the drx-RetransmissionTimerDL is running? | --> | CSI Report | 10 | P |
| 26 | In a PDCCH occasion which is X PDCCH sub frames before the PDCCH occasion in which the drx-RetransmissionTimerDL expires, with X > period of CSI Report, the SS indicates the transmission of a MBS Packet via RLC-UM for PTM transmission on the PDCCH for G-RNTI. UE stops drx-RetransmissionTimerDL. | <-- | MBS Packet. | - | - |
| 27 | Check: Does the UE transmit a CSI Report after step 26? | --> | CSI Report | 6,8 | F |

14.2.1.2.1.3.3	Specific message contents

Table 14.2.1.2.1.3.3-1: ACTIVATE TEST MODE (preamble, Table 14.2.1.2.1.3.2-1)

Derivation Path: TS 36.508 [6], Table 4.7A-1, condition UE TEST LOOP MODE C

Table 14.2.1.2.1.3.3-2: RRCReconfiguration (step 1a15, Table 14.2.1.2.1.3.2-1)

| Derivation Path: TS 38.508-1 [4],Table 4.6.1-13 and condition NR |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |  |
| radioBearerConfig |  | RadioBearerConfig with condition MRBm and UMPTP_UMPTM | m=1 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| masterCellGroup |  | CellGroupConfig with condition MRBm and UMPTP_UMPTM | m=1 |  |
| dedicatedNAS-MessageList SEQUENCE (SIZE(1..maxDRB)) OF DedicatedNAS-Message {} |  | DedicatedNAS-Message |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 14.2.1.2.1.3.3-3: RRCReconfiguration (step 1b10, Table 14.2.1.2.1.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 and condition NR |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |  |
| radioBearerConfig |  | RadioBearerConfig with condition DRBn and MRBm and UMPTP_UMPTM | n is chosen as the next available number higher or equal to 2 m=1 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| masterCellGroup |  | CellGroupConfig with condition MRBm_DRBn and UMPTP_UMPTM | n is set to the same value as for the radioBearerConfig IE above m=1 |  |
| dedicatedNAS-MessageList SEQUENCE (SIZE(1..maxDRB)) OF DedicatedNAS-Message {} |  | DedicatedNAS-Message |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 14.2.1.2.1.3.3-4: CLOSE UE TEST LOOP (step 2a1, Table 14.2.1.2.1.3.2-1)

Derivation Path: 38.508-1 [4], Table 4.7A-3, condition UE TEST LOOP MODE C and Multicast MRB

Table 14.2.1.2.1.3.3-5: RRCReconfiguration (step 3, Table 14.2.4.1.2.3.2-1)

| Derivation Path: TS 38.508-1 [4],Table 4.6.1-13 and condition NR |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| masterCellGroup |  | CellGroupConfig | Table 14.2.1.2.1.3.3-6 |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 14.2.1.2.1.3.3-6: CellGroupConfig (Table 14.2.1.2.1.3.3-5)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| mac-CellGroupConfig | MAC-CellGroupConfig | Table 14.2.1.2.1.3.3-7 |  |
| physicalCellGroupConfig | Not present |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| servCellIndex | Not present |  |  |
| reconfigurationWithSync | Not present |  |  |
| rlf-TimersAndConstants | Not present |  |  |
| rlmInSyncOutOfSyncThreshold | Not present |  |  |
| spCellConfigDedicated | ServingCellConfig | Table 14.2.1.2.1.3.3-10 |  |
| } |  |  |  |
| } |  |  |  |

Table 14.2.1.2.1.3.3-7: MAC-CellGroupConfig (Table 14.2.1.2.1.3.3-6)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-68, condition DRX and MBS_Multicast and DRX_MBS_Multicast |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MAC-CellGroupConfig ::= SEQUENCE { |  |  |  |
| drx-Config CHOICE { |  |  |  |
| setup | DRX-Config | Table 14.2.1.2.1.3.3-8 |  |
| } |  |  |  |
| g-RNTI-ConfigToAddModList-r17 SEQUENCE (SIZE (1..maxG-RNTI-r17)) OF MBS-RNTI-SpecificConfig-r17 { | 1 entry |  |  |
| MBS-RNTI-SpecificConfig-r17[1] SEQUENCE { |  | entry 1 |  |
| mbs-RNTI-SpecificConfigId-r17 | 0 |  |  |
| groupCommon-RNTI-r17 CHOICE { |  |  |  |
| g-RNTI | RNTI-Value |  |  |
| } |  |  |  |
| drx-ConfigPTM-r17 CHOICE { |  |  |  |
| setup | DRX-ConfigPTM | Table 14.2.1.2.1.3.3-9 |  |
| } |  |  |  |
| } |  |  |  |
| allowCSI-SRS-Tx-MulticastDRX-Active-r17 | true |  |  |
| } |  |  |  |

Table 14.2.1.2.1.3.3-8: DRX-Config (Table 14.2.1.2.1.3.3-7)

| Derivation Path: 38.508-1 [4], Table 4.6.3.56 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DRX-Config ::= SEQUENCE { |  |  |  |
| drx-onDurationTimer CHOICE { |  |  |  |
| milliSeconds | ms40 |  |  |
| } |  |  |  |
| drx-InactivityTimer | ms10 |  |  |
| drx-HARQ-RTT-TimerDL | 56 | 4 slots |  |
| drx-HARQ-RTT-TimerUL | 56 | 4 slots |  |
| drx-RetransmissionTimerDL | sl80 |  |  |
| drx-RetransmissionTimerUL | sl80 |  |  |
| drx-LongCycleStartOffset CHOICE { |  |  |  |
| ms640 | 7 |  |  |
| } |  |  |  |
| shortDRX | not present |  |  |
| drx-SlotOffset | 0 |  |  |
| } |  |  |  |

Table 14.2.1.2.1.3.3-9: DRX-ConfigPTM (Table 14.2.1.2.1.3.3-7)

| Derivation Path: 38.508-1 [4], Table 4.6.7-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DRX-ConfigPTM-r17 ::= SEQUENCE { |  |  |  |
| drx-onDurationTimerPTM-r17 CHOICE { |  |  |  |
| milliSeconds | ms40 |  |  |
| } |  |  |  |
| drx-InactivityTimerPTM-r17 | ms10 |  |  |
| drx-HARQ-RTT-TimerDL-PTM-r17 | Not present |  |  |
| drx-RetransmissionTimerDL-PTM-r17 | Not present |  |  |
| drx-LongCycleStartOffsetPTM-r17 CHOICE { |  |  |  |
| ms640 | 27 |  |  |
| } |  |  |  |
| drx-SlotOffsetPTM-r17 | 0 |  |  |
| } |  |  |  |

Table 14.2.1.2.1.3.3-10: ServingCellConfig (Table 14.2.1.2.1.3.3-5)

| Derivation Path: 38.508-1 [4], Table 4.6.3-167, condition MBS_Multicast |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| csi-MeasConfig CHOICE { |  |  |  |
| setup | csi-MeasConfig | Table 14.2.1.2.1.3.3-11 |  |
| } |  |  |  |
| } |  |  |  |

Table 14.2.1.2.1.3.3-11: CSI-MeasConfig (Table 14.2.1.2.1.3.3-10)

| Derivation Path: 38.508-1 [4], Table 4.6.3-38 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-MeasConfig::= SEQUENCE { |  |  |  |
| csi-SSB-ResourceSetToAddModList SEQUENCE (SIZE (1..maxNrofCSI-SSB-ResourceSets)) OF CSI-SSB-ResourceSet { | 1 entry |  |  |
| CSI-SSB-ResourceSet[1] | CSI-SSB-ResourceSet | entry 1 |  |
| } |  |  |  |
| csi-ResourceConfigToAddModList SEQUENCE (SIZE (1..maxNrofCSI-ResourceConfigurations)) OF CSI-ResourceConfig { | 1 entry |  |  |
| CSI-ResourceConfig[1] SEQUENCE { | CSI-ResourceConfig | entry 1 |  |
| csi-ResourceConfigId | 0 |  |  |
| csi-RS-ResourceSetList CHOICE { |  |  |  |
| nzp-CSI-RS-SSB SEQUENCE { |  |  |  |
| csi-SSB-ResourceSetList SEQUENCE (SIZE (1..maxNrofCSI-SSB-ResourceSetsPerConfig)) OF CSI-SSB-ResourceSetId { |  |  |  |
| CSI-SSB-ResourceSetId [1] | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| bwp-Id | 0 |  |  |
| resourceType | periodic |  |  |
| } |  |  |  |
| } |  |  |  |
| csi-ReportConfigToAddModList SEQUENCE (SIZE (1..maxNrofCSI-ReportConfigurations)) OF CSI-ReportConfig { | 1 entry |  |  |
| CSI-ReportConfig[1] | CSI-ReportConfig | entry 1 Table 14.2.1.2.1.3.3-12 |  |
| } |  |  |  |
| reportTriggerSize | 0 |  |  |
| } |  |  |  |

Table 14.2.1.2.1.3.3-12: CSI-ReportConfig (Table 14.2.1.2.1.3.3-11)

| Derivation Path: 38.508-1 [4], Table 4.6.3-39 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ReportConfig ::= SEQUENCE { |  |  |  |
| reportConfigId | 0 |  |  |
| carrier | Not present |  |  |
| resourcesForChannelMeasurement | 0 |  |  |
| csi-IM-ResourcesForInterference | Not present |  |  |
| nzp-CSI-RS-ResourcesForInterference | Not present |  |  |
| reportConfigType CHOICE { |  |  |  |
| periodic SEQUENCE { |  |  |  |
| reportSlotConfig CHOICE { |  |  |  |
| slots20 | 9 |  |  |
| } |  |  |  |
| pucch-CSI-ResourceList SEQUENCE (SIZE (1..maxNrofBWPs)) OF PUCCH-CSI-Resource { | 1 entry |  |  |
| PUCCH-CSI-Resource [1] SEQUENCE { |  | entry 1 |  |
| uplinkBandwidthPartId | 0 |  |  |
| pucch-Resource | 9 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportQuantity CHOICE { |  |  |  |
| ssb-Index-RSRP | NULL |  |  |
| } |  |  |  |
| timeRestrictionForChannelMeasurements | notConfigured |  |  |
| timeRestrictionForInterferenceMeasurements | notConfigured |  |  |
| codebookConfig | Not present |  |  |
| dummy | Not present |  |  |
| groupBasedBeamReporting CHOICE { |  |  |  |
| disabled SEQUENCE { |  |  |  |
| nrofReportedRS | n1 |  |  |
| } |  |  |  |
| } |  |  |  |
| cqi-Table | table1 |  |  |
| subbandSize | value2 |  |  |
| } |  |  |  |

Table 14.2.1.2.1.3.3-13: UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST (step 8, Table 14.2.1.2.1.3.2-1)

Derivation Path: 36.508 [6], Table 4.7A-9

Table 14.2.1.2.1.3.3-14: RRCReconfiguration (Step 21, Table 14.2.4.1.2.3.2-1)

| Derivation Path: TS 38.508-1, table 4.6.1-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| masterCellGroup |  | CellGroupConfig | Table 14.2.1.2.1.3.3-15 |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 14.2.1.2.1.3.3-15: CellGroupConfig (Table 14.2.1.2.1.3.3-14)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| mac-CellGroupConfig | MAC-CellGroupConfig | Table 14.2.1.2.1.3.3-16 |  |
| physicalCellGroupConfig | Not present |  |  |
| spCellConfig | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.2.1.2.1.3.3-16: MAC-CellGroupConfig (Table 14.2.1.2.1.3.3-15)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-68, DRX and MBS_Multicast |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MAC-CellGroupConfig ::= SEQUENCE { |  |  |  |
| drx-Config CHOICE { |  |  |  |
| setup | DRX-Config | Table 14.2.1.2.1.3.3-8 |  |
| } |  |  |  |
| g-RNTI-ConfigToAddModList-r17 SEQUENCE (SIZE (1..maxG-RNTI-r17)) OF MBS-RNTI-SpecificConfig-r17 { | 1 entry |  |  |
| MBS-RNTI-SpecificConfig-r17[1] SEQUENCE { |  | entry 1 |  |
| mbs-RNTI-SpecificConfigId-r17 | 0 |  |  |
| groupCommon-RNTI-r17 CHOICE { |  |  |  |
| g-RNTI | RNTI-Value |  |  |
| } |  |  |  |
| drx-ConfigPTM-r17 CHOICE { |  |  |  |
| release |  |  |  |
| } |  |  |  |
| } |  |  |  |
| allowCSI-SRS-Tx-MulticastDRX-Active-r17 | false |  |  |
| } |  |  |  |

<End of modified section 1>