3GPP TSG-RAN5 Meeting #98	R5-230626r2

# Athens, Greece, 27th Feb 2023- 3rd Mar 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 3522 | rev |  | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of MBS Multicast TC 14.2.3.1 and 14.2.3.2-PDCP UM MRB |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS_5MBS_5MBUSA-UEConTest |  |  |  |  |  | Date: |  |  | 2023-02-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | MBS TC 14.2.3.1 and 14.2.3.2 needs to be added to test UE using the correct initial value of state variables for PDCP when receving the first packet for UM MRB. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of MBS TC 14.2.3.1 MBS Multicast / PDCP/ PDCP HFN and SN maintenance / Non-Lossless handover / 12 bit SN and 14.2.3.2 MBS Multicast / PDCP/ PDCP HFN and SN maintenance / Non-Lossless handover / 18 bit SN |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR_MBS_5MBS_5MBUSA-UEConTest WP will be incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 14.2.3.1 and 14.2.3.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.523-2 CR 0317 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | 1st revision: Editorial update 2nd revsion: - Correct the 14.2.3.2.3.3-4/5/10 to Table 14.2.3.1.3.3-4/5/10 in 14.2.3.2 - Update the coversheet |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

14.2.3	MBS Multicast / PDCP

14.2.3.1	MBS Multicast / PDCP/ PDCP HFN and SN maintenance / Non-Lossless handover / 12 bit SN

14.2.3.1.1	Test Purpose (TP)

(1)

with { UE in RRC_Connected state and Multicast MRB established with DL only RLC-UM entity for PTM transmission and initialRX-DELIV-r17 in PDCP-Config for this Multicast MRB is not zero and PDCP configured for 12 bit SN }

ensure that {

when { UE receives a PDCP Data PDU with RCVD_COUNT < initialRX-DELIV }

then { UE discards the PDCP DATA PDU with RCVD_COUNT }

}

(2)

with { UE in RRC_Connected state and Multicast MRB established with DL only RLC-UM entity for PTM transmission and initialRX-DELIV-r17 in PDCP-Config for this Multicast MRB is not zero and PDCP configured for 12 bit SN }

ensure that {

when { UE receives a PDCP Data PDU with RCVD_COUNT = initialRX-DELIV }

then { UE delivers PDCP Data PDU to upper layers }

}

(3)

with { UE is requested to make a non-lossless handover with pdcp re-establishment and initialRX-DELIV-r17 in PDCP-Config for this Multicast MRB is not zero and PDCP configured for 12 bit SN }

ensure that {

when { UE receives a PDCP Data PDU with RCVD_COUNT < initialRX-DELIV }

then {  UE discards the PDCP DATA PDU with  RCVD_COUNT }

}

(4)

with { UE is requested to make a non-lossless handover with pdcp re-establishment and initialRX-DELIV-r17 in PDCP-Config for this Multicast MRB is not zero and PDCP configured for 12 bit SN }

ensure that {

when { UE receives a PDCP Data PDU with RCVD_COUNT = initialRX-DELIV }

then { UE delivers PDCP Data PDU to upper layers }

}

(5)

with { UE is requested to make a non-lossless handover with pdcp re-establishment and initialRX-DELIV-r17 in PDCP-Config is not configured and PDCP configured for 12 bit SN }

ensure that {

when { UE receives a PDCP Data PDU with RCVD_COUNT < RX_DELIV }

then {  UE discards the PDCP DATA PDU with RCVD_COUNT }

}

(6)

with { UE is requested to make a non-lossless handover with pdcp re-establishment and initialRX-DELIV-r17 in PDCP-Config is not configured and PDCP configured for 12 bit SN }

ensure that {

when { UE receives a PDCP Data PDU with RCVD_COUNT = RX_DELIV }

then { UE delivers PDCP Data PDU to upper layers }

}

14.2.3.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.323, clause 5.1.2, 5.2.2.1, 7.1. Unless otherwise stated these are Rel-17 requirements.

[TS 38. 323, clause 5.1.2]

When upper layers request a PDCP entity re-establishment, the receiving PDCP entity shall:

…

-	for SRBs and UM DRBs, set RX_NEXT and RX_DELIV to the initial value;

-	for UM MRBs and AM MRBs, set RX_NEXT and RX_DELIV to the initial value if initialRX-DELIV is configured in TS 38.331 [3];

…

[TS 38. 323, clause 5.2.2.1]

After determining the COUNT value of the received PDCP Data PDU = RCVD_COUNT, the receiving PDCP entity shall:

…

-	if RCVD_COUNT < RX_DELIV; or

-	if the PDCP Data PDU with COUNT = RCVD_COUNT has been received before:

-	discard the PDCP Data PDU;

If the received PDCP Data PDU with COUNT value = RCVD_COUNT is not discarded above, the receiving PDCP entity shall:

-	store the resulting PDCP SDU in the reception buffer;

-	if RCVD_COUNT >= RX_NEXT:

-	update RX_NEXT to RCVD_COUNT + 1.

…

-	if RCVD_COUNT = RX_DELIV:

-	deliver to upper layers in ascending order of the associated COUNT value after performing header decompression, if not decompressed before;

-	all stored PDCP SDU(s) with consecutively associated COUNT value(s) starting from COUNT = RX_DELIV;

-	update RX_DELIV to the COUNT value of the first PDCP SDU which has not been delivered to upper layers, with COUNT value > RX_DELIV;

…

[TS 38.323, clause 7.1]

The receiving PDCP entity shall maintain the following state variables:

a)	RX_NEXT

This state variable indicates the COUNT value of the next PDCP SDU expected to be received. The initial value is 0, except for sidelink broadcast and groupcast, for SRBs configured with state variables continuation, and for broadcast MRBs.

…

b)	RX_DELIV

This state variable indicates the COUNT value of the first PDCP SDU not delivered to the upper layers, but still waited for. The initial value is 0, except for sidelink broadcast and groupcast, for SRBs configured with state variables continuation, and for MRBs. For NR sidelink communication for broadcast and groupcast or sidelink SRB4 for broadcast and groupcast based sidelink discovery, the initial value of the SN part of RX_DELIV is (x – 0.5 × 2[sl-PDCP-SN-Size–1]) modulo (2[sl-PDCP-SN-Size]), where x is the SN of the first received PDCP Data PDU. For broadcast MRBs, the initial value of the SN part of RX_DELIV is set to (x – 0.5 × 2[PDCP-SN-SizeDL–1]) modulo (2[PDCP-SN-SizeDL]), where x is the SN of the first received PDCP Data PDU. For multicast MRBs, the initial value of RX_DELIV is set, if provided, by initialRXDELIV in TS 38.331 [3].

14.2.3.1.3	Test description

14.2.3.1.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1 is the serving cell and NR Cell 2 is a suitable neighbour intra-frequency cell.

-	System information combination NR-1 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used in NR Cell 1 and NR Cell 2.

UE:

-	None.

Preamble:

-	The UE is in state 1N-A on NR Cell 1(serving cell) according to TS 38.508-1 [4] Table 4.4A.2-3 with Test Mode = on to activate UE TEST MODE C and Test Loop Function = off.

-	The UE is made interested in receiving MBS Multicast service with MBS service ID '000101'H.

14.2.3.1.3.2	Test procedure sequence

Table 14.2.3.1.3.2-1 for FR1 and table 14.2.3.1.3.2-2 for FR2 illustrate the downlink power levels to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1" and "T2" are to be applied subsequently in the Main behaviour. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 14.2.3.1.3.2-1: Cell configuration changes over time for FR1

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | Remarks |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -88 | “Off” | Power level “Off” is defined in TS 38.508-1 [4] Table 6.2.2.1-3 |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -88 | -82 |  |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | -82 | -88 |  |

Table 14.2.3.1.3.2-2: Cell configuration changes over time for FR2

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | Remarks |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -91 | “Off” | Power level “Off” is defined in TS 38.508-1 [4] Table 6.2.2.2-2 |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -91 | -82 |  |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | -82 | -91 |  |

Table 14.2.3.1.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1a1-1b12a1 | Steps 1a1 to 1b12a1 of the generic procedures described in TS 38.508-1 subclause 4.9.34 are performed on NR Cell 1 to establish an associated PDU Session to the MBS DNN and join in MBS Multicast session. | - | - | - | - |
| 2a1-2a2 | Steps 9a1 to 9a2 of the generic procedures described in TS 38.508-1 subclause 4.5.4.2-3 are performed on NR Cell 1 with condition UE TEST LOOP MODE C and Multicast MRB. | - | - | - | - |
| 3 | SS sets TX_NEXT = (K-1). UE sets RX_NEXT = 0 and sets RX_DELIV = initialRXDELIV = K. (Note 1) | - | - | - | - |
| 4 | The SS sends the PDCP Data PDU #0 via RLC-UM of MRB with the following content to the UE: D/C field = 1 (PDCP Data PDU) and PDCP SN = (K-1). After having sent a PDU, the SS sets TX_NEXT= K. (Note 1) After receiving the PDU, UE discards it because RCVD_COUNT < RX_DELIV. | <-- | MBS Packet (PDCP Data PDU #0) | - | - |
| 5 | The SS transmits a UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST message. | <-- | NR RRC: DLInformationTransfer TC: UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST | - | - |
| 6 | UE responds with UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE. | --> | NR RRC: ULInformationTransfer TC: UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE | - | - |
| 7 | Check: Is the number of reported MBS Packets received on the MRB in step 6 equal to 0? | - | - | 1 | P |
| 8 | The SS sends the PDCP Data PDU #1 via RLC-UM of MRB with the following content to the UE: D/C field = 1 (PDCP Data PDU) and PDCP SN = K. After having sent a PDU, the SS sets TX_NEXT= (K+1). (Note 1) After receiving the PDU, UE delivers it to upper layer and sets RX_NEXT to (K+1) and sets RX_DELIV to (K+1). | <-- | MBS Packet (PDCP Data PDU #1) | - | - |
| 9 | The SS transmits a UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST message. | <-- | NR RRC: DLInformationTransfer TC: UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST | - | - |
| 10 | UE responds with UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE. | --> | NR RRC: ULInformationTransfer TC: UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE | - | - |
| 11 | Check: Is the number of reported MBS Packets received on the MRB in step 10 equal to 1? | - | - | 2 | P |
| 12 | The SS sends the PDCP Data PDU #2 via RLC-UM of MRB with the following content to the UE: D/C field = 1 (PDCP Data PDU) and PDCP SN = 0. After having sent a PDU, the SS sets TX_NEXT= (K+2). (Note 1) After receiving the PDU, UE delivers it to upper layer and sets RX_NEXT to (K+2) and sets RX_DELIV to (K+2). | <-- | MBS Packet (PDCP Data PDU #2) | - | - |
| 13 | The SS creates a PDCP Data PDU#3 (not transmitted). | - | - | - | - |
| 14 | The SS changes NR Cell 2 power level according to the row "T1" in table 14.2.3.1.3.2-1 (FR1) / 14.2.3.1.3.2-2 (FR2). | - | - | - | - |
| 15 | The SS transmits NR RRCReconfiguration message to perform PCell change from NR Cell1 to NR Cell2 and sets initialRXDELIV= K+3. (Note 1) | <-- | RRCReconfiguration | - | - |
| 16 | The UE transmits a NR RRCReconfigurationComplete message. | --> | RRCReconfigurationComplete | - | - |
| 17 | The SS sends the PDCP Data PDU #3 via RLC-UM of MRB with the following content to the UE: D/C field = 1 (PDCP Data PDU) and PDCP SN = 1. After having sent a PDU, the SS sets TX_NEXT=(K+3). (Note 1) After receiving the PDU, UE discards it because RCVD_COUNT < RX_DELIV. | <-- | MBS Packet (PDCP Data PDU #3) | - | - |
| 18 | The SS transmits a UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST message. | <-- | NR RRC: DLInformationTransfer TC: UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST | - | - |
| 19 | UE responds with UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE. | --> | NR RRC: ULInformationTransfer TC: UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE | - | - |
| 20 | Check: Is the number of reported MBS Packets received on the MRB in step 19 equal to 1? | - | - | 3 | P |
| 21 | The SS sends the PDCP Data PDU #4 via RLC-UM of MRB with the following content to the UE: D/C field = 1 (PDCP Data PDU) and PDCP SN = 2. After having sent a PDU, the SS sets TX_NEXT= (K+4). (Note 1) After receiving the PDU, UE delivers it to upper layer and sets RX_NEXT to (K+4) and sets RX_DELIV to (K+4). | <-- | MBS Packet (PDCP Data PDU #4) | - | - |
| 22 | The SS transmits a UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST message. | <-- | NR RRC: DLInformationTransfer TC: UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST | - | - |
| 23 | UE responds with UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE. | --> | NR RRC: ULInformationTransfer TC: UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE | - | - |
| 24 | Check: Is the number of reported MBS Packets received on the MRB in step 23 equal to 2? | - | - | 4 | P |
| 25 | The SS changes NR Cell 1 and NR Cell 2 power level according to the row "T2" in table 14.2.3.1.3.2-1 (FR1) / 14.2.3.1.3.2-2 (FR2). | - | - | - | - |
| 26 | The SS transmits NR RRCReconfiguration message to perform PCell change from NR Cell 2 to NR Cell 1. | <-- | RRCReconfiguration | - | - |
| 27 | The UE transmits a NR RRCReconfigurationComplete message. | --> | RRCReconfigurationComplete | - | - |
| 28 | The SS sends the PDCP Data PDU #1 via RLC-UM of MRB with the following content to the UE. After receiving the PDU, UE discards it because RCVD_COUNT < RX_DELIV. | <-- | MBS Packet (PDCP Data PDU #3) | - | - |
| 29 | The SS transmits a UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST message. | <-- | NR RRC: DLInformationTransfer TC: UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST | - | - |
| 30 | UE responds with UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE. | --> | NR RRC: ULInformationTransfer TC: UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE | - | - |
| 31 | Check: Is the number of reported MBS Packets received on the MRB in step 30 equal to 2? | - | - | 5 | P |
| 32 | The SS sends the PDCP Data PDU #5 via RLC-UM of MRB with the following content to the UE: D/C field = 1 (PDCP Data PDU) and PDCP SN = 3. After having sent a PDU, the SS sets TX_NEXT= (K+5). (Note 1) After receiving the PDU, UE delivers it to upper layer and sets RX_NEXT to (K+5) and sets RX_DELIV to (K+5). | <-- | MBS Packet (PDCP Data PDU #5) | - | - |
| 33 | The SS transmits a UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST message. | <-- | NR RRC: DLInformationTransfer TC: UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST | - | - |
| 34 | UE responds with UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE. | --> | NR RRC: ULInformationTransfer TC: UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE | - | - |
| 35 | Check: Is the number of reported MBS Packets received on the MRB in step 34 equal to 3? | - | - | 6 | P |
| Note 1: K = (2^[PDCP-SN-SizeDL])-1. If PDCP-SN-SizeDL=12, K=4095. If PDCP-SN-SizeDL=18, K= 262143. |  |  |  |  |  |

14.2.3.1.3.3	Specific message contents

Table 14.2.3.1.3.3-1: ACTIVATE TEST MODE (preamble, Table 14.2.3.1.3.2-1)

Derivation Path: TS 36.508 [6], Table 4.7A-1, condition UE TEST LOOP MODE C

Table 14.2.3.1.3.3-2: RRCReconfiguration (step 1a15, Table 14.2.3.1.3.2-1)

| Derivation Path: TS 38.508-1 [4],Table 4.6.1-13 and condition NR |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |  |
| radioBearerConfig |  | RadioBearerConfig | Table 14.2.3.1.3.3-4 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| masterCellGroup |  | CellGroupConfig with condition MRBm and UM_PTM | m=1 |  |
| dedicatedNAS-MessageList SEQUENCE (SIZE(1..maxDRB)) OF DedicatedNAS-Message {} |  | DedicatedNAS-Message |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 14.2.3.1.3.3-3: RRCReconfiguration (step 1b10, Table 14.2.3.1.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 and condition NR |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |  |
| radioBearerConfig |  | RadioBearerConfig | Table 14.2.3.1.3.3-5 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| masterCellGroup |  | CellGroupConfig with condition MRBm_DRBn and UM_PTM | n is set to the same value as for the radioBearerConfig IE above m=1 |  |
| dedicatedNAS-MessageList SEQUENCE (SIZE(1..maxDRB)) OF DedicatedNAS-Message {} |  | DedicatedNAS-Message |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 14.2.3.1.3.3-4: RadioBearerConfig (Table 14.2.3.1.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-132, condition MRBm and UM_PTM (m=1) |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioBearerConfig ::= SEQUENCE { |  |  |  |
| mrb-ToAddModList-r17 SEQUENCE (SIZE (1..maxDRB)) OF MRB-ToAddMod-r17 { | 1 entry |  |  |
| MRB-ToAddMod-r17 [1] SEQUENCE { |  | entry 1 |  |
| pdcp-Config-r17 | PDCP-Config | Table 14.2.3.1.3.3-6 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.2.3.1.3.3-5: RadioBearerConfig (Table 14.2.3.1.3.3-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-132, condition DRBn and MRBm and UM_PTM (Note 1) |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioBearerConfig ::= SEQUENCE { |  |  |  |
| mrb-ToAddModList-r17 SEQUENCE (SIZE (1..maxDRB)) OF MRB-ToAddMod-r17 { | 1 entry |  |  |
| MRB-ToAddMod-r17 [1] SEQUENCE { |  | entry 1 |  |
| pdcp-Config-r17 | PDCP-Config | Table 14.2.3.1.3.3-6 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| Note 1: n is chosen as the next available number higher or equal to 2. m =1. |  |  |  |

Table 14.2.3.1.3.3-6: PDCP-Config (Table 14.2.3.1.3.3-4, Table 14.2.3.1.3.3-5)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-99 and condition UM_MRB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCP-Config ::= SEQUENCE { |  |  |  |
| drb SEQUENCE { |  |  |  |
| pdcp-SN-Size-UL | Not present |  |  |
| pdcp-SN-Size-DL | len12bits |  |  |
| } |  |  |  |
| initialRX-DELIV-r17 | 4095 |  |  |
| } |  |  |  |

Table 14.2.3.1.3.3-7: CLOSE UE TEST LOOP (step 2a1, Table 14.2.3.1.3.2-1)

Derivation Path: 38.508-1 [4], Table 4.7A-3, condition UE TEST LOOP MODE C and Multicast MRB

Table 14.2.3.1.3.3-8: UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST (step 5, step9, step 18, step 22, step 29 and step33, Table 14.2.3.1.3.2-1)

Derivation Path: 36.508 [6], Table 4.7A-9

Table 14.2.3.1.3.3-9: RRCReconfiguration (step 15 and step 26, Table 14.2.3.1.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 and condition NR |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |  |
| radioBearerConfig |  | RadioBearerConfig | Table 14.2.3.1.3.3-10 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| masterCellGroup |  | CellGroupConfig | Table 14.2.3.1.3.3-12 |  |
| masterKeyUpdate ::= SEQUENCE { |  |  |  |  |
| keySetChangeIndicator |  | false |  |  |
| nextHopChainingCount |  | 0 |  |  |
| nas-Container |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 14.2.3.1.3.3-10: RadioBearerConfig (Table 14.2.3.1.3.3-9)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-132 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioBearerConfig ::= SEQUENCE { |  |  |  |
| srb-ToAddModList SEQUENCE (SIZE (1..2)) OF SRB-ToAddMod { | 2 entries |  |  |
| SRB-ToAddMod[1] SEQUENCE { |  | entry 1 |  |
| SRB-Identity | SRB-Identity with condition SRB1 |  |  |
| reestablishPDCP | true |  |  |
| } |  |  |  |
| SRB-ToAddMod[2] SEQUENCE { |  | entry 2 |  |
| SRB-Identity | SRB-Identity with condition SRB2 |  |  |
| reestablishPDCP | true |  |  |
| } |  |  |  |
| } |  |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod { | n entries | n is the number of DRBs |  |
| DRB-ToAddMod[k, k=1..n] SEQUENCE { |  | entry [k, k=1..n] |  |
| cnAssociation CHOICE { |  |  |  |
| sdap-Config | SDAP-Config |  |  |
| } |  |  |  |
| drb-Identity | DRB-Identity with condition DRBk |  |  |
| reestablishPDCP | true |  |  |
| pdcp-Config | PDCP-Config |  |  |
| } |  |  |  |
| } |  |  |  |
| securityConfig SEQUENCE { |  |  |  |
| securityAlgorithmConfig | SecurityAlgorithmConfig |  |  |
| keyToUse | master |  |  |
| } |  |  |  |
| mrb-ToAddModList-r17 SEQUENCE (SIZE (1..maxDRB)) OF MRB-ToAddMod-r17 { | 1 entry |  |  |
| MRB-ToAddMod-r17 [1] SEQUENCE { |  | entry 1 |  |
| mbs-SessionId-r17 | TMGI |  |  |
| mrb-Identity-r17 | MRB-Identity with condition MRBm | m=1 |  |
| reestablishPDCP-r17 | true |  |  |
| pdcp-Config-r17 | PDCP-Config | Table 14.2.3.1.3.3-11 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.2.3.1.3.3-11: PDCP-Config (Table 14.2.3.1.3.3-10)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-99, condition UM_MRB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCP-Config ::= SEQUENCE { |  |  |  |
| drb SEQUENCE { |  |  |  |
| pdcp-SN-Size-UL | Not present |  |  |
| pdcp-SN-Size-DL | len12bits |  |  |
| } |  |  |  |
| initialRX-DELIV-r17 | 4098 |  | Step 15 |
|  | Not present |  | Step 26 |
| } |  |  |  |

Table 14.2.3.1.3.3-12: CellGroupConfig (Table 14.2.3.1.3.3-9)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19, condition PCell_change |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLCH)) OF RLC-BearerConfig { | 3+n entries | n is the number of DRBs established before RRC re-establishement |  |
| RLC-BearerConfig[1] | RLC-BearerConfig with condition SRB1 and Re-establish_RLC | entry 1 |  |
| RLC-BearerConfig[2] | RLC-BearerConfig with condition SRB2 and Re-establish_RLC | entry 2 |  |
| RLC-BearerConfig[k+2, k=1..n] | RLC-BearerConfig with condition DRBk and Re-establish_RLC | entry [k+2, k=1..n] |  |
| RLC-BearerConfig[n+1] | RLC-BearerConfig with conditions UM_DLonly and PTM and MRBm and Re-establish_RLC | entry n+1 m=1 |  |
| } |  |  |  |
| mac-CellGroupConfig | MAC-CellGroupConfig with condition MBS_Multicast |  |  |
| physicalCellGroupConfig | PhysicalCellGroupConfig |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| spCellConfigDedicated | ServingCellConfig with condition MBS_Multicast |  |  |
| } |  |  |  |
| } |  |  |  |

14.2.3.2	MBS Multicast / PDCP/ PDCP HFN and SN maintenance / Non-Lossless handover / 18 bit SN

14.2.3.2.1	Test Purpose (TP)

(1)

with { UE in RRC_Connected state and Multicast MRB established with DL only RLC-UM entity for PTM transmission and initialRX-DELIV-r17 in PDCP-Config for this Multicast MRB is not zero and PDCP configured for 18 bit SN }

ensure that {

when { UE receives a PDCP Data PDU with RCVD_COUNT < initialRX-DELIV }

then { UE discards the PDCP DATA PDU with RCVD_COUNT }

}

(2)

with { UE in RRC_Connected state and Multicast MRB established with DL only RLC-UM entity for PTM transmission and initialRX-DELIV-r17 in PDCP-Config for this Multicast MRB is not zero and PDCP configured for 18 bit SN }

ensure that {

when { UE receives a PDCP Data PDU with RCVD_COUNT = initialRX-DELIV }

then { UE delivers PDCP Data PDU to upper layers }

}

(3)

with { UE is requested to make a non-lossless handover with pdcp re-establishment and initialRX-DELIV-r17 in PDCP-Config for this Multicast MRB is not zero and PDCP configured for 18 bit SN }

ensure that {

when { UE receives a PDCP Data PDU with RCVD_COUNT < initialRX-DELIV }

then {  UE discards the PDCP DATA PDU with  RCVD_COUNT }

}

(4)

with { UE is requested to make a non-lossless handover with pdcp re-establishment and initialRX-DELIV-r17 in PDCP-Config for this Multicast MRB is not zero and PDCP configured for 18 bit SN }

ensure that {

when { UE receives a PDCP Data PDU with RCVD_COUNT = initialRX-DELIV }

then { UE delivers PDCP Data PDU to upper layers }

}

(5)

with { UE is requested to make a non-lossless handover with pdcp re-establishment and initialRX-DELIV-r17 in PDCP-Config is not configured and PDCP configured for 18 bit SN }

ensure that {

when { UE receives a PDCP Data PDU with RCVD_COUNT < RX_DELIV }

then {  UE discards the PDCP DATA PDU with RCVD_COUNT }

}

(6)

with { UE is requested to make a non-lossless handover with pdcp re-establishment and initialRX-DELIV-r17 in PDCP-Config is not configured and PDCP configured for 18 bit SN }

ensure that {

when { UE receives a PDCP Data PDU with RCVD_COUNT = RX_DELIV }

then { UE delivers PDCP Data PDU to upper layers }

}

14.2.3.2.2	Conformance requirements

Same as conformance requirements in clause 14.2.3.1.2

14.2.3.2.3	Test description

14.2.3.2.3.1	Pre-test conditions

Same as pre-test conditions in clause 14.2.3.1.3.1

14.2.3.2.3.2	Test procedure sequence

Same as test procedure sequence in clause 14.2.3.1.3.2.

14.2.3.2.3.3	Specific message contents

Same as specific message contents in clause 14.2.3.1.3.3 with exception of Tables 14.2.3.1.3.3-6 and 14.2.3.1.3.3-11. Instead the Tables 14.2.3.2.3.3-6 and 14.2.3.2.3.3-11 below apply:

Table 14.2.3.2.3.3-6: PDCP-Config (Table 14.2.3.1.3.3-4, Table 14.2.3.1.3.3-5)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-99, condition UM_MRB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCP-Config ::= SEQUENCE { |  |  |  |
| drb SEQUENCE { |  |  |  |
| pdcp-SN-Size-UL | Not present |  |  |
| pdcp-SN-Size-DL | len18bits |  |  |
| } |  |  |  |
| initialRX-DELIV-r17 | 262143 |  |  |
| } |  |  |  |

Table 14.2.3.2.3.3-11: PDCP-Config (Table 14.2.3.1.3.3-10)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-99, condition UM_MRB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCP-Config ::= SEQUENCE { |  |  |  |
| drb SEQUENCE { |  |  |  |
| pdcp-SN-Size-UL | Not present |  |  |
| pdcp-SN-Size-DL | len18bits |  |  |
| } |  |  |  |
| initialRX-DELIV-r17 | 262146 |  | Step 15 |
|  | Not present |  | Step 26 |
| } |  |  |  |

<End of modified section 1>