3GPP TSG-RAN5 Meeting #98	R5-230644r1

# Athens, Greece, 27th Feb 2023- 3rd Mar 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 3531 | rev |  | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of RedCap TC 7.1.1.8.3-BWP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2023-02-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Test case only need one NR Cell, but it includes NR Cell 1 and NR Cell 2 in different part of the test case. - SearchSpace for Other Systeminformation in Table 7.1.1.8.3.3.3-5 have already been deleted from commonSearchSpaceList in the last meeting, so the IE of searchSpaceOtherSystemInformation should be set to Not present. - pucch-ResourceCommon in initialUplinkBWP is set to 0, pucch-ResourceCommonRedCap in initialUplinkBWP-RedCap is also set to 0.Non-conformant UE which always use pucch-ResourceCommon may pass. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - All the NR Cell in the test case will be changed to NR Cell 2 in order to ensure a Non-Conformat RedCap UE not using RedCap-specifc DL BWP could not receive Paging. According to default configure in 38.523-3, PF for NR Cell 2 is in Slot#0 and Slot #1. Paging in the RedCap-specifc DL BWP is configured in slot #2. [38.523-3] NR Cell 1 use SSB#1 and Paging Frame is in Slot#1 and #2; NR Cell 2 use SSB#0 and Paging Frame is in Slot#0 and #1; - Set searchSpaceOtherSystemInformation to Not present in Table 7.1.1.8.3.3.3-5. - Set pucch-ResourceCommonRedCap in initialUplinkBWP-RedCap to 1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Configuration for the NR Cell number is wrong. - Configuration for searchSpaceOtherSystemInformation is wrong. - Non-conformant UE may pass. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.8.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | 1st revision: 1.update coversheet |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

7.1.1.8.3	Separate BWP / IDLE / RedCap

7.1.1.8.3.1	Test Purpose (TP)

(1)

with { UE supporting RedCap and in NR RRC_IDLE state }

ensure that {

when { initialDownlinkBWP-RedCap is configured }

then { the UE shall monitor the PDCCH on the BWP configured by initialDownlinkBWP-RedCap }

}

(2)

with { UE supporting RedCap and in NR RRC_IDLE state }

ensure that {

when { initialUplinkBWP-RedCap is configured and the UE needs to perform the Random Access procedure }

then { the UE shall perform the Random Access procedure by using the BWP configured by initialUplinkBWP-RedCap }

}

7.1.1.8.3.2	Conformance requirements

References: The conformance requirements covered in the present test case are specified in: TS 38.321, clause 5.15.1. Unless otherwise stated these are Rel-17 requirements.

[TS 38.321, clause 5.15.1]

A RedCap UE may be configured with a RedCap-specific initial UL BWP in initialUplinkBWP-RedCap, as specified in TS 38.331 [5].

Upon initiation of the Random Access procedure on a Serving Cell, after the selection of carrier for performing Random Access procedure as specified in clause 5.1.1, the MAC entity shall for the selected carrier of this Serving Cell:

1>	if PRACH occasions are not configured for the active UL BWP:

2>	if the UE is a RedCap UE; and

2>	if initialUplinkBWP-RedCap is configured:

3>	switch the active UL BWP to BWP configured by initialUplinkBWP-RedCap.

2>	else:

3>	switch the active UL BWP to BWP indicated by initialUplinkBWP.

2>	if the Serving Cell is an SpCell:

3>	if the UE is a RedCap UE; and

3>	if initialDownlinkBWP-RedCap is configured:

4>	switch the active DL BWP to BWP configured by initialDownlinkBWP-RedCap.

3>	else:

4>	switch the active DL BWP to BWP indicated by initialDownlinkBWP.

1>	else:

2>	if the Serving Cell is an SpCell:

3>	if the active DL BWP does not have the same bwp-Id as the active UL BWP:

4>	switch the active DL BWP to the DL BWP with the same bwp-Id as the active UL BWP.

1>	stop the bwp-InactivityTimer associated with the active DL BWP of this Serving Cell, if running.

1>	if the Serving Cell is SCell:

2>	stop the bwp-InactivityTimer associated with the active DL BWP of SpCell, if running.

1>	perform the Random Access procedure on the active DL BWP of SpCell and active UL BWP of this Serving Cell.

If the MAC entity receives a PDCCH for BWP switching of a Serving Cell, the MAC entity shall:

1>	if there is no ongoing Random Access procedure associated with this Serving Cell; or

1>	if the ongoing Random Access procedure associated with this Serving Cell is successfully completed upon reception of this PDCCH addressed to C-RNTI (as specified in clauses 5.1.4, 5.1.4a, and 5.1.5):

2>	cancel, if any, triggered consistent LBT failure for this Serving Cell;

2>	perform BWP switching to a BWP indicated by the PDCCH.

…

Upon reception of RRC (re-)configuration for BWP switching for a Serving Cell, cancel any triggered LBT failure in this Serving Cell.

The MAC entity shall for each activated Serving Cell configured with bwp-InactivityTimer:

1>	if the defaultDownlinkBWP-Id is configured, and the active DL BWP is not the BWP indicated by the defaultDownlinkBWP-Id, and the active DL BWP is not the BWP indicated by the dormantBWP-Id if configured; or

1>	if the defaultDownlinkBWP-Id is not configured, and if the UE is not a RedCap UE, and the active DL BWP is not the initialDownlinkBWP, and the active DL BWP is not the BWP indicated by the dormantBWP-Id if configured; or

1>	if the defaultDownlinkBWP-Id is not configured and if the UE is a RedCap UE, and initialDownlinkBWP-RedCap is not configured, and the active DL BWP is not the initialDownlinkBWP, and the active DL BWP is not the BWP indicated by the dormantBWP-Id if configured; or

1>	if the defaultDownlinkBWP-Id is not configured and if the UE is a RedCap UE, and initialDownlinkBWP-RedCap is configured, the active DL BWP is not the initialDownlinkBWP-RedCap, and the active DL BWP is not the BWP indicated by the dormantBWP-Id if configured:

2>	if a PDCCH addressed to C-RNTI or CS-RNTI indicating downlink assignment or uplink grant is received on the active BWP; or

2>	if a PDCCH addressed to G-RNTI or G-CS-RNTI configured for multicast indicating downlink assignment is received on the active BWP; or

2>	if a PDCCH addressed to C-RNTI or CS-RNTI indicating downlink assignment or uplink grant is received for the active BWP; or

2>	if a MAC PDU is transmitted in a configured uplink grant and LBT failure indication is not received from lower layers; or

2>	if a MAC PDU is received in a configured downlink assignment for unicast or MBS multicast:

3>	if there is no ongoing Random Access procedure associated with this Serving Cell; or

3>	if the ongoing Random Access procedure associated with this Serving Cell is successfully completed upon reception of this PDCCH addressed to C-RNTI (as specified in clauses 5.1.4, 5.1.4a and 5.1.5):

4>	start or restart the bwp-InactivityTimer associated with the active DL BWP.

2>	if the bwp-InactivityTimer associated with the active DL BWP expires:

3>	if the defaultDownlinkBWP-Id is configured:

4>	perform BWP switching to a BWP indicated by the defaultDownlinkBWP-Id.

3>	else:

4>	if the UE is a RedCap UE; and

4>	if initialDownlinkBWP-RedCap is configured:

5>	perform BWP switching to the initialDownlinkBWP-RedCap.

4>	else:

5>	perform BWP switching to the initialDownlinkBWP.

NOTE:	If a Random Access procedure is initiated on an SCell, both this SCell and the SpCell are associated with this Random Access procedure.

1>	if a PDCCH for BWP switching is received, and the MAC entity switches the active DL BWP:

2>	if the defaultDownlinkBWP-Id is configured, and the MAC entity switches to the DL BWP which is not indicated by the defaultDownlinkBWP-Id and is not indicated by the dormantBWP-Id if configured; or

2>	if the defaultDownlinkBWP-Id is not configured, and the MAC entity switches to the DL BWP which is not the initialDownlinkBWP and is not indicated by the dormantBWP-Id if configured:

3>	start or restart the bwp-InactivityTimer associated with the active DL BWP.

Upon initiation of the Random Access procedure, after selection of the carrier for performing Random Access procedure as specified in clause 5.1.1, if the UE is a RedCap UE in RRC_IDLE or RRC_INACTIVE mode, the MAC entity shall:

1>	if initialUplinkBWP-RedCap is configured:

2>	perform the Random Access procedure as specified in clause 5.1 by using the BWP configured by initialUplinkBWP-RedCap.

1>	else:

2>	perform the Random Access procedure as specified in clause 5.1 by using the BWP configured by initialUplinkBWP.

1>	if initialDownlinkBWP-RedCap is configured:

2>	monitor the PDCCH on the BWP configured by initialDownlinkBWP-RedCap.

1>	else:

2>	monitor the PDCCH on the BWP configured by initialDownlinkBWP.

7.1.1.8.3.3	Test description

7.1.1.8.3.3.1	Pre-test conditions

System Simulator:

-	NR Cell 2.

UE:

-	None

Preamble:

-	The UE is in NR RRC_Idle mode (state 1N-A) on NR Cell 2 according to 38.508-1 [4] Table 4.4A.2-1.

7.1.1.8.3.3.2	Test procedure sequence

Table 7.1.1.8.3.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS changes the SIB1 of NR Cell 2 to configure initialDownlinkBWP-RedCap and initialUplinkBWP-RedCap. | - | - | - | -- |
| 2 | The SS transmits a Short message on PDCCH using P-RNTI indicating a systemInfoModification. | <-- | PDCCH (DCI 1_0): Short Message | - | - |
| 3 | Wait for 2.1* modification period second for the UE to receive new system information. (Note 1) | - | - | - | - |
| 4 | The SS transmits a Paging message within the BWP configured by initialDownlinkBWP-RedCap. | <-- | NR RRC: Paging | - | - |
| 5 | Check: Does the UE transmit an RRCSetupRequest message by setting LCID to 35? | --> | NR RRC: RRCSetupRequest | 1 | P |
| 6 | The SS transmits an RRCSetup message. | <-- | NR RRC: RRCSetup | - | - |
| 7 | Check: Does the UE transmit HARQ ACK for step 6 on PUCCH configured by initialUplinkBWP-RedCap? | --> | HARQ ACK | 2 | P |
| 8-13a1 | Steps 4-9a1 of Generic procedure as specified in TS 38.508-1 [4] Table 4.9.4.2.2-1 are performed. | - | - | - | - |
| Note 1: The modification period, expressed in number of radio frames = modificationPeriodCoeff * defaultPagingCycle. |  |  |  |  |  |

7.1.1.8.3.3.3	Specific message contents

Table 7.1.1.8.3.3.3-1: SIB1 (step 1, Table 7.1.1.8.3.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |  |
| servingCellConfigCommon |  | ServingCellConfigCommonSIB | Table 7.1.1.8.3.3.3-2 |  |
| } |  |  |  |  |

Table 7.1.1.8.3.3.3-2: ServingCellConfigCommonSIB (Table 7.1.1.8.3.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-169 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommonSIB ::= SEQUENCE { |  |  |  |
| downlinkConfigCommon | DownlinkConfigCommonSIB | Table 7.1.1.8.3.3.3-3 |  |
| uplinkConfigCommon-v1700 ::= SEQUENCE { |  |  |  |
| initialUplinkBWP-RedCap-r17 | BWP-UplinkCommon | Table 7.1.1.8.3.3.3-8 |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.8.3.3.3-3: DownlinkConfigCommonSIB (Table 7.1.1.8.3.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-53 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DownlinkConfigCommonSIB ::= SEQUENCE { |  |  |  |
| pei-Config-r17 | Not present |  |  |
| initialDownlinkBWP-RedCap-r17 | BWP-DownlinkCommon-RedCap | Table 7.1.1.8.3.3.3-4 |  |
| } |  |  |  |

Table 7.1.1.8.3.3.3-4: BWP-DownlinkCommon-RedCap (Table 7.1.1.8.3.3.3-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-10 with condition InitialBWP_SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-DownlinkCommon ::= SEQUENCE { |  |  |  |
| pdcch-ConfigCommon CHOICE { |  |  |  |
| setup | PDCCH-ConfigCommon | Table 7.1.1.8.3.3.3-5 |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.8.3.3.3-5: PDCCH-ConfigCommon (Table 7.1.1.8.3.3.3-4)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-96 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCCH-ConfigCommon ::= SEQUENCE { |  |  |  |
| commonSearchSpaceList SEQUENCE (SIZE (1..4)) OF SearchSpace { | 2 entries |  |  |
| SearchSpace[1] | SearchSpace with condition CSS as specified in TS 38.508-1 [4] Table 4.6.3-162 | entry 1 |  |
| SearchSpace[2] | SearchSpace | entry 2, Table 7.1.1.8.3.3.3-6 |  |
| } |  |  |  |
| searchSpaceOtherSystemInformation | Not present |  |  |
| pagingSearchSpace | 4 |  |  |
| } |  |  |  |

Table 7.1.1.8.3.3.3-6: SearchSpace (Table 7.1.1.8.3.3.3-5)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-162 with condition CSS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpace ::= SEQUENCE { |  |  |  |
| searchSpaceId | 4 |  |  |
| monitoringSlotPeriodicityAndOffset CHOICE { |  |  |  |
| sl10 | 2 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.8.3.3.3-7: Void

Table 7.1.1.8.3.3.3-8: BWP-UplinkCommon (Table 7.1.1.8.3.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-14 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-UplinkCommon ::= SEQUENCE { |  |  |  |
| pucch-ConfigCommon CHOICE { |  |  |  |
| setup | PUCCH-ConfigCommon | Table 7.1.1.8.3.3.3-9 |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.8.3.3.3-9: PUCCH-ConfigCommon (Table 7.1.1.8.3.3.3-8)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-113 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUCCH-ConfigCommon ::= SEQUENCE { |  |  |  |
| pucch-ResourceCommon | Not present |  |  |
| nrofPRBs | Not present |  |  |
| intra-SlotFH-r17 | fromLowerEdge |  |  |
| pucch-ResourceCommon-RedCap-r17 | 1 |  |  |
| additionalPRBOffset-r17 | n2 |  |  |
| } |  |  |  |

<End of modified section 1>