3GPP TSG-RAN5 Meeting #98	R5-230669r1

# Athens, Greece, 27th Feb 2023 - 3rd Mar 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-1 | CR | 1515 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to Annex A.2.14 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2023-02-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 181 Call is being forwarded it is used in supplementary test case scenarios when caller calls User-A and it does not get any response ( 8.41TC) or User-A has Call forwarding unconditional supplementary service activated (8.8 TC), in both cases call is forwarded to User-B. From [TS 24.604, clause 4.2.1.2]: The CFU service enables a served user to have the network redirect to another user communications which are addressed to the served user's address. From above statement can be pointed that 181 message is not sent by user-A or user-B but by the server, meaning than To tag set in 181 message cannot be one used in previous or subsequent dialogs being established. During test case execution, 181 To tag must be set to a different value only use for 181 Call is being forwarded message. Examples for 8.8 and 8.41 TCs flows can be found in RFC 5359 clause 2.7. Call Forwarding Unconditional and 2.9. Call Forwarding - No Answer. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Corrected To tag in 181 Call is being forwarded message. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.2.14 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-230669r1 : Updated the text for the change to bring it inline with 3gpp rules. |  |  |  |  |  |  |  |  |

## A.2.14	181 Call is being forwarded

| Header/param | Cond | Value/remark | Rel | Reference |
| --- | --- | --- | --- | --- |
| Status-Line |  |  |  | RFC 3261 [15] |
| SIP-Version |  | SIP/2.0 |  |  |
| Status-Code |  | 181 |  |  |
| Reason-Phrase |  | Call is being forwarded |  |  |
| Via |  |  |  | RFC 3261 [15] |
| via-parm |  | same value as received in INVITE message |  |  |
| From |  |  |  | RFC 3261 [15] |
| addr-spec |  | same value as received in INVITE message |  |  |
| tag |  | local tag of the dialog ID |  |  |
| To |  |  |  | RFC 3261 [15] |
| addr-spec |  | same value as received in INVITE message |  |  |
| tag |  | the tag shall be different than the one used in any other step, as this response is not sent from any of the UEs involved but from a server |  |  |
| History-Info |  |  |  | RFC 7044 [83] |
| hi-targeted-to-uri |  | <sip:user@company.com> |  |  |
| hi-index |  | 1 |  |  |
| Call-ID |  |  |  | RFC 3261 [15] |
| callid |  | same value as received in INVITE message |  |  |
| CSeq |  |  |  | RFC 3261 [15] |
| value |  | same value as received in INVITE message |  |  |
| Content-Length |  |  |  | RFC 3261 [15] |
| value |  | 0 |  |  |