3GPP TSG-RAN5 Meeting #98	R5-230716r1

# Athens, Greece, 27th Feb 2023 - 3rd Mar 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-2 | CR | 0435 | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add Measurement Capabilities for SFTD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-02-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The UE capabilities for SFTD measurements between the NR PCell and a configured E-UTRA PSCell is needed for some NE-DC test cases. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added UE capabilities for SFTD measurements between the NR PCell and a configured E-UTRA PSCell. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE capabilities for SFTD measurements in NE-DC will be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 38.523-2 R5-230612 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision 1: Keep only 1 PICS for SFTD measurements between the NR PCell and a configured E-UTRA PSCell. |  |  |  |  |  |  |  |  |

### A.4.3.6	Measurement Capabilities

Table A.4.3.6-1: UE Measurement Capabilities

| Item |  |  |  | UE Measurement Capabilities |  |  |  | Ref. |  |  |  | Release |  |  |  | Mnemonic |  |  |  | M |  |  |  | If indicated "Yes" the feature shall be implemented and successfully tested for the corresponding release |  |  |  | Comments |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  | Support NR measurements and events A triggered reporting |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-15 |  |  |  | pc_eventA_MeasAndReport |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  | Support two independent measurement gap configurations for FR1 and FR2 |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-15 |  |  |  | pc_independentGapConfig |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  | Support NR intra-frequency and inter-frequency measurements and at least periodical reporting |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-15 |  |  |  | pc_intraAndInterF_MeasAndReport |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
| 4 |  |  |  | Support CSI-RSRP and CSI-RSRQ measurement as specified in TS38.215 [21], where CSI-RS resource is configured with an associated SS/PBCH |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-15 |  |  |  | pc_csi_RSRP_AndRSRQ_MeasWithSSB |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 5 |  |  |  | Support inter-RAT E-UTRA measurements and events B triggered reporting |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-15 |  |  |  | pc_eventB_MeasAndReport |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
| 6 |  |  |  | Support SS-SINR measurents |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-15 |  |  |  | pc_ss_SINR_Meas |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 7 |  |  |  | Support acquisition of relevant information from a neighbouring E-UTRA cell by reading the SI of the neighbouring cell and reporting the acquired information to the network as specified in TS 38.331 [9] when the EN-DC is not configured. |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-15 |  |  |  | pc_eutra_CGI_Reporting |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
| 8 |  |  |  | Support acquisition of relevant information from a neighbouring intra-frequency or inter-frequency NR cell by reading the SI of the neighbouring cell and reporting the acquired information to the network as specified in TS 38.331 [9] when EN-DC is not configured. |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-15 |  |  |  | pc_nr_CGI_Reporting |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
| 9 |  |  |  | Support acquisition of relevant information from a neighbouring intra-frequency or inter-frequency NR cell by reading the SI of the neighbouring cell and reporting the acquired information to the network as specified in TS 38.331 [9] when the EN-DC is configured. |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-15 |  |  |  | pc_nr_CGI_Reporting_ENDC |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
| 10 |  |  |  | Support shorter measurement gap length (i.e. gp2 and gp3) for independent measurement gap configuration on FR1 and per-UE gap in (NG)EN-DC. |  |  |  | 36.331, 6.3.6 |  |  |  | Rel-15 |  |  |  | pc_gp2_gp3_en_dc |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 11 |  |  |  | Support NR supports gap pattern 4 for independent measurement gap configuration on FR1 and per-UE gap in (NG)EN-DC |  |  |  | 36.331, 6.3.6 |  |  |  | Rel-15 |  |  |  | pc_gp4_en_dc |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 12 |  |  |  | Support NR supports gap pattern 5 for independent measurement gap configuration on FR1 and per-UE gap in (NG)EN-DC |  |  |  | 36.331, 6.3.6 |  |  |  | Rel-15 |  |  |  | pc_gp5_en_dc |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 13 |  |  |  | Support NR supports gap pattern 6 for independent measurement gap configuration on FR1 and per-UE gap in (NG)EN-DC |  |  |  | 36.331, 6.3.6 |  |  |  | Rel-15 |  |  |  | pc_gp6_en_dc |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 14 |  |  |  | Support NR supports gap pattern 7 for independent measurement gap configuration on FR1 and per-UE gap in (NG)EN-DC |  |  |  | 36.331, 6.3.6 |  |  |  | Rel-15 |  |  |  | pc_gp7_en_dc |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 15 |  |  |  | Support NR supports gap pattern 8 for independent measurement gap configuration on FR1 and per-UE gap in (NG)EN-DC |  |  |  | 36.331, 6.3.6 |  |  |  | Rel-15 |  |  |  | pc_gp8_en_dc |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 16 |  |  |  | Support NR supports gap pattern 9 for independent measurement gap configuration on FR1 and per-UE gap in (NG)EN-DC |  |  |  | 36.331, 6.3.6 |  |  |  | Rel-15 |  |  |  | pc_gp9_en_dc |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 17 |  |  |  | Support NR supports gap pattern 10 for independent measurement gap configuration on FR1 and per-UE gap in (NG)EN-DC |  |  |  | 36.331, 6.3.6 |  |  |  | Rel-15 |  |  |  | pc_gp10_en_dc |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 18 |  |  |  | Support NR supports gap pattern 11 for independent measurement gap configuration on FR1 and per-UE gap in (NG)EN-DC |  |  |  | 36.331, 6.3.6 |  |  |  | Rel-15 |  |  |  | pc_gp11_en_dc |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 19 |  |  |  | Support measurement gap pattern 2 configured by NR RRC. |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-15 |  |  |  | pc_gp2_nr |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 20 |  |  |  | Support measurement gap pattern 3 configured by NR RRC. |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-15 |  |  |  | pc_gp3_nr |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 21 |  |  |  | Support measurement gap pattern 4 configured by NR RRC. |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-15 |  |  |  | pc_gp4_nr |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 22 |  |  |  | Support measurement gap pattern 5 configured by NR RRC. |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-15 |  |  |  | pc_gp5_nr |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 23 |  |  |  | Support measurement gap pattern 6 configured by NR RRC. |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-15 |  |  |  | pc_gp6_nr |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 24 |  |  |  | Support measurement gap pattern 7 configured by NR RRC. |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-15 |  |  |  | pc_gp7_nr |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 25 |  |  |  | Support measurement gap pattern 8 configured by NR RRC. |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-15 |  |  |  | pc_gp8_nr |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 26 |  |  |  | Support measurement gap pattern 9 configured by NR RRC. |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-15 |  |  |  | pc_gp9_nr |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 27 |  |  |  | Support measurement gap pattern 10 configured by NR RRC. |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-15 |  |  |  | pc_gp10_nr |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 28 |  |  |  | Support measurement gap pattern 11 configured by NR RRC. |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-15 |  |  |  | pc_gp11_nr |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 29 |  |  |  | Support measurement gap pattern 12 configured by NR RRC. |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-15 |  |  |  | pc_gp12_nr |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 30 |  |  |  | Support measurement gap pattern 15 configured by NR RRC. |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-15 |  |  |  | pc_gp15_nr |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 31 |  |  |  | Support measurement gap pattern 16 configured by NR RRC. |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-15 |  |  |  | pc_gp16_nr |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 32 |  |  |  | Support measurement gap pattern 17 configured by NR RRC. |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-15 |  |  |  | pc_gp17_nr |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 34 |  |  |  | Support measurement gap pattern 18 configured by NR RRC. |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-15 |  |  |  | pc_gp18_nr |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 35 |  |  |  | Support measurement gap pattern 19 configured by NR RRC. |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-15 |  |  |  | pc_gp19_nr |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 36 |  |  |  | Support measurement gap pattern 20 configured by NR RRC. |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-15 |  |  |  | pc_gp20_nr |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 37 |  |  |  | Support measurement gap pattern 21 configured by NR RRC. |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-15 |  |  |  | pc_gp21_nr |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 38 |  |  |  | Support measurement gap pattern 22 configured by NR RRC. |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-15 |  |  |  | pc_gp22_nr |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 39 |  |  |  | Support measurement gap pattern 23 configured by NR RRC. |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-15 |  |  |  | pc_gp23_nr |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 40 |  |  |  | Support CSI-RSRP and CSI-RSRQ measurement as specified in TS38.215 [21], where CSI-RS resource is configured without an associated SS/PBCH |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-15 |  |  |  | pc_csi_RSRP_AndRSRQ_MeasWithoutSSB |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 41 |  |  |  | Support CSI-RS based Radio Link Monitoring for FR1 |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-15 |  |  |  | pc_CSI_RS_RLM_FR1 |  |  |  | Yes |  |  |  |  |  |  |  | If the UE supports this feature, the UE needs to report maxNumberResource-CSI-RS-RLM in its capability report. If the UE doesn’t support CSI-RS based RLM, it will not include this IE in its capability report. |  |  |  |
| 41a |  |  |  | Support CSI-RS based Radio Link Monitoring for FR2 |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-15 |  |  |  | pc_CSI_RS_RLM_FR2 |  |  |  | Yes |  |  |  |  |  |  |  | If the UE supports this feature, the UE needs to report maxNumberResource-CSI-RS-RLM in its capability report. If the UE doesn’t support CSI-RS based RLM, it will not include this IE in its capability report. |  |  |  |
| 42 |  |  |  | Support of E-UTRA RS-SINR measurements |  |  |  | 38.306, 4.2.10 |  |  |  | Rel-15 |  |  |  | pc_RS_SINR_MeasEUTRA |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 43 |  |  |  | Support of SFTD measurements between a E-UTRA PCell and an NR PSCell in FDD |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-15 |  |  |  | pc_SFTD_MeasPSCell_MRDC_FDD |  |  |  | No |  |  |  |  |  |  |  | The SFTD measurement support should be indicated in MRDC capabilities for EN-DC. The support needs to be declared for FDD and TDD separately |  |  |  |
| 44 |  |  |  | Support of SFTD measurements between a E-UTRA PCell and an NR PSCell in TDD |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-15 |  |  |  | pc_SFTD_MeasPSCell_MRDC_TDD |  |  |  | No |  |  |  |  |  |  |  | The SFTD measurement support should be indicated in MRDC capabilities for EN-DC. The support needs to be declared for FDD and TDD separately |  |  |  |
| 45 |  |  |  | Support of relaxed RRM measurements of neighbour cells in RRC_IDLE/RRC_INACTIVE |  |  |  | 38.306, 5.6 |  |  |  | Rel-16 |  |  |  | pc_Relaxed_Measurement |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 46 |  |  |  | Support of SFTD measurements between a E-UTRA PCell and an NR neighbour cell in FDD |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-15 |  |  |  | pc_SFTD_MeasNR_Cell_FDD |  |  |  | No |  |  |  |  |  |  |  | The support needs to be declared for FDD and TDD separately The SFTD measurement support can only be indicated in MRDC capabilities for EN-DC |  |  |  |
| 47 |  |  |  | Support of SFTD measurements between a E-UTRA PCell and an NR neighbour cell in TDD |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-15 |  |  |  | pc_SFTD_MeasNR_Cell_TDD |  |  |  | No |  |  |  |  |  |  |  | The support needs to be declared for FDD and TDD separately The SFTD measurement support can only be indicated in MRDC capabilities for EN-DC |  |  |  |
| 48 |  |  |  | Support of SFTD measurements between a NR PCell and an NR neighbour cell in FDD |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-15 |  |  |  | pc_SFTD_MeasNR_Neigh_FDD |  |  |  | No |  |  |  |  |  |  |  | The support needs to be declared for FDD and TDD separately |  |  |  |
| 49 |  |  |  | Support of SFTD measurements between a NR PCell and an NR neighbour cell in TDD |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-15 |  |  |  | pc_SFTD_MeasNR_Neigh_TDD |  |  |  | No |  |  |  |  |  |  |  | The support needs to be declared for FDD and TDD separately |  |  |  |
| 50 |  |  |  | Support of SFTD measurements between a NR PCell and an NR PSCell in FDD |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-15 |  |  |  | pc_SFTD_MeasPSCell_NRDC_FDD |  |  |  | No |  |  |  |  |  |  |  | The SFTD measurement support should be indicated in UE-NR-Capability |  |  |  |
| 51 |  |  |  | Support of SFTD measurements between a NR PCell and an NR PSCell in TDD |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-15 |  |  |  | pc_SFTD_MeasPSCell_NRDC_TDD |  |  |  | No |  |  |  |  |  |  |  | The SFTD measurement support should be indicated in UE-NR-Capability |  |  |  |
| 52 |  |  |  | Support of acquisition of CGI related information from a neighbouring intra-frequency or inter-frequency NPN CAG cell |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-16 |  |  |  | pc_nr_CGI_Reporting_NPN_r16 |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 53 |  |  |  | Supports periodic EUTRA measurement and reporting. |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-15 |  |  |  | pc_periodicEUTRA_MeasAndReport |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
| 54 |  |  |  | Support configuration of NR SSB measurements in RRC_IDLE/RRC_INACTIVE and reporting of the corresponding results upon network request as specified in TS 38.331 [9] |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-16 |  |  |  | pc_idleInactiveNR_MeasReport |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 55 |  |  |  | Support configuration of E-UTRA measurements in RRC_IDLE/RRC_INACTIVE and reporting of the corresponding results upon network request as specified in TS 38.331 [9] |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-16 |  |  |  | pc_idleInactiveEUTRA_MeasReport |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 56 |  |  |  | Support SRS-RSRP measurements between a NR Pcell and an interfering UE, upon network request as specified in 38.331 [9] |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-16 |  |  |  | pc_nr_CLI_Reporting_r16 |  |  |  | No |  |  |  |  |  |  |  | If the UE supports this feature, the UE needs to report maxNumberCLI-SRS-RSRP-r16 and maxNumberPerSlotCLI-SRS-RSRP-r16 If the UE doesn’t support CLI SRS-RSRP measurement, it will not include this IE in its capability report. |  |  |  |
| 57 |  |  |  | Support acquisition of relevant information from a neighbouring E-UTRA cell by reading the SI of the neighbouring cell and reporting the acquired information to the network as specified in TS 38.331 [9] when the NE-DC is configured. |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-15 |  |  |  | pc_eutra_CGI_Reporting_NEDC |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 58 |  |  |  | Support acquisition of relevant information from a neighbouring E-UTRA cell by reading the SI of the neighbouring cell and reporting the acquired information to the network as specified in TS 38.331 [9] when the NR-DC is configured wherein MN and SN have different DRX cycles, or on-duration configured by MN does not contain on-duration configured by SN if the DRX cycles are the same. |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-15 |  |  |  | pc_eutra_CGI_Reporting_NRDC |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 59 |  |  |  | Support acquisition of relevant information from a neighbouring intra-frequency or inter-frequency NR cell by reading the SI of the neighbouring cell and reporting the acquired information to the network as specified in TS 38.331 [9] when the NE-DC is configured. |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-15 |  |  |  | pc_nr_CGI_Reporting_NEDC |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
| 60 |  |  |  | Support acquisition of relevant information from a neighbouring intra-frequency or inter-frequency NR cell by reading the SI of the neighbouring cell and reporting the acquired information to the network as specified in TS 38.331 [9] when the NR-DC is configured wherein MN and SN have different DRX cycles, or on-duration configured by MN does not contain on-duration configured by SN if the DRX cycles are the same. |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-15 |  |  |  | pc_nr_CGI_Reporting_NRDC |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
| 61 |  |  |  | Supports performing eNB-configured SSB-based RRM measurements for EN-DC configured NR FR1 carrier(s) in RRC_IDLE and reporting them when requested by the eNB while resuming from RRC_IDLE or in RRC_CONNECTED, as specified in TS 36.331 [5]. |  |  |  | 36.306, 4.3.6.41 |  |  |  | Rel-16 |  |  |  | pc_nrIdleInactiveNRFR1_MeasReport |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 62 |  |  |  | Supports performing eNB-configured SSB-based RRM measurements for EN-DC configured NR FR2 carrier(s) in RRC_IDLE and reporting them when requested by the eNB while resuming from RRC_IDLE or in RRC_CONNECTED, as specified in TS 36.331 [5]. |  |  |  | 36.306, 4.3.6.42 |  |  |  | Rel-16 |  |  |  | pc_nrIdleInactiveNRFR2_MeasReport |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| xx |  |  |  | Support of SFTD measurements between the NR PCell and a configured E-UTRA PSCell |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-15 |  |  |  | pc_SFTD_MeasPSCell_NEDC_FDD |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |