3GPP TSG-RAN5 Meeting #98	R5-230719r1

# Athens, Greece, 27th Feb 2023 – 3rd Mar 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 2234 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to SRS-RSRP Measurement test case 4.6.5.1 and 4.7.6.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Technologies Int |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CLI-UEConTest |  |  |  |  |  | Date: |  |  | 2022-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The Test Tolerances for CLI test cases are available in 38.903 but missing on the current test case description. Test Tolerances are available for the following configurations: - Normal conditions- Test frequency f ≤ 40.8 GHz - UE PC3- Normal conditions |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Updated the Test Requirements tables according to the corresponding Test Tolerances. - Updated the Editor’s notes to reflect the Test Tolerance addition. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case implementation will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.5.1, 4.7.6.1, Annex F |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | Y |  | Test specifications |  |  |  | TS/TR 38.903 CR R5-xxyyzz |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: Updated Rows in Annex F tables. 4.7.6.1 is upated with test tolerances |  |  |  |  |  |  |  |  |

<< Start of Changes >>

#### 4.6.5.1	EN-DC FR1 SRS-RSRP measurement with non-DRX

Editor's NOTE: This test case is incomplete in following aspects:

-	Message contents are missing.

-	TT analysis is missing.

-	Test Procedure might need update.

-	Test applicability needs to be updated

4.6.5.1.1	Test purpose

To verify that the UE makes correct reporting of SRS-RSRP measurement within SRS-RSRP measurement requirements in TS 38.133 [6] clause 9.7.2.5.

4.6.5.1.2	Test applicability

FFS.

##### 4.6.5.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 4.6.5.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.4.6.5.1.

4.6.5.1.4	Test description

4.6.5.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 4.6.5.1.4.1-1. Configure the test equipment and the DUT according to the parameters in Table 4.6.5.1.4.1-2. Test environment parameters are given in Table 4.6.5.1.4.1-3.

Table 4.6.5.1.4.1-1: EN-DC FR1 SRS-RSRP measurement supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | NR 15 kHz SRS SCS, 10 MHz bandwidth, TDD duplex mode |
| 2 | NR 30 kHz SRS SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

Table 4.6.5.1.4.1-2: General test parameters for EN-DC SRS-RSRP
event triggered reporting for PSCell in FR1

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1, 2 | E-UTRAN Cell 1 and NR Cell 2 |  |
| RF Channel Number |  | 1, 2 | 1: Cell 1 2: Cell 2 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.1 |  |
|  |  | 2 | SMTC.1 |  |
| SRS configuration |  | 1 | SRSConf.1 | Table 4.6.5.1.5-3 |
|  |  | 2 | SRSConf.2 |  |
| CP length |  | 1, 2 | Normal |  |
| i1-Threshold | dBm | 1 | -97 |  |
|  |  | 2 | -95 |  |
| Hysteresis | dB | 1, 2 | 0 |  |
| Time To Trigger | s | 1, 2 | 0 |  |
| Filter coefficient |  | 1, 2 | 0 | L3 filtering is not used |
| DRX |  | 1, 2 | OFF | Non-DRX |
| Time offset between DL from serving cell and SRS from test system | s | 1,2 | 17.67 |  |
| T1 | s | 1, 2 | 5 |  |
| T2 | s | 1, 2 | 1 |  |

Table 4.6.5.1.4.1-3: Test Environment parameters for EN-DC SRS-RSRP
event triggered reporting for PSCell in FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.2-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 4.6.5.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | For 4Rx capable UEs without any 2 Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE Part |  |  |

- Message contents are defined in clause 4.6.5.1.4.3.

- Cell 1 is the E-UTRA serving cell (PCell) for the EN-DC setup. The power levels and settings for Cell 1 are set according to Annex A.6. Cell 2 is NR FR1 cell (PSCell). Cell 2 is the target for SRS-RSRP measurements. In the measurement control information, a measurement object is configured for the frequency of the PSCell, and it is indicated to the UE that event-triggered reporting with Event I1 is used. The test consists of two successive time periods, with time duration of T1 and T2, respectively.

4.6.5.1.4.2	Test procedure

FFS

4.6.5.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

FFS

4.6.5.1.5	Test requirement

Table 4.6.5.1.5-1 defines the primary level settings including test tolerances for all tests.

Table 4.6.5.1.5-1: NR Cell specific test parameters for EN-DC SRS-RSRP
event triggered reporting for PSCell in FR1

| Parameter | Unit | Test | Cell 2 |  |
| --- | --- | --- | --- | --- |
|  |  | configuration | T1 | T2 |
| TDD configuration |  | 1 | TDDConf.1.1 |  |
|  |  | 2 | TDDConf.2.1 |  |
| PDSCH RMC |  | 1 | SR.1.1 TDD |  |
| configuration |  | 2 | SR.2.1 TDD |  |
| RMSI CORESET RMC |  | 1 | CR.1.1 TDD |  |
| configuration |  | 2 | CR.2.1 TDD |  |
| Dedicated CORESET |  | 1 | CCR.1.1 TDD |  |
| RMC configuration |  | 2 | CCR.2.1 TDD |  |
| OCNG Patterns |  | 1, 2 | OP.1 |  |
| TRS Configuration |  | 1 | TRS.1.1 TDD |  |
|  |  | 2 | TRS.1.2 TDD |  |
| Initial BWP configuration |  | 1, 2 | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2 | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2 | ULBWP.1.1 |  |
| Note 2 | dBm/15 kHz | 1 | -98+TT |  |
|  |  | 2 |  |  |
| Note 2 | dBm/SCS | 1 | -98+TT |  |
|  |  | 2 | -95+TT |  |
| Propagation Condition |  | 1, 2 | AWGN |  |
| NOTE 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. |  |  |  |  |

Table 4.6.5.1.5-2: NR Cell specific test parameters for EN-DC FR1 SRS-RSRP event triggered reporting for neighbour cell UE

| Parameter | Unit | Test | Neighbour cell UE |  |
| --- | --- | --- | --- | --- |
|  |  | configuration | T1 | T2 |
| Note 2 | dBm/15 kHz | 1 | -98+TT |  |
|  |  | 2 |  |  |
| Note 2 | dBm/SCS | 1 | -98+TT |  |
|  |  | 2 | -95+TT |  |
|  | dB | 1 | -infinity | 4+TT |
|  |  | 2 |  |  |
|  | dB | 1 | -infinity | 4+TT |
|  |  | 2 |  |  |
| SRS-RSRP Note 3 | dBm/SCS kHz | 1 | -infinity | -94+TT |
|  |  | 2 | -infinity | -91+TT |
| Io | dBm/9.36 MHz | 1 | -70.05+TT | -64.59+TT |
|  | dBm/38.16 MHz | 2 | -63.96+TT | -58.50+TT |
| Propagation Condition |  | 1, 2 | AWGN |  |
| NOTE 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SRS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

Table 4.6.5.1.5-3: SRS configuration for measurement reporting

|  | Field | SRSConf.1 | SRSConf.2 | Comments |
| --- | --- | --- | --- | --- |
| SRS- | srs-ResourceSetId | 0 | 0 |  |
| ResourceSet | srs-ResourceIdList | 0 | 0 |  |
|  | resourceType | Periodic | Periodic |  |
|  | Usage | Codebook | Codebook |  |
| SRS- | SRS-ResourceId | 0 | 0 |  |
| Resource | nrofSRS-Ports | Port1 | Port1 |  |
|  | transmissionComb | n2 | n2 |  |
|  | combOffset-n2 | 0 | 0 |  |
|  | cyclicShift-n2 | 0 | 0 |  |
|  | resourceMapping startPosition | 0 | 0 |  |
|  | resourceMapping nrofSymbols | n1 | n1 |  |
|  | resourceMapping repetitionFactor | n1 | n1 |  |
|  | freqDomainPosition | 0 | 0 |  |
|  | freqDomainShift | 0 | 0 |  |
|  | freqHopping c-SRS | 12 | 12 |  |
|  | freqHopping b-SRS | 0 | 0 |  |
|  | freqHopping b-hop | 0 | 0 |  |
|  | groupOrSequenceHopping | Neither | Neither |  |
|  | resourceType | Periodic | Periodic |  |
|  | periodicityAndOffset | sl20, 9 | sl40, 19 |  |
|  | sequenceId | 0 | 0 | Any 10 bit number |

The UE shall send one Event I1 triggered measurement report, with a measurement reporting delay less than 60 ms from the beginning of time period T2.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

<<Some Unchanged Sections>>

#### 4.7.6.1	EN-DC SRS-RSRP measurement accuracy with FR1 serving cell

Editor's Note: This test case is incomplete in following aspects:

-	Message contents are missing.

-	Test Procedure is FFS.

-	Test applicability needs to be updated

4.7.6.1.1	Test purpose

The purpose of this test is to verify that the SRS-RSRP measurement accuracy is within the specified limits with SRS-RSRP measurement requirements in TS 38.133 [6] clause 10.1.22.1.1.

4.7.6.1.2	Test applicability

FFS

4.7.6.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 4.7.6.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.4.7.6.1.

4.7.6.1.4	Test description

4.7.6.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 4.7.6.1.4.1-1. Test environment parameters are given in Table 4.7.6.2.4.1-2.

Table 4.7.6.1.4.1-1: EN-DC FR1 SRS-RSRP accuracy supported test configurations

| Config | Description |
| --- | --- |
| 1 | LTE FDD, NR 15 kHz SRS SCS, 10 MHz bandwidth, TDD duplex mode |
| 2 | LTE FDD, NR 30kHz SRS SCS, 40 MHz bandwidth, TDD duplex mode |
| 3 | LTE TDD, NR 15 kHz SRS SCS, 10 MHz bandwidth, TDD duplex mode |
| 4 | LTE TDD, NR 30kHz SRS SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

Table 4.7.6.1.4.1-2: SRS Configuration parameters for EN-DC SRS-RSRP accuracy

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.2-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 4.7.6.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | For 4Rx capable UEs without any 2 Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE Part |  |  |

- Message contents are defined in clause 4.7.6.1.4.3.

- Cell 1 is the E-UTRA serving cell (PCell) for the EN-DC setup. The power levels and settings for Cell 1 are set according to Annex A.6. Cell 2 is NR FR1 cell (PSCell). Cell 2 is the target for SRS-RSRP measurements. Before the test UE is configured to perform SRS-RSRP measurement. During the test, the test system transmits SRS resources for measurement in the DL slots according to the SRS configuration in Table 4.7.6.1.5-3. There is no measurement gap configured in the test. During the test, the test system does not transmit PDCCH/PDSCH/OCNG on SRS symbol to be transmitted and on 1 data symbol before SRS to be transmitted.

4.7.6.1.4.2	Test procedure

FFS

4.7.6.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

FFS

4.7.6.1.5	Test requirement

Table 4.7.6.1.5-1 defines the primary level settings including test tolerances for all tests.

Table 4.7.6.1.5-1: NR Cell specific test parameters for EN-DC SRS-RSRP accuracy for PSCell

| Parameter |  | Config | Unit | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- | --- | --- |
| SSB GSCN |  | 1~4 |  | freq1 | freq1 | freq1 |
| Duplex mode |  | 1~4 |  | TDD | TDD | TDD |
| TDD configuration |  | 1,3 |  | TDDConf.1.1 | TDDConf.1.1 | TDDConf.1.1 |
|  |  | 2,4 |  | TDDConf.2.1 | TDDConf.2.1 | TDDConf.2.1 |
| BWchannel |  | 1,3 | MHz | 10: NRB,c = 52 | 10: NRB,c = 52 | 10: NRB,c = 52 |
|  |  | 2,4 |  | 40: NRB,c = 106 | 40: NRB,c = 106 | 40: NRB,c = 106 |
| PDSCH Reference |  | 1,3 |  | SR.1.1 TDD | SR.1.1 TDD | SR.1.1 TDD |
| measurement channel |  | 2,4 |  | SR.2.1 TDD | SR.2.1 TDD | SR.2.1 TDD |
| RMSI CORESET Reference |  | 1,3 |  | CR.1.1 TDD | CR.1.1 TDD | CR.1.1 TDD |
| Channel |  | 2,4 |  | CR.2.1 TDD | CR.2.1 TDD | CR.2.1 TDD |
| Dedicated CORESET |  | 1,3 |  | CCR.1.1 TDD | CCR.1.1 TDD | CCR.1.1 TDD |
| Reference Channel |  | 2,4 |  | CCR.2.1 TDD | CCR.2.1 TDD | CCR.2.1 TDD |
| SSB configuration |  | 1,3 |  | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 |
|  |  | 2,4 |  | SSB.2 FR1 | SSB.2 FR1 | SSB.2 FR1 |
| OCNG Patterns |  | 1~4 |  | OP.1 | OP.1 | OP.1 |
| TRS configuration |  | 1,3 |  | TRS.1.1 TDD | TRS.1.1 TDD | TRS.1.1 TDD |
|  |  | 2,4 |  | TRS.1.2 TDD | TRS.1.2 TDD | TRS.1.2 TDD |
| Initial BWP Configuration |  | 1~4 |  | DLBWP.0.1 ULBWP.0.1 | DLBWP.0.1 ULBWP.0.1 | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration |  | 1~4 |  | DLBWP.1.1 ULBWP.1.1 | DLBWP.1.1 ULBWP.1.1 | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration |  | 1~4 |  | SMTC.1 | SMTC.1 | SMTC.1 |
| Time offset between DL from serving cell and SRS from test system |  | 1~4 | s | 17.67 | 17.67 | 17.67 |
| EPRE ratio of PSS to SSS |  | 1~4 | dB | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |
| Note2 | NR_TDD_FR1_A Note3 | 1,3 | dBm/15kHz | -106 | -88 | -114 |
|  | NR_TDD_FR1_C |  |  |  |  | -113 |
|  | NR_TDD_FR1_D |  |  |  |  | -112.5 |
|  | NR_TDD_FR1_E |  |  |  |  | -112 |
|  | NR_TDD_FR1_A Note3 | 2,4 |  | Not applicableNote4 | -91 | -114 |
|  | NR_TDD_FR1_C |  |  |  |  | -113 |
|  | NR_TDD_FR1_D |  |  |  |  | -112.5 |
|  | NR_TDD_FR1_E |  |  |  |  | -112 |
| Note2 | NR_TDD_FR1_A Note3 | 1,3 | dBm/SRS SCS | -106 | -88 | -114 |
|  | NR_TDD_FR1_C |  |  |  |  | -113 |
|  | NR_TDD_FR1_D |  |  |  |  | -112.5 |
|  | NR_TDD_FR1_E |  |  |  |  | -112 |
|  | NR_TDD_FR1_A Note3 | 2,4 |  | Not applicableNote4 | -88 | -111 |
|  | NR_TDD_FR1_C |  |  |  |  | -110 |
|  | NR_TDD_FR1_D |  |  |  |  | -109.5 |
|  | NR_TDD_FR1_E |  |  |  |  | -109 |
| Note 1: OCNG shall be used such that a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification Note 4: Test 1 is not used when testing with 30kHz SSB SCS |  |  |  |  |  |  |

Table 4.7.6.1.5-2: NR Cell specific test parameters for EN-DC FR1 SRS-RSRP accuracy for neighbour cell UE

| Parameter |  | Config | Unit | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- | --- | --- |
| Note2 | NR_TDD_FR1_A NOTE 3 | 1,3 | dBm/15kHz | -106 | -88 | -114 |
|  | NR_TDD_FR1_C |  |  |  |  | -113 |
|  | NR_TDD_FR1_D |  |  |  |  | -112.5 |
|  | NR_TDD_FR1_E |  |  |  |  | -112 |
|  | NR_TDD_FR1_A NOTE 5 | 2,4 |  | Not applicableNote 6 | -91 | -114 |
|  | NR_TDD_FR1_C |  |  |  |  | -113 |
|  | NR_TDD_FR1_D |  |  |  |  | -112.5 |
|  | NR_TDD_FR1_E |  |  |  |  | -112 |
| Note2 | NR_TDD_FR1_A NOTE 5 | 1,3 | dBm/SRS SCS | -106 | -88 | -114 |
|  | NR_TDD_FR1_C |  |  |  |  | -113 |
|  | NR_TDD_FR1_D |  |  |  |  | -112.5 |
|  | NR_TDD_FR1_E |  |  |  |  | -112 |
|  | NR_TDD_FR1_A NOTE 5 | 2,4 |  | Not applicableNote 6 | -88 | -111 |
|  | NR_TDD_FR1_C |  |  |  |  | -110 |
|  | NR_TDD_FR1_D |  |  |  |  | -109.5 |
|  | NR_TDD_FR1_E |  |  |  |  | -109 |
| on SRS |  | 1~4 | dB | 1 | 1 | 1 |
| SRS RSRP Note3 | NR_TDD_FR1_A NOTE 5 | 1,3 | dBm/SRS SCS | -105 | -87 | -113 |
|  | NR_TDD_FR1_C |  |  |  |  | -112 |
|  | NR_TDD_FR1_D |  |  |  |  | -111.5 |
|  | NR_TDD_FR1_E |  |  |  |  | -111 |
|  | NR_TDD_FR1_A NOTE 5 | 2,4 |  | Not applicableNote 6 | -87 | -110 |
|  | NR_TDD_FR1_C |  |  |  |  | -109 |
|  | NR_TDD_FR1_D |  |  |  |  | -108.5 |
|  | NR_TDD_FR1_E |  |  |  |  | -108 |
| Io Note3 | NR_TDD_FR1_A NOTE 5 | 1,3 | dBm/9.36 MHz | -74.51 | -56.51 | -82.51 |
|  | NR_TDD_FR1_C |  |  |  |  | -81.51 |
|  | NR_TDD_FR1_D |  |  |  |  | -81.01 |
|  | NR_TDD_FR1_E |  |  |  |  | -79.51 |
|  | NR_TDD_FR1_A NOTE 5 | 2,4 | dBm/38.16 MHz | Not applicableNote 6 | -53.42 | -76.42 |
|  | NR_TDD_FR1_C |  |  |  |  | -75.42 |
|  | NR_TDD_FR1_D |  |  |  |  | -74.92 |
|  | NR_TDD_FR1_E |  |  |  |  | -74.42 |
| on SRS |  | 1~4 | dB | 1 | 1 | 1 |
| Propagation condition |  | 1~4 |  | AWGN | AWGN | AWGN |
| Antenna configuration |  | 1~4 |  | 1x2 | 1x2 | 1x2 |
| SRS configuration |  | 1,3 |  | SRSConf.1 | SRSConf.1 | SRSConf.1 |
|  |  | 2,4 |  | SRSConf.2 | SRSConf.2 | SRSConf.2 |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of the test. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification Note 6: Test 1 is not used when testing with 30kHz SSB SCS |  |  |  |  |  |  |

Table 4.7.6.1.5-3: SRS configuration for FR1 SRS-RSRP accuracy

|  | Field | SRSConf.1 | SRSConf.2 |
| --- | --- | --- | --- |
| SRS- | srs-ResourceSetId | 0 | 0 |
| ResourceSet | srs-ResourceIdList | 0 | 0 |
|  | resourceType | Periodic | Periodic |
|  | Usage | Codebook | Codebook |
| SRS- | SRS-ResourceId | 0 | 0 |
| Resource | nrofSRS-Ports | Port1 | Port1 |
|  | transmissionComb | n2 | n2 |
|  | combOffset-n2 | 0 | 0 |
|  | cyclicShift-n2 | 0 | 0 |
|  | resourceMapping startPosition | 0 | 0 |
|  | resourceMapping nrofSymbols | n1 | n1 |
|  | resourceMapping repetitionFactor | n1 | n1 |
|  | freqDomainPosition | 0 | 0 |
|  | freqDomainShift | 0 | 0 |
|  | freqHopping c-SRS | 12 | 12 |
|  | freqHopping b-SRS | 0 | 0 |
|  | freqHopping b-hop | 0 | 0 |
|  | groupOrSequenceHopping | Neither | Neither |
|  | resourceType | Periodic | Periodic |
|  | periodicityAndOffset-p | sl20, 9 | sl40, 19 |
|  | sequenceId | 0 | 0 |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

<< Some Unchanged Sections>>

# F.1	Measurement uncertainties and test tolerances for FR1 and FR2

## F.1.1	Acceptable uncertainty of test system (normative)

See TS 38.521-1 [17] annex F.1.

### F.1.1.1	Measurement of test environments

See TS 38.521-1 [17] Annex F1.1.

### F.1.1.2	Measurement of RRM requirements

This clause defines the maximum test system uncertainty for the RRM requirements. The maximum uncertainty values allowed for the typical RRM measurement uncertainty contributors is defined in Table F.1.1.2-1 and Table F.1.1.2-2. Unless explicitly stated for a particular test case, these maximum uncertainty values should be used as starting point to perform the test tolerance analysis in TR 38.903 [22] for each of the test cases. Specific test cases might require a tighter measurement uncertainty value for some of the contributors. Exceptions to the general values in Table F.1.1.2-1 and Table F.1.1.2-2 shall be handled case by case.

Table F.1.1.2-1: Maximum allowed measurement uncertainty values for the test system for FR1 (up to 6 GHz) and Cell BW ≤ 40 MHz

| MU contributor | Unit | Value | Comment |
| --- | --- | --- | --- |
| AWGN absolute power, Noc averaged over BWConfig | dB | ±1.5 |  |
| AWGN absolute power, Noc for Measurement PRB | dB | ±1.5 | Measurement PRB may be for example #RBJ to RBJ+19 for SSB-based, or selected RBs in the range #RB0 to RB105 for CSI-RS-based |
| Ratio of cell X signal / AWGN, Êsx / Noc averaged over BWConfig | dB | ±0.3 | Same as in LTE |
| Ratio of cell X signal / AWGN, Êsx / Noc for Measurement PRB | dB | ±0.3 | Measurement PRB may be for example #RBJ to RBJ+19 for SSB-based, or selected RBs in the range #RB0 to RB105 for CSI-RS-based |
| Fading profile uncertainty | dB | ±0.5 for 1 Tx ±0.7 for 2 Tx | Depends on fading profile, can be referenced from TS 38.101-4 [20] |
| AWGN and signal flatness* | dB | ±3.6 | Same as in Demod Performance test cases, can be referenced from TS 38.521-4 [20] |
| AWGN and signal flatness* | dB | ±2.0 | Same as in LTE, can be referenced from TS 38.101-4 [20] |
| Uplink absolute power measurement | dB | ±1.5 | Same as in TS 38.521-1 [17] |
| Uplink relative power measurement | dB | ±0.7 | Same as in TS 38.521-1 [17] |
| Uplink signal transmit timing relative to downlink | Tc | ±112 |  |
| Relative transmit timing accuracy during UE timing adjustment | Tc | ±88 |  |
| Timing Advance Adjustment accuracy | Tc | ±88 |  |
| Note 1: The values in this table are specified per cell. Multi-cell test cases need to combined these values in the TT analysis in TR 38.903 Note 2:These values apply for cell BW ≤ 40 MHz. The maximum allowed measurement uncertainty for higher cell BW is FFS. Note 3: Void |  |  |  |

Table F.1.1.2-2: Maximum allowed measurement uncertainty values for the test system for FR2 (up to 40GHz) and Cell BW ≤ 400 MHz

| MU contributor | Unit | Value | Comment |
| --- | --- | --- | --- |
| AWGN absolute power, Noc averaged over BWConfig | dB | PC3: ±5.654,5 PC1: FFS |  |
| AWGN absolute power, Noc for Measurement PRB | dB | PC3: ±5.654,5 PC1: FFS | Measurement PRB may be for example #RBJ to RBJ+19 for SSB-based, or selected RBs in the range #RB0 to RB274 for CSI-RS-based |
| Ratio of cell X signal / AWGN, Êsx / Noc averaged over BWConfig | dB | ±0.3 |  |
| Ratio of cell X signal / AWGN, Êsx / Noc for Measurement PRB | dB | ±0.3 | Measurement PRB may be for example #RBJ to RBJ+19 for SSB-based, or selected RBs in the range #RB0 to RB274 for CSI-RS-based |
| ÊsxNote 3 absolute power, Noc averaged over BWConfig | dB | PC3: ±5.654,5 PC1: FFS |  |
| ÊsxNote 3 absolute power, Noc for Measurement PRB | dB | PC3: ±5.654,5 PC1: FFS | Measurement PRB may be for example #RBJ to RBJ+19 for SSB-based, or selected RBs in the range #RB0 to RB274 for CSI-RS-based |
| Relative power between any two Ês levels at the same frequency | dB | ±2.0 |  |
| Fading profile uncertainty | dB | ±0.5 for 1 Tx ±0.7 for 2 Tx |  |
| Uplink absolute power measurement | dB | TBD |  |
| Uplink relative power measurement | dB | TBD |  |
| Uplink signal transmit timing relative to downlink | Tc | ±48 |  |
| Relative transmit timing during UE timing adjustment | Tc | ±40 |  |
| Timing Advance Adjustment | Tc | ±40 |  |
| NOTE 1: The values in this table are specified per cell. Multi-cell test cases need to combine these values in the TT analysis in TR 38.903 NOTE 2: These values apply for cell BW ≤ 400 MHz. The maximum allowed measurement uncertainty for higher cell BW is FFS. NOTE 3: Applies for test cases that use Ês without AWGN NOTE 4: Total Expanded MU for IFF for Max device size ≤ 30cm in TR 38.903 [22] Table E.3.1.1-2 for PC3 UEs and in Table FFS for PC1 UEs. NOTE 5: If the TT analysis for a specific test case based on this MU value results in an unsolvable conflict, making the test case untestable, even after the alternative solutions listed in TR 38.903 [22] clause A.4 have been considered for the test case in TS 38.133 [6] Annex A, the TT analysis shall be repeated using a lower MU value, taking into account the values defined in clause E.3.1 TR 38.903 [22]. The test case will be applicable for the subset of the test systems meeting this reduced MU Threshold. |  |  |  |

The maximum test system uncertainty for the EN-DC FR1 test cases in chapter 4 is defined in Table F.1.1.2-3.

The maximum test system uncertainty for the NR/5GC FR1 test cases in chapter 6 is defined in Table F.1.1.2-4.

The maximum test system uncertainty for the EN-DC FR2 test cases in chapter 5 is defined in Table F.1.1.2-5.

The maximum test system uncertainty for the NR/5GC FR2 test cases in chapter 7 is defined in Table F.1.1.2-6.

The maximum test system uncertainty for the E-UTRA – NR inter-RAT with E-UTRA serving cell test cases in chapter 8 is defined in Table F.1.1.2-7.

The maximum test system uncertainty for the FR1 NR sidelink test cases in chapter 9 is defined in Table F.1.1.2-8.

The maximum test system uncertainty for the NR/5GC FR1 test cases for RedCap in chapter 16 is defined in Table F.1.1.2-9.

The maximum test system uncertainty for the NR/5GC FR2 test cases for RedCap in chapter 17 is defined in Table F.1.1.2-10.

The maximum test system uncertainty for the E-UTRA – NR inter-RAT with E-UTRA serving cell test cases for RedCap in chapter 18 is defined in Table F.1.1.2-11.

Table F.1.1.2-3: Maximum test system uncertainty for RRM requirements for EN-DC FR1 test cases

| Subclause |  |  | Maximum Test System Uncertainty1 |  |  |  |  | Derivation of Test System Uncertainty |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4.3.2.2.1 Contention based random access test in FR1 for PSCell in EN-DC |  |  | Noc ±1.5 dB Ês2 / Noc ±0.3 dB Uplink absolute power measurement ±1.5 dB Uplink relative power measurement ±0.7 dB ±112Tc Uplink signal transmit timing relative to downlink |  |  |  |  | Ês2 / Noc is the ratio of cell 2 signal / AWGN Tc = 1/(480000 x 4096) seconds, the basic timing unit defined in TS 38.211 [7] |  |  |  |
| 4.3.2.2.2 Non-contention based random access test in FR1 for PSCell in EN-DC |  |  | Same as 4.3.2.2.1 |  |  |  |  | Same as 4.3.2.2.1 |  |  |  |
| 4.3.2.2.3 EN-DC FR1 2-step contention based random access |  |  | Same as 4.3.2.2.1 |  |  |  |  | Same as 4.3.2.2.1 |  |  |  |
| 4.3.2.2.4 EN-DC FR1 2-step non-contention based random access |  |  | Same as 4.3.2.2.1 |  |  |  |  | Same as 4.3.2.2.1 |  |  |  |
| 4.4.1.1 EN-DC FR1 UE transmit timing accuracy | Noc ±1.5 dB Ês2 / Noc ±0.3 dB ±112Tc Uplink signal transmit timing relative to downlink |  |  |  |  | Ês2 / Noc is the ratio of cell 2 signal / AWGN Tc = 1/(480000 x 4096) seconds, the basic timing unit defined in TS 38.211 [7] |  |  |  |  |
| 4.4.3.1 EN-DC FR1 timing advance adjustment accuracy |  |  | Noc ±1.5 dB Ês2 / Noc ±0.3 dB ±88Tc Timing Advance Adjustment accuracy |  |  |  |  | Ês2 / Noc is the ratio of cell 2 signal / AWGN Tc = 1/(480000 x 4096) seconds, the basic timing unit defined in TS 38.211 [7] |  |  |  |
| 4.5.1.1 EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode |  |  | Average Noc2 ±1.5 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês2 / Noc2 ±0.3 dB Fading profile ±0.7 dB |  |  |  |  | Note: Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the SNR Meas PRB are the measurement PRB for SSB-based RLM #RBJ to RBJ+19 |  |  |  |
| 4.5.1.2 EN-DC FR1 radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode |  |  | Same as 4.5.1.1 |  |  |  |  | Same as 4.5.1.1 |  |  |  |
| 4.5.1.3 EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in DRX mode |  |  | Same as 4.5.1.1 |  |  |  |  | Same as 4.5.1.1 |  |  |  |
| 4.5.1.4 EN-DC FR1 radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in DRX mode |  |  | Same as 4.5.1.1 |  |  |  |  | Same as 4.5.1.1 |  |  |  |
| 4.5.1.5 EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode |  |  | Same as 4.5.1.1 |  |  |  |  | Same as 4.5.1.1 |  |  |  |
| 4.5.1.6 EN-DC FR1 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode |  |  | Same as 4.5.1.1 |  |  |  |  | Same as 4.5.1.1 |  |  |  |
| 4.5.1.7 EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in DRX mode |  |  | Same as 4.5.1.1 |  |  |  |  | Same as 4.5.1.1 |  |  |  |
| 4.5.1.8 EN-DC FR1 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM RS in DRX mode |  |  | Same as 4.5.1.1 |  |  |  |  | Same as 4.5.1.1 |  |  |  |
| 4.5.2.1 EN-DC FR1 interruptions at transitions between active and non-active during DRX in synchronous EN-DC |  |  | ± 0.6dB (AWGN conditions) |  |  |  |  | Overall system uncertainty for AWGN condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Effect of AWGN flatness and signal flatness Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±2.0 dB |  |  |  |
| 4.5.2.2 EN-DC FR1 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC |  |  | Same as 4.5.2.1 |  |  |  |  | Same as 4.5.2.1 |  |  |  |
| 4.5.2.3 EN-DC FR1 interruptions during measurements on deactivated NR SCC in synchronous EN-DC |  |  | Noc2 ±1.5 dB Noc3 ±1.5 dB Ês2 / Noc2 ±0.3 dB Ês3 / Noc3 ±0.3 dB |  |  |  |  | Note: Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês3 / Noc3 is the ratio of cell 3 signal / AWGN |  |  |  |
| 4.5.2.4 EN-DC FR1 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC |  |  | Same as 4.5.2.3 |  |  |  |  | Same as 4.5.2.3 |  |  |  |
| 4.5.2.5 EN-DC FR1 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC |  |  | Noc1 ±1.5 dB Noc2 ±1.5 dB Noc3 ±1.5 dB Ês1 / Noc ±0.3 dB Ês2 / Noc ±0.3 dB Ês3 / Noc ±0.3 dB |  |  |  |  | Note: Noc1 is the AWGN on E-UTRAN cell 1 frequency Ês1 / Noc1 is the ratio of E-UTRAN cell 1 signal / AWGN Noc2 is the AWGN on NR cell 2 frequency Ês2 / Noc2 is the ratio of NR cell 2 signal / AWGN Noc3 is the AWGN on E-UTRAN cell 3 frequency Ês3 / Noc3 is the ratio of E-UTRAN cell 3 signal / AWGN |  |  |  |
| 4.5.2.6 EN-DC FR1 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC |  |  | Same as 4.5.2.5 |  |  |  |  | Same as 4.5.2.5 |  |  |  |
| 4.5.3.1 EN-DC FR1 SCell activation and deactivation of known SCell in non-DRX for 160ms SCell measurement cycle |  |  | Average Noc2 ±1.5 dB Average Noc3 ±1.5 dB Average Ês2 / Noc2 ±0.3 dB Average Ês3 / Noc3 ±0.3 dB Meas PRB Noc2 ±1.5 dB Meas PRB Noc3 ±1.5 dB Meas PRB Ês2 / Noc2 ±0.3 dB Meas PRB Ês3 / Noc3 ±0.3 dB |  |  |  |  | Note: Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Noc3 is the AWGN on cell 3 frequency Ês3 / Noc3 is the ratio of cell 3 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |  |  |
| 4.5.3.2 EN-DC FR1 SCell activation and deactivation of known SCell in non-DRX for 640ms SCell measurement cycle |  |  | Same as 4.5.3.1 |  |  |  |  | Same as 4.5.3.1 |  |  |  |
| 4.5.3.3 EN-DC FR1 SCell activation and deactivation of unknown SCell in non-DRX |  |  | Same as 4.5.3.1 |  |  |  |  | Same as 4.5.3.1 |  |  |  |
| 4.5.4.1 EN-DC FR1 UE UL carrier RRC reconfiguration delay |  |  |  |  | Noc2 ±1.5 dB Ês2 / Noc2 ±0.3 dB Noc3 ±1.5 dB Ês3 / Noc3 ±0.3 dB |  |  |  |  | Note: Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Noc3 is the AWGN on cell 3 frequency Ês3 / Noc3 is the ratio of cell 3 signal / AWGN |  |  |
| 4.5.5.1 EN-DC FR1 SSB-based beam failure detection and link recovery in non-DRX |  |  |  |  | Average Noc2 ±1.5 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês2 / Noc2 ±0.3 dB Fading profile ±0.7 dB |  |  |  |  | Note: Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the SNR for the SSB Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |  |
| 4.5.5.2 EN-DC FR1 SSB-based beam failure detection and link recovery in DRX |  |  |  |  | Same as 4.5.5.1 |  |  |  |  | Same as 4.5.5.1 |  |  |
| 4.5.5.3 EN-DC FR1 CSI-RS-based beam failure detection and link recovery in non-DRX |  |  |  |  | Average Noc2 ±1.5 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês2 / Noc2 ±0.3 dB Fading profile ±0.7 dB |  |  |  |  | Note: Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the SNR for the CSI-RS Meas PRB is the measurement PRB for CSI-RSRP #RB0 to RB274 |  |  |
| 4.5.5.4 EN-DC FR1 CSI-RS-based beam failure detection and link recovery in DRX |  |  |  |  | Same as 4.5.5.3 |  |  |  |  | Same as 4.5.5.3 |  |  |
| 4.5.5.5 EN-DC FR1 Scell CSI-RS-based beam failure detection and SSB-based link recovery in non-DRX |  |  |  |  | Average Noc2 ±1.5 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês2 / Noc2 ±0.3 dB Fading profile ±0.7 dB |  |  |  |  | Note: Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the SNR for the SSB Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the SNR for the CSI-RS Meas PRB is the measurement PRB for CSI-RSRP #RB0 to RB274 |  |  |
| 4.5.5.6 EN-DC FR1 Scell CSI-RS-based beam failure detection and SSB-based link recovery in DRX |  |  |  |  | Same as 4.5.5.5 |  |  |  |  | Same as 4.5.5.5 |  |  |
| 4.5.6.1.1 EN-DC FR1 DCI-based DL active BWP switch in non-DRX in synchronous EN-DC |  |  |  |  | ±0.6 dB |  |  |  |  | Overall system uncertainty for AWGN conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Effect of AWGN flatness and signal flatness Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty^2 + (0.25 x AWGN flatness and signal flatness)^2) Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±2.0 dB |  |  |
| 4.5.6.1.2 EN-DC FR1 DCI-based DL active BWP switch with SCell in non-DRX in synchronous EN-DC |  |  |  |  | Same as 4.5.6.1.1 |  |  |  |  | Same as 4.5.6.1.1 |  |  |
| 4.5.6.2.1 EN-DC FR1 RRC-based DL active BWP switch in non-DRX in synchronous EN-DC |  |  |  |  | ±0.6 dB |  |  |  |  | Overall system uncertainty for AWGN conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Effect of AWGN flatness and signal flatness Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty^2 + (0.25 x AWGN flatness and signal flatness)^2) Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±2.0 dB |  |  |
| 4.5.7.1 EN-DC FR1 addition and release delay of known PSCell |  | Average Noc1 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Meas PRB Noc1 ±1.5 dB Meas PRB Ês1 / Noc1 ±0.3 dB |  |  |  |  | Noc1 is the AWGN on NR freq1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |  |  |  |
| 4.5.8.1 EN-DC FR1 interruptions at switching between two uplink carriers |  | Average Noc ±1.5 dB Average Ês / Noc ±0.3 dB |  |  |  |  | Noc is the AWGN on cell 2 frequency Ês / Noc is the ratio of cell 2 signal / AWGN |  |  |  |  |
| 4.6.1.1 EN-DC FR1 event-triggered reporting without gap in non-DRX |  |  | Noc ±1.5 dB Ês2 / Noc ±0.3 dB Ês3 / Noc ±0.3 dB |  |  |  |  | Note: Ês2 / Noc is the ratio of cell 2 signal / AWGN Ês3 / Noc is the ratio of cell 3 signal / AWGN |  |  |  |
| 4.6.1.2 EN-DC FR1 event-triggered reporting without gap in DRX |  |  | Same as 4.6.1.1 |  |  |  |  | Same as 4.6.1.1 |  |  |  |
| 4.6.1.3 EN-DC FR1 event-triggered reporting with gap in non-DRX |  |  | Same as 4.6.1.1 |  |  |  |  | Same as 4.6.1.1 |  |  |  |
| 4.6.1.4 EN-DC FR1 event-triggered reporting with gap in DRX |  |  | Same as 4.6.1.1 |  |  |  |  | Same as 4.6.1.1 |  |  |  |
| 4.6.1.5 EN-DC FR1 event-triggered reporting without gap in non-DRX with SSB time index detection | Same as 4.6.1.1 |  |  |  |  | Same as 4.6.1.1 |  |  |  |  |
| 4.6.1.6 EN-DC FR1 event-triggered reporting with gap in non-DRX with SSB time index detection | Same as 4.6.1.1 |  |  |  |  | Same as 4.6.1.1 |  |  |  |  |
| 4.6.1.7 EN-DC FR1 event-triggered reporting without gap in DRX for UE configured with highSpeedMeasFlag-r16 |  |  | Same as 4.6.1.1 |  |  |  |  | Same as 4.6.1.1 |  |  |  |
| 4.6.2.1 EN-DC FR1-FR1 event-triggered reporting in non-DRX |  |  | Average Noc2 ±1.5 dB Average Noc3 ±1.5 dB Average Ês2 / Noc2 ±0.3 dB Average Ês3 / Noc3 ±0.3 dB Meas PRB Noc2 ±1.5 dB Meas PRB Noc3 ±1.5 dB Meas PRB Ês2 / Noc2 ±0.3 dB Meas PRB Ês3 / Noc3 ±0.3 dB |  |  |  |  | Noc2 is the AWGN on NR freq2 Noc3 is the AWGN on NR freq3 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês3 / Noc3 is the ratio of cell 3 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |  |  |
| 4.6.2.2 EN-DC FR1-FR1 event-triggered reporting in DRX |  |  | Same as 4.6.2.1 |  |  |  |  | Same as 4.6.2.1 |  |  |  |
| 4.6.2.5 EN-DC FR1-FR1 event-triggered reporting in non-DRX with SSB time index detection |  |  | Same as 4.6.2.1 |  |  |  |  | Same as 4.6.2.1 |  |  |  |
| 4.6.2.6 EN-DC FR1-FR1 event-triggered reporting in DRX with SSB time index detection |  |  | Same as 4.6.2.1 |  |  |  |  | Same as 4.6.2.1 |  |  |  |
| 4.6.4.1 EN-DC FR1 SSB-based L1-RSRP measurement in non-DRX |  |  | Average Noc ±1.5 dB Average Ês0 / Noc ±0.3 dB Average Ês1 / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês0 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB |  |  |  |  | Noc is the AWGN on NR freq1 Ês0 / Noc is the SNR for the SSB#0 Ês1 / Noc is the SNR for the SSB#1 Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |  |  |
| 4.6.4.2 EN-DC FR1 SSB-based L1-RSRP measurement in DRX |  |  | Same as 4.6.4.1 |  |  |  |  | Same as 4.6.4.1 |  |  |  |
| 4.6.4.3 EN-DC FR1 CSI-RS-based L1-RSRP measurement in non-DRX |  |  | Average Noc ±1.5 dB Average Ês0 / Noc ±0.3 dB Average Ês1 / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês0 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB |  |  |  |  | Noc is the AWGN on NR freq1 Ês0 / Noc is the SNR for the CSI-RS#0 Ês1 / Noc is the SNR for the CSI-RS#1 Meas PRB is the measurement PRB for CSI-RSRP #RB0 to RB274 |  |  |  |
| 4.6.4.4 EN-DC FR1 CSI-RS-based L1-RSRP measurement in DRX |  |  | Same as 4.6.4.3 |  |  |  |  | Same as 4.6.4.3 |  |  |  |
| 4.6.4.5 EN-DC FR1 SSB-based L1-RSRP measurement in DRX for UE configured with highSpeedMeasFlag-r16 |  |  | Same as 4.6.4.1 |  |  |  |  | Same as 4.6.4.1 |  |  |  |
| 4.6.5.1 EN-DC FR1 SRS-RSRP measurement with non-DRX |  |  | Average Noc ±1.5 dB Average Ês / Noc2 ±0.3 dB |  |  |  |  | Note: Noc is the AWGN on NR frequency Ês / Noc is the SNR |  |  |  |
| 4.6.5.2 EN-DC FR1 CLI-RSSI measurement with non-DRX |  |  | Noc2 ±1.5 dB Ês2 / Noc2 ±0.3 dB |  |  |  |  | Note: Noc2 is the AWGN on cell 2 (NR) frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN |  |  |  |
| 4.6.7.1 EN-DC FR1 CSI-RS based CMR and no dedicated IMR L1-SINR measurement in non-DRX |  |  |  | Average Noc ±1.5 dB Average Ês0 / Noc ±0.3 dB Average Ês1 / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês0 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB |  |  |  |  | Noc is the AWGN on Cell 2 frequency Ês0 / Noc is the SNR for the CSI-RS#0 Ês1 / Noc is the SNR for the CSI-RS#1 Meas PRB is the measurement PRB for CSI-SINR #RB0 to RB275 |  |  |  |
| 4.6.7.2 EN-DC FR1 SSB based CMR and dedicated IMR L1-SINR measurement in DRX |  |  |  | Average Noc ±1.5 dB Average Ês0 / Noc ±0.3 dB Average Ês1 / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês0 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB |  |  |  |  | Noc is the AWGN on Cell 2 frequency Ês0 / Noc is the SNR for the SSB#0 Ês1 / Noc is the SNR for the SSB#1 Meas PRB is the measurement PRB for SS-SINR #RBJ to RBJ+19 |  |  |  |
| 4.6.7.3 EN-DC FR1 CSI-RS based CMR and dedicated IMR L1-SINR measurement in DRX |  |  |  | Average Noc ±1.5 dB Average Ês0 / Noc ±0.3 dB Average Ês1 / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês0 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB |  |  |  |  | Noc is the AWGN on Cell 2 frequency Ês0 / Noc is the SNR for the CSI-RS#0 Ês1 / Noc is the SNR for the CSI-RS#1 Meas PRB is the measurement PRB for CSI-SINR #RB0 to RB275 |  |  |  |
| 4.7.1.1.1 EN-DC FR1 SS-RSRP absolute measurement accuracy |  |  | Average Noc ±1.5 dB Average Ês2 / Noc ±0.3 dB Average Ês3 / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês2 / Noc ±0.3 dB Meas PRB Ês3 / Noc ±0.3 dB |  |  |  |  | Noc is the AWGN on NR freq1 Ês2 / Noc is the cell 2 SNR Ês3 / Noc is the cell 3 SNR Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |  |  |
| 4.7.1.1.2 EN-DC FR1 SS-RSRP relative measurement accuracy |  |  | Same as 4.7.1.1.1 |  |  |  |  | Same as 4.7.1.1.1 |  |  |  |
| 4.7.1.2.1 EN-DC FR1-FR1 SS-RSRP absolute measurement accuracy |  |  | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB Average Ês2 / Noc1 ±0.3 dB Average Ês3 / Noc2 ±0.3 dB Meas PRB Noc1 ±1.5 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês2 / Noc1 ±0.3 dB Meas PRB Ês3 / Noc2 ±0.3 dB |  |  |  |  | Noc1 is the AWGN on NR freq1 Noc2 is the AWGN on NR freq2 Ês2 / Noc1 is the cell 2 SNR Ês3 / Noc2 is the cell 3 SNR Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |  |  |
| 4.7.1.2.2 EN-DC FR1-FR1 SS-RSRP relative measurement accuracy |  |  | Same as 4.7.1.2.1 |  |  |  |  | Same as 4.7.1.2.1 |  |  |  |
| 4.7.2.1 EN-DC FR1 SS-RSRQ measurement accuracy |  |  | Same as 4.7.1.1.1 |  |  |  |  | Same as 4.7.1.1.1 |  |  |  |
| 4.7.2.2.1 EN-DC FR1-FR1 SS-RSRQ absolute measurement accuracy |  |  | Same as 4.7.1.2.1 |  |  |  |  | Same as 4.7.1.2.1 |  |  |  |
| 4.7.2.2.2 EN-DC FR1-FR1 SS-RSRQ relative measurement accuracy |  |  | Same as 4.7.1.2.1 |  |  |  |  | Same as 4.7.1.2.1 |  |  |  |
| 4.7.3.1 EN-DC FR1 SS-SINR measurement accuracy |  |  | Same as 4.7.1.1.1 |  |  |  |  | Same as 4.7.1.1.1 |  |  |  |
| 4.7.3.2.1 EN-DC FR1-FR1 SS-SINR absolute measurement accuracy |  |  | Same as 4.7.1.2.1 |  |  |  |  | Same as 4.7.1.2.1 |  |  |  |
| 4.7.3.2.2 EN-DC FR1-FR1 SS-SINR relative measurement accuracy |  |  | Same as 4.7.1.2.1 |  |  |  |  | Same as 4.7.1.2.1 |  |  |  |
| 4.7.4.1.1 EN-DC FR1 SSB based L1-RSRP absolute measurement accuracy |  |  | Average Noc ±1.5 dB Average Ês2 / Noc ±0.3 dB Meas PRB Ês2 / Noc ±0.8 dB |  |  |  |  | Noc is the AWGN on NR freq1 Ês2 / Noc is the cell 2 SNR Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |  |  |
| 4.7.4.1.2 EN-DC FR1 SSB based L1-RSRP relative measurement accuracy |  |  | Same as 4.7.4.1.1 |  |  |  |  | Same as 4.7.4.1.1 |  |  |  |
| 4.7.4.2.1 EN-DC FR1 CSI-RS based L1-RSRP absolute measurement accuracy |  |  | Same as 4.7.4.1.1 |  |  |  |  | Same as 4.7.4.1.1 |  |  |  |
| 4.7.4.2.2 EN-DC FR1 CSI-RS based L1-RSRP relative measurement accuracy |  |  | Same as 4.7.4.1.1 |  |  |  |  | Same as 4.7.4.1.1 |  |  |  |
| 4.7.5.1 EN-DC FR1 SFTD measurement accuracy |  |  | Average Noc1 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Meas PRB Noc1 ±1.5 dB Meas PRB Ês1 / Noc1 ±0.3 dB Average Noc2 ±1.5 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês2 / Noc2 ±0.8 dB |  |  |  |  | Noc1 is the AWGN on NR freq1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 Noc2 is the AWGN on E-UTRA freq2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RB22 to RB27 |  |  |  |
| 4.7.6.1 EN-DC FR1 SRS-RSRP measurement accuracy |  |  | Average Noc ±1.5 dB Average Ês / Noc2 ±0.3 dB |  |  |  |  | Note: Noc is the AWGN on NR frequency Ês / Noc is the SNR |  |  |  |
| 4.7.6.2 EN-DC CLI-RSSI measurement accuracy with FR1 serving cell |  |  | Same as 4.7.6.1 |  |  |  |  | Same as 4.7.6.1 |  |  |  |
| 4.7.7.1.1 EN-DC FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR absolute measurement accuracy |  |  | Average Noc ±1.5 dB Average Ês2 / Noc ±0.3 dB Meas PRB Ês2 / Noc ±0.8 dB |  |  |  |  | Noc is the AWGN on NR freq1 Ês2 / Noc is the SNR for the CSI-RS#0 & CSI-RS#1 Meas PRB is the measurement PRB for CSI-SINR #RB0 to RB275 |  |  |  |
| 4.7.7.1.2 EN-DC FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR relative measurement accuracy |  |  | Same as 4.7.7.1.1 |  |  |  |  | Same as 4.7.7.1.1 |  |  |  |
| 4.7.7.2 EN-DC FR1 SSB based CMR and dedicated IMR L1-SINR absolute measurement accuracy |  |  | Average Noc ±1.5 dB Average Ês2 / Noc ±0.3 dB Meas PRB Ês2 / Noc ±0.8 dB |  |  |  |  | Noc is the AWGN on NR freq1 Ês2 / Noc is the SNR for the SSB#0 & SSB#1 Meas PRB is the measurement PRB for SS-SINR #RBJ to RBJ+19 |  |  |  |
| 4.7.7.3.1 EN-DC FR1 CSI-RS based CMR and dedicated IMR L1-SINR absolute measurement accuracy |  |  | Same as 4.7.7.1.1 |  |  |  |  | Same as 4.7.7.1.1 |  |  |  |
| 4.7.7.3.2 EN-DC FR1 CSI-RS based CMR and dedicated IMR L1-SINR relative measurement accuracy |  |  | Same as 4.7.7.1.1 |  |  |  |  | Same as 4.7.7.1.1 |  |  |  |

Table F.1.1.2-4 Maximum test system uncertainty for RRM requirements for SA FR1 test cases

| Subclause | Maximum Test System Uncertainty1 |  | Derivation of Test System Uncertainty |
| --- | --- | --- | --- |
| 6.1.1.1 NR SA FR1 cell re-selection | Noc ±1.5 dB Ês1 / Noc ±0.3 dB Ês2 / Noc ±0.3 dB |  | Note: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |
| 6.1.1.2 NR SA FR1-FR1 cell re-selection | Noc1 ±1.5 dB Ês1 / Noc1 ±0.3 dB Noc2 ±1.5 dB Ês2 / Noc2 ±0.3 dB |  | Note: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN |
| 6.1.1.3 NR SA FR1 cell re-selection for UE fulfilling low mobility relaxed measurement criterion | Same as 6.1.1.1 |  | Same as 6.1.1.1 |
| 6.1.1.4 NR SA FR1 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion | Same as 6.1.1.1 |  | Same as 6.1.1.1 |
| 6.1.1.5 NR SA FR1-FR1 cell re-selection for UE fulfilling low mobility relaxed measurement criterion | Same as 6.1.1.2 |  | Same as 6.1.1.2 |
| 6.1.1.6 NR SA FR1-FR1 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion | Same as 6.1.1.2 |  | Same as 6.1.1.2 |
| 6.1.1.7 NR SA FR1 cell re-selection for UE configured with highSpeedMeasFlag-r16 | Same as 6.1.1.1 |  | Same as 6.1.1.1 |  |
| 6.1.2.1 NR SA FR1 – E-UTRA cell re-selection to higher priority E-UTRA | Noc1 ±1.5 dB Ês1 / Noc1 ±0.3 dB Noc2 ±1.5 dB Ês2 / Noc2 ±0.3 dB |  | Note: Noc1 is the AWGN on cell 1 (NR) frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on cell 2 (E-UTRAN) frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN |
| 6.1.2.2 NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRA | Same as 6.1.2.1 |  | Same as 6.1.2.1 |
| 6.1.2.3 NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRAN for UE fulfilling low mobility relaxed measurement criterion | Same as 6.1.2.1 |  | Same as 6.1.2.1 |
| 6.1.2.4 NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRAN for UE fulfilling not-at-cell edge relaxed measurement criterion | Same as 6.1.2.1 |  | Same as 6.1.2.1 |
| 6.1.2.5 NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRA for UE configured with highSpeedMeasFlag-r16 | Same as 6.1.2.1 | Same as 6.1.2.1 |  |
| 6.3.1.1 NR SA FR1 handover with known target cell | Average Noc ±1.5 dB Average Ês1 / Noc ±0.3 dB Average Ês2 / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês1 / Noc ±0.3 dB Meas PRB Ês2 / Noc ±0.3 dB | Note: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |
| 6.3.1.2 NR SA FR1 handover with unknown target cell | Same as 6.3.1.1 | Same as 6.3.1.1 |  |
| 6.3.1.3 NR SA FR1-FR1 Handover with unknown Target Cell | Noc1 ±1.5 dB Ês1 / Noc1 ±0.3 dB Noc2 ±1.5 dB Ês2 / Noc2 ±0.3 dB | Note: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN |  |
| 6.3.1.4 NR SA FR1 – E-UTRA handover with known target cell | Same as 6.1.2.1 |  | Same as 6.1.2.1 |
| 6.3.1.5 NR SA FR1 – E-UTRA handover with unknown target cell | Same as 6.1.2.1 |  | Same as 6.1.2.1 |
| 6.3.1.6 NR SA FR1 – UTRAN FDD handover with known target cell | Noc ±1.5 dB Ês / Noc ±0.3 dB Ioc ±0.7 dB Îor / Ioc ± 0.3dB CPICH Ec / Ior ± 0.1dB SCH Ec / Ior ± 0.1dB |  | Note: Noc is the AWGN on cell 1 frequency Ês / Noc is the ratio of cell 1 signal / AWGN Ioc is the AWGN on cell 2 frequency Îor / Ioc is the ratio of cell 2 signal / AWGN CPICH Ec / Ior is the ratio of cell 2 CPICH code level / Ior SCH Ec / Ior is the ratio of cell 2 SCH code level / Ior |
| 6.3.1.7 NR SA FR1 synchronous DAPS handover | Same as 6.3.1.1 |  | Same as 6.3.1.1 |
| 6.3.1.8 NR SA FR1 asynchronous DAPS handover | Same as 6.3.1.1 |  | Same as 6.3.1.1 |
| 6.3.1.9 NR SA FR1 Intra-band inter-frequency synchronous DAPS handover | Same as 6.3.1.3 |  | Same as 6.3.1.3 |
| 6.3.1.10 NR SA FR1 Intra-band inter-frequency asynchronous DAPS handover | Same as 6.3.1.3 |  | Same as 6.3.1.3 |
| 6.3.1.11 NR SA FR1 Inter-band inter-frequency synchronous DAPS handover | Same as 6.3.1.3 |  | Same as 6.3.1.3 |  |
| 6.3.1.12 NR SA FR1 Inter-band inter-frequency asynchronous DAPS handover | Same as 6.3.1.3 |  | Same as 6.3.1.3 |  |
| 6.3.2.1.1 NR SA FR1 RRC re-establishment | Same as 6.1.1.1 |  | Same as 6.1.1.1 |
| 6.3.2.1.2 NR SA FR1 – FR1 RRC re-establishment | Same as 6.1.1.2 |  | Same as 6.1.1.2 |
| 6.3.2.1.3 NR SA FR1 RRC re-establishment without serving cell timing | Average Noc ±1.5 dB Average Ês1 / Noc ±0.3 dB Average Ês2 / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês1 / Noc ±0.3 dB Meas PRB Ês2 / Noc ±0.3 dB |  | Note: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 6.3.2.2.1 | Noc ±1.5 dB Ês / Noc ±0.3 dB Uplink absolute power measurement ±1.5 dB Uplink relative power measurement ±0.7 dB ±112Tc Uplink signal transmit timing relative to downlink |  | Ês1 / Noc is the ratio of cell 1 signal / AWGN Tc = 1/(480000 x 4096) seconds, the basic timing unit defined in TS 38.211 [7] |
| 6.3.2.2.2 | Same as 6.3.2.2.1 |  | Same as 6.3.2.2.1 |
| 6.3.2.2.3 | Same as 6.3.2.2.1 |  | Same as 6.3.2.2.1 |
| 6.3.2.2.4 | Same as 6.3.2.2.1 |  | Same as 6.3.2.2.1 |
| 6.3.2.3.1 NR SA FR1 RRC connection release with redirection | Same as 6.1.1.2 |  | Same as 6.1.1.2 |
| 6.3.2.3.2 NR SA FR1 – E-UTRA RRC connection release with redirection | Same as 6.1.2.1 |  | Same as 6.1.2.1 |
| 6.3.3.1 NR SA FR1 conditional handover | Same as 6.3.1.1 |  | Same as 6.3.1.1 |
| 6.3.3.2 NR SA FR1-FR1 conditional handover | Same as 6.3.1.3 |  | Same as 6.3.1.3 |
| 6.4.1.1 EN-DC FR1 UE transmit timing accuracy | Noc ±1.5 dB Ês1 / Noc ±0.3 dB ±112Tc Uplink signal transmit timing relative to downlink |  | Ês1 / Noc is the ratio of cell 1 signal / AWGN Tc = 1/(480000 x 4096) seconds, the basic timing unit defined in TS 38.211 [7] |
| 6.4.3.1 NR SA FR1 timing advance adjustment accuracy | Noc ±1.5 dB Ês1 / Noc ±0.3 dB ±88Tc Timing Advance Adjustment accuracy |  | Ês1 / Noc is the ratio of cell 1 signal / AWGN Tc = 1/(480000 x 4096) seconds, the basic timing unit defined in TS 38.211 [7] |
| 6.5.1.1 NR SA FR1 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in non-DRX mode | Average Noc ±1.5 dB Average Ês / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês / Noc ±0.3 dB Fading profile ±0.7 dB |  | Noc is the AWGN on cell 1 frequency Ês / Noc is the SNR Meas PRB are the measurement PRB for SSB-based RLM #RBJ to RBJ+19 |
| 6.5.1.2 NR SA FR1 radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in non-DRX mode | Same as 6.5.1.1 |  | Same as 6.5.1.1 |  |
| 6.5.1.3 NR SA FR1 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in DRX mode | Same as 6.5.1.1 |  | Same as 6.5.1.1 |  |
| 6.5.1.4 NR SA FR1 radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in DRX mode | Same as 6.5.1.1 |  | Same as 6.5.1.1 |  |
| 6.5.1.5 NR SA FR1 radio link monitoring out-of-sync test for PCell configured with CSI-RS-based RLM RS in non-DRX mode | Same as 6.5.1.1 |  | Same as 6.5.1.1 |  |
| 6.5.1.6 NR SA FR1 radio link monitoring in-sync test for PCell configured with CSI-RS-based RLM RS in non-DRX mode | Same as 6.5.1.1 |  | Same as 6.5.1.1 |  |
| 6.5.1.7 NR SA FR1 radio link monitoring out-of-sync test for PCell configured with CSI-RS-based RLM RS in DRX mode | Same as 6.5.1.1 |  | Same as 6.5.1.1 |  |
| 6.5.1.8 NR SA FR1 radio link monitoring in-sync test for PCell configured with CSI-RS-based RLM RS in DRX mode | Same as 6.5.1.1 |  | Same as 6.5.1.1 |  |
| 6.5.2.1 NR SA FR1 interruptions during measurements on deactivated NR SCC | Noc1 ±1.5 dB Noc2 ±1.5 dB Ês1 / Noc1 ±0.3 dB Ês2 / Noc2 ±0.3 dB |  | Note: Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês2 / Noc2 is the ratio of cell 2 signal / AWGN |  |
| 6.5.3.1 NR SA FR1 SCell activation and deactivation of known SCell in non-DRX for 160ms SCell measurement cycle | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc1 ±1.5 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês1 / Noc1 ±0.3 dB Meas PRB Ês2 / Noc2 ±0.3 dB |  | Note: Noc1 is the AWGN on cell 1 frequency Noc2 is the AWGN on cell 2 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |
| 6.5.3.2 NR SA FR1 SCell activation and deactivation of known SCell in non-DRX for 640ms SCell measurement cycle | Same as 6.5.3.1 |  | Same as 6.5.3.1 |  |
| 6.5.3.3 NR SA FR1 SCell activation and deactivation of unknown SCell in non-DRX | Same as 6.5.3.1 |  | Same as 6.5.3.1 |  |
| 6.5.5.1 NR SA FR1 SSB-based beam failure detection and link recovery in non-DRX | Average Noc1 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Meas PRB Noc1 ±1.5 dB Meas PRB Ês1 / Noc1 ±0.3 dB Fading profile ±0.7 dB |  | Note: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the SNR for the SSB Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |
| 6.5.5.2 NR SA FR1 SSB-based beam failure detection and link recovery in DRX | Same as 6.5.5.1 |  | Same as 6.5.5.1 |  |
| 6.5.5.3 NR SA FR1 CSI-RS-based beam failure detection and link recovery in non-DRX | Average Noc1 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Meas PRB Noc1 ±1.5 dB Meas PRB Ês1 / Noc1 ±0.3 dB Fading profile ±0.7 dB |  | Note: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the SNR for the CSI-RS Meas PRB is the measurement PRB for CSI-RSRP #RB0 to RB274 |  |
| 6.5.5.4 NR SA FR1 CSI-RS-based beam failure detection and link recovery in DRX | Same as 6.5.5.3 |  | Same as 6.5.5.3 |  |
| 6.5.5.5 NR SA FR1 Scell CSI-RS-based beam failure detection and SSB-based link recovery in non-DRX | Average Noc2 ±1.5 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês2 / Noc2 ±0.3 dB Fading profile ±0.7 dB |  | Note: Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the SNR for the SSB and CSI-RS Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |
| 6.5.5.6 NR SA FR1 Scell CSI-RS-based beam failure detection and SSB-based link recovery in DRX | Same as 6.5.5.5 |  | Same as 6.5.5.5 |  |
| 6.5.6.1.1 NR SA FR1-FR1 DCI-based DL active BWP switch in non-DRX | ±0.6 dB |  | Overall system uncertainty for AWGN conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Effect of AWGN flatness and signal flatness Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty^2 + (0.25 x AWGN flatness and signal flatness)^2) Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±2.0 dB |  |
| 6.5.6.1.2 NR SA FR1 DCI-based DL active BWP switch in non-DRX | Same as 6.5.6.1.1 |  | Same as 6.5.6.1.1 |  |
| 6.5.6.2.1 NR SA FR1 RRC-based DL active BWP switch in non-DRX | ±0.6 dB |  | Overall system uncertainty for AWGN conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Effect of AWGN flatness and signal flatness Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty^2 + (0.25 x AWGN flatness and signal flatness)^2) Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±2.0 dB |  |
| 6.5.7.1 NR SA FR1 DL Interruptions at switching between two uplink carriers in FDD-TDD CA | Noc1 ±1.5 dB Noc2 ±1.5 dB Ês1 / Noc ±0.3 dB Ês2 / Noc ±0.3 dB |  | Note: Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês2 / Noc2 is the ratio of cell 2 signal / AWGN |  |
| 6.5.7.2 NR SA FR1 DL Interruptions at switching between two uplink carriers in TDD-TDD CA | Same as 6.5.7.1 |  | Same as 6.5.7.1 |  |
| 6.6.1.1 SA event triggered reporting tests without gap under non-DRX | Noc ±1.5 dB Ês1 / Noc ±0.3 dB Ês2 / Noc ±0.3 dB |  | Note: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |  |
| 6.6.1.2 SA event triggered reporting tests without gap under DRX | Same as 6.6.1.1 |  | Same as 6.6.1.1 |  |
| 6.6.1.3 SA event triggered reporting tests with per-UE gaps under non-DRX | Same as 6.6.1.1 |  | Same as 6.6.1.1 |  |
| 6.6.1.4 SA event triggered reporting tests with per-UE gaps under DRX | Same as 6.6.1.1 |  | Same as 6.6.1.1 |  |
| 6.6.1.5 SA event triggered reporting tests without gap under non-DRX with SSB index reading | Same as 6.6.1.1 |  | Same as 6.6.1.1 |  |
| 6.6.1.6 SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading | Same as 6.6.1.1 |  | Same as 6.6.1.1 |  |
| 6.6.1.7 NR SA FR1 event-triggered reporting without gap in DRX for UE configured with highSpeedMeasFlag-r16 | Same as 6.6.1.1 |  | Same as 6.6.1.1 |  |
| 6.6.2.1 NR SA FR1-FR1 event-triggered reporting in non-DRX | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc1 ±1.5 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês1 / Noc1 ±0.3 dB Meas PRB Ês2 / Noc2 ±0.3 dB |  | Noc1 is the AWGN on NR freq1 Noc2 is the AWGN on NR freq2 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |
| 6.6.2.2 NR SA FR1-FR1 event-triggered reporting in DRX | Same as 6.6.2.1 |  | Same as 6.6.2.1 |  |
| 6.6.2.5 NR SA FR1-FR1 event-triggered reporting in non-DRX with SSB time index detection | Same as 6.6.2.1 |  | Same as 6.6.2.1 |  |
| 6.6.2.6 NR SA FR1-FR1 event-triggered reporting in DRX with SSB time index detection | Same as 6.6.2.1 |  | Same as 6.6.2.1 |  |
| 6.6.3.1 NR SA FR1 – E-UTRAN event-triggered reporting in non-DRX | Freq 1 Noc ±1.5 dB Freq 2 Noc ±1.5 dB Ês1 / Noc ±0.3 dB Ês2 / Noc ±0.3 dB Fading ±0.5 dB |  | Note: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |  |
| 6.6.3.2 NR SA FR1 – E-UTRAN event-triggered reporting in DRX | Same as 6.6.3.1 |  | Same as 6.6.3.1 |  |
| 6.6.3.3 NR SA FR1 – E-UTRAN event-triggered reporting in DRX for UE configured with highSpeedMeasFlag-r16 | Freq 1 Noc ±1.5 dB Freq 2 Noc ±1.5 dB Ês1 / Noc ±0.3 dB Ês2 / Noc ±0.3 dB |  | Note: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |  |
| 6.6.4.1 NR SA FR1 SSB-based L1-RSRP measurement in non-DRX | Average Noc ±1.5 dB Average Ês0 / Noc ±0.3 dB Average Ês1 / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês0 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB |  | Noc is the AWGN on NR freq1 Ês0 / Noc is the SNR for the SSB#0 Ês1 / Noc is the SNR for the SSB#1 Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |
| 6.6.4.2 NR SA FR1 SSB-based L1-RSRP measurement in DRX | Same as 6.6.4.1 |  | Same as 6.6.4.1 |  |
| 6.6.4.3 NR SA FR1 CSI-RS-based L1-RSRP measurement in non-DRX | Average Noc ±1.5 dB Average Ês0 / Noc ±0.3 dB Average Ês1 / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês0 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB |  | Noc is the AWGN on NR freq1 Ês0 / Noc is the SNR for the CSI-RS#0 Ês1 / Noc is the SNR for the CSI-RS#1 Meas PRB is the measurement PRB for CSI-RSRP #RB0 to RB274 |  |
| 6.6.4.4 NR SA FR1 CSI-RS-based L1-RSRP measurement in DRX | Same as 6.6.4.3 |  | Same as 6.6.4.3 |  |
| 6.6.4.5 NR SA FR1 SSB-based L1-RSRP measurement in DRX for UE configured with highSpeedMeasFlag-r16 | Same as 6.6.4.1 |  | Same as 6.6.4.1 |  |
| 6.6.5.1 NR SA FR1 – UTRAN event-triggered reporting in non-DRX | Noc ±1.5 dB Ês / Noc ±0.3 dB Ioc ±0.7 dB Îor / Ioc ± 0.3dB CPICH Ec / Ior ± 0.1dB SCH Ec / Ior ± 0.1dB |  | Note: Noc is the AWGN on cell 1 frequency Ês / Noc is the ratio of cell 1 signal / AWGN Ioc is the AWGN on cell 2 frequency Îor / Ioc is the ratio of cell 2 signal / AWGN CPICH Ec / Ior is the ratio of cell 2 CPICH code level / Ior SCH Ec / Ior is the ratio of cell 2 SCH code level / Ior |  |
| 6.6.6.1 NR SA FR1 SRS-RSRP measurement in non-DRX | Average Noc ±1.5 dB Average Ês / Noc ±0.3 dB |  | Note: Noc is the AWGN on NR frequency Ês / Noc is the SNR |  |
| 6.6.6.2 NR SA FR1 CLI-RSSI measurement in non-DRX | Same as 6.6.6.1 |  | Same as 6.6.6.1 |  |
| 6.6.8.1 NR SA FR1 CSI-RS based CMR and no dedicated IMR L1-SINR measurement in DRX | Average Noc ±1.5 dB Average Ês0 / Noc ±0.3 dB Average Ês1 / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês0 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB |  | Noc is the AWGN on Cell 1 frequency Ês0 / Noc is the SNR for the CSI-RS#0 Ês1 / Noc is the SNR for the CSI-RS#1 Meas PRB is the measurement PRB for CSI-SINR #RB0 to RB275 |  |
| 6.6.8.2 NR SA FR1 SSB based CMR and dedicated IMR L1-SINR measurement in non-DRX | Average Noc ±1.5 dB Average Ês0 / Noc ±0.3 dB Average Ês1 / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês0 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB |  | Noc is the AWGN on Cell 1 frequency Ês0 / Noc is the SNR for the SSB#0 Ês1 / Noc is the SNR for the SSB#1 Meas PRB is the measurement PRB for SS-SINR #RBJ to RBJ+19 |  |
| 6.6.8.3 NR SA FR1 CSI-RS based CMR and dedicated IMR L1-SINR measurement in non-DRX | Average Noc ±1.5 dB Average Ês0 / Noc ±0.3 dB Average Ês1 / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês0 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB |  | Noc is the AWGN on Cell 1 frequency Ês0 / Noc is the SNR for the CSI-RS#0 Ês1 / Noc is the SNR for the CSI-RS#1 Meas PRB is the measurement PRB for CSI-SINR #RB0 to RB275 |  |
| 6.6.9.1 NR SA FR1 Idle mode CA/DC measurement for FR1 | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc1 ±1.5 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês1 / Noc1 ±0.3 dB Meas PRB Ês2 / Noc2 ±0.3 dB |  | Noc1 is the AWGN on NR freq1 Noc2 is the AWGN on NR freq2 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |
| 6.6.15.1 NR SA FR1 Idle Mode measurements of inter-RAT CA candidate cells for early reporting | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc1 ±1.5 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês1 / Noc1 ±0.3 dB Meas PRB Ês2 / Noc2 ±0.3 dB |  | Noc1 is the AWGN on NR freq1 Noc2 is the AWGN on LTE freq2 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |
| 6.7.1.1.1 NR SA FR1 SS-RSRP absolute measurement accuracy | Average Noc ±1.5 dB Average Ês1 / Noc ±0.3 dB Average Ês2 / Noc ±0.3 dB meas PRB Ês1 / Noc ±0.3 dB meas PRB Ês2 / Noc ±0.3 dB |  | Noc is the AWGN on NR freq1 Ês1 / Noc is the cell 1 SNR Ês2 / Noc is the cell 2 SNR meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |
| 6.7.1.1.2 NR SA FR1 SS-RSRP relative measurement accuracy | Same as 6.7.1.1.1 |  | Same as 6.7.1.1.1 |  |
| 6.7.1.2.1 NR SA FR1-FR1 SS-RSRP absolute measurement accuracy | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Average Ês2 / Noc2 ±0.3 dB meas PRB Ês1 / Noc1 ±0.3 dB meas PRB Ês2 / Noc2 ±0.3 dB |  | Noc1 is the AWGN on NR freq1 Noc2 is the AWGN on NR freq2 Ês1 / Noc1 is the cell 2 SNR Ês2 / Noc2 is the cell 3 SNR meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 6.7.1.2.2 NR SA FR1-FR1 SS-RSRP relative measurement accuracy | Same as 6.7.1.2.1 |  | Same as 6.7.1.2.1 |
| 6.7.2.1 NR SA FR1 SS-RSRQ measurement accuracy | Same as 6.7.1.1.1 |  | Same as 6.7.1.1.1 |
| 6.7.2.2.1 NR SA FR1-FR1 SS-RSRQ absolute measurement accuracy | Same as 6.7.1.2.1 |  | Same as 6.7.1.2.1 |
| 6.7.2.2.2 NR SA FR1-FR1 SS-RSRQ relative measurement accuracy | Same as 6.7.1.2.1 |  | Same as 6.7.1.2.1 |
| 6.7.3.1 NR SA FR1 SS-SINR measurement accuracy | Same as 6.7.1.1.1 |  | Same as 6.7.1.1.1 |
| 6.7.3.2.1 NR SA FR1-FR1 SS-SINR absolute measurement accuracy | Same as 6.7.1.2.1 |  | Same as 6.7.1.2.1 |
| 6.7.3.2.2 NR SA FR1-FR1 SS-SINR relative measurement accuracy | Same as 6.7.1.2.1 |  | Same as 6.7.1.2.1 |
| 6.7.4.1.1 NR SA FR1 SSB based L1-RSRP absolute measurement accuracy | Average Noc ±1.5 dB Average Ês1 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.8 dB |  | Noc is the AWGN on NR freq1 Ês2 / Noc is the cell 1 SNR Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 6.7.4.1.2 NR SA FR1 SSB based L1-RSRP relative measurement accuracy | Same as 6.7.4.1.1 |  | Same as 6.7.4.1.1 |
| 6.7.4.2.1 NR SA FR1 CSI-RS based L1-RSRP absolute measurement accuracy | Same as 6.7.4.1.1 |  | Same as 6.7.4.1.1 |
| 6.7.4.2.2 NR SA FR1 CSI-RS based L1-RSRP relative measurement accuracy | Same as 6.7.4.1.1 |  | Same as 6.7.4.1.1 |
| 6.7.5.1 NR SA FR1 – E-UTRAN RSRP absolute measurement accuracy | Average Noc ±1.5 dB Average Ês / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês / Noc ±0.8 dB |  | Noc is the AWGN on E-UTRA freq1 Ês / Noc is the ratio of cell 1 signal / AWGN Meas PRB are the measurement PRB for RSRP #RB22 to RB27 |
| 6.7.6.1 NR SA FR1 – E-UTRAN RSRQ absolute measurement accuracy | Same as 6.7.5.1 |  | Same as 6.7.5.1 |
| 6.7.7.1 NR SA FR1 – E-UTRAN RS-SINR absolute measurement accuracy | Same as 6.7.5.1 |  | Same as 6.7.5.1 |
| 6.7.8.1 NR SA FR1 SRS-RSRP measurement accuracy | Average Noc ±1.5 dB Average Ês / Noc ±0.3 dB |  | Note: Noc is the AWGN on NR frequency Ês / Noc is the SNR |
| 6.7.8.2 NR SA FR1 CLI-RSSI measurement accuracy | Same as 6.7.8.1 |  | Same as 6.7.8.1 |
| 6.7.9.1.1 NR SA FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR absolute measurement accuracy | Average Noc ±1.5 dB Average Ês2 / Noc ±0.3 dB Meas PRB Ês2 / Noc ±0.8 dB |  | Noc is the AWGN on NR freq1 Ês2 / Noc is the SNR for the CSI-RS#0 & CSI-RS#1 Meas PRB is the measurement PRB for CSI-SINR #RB0 to RB275 |
| 6.7.9.1.2 NR SA FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR relative measurement accuracy | Same as 6.7.9.1.1 |  | Same as 6.7.9.1.1 |
| 6.7.9.2 NR SA FR1 SSB based CMR and dedicated IMR L1-SINR absolute measurement accuracy | Average Noc ±1.5 dB Average Ês2 / Noc ±0.3 dB Meas PRB Ês2 / Noc ±0.8 dB |  | Noc is the AWGN on NR freq1 Ês2 / Noc is the SNR for the SSB#0 & SSB#1 Meas PRB is the measurement PRB for SS-SINR #RBJ to RBJ+19 |
| 6.7.9.3.1 NR SA FR1 CSI-RS based CMR and dedicated IMR L1-SINR absolute measurement accuracy | Same as 6.7.9.1.1 |  | Same as 6.7.9.1.1 |
| 6.7.9.3.2 NR SA FR1 CSI-RS based CMR and dedicated IMR L1-SINR relative measurement accuracy | Same as 6.7.9.1.1 |  | Same as 6.7.9.1.1 |

Table F.1.1.2-5: Maximum test system uncertainty for RRM requirements for EN-DC FR2 test cases

| Subclause | Maximum Test System Uncertainty1 | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 5.3.2.2.1 EN-DC FR2 contention based random access | Average Ês ±6.0 dB Meas PRB Ês ±6.0 dB Uplink absolute power measurement ±6.0 dB Uplink relative power measurement ±FFS dB ±[48]Tc Uplink signal transmit timing relative to downlink | Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 Tc = 1/(480000 x 4096) seconds, the basic timing unit defined in TS 38.211 [7] |
| 5.3.2.2.2 EN-DC FR2 non-contention based random access | Same as 5.3.2.2.1 | Same as 5.3.2.2.1 |
| 5.4.1.1 EN-DC FR2 UE transmit timing accuracy | Noc ±5.65 dB, f<=40.8 GHz Ês2 / Noc ±0.3 dB ±48Tc Uplink signal transmit timing relative to downlink ±40Tc Relative during UE timing adjustment | Ês2 / Noc is the ratio of cell 2 signal / AWGN Tc = 1/(480000 x 4096) seconds, the basic timing unit defined in TS 38.211 [7] |
| 5.4.3.1 EN-DC FR2 UE timing advance adjustment accuracy | Noc ±5.65 dB, f<=40.8 GHz Ês1 / Noc ±0.3 dB ±40Tc timing advance adjustment | Ês1 / Noc is the ratio of cell 1 signal / AWGN Tc = 1/(480000 x 4096) seconds, the basic timing unit defined in TS 38.211 [7] |
| 5.5.1.1 EN-DC FR2 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode | Average Noc ±5.65 dB, f<=40.8 GHz Average Ês1 / Noc ±0.3 dB Average Ês2 / Noc ±0.3 dB Fading profile ±0.5 dB for 1Tx | Ês1 / Noc is the SNR of RLM-RS1 Ês2 / Noc is the SNR of RLM-RS2 |
| 5.5.1.2 EN-DC FR2 radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode | Same as 5.5.1.1 | Same as 5.5.1.1 |
| 5.5.1.3 EN-DC FR2 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in DRX mode | Same as 5.5.1.1 | Same as 5.5.1.1 |
| 5.5.1.4 EN-DC FR2 radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in DRX mode | Same as 5.5.1.1 | Same as 5.5.1.1 |
| 5.5.1.5 EN-DC FR2 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode | Average Noc ±5.65 dB, f<=40.8 GHz Average Ês1 / Noc ±0.3 dB Average Ês2 / Noc ±0.3 dB Fading profile ±0.5 dB for 1Tx | Ês1 / Noc is the SNR of RLM-RS1 Ês2 / Noc is the SNR of RLM-RS2 |
| 5.5.1.6 EN-DC FR2 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode | Same as 5.5.1.5 | Same as 5.5.1.5 |
| 5.5.1.7 EN-DC FR2 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in DRX mode | Same as 5.5.1.5 | Same as 5.5.1.5 |
| 5.5.1.8 EN-DC FR2 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM RS in DRX mode | Same as 5.5.1.5 | Same as 5.5.1.5 |
| 5.5.2.1 EN-DC FR2 interruptions at transitions between active and non-active during DRX in synchronous EN-DC | Noc ±5.65 dB, f<=40.8 GHz Ês2 / Noc ±0.3 dB | Ês2 / Noc is the ratio of cell 2 signal / AWGN |
| 5.5.2.2 EN-DC FR2 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC | Same as 5.5.2.1 | Same as 5.5.2.1 |
| 5.5.2.3 EN-DC FR2 interruptions during measurements on deactivated NR SCC in synchronous EN-DC | Noc ±5.65 dB, f<=40.8 GHz Ês1 / Noc ±0.3 dB Ês2 / Noc ±0.3 dB | Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |
| 5.5.2.4 EN-DC FR2 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC | Same as 5.5.2.4 | Same as 5.5.2.4 |
| 5.5.5.1 EN-DC FR2 SSB-based beam failure detection and link recovery in non-DRX | Noc averaged over BWConfig, ±5.65 dB, f<=40.8 GHz Noc averaged over measured PRBs, ±5.65 dB, f<=40.8 GHz Ês / Noc averaged over BWConfig: ±0.3 dB Ês / Noc averaged over measured PRBs: ±0.3 dB Fading profile ±0.7 dB, 2Tx | Noc averaged over BWConfig is the AWGN absolute power averaged over BWConfig Noc averaged over measured PRBs is the AWGN absolute power averaged over measured PRBs Ês / Noc averaged over BWConfig is the Ratio of cell signal / AWGN averaged over BWConfig Ês / Noc averaged over measured PRBs is the Ratio of cell signal / AWGN averaged over measured PRBs |
| 5.5.5.2 EN-DC FR2 SSB-based beam failure detection and link recovery in DRX | Same as 5.5.5.1 | Same as 5.5.5.1 |
| 5.5.5.3 EN-DC FR2 CSI-RS-based beam failure detection and link recovery in non-DRX | Noc ±5.65 dB, f<=40.8 GHz Ês2 / Noc ±0.3 dB Fading profile ±0.7 dB for 2Tx | Note: Noc is the AWGN on cell 2 frequency Ês2 / Noc is the SNR for the CSI-RS |
| 5.5.5.4 EN-DC FR2 CSI-RS-based beam failure detection and link recovery in DRX | Same as 5.5.5.3 | Same as 5.5.5.3 |
| 5.5.5.5 EN-DC FR2 scheduling available restriction during SSB-based beam failure detection and link recovery in non-DRX | Same as 5.5.5.1 | Same as 5.5.5.1 |
| 5.5.5.6 EN-DC FR2 SCell CSI-RS-based beam failure detection and link recovery in non-DRX | Noc ±5.65 dB, f<=40.8 GHz Ês3 / Noc ±0.3 dB Fading profile ±0.7 dB for 2Tx | Note: Noc is the AWGN on cell 3 frequency Ês3 / Noc is the SNR for the CSI-RS |
| 5.5.5.7 EN-DC FR2 SCell CSI-RS-based beam failure detection and link recovery in DRX | Same as 5.5.5.6 | Same as 5.5.5.6 |
| 5.5.6.2.1 EN-DC FR2 RRC-based DL active BWP switch in non-DRX in synchronous EN-DC | ±1.0 dB | Overall system uncertainty for fading conditions comprises two quantities: 1. gNB emulator Signal-to-noise ratio uncertainty 2. Effect of AWGN flatness and signal flatness Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (gNB emulator Signal-to-noise ratio uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) gNB emulator Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±3.6 dB |
| 5.6.1.1 EN-DC FR2 event-triggered reporting without gap in non-DRX | Ês2 averaged over BW ±5.65 dB, f <= 40.8 GHz Ês2 averaged over measured PRB ±5.65 dB, f <= 40.8 GHz Ês3 averaged over BW ±5.65 dB, f <= 40.8 GHz Ês3 averaged over measured PRB ±5.65 dB, f <= 40.8 GHz | Ês2 averaged over BW is the absolute power of cell 2 signal averaged over BW Ês2 averaged over measured PRB is the absolute power of cell 2 signal averaged over measured PRB Ês3 averaged over BW is the absolute power of cell 3 signal averaged over BW Ês3 averaged over measured PRB is the absolute power of cell 3 signal averaged over measured PRB |
| 5.6.1.2 EN-DC FR2 event-triggered reporting without gap in DRX | Noc ±5.65 dB Ês2 / Noc ±0.3 dB Ês3 / Noc ±0.3 dB | Ês2 / Noc is the ratio of cell 2 signal / AWGN Ês3 / Noc is the ratio of cell 3 signal / AWGN |
| 5.6.1.3 EN-DC FR2 event-triggered reporting without gap in non-DRX | Same as 5.6.1.1 | Same as 5.6.1.1 |
| 5.6.1.4 EN-DC FR2 event-triggered reporting with gap in DRX | Same as 5.6.1.2 | Same as 5.6.1.2 |
| 5.6.2.1 EN-DC FR2-FR2 event-triggered reporting in non-DRX | Average Ês1 ±5.65 dB, f <= 40.8 GHz Meas PRB Ês1 ±5.65 dB, f <= 40.8 GHz Average Ês2 ±5.65 dB Meas PRB Ês2 ±5.65 dB | Ês1 is NR cell 2 signal Ês2 is NR cell 3 signal Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 5.6.2.2 EN-DC FR2-FR2 event-triggered reporting in DRX | Average Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Average Ês1 / Noc ±0.3 dB Ês1 / Noc over meas PRBs ±0.3 dB Average Ês2 / Noc ±0.3 dB Ês2 / Noc over meas PRBs ±0.3 dB | Ês1 is NR cell 2 signal Ês2 is NR cell 3 signal Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 5.6.2.3 EN-DC FR2-FR2 event-triggered reporting in non-DRX with SSB time index detection | Same as 5.6.2.1 | Same as 5.6.2.1 |
| 5.6.2.4 EN-DC FR2-FR2 event-triggered reporting in DRX with SSB time index detection | Same as 5.6.2.2 | Same as 5.6.2.2 |
| 5.6.2.5 EN-DC FR1-FR2 event-triggered reporting in non-DRX | Average Ês2 ±5.65, f <= 40.8 GHz | Ês2 is NR cell 2 signal Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 5.6.2.6 EN-DC FR1-FR2 event-triggered reporting in DRX | Noc ±5.65 dB, f <= 40.8 GHz Ês2 / Noc ±0.3 dB | Ês2 / Noc is the ratio of cell 2 signal / AWGN |
| 5.6.2.7 EN-DC FR1-FR2 event-triggered reporting in non-DRX with SSB time index detection | Same as 5.6.2.5 | Same as 5.6.2.5 |
| 5.6.2.8 EN-DC FR1-FR2 event-triggered reporting in DRX with SSB time index detection | Same as 5.6.2.6 | Same as 5.6.2.6 |
| 5.6.3.1 EN-DC FR2 SSB-based L1-RSRP measurement in non-DRX | Average Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Average Ês0 / Noc ±0.3 dB Ês0 / Noc over meas PRBs ±0.3 dB Average Ês1 / Noc ±0.3 dB Ês1 / Noc over meas PRBs ±0.3 dB | Ês0 / Noc is the SNR for the SSB#0 Ês1 / Noc is the SNR for the SSB#1 Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 5.6.3.2 EN-DC FR2 SSB-based L1-RSRP measurement in DRX | Same as 5.6.3.1 | Same as 5.6.3.1 |
| 5.6.3.3 EN-DC FR2 CSI-RS-based L1-RSRP measurement in non-DRX | Average Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Average Ês0 / Noc ±0.3 dB Ês0 / Noc over meas PRBs ±0.3 dB Average Ês1 / Noc ±0.3 dB Ês1 / Noc over meas PRBs ±0.3 dB | Ês0 / Noc is the SNR for the CSI-RS#0 Ês1 / Noc is the SNR for the CSI-RS#1 Meas PRB is the measurement PRB for CSI-RS-RSRP |
| 5.6.3.4 EN-DC FR2 CSI-RS-based L1-RSRP measurement in DRX | Same as 5.6.3.3 | Same as 5.6.3.3 |
| 5.6.4.1 EN-DC FR2 SRS-RSRP measurement in non-DRX | Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Ês1 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB | Noc is the AWGN on the NR freq Ês1 / Noc is the ratio of cell2 signal/AWGN |
| 5.6.4.2 EN-DC FR2 CLI-RSSI measurement in non-DRX | Same as 5.6.4.1 | Same as 5.6.4.1 |
| 5.6.6.1 EN-DC FR2 CSI-RS based CMR and no dedicated IMR L1-SINR measurement in DRX | Average Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Average Ês0 / Noc ±0.3 dB Ês0 / Noc over meas PRBs ±0.3 dB Average Ês1 / Noc ±0.3 dB Ês1 / Noc over meas PRBs ±0.3 dB | Ês0 / Noc is the SNR for the CSI-RS#0 Ês1 / Noc is the SNR for the CSI-RS#1 Meas PRB is the measurement PRB for CSI-RS-SINR #RBJ to RBJ+275 |
| 5.6.6.2 EN-DC FR2 SSB based CMR and dedicated IMR L1-SINR measurement in non-DRX | Average Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Average Ês0 / Noc ±0.3 dB Ês0 / Noc over meas PRBs ±0.3 dB Average Ês1 / Noc ±0.3 dB Ês1 / Noc over meas PRBs ±0.3 dB | Ês0 / Noc is the SNR for the SSB#0 Ês1 / Noc is the SNR for the SSB#1 Meas PRB is the measurement PRB for SS-SINR#RBJ to RBJ+19 |
| 5.6.6.3 EN-DC FR2 CSI-RS based CMR and dedicated IMR L1-SINR measurement in non-DRX | Same as 5.6.6.1 | Same as 5.6.6.1 |
| 5.7.1.1 EN-DC FR2 SS-RSRP measurement accuracy | Test 1 Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Ês1 / Noc ±0.3 dB Ês2 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB Meas PRB Ês2 / Noc ±0.3 dB Test 2 Average Ês1 ±5.65 dB, f <= 40.8 GHz Meas PRB Ês1 ±5.65 dB, f <= 40.8 GHz Average Ês2 ±5.65 dB, f <= 40.8 GHz Meas PRB Ês2 ±5.65 dB, f <= 40.8 GHz | Test 1 Noc is the AWGN on the NR freq Ês1 / Noc is the ratio of cell 2 signal / AWGN Ês2 / Noc is the ratio of cell 3 signal / AWGN Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 Test 2 Ês1 is NR cell 2 signal Ês2 is NR cell 3 signal Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 5.7.1.2 EN-DC FR2-FR2 SS-RSRP measurement accuracy | Noc1 ±5.65 dB, f <= 40.8 GHz Noc1 over measPRBs ±5.65 dB, f <= 40.8 GHz Noc2 ±5.65 dB, f <= 40.8 GHz Noc2 over measPRBs ±5.65 dB, f <= 40.8 GHz Ês1 / Noc1 ±0.3 dB Ês2 / Noc2 ±0.3 dB Meas PRB Ês1 / Noc1 ±0.3 dB Meas PRB Ês2 / Noc2 ±0.3 dB | Noc1 is the AWGN on the NR freq 1 Noc2 is the AWGN on the NR freq 2 Ês1 / Noc1 is the ratio of cell 2 signal / AWGN Ês2 / Noc2 is the ratio of cell 3 signal / AWGN Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 5.7.1.3 EN-DC FR1-FR2 SS-RSRP measurement accuracy | Test 1 Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Ês2 / Noc ±0.3 dB Meas PRB Ês2 / Noc ±0.3 dB Test 2 Average Ês2 ±5.65 dB, f <= 40.8 GHz Meas PRB Ês2 ±5.65 dB, f <= 40.8 GHz | Test 1 Noc is the AWGN on the NR freq Ês2 / Noc is the ratio of cell 3 signal / AWGN Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 Test 2 Ês2 is NR cell 3 signal Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 5.7.2.1 EN-DC FR2 SS-RSRQ measurement accuracy | Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB, f <= 40.8 GHz Ês1 / Noc ±0.3 dB Ês2 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB Meas PRB Ês2 / Noc ±0.3 dB | Noc is the AWGN on the NR freq Ês1 / Noc is the ratio of cell 2 signal / AWGN Ês2 / Noc is the ratio of cell 3 signal / AWGN Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 5.7.2.2 EN-DC FR2-FR2 SS-RSRQ measurement accuracy | Same as 5.7.1.2 | Same as 5.7.1.2 |
| 5.7.3.1 EN-DC FR2 SS-SINR measurement accuracy | Same as 5.7.2.1 | Same as 5.7.2.1 |
| 5.7.3.2 EN-DC FR2-FR2 SS-SINR measurement accuracy | Same as 5.7.1.2 | Same as 5.7.1.2 |
| 5.7.4.1 EN-DC FR2 SSB based L1-RSRP measurement accuracy | Test 1 Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Ês1 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB Test 2 Average Ês1 ±5.65 dB, f <= 40.8 GHz Meas PRB Ês1 ±5.65 dB, f <= 40.8 GHz | Test 1 Noc is the AWGN on the NR freq Ês1 / Noc is the ratio of cell 2 signal / AWGN Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 Test 2 Ês1 is NR cell 2 signal Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 5.7.4.2 EN-DC FR2 CSI-RS based L1-RSRP measurement accuracy | Test 1 Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Ês1 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB Test 2 Average Ês1 ±5.65 dB, f <= 40.8 GHz Meas PRB Ês1 ±5.65 dB, f <= 40.8 GHz | Test 1 Noc is the AWGN on the NR freq Ês1 / Noc is the ratio of cell 2 signal / AWGN Meas PRB is the measurement PRB for CSI-RS-RSRP Test 2 Ês1 is NR cell 2 signal Meas PRB is the measurement PRB for CSI-RS-RSRP |
| 5.7.5.1 EN-DC FR2 SRS-RSRP measurement accuracy | Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Ês1 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB | Noc is the AWGN on the NR freq Ês1 / Noc is the ratio of cell2 signal/AWGN |
| 5.7.5.2 EN-DC FR2 CLI-RSSI measurement accuracy | Same as 5.7.5.1 | Same as 5.7.5.1 |
| 5.7.6.1 EN-DC FR2 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR measurement accuracy | Test 1 Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Ês1 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB | Test 1 Noc is the AWGN on the NR freq Ês1 / Noc is the ratio of cell 2 signal / AWGN Meas PRB is the measurement PRB for CSI-RS-RSRP |
| 5.7.6.2 EN-DC FR2 SSB based CMR and dedicated IMR L1-SINR absolute measurement accuracy | Test 1 Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Ês1 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB | Test 1 Noc is the AWGN on the NR freq Ês1 / Noc is the ratio of cell 2 signal / AWGN Meas PRB is the measurement PRB for SSB or CSI-RS |
| 5.7.6.3 EN-DC FR2 CSI-RS based CMR and dedicated IMR L1-SINR measurement accuracy | Test 1 Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Ês1 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB | Test 1 Noc is the AWGN on the NR freq Ês1 / Noc is the ratio of cell 2 signal / AWGN Meas PRB is the measurement PRB for CSI-RS-RSRP |

Table F.1.1.2-6: Maximum test system uncertainty for RRM requirements for SA FR2 test cases

| Subclause | Maximum Test System Uncertainty1 | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 7.1.1.1 NR SA FR2 cell re-selection | Noc, averaged over BWConfig, ±5.65 dB Noc, averaged over Measurement PRB, ±5.65 dB Es1/Noc averaged over BWConfig ±0.3 dB Es1/Noc averaged over Measurement PRB ±0.3 dB Es2/Noc averaged over BWConfig ±0.3 dB Es2/Noc averaged over Measurement PRB ±0.3 dB | Noc averaged over BWConfig is the AWGN absolute power of RF channel 1 averaged over BWConfig Noc averaged over Measurement PRB is the AWGN absolute power of RF channel 1 averaged over Measurement PRB Es1/Noc averaged over BWConfig is Ratio of cell 1 signal / AWGN averaged over BWConfig Es1/Noc averaged over Measurement PRB is Ratio of cell 1 signal / AWGN averaged over Measurement PRB Es2/Noc averaged over BWConfig is Ratio of cell 2 signal / AWGN averaged over BWConfig Es2/Noc averaged over Measurement PRB is Ratio of cell 2 signal / AWGN averaged over Measurement PRB |
| 7.1.1.2 NR SA FR2-FR2 cell re-selection | Noc1, averaged over BWConfig, ±5.65 dB Noc1, averaged over Measurement PRB, ±5.65 dB Noc2, averaged over BWConfig, ±5.65 dB Noc2, averaged over Measurement PRB, ±5.65 dB Es1/Noc1 averaged over BWConfig ±0.3 dB Es1/Noc1 averaged over Measurement PRB ±0.3 dB Es2/Noc2 averaged over BWConfig ±0.3 dB Es2/Noc2 averaged over Measurement PRB ±0.3 dB | Noc1 averaged over BWConfig is the AWGN absolute power of RF channel 1 averaged over BWConfig Noc1 averaged over Measurement PRB is the AWGN absolute power of RF channel 1 averaged over Measurement PRB Noc2 averaged over BWConfig is the AWGN absolute power of RF channel 2 averaged over BWConfig Noc2 averaged over Measurement PRB is the AWGN absolute power of RF channel 2 averaged over Measurement PRB Es1/Noc1 averaged over BWConfig is Ratio of cell 1 signal / AWGN averaged over BWConfig Es1/Noc1 averaged over Measurement PRB is Ratio of cell 1 signal / AWGN averaged over Measurement PRB Es2/Noc2 averaged over BWConfig is Ratio of cell 2 signal / AWGN averaged over BWConfig Es2/Noc2 averaged over Measurement PRB is Ratio of cell 2 signal / AWGN averaged over Measurement PRB |
| 7.1.1.3 NR SA FR2 cell re-selection for UE fulfilling low mobility relaxed measurement criterion | Same as 7.1.1.1 | Same as 7.1.1.1 |
| 7.1.1.4 NR SA FR2 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion | Same as 7.1.1.1 | Same as 7.1.1.1 |
| 7.1.1.5 NR SA FR2-FR2 cell re-selection for UE fulfilling low mobility relaxed measurement criterion | Same as 7.1.1.2 | Same as 7.1.1.2 |
| 7.1.1.6 NR SA FR2-FR2 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion | Same as 7.1.1.2 | Same as 7.1.1.2 |
| 7.3.1.4 NR SA FR1-FR2 synchronous DAPS handover | Noc, averaged over BWConfig, ±5.65 dB Noc, averaged over Measurement PRB, ±5.65 dB Es2/Noc averaged over BWConfig ±0.3 dB Es2/Noc averaged over Measurement PRB ±0.3 dB | Noc averaged over BWConfig is the AWGN absolute power of RF channel 2 averaged over BWConfig Noc averaged over Measurement PRB is the AWGN absolute power of RF channel 2 averaged over Measurement PRB Es2/Noc averaged over BWConfig is Ratio of cell 2 signal / AWGN averaged over BWConfig Es2/Noc averaged over Measurement PRB is Ratio of cell 2 signal / AWGN averaged over Measurement PRB |
| 7.3.1.5 NR SA FR1-FR2 asynchronous DAPS handover | Same as 7.3.1.4 | Same as 7.3.1.4 |
| 7.3.2.1.1 NR SA FR2 RRC re-establishment | Average Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Average Ês1 / Noc ±0.3 dB Ês1 / Noc over meas PRBs ±0.3 dB Average Ês2 / Noc ±0.3 dB Ês2 / Noc over meas PRBs ±0.3 dB | Ês1 is NR cell 1 signal Ês2 is NR cell 2 signal Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 7.3.2.1.2 NR SA FR2 - FR2 RRC re-establishment | Average Noc1 ±5.65 dB, f <= 40.8 GHz Noc1 over meas PRBs ±5.65 dB f <= 40.8 GHz Average Noc2 ±5.65 dB, f <= 40.8 GHz Noc2 over meas PRBs ±5.65 dB f <= 40.8 GHz Average Ês1 / Noc1 ±0.3 dB Ês1 / Noc over meas PRBs ±0.3 dB Average Ês2 / Noc2 ±0.3 dB Ês2 / Noc over meas PRBs ±0.3 dB | Ês1 is NR cell 1 signal Ês2 is NR cell 2 signal Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 7.3.2.2.1 NR SA FR2 contention based random access | Average Ês ±6.0 dB Meas PRB Ês ±6.0 dB Uplink absolute power measurement ±6.0 dB Uplink relative power measurement ±FFS dB ±[48]Tc Uplink signal transmit timing relative to downlink | Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 Tc = 1/(480000 x 4096) seconds, the basic timing unit defined in TS 38.211 [7] |
| 7.3.2.2.2 NR SA FR2 non-contention based random access | Same as 7.3.2.2.1 | Same as 7.3.2.2.1 |
| 7.3.3.1 NR SA FR2 conditional handover | Noc averaged over BWConfig, ±5.65 dB Noc, averaged over Measurement PRB, ±5.65 dB Es1/Noc averaged over BWConfig ±0.3 dB Es1/Noc averaged over Measurement PRB ±0.3 dB Es2/Noc ±0.3 dB Noc, averaged over BWConfig, ±5.65 dB Es2/Noc averaged over Measurement PRB ±0.3 dB | Noc averaged over BWConfig,is the AWGN absolute power of RF channel 1 averaged over BWConfig, Noc averaged over Measurement PRB,is the AWGN absolute power of RF channel 1 averaged over Measurement PRB, Es1/Noc averaged over BWConfig is Ratio of cell 1 signal / AWGN averaged over BWConfig Es1/Noc averaged over Measurement PRB is Ratio of cell 1 signal / AWGN averaged over Measurement PRB Es2/Noc averaged over BWConfig is Ratio of cell 2 signal / AWGN averaged over BWConfig Es2/Noc averaged over Measurement PRB is Ratio of cell 2 signal / AWGN averaged over Measurement PRB |
| 7.4.1.1 SA FR2 UE transmit timing accuracy | Noc ±5.65 dB Ês1 / Noc ±0.3 dB ±48Tc Uplink signal transmit timing relative to downlink ±40Tc Relative during UE timing adjustment | Ês1 / Noc is the ratio of cell 1 signal / AWGN Tc = 1/(480000 x 4096) seconds, the basic timing unit defined in TS 38.211 [7] |
| 7.4.3.1 NR SA FR2 UE timing advance adjustment accuracy | Noc ±5.65 dB Ês1 / Noc ±0.3 dB ±40Tc timing advance adjustment | Ês1 / Noc is the ratio of cell 1 signal / AWGN Tc = 1/(480000 x 4096) seconds, the basic timing unit defined in TS 38.211 [7] |
| 7.5.1.1 Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode | Same as 5.5.1.1 | Same as 5.5.1.1 |
| 7.5.1.2 Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode | Same as 5.5.1.1 | Same as 5.5.1.1 |
| 7.5.1.3 Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode | Same as 5.5.1.1 | Same as 5.5.1.1 |
| 7.5.1.4 Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode | Same as 5.5.1.1 | Same as 5.5.1.1 |
| 7.5.5.1 NR SA FR2 SSB-based beam failure detection and link recovery in non-DRX | Same as 5.5.5.1 | Same as 5.5.5.1 |
| 7.5.5.2 NR SA FR2 SSB-based beam failure detection and link recovery in DRX | Same as 5.5.5.1 | Same as 5.5.5.1 |
| 7.5.5.3 NR SA FR2 CSI-RS-based beam failure detection and link recovery in non-DRX | Noc ±5.65 dB, f<=40.8 GHz Ês1 / Noc ±0.3 dB Fading profile ±0.7 dB for 2Tx | Note: Noc is the AWGN on cell 1 frequency Ês1 / Noc is the SNR for the CSI-RS |
| 7.5.5.4 NR SA FR2 CSI-RS-based beam failure detection and link recovery in DRX | Same as 7.5.5.3 | Same as 7.5.5.3 |
| 7.5.5.5 NR SA FR2 scheduling available restriction during SSB-based beam failure detection and link recovery in non-DRX | Same as 5.5.5.1 | Same as 5.5.5.1 |
| 7.5.5.6 NR SA FR2 SCell CSI-RS-based beam failure detection and link recovery in non-DRX | Noc ±5.65 dB, f<=40.8 GHz Ês2 / Noc ±0.3 dB Fading profile ±0.7 dB for 2Tx | Note: Noc is the AWGN on cell 2 frequency Ês2 / Noc is the SNR for the CSI-RS |
| 7.5.5.7 NR SA FR2 SCell CSI-RS-based beam failure detection and link recovery in DRX | Same as 7.5.5.6 | Same as 7.5.5.6 |
| 7.5.6.2.1 NR SA FR2 RRC-based DL active BWP switch in non-DRX | ±1.0 dB | Overall system uncertainty for fading conditions comprises two quantities: 1. gNB emulator Signal-to-noise ratio uncertainty 2. Effect of AWGN flatness and signal flatness Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (gNB emulator Signal-to-noise ratio uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) gNB emulator Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±3.6 dB |
| 7.6.1.1 NR SA FR2 event-triggered reporting without gap in non-DRX | Ês1 averaged over BW ±5.65 dB, f <= 40.8 GHz Ês1 averaged over measured PRB ±5.65 dB, f <= 40.8 GHz Ês2 averaged over BW ±5.65 dB, f <= 40.8 GHz Ês2 averaged over measured PRB ±5.65 dB, f <= 40.8 GHz | Ês1 averaged over BW is the absolute power of cell 1 signal averaged over BW Ês1 averaged over measured PRB is the absolute power of cell 1 signal averaged over measured PRB Ês2 averaged over BW is the absolute power of cell 2 signal averaged over BW Ês2 averaged over measured PRB is the absolute power of cell 2 signal averaged over measured PRB |
| 7.6.1.2 NR SA FR2 event-triggered reporting without gap in DRX | Noc averaged over BW, ±5.65 dB, f <= 40.8 GHz Noc averaged over measured PRB, ±5.65 dB, f <= 40.8 GHz Ês1 / Noc averaged over BW, ±0.3 dB, f <= 40.8 GHz Ês1 / Noc averaged over measured PRB, ±0.3 dB, f <= 40.8 GHz Ês2 / Noc averaged over BW, ±0.3 dB, f <= 40.8 GHz Ês2 / Noc averaged over measured PRB, ±0.3 dB, f <= 40.8 GHz | Noc averaged over BW is AWGN absolute power averaged over BW. Noc averaged over measured PRB is AWGN absolute power averaged over measured PRB. Ês1 / Noc averaged over BW is the ratio of cell 1 signal / AWGN averaged over BW Ês1 / Noc averaged over measured PRB is the ratio of cell 1 signal / AWGN averaged over measured PRB Ês2 / Noc averaged over BW is the ratio of cell 2 signal / AWGN averaged over BW. Ês2 / Noc averaged over measured PRB is the ratio of cell 2 signal / AWGN averaged over measured PRB. |
| 7.6.1.3 NR SA FR2 event-triggered reporting without gap in non-DRX | Same as 7.6.1.1 | Same as 7.6.1.1 |
| 7.6.1.4 NR SA FR2 event-triggered reporting with gap in DRX | Same as 7.6.1.2 | Same as 7.6.1.2 |
| 7.6.2.1 NR SA FR2-FR2 event-triggered reporting in non-DRX | Average Ês1 ±5.65 dB, f <= 40.8 GHz Meas PRB Ês1 ±5.65 dB, f <= 40.8 GHz Average Ês2 ±5.65 dB Meas PRB Ês2 ±5.65 dB | Ês1 is NR cell 1 signal Ês2 is NR cell 2 signal Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 7.6.2.2 NR SA FR2-FR2 event-triggered reporting in DRX | Average Noc1 ±5.65 dB, f <= 40.8 GHz Noc1 over meas PRBs ±5.65 dB f <= 40.8 GHz Average Noc2 ±5.65 dB, f <= 40.8 GHz Noc2 over meas PRBs ±5.65 dB f <= 40.8 GHz Average Ês1 / Noc1 ±0.3 dB Ês1 / Noc over meas PRBs ±0.3 dB Average Ês2 / Noc2 ±0.3 dB Ês2 / Noc2 over meas PRBs ±0.3 dB | Ês1 is NR cell 1 signal Ês2 is NR cell 2 signal Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 7.6.2.3 NR SA FR2-FR2 event-triggered reporting in non-DRX with SSB time index detection | Same as 7.6.2.1 | Same as 7.6.2.1 |
| 7.6.2.4 NR SA FR2-FR2 event-triggered reporting in DRX with SSB time index detection | Same as 7.6.2.2 | Same as 7.6.2.2 |
| 7.6.2.5 NR SA FR1-FR2 event-triggered reporting in non-DRX | Average Ês2 ±5.65 dB, f <= 40.8 GHz | Ês2 is NR cell 2 signal Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 7.6.2.6 NR SA FR1-FR2 event-triggered reporting in DRX | Noc ±5.65 dB, f <= 40.8 GHz Ês2 / Noc ±0.3 dB | Ês2 / Noc is the ratio of cell 2 signal / AWGN |
| 7.6.2.7 NR SA FR1-FR2 event-triggered reporting in non-DRX with SSB time index detection | Same as 7.6.2.5 | Same as 7.6.2.5 |
| 7.6.2.8 NR SA FR1-FR2 event-triggered reporting in DRX with SSB time index detection | Same as 7.6.2.6 | Same as 7.6.2.6 |
| 7.6.3.1 NR SA FR2 SSB-based L1-RSRP measurement in non-DRX | Average Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Average Ês0 / Noc ±0.3 dB Ês0 / Noc over meas PRBs ±0.3 dB Average Ês1 / Noc ±0.3 dB Ês1 / Noc over meas PRBs ±0.3 dB | Ês0 / Noc is the SNR for the SSB#0 Ês1 / Noc is the SNR for the SSB#1 Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 7.6.3.2 NR SA FR2 SSB-based L1-RSRP measurement in DRX | Same as 7.6.3.1 | Same as 7.6.3.1 |
| 7.6.3.3 NR SA FR2 CSI-RS-based L1-RSRP measurement in non-DRX | Average Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Average Ês0 / Noc ±0.3 dB Ês0 / Noc over meas PRBs ±0.3 dB Average Ês1 / Noc ±0.3 dB Ês1 / Noc over meas PRBs ±0.3 dB | Ês0 / Noc is the SNR for the CSI-RS#0 Ês1 / Noc is the SNR for the CSI-RS#1 Meas PRB is the measurement PRB for CSI-RS-RSRP |
| 7.6.3.4 NR SA FR2 CSI-RS-based L1-RSRP measurement in DRX | Same as 7.6.3.3 | Same as 7.6.3.3 |
| 7.6.4.1 NR SA FR2 SRS-RSRP measurement in non-DRX | Same as 5.6.4.1 | Same as 5.6.4.1 |
| 7.6.4.2 NR SA FR2 CLI-RSSI measurement in non-DRX | Same as 5.6.4.1 | Same as 5.6.4.1 |
| 7.6.6.1 NR SA FR2 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR measurement accuracy | Average Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Average Ês0 / Noc ±0.3 dB Ês0 / Noc over meas PRBs ±0.3 dB Average Ês1 / Noc ±0.3 dB Ês1 / Noc over meas PRBs ±0.3 dB | Ês0 / Noc is the SNR for the CSI-RS#0 Ês1 / Noc is the SNR for the CSI-RS#1 Meas PRB is the measurement PRB for CSI-RS-SINR #RBJ to RBJ+275 |
| 7.6.6.2 NR SA FR2 SSB based CMR and dedicated IMR L1-SINR measurement accuracy | Average Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Average Ês0 / Noc ±0.3 dB Ês0 / Noc over meas PRBs ±0.3 dB Average Ês1 / Noc ±0.3 dB Ês1 / Noc over meas PRBs ±0.3 dB | Ês0 / Noc is the SNR for the SSB#0 Ês1 / Noc is the SNR for the SSB#1 Meas PRB is the measurement PRB for SS-SINR #RBJ to RBJ+19 |
| 7.6.6.3 NR SA FR2 CSI-RS based CMR and dedicated IMR L1-SINR measurement accuracy | Same as 7.6.6.1 | Same as 7.6.6.1 |
| 7.7.1.1 NR SA FR2 SS-RSRP measurement accuracy | Test 1 Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Ês1 / Noc ±0.3 dB Ês2 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB Meas PRB Ês2 / Noc ±0.3 dB Test 2 Average Ês1 ±5.65 dB, f <= 40.8 GHz Meas PRB Ês1 ±5.65 dB, f <= 40.8 GHz Average Ês2 ±5.65 dB, f <= 40.8 GHz Meas PRB Ês2 ±5.65 dB, f <= 40.8 GHz | Test 1 Noc is the AWGN on the NR freq Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 1 signal / AWGN Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 Test 2 Ês1 is NR cell 1 signal Ês2 is NR cell 2 signal Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 7.7.1.2 NR SA FR2-FR2 SS-RSRP measurement accuracy | Noc1 ±5.65 dB, f <= 40.8 GHz Noc1 over measPRBs ±5.65 dB, f <= 40.8 GHz Noc2 ±5.65 dB, f <= 40.8 GHz Noc2 over measPRBs ±5.65 dB, f <= 40.8 GHz Ês1 / Noc1 ±0.3 dB Ês2 / Noc2 ±0.3 dB Meas PRB Ês1 / Noc1 ±0.3 dB Meas PRB Ês2 / Noc2 ±0.3 dB | Noc1 is the AWGN on the NR freq 1 Noc2 is the AWGN on the NR freq 2 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 7.7.1.3 NR SA FR1-FR2 SS-RSRP measurement accuracy | Test 1 Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Ês2 / Noc ±0.3 dB Meas PRB Ês2 / Noc ±0.3 dB Test 2 Average Ês2 ±5.65 dB, f <= 40.8 GHz Meas PRB Ês2 ±5.65 dB, f <= 40.8 GHz | Test 1 Noc is the AWGN on the NR freq Ês2 / Noc is the ratio of cell 1 signal / AWGN Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 Test 2 Ês2 is NR cell 2 signal Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 7.7.2.1 NR SA FR2 SS-RSRQ measurement accuracy | Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB, f <= 40.8 GHz Ês1 / Noc ±0.3 dB Ês2 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB Meas PRB Ês2 / Noc ±0.3 dB | Noc is the AWGN on the NR freq Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 7.7.2.2 NR SA FR2-FR2 SS-RSRQ measurement accuracy | Same as 7.7.1.2 | Same as 7.7.1.2 |
| 7.7.3.1 NR SA FR2 SS-SINR measurement accuracy | Same as 7.7.2.1 | Same as 7.7.2.1 |
| 7.7.3.3 NR SA FR2-FR2 SS-SINR measurement accuracy | Same as 7.7.1.2 | Same as 7.7.1.2 |
| 7.7.4.1 NR SA FR2 SSB based L1-RSRP measurement accuracy | Test 1 Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Ês1 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB Test 2 Average Ês1 ±5.65 dB, f <= 40.8 GHz Meas PRB Ês1 ±5.65 dB, f <= 40.8 GHz | Test 1 Noc is the AWGN on the NR freq Ês1 / Noc is the ratio of cell 1 signal / AWGN Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 Test 2 Ês1 is NR cell 1 signal Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 7.7.4.2 NR SA FR2 CSI-RS based L1-RSRP measurement accuracy | Test 1 Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Ês1 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB Test 2 Average Ês1 ±5.65 dB, f <= 40.8 GHz Meas PRB Ês1 ±5.65 dB, f <= 40.8 GHz | Test 1 Noc is the AWGN on the NR freq Ês1 / Noc is the ratio of cell 1 signal / AWGN Meas PRB is the measurement PRB for CSI-RS-RSRP Test 2 Ês1 is NR cell 1 signal Meas PRB is the measurement PRB for CSI-RS-RSRP |
| 7.7.5.1 NR SA FR2 SRS-RSRP measurement accuracy | Same as 5.7.5.1 | Same as 5.7.5.1 |
| 7.7.5.2 NR SA FR2 CLI-RSSI measurement accuracy | Same as 5.7.5.1 | Same as 5.7.5.1 |
| 7.7.6.1 NR SA FR2 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR measurement accuracy | Same as 7.6.6.1 | Same as 7.6.6.1 |
| 7.7.6.2 NR SA FR2 SSB based CMR and dedicated IMR L1-SINR measurement accuracy | Same as 7.6.6.2 | Same as 7.6.6.2 |
| 7.7.6.3 NR SA FR2 CSI-RS based CMR and dedicated IMR L1-SINR measurement accuracy | Same as 7.6.6.1 | Same as 7.6.6.1 |

### F.1.3.2	Measurement of RRM requirements

Because the relationships between the test system uncertainties and the test tolerances are often complex, it is not always possible to give a simple derivation of the test requirement in this document. The analysis is recorded in 3GPP TR 38 903 [22].

The derivation of the test requirements for the EN-DC FR1 test cases in chapter 4 is defined in Table F.1.3.2-1.

The derivation of the test requirements for the NR/5GC FR1 test cases in chapter 6 is defined in Table F.1.3.2-2.

The derivation of the test requirements for the EN-DC FR2 test cases in chapter 5 is defined in Table F.1.3.2-3.

The derivation of the test requirements for the NR/5GC FR2 test cases in chapter 7 is defined in Table F.1.3.2-4.

The derivation of the test requirements for the E-UTRA – NR inter-RAT with E-UTRA serving cell test cases in chapter 8 is defined in Table F.1.3.2-5.

The derivation of the test requirements for the FR1 NR sidelink test cases in chapter 9 is defined in Table F.1.3.2-6.

The derivation of the test requirements for the NR/5GC FR1 test cases for RedCap in chapter 16 is defined in Table F.1.3.2-7.

The derivation of the test requirements for the NR/5GC FR2 test cases for RedCap in chapter 17 is defined in Table F.1.3.2-8.

The derivation of the test requirements for the E-UTRA – NR inter-RAT with E-UTRA serving cell test cases for RedCap in chapter 18 is defined in Table F.1.3.2-9.

Table F.1.3.2-1: Derivation of test requirements for EN-DC FR1 RRM tests

| Test |  | Minimum requirement in TS 38.133 [6] |  | Test tolerance (TT) |  | Test requirement in TS 38.533 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4.3.2.2.1 |  | Absolute uplink power: Normal conditions ±9dB Relative uplink power step: Normal conditions ±2.5dB Uplink timing: 15kHz SCS Te ±12*64*Tc 30kHz SCS Te ±8*64*Tc |  | 2.1dB 0.7dB 112Tc 112Tc |  | Absolute uplink power: Normal conditions ±11.1dB Relative uplink power step: Normal conditions ±3.2dB Uplink timing: 15kHz SCS Te ±880*Tc 30kHz SCS Te ±624*Tc |  |  |
| 4.3.2.2.2 |  | Same as 4.3.2.2.1 |  | Same as 4.3.2.2.1 |  | Same as 4.3.2.2.1 |  |  |
| 4.3.2.2.3 |  | Same as 4.3.2.2.1 |  | Same as 4.3.2.2.1 |  | Same as 4.3.2.2.1 |  |  |
| 4.3.2.2.4 |  | Same as 4.3.2.2.1 |  | Same as 4.3.2.2.1 |  | Same as 4.3.2.2.1 |  |  |
| 4.4.1.1 EN-DC FR1 UE transmit timing accuracy |  | Test 1 (no DRX): Uplink timing: ±12*64 Tc for 15 KHz SSB SCS,15 kHz UL SCS ±10*64 Tc for 15 KHz SSB SCS,30 kHz UL SCS ±10*64 Tc for 15 KHz SSB SCS,60 kHz UL SCS ±8*64 Tc for 30 KHz SSB SCS,15 kHz UL SCS ±8*64 Tc for 30 KHz SSB SCS,30 kHz UL SCS ±7*64 Tc for 30 KHz SSB SCS,60 kHz UL SCS Max step size Tq: 5.5*64*Tc Min adjust rate Tp: 5.5*64*Tc Max adjust rate: 5.5*64*Tc Ês2 / Noc: +3.00dB Noc = -98 dBm/15 kHz (Config 1,2,3) Test 2 (with DRX): ±12*64 Tc for 15 KHz SSB SCS,15 kHz UL SCS ±10*64 Tc for 15 KHz SSB SCS,30 kHz UL SCS ±10*64 Tc for 15 KHz SSB SCS,60 kHz UL SCS ±8*64 Tc for 30 KHz SSB SCS,15 kHz UL SCS ±8*64 Tc for 30 KHz SSB SCS,30 kHz UL SCS ±7*64 Tc for 30 KHz SSB SCS,60 kHz UL SCS Ês2 / Noc: +3.00dB |  | ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc +0.5*64Tc -3.6*64*Tc +1.1*64*Tc +0.3 dB +1.5 dB ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc +0.3dB |  | Test 1 (10MHz Ch BW): Uplink timing: ±13.75*64*Tc Uplink timing: ±11.75*64*Tc Uplink timing: ±11.75*64*Tc Uplink timing: ±9.75*64*Tc Uplink timing: ±9.75*64*Tc Uplink timing: ±8.75*64*Tc Max step size Tq: 6.0*64*Tc Min adjust rate: 1.9*64*Tc Max adjust rate: 6.6*64*Tc Ês2 / Noc: +3.30dB Noc = -98 dBm/15 kHz (Config 1,2,3) +1.5 dB Test 2 (with DRX): Uplink timing: ±13.75*64*Tc Uplink timing: ±11.75*64*Tc Uplink timing: ±11.75*64*Tc Uplink timing: ±9.75*64*Tc Uplink timing: ±9.75*64*Tc Uplink timing: ±8.75*64*Tc Ês2 / Noc: +3.30dB |  |  |
| 4.4.3.1 EN-DC FR1 timing advance adjustment accuracy |  | Noc = -98 dBm/15 kHz (Config 1, 2, 4, 5) Noc = -95 dBm/15 kHz (Config 3, 6) Êsx / Noc = 3 dB UE Timing Advance Adjustment Accuracy for 15kHz SCS = ±256 Tc+ TT UE Timing Advance Adjustment Accuracy for 30kHz SCS = ±256 Tc+ TT |  | 0 0 0 +/- 88 Tc +/- 88 Tc |  | Noc = -98 dBm/15 kHz (Config 1, 2, 4, 5) Noc = -95 dBm/15 kHz (Config 3, 6) Êsx / Noc = 3 dB UE TAAA for 15kHz SCS = ±344 Tc UE TAAA for 30kHz SCS = ±344 Tc |  |  |
| 4.5.1.1 EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode |  | SNR during T1: 1dB T2: -7dB T3: -15dB |  | Offset during T1: +0.8dB T2: +0.8dB T3: -0.8dB |  | SNR during T1: 1.8dB T2: -6.2dB T3: -15.8dB |  |  |
| 4.5.1.2 EN-DC FR1 radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode |  | SNR during T1: 1dB T2: -7dB T3: -15dB T4: -4.5dB T5: 1dB |  | Offset during T1: +0.8dB T2: +0.8dB T3: -0.8dB T4: -0.8dB T5: +0.8dB |  | SNR during T1: 1.8dB T2: -6.2dB T3: -15.8dB T4: -5.3dB T5: 1.8dB For testing of a UE which supports 4RX on all bands, the SNR during T3 and T4 is modified as specified in clause D.4.1.1 |  |  |
| 4.5.1.3 EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in DRX mode |  | Same as 4.5.1.1 |  | Same as 4.5.1.1 |  | Same as 4.5.1.1 |  |  |
| 4.5.1.4 EN-DC FR1 radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in DRX mode |  | Same as 4.5.1.2 |  | Same as 4.5.1.2 |  | Same as 4.5.1.2 |  |  |
| 4.5.1.5 EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode |  | Same as 4.5.1.1 |  | Same as 4.5.1.1 |  | Same as 4.5.1.1 |  |  |
| 4.5.1.6 EN-DC FR1 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode |  | Same as 4.5.1.2 |  | Same as 4.5.1.2 |  | Same as 4.5.1.2 |  |  |
| 4.5.1.7 EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in DRX mode |  | Same as 4.5.1.1 |  | Same as 4.5.1.1 |  | Same as 4.5.1.1 |  |  |
| 4.5.1.8 EN-DC FR1 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM RS in DRX mode |  | Same as 4.5.1.2 |  | Same as 4.5.1.2 |  | Same as 4.5.1.2 |  |  |
| 4.5.2.1 EN-DC FR1 interruptions at transitions between active and non-active during DRX in synchronous EN-DC |  | SNRs as specified |  | 0.6dB |  | Formula: SNR + TT |  |  |
| 4.5.2.2 EN-DC FR1 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC |  | Same as 4.5.2.1 |  | Same as 4.5.2.1 |  | Same as 4.5.2.1 |  |  |
| 4.5.2.3 EN-DC FR1 interruptions during measurements on deactivated NR SCC in synchronous EN-DC |  | During T1: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB |  | During T1: 0dB 0dB 0dB 0dB |  | During T1: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB |  |  |
| 4.5.2.4 EN-DC FR1 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC |  | Same as 4.5.2.3 |  | Same as 4.5.2.3 |  | Same as 4.5.2.3 |  |  |
| 4.5.2.5 EN-DC FR1 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC |  | During T1: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB |  | During T1: 0dB 0dB 0dB 0dB |  | During T1: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB |  |  |
| 4.5.2.6 EN-DC FR1 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC |  | Same as 4.5.2.5 |  | Same as 4.5.2.5 |  | Same as 4.5.2.5 |  |  |
| 4.5.3.1 EN-DC FR1 SCell activation and deactivation of known SCell in non-DRX for 160ms SCell measurement cycle |  | During T1: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB During T2: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB During T3: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB |  | During T1: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB During T2: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB During T3: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB |  |  |
| 4.5.3.2 EN-DC FR1 SCell activation and deactivation of known SCell in non-DRX for 640ms SCell measurement cycle |  | Same as 4.5.3.1 |  | Same as 4.5.3.1 |  | Same as 4.5.3.1 |  |  |
| 4.5.3.3 EN-DC FR1 SCell activation and deactivation of unknown SCell in non-DRX |  | Same as 4.5.3.1 |  | Same as 4.5.3.1 |  | Same as 4.5.3.1 |  |  |
| 4.5.4.1 EN-DC FR1 UE UL carrier RRC reconfiguration delay |  | During T1: Noc2: -102dBm/15kHz Noc3: -102dBm/15kHz Ês2 / Noc2: +16dB Ês3 / Noc3: +16dB During T2: Noc2: -102dBm/15kHz Noc3: -102dBm/15kHz Ês2 / Noc2: +16dB Ês3 / Noc3: +16dB During T3: Noc2: -102dBm/15kHz Noc3: -102dBm/15kHz Ês2 / Noc2: +16dB Ês3 / Noc3: +16dB |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB |  | During T1: Noc2: -102dBm/15kHz Noc3: -102dBm/15kHz Ês2 / Noc2: +16dB Ês3 / Noc3: +16dB During T2: Noc2: -102dBm/15kHz Noc3: -102dBm/15kHz Ês2 / Noc2: +16dB Ês3 / Noc3: +16dB During T3: Noc2: -102dBm/15kHz Noc3: -102dBm/15kHz Ês2 / Noc2: +16dB Ês3 / Noc3: +16dB |  |  |
| 4.5.5.1 EN-DC FR1 SSB-based beam failure detection and link recovery in non-DRX |  | q0 SSB SNR during: T1: 5dB T2: -3dB T3: -12dB T4: -12dB T5: -12dB q1 SSB during T1: Noc2: -98dBm/15kHz Ês2 / Noc2: -10dB q1 SSB during T2: Noc2: -98dBm/15kHz Ês2 / Noc2: -10dB q1 SSB during T3, T4 and T5: Noc2: -98dBm/15kHz Ês2 / Noc2: +10dB |  | Offset during: T1: +0.8dB T2: +0.8dB T3: -0.8dB T4: -0.8dB T5: -0.8dB q1 SSB during T1: 0dB -0.2dB q1 SSB during T2: 0dB -0.2dB q1 SSB during T3, T4 and T5: 0dB +0.2dB |  | SNR during: T1: +5.8dB T2: -2.2dB T3: -12.8dB T4: -12.8dB T5: -12.8dB For testing of a UE which supports 4RX on all bands, the SNR during T3, T4 and T5 is modified as specified in clause D.4.1.1 q1 SSB during T1: Noc2: -98dBm/15kHz Ês2 / Noc2: -10.2dB q1 SSB during T2: Noc2: -98dBm/15kHz Ês2 / Noc2: -10.2dB q1 SSB during T3, T4 and T5: Noc2: -98dBm/15kHz Ês2 / Noc2: +10.2dB |  |  |
| 4.5.5.2 EN-DC FR1 SSB-based beam failure detection and link recovery in DRX |  | Same as 4.5.5.1 |  | Same as 4.5.5.1 |  | Same as 4.5.5.1 |  |  |
| 4.5.5.3 EN-DC FR1 CSI-RS-based beam failure detection and link recovery in non-DRX |  | q0 CSI-RS SNR during: T1: 5dB T2: -3dB T3: -12dB T4: -12dB T5: -12dB q1 CSI-RS during T1: Noc2: -98dBm/15kHz Ês2 / Noc2: -10dB q1 CSI-RS during T2: Noc2: -98dBm/15kHz Ês2 / Noc2: -10dB q1 CSI-RS during T3, T4 and T5: Noc2: -98dBm/15kHz Ês2 / Noc2: +10dB |  | Offset during: T1: +0.8dB T2: +0.8dB T3: -0.8dB T4: -0.8dB T5: -0.8dB q1 CSI-RS during T1: 0dB -0.2dB q1 CSI-RS during T2: 0dB -0.2dB q1 CSI-RS during T3, T4 and T5: 0dB +0.2dB |  | SNR during: T1: +5.8dB T2: -2.2dB T3: -12.8dB T4: -12.8dB T5: -12.8dB For testing of a UE which supports 4RX on all bands, the SNR during T3, T4 and T5 is modified as specified in clause D.4.1.1 q1 CSI-RS during T1: Noc2: -98dBm/15kHz Ês2 / Noc2: -10.2dB q1 CSI-RS during T2: Noc2: -98dBm/15kHz Ês2 / Noc2: -10.2dB q1 CSI-RS during T3, T4 and T5: Noc2: -98dBm/15kHz Ês2 / Noc2: +10.2dB |  |  |
| 4.5.5.4 EN-DC FR1 CSI-RS-based beam failure detection and link recovery in DRX |  | Same as 4.5.5.3 |  | Same as 4.5.5.3 |  | Same as 4.5.5.3 |  |  |
| 4.5.5.5 EN-DC FR1 Scell CSI-RS-based beam failure detection and SSB-based link recovery in non-DRX |  | q0 SSB SNR during: T1: 5dB T2: -3dB T3: -12dB T4: -12dB T5: -12dB q1 SSB during T1: Noc2: -98dBm/15kHz Ês2 / Noc2: -10dB q1 SSB during T2: Noc2: -98dBm/15kHz Ês2 / Noc2: -10dB q1 SSB during T3, T4 and T5: Noc2: -98dBm/15kHz Ês2 / Noc2: +10dB q1 CSI-RS during T1: Noc2: -98dBm/15kHz Ês2 / Noc2: -10dB q1 CSI-RS during T2: Noc2: -98dBm/15kHz Ês2 / Noc2: -10dB q1 CSI-RS during T3, T4 and T5: Noc2: -98dBm/15kHz Ês2 / Noc2: +10dB |  | Offset during: T1: +0.8dB T2: +0.8dB T3: -0.8dB T4: -0.8dB T5: -0.8dB q1 SSB during T1: 0dB -0.2dB q1 SSB during T2: 0dB -0.2dB q1 SSB during T3, T4 and T5: 0dB +0.2dB q1 CSI-RS during T1: 0dB -0.2dB q1 CSI-RS during T2: 0dB -0.2dB q1 CSI-RS during T3, T4 and T5: 0dB +0.2dB |  | SNR during: T1: +5.8dB T2: -2.2dB T3: -12.8dB T4: -12.8dB T5: -12.8dB For testing of a UE which supports 4RX on all bands, the SNR during T3, T4 and T5 is modified as specified in section D.4.1.1 q1 SSB during T1: Noc2: -98dBm/15kHz Ês2 / Noc2: -10.2dB q1 SSB during T2: Noc2: -98dBm/15kHz Ês2 / Noc2: -10.2dB q1 SSB during T3, T4 and T5: Noc2: -98dBm/15kHz Ês2 / Noc2: +10.2dB q1 CSI-RS during T1: Noc2: -98dBm/15kHz Ês2 / Noc2: -10.2dB q1 CSI-RS during T2: Noc2: -98dBm/15kHz Ês2 / Noc2: -10.2dB q1 CSI-RS during T3, T4 and T5: Noc2: -98dBm/15kHz Ês2 / Noc2: +10.2dB |  |  |
| 4.5.5.6 EN-DC FR1 Scell CSI-RS-based beam failure detection and SSB-based link recovery in DRX |  | Same as 4.5.5.5 |  | Same as 4.5.5.5 |  | Same as 4.5.5.5 |  |  |
| 4.5.6.1.1 EN-DC FR1 DCI-based DL active BWP switch in non-DRX in synchronous EN-DC |  | During T1: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB During T2: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB During T3: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB |  | During T1: 0dB 0dB During T2: 0dB 0dB During T3: 0dB 0dB |  | During T1: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB During T2: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB During T3: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB |  |  |
| 4.5.6.1.2 EN-DC FR1 DCI-based DL active BWP switch with SCell in non-DRX in synchronous EN-DC |  | During T1: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB Noc3: -104dBm/15kHz Ês3 / Noc3: +17dB During T2: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB Noc3: -104dBm/15kHz Ês3 / Noc3: +17dB During T3: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB Noc3: -104dBm/15kHz Ês3 / Noc3: +17dB |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB |  | During T1: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB Noc3: -104dBm/15kHz Ês3 / Noc3: +17dB During T2: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB Noc3: -104dBm/15kHz Ês3 / Noc3: +17dB During T3: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB Noc3: -104dBm/15kHz Ês3 / Noc3: +17dB |  |  |
| 4.5.6.2.1 EN-DC FR1 RRC-based DL active BWP switch in non-DRX in synchronous EN-DC |  | During T1: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB |  | During T1: 0dB 0dB |  | During T1: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB |  |  |
| 4.5.7.1 EN-DC FR1 addition and release delay of known PSCell |  |  | For Test Configurations 1,2,4,5 Noc: -88dBm/15kHz Ês / Noc: 0.0dB B1-Threshold: -96 dBm/SCS For Test Configurations 3,6 Noc: -88dBm/15kHz Ês / Noc: 0.0dB B1-Threshold: -93 dBm/SCS |  | -0.6dB 0dB -3dB -0.6dB 0dB -3dB |  |  | Noc: -88.6dBm/15kHz Ês / Noc: 0.0dB B1-Threshold: -99 dBm/SCS Noc: -88.6dBm/15kHz Ês / Noc: 0.0dB B1-Threshold: -96 dBm/SCS |  |  |
| 4.5.8.1 EN-DC FR1 interruptions at switching between two uplink carriers |  |  | Noc: -104dBm/15kHz Ês / Noc: 17dB |  | 0dB 0dB |  |  | Noc: -104dBm/15kHz Ês / Noc: 17dB |  |  |
| 4.6.1.1 EN-DC FR1 event-triggered reporting without gap in non-DRX |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +4.00dB |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +4.00dB |  |  |
| 4.6.1.2 EN-DC FR1 event-triggered reporting without gap in DRX |  | Same as 4.6.1.1 |  | Same as 4.6.1.1 |  | Same as 4.6.1.1 |  |  |
| 4.6.1.3 EN-DC FR1 event-triggered reporting with gap in non-DRX |  | Same as 4.6.1.1 |  | Same as 4.6.1.1 |  | Same as 4.6.1.1 |  |  |
| 4.6.1.4 EN-DC FR1 event-triggered reporting with gap in DRX |  | Same as 4.6.1.1 |  | Same as 4.6.1.1 |  | Same as 4.6.1.1 |  |  |
| 4.6.1.5 EN-DC FR1 event-triggered reporting without gap in non-DRX with SSB time index detection |  | Same as 4.6.1.1 |  | Same as 4.6.1.1 |  | Same as 4.6.1.1 |  |  |
| 4.6.1.6 EN-DC FR1 event-triggered reporting with gap in non-DRX with SSB time index detection |  | Same as 4.6.1.1 |  | Same as 4.6.1.1 |  | Same as 4.6.1.1 |  |  |
| 4.6.1.7 EN-DC FR1 event-triggered reporting without gap in DRX for UE configured with highSpeedMeasFlag-r16 |  | Same as 4.6.1.1 |  | Same as 4.6.1.1 |  | Same as 4.6.1.1 |  |  |
| 4.6.2.1 EN-DC FR1-FR1 event-triggered reporting in non-DRX |  | During T1: Freq 2 Noc: -98dBm/15kHz Freq 3 Noc: -98dBm/15kHz Ês2 / Noc: +4.00dB Ês3 / Noc: -infinity During T2: Freq 2 Noc: -98dBm/15kHz Freq 3 Noc: -98dBm/15kHz Ês2 / Noc: +4.00dB Ês3 / Noc: +7.00dB |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB |  | During T1: Freq 2 Noc: -98dBm/15kHz Freq 3 Noc: -98dBm/15kHz Ês2 / Noc: +4.00dB Ês3 / Noc: -infinity During T2: Freq 2 Noc: -98dBm/15kHz Freq 3 Noc: -98dBm/15kHz Ês2 / Noc: +4.00dB Ês3 / Noc: +7.00dB |  |  |
| 4.6.2.2 EN-DC FR1-FR1 event-triggered reporting in DRX |  | Same as 4.6.2.1 |  | Same as 4.6.2.1 |  | Same as 4.6.2.1 |  |  |
| 4.6.2.5 EN-DC FR1-FR1 event-triggered reporting in non-DRX with SSB time index detection |  | Same as 4.6.2.1 |  | Same as 4.6.2.1 |  | Same as 4.6.2.1 |  |  |
| 4.6.2.6 EN-DC FR1-FR1 event-triggered reporting in DRX with SSB time index detection |  | Same as 4.6.2.1 |  | Same as 4.6.2.1 |  | Same as 4.6.2.1 |  |  |
| 4.6.4.1 EN-DC FR1 SSB-based L1-RSRP measurement in non-DRX |  | During T1: Noc: -94.65dBm/15kHz Ês0 / Noc: +0.00dB Ês1 / Noc: -infinity During T2: Noc: -94.65dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.00dB Reported SSB#0 SS-RSRP values ± 10dB Reported SSB#1 SS-RSRP values ± 10dB Reported Differential SS-RSRP values ± 3dB |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0.5dB Via mapping |  | During T1: Noc: -94.65dBm/15kHz Ês0 / Noc: +0.00dB Ês1 / Noc: -infinity During T2: Noc: -94.65dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.50dB For configuration 1,2,4,5 SSB#1: RSRP_55 to RSRP_75 SSB#0: DIFFRSRP_0 to DIFFRSRP_3 For configuration 3,6 SSB#1: RSRP_58 to RSRP_78 SSB#0: DIFFRSRP_0 to DIFFRSRP_3 |  |  |
| 4.6.4.2 EN-DC FR1 SSB-based L1-RSRP measurement in DRX |  | Same as 4.6.4.1 |  | Same as 4.6.4.1 |  | Same as 4.6.4.1 |  |  |
| 4.6.4.3 EN-DC FR1 CSI-RS-based L1-RSRP measurement in non-DRX |  | Noc: -94.65dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.00dB Reported CSI-RSRP values ± 10dB Reported Differential CSI-RSRP values ± 3dB |  | 0dB 0dB 0.5dB Via mapping |  | Noc: -94.65dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.50dB For configuration 1,2,4,5 CSI-RS#1: RSRP_55 to RSRP_75 CSI-RS#0: DIFFRSRP_0 to DIFFRSRP_3 For configuration 3,6 CSI-RS#1: RSRP_58 to RSRP_78 CSI-RS#0: DIFFRSRP_0 to DIFFRSRP_3 |  |  |
| 4.6.4.4 EN-DC FR1 CSI-RS-based L1-RSRP measurement in DRX |  | Same as 4.6.4.3 |  | Same as 4.6.4.3 |  | Same as 4.6.4.3 |  |  |
| 4.6.4.5 EN-DC FR1 SSB-based L1-RSRP measurement in DRX for UE configured with highSpeedMeasFlag-r16 |  | Same as 4.6.4.1 |  | Same as 4.6.4.1 |  | Same as 4.6.4.1 |  |  |
| 4.6.5.2 EN-DC FR1 CLI-RSSI measurement with non-DRX |  |  | During T1: Noc on CLI RSSI: -116 dBm/15kHz Io on CLI RSSI : -97.43 dBm/ 1.08 MHz During T2: Noc on CLI RSSI: -108dBm/15kHz Io on CLI RSSI : -89.43 dBm/ 1.08 Mhz |  | During T1: 0dB 0dB During T2: 0dB 0.5dB |  |  | During T1: Noc: -116 dBm/15kHz Io on CLI RSSI : -97.43 dBm/ 1.08 MHz During T2: Noc: -116 dBm/15kHz Io on CLI RSSI : -88.93 dBm/ 1.08 MHz |  |  |
| 4.6.7.1 EN-DC FR1 CSI-RS based CMR and no dedicated IMR L1-SINR measurement in non-DRX |  |  | Noc: -94.65dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.00dB Reported CSI-SINR values ± 5.5dB Reported Differential CSI-SINR values ± 4.5dB |  | 0dB 0dB 0dB Via mapping |  |  | Noc: -94.65dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.00dB For configuration 1,2,3,4,5,6 CSI-RS#1: SINR_41 to SINR_64 CSI-RS#0: DIFFSINR_0 to DIFFSINR_7 |  |  |
| 4.6.7.2 EN-DC FR1 SSB based CMR and dedicated IMR L1-SINR measurement in DRX |  |  | Noc: -94.65dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.00dB Reported SS-SINR values ±4.5dB Reported Differential SS-SINR values ±3.5dB |  | 0dB 0dB 0dB Via mapping |  |  | Noc: -94.65dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.00dB For configuration 1,2,3,4,5,6 SSB#1: SINR_43 to SINR_62 SSB#0: DIFFSINR_0 to DIFFSINR_6 |  |  |
| 4.6.7.3 EN-DC FR1 CSI-RS based CMR and dedicated IMR L1-SINR measurement in DRX |  |  | Noc: -94.65dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.00dB Reported CSI-SINR values ±4dB Reported Differential CSI-SINR values ±3dB |  | 0dB 0.5dB 0dB Via mapping |  |  | Noc: -94.65dBm/15kHz Ês0 / Noc: 0.50dB Ês1 / Noc: +3.00dB For configuration 1,2,3,4,5,6 CSI-RS#1: SINR_44 to SINR_61 CSI-RS#0: DIFFSINR_0 to DIFFSINR_5 |  |  |
| 4.7.1.1.1 EN-DC FR1 SS-RSRP absolute measurement accuracy |  | TEST CONFIGURATION 1, 2, 4, 5 |  |  |  |  |  |  |
|  |  | Test 1: Noc: -106dBm/15kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +1.0dB Reported RSRP values: ±4.5dB Test 2: Noc: -88dBm/15kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +1.0dB Reported RSRP values: ±8dB Test 3: Noc: -116dBm/15kHz + ΔBG_offset Ês2 / Noc: +3.0dB Ês3 / Noc: -1.0dB Reported RSRP values: ±6dB |  | Test 1: -1.5dB 0dB +0.4dB Via mapping Test 2: 0dB 0dB +0.4dB Via mapping Test 3: 0dB 0dB +0.2dB Via mapping |  | Test 1: Noc: -107.5dBm/15kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +1.4dB RSRP_44 to RSRP_56 Test 2: Noc: -88dBm/15kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +1.4dB RSRP_60 to RSRP_79 Test 3: Noc: -116dBm/15kHz + ΔBG_offset Ês2 / Noc: +3.0dB Ês3 / Noc: -0.8dB RSRP_34 to RSRP_46 RSRP_34 to RSRP_46 RSRP_35 to RSRP_47 RSRP_35 to RSRP_47 RSRP_36 to RSRP_48 RSRP_37 to RSRP_49 RSRP_37 to RSRP_49 depending on operating band |  |  |
|  |  | TEST CONFIGURATION 3, 6 |  |  |  |  |  |  |
|  |  | Test 1: Noc: -110dBm/30kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +1.0dB Reported RSRP values: ±4.5dB Test 2: Noc: -91dBm/30kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +1.0dB Reported RSRP values: ±8dB Test 3: Noc: -113dBm/30kHz + ΔBG_offset Ês2 / Noc: +3.0dB Ês3 / Noc: -1.0dB Reported RSRP values: ±6dB |  | Test 1: -0.8dB 0dB +0.4dB Via mapping Test 2: 0dB 0dB +0.4dB Via mapping Test 3: 0dB 0dB +0.2dB Via mapping |  | Test 1: Noc: -110.8dBm/30kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +1.4dB RSRP_41 to RSRP_53 Test 2: Noc: -91dBm/30kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +1.4dB RSRP_57 to RSRP_76 Test 3: Noc: -113dBm/30kHz + ΔBG_offset Ês2 / Noc: +3.0dB Ês3 / Noc: -0.8dB RSRP_37 to RSRP_49 RSRP_37 to RSRP_49 RSRP_38 to RSRP_50 RSRP_38 to RSRP_50 RSRP_39 to RSRP_51 RSRP_40 to RSRP_52 RSRP_40 to RSRP_52 depending on operating band |  |  |
| The derivation of the RSRP values takes into account the uncertainty in Cell 3 SS-RSRP from Noc and Ês3 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The SS-RSRP values given above are for normal conditions. In all cases the RSRP values are 4.5dB wider at each end for extreme conditions. |  |  |  |  |  |  |  |  |
| 4.7.1.1.2 EN-DC FR1 SS-RSRP relative measurement accuracy |  | Test 1: Noc: Test configuration 1, 2, 4, 5:-106dBm/15kHz Test configuration 3, 6:-110dBm/30kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +1.0dB Reported relative SS-RSRP values: ±3dB Test 2: Noc: -88dBm/15kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +1.0dB Reported relative SS-RSRP values: ±3dB Test 3: Noc: -116dBm/15kHz + ΔBG_offset Ês2 / Noc: +3.0dB Ês3 / Noc: -1.0dB Reported relative SS-RSRP values: ±3dB |  | Test 1: 0 dB 0 dB 0 dB +1.0dB Via mapping Test 2: 0dB 0dB +1.0dB Via mapping Test 3: 0dB 0dB +1.0dB Via mapping |  | Test 1: Noc: -106 dBm/15kHz Noc: -110 dBm/30kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +2.0dB RSRP_x-8 to RSRP_x-1 Test 2: Noc: -88dBm/15kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +2.0dB RSRP_x-8 to RSRP_x-1 Test 3: Noc: -116dBm/15kHz + ΔBG_offset Ês2 / Noc: +3.0dB Ês3 / Noc: 0dB RSRP_x-7 to RSRP_x+1 |  |  |
| The derivation of the SS-RSRP values takes into account the uncertainty in Cell 2 and Cell 3 RSRP from Noc, Ês2 / Noc and Ês3 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The RSRP values given above are for both normal and extreme conditions. |  |  |  |  |  |  |  |  |
| 4.7.1.2.1 EN-DC FR1-FR1 SS-RSRP absolute measurement accuracy |  | TEST CONFIGURATION 1, 2, 4, 5 |  |  |  |  |  |  |
|  |  | Test 1: Noc1: -94.65 dBm/15kHz Noc2: -94.65 dBm/15kHz Ês2 / Noc1: +10.0dB Ês3 / Noc2: +10.0dB Reported RSRP values: ±8dB Test 2: Noc1: -107dBm/15kHz + ΔBG_offset Noc2: -118dBm/15kHz + ΔBG_offset Ês2 / Noc1: +13.0dB Ês3 / Noc2: -3.0dB Reported RSRP values: ±4.5dB |  | Test 1: 0dB 0dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping |  | Test 1: Noc1: -94.65dBm/15kHz Noc2: -94.65dBm/15kHz Ês2 / Noc1: +10.0dB Ês3 / Noc2: +10.0dB RSRP_62 to RSRP_81 Test 2: Noc1: -107dBm/15kHz + ΔBG_offset Noc2: -118dBm/15kHz + ΔBG_offset Ês2 / Noc1: +13.0dB Ês3 / Noc2: -3.0dB RSRP_32 to RSRP_45 RSRP_33 to RSRP_45 RSRP_33 to RSRP_46 RSRP_34 to RSRP_46 RSRP_34 to RSRP_47 RSRP_35 to RSRP_48 RSRP_36 to RSRP_48 depending on operating band |  |  |
|  |  | TEST CONFIGURATION 3, 6 |  |  |  |  |  |  |
|  |  | Test 1: Noc1: -94.65 dBm/15kHz Noc2: -94.65 dBm/15kHz Ês2 / Noc1: +10.0dB Ês3 / Noc2: +10.0dB Reported RSRP values: ±8dB Test 2: Noc1: -107dBm/15kHz + ΔBG_offset Noc1: -118dBm/15kHz + ΔBG_offset Ês2 / Noc1: +13.0dB Ês3 / Noc2: -3.0dB Reported RSRP values: ±4.5dB |  | Test 1: -1.35dB -1.35dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping |  | Test 1: Noc1: -96dBm/15kHz Noc2: -96dBm/15kHz Ês2 / Noc1: +10.0dB Ês3 / Noc2: +10.0dB RSRP_64 to RSRP_83 Test 2: Noc: -107dBm/15kHz + ΔBG_offset Noc1: -118dBm/15kHz + ΔBG_offset Ês2 / Noc1: +13.0dB Ês3 / Noc2: -3.0dB RSRP_35 to RSRP_48 RSRP_36 to RSRP_48 RSRP_36 to RSRP_49 RSRP_37 to RSRP_49 RSRP_37 to RSRP_50 RSRP_38 to RSRP_51 RSRP_39 to RSRP_51 depending on operating band |  |  |
| The derivation of the RSRP values takes into account the uncertainty in Cell 3 SS-RSRP from Noc2 and Ês3 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. The SS-RSRP values given above are for normal conditions. For extreme conditions, the RSRP values are 3.0dB wider at each end for Test 1, and 4.5 dB wider at each for Test 2. |  |  |  |  |  |  |  |  |
| 4.7.1.2.2 EN-DC FR1-FR1 SS-RSRP relative measurement accuracy |  | Test 1: Noc1: -94.65 dBm/15kHz Noc2: -94.65 dBm/15kHz Ês2 / Noc1: +10.0dB Ês3 / Noc2: +10.0dB Reported relative RSRP values: ±4.5dB Test 2: Noc1: -107dBm/15kHz + ΔBG_offset Noc1: -118dBm/15kHz + ΔBG_offset Ês2 / Noc1: +13.0dB Ês3 / Noc2: -3.0dB Reported relative RSRP values: ±4.5dB |  | Test 1: 0dB 0dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping |  | Test 1: Noc1: -94.65dBm/15kHz Noc2: -94.65dBm/15kHz Ês2 / Noc1: +10.0dB Ês3 / Noc2: +10.0dB RSRP_x-7 to RSRP_x+7 Test 2: Noc: -107dBm/15kHz + ΔBG_offset Noc1: -118dBm/15kHz + ΔBG_offset Ês2 / Noc1: +13.0dB Ês3 / Noc2: -3.0dB RSRP_x-31 to RSRP_x-18 |  |  |
| The derivation of the SS-RSRP values takes into account the uncertainty in Cell 2 and Cell 3 RSRP from Noc, Ês2 / Noc and Ês3 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The SS-RSRP values given above are for normal conditions. For extreme conditions, the RSRP values are 1.5dB wider at each end. |  |  |  |  |  |  |  |  |
| 4.7.2.1 EN-DC FR1 SS-RSRQ measurement accuracy |  | TEST CONFIGURATION 1, 2, 4, 5 |  |  |  |  |  |  |
|  |  | Test 1: Noc: -85dBm/15kHz Ês2 / Noc: +3.0dB Ês3 / Noc: +3.0dB Reported RSRQ values: ±2.5dB Test 2: Noc: -101dBm/15kHz Ês2 / Noc: -2.9dB Ês3 / Noc: -2.9dB Reported RSRQ values: ±3.5dB Test 3: Noc: -114dBm/15kHz + ΔBG_offset Ês2 / Noc: -4.0dB Ês3 / Noc: -4.0dB Reported RSRQ values: ±3.5dB |  | Test 1: -1.5dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0dB Via mapping |  | Test 1: Noc: -86.5dBm/15kHz Ês2 / Noc: +3.0dB Ês3 / Noc: +3.0dB RSRQ_52 to RSRQ_62 Test 2: Noc: -101dBm/15kHz Ês2 / Noc: -2.9dB Ês3 / Noc: -2.9dB RSRQ_46 to RSRQ_60 Test 3: Noc: -114dBm/15kHz + ΔBG_offset Ês2 / Noc: -4.0dB Ês3 / Noc: -4.0dB RSRQ_44 to RSRQ_59 |  |  |
|  |  | TEST CONFIGURATION 3, 6 |  |  |  |  |  |  |
|  |  | Test 1: Noc: -91dBm/15kHz Ês2 / Noc: +3.0dB Ês3 / Noc: +3.0dB Reported RSRQ values: ±2.5dB Test 2: N/A Test 3: Noc: -114dBm/15kHz + ΔBG_offset Ês2 / Noc: -4.0dB Ês3 / Noc: -4.0dB Reported RSRQ values: ±3.5dB |  | Test 1: -1.6dB 0dB 0dB Via mapping Test 2: N/A Test 3: 0dB 0dB 0dB Via mapping |  | Test 1: Noc: -92.6dBm/15kHz Ês2 / Noc: +3.0dB Ês3 / Noc: +3.0dB RSRQ_52 to RSRQ_62 Test 2: N/A Test 3: Noc: -114dBm/15kHz + ΔBG_offset Ês2 / Noc: -4.0dB Ês3 / Noc: -4.0dB RSRQ_44 to RSRQ_59 |  |  |
| The derivation of the RSRQ values takes into account the uncertainty in Cell 3 SS-RSRQ from Noc and Ês3 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The SS-RSRQ values given above are for normal conditions. For extreme conditions, the SS-RSRQ values are 1.5dB wider at each end for Test 1, and 0.5 dB wider at each for Tests 2 and 3. |  |  |  |  |  |  |  |  |
| 4.7.2.2.1 EN-DC FR1-FR1 SS-RSRQ absolute measurement accuracy |  | TEST CONFIGURATION 1, 2, 4, 5 |  |  |  |  |  |  |
|  |  | Test 1: Noc1: -80.18 dBm/15kHz Noc2: -80.18 dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês2 / Noc1: 3dB Ês3 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB |  | Test 1: -1.5dB -1.5dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0dB 0dB Via mapping |  | Test 1: Noc1: -81.68dBm/15kHz Noc2: -81.68dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB RSRQ_52 to RSRQ_62 Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB RSRQ_52 to RSRQ_62 Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês2 / Noc1: 3dB Ês3 / Noc2: -1.75dB RSRQ_52 to RSRQ_62 |  |  |
|  |  | TEST CONFIGURATION 3, 6 |  |  |  |  |  |  |
|  |  | Test 1: Noc1: -86.27 dBm/15kHz Noc2: -86.27 dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês2 / Noc1: 3dB Ês3 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB |  | Test 1: -1.53dB -1.53dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0dB 0dB Via mapping |  | Test 1: Noc1: -87.8dBm/15kHz Noc2: -87.8dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB RSRQ_52 to RSRQ_62 Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB RSRQ_52 to RSRQ_62 Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês2 / Noc1: 3dB Ês3 / Noc2: -1.75dB RSRQ_52 to RSRQ_62 |  |  |
| The derivation of the RSRQ values takes into account the uncertainty in Cell 3 SS-RSRQ from Noc2 and Ês3 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. The SS-RSRQ values given above are for normal conditions. For extreme conditions, the RSRQ values are 1.5dB wider at each end. |  |  |  |  |  |  |  |  |
| 4.7.2.2.2 EN-DC FR1-FR1 SS-RSRQ relative measurement accuracy |  | TEST CONFIGURATION 1, 2, 4, 5 |  |  |  |  |  |  |
|  |  | Test 1: Noc1: -80.18 dBm/15kHz Noc2: -80.18 dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB Reported RSRQ values: ±3dB Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB Reported RSRQ values: ±3dB Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês2 / Noc1: 3dB Ês3 / Noc2: -1.75dB Reported RSRQ values: ±3dB |  | Test 1: -1.5dB -1.5dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0dB 0dB Via mapping |  | Test 1: Noc1: -81.68dBm/15kHz Noc2: -81.68dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB RSRQ_x-7 to RSRQ_x+7 Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB RSRQ_x-7 to RSRQ_x+7 Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês2 / Noc1: 3dB Ês3 / Noc2: -1.75dB RSRQ_x-11 to RSRQ_x+2 |  |  |
|  |  | TEST CONFIGURATION 3, 6 |  |  |  |  |  |  |
|  |  | Test 1: Noc1: -86.27 dBm/15kHz Noc2: -86.27 dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB Reported RSRQ values: ±3dB Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês2 / Noc1: 3dB Ês3 / Noc2: -1.75dB Reported RSRQ values: ±3dB |  | Test 1: -1.53dB -1.53dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0dB 0dB Via mapping |  | Test 1: Noc1: -87.8dBm/15kHz Noc2: -87.8dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB RSRQ_x-7 to RSRQ_x+7 Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB RSRQ_x-7 to RSRQ_x+7 Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês2 / Noc1: 3dB Ês3 / Noc2: -1.75dB RSRQ_x-11 to RSRQ_x+2 |  |  |
| The derivation of the RSRQ values takes into account the uncertainty in Cell 2 and Cell 3 SS-RSRQ from Noc2 and Ês3 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. The SS-RSRQ values given above are for normal conditions. For extreme conditions, the RSRQ values are 1dB wider at each end. |  |  |  |  |  |  |  |  |
| 4.7.3.1 EN-DC FR1 SS-SINR measurement accuracy |  | TEST CONFIGURATION 1, 2, 4, 5 |  |  |  |  |  |  |
|  |  | Test 1: Noc: -93dBm/15kHz Ês2 / Noc: +4.54dB Ês3 / Noc: +4.54dB Reported SINR values: ±3.5dB Test 2: Noc: -116dBm/15kHz+ ΔBG_offset Ês2 / Noc: -4dB Ês3 / Noc: -4dB Reported SINR values: ±3.5dB |  | Test 1: 0dB 0dB 0dB Via mapping Test 2: 0dB 0.5dB 0.5dB Via mapping |  | Test 1: Noc: -93dBm/15kHz Ês2 / Noc: +4.54dB Ês3 / Noc: +4.54dB SINR_31 to SINR_49 Test 2: Noc: -116dBm/15kHz+ ΔBG_offset Ês2 / Noc: -3.5dB Ês3 / Noc: -3.5dB SINR_28 to SINR_45 |  |  |
|  |  | TEST CONFIGURATION 3, 6 |  |  |  |  |  |  |
|  |  | Test 1: Noc: -93dBm/15kHz Ês2 / Noc: +4.54dB Ês3 / Noc: +4.54dB Reported SINR values: ±3.5dB Test 2: Noc: -116dBm/15kHz+ ΔBG_offset Ês2 / Noc: -4dB Ês3 / Noc: -4dB Reported SINR values: ±3.5dB |  | Test 1: -0.2dB 0dB 0dB Via mapping Test 2: 0dB 0.5dB 0.5dB Via mapping |  | Test 1: Noc: -93.2dBm/15kHz Ês2 / Noc: +4.54dB Ês3 / Noc: +4.54dB SINR_31 to SINR_49 Test 2: Noc: -116dBm/15kHz+ ΔBG_offset Ês2 / Noc: -3.5dB Ês3 / Noc: -3.5dB SINR_28 to SINR_45 |  |  |
| The derivation of the SINR values takes into account the uncertainty in Cell 3 SS- SINR from Noc and Ês3 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The SS- SINR values given above are for normal conditions. For extreme conditions, the SS- SINR values are 0.5dB wider at each end. |  |  |  |  |  |  |  |  |
| 4.7.3.2.1 EN-DC FR1-FR1 SS-SINR absolute measurement accuracy |  | TEST CONFIGURATION 1, 2, 3, 4, 5, 6 |  |  |  |  |  |  |
|  |  | Test 1: Noc1: -88 dBm/15kHz Noc2: -88 dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB Reported SINR values: ±3dB Test 2: Noc1: -108.5dBm/15kHz Noc2: -108.5dBm/15kHz Ês2 / Noc1: 20dB Ês3 / Noc2: 20dB Reported SINR values: ±3dB Test 3: Noc1: -119.5dBm/15kHz + ΔBG_offset Noc2: -119.5dBm/15kHz + ΔBG_offset Ês2 / Noc1: -4dB Ês3 / Noc2: -4dB Reported SINR values: ±3.5dB |  | Test 1: 0dB 0dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0.8dB 0.8dB Via mapping |  | Test 1: Noc1: -88 dBm/15kHz Noc2: -88 dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB SINR_35 to SINR_51 Test 2: Noc1: -108.5dBm/15kHz Noc2: -108.5dBm/15kHz Ês2 / Noc1: 20dB Ês3 / Noc2: 20dB SINR_79 to SINR_94 Test 3: Noc1: -119.5dBm/15kHz + ΔBG_offset Noc2: -119.5dBm/15kHz + ΔBG_offset Ês2 / Noc1: -3.2dB Ês3 / Noc2: -3.2dB SINR_32 to SINR_49 |  |  |
| The derivation of the SINR values takes into account the uncertainty in Cell 3 SS- SINR from Noc and Ês3 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The SS- SINR values given above are for normal conditions. For extreme conditions, the SS- SINR values are 1dB wider at each for Tests 1 and 2 and 0.5dB wider at each end for Test 3. |  |  |  |  |  |  |  |  |
| 4.7.3.2.2 EN-DC FR1-FR1 SS-SINR relative measurement accuracy |  | TEST CONFIGURATION 1, 2, 3, 4, 5, 6 |  |  |  |  |  |  |
|  |  | Test 1: Noc1: -88 dBm/15kHz Noc2: -88 dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB Reported SINR values: ±3.5dB Test 2: Noc1: -108.5dBm/15kHz Noc2: -108.5dBm/15kHz Ês2 / Noc1: 20dB Ês3 / Noc2: 20dB Reported SINR values: ±3.5dB Test 3: Noc1: -119.5dBm/15kHz + ΔBG_offset Noc2: -119.5dBm/15kHz + ΔBG_offset Ês2 / Noc1: -4dB Ês3 / Noc2: -4dB Reported SINR values: ±4dB |  | Test 1: 0dB 0dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0.8dB 0.8dB Via mapping |  | Test 1: Noc1: -88 dBm/15kHz Noc2: -88 dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB SINR_x-10 to SINR_x+10 Test 2: Noc1: -108.5dBm/15kHz Noc2: -108.5dBm/15kHz Ês2 / Noc1: 20dB Ês3 / Noc2: 20dB SINR_x-10 to SINR_x+10 Test 3: Noc1: -119.5dBm/15kHz + ΔBG_offset Noc2: -119.5dBm/15kHz + ΔBG_offset Ês2 / Noc1: -3.2dB Ês3 / Noc2: -3.2dB SINR_x-11 to SINR_x+11 |  |  |
| The derivation of the SINR values takes into account the uncertainty in Cell 3 SS- SINR from Noc and Ês3 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The SS- SINR values given above are for normal conditions. For extreme conditions, the SS- SINR values are 0.5dB wider at each for Tests 1 and 2 and 0dB wider at each end for Test 3. |  |  |  |  |  |  |  |  |
| 4.7.4.1.1 EN-DC FR1 SSB based L1-RSRP absolute measurement accuracy |  |  |  |  |  |  |  |  |
| Test Configuration 1,2,4,5 |  | Test 1: Noc1: -94.65 dBm/15kHz Ês2 / Noc1: 10dB Reported L1-RSRP values: ±8.5dB (±4.5dB additionally for extreme conditions) Test 2: Noc1: -119.5dBm/15kHz + ΔBG_offset Ês2 / Noc1: -3dB Reported L1-RSRP values: ±5dB (±3dB additionally for extreme conditions) |  | Test 1: 0dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping |  | Test 1: Noc1: -94.65dBm/15kHz Ês2 / Noc1: 10dB RSRP_62 to RSRP_82 Test 2: Noc1: -119.2dBm/15kHz + ΔBG_offset Ês2 / Noc1: -2.2dB RSRP_31 to RSRP_44 RSRP_31 to RSRP_45 RSRP_32 to RSRP_45 RSRP_32 to RSRP_46 RSRP_33 to RSRP_46 RSRP_34 to RSRP_47 RSRP_34 to RSRP_48 depending on operating band |  |  |
| Test Configuration 3,6 |  | Test 1: Noc1: -94.65 dBm/15kHz Ês2 / Noc1: 10dB Reported L1-RSRP values: ±8.5dB (±4.5dB additionally for extreme conditions) Test 2: Noc1: -119.5dBm/15kHz + ΔBG_offset Ês2 / Noc1: -3dB Reported L1-RSRP values: ±5dB (±3dB additionally for extreme conditions) |  | Test 1: -1.35dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping |  | Test 1: Noc1: -96dBm/15kHz Ês2 / Noc1: 10dB RSRP_63 to RSRP_84 Test 2: Noc1: -116.2dBm/15kHz + ΔBG_offset Ês2 / Noc1: -2.2dB RSRP_34 to RSRP_47 RSRP_34 to RSRP_48 RSRP_35 to RSRP_48 RSRP_35 to RSRP_49 RSRP_36 to RSRP_49 RSRP_37 to RSRP_50 RSRP_37 to RSRP_51 depending on operating band |  |  |
| 4.7.4.1.2 EN-DC FR1 SSB based L1-RSRP relative measurement accuracy |  |  |  |  |  |  |  |  |
| Test Configuration 1,2,4,5 |  | Test 1: Noc1: -94.65 dBm/15kHz Ês2 / Noc1: 10dB Reported L1-RSRP values: ±8.5dB (±1dB additionally for extreme conditions) Test 2: Noc1: -119.5dBm/15kHz + ΔBG_offset Ês2 / Noc1: -3dB Reported L1-RSRP values: ±5dB (±1dB additionally for extreme conditions) |  | Test 1: 0dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping |  | Test 1: Noc1: -94.65dBm/15kHz Ês2 / Noc1: 10dB RSRP_x-3 to RSRP_x+3 Test 2: Noc1: -119.2dBm/15kHz + ΔBG_offset Ês2 / Noc1: -2.2dB RSRP_x-3 to RSRP_x+3 |  |  |
| Test Configuration 3,6 |  | Test 1: Noc1: -94.65 dBm/15kHz Ês2 / Noc1: 10dB Reported L1-RSRP values: ±8.5dB (±1dB additionally for extreme conditions) Test 2: Noc1: -119.5dBm/15kHz + ΔBG_offset Ês2 / Noc1: -3dB Reported L1-RSRP values: ±5dB (±1dB additionally for extreme conditions) |  | Test 1: -1.35dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping |  | Test 1: Noc1: -96dBm/15kHz Ês2 / Noc1: 10dB RSRP_x-3 to RSRP_x+3 Test 2: Noc1: -116.2dBm/15kHz + ΔBG_offset Ês2 / Noc1: -2.2dB RSRP_x-3 to RSRP_x+3 |  |  |
| 4.7.4.2.1 EN-DC FR1 CSI-RS based L1-RSRP absolute measurement accuracy |  | Same as 4.7.4.1.1 |  | Same as 4.7.4.1.1 |  | Same as 4.7.4.1.1 |  |  |
| 4.7.4.2.2 EN-DC FR1 CSI-RS based L1-RSRP relative measurement accuracy |  | Same as 4.7.4.1.2 |  | Same as 4.7.4.1.2 |  | Same as 4.7.4.1.2 |  |  |
| 4.7.5.1 EN-DC FR1 SFTD measurement accuracy |  | Noc1: -104dBm/15kHz Ês1 / Noc1: -3.0dB Noc2: -104dBm/15kHz Ês2 / Noc2: 17.0dB |  | 0dB 0.3dB 0dB 0dB |  | Noc1: -104dBm/15kHz Ês1 / Noc1: -2.7dB Noc2: -104dBm/15kHz Ês2 / Noc2: 17.0dB |  |  |
| 4.7.7.1.1 EN-DC FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR measurement accuracy |  |  |  |  |  |  |  |  |  |  |
| Test Configuration 1,2,4,5 |  | Test 1: Noc1: -94.65 dBm/15kHz Ês2 / Noc1: 10dB Reported CSI-SINR values: ±5.5dB (±1dB additionally for extreme conditions) Test 2: Noc1: -119.5dBm/15kHz + ΔBG_offset Ês2 / Noc1: -3dB Reported CSI-SINR values: ±5.5dB (±1dB additionally for extreme conditions) |  |  | Test 1: 0dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping |  |  | Test 1: Noc1: -94.65dBm/15kHz Ês2 / Noc1: 10dB All bands: L1-SINR_28 to L1-SINR_57 Test 2: Noc1: -119.2dBm/15kHz + ΔBG_offset Ês2 / Noc1: -2.2dB All bands: L1-SINR_27 to L1-SINR_58 |  |  |
| Test Configuration 3,6 |  | Test 1: Noc1: -94.65 dBm/15kHz Ês2 / Noc1: 10dB Reported CSI-SINR values: ±5.5dB (±1dB additionally for extreme conditions) Test 2: Noc1: -119.5dBm/15kHz + ΔBG_offset Ês2 / Noc1: -3dB Reported CSI-SINR values: ±5.5dB (±1dB additionally for extreme conditions) |  |  | Test 1: -1.35dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping |  |  | Test 1: Noc1: -96dBm/15kHz Ês2 / Noc1: 10dB All bands: L1-SINR_28 to L1-SINR_57 Test 2: Noc1: -116.2dBm/15kHz + ΔBG_offset Ês2 / Noc1: -2.2dB All bands: L1-SINR_27 to L1-SINR_58 |  |  |
| 4.7.7.1.2 EN-DC FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR relative measurement accuracy |  |  |  |  |  |  |  |  |  |  |
| Same as 4.7.7.1.1 |  | Same as 4.7.7.1.1 |  |  | Same as 4.7.7.1.1 |  |  | Test 1: Noc1: -94.65dBm/15kHz Ês2 / Noc1: 10dB L1-SINR_x-9 to L1-SINR_x+9 Test 2: Noc1: -119.2dBm/15kHz + ΔBG_offset Ês2 / Noc1: -2.2dB L1-SINR_x-9 to L1-SINR_x+9 |  |  |
| Same as 4.7.7.1.1 |  | Same as 4.7.7.1.1 |  |  | Same as 4.7.7.1.1 |  |  | Test 1: Noc1: -96dBm/15kHz Ês2 / Noc1: 10dB L1-SINR_x-9 to L1-SINR_x+9 Test 2: Noc1: -116.2dBm/15kHz + ΔBG_offset Ês2 / Noc1: -2.2dB L1-SINR_x-9 to L1-SINR_x+9 |  |  |
| 4.7.7.2 EN-DC FR1 SSB based CMR and dedicated IMR L1-SINR absolute measurement accuracy |  |  |  |  |  |  |  |  |  |  |
| Test Configuration 1,2,4,5 |  | Test 1: Noc1: -94.65 dBm/15kHz Ês2 / Noc1: 10dB Reported SS-SINR values: ±4.5dB (±1dB additionally for extreme conditions) Test 2: Noc1: -119.5dBm/15kHz + ΔBG_offset Ês2 / Noc1: -3dB Reported SS-SINR values: ±4.5dB (±1dB additionally for extreme conditions) |  |  | Test 1: 0dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping |  |  | Test 1: Noc1: -94.65dBm/15kHz Ês2 / Noc1: 10dB All bands: L1-SINR_30 to L1-SINR_55 Test 2: Noc1: -119.2dBm/15kHz + ΔBG_offset Ês2 / Noc1: -2.2dB All bands: L1-SINR_29 to L1-SINR_56 |  |  |
| Test Configuration 3,6 |  | Test 1: Noc1: -94.65 dBm/15kHz Ês2 / Noc1: 10dB Reported CSI-SINR values: ±4.5dB (±1dB additionally for extreme conditions) Test 2: Noc1: -119.5dBm/15kHz + ΔBG_offset Ês2 / Noc1: -3dB Reported CSI-SINR values: ±4.5dB (±1dB additionally for extreme conditions) |  |  | Test 1: -1.35dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping |  |  | Test 1: Noc1: -96dBm/15kHz Ês2 / Noc1: 10dB All bands: L1-SINR_30 to L1-SINR_55 Test 2: Noc1: -116.2dBm/15kHz + ΔBG_offset Ês2 / Noc1: -2.2dB All bands: L1-SINR_29 to L1-SINR_56 |  |  |
| 4.7.7.3.1 EN-DC FR1 CSI-RS based CMR and dedicated IMR L1-SINR absolute measurement accuracy |  |  |  |  |  |  |  |  |  |  |
| Test Configuration 1,2,4,5 |  | Test 1: Noc1: -94.65 dBm/15kHz Ês2 / Noc1: 10dB Reported CSI-SINR values: ±4.5dB (±1dB additionally for extreme conditions) Test 2: Noc1: -119.5dBm/15kHz + ΔBG_offset Ês2 / Noc1: -3dB Reported CSI-SINR values: ±4.5dB (±1dB additionally for extreme conditions) |  |  | Test 1: 0dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping |  |  | Test 1: Noc1: -94.65dBm/15kHz Ês2 / Noc1: 10dB All bands: L1-SINR_30 to L1-SINR_55 Test 2: Noc1: -119.2dBm/15kHz + ΔBG_offset Ês2 / Noc1: -2.2dB All bands: L1-SINR_29 to L1-SINR_56 |  |  |
| Test Configuration 3,6 |  | Test 1: Noc1: -94.65 dBm/15kHz Ês2 / Noc1: 10dB Reported CSI-SINR values: ±4.5dB (±1dB additionally for extreme conditions) Test 2: Noc1: -119.5dBm/15kHz + ΔBG_offset Ês2 / Noc1: -3dB Reported CSI-SINR values: ±4.5dB (±1dB additionally for extreme conditions) |  |  | Test 1: -1.35dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping |  |  | Test 1: Noc1: -96dBm/15kHz Ês2 / Noc1: 10dB All bands: L1-SINR_30 to L1-SINR_55 Test 2: Noc1: -116.2dBm/15kHz + ΔBG_offset Ês2 / Noc1: -2.2dB All bands: L1-SINR_29 to L1-SINR_56 |  |  |
| 4.7.7.3.2 EN-DC FR1 CSI-RS based CMR and dedicated IMR L1-SINR relative measurement accuracy |  |  |  |  |  |  |  |  |  |  |
| Same as 4.7.7.1.2 |  | Same as 4.7.7.1.2 |  |  | Same as 4.7.7.1.2 |  |  | Same as 4.7.7.1.2 |  |  |
| Same as 4.7.7.1.2 |  | Same as 4.7.7.1.2 |  |  | Same as 4.7.7.1.2 |  |  | Same as 4.7.7.1.2 |  |  |

Table F.1.3.2-2: Derivation of test requirements for NR SA FR1 RRM tests

| Test |  |  | Minimum requirement in TS 38.133 [6] |  | Test tolerance (TT) |  | Test requirement in TS 38.533 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6.1.1.1 NR SA FR1 cell re-selection |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +16dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +13dB Ês2 / Noc: +16dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +16dB Ês2 / Noc: +13dB |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0.45dB During T3: 0dB 0.45dB 0dB |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +16dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +13dB Ês2 / Noc: +16.45dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +16.45dB Ês2 / Noc: +13dB |  |
| 6.1.1.2 NR SA FR1-FR1 cell re-selection |  |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -4dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -infinity During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: +12dB |  | During T1: 0dB -2dB 1.6dB 0.4dB During T2: 0dB 0dB 1.6dB 0dB During T3: 0dB 0dB 1.6dB 1.6dB |  | During T1: Noc1: -98dBm/15kHz Noc2: -100dBm/15kHz Ês1 / Noc1: +15.6dB Ês2 / Noc2: -3.6dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +15.6dB Ês2 / Noc2: -infinity During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +15.6dB Ês2 / Noc2: 13.6dB |  |
| 6.1.1.3 NR SA FR1 cell re-selection for UE fulfilling low mobility relaxed measurement criterion |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +13dB Ês2 / Noc: +16dB During T2: Noc: -98dBm/15kHz Ês1 / Noc: +16dB Ês2 / Noc: +13dB In Signalling SSearchDeltaP = 3dB |  | During T1: 0dB -0.45dB 0dB During T2: 0dB 0dB -0.45dB In Signalling 3dB |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +12.55dB Ês2 / Noc: +16dB During T2: Noc: -98dBm/15kHz Ês1 / Noc: +16dB Ês2 / Noc: +12.55dB In Signalling SSearchDeltaP = 6dB |  |
| 6.1.1.4 NR SA FR1 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +13dB Ês2 / Noc: +16dB During T2: Noc: -98dBm/15kHz Ês1 / Noc: +16dB Ês2 / Noc: +13dB |  | During T1: 0dB -0.45dB 0dB During T2: 0dB 0dB -0.45dB |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +12.55dB Ês2 / Noc: +16dB During T2: Noc: -98dBm/15kHz Ês1 / Noc: +16dB Ês2 / Noc: +12.55dB |  |
| 6.1.1.5 NR SA FR1-FR1 cell re-selection for UE fulfilling low mobility relaxed measurement criterion |  |  | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +14dB Noc2: -98dBm/15kHz Ês2 / Noc2: -4dB During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +14dB Noc2: -98dBm/15kHz Ês2 / Noc2: +12dB In signalling: Thresh_X_HighP of Cell 1 = 48dB Thresh_X_HighP of Cell 2 = 48dB Thresh_X_LowP of Cell 1 = 50dB Thresh_X_LowP of Cell 2 = 50dB Thresh_Serving_LowP of Cell 1 = 44dB Thresh_Serving_LowP of Cell 2 = 44dB SSearchDeltaP of Cell 1 = 3dB SSearchDeltaP of Cell 2 = 3dB |  | During T1: 0dB 0dB 0dB 0.3dB During T2: 0dB 0dB 0dB -2.25dB In signalling: 0dB -4dB -2dB 0dB 0dB 2dB 0dB 12dB |  | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +14dB Noc2: -98dBm/15kHz Ês2 / Noc2: -3.7dB During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +14dB Noc1: -98dBm/15kHz Ês2 / Noc2: +9.75dB In signalling: Thresh_X_HighP of Cell 1 = 48dB Thresh_X_HighP of Cell 2 = 44dB Thresh_X_LowP of Cell 1 = 48dB Thresh_X_LowP of Cell 2 = 50dB Thresh_Serving_LowP of Cell 1 = 44dB Thresh_Serving_LowP of Cell 2 = 46dB SSearchDeltaP of Cell 1 = 3dB SSearchDeltaP of Cell 2 = 15dB |  |
| 6.1.1.6 NR SA FR1-FR1 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion |  |  | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +14dB Noc2: -98dBm/15kHz Ês2 / Noc2: -4dB During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +14dB Noc2: -98dBm/15kHz Ês2 / Noc2: +12dB In signalling: SSearchThresholdP of Cell 1 = 50dB SSearchThresholdP of Cell 2 = 50dB |  | During T1: 0dB 2dB -2dB 0.35dB During T2: 0dB 0dB -2dB 4dB In signalling: 0dB -16dB |  | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +16dB Noc2: -100dBm/15kHz Ês2 / Noc2: -3.65dB During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +14dB Noc1: -100dBm/15kHz Ês2 / Noc2: +16dB In signalling: SSearchThresholdP of Cell 1 = 50dB SSearchThresholdP of Cell 2 = 34dB |  |
| 6.1.1.7 NR SA FR1 cell re-selection for UE configured with highSpeedMeasFlag-r16 |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +16dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +13dB Ês2 / Noc: +16dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +16dB Ês2 / Noc: +13dB |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0.45dB During T3: 0dB 0.45dB 0dB |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +16dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +13dB Ês2 / Noc: +16.45dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +16.45dB Ês2 / Noc: +13dB |  |
| 6.1.2.1 NR SA FR1 – E-UTRA cell re-selection to higher priority E-UTRA |  |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: +12dB During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -4dB |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 1.6dB 1.6dB During T3: 0dB -2dB 1.6dB 0.4dB |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +15.6dB Ês2 / Noc2: 13.6dB During T3: Noc1: -98dBm/15kHz Noc2: -100dBm/15kHz Ês1 / Noc1: +15.6dB Ês2 / Noc2: -3.6dB |  |
| 6.1.2.2 NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRA |  |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -4dB Ês2 / Noc2: +14dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +12dB Ês2 / Noc2: +14dB |  | During T1: -2dB 0dB 0.4dB 1.6dB During T2: 0dB 0dB 1.6dB 0dB |  | During T1: Noc1: -100dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -3.6dB Ês2 / Noc2: +15.6dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +13.6dB Ês2 / Noc2: +14dB |  |
| 6.1.2.3 NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRAN for UE fulfilling low mobility relaxed measurement criterion |  |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -4dB Ês2 / Noc2: +14dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +12dB Ês2 / Noc2: +14dB |  | During T1: -2dB 0dB 0.4dB 1.6dB During T2: 0dB 0dB 1.6dB 0dB |  | During T1: Noc1: -100dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -3.6dB Ês2 / Noc2: +15.6dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +13.6dB Ês2 / Noc2: +14dB |  |
| 6.1.2.4 NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRAN for UE fulfilling not-at-cell edge relaxed measurement criterion |  |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -4dB Ês2 / Noc2: +14dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +12dB Ês2 / Noc2: +14dB |  | During T1: -2dB 0dB 0.4dB 1.6dB During T2: 0dB 0dB 1.6dB 0dB |  | During T1: Noc1: -100dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -3.6dB Ês2 / Noc2: +15.6dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +13.6dB Ês2 / Noc2: +14dB |  |
| 6.1.2.5 NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRA for UE configured with highSpeedMeasFlag-r16 |  |  | Same as 6.1.2.2 |  | Same as 6.1.2.2 |  | Same as 6.1.2.2 |  |
| 6.3.1.1 NR SA FR1 handover with known target cell |  |  | TEST CONFIGURATION 1, 2 A3-Offset: 0dB |  | -1dB |  | A3-Offset: -1dB |  |
|  |  |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 8dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +11dB During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +11dB |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB -1dB During T3: 0dB 0dB 0dB -1dB |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 8dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +10dB During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +10dB |  |
|  |  |  | TEST CONFIGURATION 3 A3-Offset: 0dB |  | -1dB |  | A3-Offset: -1dB |  |
|  |  |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 8dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +11dB During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +11dB |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB -1dB During T3: 0dB 0dB 0dB -1dB |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 8dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +10dB During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +10dB |  |
| 6.3.1.2 NR SA FR1 handover with unknown target cell |  |  | A3-Offset: 0dB During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 8dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +8dB |  | -1dB During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 1.5dB |  | A3-Offset: -1dB During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 8dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +9.5dB |  |
| 6.3.1.3 NR SA FR1-FR1 Handover with unknown Target Cell |  |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 4dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: +5dB |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 1.7dB |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 4dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: +6.7dB |  |
| 6.3.1.4 NR SA FR1 – E-UTRA handover with known target cell |  |  | During T1: Noc1: -100dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +12dB Ês2 / Noc2: -infinity During T2: Noc1: -100dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -4dB Ês2 / Noc2: +8dB During T3: Noc1: -100dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -4dB Ês2 / Noc2: +8dB |  | During T1: 0dB 0dB 1.55dB 0dB During T2: 0dB 0dB -1.55dB 1.55dB During T3: 0dB 0dB -1.55dB 1.55dB |  | During T1: Noc1: -100dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +13.55dB Ês2 / Noc2: -infinity During T2: Noc1: -100dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -5.55dB Ês2 / Noc2: +9.55dB During T3: Noc1: -100dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -5.55dB Ês2 / Noc2: +9.55dB |  |
| 6.3.1.5 NR SA FR1 – E-UTRA handover with unknown target cell |  |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 0dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 0dB Ês2 / Noc2: +7dB |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 0dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 0dB Ês2 / Noc2: +7dB |  |
| 6.3.1.6 NR SA FR1 – UTRAN FDD handover with known target cell |  |  | During T1: Noc: -100dBm/15kHz Ês / Noc: 12dB Ioc: -70dBm/3.84MHz Îor / Ioc: -infinity CPICH Ec / Ior: -10dB SCH Ec / Ior: -12dB During T2: Noc: -100dBm/15kHz Ês / Noc: -4dB Ioc: -70dBm/3.84MHz Îor / Ioc: -infinity CPICH Ec / Ior: -10dB SCH Ec / Ior: -12dB During T3: Noc: -100dBm/15kHz Ês / Noc: -4dB Ioc: -70dBm/3.84MHz Îor / Ioc: -infinity CPICH Ec / Ior: -10dB SCH Ec / Ior: -12dB |  | During T1: -1.54dB 0dB 0dB 0dB 0dB 0dB During T2: -1.54dB 0dB 0dB 0dB 0dB 0dB During T3: -1.54dB 0dB 0dB 0dB 0dB 0dB |  | During T1: Noc: -101.54dBm/15kHz Ês / Noc: 12dB Ioc: -70dBm/3.84MHz Îor / Ioc: -infinity CPICH Ec / Ior: -10dB SCH Ec / Ior: -12dB During T2: Noc: -101.54dBm/15kHz Ês / Noc: -4dB Ioc: -70dBm/3.84MHz Îor / Ioc: -infinity CPICH Ec / Ior: -10dB SCH Ec / Ior: -12dB During T3: Noc: -101.54dBm/15kHz Ês / Noc: -4dB Ioc: -70dBm/3.84MHz Îor / Ioc: -infinity CPICH Ec / Ior: -10dB SCH Ec / Ior: -12dB |  |
| 6.3.1.7 NR SA FR1 synchronous DAPS handover |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +9dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +9dB During T4: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +9dB During T5: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +9dB In signaling: A3-offset: 0dB |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 1dB During T3: 0dB 0dB 1dB During T4: 0dB 0dB 1dB During T5: 0dB 0dB 1dB In signaling: -1dB |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +10dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +10dB During T4: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +10dB During T5: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +10dB In signaling: A3-offset: -1dB |  |
| 6.3.1.8 NR SA FR1 asynchronous DAPS handover |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +9dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +9dB During T4: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +9dB During T5: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +9dB In signaling: A3-offset: 0dB |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 1dB During T3: 0dB 0dB 1dB During T4: 0dB 0dB 1dB During T5: 0dB 0dB 1dB In signaling: -1dB |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +10dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +10dB During T4: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +10dB During T5: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +10dB In signaling: A3-offset: -1dB |  |
| 6.3.1.9 NR SA FR1 Intra-band inter-frequency synchronous DAPS handover |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +8dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +8dB During T4: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +8dB During T5: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +8dB In signaling: A3-offset: 0dB |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 3.7dB During T3: 0dB 0dB 3.7dB During T4: 0dB 0dB 3.7dB During T5: 0dB 0dB 3.7dB In signaling: -3dB |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +11.7dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +11.7dB During T4: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +11.7dB During T5: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +11.7dB In signaling: A3-offset: -3dB |  |
| 6.3.1.10 NR SA FR1 Intra-band inter-frequency asynchronous DAPS handover |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +8dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +8dB During T4: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +8dB During T5: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +8dB In signaling: A3-offset: 0dB |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 3.7dB During T3: 0dB 0dB 3.7dB During T4: 0dB 0dB 3.7dB During T5: 0dB 0dB 3.7dB In signaling: -3dB |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +11.7dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +11.7dB During T4: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +11.7dB During T5: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +11.7dB In signaling: A3-offset: -3dB |  |
| 6.3.1.11 NR SA FR1 Inter-band inter-frequency synchronous DAPS handover |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: +4dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: +4dB During T4: Noc: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: +4dB During T5: Noc: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: +4dB In signaling: A3-offset: -6dB |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB During T3: 0dB 0dB 0dB During T4: 0dB 0dB 0dB During T5: 0dB 0dB 0dB In signaling: -1dB for config 1,2,4,5,9 In signaling: -4dB for config 3,6 In signaling: 2dB for config 7,8 |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: +4dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: +4dB During T4: Noc: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: +4dB During T5: Noc: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: +4dB In signaling: -7dB for config 1,2,4,5,9 In signaling: -10dB for config 3,6 In signaling: -4dB for config 7,8 |  |
| 6.3.1.12 NR SA FR1 Inter-band inter-frequency asynchronous DAPS handover |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: +4dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: +4dB During T4: Noc: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: +4dB During T5: Noc: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: +4dB In signaling: A3-offset: -4dB |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB During T3: 0dB 0dB 0dB During T4: 0dB 0dB 0dB During T5: 0dB 0dB 0dB In signaling: -3dB for config 1,2,4,5,9 In signaling: -6dB for config 3,6 In signaling: 0dB for config 7,8 |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: +4dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: +4dB During T4: Noc: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: +4dB During T5: Noc: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: +4dB In signaling: -7dB for config 1,2,4,5,9 In signaling: -10dB for config 3,6 In signaling: -4dB for config 7,8 |  |
| 6.3.2.1.1 NR SA FR1 RRC re-establishment |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +7dB Ês2 / Noc: +4dB During T2: Noc: -98dBm/15kHz Ês1 / Noc: -infinity Ês2 / Noc: +4dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: -infinity Ês2 / Noc: +4dB |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB During T3: 0dB 0dB 0dB |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +7dB Ês2 / Noc: +4dB During T2: Noc: -98dBm/15kHz Ês1 / Noc: -infinity Ês2 / Noc: +4dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: -infinity Ês2 / Noc: +4dB |  |
| 6.3.2.1.2 NR SA FR1 - FR1 RRC re-establishment |  |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -infinity Ês2 / Noc2: -infinity During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -infinity Ês2 / Noc2: +7dB |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -infinity Ês2 / Noc2: -infinity During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -infinity Ês2 / Noc2: +7dB |  |
| 6.3.2.1.3 NR SA FR1 RRC re-establishment without serving cell timing |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: -infinity Ês2 / Noc: -infinity During T3: Noc: -98dBm/15kHz Ês1 / Noc: -infinity Ês2 / Noc: +4dB |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB During T3: 0dB 0dB 0dB |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: -infinity Ês2 / Noc: -infinity During T3: Noc: -98dBm/15kHz Ês1 / Noc: -infinity Ês2 / Noc: +4dB |  |
| 6.3.2.2.1 |  |  | Absolute uplink power: Normal conditions ±9dB Relative uplink power step: Normal conditions ±2.5dB Uplink timing: 15kHz SCS Te ±12*64*Tc 30kHz SCS Te ±8*64*Tc |  | 2.1dB 0.7dB 112Tc 112Tc |  | Absolute uplink power: Normal conditions ±11.1dB Relative uplink power step: Normal conditions ±3.2dB Uplink timing: 15kHz SCS Te ±880*Tc 30kHz SCS Te ±624*Tc |  |
| 6.3.2.2.2 |  |  | Same as 6.3.2.2.1 |  | Same as 6.3.2.2.1 |  | Same as 6.3.2.2.1 |  |
| 6.3.2.2.3 |  |  | Same as 6.3.2.2.1 |  | Same as 6.3.2.2.1 |  | Same as 6.3.2.2.1 |  |
| 6.3.2.2.4 |  |  | Same as 6.3.2.2.1 |  | Same as 6.3.2.2.1 |  | Same as 6.3.2.2.1 |  |
| 6.3.2.3.1 NR SA FR1 RRC connection release with redirection |  |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: +4dB |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: +4dB |  |
| 6.3.2.3.2 NR SA FR1 – E-UTRA RRC connection release with redirection |  |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: +4dB |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: +4dB |  |
| 6.3.3.1 NR SA FR1 conditional handover |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +11dB In signaling: A3-offset: 0dB |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB -1dB In signaling: -1dB |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +10dB In signaling: A3-offset: -1dB |  |
| 6.3.3.2 NR SA FR1-FR1 conditional handover |  |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: +5dB |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 1.7dB |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: +6.7dB |  |
| 6.4.1.1 EN-DC FR1 UE transmit timing accuracy |  |  | Test 1 (no DRX): Uplink timing: ±12*64 Tc for 15 KHz SSB SCS,15 kHz UL SCS ±10*64 Tc for 15 KHz SSB SCS,30 kHz UL SCS ±10*64 Tc for 15 KHz SSB SCS,60 kHz UL SCS ±8*64 Tc for 30 KHz SSB SCS,15 kHz UL SCS ±8*64 Tc for 30 KHz SSB SCS,30 kHz UL SCS ±7*64 Tc for 30 KHz SSB SCS,60 kHz UL SCS Max step size Tq: 5.5*64*Tc Min adjust rate Tp: 5.5*64*Tc Max adjust rate: 5.5*64*Tc Ês2 / Noc: +3.00dB Noc = -98 dBm/15 kHz (Config 1,2,3) Test 2 (with DRX): ±12*64 Tc for 15 KHz SSB SCS,15 kHz UL SCS ±10*64 Tc for 15 KHz SSB SCS,30 kHz UL SCS ±10*64 Tc for 15 KHz SSB SCS,60 kHz UL SCS ±8*64 Tc for 30 KHz SSB SCS,15 kHz UL SCS ±8*64 Tc for 30 KHz SSB SCS,30 kHz UL SCS ±7*64 Tc for 30 KHz SSB SCS,60 kHz UL SCS Ês1 / Noc: +3.00dB |  | ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc +0.5*64Tc -3.6*64*Tc +1.1*64*Tc +0.3 dB +1.5 dB ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc +0.3dB |  | Test 1 (10MHz Ch BW): Uplink timing: ±13.75*64*Tc Uplink timing: ±11.75*64*Tc Uplink timing: ±11.75*64*Tc Uplink timing: ±9.75*64*Tc Uplink timing: ±9.75*64*Tc Uplink timing: ±8.75*64*Tc Max step size Tq: 6.0*64*Tc Min adjust rate: 1.9*64*Tc Max adjust rate: 6.6*64*Tc Ês1 / Noc: +3.30dB Noc = -98 dBm/15 kHz (Config 1,2,3) +1.5 dB Test 2 (with DRX): Uplink timing: ±13.75*64*Tc Uplink timing: ±11.75*64*Tc Uplink timing: ±11.75*64*Tc Uplink timing: ±9.75*64*Tc Uplink timing: ±9.75*64*Tc Uplink timing: ±8.75*64*Tc Ês1 / Noc: +3.30dB |  |
| 6.4.3.1 NR SA FR1 timing advance adjustment accuracy |  |  | Noc = -98 dBm/15 kHz (Config 1, 2, 4, 5) Noc = -95 dBm/15 kHz (Config 3, 6) Êsx / Noc = 3 dB UE Timing Advance Adjustment Accuracy for 15kHz SCS = ±256 Tc+ TT UE Timing Advance Adjustment Accuracy for 30kHz SCS = ±256 Tc+ TT |  | 0 0 0 +/- 88 Tc +/- 88 Tc |  | Noc = -98 dBm/15 kHz (Config 1, 2) Noc = -95 dBm/15 kHz (Config 3) Êsx / Noc = 3 dB UE TAAA for 15kHz SCS = ±344 Tc UE TAAA for 30kHz SCS = ±344 Tc |  |
| 6.5.1.1 NR SA FR1 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in non-DRX mode |  |  | SNR during T1: 1dB T2: -7dB T3: -15dB |  | Offset during T1: +0.8dB T2: +0.8dB T3: -0.8dB |  | SNR during T1: 1.8dB T2: -6.2dB T3: -15.8dB |  |
| 6.5.1.2 NR SA FR1 radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in non-DRX mode |  |  | SNR during T1: 1dB T2: -7dB T3: -15dB T4: -4.5dB T5: 1dB |  | Offset during T1: +0.8dB T2: +0.8dB T3: -0.8dB T4: -0.8dB T5: +0.8dB |  | SNR during T1: 1.8dB T2: -6.2dB T3: -15.8dB T4: -5.3dB T5: 1.8dB For testing of a UE which supports 4RX on all bands, the SNR during T3 and T4 is modified as specified in clause D.4.1.1 |  |
| 6.5.1.3 NR SA FR1 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in DRX mode |  |  | Same as 6.5.1.1 |  | Same as 6.5.1.1 |  | Same as 6.5.1.1 |  |
| 6.5.1.4 NR SA FR1 radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in DRX mode |  |  | Same as 6.5.1.2 |  | Same as 6.5.1.2 |  | Same as 6.5.1.2 |  |
| 6.5.1.5 NR SA FR1 radio link monitoring out-of-sync test for PCell configured with CSI-RS-based RLM RS in non-DRX mode |  |  | Same as 6.5.1.1 |  | Same as 6.5.1.1 |  | Same as 6.5.1.1 |  |
| 6.5.1.6 NR SA FR1 radio link monitoring in-sync test for PCell configured with CSI-RS-based RLM RS in non-DRX mode |  |  | Same as 6.5.1.2 |  | Same as 6.5.1.2 |  | Same as 6.5.1.2 |  |
| 6.5.1.7 NR SA FR1 radio link monitoring out-of-sync test for PCell configured with CSI-RS-based RLM RS in DRX mode |  |  | Same as 6.5.1.1 |  | Same as 6.5.1.1 |  | Same as 6.5.1.1 |  |
| 6.5.1.8 NR SA FR1 radio link monitoring in-sync test for PCell configured with CSI-RS-based RLM RS in DRX mode |  |  | Same as 6.5.1.2 |  | Same as 6.5.1.2 |  | Same as 6.5.1.2 |  |
| 6.5.2.1 NR SA FR1 interruptions during measurements on deactivated NR SCC |  |  | During T1: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB |  | During T1: 0dB 0dB 0dB 0dB |  | During T1: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB |  |
| 6.5.3.1 NR SA FR1 SCell activation and deactivation of known SCell in non-DRX for 160ms SCell measurement cycle |  |  | During T1: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB During T2: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB During T3: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB |  | During T1: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB During T2: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB During T3: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB |  |
| 6.5.3.2 NR SA FR1 SCell activation and deactivation of known SCell in non-DRX for 640ms SCell measurement cycle |  |  | Same as 6.5.3.1 |  | Same as 6.5.3.1 |  | Same as 6.5.3.1 |  |
| 6.5.3.3 NR SA FR1 SCell activation and deactivation of unknown SCell in non-DRX |  |  | Same as 6.5.3.1 |  | Same as 6.5.3.1 |  | Same as 6.5.3.1 |  |
| 6.5.5.1 NR SA FR1 SSB-based beam failure detection and link recovery in non-DRX |  |  | q0 SSB SNR during: T1: 5dB T2: -3dB T3: -12dB T4: -12dB T5: -12dB q1 SSB during T1: Noc1: -98dBm/15kHz Ês1 / Noc1: -10dB q1 SSB during T2: Noc1: -98dBm/15kHz Ês1 / Noc1: -10dB q1 SSB during T3, T4 and T5: Noc1: -98dBm/15kHz Ês1 / Noc1: +10dB |  | Offset during: T1: +0.8dB T2: +0.8dB T3: -0.8dB T4: -0.8dB T5: -0.8dB q1 SSB during T1: 0dB -0.2dB q1 SSB during T2: 0dB -0.2dB q1 SSB during T3, T4 and T5: 0dB +0.2dB |  | SNR during: T1: +5.8dB T2: -2.2dB T3: -12.8dB T4: -12.8dB T5: -12.8dB For testing of a UE which supports 4RX on all bands, the SNR during T3, T4 and T5 is modified as specified in clause D.4.1.1 q1 SSB during T1: Noc1: -98dBm/15kHz Ês1 / Noc1: -10.2dB q1 SSB during T2: Noc1: -98dBm/15kHz Ês1 / Noc1: -10.2dB q1 SSB during T3, T4 and T5: Noc1: -98dBm/15kHz Ês1 / Noc1: +10.2dB |  |
| 6.5.5.2 NR SA FR1 SSB-based beam failure detection and link recovery in DRX |  |  | Same as 6.5.5.1 |  | Same as 6.5.5.1 |  | Same as 6.5.5.1 |  |
| 6.5.5.3 NR SA FR1 CSI-RS-based beam failure detection and link recovery in non-DRX |  |  | q0 CSI-RS SNR during: T1: 5dB T2: -3dB T3: -12dB T4: -12dB T5: -12dB q1 CSI-RS during T1: Noc1: -98dBm/15kHz Ês1 / Noc1: -10dB q1 CSI-RS during T2: Noc2: -98dBm/15kHz Ês1 / Noc1: -10dB q1 CSI-RS during T3, T4 and T5: Noc2: -98dBm/15kHz Ês1 / Noc1: +10dB |  | Offset during: T1: +0.8dB T2: +0.8dB T3: -0.8dB T4: -0.8dB T5: -0.8dB q1 CSI-RS during T1: 0dB -0.2dB q1 CSI-RS during T2: 0dB -0.2dB q1 CSI-RS during T3, T4 and T5: 0dB +0.2dB |  | SNR during: T1: +5.8dB T2: -2.2dB T3: -12.8dB T4: -12.8dB T5: -12.8dB For testing of a UE which supports 4RX on all bands, the SNR during T3, T4 and T5 is modified as specified in clause D.4.1.1 q1 CSI-RS during T1: Noc1: -98dBm/15kHz Ês1 / Noc1: -10.2dB q1 CSI-RS during T2: Noc1: -98dBm/15kHz Ês1 / Noc1: -10.2dB q1 CSI-RS during T3, T4 and T5: Noc1: -98dBm/15kHz Ês1 / Noc1: +10.2dB |  |
| 6.5.5.4 NR SA FR1 CSI-RS-based beam failure detection and link recovery in DRX |  |  | Same as 6.5.5.3 |  | Same as 6.5.5.3 |  | Same as 6.5.5.3 |  |
| 6.5.5.5 NR SA FR1 Scell CSI-RS-based beam failure detection and SSB-based link recovery in non-DRX |  |  | On the SCell: q0 CSI-RS SNR during: T1: 5dB T2: -3dB T3: -12dB T4: -12dB T5: -12dB q1 SSB during T1: Noc1: -98dBm/15kHz Ês1 / Noc1: -10dB q1 SSB during T2: Noc1: -98dBm/15kHz Ês1 / Noc1: -10dB q1 SSB during T3, T4 and T5: Noc1: -98dBm/15kHz Ês1 / Noc1: +10dB |  | On the SCell: Offset during: T1: +0.8dB T2: +0.8dB T3: -0.8dB T4: -0.8dB T5: -0.8dB q1 SSB during T1: 0dB -0.2dB q1 SSB during T2: 0dB -0.2dB q1 SSB during T3, T4 and T5: 0dB +0.2dB |  | On the SCell: T1: +5.8dB T2: -2.2dB T3: -12.8dB T4: -12.8dB T5: -12.8dB For testing of a UE which supports 4RX on all bands, the SNR during T3, T4 and T5 is modified as specified in clause D.4.1.1 q1 SSB during T1: Noc1: -98dBm/15kHz Ês1 / Noc1: -10.2dB q1 SSB during T2: Noc1: -98dBm/15kHz Ês1 / Noc1: -10.2dB q1 SSB during T3, T4 and T5: Noc1: -98dBm/15kHz Ês1 / Noc1: +10.2dB |  |
| 6.5.5.6 NR SA FR1 Scell CSI-RS-based beam failure detection and SSB-based link recovery in DRX |  |  | Same as 6.5.5.5 |  | Same as 6.5.5.5 |  | Same as 6.5.5.5 |  |
| 6.5.6.1.1 NR SA FR1-FR1 DCI-based DL active BWP switch in non-DRX |  |  | During T1: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB During T2: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB During T3: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB |  | During T1: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB During T2: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB During T3: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB |  |
| 6.5.6.1.2 NR SA FR1 DCI-based DL active BWP switch in non-DRX |  |  | During T1: Noc1: -104dBm/15kHz Ês1 / Noc1: +17dB During T2: Noc1: -104dBm/15kHz Ês1 / Noc1: +17dB During T3: Noc1: -104dBm/15kHz Ês1 / Noc1: +17dB |  | During T1: 0dB 0dB During T2: 0dB 0dB During T3: 0dB 0dB |  | During T1: Noc1: -104dBm/15kHz Ês1 / Noc1: +17dB During T2: Noc1: -104dBm/15kHz Ês1 / Noc1: +17dB During T3: Noc1: -104dBm/15kHz Ês1 / Noc1: +17dB |  |
| 6.5.6.2.1 NR SA FR1 RRC-based DL active BWP switch in non-DRX |  |  | During T1: Noc1: -104dBm/15kHz Ês1 / Noc1: +17dB |  | During T1: 0dB 0dB |  | During T1: Noc1: -104dBm/15kHz Ês1 / Noc1: +17dB |  |
| 6.6.1.1 SA event triggered reporting tests without gap under non-DRX |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +4.00dB |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +4.00dB |  |
| 6.6.1.2 SA event triggered reporting tests without gap under DRX |  |  | Same as 6.6.1.1 |  | Same as 6.6.1.1 |  | Same as 6.6.1.1 |  |
| 6.6.1.3 SA event triggered reporting tests with per-UE gaps under non-DRX |  |  | Same as 64.6.1.1 |  | Same as 6.6.1.1 |  | Same as 6.6.1.1 |  |
| 6.6.1.4 SA event triggered reporting tests with per-UE gaps under DRX |  |  | Same as 6.6.1.1 |  | Same as 6.6.1.1 |  | Same as 6.6.1.1 |  |
| 6.6.1.5 SA event triggered reporting tests without gap under non-DRX with SSB index reading |  |  | Same as 6.6.1.1 |  | Same as 6.6.1.1 |  | Same as 6.6.1.1 |  |
| 6.6.1.6 SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading |  |  | Same as 6.6.1.1 |  | Same as 6.6.1.1 |  | Same as 6.6.1.1 |  |
| 6.6.1.7 NR SA FR1 event-triggered reporting without gap in DRX for UE configured with highSpeedMeasFlag-r16 |  |  | Same as 6.6.1.1 |  | Same as 6.6.1.1 |  | Same as 6.6.1.1 |  |
| 6.5.3.1 |  |  | During T1: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB During T2: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB During T3: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB |  | During T1: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB During T2: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB During T3: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB |  |
| 6.5.3.2 |  |  | Same as 6.5.3.1 |  | Same as 6.5.3.1 |  | Same as 6.5.3.1 |  |
| 6.5.3.3 |  |  | Same as 6.5.3.1 |  | Same as 6.5.3.1 |  | Same as 6.5.3.1 |  |
| 6.5.7.1 NR SA FR1 DL Interruptions at switching between two uplink carriers in FDD-TDD CA |  |  | Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc: +17dB Ês2 / Noc: +17dB Reported CSI-RSRP values ± 10dB |  | -1.10dB -1.10dB 0dB 0dB Via mapping |  | Noc1: -105.10dBm/15kHz Noc2: -105.10dBm/15kHz Ês1 / Noc: +17dB Ês2 / Noc: +17dB CSI-RS#0: RSRP_64 to RSRP_84 CSI-RS#1: RSRP_67 to RSRP_87 |  |
| 6.5.7.2 NR SA FR1 DL Interruptions at switching between two uplink carriers in TDD-TDD CA |  |  | Same as 6.5.7.1 |  | Same as 6.5.7.1 |  | Noc1: -105.10dBm/15kHz Noc2: -105.10dBm/15kHz Ês1 / Noc: +17dB Ês2 / Noc: +17dB CSI-RS#0: RSRP_67 to RSRP_87 CSI-RS#1: RSRP_67 to RSRP_87 |  |
| 6.6.2.1 NR SA FR1-FR1 event-triggered reporting in non-DRX |  |  | During T1: Freq 1 Noc: -98dBm/15kHz Freq 2 Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: -infinity During T2: Freq 1 Noc: -98dBm/15kHz Freq 2 Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +7.00dB |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB |  | During T1: Freq 1 Noc: -98dBm/15kHz Freq 2 Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: -infinity During T2: Freq 1 Noc: -98dBm/15kHz Freq 2 Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +7.00dB |  |
| 6.6.2.2 NR SA FR1-FR1 event-triggered reporting in DRX |  |  | Same as 6.6.2.1 |  | Same as 6.6.2.1 |  | Same as 6.6.2.1 |  |
| 6.6.2.5 NR SA FR1-FR1 event-triggered reporting in non-DRX with SSB time index detection |  |  | Same as 6.6.2.1 |  | Same as 6.6.2.1 |  | Same as 6.6.2.1 |  |
| 6.6.2.6 NR SA FR1-FR1 event-triggered reporting in DRX with SSB time index detection |  |  | Same as 6.6.2.1 |  | Same as 6.6.2.1 |  | Same as 6.6.2.1 |  |
| 6.6.3.1 NR SA FR1 – E-UTRAN event-triggered reporting in non-DRX |  |  | During T1: Freq 1 Noc: -106dBm/15kHz Freq 2 Noc: -106dBm/15kHz Ês1 / Noc: +18.00dB Ês2 / Noc: -infinity During T2: Freq 1 Noc: -106dBm/15kHz Freq 2 Noc: -106dBm/15kHz Ês1 / Noc: -2dB Ês2 / Noc: +19.00dB |  | During T1: 0dB 0dB +1.65dB 0dB During T2: 0dB 0dB -1.65dB 1.65dB |  | During T1: Freq 1 Noc: -106dBm/15kHz Freq 2 Noc: -106dBm/15kHz Ês1 / Noc: +19.65dB Ês2 / Noc: -infinity During T2: Freq 1 Noc: -106dBm/15kHz Freq 2 Noc: -106dBm/15kHz Ês1 / Noc: -3.65dB Ês2 / Noc: +20.65dB |  |
| 6.6.3.2 NR SA FR1 – E-UTRAN event-triggered reporting in DRX |  |  | Same as 6.6.3.1 |  | Same as 6.6.3.1 |  | Same as 6.6.3.1 |  |
| 6.6.3.3 NR SA FR1 – E-UTRAN event-triggered reporting in DRX for UE configured with highSpeedMeasFlag-r16 |  |  | During T1: Freq 1 Noc: -106dBm/15kHz Freq 2 Noc: -106dBm/15kHz Ês1 / Noc: +18.00dB Ês2 / Noc: -infinity During T2: Freq 1 Noc: -106dBm/15kHz Freq 2 Noc: -106dBm/15kHz Ês1 / Noc: -2dB Ês2 / Noc: +19.00dB |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB |  | During T1: Freq 1 Noc: -106dBm/15kHz Freq 2 Noc: -106dBm/15kHz Ês1 / Noc: +18.00dB Ês2 / Noc: -infinity During T2: Freq 1 Noc: -106dBm/15kHz Freq 2 Noc: -106dBm/15kHz Ês1 / Noc: -2dB Ês2 / Noc: +19.00dB |  |
| 6.6.4.1 NR SA FR1 SSB-based L1-RSRP measurement in non-DRX |  |  | During T1: Noc: -98dBm/15kHz Ês0 / Noc: +0.00dB Ês1 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.00dB Reported SSB#0 SS-RSRP values ± 10dB Reported SSB#1 SS-RSRP values ± 10dB Reported Differential SS-RSRP values ± 3dB |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0.5dB Via mapping |  | During T1: Noc: -94.65dBm/15kHz Ês0 / Noc: +0.00dB Ês1 / Noc: -infinity During T2: Noc: -94.65dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.50dB For configuration 1,2,4,5 SSB#1: RSRP_55 to RSRP_75 SSB#0: DIFFRSRP_0 to DIFFRSRP_3 For configuration 3,6 SSB#1: RSRP_58 to RSRP_78 SSB#0: DIFFRSRP_0 to DIFFRSRP_3 |  |
| 6.6.4.2 NR SA FR1 SSB-based L1-RSRP measurement in DRX |  |  | Same as 6.6.4.1 |  | Same as 6.6.4.1 |  | Same as 6.6.4.1 |  |
| 6.6.4.3 NR SA FR1 CSI-RS-based L1-RSRP measurement in non-DRX |  |  | Noc: -98dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.00dB Reported CSI-RSRP values ± 10dB Reported Differential CSI-RSRP values ± 3dB |  | 0dB 0dB 0.5dB Via mapping |  | Noc: -94.65dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.50dB For configuration 1,2,4,5 CSI-RS#1: RSRP_55 to RSRP_75 CSI-RS#0: DIFFRSRP_0 to DIFFRSRP_3 For configuration 3,6 CSI-RS#1: RSRP_58 to RSRP_78 CSI-RS#0: DIFFRSRP_0 to DIFFRSRP_3 |  |
| 6.6.4.4 NR SA FR1 CSI-RS-based L1-RSRP measurement in DRX |  |  | Same as 6.6.4.3 |  | Same as 6.6.4.3 |  | Same as 6.6.4.3 |  |
| 6.6.4.5 NR SA FR1 SSB-based L1-RSRP measurement in DRX for UE configured with highSpeedMeasFlag-r16 |  |  | Same as 6.6.4.1 |  | Same as 6.6.4.1 |  | Same as 6.6.4.1 |  |
| 6.6.5.1 NR SA FR1 – UTRAN event-triggered reporting in non-DRX |  |  | During T1: Noc: -106dBm/15kHz Ês / Noc: 18dB Ioc: -70dBm/3.84MHz Îor / Ioc: -infinity CPICH Ec / Ior: -10dB SCH Ec / Ior: -12dB During T2: Noc: -106dBm/15kHz Ês / Noc: -2dB Ioc: -70dBm/3.84MHz Îor / Ioc: -1.8dB CPICH Ec / Ior: -10dB SCH Ec / Ior: -12dB |  | During T1: 0dB 0dB 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB 0dB 0dB |  | During T1: Noc: -106dBm/15kHz Ês / Noc: 18dB Ioc: -70dBm/3.84MHz Îor / Ioc: -infinity CPICH Ec / Ior: -10dB SCH Ec / Ior: -12dB During T2: Noc: -106dBm/15kHz Ês / Noc: -2dB Ioc: -70dBm/3.84MHz Îor / Ioc: -1.8dB CPICH Ec / Ior: -10dB SCH Ec / Ior: -12dB |  |
| 6.6.8.1 NR SA FR1 CSI-RS based CMR and no dedicated IMR L1-SINR measurement in DRX |  |  | Noc: -94.65dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.00dB Reported CSI-SINR values ±5.5dB Reported Differential CSI-SINR values ±4.5dB |  | 0dB 0dB 0dB Via mapping |  | Noc: -94.65dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.00dB For configuration 1,2,3,4,5,6 CSI-RS#1: SINR_41 to SINR_64 CSI-RS#0: DIFFSINR_0 to DIFFSINR_7 |  |
| 6.6.8.2 NR SA FR1 SSB based CMR and dedicated IMR L1-SINR measurement in non-DRX |  |  | Noc: -94.65dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.00dB Reported SS-SINR values ±4dB Reported Differential SS-SINR values ±3dB |  | 0dB 0.5dB 0dB Via mapping |  | Noc: -94.65dBm/15kHz Ês0 / Noc: 0.50dB Ês1 / Noc: +3.00dB For configuration 1,2,3,4,5,6 SSB#1: SINR_44 to SINR_61 SSB#0: DIFFSINR_0 to DIFFSINR_5 |  |
| 6.6.8.3 NR SA FR1 CSI-RS based CMR and dedicated IMR L1-SINR measurement in non-DRX |  |  | Noc: -94.65dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.00dB Reported CSI-SINR values ±4.5dB Reported Differential CSI-SINR values ±3.5dB |  | 0dB 0dB 0dB Via mapping |  | Noc: -94.65dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.00dB For configuration 1,2,3,4,5,6 CSI-RS#1: SINR_43 to SINR_62 CSI-RS#0: DIFFSINR_0 to DIFFSINR_6 |  |
| 6.6.9.1 NR SA FR1 Idle mode CA/DC measurement for FR1 |  |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +7dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +7dB Ês2 / Noc2: 0dB During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +7dB Ês2 / Noc2: 0dB Reported SS-RSRP and SS-RSRQ |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB Via mapping |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +7dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +7dB Ês2 / Noc2: 0dB During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +7dB Ês2 / Noc2: 0dB Configuration 1 and 2: Cell 1: RSRP_54 to RSRP_77 RSRQ_55 to RSRQ_72 Cell 2: RSRP_47 to RSRP_70 RSRQ_51 to RSRQ_67 Configuration 3: Cell 1: RSRP_57 to RSRP_80 RSRQ_55 to RSRQ_72 Cell 2: RSRP_50 to RSRP_73 RSRQ_51 to RSRQ_67 |  |
| 6.6.15.1 NR SA FR1 Idle Mode measurements of inter-RAT CA candidate cells for early reporting |  |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +5dB Ês2 / Noc2: +5dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +5dB Ês2 / Noc2: +5dB During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +5dB Ês2 / Noc2: -3dB During T4: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +5dB Ês2 / Noc2: +8dB During T5: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +5dB Ês2 / Noc2: +5dB Thresh_X_HighP 48dB Reported SS-RSRP and SS-RSRQ |  | FFS |  | FFS |  |
| 6.7.1.1.1 NR SA FR1 SS-RSRP absolute measurement accuracy |  |  | TEST CONFIGURATION 1, 2, |  |  |  |  |  |
|  |  |  | Test 1: Noc: -106dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported RSRP values: ±4.5dB Test 2: Noc: -88dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported RSRP values: ±8dB Test 3: Noc: -116dBm/15kHz + ΔBG_offset Ês1 / Noc: +3.0dB Ês2 / Noc: -1.0dB Reported RSRP values: ±6dB |  | Test 1: -1.5dB 0dB +0.4dB Via mapping Test 2: 0dB 0dB +0.4dB Via mapping Test 3: 0dB 0dB +0.2dB Via mapping |  | Test 1: Noc: -107.5dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.4dB RSRP_44 to RSRP_56 Test 2: Noc: -88dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.4dB RSRP_60 to RSRP_79 Test 3: Noc: -116dBm/15kHz + ΔBG_offset Ês1 / Noc: +3.0dB Ês2 / Noc: -0.8dB RSRP_34 to RSRP_46 RSRP_34 to RSRP_46 RSRP_35 to RSRP_47 RSRP_35 to RSRP_47 RSRP_36 to RSRP_48 RSRP_37 to RSRP_49 RSRP_37 to RSRP_49 depending on operating band |  |
|  |  |  | TEST CONFIGURATION 3 |  |  |  |  |  |
|  |  |  | Test 1: Noc: -110dBm/30kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported RSRP values: ±4.5dB Test 2: Noc: -91dBm/30kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported RSRP values: ±8dB Test 3: Noc: -113dBm/30kHz + ΔBG_offset Ês1 / Noc: +3.0dB Ês2 / Noc: -1.0dB Reported RSRP values: ±6dB |  | Test 1: -0.8dB 0dB +0.4dB Via mapping Test 2: 0dB 0dB +0.4dB Via mapping Test 3: 0dB 0dB +0.2dB Via mapping |  | Test 1: Noc: -110.8dBm/30kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.4dB RSRP_41 to RSRP_53 Test 2: Noc: -91dBm/30kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.4dB RSRP_57 to RSRP_76 Test 3: Noc: -113dBm/30kHz + ΔBG_offset Ês1 / Noc: +3.0dB Ês2 / Noc: -0.8dB RSRP_37 to RSRP_49 RSRP_37 to RSRP_49 RSRP_38 to RSRP_50 RSRP_38 to RSRP_50 RSRP_39 to RSRP_51 RSRP_40 to RSRP_52 RSRP_40 to RSRP_52 depending on operating band |  |
| The derivation of the RSRP values takes into account the uncertainty in Cell 2 SS-RSRP from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The SS-RSRP values given above are for normal conditions. In all cases the RSRP values are 4.5dB wider at each end for extreme conditions. |  |  |  |  |  |  |  |  |
| 6.7.1.1.2 NR SA FR1 SS-RSRP relative measurement accuracy |  |  | Test 1: Noc: Test configuration 1, 2:-106dBm/15kHz Test configuration 3:-110dBm/30kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported rel. SS-RSRP values: ±3dB Test 2: Noc: -88dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported rel. SS-RSRP values: ±3dB Test 3: Noc: -116dBm/15kHz + ΔBG_offset Ês1 / Noc: +3.0dB Ês2 / Noc: -1.0dB Reported re. SS-RSRP values: ±3dB |  | Test 1: 0 dB 0 dB 0 dB +1.0dB Via mapping Test 2: 0dB 0dB +1.0dB Via mapping Test 3: 0dB 0dB +1.0dB Via mapping |  | Test 1: Noc: -106 dBm/15kHz Noc: -110 dBm/30kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +2.0dB RSRP_x-8 to RSRP_x-1 Test 2: Noc: -88dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +2.0dB RSRP_x-8 to RSRP_x-1 Test 3: Noc: -116dBm/15kHz + ΔBG_offset Ês1 / Noc: +3.0dB Ês2 / Noc: 0dB RSRP_x-7 to RSRP_x+1 |  |
| The derivation of the SS-RSRP values takes into account the uncertainty in Cell 1 and Cell 2 RSRP from Noc, Ês1 / Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The RSRP values given above are for both normal and extreme conditions. |  |  |  |  |  |  |  |  |
| 6.7.1.2.1 NR SA FR1-FR1 SS-RSRP absolute measurement accuracy | TEST CONFIGURATION 1, 2, 4, 5 |  |  |  |  |  |  |  |
|  | Test 1: Noc1: -94.65 dBm/15kHz Noc2: -94.65 dBm/15kHz Ês1 / Noc1: +10.0dB Ês2 / Noc2: +10.0dB Reported RSRP values: ±8dB Test 2: Noc1: -107dBm/15kHz + ΔBG_offset Noc2: -118dBm/15kHz + ΔBG_offset Ês1 / Noc1: +13.0dB Ês2 / Noc2: -3.0dB Reported RSRP values: ±4.5dB |  |  |  | Test 1: 0dB 0dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping |  | Test 1: Noc1: -94.65dBm/15kHz Noc2: -94.65dBm/15kHz Ês1 / Noc1: +10.0dB Ês2 / Noc2: +10.0dB RSRP_62 to RSRP_81 Test 2: Noc1: -107dBm/15kHz + ΔBG_offset Noc2: -118dBm/15kHz + ΔBG_offset Ês1 / Noc1: +13.0dB Ês2 / Noc2: -3.0dB RSRP_32 to RSRP_45 RSRP_33 to RSRP_45 RSRP_33 to RSRP_46 RSRP_34 to RSRP_46 RSRP_34 to RSRP_47 RSRP_35 to RSRP_48 RSRP_36 to RSRP_48 depending on operating band |  |
|  | TEST CONFIGURATION 3, 6 |  |  |  |  |  |  |  |
|  | Test 1: Noc1: -94.65 dBm/15kHz Noc2: -94.65 dBm/15kHz Ês1 / Noc1: +10.0dB Ês2 / Noc2: +10.0dB Reported RSRP values: ±8dB Test 2: Noc1: -107dBm/15kHz + ΔBG_offset Noc1: -118dBm/15kHz + ΔBG_offset Ês1 / Noc1: +13.0dB Ês2 / Noc2: -3.0dB Reported RSRP values: ±4.5dB |  |  |  | Test 1: -1.35dB -1.35dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping |  | Test 1: Noc1: -96dBm/15kHz Noc2: -96dBm/15kHz Ês1 / Noc1: +10.0dB Ês2 / Noc2: +10.0dB RSRP_64 to RSRP_83 Test 2: Noc: -107dBm/15kHz + ΔBG_offset Noc1: -118dBm/15kHz + ΔBG_offset Ês1 / Noc1: +13.0dB Ês2 / Noc2: -3.0dB RSRP_35 to RSRP_48 RSRP_36 to RSRP_48 RSRP_36 to RSRP_49 RSRP_37 to RSRP_49 RSRP_37 to RSRP_50 RSRP_38 to RSRP_51 RSRP_39 to RSRP_51 depending on operating band |  |
| The derivation of the RSRP values takes into account the uncertainty in Cell 2 SS-RSRP from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. The SS-RSRP values given above are for normal conditions. For extreme conditions, the RSRP values are 3.0 dB wider at each end for Test 1, and 4.5 dB wider at each for Test 2. |  |  |  |  |  |  |  |  |
| 6.7.1.2.2 NR SA FR1-FR1 SS-RSRP relative measurement accuracy | Test 1: Noc1: -94.65 dBm/15kHz Noc2: -94.65 dBm/15kHz Ês1 / Noc1: +10.0dB Ês2 / Noc2: +10.0dB Reported relative RSRP values: ±4.5dB Test 2: Noc1: -107dBm/15kHz + ΔBG_offset Noc1: -118dBm/15kHz + ΔBG_offset Ês1 / Noc1: +13.0dB Ês2 / Noc2: -3.0dB Reported relative RSRP values: ±4.5dB |  |  |  | Test 1: 0dB 0dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping |  | Test 1: Noc1: -94.65dBm/15kHz Noc2: -94.65dBm/15kHz Ês1 / Noc1: +10.0dB Ês2 / Noc2: +10.0dB RSRP_x-7 to RSRP_x+7 Test 2: Noc: -107dBm/15kHz + ΔBG_offset Noc1: -118dBm/15kHz + ΔBG_offset Ês1 / Noc1: +13.0dB Ês2 / Noc2: -3.0dB RSRP_x-31 to RSRP_x-18 |  |
| The derivation of the SS-RSRP values takes into account the uncertainty in Cell 1 and Cell 2 RSRP from Noc, Ês1 / Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The SS-RSRP values given above are for normal conditions. For extreme conditions, the RSRP values are 1.5dB wider at each end. |  |  |  |  |  |  |  |  |
| 6.7.2.1 NR SA FR1 SS-RSRQ measurement accuracy | TEST CONFIGURATION 1, 2, 4, 5 |  |  |  |  |  |  |  |
|  | Test 1: Noc: -85dBm/15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: +3.0dB Reported RSRQ values: ±2.5dB Test 2: Noc: -101dBm/15kHz Ês1 / Noc: -2.9dB Ês2 / Noc: -2.9dB Reported RSRQ values: ±3.5dB Test 3: Noc: -114dBm/15kHz + ΔBG_offset Ês1 / Noc: -4.0dB Ês2 / Noc: -4.0dB Reported RSRQ values: ±3.5dB |  |  |  | Test 1: -1.5dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0dB Via mapping |  | Test 1: Noc: -86.5dBm/15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: +3.0dB RSRQ_52 to RSRQ_62 Test 2: Noc: -101dBm/15kHz Ês1 / Noc: -2.9dB Ês2 / Noc: -2.9dB RSRQ_46 to RSRQ_60 Test 3: Noc: -114dBm/15kHz + ΔBG_offset Ês1 / Noc: -3.5dB Ês2 / Noc: -3.5dB RSRQ_44 to RSRQ_59 |  |
|  | TEST CONFIGURATION 3, 6 |  |  |  |  |  |  |  |
|  | Test 1: Noc: -91dBm/15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: +3.0dB Reported RSRQ values: ±2.5dB Test 2: N/A Test 3: Noc: -114dBm/15kHz + ΔBG_offset Ês1 / Noc: -4.0dB Ês2 / Noc: -4.0dB Reported RSRQ values: ±3.5dB |  |  |  | Test 1: -1.6dB 0dB 0dB Via mapping Test 2: N/A Test 3: 0dB 0dB 0dB Via mapping |  | Test 1: Noc: -92.6dBm/15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: +3.0dB RSRQ_52 to RSRQ_62 Test 2: N/A Test 3: Noc: -114dBm/15kHz + ΔBG_offset Ês1 / Noc: -3.5dB Ês2 / Noc: -3.5dB RSRQ_44 to RSRQ_59 |  |
| The derivation of the RSRQ values takes into account the uncertainty in Cell 2 SS-RSRQ from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The SS-RSRQ values given above are for normal conditions. For extreme conditions, the SS-RSRQ values are 1.5dB wider at each end for Test 1, and 0.5 dB wider at each for Tests 2 and 3. |  |  |  |  |  |  |  |  |
| 6.7.2.2.1 NR SA FR1-FR1 SS-RSRQ absolute measurement accuracy | TEST CONFIGURATION 1, 2, 4, 5 |  |  |  |  |  |  |  |
|  | Test 1: Noc1: -80.18 dBm/15kHz Noc2: -80.18 dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês1 / Noc1: 3dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB |  |  |  | Test 1: -1.5dB -1.5dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0dB 0dB Via mapping |  | Test 1: Noc1: -81.68dBm/15kHz Noc2: -81.68dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB RSRQ_52 to RSRQ_62 Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB RSRQ_52 to RSRQ_62 Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês1 / Noc1: 3dB Ês2 / Noc2: -1.75dB RSRQ_52 to RSRQ_62 |  |
|  | TEST CONFIGURATION 3, 6 |  |  |  |  |  |  |  |
|  | Test 1: Noc1: -86.27 dBm/15kHz Noc2: -86.27 dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês1 / Noc1: 3dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB |  |  |  | Test 1: -1.53dB -1.53dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0dB 0dB Via mapping |  | Test 1: Noc1: -87.8dBm/15kHz Noc2: -87.8dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB RSRQ_52 to RSRQ_62 Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB RSRQ_52 to RSRQ_62 Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês1 / Noc1: 3dB Ês2 / Noc2: -1.75dB RSRQ_52 to RSRQ_62 |  |
| The derivation of the RSRQ values takes into account the uncertainty in Cell 2 SS-RSRQ from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. The SS-RSRQ values given above are for normal conditions. For extreme conditions, the RSRQ values are 1.5dB wider at each end. |  |  |  |  |  |  |  |  |
| 6.7.2.2.2 NR SA FR1-FR1 SS-RSRQ relative measurement accuracy | TEST CONFIGURATION 1, 2, 4, 5 |  |  |  |  |  |  |  |
|  | Test 1: Noc1: -80.18 dBm/15kHz Noc2: -80.18 dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±3dB Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±3dB Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês1 / Noc1: 3dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±3dB |  |  |  | Test 1: -1.5dB -1.5dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0dB 0dB Via mapping |  | Test 1: Noc1: -81.68dBm/15kHz Noc2: -81.68dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB RSRQ_x-7 to RSRQ_x+7 Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB RSRQ_x-7 to RSRQ_x+7 Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês1 / Noc1: 3dB Ês2 / Noc2: -1.75dB RSRQ_x-11 to RSRQ_x+2 |  |
|  | TEST CONFIGURATION 3, 6 |  |  |  |  |  |  |  |
|  | Test 1: Noc1: -86.27 dBm/15kHz Noc2: -86.27 dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±3dB Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês1 / Noc1: 3dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±3dB |  |  |  | Test 1: -1.53dB -1.53dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0dB 0dB Via mapping |  | Test 1: Noc1: -87.8dBm/15kHz Noc2: -87.8dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB RSRQ_x-7 to RSRQ_x+7 Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB RSRQ_x-7 to RSRQ_x+7 Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês1 / Noc1: 3dB Ês2 / Noc2: -1.75dB RSRQ_x-11 to RSRQ_x+2 |  |
| The derivation of the RSRQ values takes into account the uncertainty in Cell 1 and Cell 2 SS-RSRQ from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. The SS-RSRQ values given above are for normal conditions. For extreme conditions, the RSRQ values are 1dB wider at each end. |  |  |  |  |  |  |  |  |
| 6.7.3.1 NR SA FR1 SS-SINR measurement accuracy | TEST CONFIGURATION 1, 2, 4, 5 |  |  |  |  |  |  |  |
|  | Test 1: Noc: -93dBm/15kHz Ês1 / Noc: +4.54dB Ês2 / Noc: +4.54dB Reported SINR values: ±3.5dB Test 2: Noc: -116dBm/15kHz+ ΔBG_offset Ês1 / Noc: -4dB Ês2 / Noc: -4dB Reported SINR values: ±3.5dB |  |  |  | Test 1: 0dB 0dB 0dB Via mapping Test 2: 0dB 0.5dB 0.5dB Via mapping |  | Test 1: Noc: -93dBm/15kHz Ês1 / Noc: +4.54dB Ês2 / Noc: +4.54dB SINR_31 to SINR_49 Test 2: Noc: -116dBm/15kHz+ ΔBG_offset Ês1 / Noc: -3.5dB Ês2 / Noc: -3.5dB SINR_28 to SINR_45 |  |
|  | TEST CONFIGURATION 3, 6 |  |  |  |  |  |  |  |
|  | Test 1: Noc: -93dBm/15kHz Ês1 / Noc: +4.54dB Ês2 / Noc: +4.54dB Reported SINR values: ±3.5dB Test 2: Noc: -116dBm/15kHz+ ΔBG_offset Ês1 / Noc: -4dB Ês2 / Noc: -4dB Reported SINR values: ±3.5dB |  |  |  | Test 1: -0.2dB 0dB 0dB Via mapping Test 2: 0dB 0.5dB 0.5dB Via mapping |  | Test 1: Noc: -93.2dBm/15kHz Ês1 / Noc: +4.54dB Ês2 / Noc: +4.54dB SINR_31 to SINR_49 Test 2: Noc: -116dBm/15kHz+ ΔBG_offset Ês1 / Noc: -3.5dB Ês2 / Noc: -3.5dB SINR_28 to SINR_45 |  |
| The derivation of the SINR values takes into account the uncertainty in Cell 2 SS- SINR from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The SS- SINR values given above are for normal conditions. For extreme conditions, the SS- SINR values are 0.5dB wider at each end. |  |  |  |  |  |  |  |  |
| 6.7.3.2.1 NR SA FR1-FR1 SS-SINR absolute measurement accuracy | TEST CONFIGURATION 1, 2, 3, 4, 5, 6 |  |  |  |  |  |  |  |
|  | Test 1: Noc1: -88 dBm/15kHz Noc2: -88 dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported SINR values: ±3dB Test 2: Noc1: -108.5dBm/15kHz Noc2: -108.5dBm/15kHz Ês1 / Noc1: 20dB Ês2 / Noc2: 20dB Reported SINR values: ±3dB Test 3: Noc1: -119.5dBm/15kHz + ΔBG_offset Noc2: -119.5dBm/15kHz + ΔBG_offset Ês1 / Noc1: -4dB Ês2 / Noc2: -4dB Reported SINR values: ±3.5dB |  |  |  | Test 1: 0dB 0dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0.8dB 0.8dB Via mapping |  | Test 1: Noc1: -88 dBm/15kHz Noc2: -88 dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB SINR_35 to SINR_51 Test 2: Noc1: -108.5dBm/15kHz Noc2: -108.5dBm/15kHz Ês1 / Noc1: 20dB Ês2 / Noc2: 20dB SINR_79 to SINR_94 Test 3: Noc1: -119.5dBm/15kHz + ΔBG_offset Noc2: -119.5dBm/15kHz + ΔBG_offset Ês1 / Noc1: -3.2dB Ês2 / Noc2: -3.2dB SINR_32 to SINR_49 |  |
| The derivation of the SINR values takes into account the uncertainty in Cell 2 SS- SINR from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The SS- SINR values given above are for normal conditions. For extreme conditions, the SS- SINR values are 1dB wider at each for Tests 1 and 2 and 0.5dB wider at each end for Test 3. |  |  |  |  |  |  |  |  |
| 6.7.3.2.2 NR SA FR1-FR1 SS-SINR relative measurement accuracy | TEST CONFIGURATION 1, 2, 3, 4, 5, 6 |  |  |  |  |  |  |  |
|  | Test 1: Noc1: -88 dBm/15kHz Noc2: -88 dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported SINR values: ±3.5dB Test 2: Noc1: -108.5dBm/15kHz Noc2: -108.5dBm/15kHz Ês1 / Noc1: 20dB Ês2 / Noc2: 20dB Reported SINR values: ±3.5dB Test 3: Noc1: -119.5dBm/15kHz + ΔBG_offset Noc2: -119.5dBm/15kHz + ΔBG_offset Ês1 / Noc1: -4dB Ês2 / Noc2: -4dB Reported SINR values: ±4dB |  |  |  | Test 1: 0dB 0dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0.8dB 0.8dB Via mapping |  | Test 1: Noc1: -88 dBm/15kHz Noc2: -88 dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB SINR_x-10 to SINR_x+10 Test 2: Noc1: -108.5dBm/15kHz Noc2: -108.5dBm/15kHz Ês1 / Noc1: 20dB Ês2 / Noc2: 20dB SINR_x-10 to SINR_x+10 Test 3: Noc1: -119.5dBm/15kHz + ΔBG_offset Noc2: -119.5dBm/15kHz + ΔBG_offset Ês1 / Noc1: -3.2dB Ês2 / Noc2: -3.2dB SINR_x-11 to SINR_x+11 |  |
| The derivation of the SINR values takes into account the uncertainty in Cell 1 and Cell 2 SS- SINR from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The SS- SINR values given above are for normal conditions. For extreme conditions, the SS- SINR values are 0.5dB wider at each for Tests 1 and 2 and 0dB wider at each end for Test 3. |  |  |  |  |  |  |  |  |
| 6.7.4.1.1 NR SA FR1 SSB based L1-RSRP absolute measurement accuracy |  |  |  |  |  |  |  |  |
| Test Configuration 1,2,4,5 | Test 1: Noc1: -94.65 dBm/15kHz Ês1 / Noc1: 10dB Reported L1-RSRP values: ±8.5dB (±4.5dB additionally for extreme conditions) Test 2: Noc1: -119.5dBm/15kHz + ΔBG_offset Ês1 / Noc1: -3dB Reported L1-RSRP values: ±5dB (±3dB additionally for extreme conditions) |  |  |  | Test 1: 0dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping |  | Test 1: Noc1: -94.65dBm/15kHz Ês1 / Noc1: 10dB RSRP_62 to RSRP_82 Test 2: Noc1: -119.2dBm/15kHz + ΔBG_offset Ês1 / Noc1: -2.2dB RSRP_31 to RSRP_44 RSRP_31 to RSRP_45 RSRP_32 to RSRP_45 RSRP_32 to RSRP_46 RSRP_33 to RSRP_46 RSRP_34 to RSRP_47 RSRP_34 to RSRP_48 depending on operating band |  |
| Test Configuration 3,6 | Test 1: Noc1: -94.65 dBm/15kHz Ês1 / Noc1: 10dB Reported L1-RSRP values: ±8.5dB (±4.5dB additionally for extreme conditions) Test 2: Noc1: -119.5dBm/15kHz + ΔBG_offset Ês1 / Noc1: -3dB Reported L1-RSRP values: ±5dB (±3dB additionally for extreme conditions) |  |  |  | Test 1: -1.35dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping |  | Test 1: Noc1: -96dBm/15kHz Ês1 / Noc1: 10dB RSRP_63 to RSRP_84 Test 2: Noc1: -116.2dBm/15kHz + ΔBG_offset Ês1 / Noc1: -2.2dB RSRP_34 to RSRP_47 RSRP_34 to RSRP_48 RSRP_35 to RSRP_48 RSRP_35 to RSRP_49 RSRP_36 to RSRP_49 RSRP_37 to RSRP_50 RSRP_37 to RSRP_51 depending on operating band |  |
| 6.7.4.1.2 NR SA FR1 SSB based L1-RSRP relative measurement accuracy |  |  |  |  |  |  |  |  |
| Test Configuration 1,2,4,5 | Test 1: Noc1: -94.65 dBm/15kHz Ês1 / Noc1: 10dB Reported L1-RSRP values: ±8.5dB (±1dB additionally for extreme conditions) Test 2: Noc1: -119.5dBm/15kHz + ΔBG_offset Ês1 / Noc1: -3dB Reported L1-RSRP values: ±5dB (±1dB additionally for extreme conditions) |  |  |  | Test 1: 0dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping |  | Test 1: Noc1: -94.65dBm/15kHz Ês1 / Noc1: 10dB RSRP_x-3 to RSRP_x+3 Test 2: Noc1: -119.2dBm/15kHz + ΔBG_offset Ês1 / Noc1: -2.2dB RSRP_x-3 to RSRP_x+3 |  |
| Test Configuration 3,6 | Test 1: Noc1: -94.65 dBm/15kHz Ês1 / Noc1: 10dB Reported L1-RSRP values: ±8.5dB (±1dB additionally for extreme conditions) Test 2: Noc1: -119.5dBm/15kHz + ΔBG_offset Ês1 / Noc1: -3dB Reported L1-RSRP values: ±5dB (±1dB additionally for extreme conditions) |  |  |  | Test 1: -1.35dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping |  | Test 1: Noc1: -96dBm/15kHz Ês1 / Noc1: 10dB RSRP_x-3 to RSRP_x+3 Test 2: Noc1: -116.2dBm/15kHz + ΔBG_offset Ês1 / Noc1: -2.2dB RSRP_x-3 to RSRP_x+3 |  |
| 6.7.4.2.1 NR SA FR1 CSI-RS based L1-RSRP absolute measurement accuracy | Same as 6.7.4.1.1 |  |  |  | Same as 6.7.4.1.1 |  | Same as 6.7.4.1.1 |  |
| 6.7.4.2.2 NR SA FR1 CSI-RS based L1-RSRP relative measurement accuracy | Same as 6.7.4.1.2 |  |  |  | Same as 6.7.4.1.2 |  | Same as 6.7.4.1.2 |  |
| 6.7.5.1 NR SA FR1 – E-UTRAN RSRP absolute measurement accuracy | Test 1: Noc: -91.65dBm/15kHz Ês / Noc: +10.0dB Reported RSRP values: ±9dB (±2dB additionally for extreme conditions) Test 2: Noc: -117dBm/15kHz+ ΔBG_offset Ês / Noc: -4.0dB Reported RSRP values: ±4.5dB (±3.5dB additionally for extreme conditions) |  |  |  | Test 1: 0dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping |  | Test 1: Noc: -91.65dBm/15kHz Ês / Noc: 10dB RSRP_48 to RSRP_70 Test 2: Noc: -117dBm/15kHz+ ΔBG_offset Ês / Noc: -3.2dB RSRP_14 to RSRP_27 RSRP_15 to RSRP_27 RSRP_15 to RSRP_28 RSRP_16 to RSRP_28 RSRP_16 to RSRP_29 RSRP_17 to RSRP_30 RSRP_18 to RSRP_30 Depending on operating band |  |
| 6.7.6.1 NR SA FR1 – E-UTRAN RSRQ absolute measurement accuracy | Test 1: Noc: -83dBm/15kHz Ês / Noc: -1.75dB Reported RSRQ values: ±2.5dB (±1.5dB additionally for extreme conditions) Test 2: Noc: -104.7dBm/15kHz Ês / Noc: -4.0dB Reported RSRQ values: ±3.5dB (±0.5dB additionally for extreme conditions) Test 3: Noc: -119.5dBm/15kHz+ ΔBG_offset Ês / Noc: -4.0dB Reported RSRQ values: ±3.5dB (±0.5dB additionally for extreme conditions) |  |  |  | Test 1: 0dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping Test 3: 0dB 0.8dB Via mapping |  | Test 1: Noc: -83dBm/15kHz Ês / Noc: -1.75dB RSRQ_4 to RSRQ_16 Test 2: Noc: -104.7dBm/15kHz Ês / Noc: -3.2dB RSRQ_0 to RSRQ_16 Test 3: Noc: -119.5dBm/15kHz+ ΔBG_offset Ês / Noc: -3.2dB RSRQ_0 to RSRQ_16 |  |
| 6.7.7.1 NR SA FR1 – E-UTRAN RS-SINR absolute measurement accuracy | Test 1: Noc: -88dBm/15kHz Ês / Noc: -1.75dB Reported RS-SINR values: ±3.0dB (±1.0dB additionally for extreme conditions) Test 2: Noc: -108.5dBm/15kHz Ês / Noc: 20.0dB Reported RS-SINR values: ±3.0dB (±1.0dB additionally for extreme conditions) Test 3: Noc: -119.5dBm/15kHz+ ΔBG_offset Ês / Noc: -4.0dB Reported RS-SINR values: ±3.5dB (±0.5dB additionally for extreme conditions) |  |  |  | Test 1: 0dB 0dB Via mapping Test 2: 0dB 0dB Via mapping Test 3: 0dB 0.8dB Via mapping |  | Test 1: Noc: -88dBm/15kHz Ês / Noc: -1.75dB SINR_35 to SINR_51 Test 2: Noc: -108.5dBm/15kHz Ês / Noc: 20.0dB SINR_79 to SINR_94 Test 3: Noc: -119.5dBm/15kHz+ ΔBG_offset Ês / Noc: -3.2dB SINR_32 to SINR_49 |  |
| 6.7.9.1.1 NR SA FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR absolute measurement accuracy |  |  |  |  |  |  |  |  |
| Test Configuration 1,2,4,5 |  |  | Same as 4.7.7.1 |  | Same as 4.7.7.1 |  | Same as 4.7.7.1 |  |
| Test Configuration 3,6 |  |  | Same as 4.7.7.1 |  | Same as 4.7.7.1 |  | Same as 4.7.7.1 |  |
| 6.7.9.2 NR SA FR1 SSB based CMR and dedicated IMR L1-SINR absolute measurement accuracy |  |  |  |  |  |  |  |  |
| Test Configuration 1,2,4,5 |  |  | Same as 4.7.7.2 |  | Same as 4.7.7.2 |  | Same as 4.7.7.2 |  |
| Test Configuration 3,6 |  |  | Same as 4.7.7.2 |  | Same as 4.7.7.2 |  | Same as 4.7.7.2 |  |
| 6.7.9.3 NR SA FR1 CSI-RS based CMR and dedicated IMR L1-SINR measurement accuracy |  |  |  |  |  |  |  |  |
| Test Configuration 1,2,4,5 |  |  | Same as 4.7.7.3 |  | Same as 4.7.7.3 |  | Same as 4.7.7.3 |  |
| Test Configuration 3,6 |  |  | Same as 4.7.7.3 |  | Same as 4.7.7.3 |  | Same as 4.7.7.3 |  |

Table F.1.3.2-3: Derivation of test requirements for EN-DC FR2 RRM tests

| Test | Minimum requirement in TS 38.133 [6] | Test tolerance (TT) | Test requirement in TS 38.533 |
| --- | --- | --- | --- |
| 5.3.2.2.1 EN-DC FR2 contention based random access | Absolute uplink power: Pint ≥ P ≥ Pmin ±14.0dB PUMAX ≥ P > Pint ±12.0dB Relative uplink power step: Pint ≥ P ≥ Pmin ±6.0dB PUMAX ≥ P > Pint ±4.0dB Uplink timing: 120kHz SCS Te ±3.5*64*Tc | Not applicable due to UL calibration FFS dB FFS dB [48]Tc | Absolute uplink power: ±FFSdB Relative uplink power step: ±(6+FFS)dB, either PRACH being compared ≤ (Pmax – 6dB) ±(4+FFS)dB, both PRACHs being compared > (Pmax – 6dB) Uplink timing: 120kHz SCS Te ±224±[48]*Tc |
| 5.3.2.2.2 EN-DC FR2 non-contention based random access | Same as 5.3.2.2.1 | Same as 5.3.2.2.1 | Same as 5.3.2.2.1 |
| 5.4.1.1 EN-DC FR2 UE transmit timing accuracy | Test 1 (no DRX): Uplink timing: ±3*64*Tc for 240 kHz SSB SCS Max step size Tq: 2.5*64*Tc Min adjust rate Tp: 2.5*64*Tc Max adjust rate: 2.5*64*Tc Noc = -112 dBm/15 kHz Ês2 / Noc: +4.0dB Test 2 (with DRX): Uplink timing: ±3*64*Tc for 240 kHz SSB SCS Noc = -112 dBm/15 kHz Ês2 / Noc: +4.0dB | ±0.75*64*Tc +0.625*64Tc -3.725*64*Tc +1.225*64*Tc 0dB 0dB ±0.75*64*Tc 0dB 0dB | Test 1 (no DRX): Uplink timing: ±3.75*64*Tc Max step size Tq: 3.125*64*Tc Min adjust rate: -1.225*64*Tc Max adjust rate: 3.725*64*Tc Noc = -112 dBm/15 kHz Ês2 / Noc: +4.0dB Test 2 (with DRX): Uplink timing: ±3.75*64*Tc Noc = -112 dBm/15 kHz Ês2 / Noc: +4.0dB |
| 5.4.3.1 EN-DC FR2 UE timing advance adjustment accuracy | Noc = -112 dBm/15 kHz Ês1 / Noc = 4 dB UE Timing Advance Adjustment Accuracy for 120kHz SCS = ±32 Tc | 0dB 0dB +/- 40 Tc | Noc = -112 dBm/15 kHz Ês1 / Noc = 4 dB UE TAAA for 120kHz SCS = ±72 Tc |
| 5.5.1.1 EN-DC FR2 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode | During T1: Noc: -92.1 dBm/15kHz Ês1 / Noc: +2 dB Ês2 / Noc: +2 dB During T2: Noc: -92.1dBm/15kHz Ês1 / Noc: -6 dB Ês2 / Noc: -15 dB During T3: Noc: -92.1dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: -15 dB | During T1: -2.7 dB 2.1 dB 2.1 dB During T2: -2.7 dB 2.1 dB 0 dB During T3: -2.7 dB 0 dB 0 dB | During T1: Noc: -94.8 dBm/15kHz Ês1 / Noc: +4.1 dB Ês2 / Noc: +4.1 dB During T2: Noc: -94.8 dBm/15kHz Ês1 / Noc: -3.9 dB Ês2 / Noc: -15 dB During T3: Noc: -94.8 dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: -15 dB |
| 5.5.1.2 EN-DC FR2 radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode | During T1: Noc: -92.1 dBm/15kHz Ês1 / Noc: +2 dB Ês2 / Noc: +2 dB During T2: Noc: -92.1dBm/15kHz Ês1 / Noc: -6 dB Ês2 / Noc: -15 dB During T3: Noc: -92.1dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: -15 dB During T4: Noc: -92.1dBm/15kHz Ês1 / Noc: -4.5 dB Ês2 / Noc: -15 dB During T5: Noc: -92.1dBm/15kHz Ês1 / Noc: +2 dB Ês2 / Noc: -15 dB | During T1: -2.7 dB 2.1 dB 2.1 dB During T2: -2.7 dB 2.1 dB 0 dB During T3: -2.7 dB 0 dB 0 dB During T4: -2.7 dB 0 dB 0 dB During T5: -2.7 dB 2.1 dB 0 dB | During T1: Noc: -94.8 dBm/15kHz Ês1 / Noc: +4.1 dB Ês2 / Noc: +4.1 dB During T2: Noc: -94.8 dBm/15kHz Ês1 / Noc: -3.9 dB Ês2 / Noc: -15 dB During T3: Noc: -94.8 dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: -15 dB During T4: Noc: -94.8 dBm/15kHz Ês1 / Noc: -4.5 dB Ês2 / Noc: -15 dB During T5: Noc: -94.8 dBm/15kHz Ês1 / Noc: 4.1 dB Ês2 / Noc: -15 dB |
| 5.5.1.3 EN-DC FR2 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in DRX mode | During T1: Noc: -104.7 dBm/15kHz Ês1 / Noc: +2 dB Ês2 / Noc: +2 dB During T2: Noc: -104.7 dBm/15kHz Ês1 / Noc: -6 dB Ês2 / Noc: -15 dB During T3: Noc: -104.7 dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: -15 dB | During T1: 0 dB 1.3 dB 1.3 dB During T2: 0 dB 1.3 dB -0.4 dB During T3: 0 dB -0.4 dB -0.4 dB | During T1: Noc: -104.7 dBm/15kHz Ês1 / Noc: +3.3 dB Ês2 / Noc: +3.3 dB During T2: Noc: -104.7 dBm/15kHz Ês1 / Noc: -4.7 dB Ês2 / Noc: -15.4 dB During T3: Noc: -104.7 dBm/15kHz Ês1 / Noc: -15.4 dB Ês2 / Noc: -15.4 dB |
| 5.5.1.4 EN-DC FR2 radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in DRX mode | During T1: Noc: -104.7 dBm/15kHz Ês1 / Noc: +2 dB Ês2 / Noc: +2 dB During T2: Noc: -104.7 dBm/15kHz Ês1 / Noc: -6 dB Ês2 / Noc: -15 dB During T3: Noc: -104.7 dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: -15 dB During T4: Noc: -104.7 dBm/15kHz Ês1 / Noc: -4.5 dB Ês2 / Noc: -15 dB During T5: Noc: -104.7 dBm/15kHz Ês1 / Noc: +2 dB Ês2 / Noc: -14 dB | During T1: 0 dB 1.3 dB 1.3 dB During T2: 0 dB 1.3 dB -0.4 dB During T3: 0 dB -0.4 dB -0.4 dB During T4: 0 dB -0.4 dB -0.4 dB During T5: 0 dB 1.3 dB -0.4 dB | During T1: Noc: -104.7 dBm/15kHz Ês1 / Noc: +3.3 dB Ês2 / Noc: +3.3 dB During T2: Noc: -104.7 dBm/15kHz Ês1 / Noc: -4.7 dB Ês2 / Noc: -15.4 dB During T3: Noc: -104.7 dBm/15kHz Ês1 / Noc: -15.4 dB Ês2 / Noc: -15.4 dB During T4: Noc: -104.7 dBm/15kHz Ês1 / Noc: -4.9 dB Ês2 / Noc: -15.4 dB During T5: Noc: -104.7 dBm/15kHz Ês1 / Noc: 3.3 dB Ês2 / Noc: -15.4 dB |
| 5.5.1.5 EN-DC FR2 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode | During T1: Noc: -92.1 dBm/15kHz Ês1 / Noc: +2 dB Ês2 / Noc: +2 dB During T2: Noc: -92.1dBm/15kHz Ês1 / Noc: -6 dB Ês2 / Noc: -14 dB During T3: Noc: -92.1dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: -15 dB | During T1: -2.7 dB 2.1 dB 2.1 dB During T2: -2.7 dB 2.1 dB 0 dB During T3: -2.7 dB 0 dB 0 dB | During T1: Noc: -94.8 dBm/15kHz Ês1 / Noc: +4.1 dB Ês2 / Noc: +4.1 dB During T2: Noc: -94.8 dBm/15kHz Ês1 / Noc: -3.9 dB Ês2 / Noc: -14 dB During T3: Noc: -94.8 dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: -15 dB |
| 5.5.1.6 EN-DC FR2 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode | During T1: Noc: -92.1 dBm/15kHz Ês1 / Noc: +2 dB Ês2 / Noc: +2 dB During T2: Noc: -92.1dBm/15kHz Ês1 / Noc: -6 dB Ês2 / Noc: -14 dB During T3: Noc: -92.1dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: -15 dB During T4: Noc: -92.1dBm/15kHz Ês1 / Noc: -4.5 dB Ês2 / Noc: -15 dB During T5: Noc: -92.1dBm/15kHz Ês1 / Noc: +2 dB Ês2 / Noc: -14 dB | During T1: -2.7 dB 2.1 dB 2.1 dB During T2: -2.7 dB 2.1 dB 0 dB During T3: -2.7 dB 0 dB 0 dB During T4: -2.7 dB 0 dB 0 dB During T5: -2.7 dB 2.1 dB 0 dB | During T1: Noc: -94.8 dBm/15kHz Ês1 / Noc: +4.1 dB Ês2 / Noc: +4.1 dB During T2: Noc: -94.8 dBm/15kHz Ês1 / Noc: -3.9 dB Ês2 / Noc: -14 dB During T3: Noc: -94.8 dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: -15 dB During T4: Noc: -94.8 dBm/15kHz Ês1 / Noc: -4.5 dB Ês2 / Noc: -15 dB During T5: Noc: -94.8 dBm/15kHz Ês1 / Noc: 4.1 dB Ês2 / Noc: -14 dB |
| 5.5.1.7 EN-DC FR2 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in DRX mode | During T1: Noc: -104.7 dBm/15kHz Ês1 / Noc: +2 dB Ês2 / Noc: +2 dB During T2: Noc: -104.7dBm/15kHz Ês1 / Noc: -6 dB Ês2 / Noc: -14 dB During T3: Noc: -104.7dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: -15 dB | During T1: 0 dB 1.3 dB 1.3 dB During T2: 0 dB 1.3 dB -0.4 dB During T3: 0 dB -0.4 dB -0.4 dB | During T1: Noc: -104.7 dBm/15kHz Ês1 / Noc: +3.3 dB Ês2 / Noc: +3.3 dB During T2: Noc: -104.7 dBm/15kHz Ês1 / Noc: -4.7 dB Ês2 / Noc: -14.4 dB During T3: Noc: -104.7 dBm/15kHz Ês1 / Noc: -15.4 dB Ês2 / Noc: -15.4 dB |
| 5.5.1.8 EN-DC FR2 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM RS in DRX mode | During T1: Noc: -104.7 dBm/15kHz Ês1 / Noc: +2 dB Ês2 / Noc: +2 dB During T2: Noc: -104.7dBm/15kHz Ês1 / Noc: -6 dB Ês2 / Noc: -14 dB During T3: Noc: -104.7dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: -15 dB During T4: Noc: -104.7dBm/15kHz Ês1 / Noc: -4.5 dB Ês2 / Noc: -15 dB During T5: Noc: -104.7dBm/15kHz Ês1 / Noc: +2 dB Ês2 / Noc: -14 dB | During T1: 0 dB 1.3 dB 1.3 dB During T2: 0 dB 1.3 dB -0.4 dB During T3: 0 dB -0.4 dB -0.4 dB During T4: 0 dB -0.4 dB -0.4 dB During T5: 0 dB 1.3 dB -0.4 dB | During T1: Noc: -104.7 dBm/15kHz Ês1 / Noc: +3.3 dB Ês2 / Noc: +3.3 dB During T2: Noc: -104.7 dBm/15kHz Ês1 / Noc: -4.7 dB Ês2 / Noc: -14.4 dB During T3: Noc: -104.7 dBm/15kHz Ês1 / Noc: -15.4 dB Ês2 / Noc: -15.4 dB During T4: Noc: -104.7 dBm/15kHz Ês1 / Noc: -4.9 dB Ês2 / Noc: -15.4 dB During T5: Noc: -104.7 dBm/15kHz Ês1 / Noc: 3.3 dB Ês2 / Noc: -14.4 dB |
| 5.5.2.3 EN-DC FR2 interruptions during measurements on deactivated NR SCC in synchronous EN-DC | During T1: Noc1: -111.7 dBm/15kHz Noc2: -104.7 dBm/15kHz Ês1 / Noc: +12 dB Ês2 / Noc: +5 dB | During T1: 0 dB 0 dB 0 dB 0 dB | During T1: Noc1: -111.7 dBm/15kHz Noc2: -104.7 dBm/15kHz Ês1 / Noc: +12 dB Ês2 / Noc: +5 dB |
| 5.5.2.3 EN-DC FR2 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC | During T1: Noc1: -111.7 dBm/15kHz Noc2: -104.7 dBm/15kHz Ês1 / Noc: +12 dB Ês2 / Noc: +5 dB | During T1: 0 dB 0 dB 0 dB 0 dB | During T1: Noc1: -111.7 dBm/15kHz Noc2: -104.7 dBm/15kHz Ês1 / Noc: +12 dB Ês2 / Noc: +5 dB |
| 5.5.5.1 EN-DC FR2 SSB-based beam failure detection and link recovery in non-DRX | During T1: SNR_SSB q0: 5 dB SNR_SSB q1: 0.2 dB During T2: SNR_SSB q0: -3 dB SNR_SSB q1: 0.2 dB During T3-T5: SNR_SSB q0: -12 dB SNR_SSB q1: 20.2 dB In signaling: rsrp-ThresholdSSB: -95 dBm/120kHz for Config 1,2 rsrp-ThresholdSSB: -92 dBm/240kHz for Config 3,4 | +8.70 dB 0 dB +8.70 dB 0 dB 0 dB -0.2 dB -14 dB -14 dB | During T1: SNR_SSB q0: 13.7 dB SNR_SSB q1: 0.2 dB During T2: SNR_SSB q0: 5.7 dB SNR_SSB q1: 0.2 dB During T3-T5: SNR_SSB q0: -12 dB SNR_SSB q1: 20 dB In signaling: rsrp-ThresholdSSB: -109 dBm/120kHz for Config 1,2 srp-ThresholdSSB: -106 dBm/240kHz for Config 3,4 |
| 5.5.5.2 EN-DC FR2 SSB-based beam failure detection and link recovery in DRX | Same as 5.5.5.1 | Same as 5.5.5.1 | Same as 5.5.5.1 |
| 5.5.5.3 EN-DC FR2 CSI-RS-based beam failure detection and link recovery in non-DRX | Noc = -104.7 dBm/120 kHz Ês2 / Noc for CSI-RS set q0 T1: 5 dB T2: -3 dB T3: -12 dB T4: -12 dB T5: -12 dB Ês2 / Noc for CSI-RS set q1 T1: 0.2 dB T2: 0.2 dB T3: 20.2 dB T4: 20.2 dB T5: 20.2 dB rsrp-ThresholdSSB: -95 dBm/SCS kHz | 0 dB 8.7 dB 8.7 dB 0 dB 0 dB 0 dB 0 dB 0 dB -0.2 dB -0.2 dB -0.2 dB -14 dB | Noc = -104.7 dBm/120 kHz Ês2 / Noc for CSI-RS set q0 T1: 13.7 dB T2: 5.7 dB T3: -12 dB T4: -12 dB T5: -12 dB Ês2 / Noc for CSI-RS set q1 T1: 0.2 dB T2: 0.2 dB T3: 20 dB T4: 20 dB T5: 20 dB rsrp-ThresholdSSB: -109.0 dBm/SCS kHz |
| 5.5.5.4 EN-DC FR2 CSI-RS-based beam failure detection and link recovery in DRX | Same as 5.5.5.3 | Same as 5.5.5.3 | Same as 5.5.5.3 |
| 5.5.5.5 EN-DC FR2 scheduling available restriction during SSB-based beam failure detection and link recovery in non-DRX | Same as 5.5.5.1 | Same as 5.5.5.1 | Same as 5.5.5.1 |
| 5.5.5.6 EN-DC FR2 SCell CSI-RS-based beam failure detection and link recovery in non-DRX | Noc = -104.7 dBm/120 kHz Ês3 / Noc for CSI-RS set q0 T1: 5 dB T2: -3 dB T3: -12 dB T4: -12 dB T5: -12 dB Ês3 / Noc for CSI-RS set q1 T1: 0.2 dB T2: 0.2 dB T3: 20.2 dB T4: 20.2 dB T5: 20.2 dB rsrp-ThresholdBFR: -95 dBm/SCS kHz | 0 dB 8.7 dB 8.7 dB 0 dB 0 dB 0 dB 0 dB 0 dB -0.2 dB -0.2 dB -0.2 dB -14 dB | Noc = -104.7 dBm/120 kHz Ês3 / Noc for CSI-RS set q0 T1: 13.7 dB T2: 5.7 dB T3: -12 dB T4: -12 dB T5: -12 dB Ês3 / Noc for CSI-RS set q1 T1: 0.2 dB T2: 0.2 dB T3: 20 dB T4: 20 dB T5: 20 dB rsrp-ThresholdBFR: -109.0 dBm/SCS kHz |
| 5.5.5.7 EN-DC FR2 SCell CSI-RS-based beam failure detection and link recovery in DRX | Same as 5.5.5.6 | Same as 5.5.5.6 | Same as 5.5.5.6 |
| 5.5.6.2.1 EN-DC FR2 RRC-based DL active BWP switch in non-DRX in synchronous EN-DC | During T1: Noc2: -112dBm/15kHz Ês2 / Iot2: +18dB | During T1: 0dB 0dB | During T1: Noc2: -112dBm/15kHz Ês2 / Iot2: +18dB |
| 5.6.1.1 EN-DC FR2 event-triggered reporting without gap in non-DRX | During T1: Ês2: -86dBm/120kHz Ês3: -infinity During T2: Ês2: -86dBm/120kHz Ês3: -86dBm/120kHz | During T1: 0dB 0dB During T2: 0dB 0dB | During T1: Ês2: -86dBm/120kHz Ês3: -infinity During T2: Ês2: -86dBm/120kHz Ês3: -86dBm/120kHz |
| 5.6.1.2 EN-DC FR2 event-triggered reporting without gap in DRX | During T1: Noc: -98dBm/15kHz Ês2 / Noc: +4.00dB Ês3 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês2 / Noc: +4.00dB Ês3 / Noc: +4.00dB Signalled A3-offset: -6.0 | During T1: -3.5dB 0dB 0dB During T2: -3.5dB 0dB 0dB Signalled A3-offset: -1.0 | During T1: Noc: -101.5dBm/15kHz Ês2 / Noc: +4.00dB Ês3 / Noc: -infinity During T2: Noc: -101.5dBm/15kHz Ês2 / Noc: +4.00dB Ês3 / Noc: +4.00dB Signalled A3-offset: -7.0 |
| 5.6.1.3 EN-DC FR2 event-triggered reporting with gap in non-DRX | Same as 5.6.1.1 | Same as 5.6.1.1 | Same as 5.6.1.1 |
| 5.6.1.4 EN-DC FR2 event-triggered reporting with gap in DRX | Same as 5.6.1.2 | Same as 5.6.1.2 | Same as 5.6.1.2 |
| 5.6.2.1 EN-DC FR2-FR2 event-triggered reporting in non-DRX | During T1: Ês1: -87 dBm/120kHz Ês2: - infinity During T2: Ês1: -87 dBm/120kHz Ês2: -87 dBm/120kHz | During T1: 0dB 0dB During T2: 0dB 0dB | During T1: Ês1: -87 dBm/120kHz Ês2: -- infinity During T2: Ês1: -87 dBm/120kHz Ês2: -87 dBm/120kHz |
| 5.6.2.2 EN-DC FR2-FR2 event-triggered reporting in DRX | During T1: Noc: -104.70dBm/15kHz Ês1 / Noc: +6.00dB Ês2 / Noc: -infinity During T2: Noc: -104.70dBm/15kHz Ês1 / Noc: +6.00dB Ês2 / Noc: +9.00dB Signalled A3-offset: -6.0 | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB Signalled A3-offset: -6.0 | During T1: Noc: -104.70dBm/15kHz Ês1 / Noc: +6.00dB Ês2 / Noc: -infinity During T2: Noc: -104.70dBm/15kHz Ês1 / Noc: +6.00dB Ês2 / Noc: +9.00dB Signalled A3-offset: -12.0 |
| 5.6.2.3 EN-DC FR2-FR2 event-triggered reporting in non-DRX with SSB time index detection | Same as 5.6.2.1 | Same as 5.6.2.1 | Same as 5.6.2.1 |
| 5.6.2.4 EN-DC FR2-FR2 event-triggered reporting in DRX with SSB time index detection | Same as 5.6.2.2 | Same as 5.6.2.2 | Same as 5.6.2.2 |
| 5.6.2.5 EN-DC FR1-FR2 event-triggered reporting in non-DRX | During T1: Ês2: - Infinity During T2: Ês2: -87 dBm/120kHz | During T1: 0dB During T2: 0dB | During T1: Ês2: - Infinity During T2: Ês2: -87 dBm/120kHz |
| 5.6.2.6 EN-DC FR1-FR2 event-triggered reporting in DRX | During T1: Noc = -104.7 dBm/15 kHz Ês2 / Noc = - Infinity During T2: Noc = -104.7 dBm/15 kHz Ês2 / Noc = 9 dB | During T1: 0dB 0dB During T2: 0dB 0dB | During T1: Noc = -104.7 dBm/15 kHz Ês2 / Noc = - Infinity During T2: Noc = -104.7 dBm/15 kHz Ês2 / Noc = 9 dB |
| 5.6.2.7 EN-DC FR1-FR2 event-triggered reporting in non-DRX with SSB time index detection | Same as 5.6.2.5 | Same as 5.6.2.5 | Same as 5.6.2.5 |
| 5.6.2.8 EN-DC FR1-FR2 event-triggered reporting in DRX with SSB time index detection | Same as 5.6.2.6 | Same as 5.6.2.6 | Same as 5.6.2.6 |
| 5.6.3.1 EN-DC FR2 SSB-based L1-RSRP measurement in non-DRX | During T1: Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: -infinity During T2: Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +9.00dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB | During T1: Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: -infinity During T2: Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +9.00dB |
| 5.6.3.2 EN-DC FR2 SSB-based L1-RSRP measurement in DRX | Same as 5.6.3.1 | Same as 5.6.3.1 | Same as 5.6.3.1 |
| 5.6.3.3 EN-DC FR2 CSI-RS-based L1-RSRP measurement in non-DRX | Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +9.00dB Reported CSI-RSRP values ± 29dB Reported Differential CSI-RSRP values ± 7dB | 0dB 0dB 0dB Via mapping | Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +9.00dB CSI-RS#1: RSRP_40 to RSRP_99 CSI-RS#0: DIFFRSRP_1 to DIFFRSRP_7 |
| 5.6.3.4 EN-DC FR2 CSI-RS-based L1-RSRP measurement in DRX | Same as 5.6.3.3 | Same as 5.6.3.3 | Same as 5.6.3.3 |
| 5.6.4.1 EN-DC FR2 SRS-RSRP measurement in non-DRX | During T1: Noc: -98 dBm/15kHz Neighbour cell UE Es/Noc: -infinity Io: -70.01 dBm/ 95.04 MHz During T2: Noc: -98 dBm/15kHz Neighbour cell UE Es/Noc: 4dB Io: -68.82 dBm/ 95.04 MHz In signalling: i1-threshold: -103dBm/SCS | During T1: -2.5dB 3dB 0dB During T2: -2.5dB 3dB 0dB -9dB | During T1: Noc: -100.5 dBm/15kHz Neighbour cell UE Es/Noc: -infinity Io: -70.01 dBm/ 95.04 MHz During T2: Noc: -100.5 dBm/15kHz Neighbour cell UE Es/Noc: 7dB Io: -68.82 dBm/ 95.04 MHz i1-threshold: -112dBm/SCS |
| 5.6.6.1 EN-DC FR2 CSI-RS based CMR and no dedicated IMR L1-SINR measurement in DRX | Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +9.00dB Reported CSI-SINR values ± 5.5dB Reported Differential CSI-SINR values ± 4.5dB | 0dB 0dB 0dB Via mapping | Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +9.00dB CSI-RS#1: SINR_51 to SINR_74 CSI-RS#0: DIFFSINR_4 to DIFFSINR_13 |
| 5.6.6.2 EN-DC FR2 SSB based CMR and dedicated IMR L1-SINR measurement in non-DRX | During T1: Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: -infinity During T2: Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +9.00dB Reported SS-SINR values ± 4.0dB Reported Differential SS-SINR values ± 3.0dB | During T1: 0dB 1.5dB 0dB During T2: 0dB 1.5dB 0dB Via mapping | During T1: Noc: -105dBm/15kHz Ês0 / Noc: +1.50dB Ês1 / Noc: -infinity During T2: Noc: -105dBm/15kHz Ês0 / Noc: +1.50dB Ês1 / Noc: +9.00dB SSB#1: SINR_54 to SINR_71 SSB#0: DIFFSINR_4 to DIFFSINR_10 |
| 5.6.6.3 EN-DC FR2 CSI-RS based CMR and dedicated IMR L1-SINR measurement in non-DRX | Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +9.00dB Reported CSI-SINR values ± 4.5dB Reported Differential CSI-SINR values ± 3.5dB | 0dB 0dB 0dB Via mapping | Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +9.00dB CSI-RS#1: SINR_53 to SINR_72 CSI-RS#0: DIFFSINR_5 to DIFFSINR_12 |
| 5.7.1.1 EN-DC FR2 SS-RSRP measurement accuracy | Test 1: Noc: -91.60dBm/15kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +1.0dB Reported absolute RSRP values: ±8dB Reported relative RSRP values: ±6dB For extreme conditions allow ±3dB + FFS MU additionally Test 2: n257 Ês2: -110.0dBm/120kHz n257 Ês3: -110.0dBm/120kHz n260 Ês2: -107.4dBm/120kHz n260 Ês3: -107.4dBm/120kHz Reported absolute RSRP values: ±8dB Reported relative RSRP values: ±6dB For extreme conditions allow ±3dB+ FFS MU additionally | Test 1: -5.80dB 0dB 0.4dB Via Mapping Test 2: 7.70dB 7.70dB 7.70dB 7.70dB Via Mapping | Test 1: Noc: -97.40dBm/15kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +1.4dB Cell 2: RSRP_50 to RSRP_108 Cell 3: RSRP_46 to RSRP_103 Cell 3: RSRP_x-12 to RSRP_X+2 Test 2: n257 Ês2: -102.3dBm/120kHz n257 Ês3: -102.3dBm/120kHz n260 Ês2: -99.7dBm/120kHz n260 Ês3: -99.7dBm/120kHz Band dependent. See test case tables in clause 5.7.1.1.5 |
| 5.7.1.2 EN-DC FR2-FR2 SS-RSRP measurement accuracy | Test 1 Config 1: Noc1: -90.60dBm/15kHz Noc2: -90.60dBm/15kHz Ês1 / Noc1: +6.0dB Ês2 / Noc2: +6.0dB Reported absolute RSRP values: ±8.0dB For extreme conditions allow ±3dB+ FFS MU additionally Reported relative RSRP values: ±6.0dB For extreme conditions allow ±3dB+ FFS MU additionally Test 2 Config 1: n257 Noc1: -108.13dBm/15kHz n257 Noc2: -113.13dBm/15kHz n260 Noc1: -105.53dBm/15kHz n260 Noc2: -110.53dBm/15kHz Ês1 / Noc1: 17.0dB Ês2 / Noc2: -1.0dB Reported absolute RSRP values: ±8.0dB For extreme conditions allow ±3dB+ FFS MU additionally Reported relative RSRP values: ±6.0dB For extreme conditions allow ±3dB+ FFS MU additionally Test 1 Config 2: Noc1: -93.70dBm/15kHz Noc2: -93.70dBm/15kHz Ês1 / Noc1: +6.0dB Ês2 / Noc2: +6.0dB Reported absolute RSRP values: ±8.0dB For extreme conditions allow ±3dB+ FFS MU additionally Reported relative RSRP values: ±6.0dB For extreme conditions allow ±3dB+ FFS MU additionally Test 2 Config 2: n257 Noc1: -108.13dBm/15kHz n257 Noc2: -113.13dBm/15kHz n260 Noc1: -105.53dBm/15kHz n260 Noc2: -110.53dBm/15kHz Ês1 / Noc1: 17.0dB Ês2 / Noc2: -1.0dB Reported absolute RSRP values: ±8.0dB For extreme conditions allow ±3dB+ FFS MU additionally Reported relative RSRP values: ±6.0dB For extreme conditions allow ±3dB+ FFS MU additionally | Test 1 Config 1: -5.70dB -5.70dB 0dB 0dB Via Mapping Via Mapping Test 2 Config 1: -6.60dB 0dB -6.60dB 0dB 0dB 2dB Via Mapping Via Mapping Test 1 Config 2: -5.60dB -5.60dB 0dB 0dB Via Mapping Via Mapping Test 2 Config 2: -6.60dB 0dB -6.60dB 0dB 0dB 2dB Via Mapping Via Mapping | Test 1 Config 1: Noc1: -96.30dBm/15kHz Noc2: -96.30dBm /15kHz Ês1 / Noc1: +6.0dB Ês2 / Noc2: +6.0dB Cell 2 n257: RSRP_41 to RSRP_109 n260: RSRP_39 to RSRP_109 Cell 3: RSRP_52 to RSRP_109 Cell 3: n257: RSRP_x-15 to RSRP_x+25 n260: RSRP_x-15 to RSRP_x+27 Test 2 Config 1: -114.73dBm/15kHz -113.13dBm/15kHz -112.13dBm/15kHz -110.53dBm/15kHz Ês1 / Noc1: 17.0dB Ês2 / Noc2: 1.0dB Cell 2 n257: RSRP_33 to RSRP_101 n260: RSRP_34 to RSRP_104 Cell 3: n257: RSRP_32 to RSRP_87 n260: RSRP_34 to RSRP_90 Cell 3: n257: RSRP_x-29 to RSRP_x+11 n260: RSRP_x-29 to RSRP_x+13 Test 1 Config 2: Noc1: -99.30dBm/15kHz Noc2: -99.30dBm /15kHz Ês1 / Noc1: +6.0dB Ês2 / Noc2: +6.0dB Cell 12 n257: RSRP_41 to RSRP_109 n260: RSRP_39 to RSRP_109 Cell 3: RSRP_52 to RSRP_109 Cell 3: n257: RSRP_x-15 to RSRP_x+25 n260: RSRP_x-15 to RSRP_x+27 Test 2 Config 2: -114.73dBm/15kHz -113.13dBm/15kHz -112.13dBm/15kHz -110.53dBm/15kHz Ês1 / Noc1: 17.0dB Ês2 / Noc2: 1.0dB Cell 2 n257: RSRP_36 to RSRP_104 n260: RSRP_37 to RSRP_107 Cell 3: n257: RSRP_35 to RSRP_90 n260: RSRP_37 to RSRP_93 Cell 3: n257: RSRP_x-29 to RSRP_x+11 n260: RSRP_x-29 to RSRP_x+13 |
| 5.7.1.3 EN-DC FR1-FR2 SS-RSRP measurement accuracy | Test 1: Noc: -90dBm/15kHz Ês3 / Noc: +5.0dB Reported absolute RSRP values: ±8dB For extreme conditions allow ±3dB + FFS MU additionally Test 2: n257 Ês3: -98.2dBm/120kHz n260 Ês3: -93.9dBm/120kHz Reported absolute RSRP values: ±8dBFor extreme conditions allow ±3dB+ FFS MU additionally | Test 1: -5.50dB 0dB Via Mapping Test 2: 5.50dB 5.50dB Via Mapping | Test 1: Noc: -95.50dBm/15kHz Ês3 / Noc: +5dB Band dependent. See test case tables in clause 5.7.1.3.5 Test 2: n257 Ês3: -92.7dBm/120kHz n260 Ês3: -88.4dBm/120kHz Band dependent. See test case tables in clause 5.7.1.3.5 |
| 5.7.2.1 EN-DC FR2 SS-RSRQ measurement accuracy | Test 1: Noc: -95.0dBm/15kHz Ês2 / Noc: +3.0dB Ês3 / Noc: +3.0dB Reported absolute RSRQ values: ±2.5dB For extreme conditions allow ±1.5dB+ FFS MU additionally Test 2: Noc: -95.0dBm/15kHz Ês2 / Noc: -3.0dB Ês3 / Noc: -3.0dB Reported absolute RSRQ values: ±3.5dB For extreme conditions allow ±0.5dB+ FFS MU additionally | Test 1: -5.70dB 0dB 0dB Via Mapping Test 2: -1.70dB 0dB 0dB Via Mapping | Test 1: Noc: -100.70dBm/15kHz Ês2 / Noc: +3.0dB Ês3 / Noc: +3.0dB Cell 3: All bands: RSRQ_41 to RSRQ_73 Test 2: Noc: -96.70dBm/15kHz Ês2 / Noc: -3.0dB Ês3 / Noc: -3.0dB Cell 3: n257, n258, n261: RSRQ_35 to RSRQ_71 n260: RSRQ_34 to RSRQ_71 |
| 5.7.2.2 EN-DC FR2-FR2 SS-RSRQ measurement accuracy | Test 1: Noc1: -94.03dBm/15kHz Noc2: -94.03dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported absolute RSRQ values: ±2.5dB For extreme conditions allow ±1.5dB+ FFS MU additionally Reported relative RSRQ values: ±3.0dB For extreme conditions allow ±1.0dB+ FFS MU additionally Test 2: Noc1: -94.03dBm/15kHz Noc2: -94.03dBm/15kHz Ês1 / Noc1: -3.0dB Ês2 / Noc2: -3.0dB Reported absolute RSRQ values: ±3.5dB For extreme conditions allow ±0.5dB+ FFS MU additionally Reported relative RSRQ values: ±4.0dB For extreme conditions allow ±0.0dB+ FFS MU additionally | Test 1: -1.90dB -1.90dB 0dB 0dB Via Mapping Via Mapping Test 2: -1.41dB -1.41dB 0dB 0dB Via Mapping Via Mapping | Test 1: Noc1: -95.93dBm/15kHz Noc2: -95.93dBm /15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Cell 2 and Cell 3: RSRQ_41 to RSRQ_73 Cell 3: RSRQ_x-7 to RSRQ_x+7 Test 2: Noc1: -95.44dBm/15kHz Noc2: -95.44dBm /15kHz Ês1 / Noc1: -3.0dB Ês2 / Noc2: -3.0dB Cell 2 and Cell 3: RSRQ_37 to RSRQ_74 Cell 3: RSRQ_x-9 to RSRQ_x+9 |
| 5.7.3.1 EN-DC FR2 SS-SINR measurement accuracy | Test 1: Noc: -105.0dBm/15kHz Ês2 / Noc: +4.54dB Ês3 / Noc: +2.66dB Reported absolute SINR values: ±3.5dB For extreme conditions allow ±0.5dB+ FFS MU additionally Test 2: Noc: -105.0dBm/15kHz Ês2 / Noc: -3.0dB Ês3 / Noc: -3.0dB Reported absolute SINR values: ±3.5dB For extreme conditions allow ±0.5dB+ FFS MU additionally | Test 1: 0dB 0dB 0dB Via Mapping Test 2: 0dB 0.2dB 0.2dB Via Mapping | Test 1: Noc: -105.0dBm/15kHz Ês2 / Noc: +4.54dB Ês3 / Noc: +2.66dB Cell 3: n257, n258, n261: SINR_22 to RSRQ_58 n260: SINR_21 to RSRQ_58 Test 2: Noc: -105.0dBm/15kHz Ês2 / Noc: -2.8dB Ês3 / Noc: -2.8dB Cell 3: n257, n258, n261: SINR_18 to RSRQ_55 n260: SINR_18 to RSRQ_54 |
| 5.7.3.2 EN-DC FR2-FR2 SS-SINR measurement accuracy | Test 1: Noc1: -105.0dBm/15kHz Noc2: -105.0dBm/15kHz Ês1 / Noc1: -0.5dB Ês2 / Noc2: -0.5dB Reported absolute SINR values: ±3.0dB For extreme conditions allow ±1.0dB+ FFS MU additionally Reported absolute SINR values: ±3.5dB For extreme conditions allow ±0.5dB+ FFS MU additionally Test 2: Noc1: -105.0dBm/15kHz Noc2: -105.0dBm/15kHz Ês1 / Noc1: +11.0dB Ês2 / Noc2: +11.0dB Reported absolute SINR values: ±3.0dB For extreme conditions allow ±1.0dB+ FFS MU additionally Reported absolute SINR values: ±3.5dB For extreme conditions allow ±0.5dB+ FFS MU additionally Test 3: Noc1: -105.0dBm/15kHz Noc2: -105.0dBm/15kHz Ês1 / Noc1: -3.0dB Ês2 / Noc2: -3.0dB Reported absolute SINR values: ±3.5dB For extreme conditions allow ±0.5dB+ FFS MU additionally Reported absolute SINR values: ±4.0dB For extreme conditions allow ±0.0dB+ FFS MU additionally | Test 1: 0dB 0dB 0dB 0dB Via Mapping Via Mapping Test 2: -0.1dB -0.1dB 0dB 0dB Via Mapping Via Mapping Test 3: 0dB 0dB 0.9dB 0.9dB Via Mapping Via Mapping | Test 1: Noc1: -105.0dBm/15kHz Noc2: -105.0dBm/15kHz Ês1 / Noc1: -0.5dB Ês2 / Noc2: -0.5dB Cell 2 and Cell 3: n257, n258, n261: SINR_27 to SINR_62 n260: SINR_27 to SINR_61 Cell 3: SINR_x-8 to SINR_x+8 Test 2: Noc1: -105.1dBm/15kHz Noc2: -105.1dBm/15kHz Ês1 / Noc1: +11.0dB Ês2 / Noc2: +11.0dB Cell 2 and Cell 3: n257, n258, n261: SINR_48 to SINR_87 n260: SINR_48 to SINR_86 Cell 3: SINR_x-17 to SINR_x+17 Test 3: Noc1: -105.0dBm/15kHz Noc2: -105.0dBm/15kHz Ês1 / Noc1: -2.1dB Ês2 / Noc2: -2.1dB Cell 2 and Cell 3: n257, n258, n261: SINR_23 to SINR_60 n260: SINR_22 to SINR_59 Cell 3: SINR_x-9 to SINR_x+9 |
| 5.7.4.1 EN-DC FR2 SSB based L1-RSRP measurement accuracy | Test 1: Noc: -100dBm/15kHz SSB#0 Ês / Noc: +10.0dB SSB#1 Ês / Noc: -2.0dB Reported absolute RSRP values: ±8.5dB Reported relative RSRP values: ±6.5dB For extreme conditions allow ±3dB + FFS MU additionally Test 2: n257 SSB#0 Ês: -108.1dBm/120kHz n257 SSB#1 Ês: -105.5dBm/120kHz n260 SSB#0 Ês: -108.1dBm/120kHz n260 SSB#1 Ês: -105.5dBm/120kHz Reported absolute RSRP values: -6.5 +8.5dB Reported relative RSRP values: ±6.5dB For extreme conditions allow ±3dB+ FFS MU additionally | Test 1: -4.1dB 0dB 0.4dB Test 2: 5.7dB | Test 1: Noc: -104.1dBm/15kHz SSB#0 Ês / Noc: +10.0dB SSB#1 Ês / Noc: -1.6dB SSB#0: RSRP_42 to RSRP_101 SSB#1: RSRP_31 to RSRP_89 SSB#1-SSB#0: -9 to -2 Test 2: n257 SSB#0 Ês: -102.4dBm/120kHz n257 SSB#1 Ês: -99.8dBm/120kHz n260 SSB#0 Ês: -102.4dBm/120kHz n260 SSB#1 Ês: -99.8dBm/120kHz SSB#0 and SSB#1 n257: RSRP_27 to RSRP_83 n260: RSRP_30 to RSRP_86 SSB#1-SSB#0: -4 to +4 |
| 5.7.4.2 EN-DC FR2 CSI-RS based L1-RSRP measurement accuracy | Test 1: Noc: -100dBm/15kHz CSI-RS#0 Ês / Noc: +10.0dB CSI-RS#1 Ês / Noc: -2.0dB Reported absolute RSRP values: ±8.5dB Reported relative RSRP values: ±6.5dB For extreme conditions allow ±3dB + FFS MU additionally Test 2: n257 CSI-RS#0 Ês: -108.1dBm/120kHz n257 CSI-RS#1 Ês: -105.5dBm/120kHz n260 CSI-RS#0 Ês: -108.1dBm/120kHz n260 CSI-RS#1 Ês: -105.5dBm/120kHz Reported absolute RSRP values: -6.5 +8.5dB Reported relative RSRP values: ±6.5dB For extreme conditions allow ±3dB+ FFS MU additionally | Test 1: -4.1dB 0dB 0.2dB Test 2: 5.7dB | Test 1: Noc: -104.1dBm/15kHz CSI-RS#0 Ês / Noc: +10.0dB CSI-RS#1 Ês / Noc: -1.8dB CSI-RS#0: RSRP_42 to RSRP_101 CSI-RS#1: RSRP_30 to RSRP_89 CSI-RS#1-CSI-RS#0: -9 to -2 Test 2: n257 CSI-RS#0 Ês: -102.4dBm/120kHz n257 CSI-RS#1 Ês: -99.8dBm/120kHz n260 CSI-RS#0 Ês: -102.4dBm/120kHz n260 CSI-RS#1 Ês: -99.8dBm/120kHz CSI-RS#0 and CSI-RS#1 n257: RSRP_27 to RSRP_83 n260: RSRP_30 to RSRP_86 CSI-RS#1-CSI-RS#0: -4 to +4 |
| 5.7.5.1 EN-DC FR2 SRS-RSRP measurement accuracy | During T1: Noc: -100 dBm/15kHz SRS Es/Noc: 2dB Io: -70.01 dBm/ 95.04 MHz During T2: Noc: -100 dBm/15kHz Io: -68.82 dBm/ 95.04 MHz Reported absolute SRS-RSRP values: -89 ± 11.5dB | During T1: 0dB 0dB During T2: 0dB Via mapping | During T1: Noc: -100 dBm/15kHz SRS Es/Noc: 2dB Io: -70.01 dBm/ 95.04 MHz During T2: Noc: -100 dBm/15kHz Io: -68.82 dBm/ 95.04 MHz Reported SRS-RSRP range n257: SRS_RSRP_50 to SRS_RSRP_85 n260: SRS_RSRP_50 to SRS_RSRP_85 |
| 5.7.6.1 EN-DC FR2 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR measurement accuracy | Test 1: Noc: -100dBm/15kHz CSI-RS#0 Ês / Noc: +10.0dB CSI-RS#1 Ês / Noc: -2.0dB Reported absolute SINR values: ±5.5dB Reported relative SINR values: ±4.5dB For extreme conditions allow ±1dB + FFS MU additionally | Test 1: -4.1dB 0dB 0.2dB | Test 1: Noc: -104.1dBm/15kHz CSI-RS#0 Ês / Noc: +10.0dB CSI-RS#1 Ês / Noc: -1.8dB CSI-RS#0: SINR_53 to SINR_76 CSI-RS#1-CSI-RS#0: 6 to 15 |
| 5.7.6.2 EN-DC FR2 SSB based CMR and dedicated IMR L1-SINR absolute measurement accuracy | Test 1: Noc: -100dBm/15kHz SSB#0 Ês / Noc: +10.0dB SSB#1 Ês / Noc: 0.0dB CSI-RS#0 Ês / Noc: +10.0dB CSI-RS#1 Ês / Noc: 0.0dB Reported absolute SINR values: ±4dB Reported relative SINR values: ±3dB For extreme conditions allow ±1dB + FFS MU additionally | Test 1: -4.1dB 0dB 0.2dB 0dB 0.2dB | Test 1: Noc: -104.1dBm/15kHz SSB#0 Ês / Noc: +10.0dB SSB#1 Ês / Noc: 1.2dB CSI-RS#0 Ês / Noc: +10.0dB CSI-RS#1 Ês / Noc: 1.2dB SSB#0: SINR_56 to SINR_73 SSB#1-SSB#0: 5 to 12 |
| 5.7.6.3 EN-DC FR2 CSI-RS based CMR and dedicated IMR L1-SINR measurement accuracy | Test 1: Noc: -100dBm/15kHz CSI-RS#0 Ês / Noc: +10.0dB CSI-RS#1 Ês / Noc: -2.0dB Reported absolute SINR values: ±4.5dB Reported relative SINR values: ±3.5dB For extreme conditions allow ±1dB + FFS MU additionally | Test 1: -4.1dB 0dB 0.2dB | Test 1: Noc: -104.1dBm/15kHz CSI-RS#0 Ês / Noc: +10.0dB CSI-RS#1 Ês / Noc: -1.8dB CSI-RS#0: SINR_55 to SINR_74 CSI-RS#1-CSI-RS#0: 7 to 15 |

Table F.1.3.2-4: Derivation of test requirements for NR SA FR2 RRM tests

| Test | Minimum requirement in TS 38.133 [6] | Test tolerance (TT) | Test requirement in TS 38.533 |
| --- | --- | --- | --- |
| 7.1.1.1 NR SA FR2 cell re-selection | During T1: Noc = -102 dBm/15kHz Es1/Noc = 8dB Es2/Noc = -infinity During T2: Noc = -102 dBm/15kHz Es1/Noc = -3dB Es2/Noc = 1.5dB During T3: Noc = -102 dBm/15kHz Es1/Noc = 1.5dB Es2/Noc = -3dB In signalling: Qrxlevmin = -138dBm/SCS for config. 1 Qrxlevmin = -135dBm/SCS for config. 2 | During T1: 0dB 0dB N/A During T2: 0dB 0.1dB 0.55dB During T3: 0dB 0.55dB 0.1dB In signalling: 18 dB 18 dB | During T1: Noc = -102 dBm/15kHz Es1/Noc = 8dB Es2/Noc = -infinity During T2: Noc = -102 dBm/15kHz Es1/Noc = -2.90dB Es2/Noc = 2.05dB During T3: Noc = -102 dBm/15kHz Es1/Noc = 2.05dB Es2/Noc = -2.90dB In signalling: Qrxlevmin = -120dBm/SCS for config. 1 Qrxlevmin = -117dBm/SCS for config. 2 |
| 7.1.1.2 NR SA FR2-FR2 cell re-selection | During T1: Noc1 = -102 dBm/15kHz Noc2 = -102 dBm/15kHz Es1/Noc1 = 10.5dB Es2/Noc2 = -10.5dB During T2: Noc1 = -102 dBm/15kHz Noc2 = -102 dBm/15kHz Es1/Noc1 = 10.5dB Es2/Noc2 = -infinity During T3: Noc1 = -102 dBm/15kHz Noc2 = -102 dBm/15kHz Es1/Noc1 = 8dB Es2/Noc2 = 8.5dB In signalling: Cell 2 Qrxlevmin = -138dBm/SCS for config. 1 Cell 2 Qrxlevmin = -135dBm/SCS for config. 2 Cell 2 SnonintraSearchP = 50dB Cell 1 Threshx, highP = 48dB Cell 2 Threshserving, lowP = 44dB Cell 2 Threshx, lowP = 50dB | During T1: 0dB 0dB 0dB 7.6dB During T2: 0dB 0dB 0dB N/A During T3: 0dB 0dB 0dB 0dB In signalling: 16 dB 16 dB 12dB -16dB 18dB -32dB | During T1: Noc1 = -102 dBm/15kHz Noc2 = -102 dBm/15kHz Es1/Noc1 = 10.5dB Es2/Noc2 = -2.9dB During T2: Noc1 = -102 dBm/15kHz Noc2 = -102 dBm/15kHz Es1/Noc1 = 10.5dB Es2/Noc2 = -infinity During T3: Noc1 = -102 dBm/15kHz Noc2 = -102 dBm/15kHz Es1/Noc1 = 8dB Es2/Noc2 = 8.5dB In signalling: Qrxlevmin = -124dBm/SCS for config. 1 Qrxlevmin = -121dBm/SCS for config. 2 Cell 2 SnonintraSearchP = 62dB Cell 1 Threshx, highP = 32dB Cell 2 Threshserving, lowP = 62dB Cell 2 Threshx, lowP = 18dB |
| 7.1.1.3 NR SA FR2 cell re-selection for UE fulfilling low mobility relaxed measurement criterion | During T1: Noc = -102 dBm/15kHz Es1/Noc = -3dB Es2/Noc = 1.5dB During T2: Noc = -102 dBm/15kHz Es1/Noc = 1.5dB Es2/Noc = -3dB In signalling: Qrxlevmin = -140dBm/SCS for config. 1 Qrxlevmin = -137dBm/SCS for config. 2 | During T1: 0dB 0.1dB 0.55dB During T2: 0dB 0.55dB 0.1dB In signalling: 20dB 20dB | During T1: Noc = -102 dBm/15kHz Es1/Noc = -2.90dB Es2/Noc = 2.05dB During T2: Noc = -102 dBm/15kHz Es1/Noc = 2.05dB Es2/Noc = -2.90dB In signalling: Qrxlevmin = -120dBm/SCS for config. 1 Qrxlevmin = -117dBm/SCS for config. 2 |
| 7.1.1.4 NR SA FR2 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion | During T1: Noc = -102 dBm/15kHz Es1/Noc = -3dB Es2/Noc = 1.5dB During T2: Noc = -102 dBm/15kHz Es1/Noc = 1.5dB Es2/Noc = -3dB In signalling: Qrxlevmin = -140dBm/SCS for config. 1 Qrxlevmin = -137dBm/SCS for config. 2 SsearchThresholdP = 35dB | During T1: 0dB 0.1dB 0.55dB During T2: 0dB 0.55dB 0.1dB In signalling: 20dB 20dB -27dB | During T1: Noc = -102 dBm/15kHz Es1/Noc = -2.90dB Es2/Noc = 2.05dB During T2: Noc = -102 dBm/15kHz Es1/Noc = 2.05dB Es2/Noc = -2.90dB In signalling: Qrxlevmin = -120dBm/SCS for config. 1 Qrxlevmin = -117dBm/SCS for config. 2 SsearchThresholdP = 8dB |
| 7.1.1.5 NR SA FR2-FR2 cell re-selection for UE fulfilling low mobility relaxed measurement criterion | During T1: Noc1 = -102 dBm/15kHz Noc2 = -102 dBm/15kHz Es1/Noc1 = 10.5dB Es2/Noc2 = -10.5dB During T2: Noc1 = -102 dBm/15kHz Noc2 = -102 dBm/15kHz Es1/Noc1 = 8dB Es2/Noc2 = 8.5dB In signalling: Cell 2 Qrxlevmin = -138dBm/SCS for config. 1 Cell 2 Qrxlevmin = -135dBm/SCS for config. 2 Cell 2 SnonintraSearchP = 50dB Cell 1 Threshx, highP = 48dB Cell 2 Threshserving, lowP = 44dB Cell 2 Threshx, lowP = 50dB SSearchDeltaP = 6dB | During T1: 0dB 0dB 0dB 7.6dB During T2: 0dB 0dB 0dB 0dB In signalling: 16 dB 16 dB 12dB -16dB 18dB -32dB 6dB | During T1: Noc1 = -102 dBm/15kHz Noc2 = -102 dBm/15kHz Es1/Noc1 = 10.5dB Es2/Noc2 = -2.9dB During T2: Noc1 = -102 dBm/15kHz Noc2 = -102 dBm/15kHz Es1/Noc1 = 8dB Es2/Noc2 = 8.5dB In signalling: Qrxlevmin = -124dBm/SCS for config. 1 Qrxlevmin = -121dBm/SCS for config. 2 Cell 2 SnonintraSearchP = 62dB Cell 1 Threshx, highP = 32dB Cell 2 Threshserving, lowP = 62dB Cell 2 Threshx, lowP = 18dB SSearchDeltaP = 12dB |
| 7.1.1.6 NR SA FR2-FR2 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion | During T1: Noc1 = -102 dBm/15kHz Noc2 = -102 dBm/15kHz Es1/Noc1 = 10.5dB Es2/Noc2 = -10.5dB During T2: Noc1 = -102 dBm/15kHz Noc2 = -102 dBm/15kHz Es1/Noc1 = 8dB Es2/Noc2 = 8.5dB In signalling: Cell 2 Qrxlevmin = -138dBm/SCS for config. 1 Cell 2 Qrxlevmin = -135dBm/SCS for config. 2 Cell 2 SnonintraSearchP = 50dB Cell 1 Threshx, highP = 48dB Cell 2 Threshserving, lowP = 44dB Cell 2 Threshx, lowP = 50dB Cell 2 SsearchThresholdP = 29dB | During T1: 0dB 0dB 0dB 7.6dB During T2: 0dB 0dB 0dB 0dB In signalling: 16 dB 16 dB 12dB -16dB 18dB -32dB -17dB | During T1: Noc1 = -102 dBm/15kHz Noc2 = -102 dBm/15kHz Es1/Noc1 = 10.5dB Es2/Noc2 = -2.9dB During T2: Noc1 = -102 dBm/15kHz Noc2 = -102 dBm/15kHz Es1/Noc1 = 8dB Es2/Noc2 = 8.5dB In signalling: Qrxlevmin = -124dBm/SCS for config. 1 Qrxlevmin = -121dBm/SCS for config. 2 Cell 2 SnonintraSearchP = 62dB Cell 1 Threshx, highP = 32dB Cell 2 Threshserving, lowP = 62dB Cell 2 Threshx, lowP = 18dB Cell 2 SsearchThresholdP = 12dB |
| 7.3.1.4 NR SA FR1-FR2 synchronous DAPS handover | During T1: Noc -104.7dBm/15kHz Es2/Noc -infinity During T2-T5: Noc -104.7 dBm/15kHz Es2/Noc 10 dB | During T1: 0 dB 0 dB During T2-T5: 0 dB 0 dB | During T1: Noc -104.7dBm/15kHz Es2/Noc -infinity During T2-T5: Noc -104.7 dBm/15kHz Es2/Noc 10 dB |
| 7.3.1.5 NR SA FR1-FR2 asynchronous DAPS handover | Same as 7.3.1.4 | Same as 7.3.1.4 | Same as 7.3.1.4 |
| 7.3.2.1.1 NR SA FR2 RRC re-establishment | During T1: Noc = -104.7 dBm/15 kHz Ês1 / Noc = 4 dB Ês2 / Noc = 2 dB During T2: Noc = -104.7 dBm/15 kHz Ês1 / Noc = - Infinity Ês2 / Noc = 2 dB During T3: Noc = -104.7 dBm/15 kHz Ês1 / Noc = - Infinity Ês2 / Noc = 2 dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB During T3: 0dB 0dB 0dB | During T1: Noc = -104.7 dBm/15 kHz Ês1 / Noc = 4 dB Ês2 / Noc = 2 dB During T2: Noc = -104.7 dBm/15 kHz Ês1 / Noc = - Infinity Ês2 / Noc = 2 dB During T3: Noc = -104.7 dBm/15 kHz Ês1 / Noc = - Infinity Ês2 / Noc = 2 dB |
| 7.3.2.1.2 NR SA FR2 - FR2 RRC re-establishment | During T1: Noc1 = -92.1 dBm/15 kHz Noc2 = -92.1 dBm/15 kHz Ês1 / Noc1 = 0 dB Ês2 / Noc 2= - Infinity During T2: Noc1 = -92.1 dBm/15 kHz Noc2 = -92.1 dBm/15 kHz Ês1 / Noc1 = - Infinity Ês2 / Noc2 = - Infinity During T3: Noc1 = -92.1 dBm/15 kHz Noc2 = -92.1 dBm/15 kHz Ês1 / Noc1 = - Infinity Ês2 / Noc2 = 0 dB | During T1: -0.2dB -0.2dB 0dB 0dB During T2: -0.2dB -0.2dB 0dB 0dB During T3: -0.2dB -0.2dB 0dB 0dB | During T1: Noc1 = -92.3 dBm/15 kHz Noc2 = -92.3 dBm/15 kHz Ês1 / Noc1 = 0 dB Ês2 / Noc 2= - Infinity During T2: Noc1 = -92.3 dBm/15 kHz Noc2 = -92.3 dBm/15 kHz Ês1 / Noc1 = - Infinity Ês2 / Noc2 = - Infinity During T3: Noc1 = -92.3 dBm/15 kHz Noc2 = -92.3 dBm/15 kHz Ês1 / Noc1 = - Infinity Ês2 / Noc2 = 0 dB |
| 7.3.2.2.1 NR SA FR2 contention based random access | Absolute uplink power: Pint ≥ P ≥ Pmin ±14.0dB PUMAX ≥ P > Pint ±12.0dB Relative uplink power step: Pint ≥ P ≥ Pmin ±6.0dB PUMAX ≥ P > Pint ±4.0dB Uplink timing: 120kHz SCS Te ±3.5*64*Tc | Not applicable due to UL calibration FFS dB FFS dB [48]Tc | Absolute uplink power: ±FFSdB Relative uplink power step: ±(6+FFS)dB, either PRACH being compared ≤ (Pmax – 6dB) ±(4+FFS)dB, both PRACHs being compared > (Pmax – 6dB) Uplink timing: 120kHz SCS Te ±224±[48]*Tc |
| 7.3.2.2.2 NR SA FR2 non-contention based random access | Same as 7.3.2.2.1 | Same as 7.3.2.2.1 | Same as 7.3.2.2.1 |
| 7.3.3.1 NR SA FR2 conditional handover | During T1: Noc -104.7dBm/15kHz Es1/Noc 6 dB Es2/Noc -infinity During T2: Noc -104.7dBm/15kHz Es1/Noc 6 dB Es2/Noc 11dB In signaling: A3-offset = -1dB | During T1 -1.5dB 0dB 0dB During T2 -1.5dB 0dB 0dB In signaling: -1dB | During T1: Noc -106.2 dBm/15kHz Es1/Noc 6 dB Es2/Noc -infinity During T2: Noc -106.2 dBm/15kHz Es1/Noc 6 dB Es2/Noc 11dB In signaling: A3-offset = -2dB |
| 7.4.1.1 SA FR2 UE transmit timing accuracy | Test 1 (no DRX): Uplink timing: ±3*64*Tc for 240 kHz SSB SCS Max step size Tq: 2.5*64*Tc Min adjust rate Tp: 2.5*64*Tc Max adjust rate: 2.5*64*Tc Noc = -112 dBm/15 kHz Ês1 / Noc: +4.0dB Test 2 (with DRX): Uplink timing: ±3*64*Tc for 240 kHz SSB SCS Noc = -112 dBm/15 kHz Ês1 / Noc: +4.0dB | ±0.75*64*Tc +0.625*64Tc -3.725*64*Tc +1.225*64*Tc 0dB 0dB ±0.75*64*Tc 0dB 0dB | Test 1 (no DRX): Uplink timing: ±3.75*64*Tc Max step size Tq: 3.125*64*Tc Min adjust rate: -1.225*64*Tc Max adjust rate: 3.725*64*Tc Noc = -112 dBm/15 kHz Ês1 / Noc: +4.0dB Test 2 (with DRX): Uplink timing: ±3.75*64*Tc Noc = -112 dBm/15 kHz Ês1 / Noc: +4.0dB |
| 7.4.3.1 NR SA FR2 UE timing advance adjustment accuracy | Noc = -112 dBm/15 kHz Ês1 / Noc = 4 dB UE Timing Advance Adjustment Accuracy for 120kHz SCS = ±32 Tc | 0dB 0dB +/- 40 Tc | Noc = -112 dBm/15 kHz Ês1 / Noc = 4 dB UE TAAA for 120kHz SCS = ±72 Tc |
| 7.5.1.1 Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode | During T1: Noc: -92.1 dBm/15kHz Ês1 / Noc: +2 dB Ês2 / Noc: +2 dB During T2: Noc: -92.1dBm/15kHz Ês1 / Noc: -6 dB Ês2 / Noc: -15 dB During T3: Noc: -92.1dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: -15 dB | During T1: -2.7 dB 2.1 dB 2.1 dB During T2: -2.7 dB 2.1 dB 0 dB During T3: -2.7 dB 0 dB 0 dB | During T1: Noc: -94.8 dBm/15kHz Ês1 / Noc: +4.1 dB Ês2 / Noc: +4.1 dB During T2: Noc: -94.8 dBm/15kHz Ês1 / Noc: -3.9 dB Ês2 / Noc: -15 dB During T3: Noc: -94.8 dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: -15 dB |
| 7.5.1.2 Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode | During T1: Noc: -92.1 dBm/15kHz Ês1 / Noc: +2 dB Ês2 / Noc: +2 dB During T2: Noc: -92.1dBm/15kHz Ês1 / Noc: -6 dB Ês2 / Noc: -15 dB During T3: Noc: -92.1dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: -15 dB During T4: Noc: -92.1dBm/15kHz Ês1 / Noc: -4.5 dB Ês2 / Noc: -15 dB During T5: Noc: -92.1dBm/15kHz Ês1 / Noc: +2 dB Ês2 / Noc: -15 dB | During T1: -2.7 dB 2.1 dB 2.1 dB During T2: -2.7 dB 2.1 dB 0 dB During T3: -2.7 dB 0 dB 0 dB During T4: -2.7 dB 0 dB 0 dB During T5: -2.7 dB 2.1 dB 0 dB | During T1: Noc: -94.8 dBm/15kHz Ês1 / Noc: +4.1 dB Ês2 / Noc: +4.1 dB During T2: Noc: -94.8 dBm/15kHz Ês1 / Noc: -3.9 dB Ês2 / Noc: -15 dB During T3: Noc: -94.8 dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: -15 dB During T4: Noc: -94.8 dBm/15kHz Ês1 / Noc: -4.5 dB Ês2 / Noc: -15 dB During T5: Noc: -94.8 dBm/15kHz Ês1 / Noc: 4.1 dB Ês2 / Noc: -15 dB |
| 7.5.1.3 Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode | During T1: Noc: -104.7 dBm/15kHz Ês1 / Noc: +2 dB Ês2 / Noc: +2 dB During T2: Noc: -104.7 dBm/15kHz Ês1 / Noc: -6 dB Ês2 / Noc: -15 dB During T3: Noc: -104.7 dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: -15 dB | During T1: 0 dB 1.3 dB 1.3 dB During T2: 0 dB 1.3 dB -0.4 dB During T3: 0 dB -0.4 dB -0.4 dB | During T1: Noc: -104.7 dBm/15kHz Ês1 / Noc: +3.3 dB Ês2 / Noc: +3.3 dB During T2: Noc: -104.7 dBm/15kHz Ês1 / Noc: -4.7 dB Ês2 / Noc: -15.4 dB During T3: Noc: -104.7 dBm/15kHz Ês1 / Noc: -15.4 dB Ês2 / Noc: -15.4 dB |
| 7.5.1.4 Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode | During T1: Noc: -104.7 dBm/15kHz Ês1 / Noc: +2 dB Ês2 / Noc: +2 dB During T2: Noc: -104.7 dBm/15kHz Ês1 / Noc: -6 dB Ês2 / Noc: -15 dB During T3: Noc: -104.7 dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: -15 dB During T4: Noc: -104.7 dBm/15kHz Ês1 / Noc: -4.5 dB Ês2 / Noc: -15 dB During T5: Noc: -104.7 dBm/15kHz Ês1 / Noc: +2 dB Ês2 / Noc: -14 dB | During T1: 0 dB 1.3 dB 1.3 dB During T2: 0 dB 1.3 dB -0.4 dB During T3: 0 dB -0.4 dB -0.4 dB During T4: 0 dB -0.4 dB -0.4 dB During T5: 0 dB 1.3 dB -0.4 dB | During T1: Noc: -104.7 dBm/15kHz Ês1 / Noc: +3.3 dB Ês2 / Noc: +3.3 dB During T2: Noc: -104.7 dBm/15kHz Ês1 / Noc: -4.7 dB Ês2 / Noc: -15.4 dB During T3: Noc: -104.7 dBm/15kHz Ês1 / Noc: -15.4 dB Ês2 / Noc: -15.4 dB During T4: Noc: -104.7 dBm/15kHz Ês1 / Noc: -4.9 dB Ês2 / Noc: -15.4 dB During T5: Noc: -104.7 dBm/15kHz Ês1 / Noc: 3.3 dB Ês2 / Noc: -15.4 dB |
| 7.5.5.1 NR SA FR2 SSB-based beam failure detection and link recovery in non-DRX | During T1: SNR_SSB q0: 5 dB SNR_SSB q1: 0.2 dB During T2: SNR_SSB q0: -3 dB SNR_SSB q1: 0.2 dB During T3-T5: SNR_SSB q0: -12 dB SNR_SSB q1: 20.2 dB In signaling: rsrp-ThresholdSSB: -95 dBm/120kHz for Config 1 rsrp-ThresholdSSB: -92 dBm/240kHz for Config 2 | +8.70 dB 0 dB +8.70 dB 0 dB 0 dB -0.2 dB -14 dB -14 dB | During T1: SNR_SSB q0: 13.7 dB SNR_SSB q1: 0.2 dB During T2: SNR_SSB q0: 5.7 dB SNR_SSB q1: 0.2 dB During T3-T5: SNR_SSB q0: -12 dB SNR_SSB q1: 20 dB In signaling: rsrp-ThresholdSSB: -109 dBm/120kHz for Config 1 srp-ThresholdSSB: -106 dBm/240kHz for Config 2 |
| 7.5.5.2 NR SA FR2 SSB-based beam failure detection and link recovery in DRX | Same as 7.5.5.1 | Same as 7.5.5.1 | Same as 7.5.5.1 |
| 7.5.5.3 NR SA FR2 CSI-RS-based beam failure detection and link recovery in non-DRX | Noc = -104.7 dBm/120 kHz Ês1 / Noc for CSI-RS set q0 T1: 5 dB T2: -3 dB T3: -12 dB T4: -12 dB T5: -12 dB Ês1 / Noc for CSI-RS set q1 T1: 0.2 dB T2: 0.2 dB T3: 20.2 dB T4: 20.2 dB T5: 20.2 dB rsrp-ThresholdSSB: -95 dBm/SCS kHz | 0 dB 8.7 dB 8.7 dB 0 dB 0 dB 0 dB 0 dB 0 dB -0.2 dB -0.2 dB -0.2 dB -14 dB | Noc = -104.7 dBm/120 kHz Ês1 / Noc for CSI-RS set q0 T1: 13.7 dB T2: 5.7 dB T3: -12 dB T4: -12 dB T5: -12 dB Ês1 / Noc for CSI-RS set q1 T1: 0.2 dB T2: 0.2 dB T3: 20 dB T4: 20 dB T5: 20 dB rsrp-ThresholdSSB: -109.0 dBm/SCS kHz |
| 7.5.5.4 NR SA FR2 CSI-RS-based beam failure detection and link recovery in DRX | Same as 7.5.5.3 | Same as 7.5.5.3 | Same as 7.5.5.3 |
| 7.5.5.5 NR SA FR2 scheduling available restriction during SSB-based beam failure detection and link recovery in non-DRX | Same as 7.5.5.1 | Same as 7.5.5.1 | Same as 7.5.5.1 |
| 7.5.5.6 NR SA FR2 SCell CSI-RS-based beam failure detection and link recovery in non-DRX | Noc = -104.7 dBm/120 kHz Ês2 / Noc for CSI-RS set q0 T1: 5 dB T2: -3 dB T3: -12 dB T4: -12 dB T5: -12 dB Ês2 / Noc for CSI-RS set q1 T1: 0.2 dB T2: 0.2 dB T3: 20.2 dB T4: 20.2 dB T5: 20.2 dB rsrp-ThresholdBFR: -95 dBm/SCS kHz | 0 dB 8.7 dB 8.7 dB 0 dB 0 dB 0 dB 0 dB 0 dB -0.2 dB -0.2 dB -0.2 dB -14 dB | Noc = -104.7 dBm/120 kHz Ês2 / Noc for CSI-RS set q0 T1: 13.7 dB T2: 5.7 dB T3: -12 dB T4: -12 dB T5: -12 dB Ês2 / Noc for CSI-RS set q1 T1: 0.2 dB T2: 0.2 dB T3: 20 dB T4: 20 dB T5: 20 dB rsrp-ThresholdBFR: -109.0 dBm/SCS kHz |
| 7.5.5.7 NR SA FR2 SCell CSI-RS-based beam failure detection and link recovery in DRX | Same as 7.5.5.6 | Same as 7.5.5.6 | Same as 7.5.5.6 |
| 7.5.6.2.1 NR SA FR2 RRC-based DL active BWP switch in non-DRX | During T1: Noc2: -112dBm/15kHz Ês2 / Iot2: +18dB | During T1: 0dB 0dB | During T1: Noc2: -112dBm/15kHz Ês2 / Iot2: +18dB |
| 7.6.1.1 NR SA FR2 event-triggered reporting without gap in non-DRX | During T1: Ês1: -86dBm/120kHz Ês2: -infinity During T2: Ês1: -86dBm/120kHz Ês2: -86dBm/120kHz | During T1: 0dB 0dB During T2: 0dB 0dB | During T1: Ês1: -86dBm/120kHz Ês2: -infinity During T2: Ês1: -86dBm/120kHz Ês2: -86dBm/120kHz |
| 7.6.1.2 NR SA FR2 event-triggered reporting without gap in DRX | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +4.00dB Signalled A3-offset: -6.0 | During T1: -3.5dB 0dB 0dB During T2: -3.5dB 0dB 0dB Signalled A3-offset: -1.0 | During T1: Noc: -101.5dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: -infinity During T2: Noc: -101.5dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +4.00dB Signalled A3-offset: -7.0 |
| 7.6.1.3 NR SA FR2 event-triggered reporting with gap in non-DRX | Same as 7.6.1.1 | Same as 7.6.1.1 | Same as 7.6.1.1 |
| 7.6.1.4 NR SA FR2 event-triggered reporting with gap in DRX | Same as 7.6.1.2 | Same as 7.6.1.2 | Same as 7.6.1.2 |
| 7.6.2.1 NR SA FR2-FR2 event-triggered reporting in non-DRX | During T1: Ês1: -87 dBm/120kHz Ês2: - infinity During T2: Ês1: -87 dBm/120kHz Ês2: -87 dBm/120kHz | During T1: 0dB 0dB During T2: 0dB 0dB | During T1: Ês1: -87 dBm/120kHz Ês2: - infinity During T2: Ês1: -87 dBm/120kHz Ês2: -87 dBm/120kHz |
| 7.6.2.3 NR SA FR2-FR2 event-triggered reporting in non-DRX with SSB time index detection | Same as 7.6.2.1 | Same as 7.6.2.1 | Same as 7.6.2.1 |
| 7.6.2.5 SA event triggered reporting tests for FR2 without SSB time index detection when DRX is not used (PCell in FR1) | During T1: Ês2: - Infinity During T2: Ês2: -87 dBm/120kHz | During T1: 0dB During T2: 0dB | During T1: Ês2: - Infinity During T2: Ês2: -87 dBm/120kHz |
| 7.6.2.6 SA event triggered reporting tests for FR2 without SSB time index detection when DRX is used (PCell in FR1) | During T1: Noc = -104.7 dBm/15 kHz Ês2 / Noc = - Infinity During T2: Noc = -104.7 dBm/15 kHz Ês2 / Noc = 9 dB | During T1: 0dB 0dB During T2: 0dB 0dB | During T1: Noc = -104.7 dBm/15 kHz Ês2 / Noc = - Infinity During T2: Noc = -104.7 dBm/15 kHz Ês2 / Noc = 9 dB |
| 7.6.2.7 SA event triggered reporting tests for FR2 with SSB time index detection when DRX is not used (PCell in FR1) | Same as 5.6.2.5 | Same as 5.6.2.5 | Same as 5.6.2.5 |
| 7.6.2.8 SA event triggered reporting tests for FR2 with SSB time index detection when DRX is used (PCell in FR1) | Same as 5.6.2.6 | Same as 5.6.2.6 | Same as 5.6.2.6 |
| 7.6.3.1 NR SA FR2 SSB-based L1-RSRP measurement in non-DRX | Same as 5.6.3.1 | Same as 5.6.3.1 | Same as 5.6.3.1 |
| 7.6.3.2 NR SA FR2 SSB-based L1-RSRP measurement in DRX | Same as 5.6.3.1 | Same as 5.6.3.1 | Same as 5.6.3.1 |
| 7.6.3.3 NR SA FR2 CSI-RS-based L1-RSRP measurement in non-DRX | Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +9.00dB Reported CSI-RSRP values ± 29dB Reported Differential CSI-RSRP values ± 7dB | 0dB 0dB 0dB Via mapping | Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +9.00dB CSI-RS#1: RSRP_40 to RSRP_99 CSI-RS#0: DIFFRSRP_1 to DIFFRSRP_7 |
| 7.6.3.4 NR SA FR2 CSI-RS-based L1-RSRP measurement in DRX | Same as 7.6.3.3 | Same as 7.6.3.3 | Same as 7.6.3.3 |
| 7.6.4.1 NR SA FR2 SRS-RSRP measurement in non-DRX | Same as 5.6.4.1 | Same as 5.6.4.1 | Same as 5.6.4.1 |
| 7.6.6.1 NR SA FR2 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR measurement accuracy | Same as 5.6.6.1 | Same as 5.6.6.1 | Same as 5.6.6.1 |
| 7.6.6.2 NR SA FR2 SSB based CMR and dedicated IMR L1-SINR measurement accuracy | During T1: Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: -infinity During T2: Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +9.00dB Reported SS-SINR values ± 4.5dB Reported Differential SS-SINR values ± 3.5dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB Via mapping | During T1: Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: -infinity During T2: Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +9.00dB SSB#1: SINR_53 to SINR_72 SSB#0: DIFFSINR_5 to DIFFSINR_12 |
| 7.6.6.3 NR SA FR2 CSI-RS based CMR and dedicated IMR L1-SINR measurement accuracy | Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +9.00dB Reported CSI-SINR values ± 4.0dB Reported Differential CSI-SINR values ± 3.0dB | 0dB 1.5dB 0dB Via mapping | Noc: -105dBm/15kHz Ês0 / Noc: +1.50dB Ês1 / Noc: +9.00dB CSI-RS#1: SINR_54 to SINR_71 CSI-RS#0: DIFFSINR_4 to DIFFSINR_10 |
| 7.7.1.1 NR SA FR2 SS-RSRP measurement accuracy | Test 1: Noc: -91.60dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported absolute RSRP values: ±8dB Reported relative RSRP values: ±6dB For extreme conditions allow ±3dB + FFS MU additionally Test 2: n257 Ês1: -110.0dBm/120kHz n257 Ês2: -110.0dBm/120kHz n260 Ês1: -107.4dBm/120kHz n260 Ês2: -107.4dBm/120kHz Reported absolute RSRP values: ±8dB Reported relative RSRP values: ±6dB For extreme conditions allow ±3dB+ FFS MU additionally | Test 1: -5.80dB 0dB 0.4dB Via Mapping Test 2: 7.70dB 7.70dB 7.70dB 7.70dB Via Mapping | Test 1: Noc: -97.40dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.4dB Cell 1: RSRP_50 to RSRP_108 Cell 2: RSRP_46 to RSRP_103 Cell 2: RSRP_x-12 to RSRP_X+2 Test 2: n257 Ês1: -102.3dBm/120kHz n257 Ês2: -102.3dBm/120kHz n260 Ês1: -99.7dBm/120kHz n260 Ês2: -99.7dBm/120kHz Band dependent. See test case tables in clause 5.7.1.1.5 |
| 7.7.1.2 NR SA FR2-FR2 SS-RSRP measurement accuracy | Test 1 Config 1: Noc1: -90.60dBm/15kHz Noc2: -90.60dBm/15kHz Ês1 / Noc1: +6.0dB Ês2 / Noc2: +6.0dB Reported absolute RSRP values: ±8.0dB For extreme conditions allow ±3dB+ FFS MU additionally Reported relative RSRP values: ±6.0dB For extreme conditions allow ±3dB+ FFS MU additionally Test 2 Config 1: n257 Noc1: -108.13dBm/15kHz n257 Noc2: -113.13dBm/15kHz n260 Noc1: -105.53dBm/15kHz n260 Noc2: -110.53dBm/15kHz Ês1 / Noc1: 17.0dB Ês2 / Noc2: -1.0dB Reported absolute RSRP values: ±8.0dB For extreme conditions allow ±3dB+ FFS MU additionally Reported relative RSRP values: ±6.0dB For extreme conditions allow ±3dB+ FFS MU additionally Test 1 Config 2: Noc1: -93.70dBm/15kHz Noc2: -93.70dBm/15kHz Ês1 / Noc1: +6.0dB Ês2 / Noc2: +6.0dB Reported absolute RSRP values: ±8.0dB For extreme conditions allow ±3dB+ FFS MU additionally Reported relative RSRP values: ±6.0dB For extreme conditions allow ±3dB+ FFS MU additionally Test 2 Config 2: n257 Noc1: -108.13dBm/15kHz n257 Noc2: -113.13dBm/15kHz n260 Noc1: -105.53dBm/15kHz n260 Noc2: -110.53dBm/15kHz Ês1 / Noc1: 17.0dB Ês2 / Noc2: -1.0dB Reported absolute RSRP values: ±8.0dB For extreme conditions allow ±3dB+ FFS MU additionally Reported relative RSRP values: ±6.0dB For extreme conditions allow ±3dB+ FFS MU additionally | Test 1 Config 1: -5.70dB -5.70dB 0dB 0dB Via Mapping Via Mapping Test 2 Config 1: -6.60dB 0dB -6.60dB 0dB 0dB 2dB Via Mapping Via Mapping Test 1 Config 2: -5.60dB -5.60dB 0dB 0dB Via Mapping Via Mapping Test 2 Config 2: -6.60dB 0dB -6.60dB 0dB 0dB 2dB Via Mapping Via Mapping | Test 1 Config 1: Noc1: -96.30dBm/15kHz Noc2: -96.30dBm /15kHz Ês1 / Noc1: +6.0dB Ês2 / Noc2: +6.0dB Cell 1 n257: RSRP_41 to RSRP_109 n260: RSRP_39 to RSRP_109 Cell 2: RSRP_52 to RSRP_109 Cell 2: n257: RSRP_x-15 to RSRP_x+25 n260: RSRP_x-15 to RSRP_x+27 Test 2 Config 1: -114.73dBm/15kHz -113.13dBm/15kHz -112.13dBm/15kHz -110.53dBm/15kHz Ês1 / Noc1: 17.0dB Ês2 / Noc2: 1.0dB Cell 1 n257: RSRP_33 to RSRP_101 n260: RSRP_34 to RSRP_104 Cell 2: n257: RSRP_32 to RSRP_87 n260: RSRP_34 to RSRP_90 Cell 2: n257: RSRP_x-29 to RSRP_x+11 n260: RSRP_x-29 to RSRP_x+13 Test 1 Config 2: Noc1: -99.30dBm/15kHz Noc2: -99.30dBm /15kHz Ês1 / Noc1: +6.0dB Ês2 / Noc2: +6.0dB Cell 1 n257: RSRP_41 to RSRP_109 n260: RSRP_39 to RSRP_109 Cell 2: RSRP_52 to RSRP_109 Cell 2: n257: RSRP_x-15 to RSRP_x+25 n260: RSRP_x-15 to RSRP_x+27 Test 2 Config 2: -114.73dBm/15kHz -113.13dBm/15kHz -112.13dBm/15kHz -110.53dBm/15kHz Ês1 / Noc1: 17.0dB Ês2 / Noc2: 1.0dB Cell 1 n257: RSRP_36 to RSRP_104 n260: RSRP_37 to RSRP_107 Cell 2: n257: RSRP_35 to RSRP_90 n260: RSRP_37 to RSRP_93 Cell 2: n257: RSRP_x-29 to RSRP_x+11 n260: RSRP_x-29 to RSRP_x+13 |
| 7.7.1.3 NR SA FR1-FR2 SS-RSRP measurement accuracy | Test 1: Noc: -90dBm/15kHz Ês2 / Noc: +5.0dB Reported absolute RSRP values: ±8dB For extreme conditions allow ±3dB + FFS MU additionally Test 2: n257 Ês2: -98.2dBm/120kHz n260 Ês2: -93.9dBm/120kHz Reported absolute RSRP values: ±8dBFor extreme conditions allow ±3dB+ FFS MU additionally | Test 1: -5.50dB 0dB Via Mapping Test 2: 5.50dB 5.50dB Via Mapping | Test 1: Noc: -95.50dBm/15kHz Ês2 / Noc: +5dB Band dependent. See test case tables in clause 7.7.1.3.5 Test 2: n257 Ês2: -92.7dBm/120kHz n260 Ês2: -88.4dBm/120kHz Band dependent. See test case tables in clause 7.7.1.3.5 |
| 7.7.2.1 NR SA FR2 SS-RSRQ measurement accuracy | Test 1: Noc: -95.0dBm/15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: +3.0dB Reported absolute RSRQ values: ±2.5dB For extreme conditions allow ±1.5dB+ FFS MU additionally Test 2: Noc: -95.0dBm/15kHz Ês1 / Noc: -3.0dB Ês2 / Noc: -3.0dB Reported absolute RSRQ values: ±3.5dB For extreme conditions allow ±0.5dB+ FFS MU additionally | Test 1: -5.70dB 0dB 0dB Via Mapping Test 2: -1.70dB 0dB 0dB Via Mapping | Test 1: Noc: -100.70dBm/15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: +3.0dB Cell 2: All bands: RSRQ_41 to RSRQ_73 Test 2: Noc: -96.70dBm/15kHz Ês1 / Noc: -3.0dB Ês2 / Noc: -3.0dB Cell 2: n257, n258, n261: RSRQ_35 to RSRQ_71 n260: RSRQ_34 to RSRQ_71 |
| 7.7.2.2 NR SA FR2-FR2 SS-RSRQ measurement accuracy | Test 1: Noc1: -94.03dBm/15kHz Noc2: -94.03dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported absolute RSRQ values: ±2.5dB For extreme conditions allow ±1.5dB+ FFS MU additionally Reported relative RSRQ values: ±3.0dB For extreme conditions allow ±1.0dB+ FFS MU additionally Test 2: Noc1: -94.03dBm/15kHz Noc2: -94.03dBm/15kHz Ês1 / Noc1: -3.0dB Ês2 / Noc2: -3.0dB Reported absolute RSRQ values: ±3.5dB For extreme conditions allow ±0.5dB+ FFS MU additionally Reported relative RSRQ values: ±4.0dB For extreme conditions allow ±0.0dB+ FFS MU additionally | Test 1: -1.90dB -1.90dB 0dB 0dB Via Mapping Via Mapping Test 2: -1.41dB -1.41dB 0dB 0dB Via Mapping Via Mapping | Test 1: Noc1: -95.93dBm/15kHz Noc2: -95.93dBm /15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Cell 1 and Cell 2: RSRQ_41 to RSRQ_73 Cell 2: RSRQ_x-7 to RSRQ_x+7 Test 2: Noc1: -95.44dBm/15kHz Noc2: -95.44dBm /15kHz Ês1 / Noc1: -3.0dB Ês2 / Noc2: -3.0dB Cell 1 and Cell 2: RSRQ_37 to RSRQ_74 Cell 2: RSRQ_x-9 to RSRQ_x+9 |
| 7.7.3.1 NR SA FR2 SS-SINR measurement accuracy | Test 1: Noc: -105.0dBm/15kHz Ês1 / Noc: +4.54dB Ês2 / Noc: +2.66dB Reported absolute SINR values: ±3.5dB For extreme conditions allow ±0.5dB+ FFS MU additionally Test 2: Noc: -105.0dBm/15kHz Ês1 / Noc: -3.0dB Ês2 / Noc: -3.0dB Reported absolute SINR values: ±3.5dB For extreme conditions allow ±0.5dB+ FFS MU additionally | Test 1: 0dB 0dB 0dB Via Mapping Test 2: 0dB 0.2dB 0.2dB Via Mapping | Test 1: Noc: -105.0dBm/15kHz Ês1 / Noc: +4.54dB Ês2 / Noc: +2.66dB Cell 2: n257, n258, n261: SINR_22 to RSRQ_58 n260: SINR_21 to RSRQ_58 Test 2: Noc: -105.0dBm/15kHz Ês1 / Noc: -2.8dB Ês2 / Noc: -2.8dB Cell 2: n257, n258, n261: SINR_18 to RSRQ_55 n260: SINR_18 to RSRQ_54 |
| 7.7.3.2 NR SA FR2-FR2 SS-SINR measurement accuracy | Test 1: Noc1: -105.0dBm/15kHz Noc2: -105.0dBm/15kHz Ês1 / Noc1: -0.5dB Ês2 / Noc2: -0.5dB Reported absolute SINR values: ±3.0dB For extreme conditions allow ±1.0dB+ FFS MU additionally Reported absolute SINR values: ±3.5dB For extreme conditions allow ±0.5dB+ FFS MU additionally Test 2: Noc1: -105.0dBm/15kHz Noc2: -105.0dBm/15kHz Ês1 / Noc1: +11.0dB Ês2 / Noc2: +11.0dB Reported absolute SINR values: ±3.0dB For extreme conditions allow ±1.0dB+ FFS MU additionally Reported absolute SINR values: ±3.5dB For extreme conditions allow ±0.5dB+ FFS MU additionally Test 3: Noc1: -105.0dBm/15kHz Noc2: -105.0dBm/15kHz Ês1 / Noc1: -3.0dB Ês2 / Noc2: -3.0dB Reported absolute SINR values: ±3.5dB For extreme conditions allow ±0.5dB+ FFS MU additionally Reported absolute SINR values: ±4.0dB For extreme conditions allow ±0.0dB+ FFS MU additionally | Test 1: 0dB 0dB 0dB 0dB Via Mapping Via Mapping Test 2: -0.1dB -0.1dB 0dB 0dB Via Mapping Via Mapping Test 3: 0dB 0dB 0.9dB 0.9dB Via Mapping Via Mapping | Test 1: Noc1: -105.0dBm/15kHz Noc2: -105.0dBm/15kHz Ês1 / Noc1: -0.5dB Ês2 / Noc2: -0.5dB Cell 1 and Cell 2: n257, n258, n261: SINR_27 to SINR_62 n260: SINR_27 to SINR_61 Cell 2: SINR_x-8 to SINR_x+8 Test 2: Noc1: -105.1dBm/15kHz Noc2: -105.1dBm/15kHz Ês1 / Noc1: +11.0dB Ês2 / Noc2: +11.0dB Cell 1 and Cell 2: n257, n258, n261: SINR_48 to SINR_87 n260: SINR_48 to SINR_86 Cell 2: SINR_x-17 to SINR_x+17 Test 3: Noc1: -105.0dBm/15kHz Noc2: -105.0dBm/15kHz Ês1 / Noc1: -2.1dB Ês2 / Noc2: -2.1dB Cell 1 and Cell 2: n257, n258, n261: SINR_23 to SINR_60 n260: SINR_22 to SINR_59 Cell 2: SINR_x-9 to SINR_x+9 |
| 7.7.4.1 NR SA FR2 SSB based L1-RSRP measurement accuracy | Test 1: Noc: -100dBm/15kHz SSB#0 Ês / Noc: +10.0dB SSB#1 Ês / Noc: -2.0dB Reported absolute RSRP values: ±8.5dB Reported relative RSRP values: ±6.5dB For extreme conditions allow ±3dB + FFS MU additionally Test 2: n257 SSB#0 Ês: -108.1dBm/120kHz n257 SSB#1 Ês: -105.5dBm/120kHz n260 SSB#0 Ês: -108.1dBm/120kHz n260 SSB#1 Ês: -105.5dBm/120kHz Reported absolute RSRP values: -6.5 +8.5dB Reported relative RSRP values: ±6.5dB For extreme conditions allow ±3dB+ FFS MU additionally | Test 1: -4.1dB 0dB 0.4dB Test 2: 5.7dB | Test 1: Noc: -104.1dBm/15kHz SSB#0 Ês / Noc: +10.0dB SSB#1 Ês / Noc: -1.6dB SSB#0: RSRP_42 to RSRP_101 SSB#1: RSRP_31 to RSRP_89 SSB#1-SSB#0: -9 to -2 Test 2: n257 SSB#0 Ês: -102.4dBm/120kHz n257 SSB#1 Ês: -99.8dBm/120kHz n260 SSB#0 Ês: -102.4dBm/120kHz n260 SSB#1 Ês: -99.8dBm/120kHz SSB#0 and SSB#1 n257: RSRP_27 to RSRP_83 n260: RSRP_30 to RSRP_86 SSB#1-SSB#0: -4 to +4 |
| 7.7.4.2 NR SA FR2 CSI-RS based L1-RSRP measurement accuracy | Test 1: Noc: -100dBm/15kHz CSI-RS#0 Ês / Noc: +10.0dB CSI-RS#1 Ês / Noc: -2.0dB Reported absolute RSRP values: ±8.5dB Reported relative RSRP values: ±6.5dB For extreme conditions allow ±3dB + FFS MU additionally Test 2: n257 CSI-RS#0 Ês: -108.1dBm/120kHz n257 CSI-RS#1 Ês: -105.5dBm/120kHz n260 CSI-RS#0 Ês: -108.1dBm/120kHz n260 CSI-RS#1 Ês: -105.5dBm/120kHz Reported absolute RSRP values: -6.5 +8.5dB Reported relative RSRP values: ±6.5dB For extreme conditions allow ±3dB+ FFS MU additionally | Test 1: -4.1dB 0dB 0.2dB Test 2: 5.7dB | Test 1: Noc: -104.1dBm/15kHz CSI-RS#0 Ês / Noc: +10.0dB CSI-RS#1 Ês / Noc: -1.8dB CSI-RS#0: RSRP_42 to RSRP_101 CSI-RS#1: RSRP_30 to RSRP_89 CSI-RS#1-CSI-RS#0: -9 to -2 Test 2: n257 CSI-RS#0 Ês: -102.4dBm/120kHz n257 CSI-RS#1 Ês: -99.8dBm/120kHz n260 CSI-RS#0 Ês: -102.4dBm/120kHz n260 CSI-RS#1 Ês: -99.8dBm/120kHz CSI-RS#0 and CSI-RS#1 n257: RSRP_27 to RSRP_83 n260: RSRP_30 to RSRP_86 CSI-RS#1-CSI-RS#0: -4 to +4 |
| 7.7.5.1 NR SA FR2 SRS-RSRP measurement accuracy | Same as 5.7.5.1 | Same as 5.7.5.1 | Same as 5.7.5.1 |
| 7.7.6.1 NR SA FR2 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR measurement accuracy | Noc: -100dBm/15kHz Ês0 / Noc: 10.00dB Ês1 / Noc: -2.00dB Reported CSI-SINR values ± 5.5dB Reported Differential CSI-SINR values ± 4.5dB | -4.1dB 0dB 0.2dB Via mapping | Noc: -104.1dBm/15kHz Ês0 / Noc: 10.00dB Ês1 / Noc: -1.8dB CSI-RS#0: SINR_53 to SINR_76 CSI-RS#0: DIFFSINR_6 to DIFFSINR_15 |
| 7.7.6.2 NR SA FR2 SSB based CMR and dedicated IMR L1-SINR measurement accuracy | Noc: -100dBm/15kHz Ês0 / Noc: 10.00dB Ês1 / Noc: -2.00dB Reported SS-SINR values ± 4.5dB Reported Differential SS-SINR values ± 3.5dB | -4.1dB 0dB 0.2dB Via mapping | Noc: -104.1dBm/15kHz Ês0 / Noc: 10.00dB Ês1 / Noc: -1.8dB SSB#0: SINR_55 to SINR_74 SSB#0: DIFFSINR_7 to DIFFSINR_15 |
| 7.7.6.3 NR SA FR2 CSI-RS based CMR and dedicated IMR L1-SINR measurement accuracy | Noc: -100dBm/15kHz Ês0 / Noc: 10.00dB Ês1 / Noc: 0.00dB Reported CSI-SINR values ± 4.5dB Reported Differential CSI-SINR values ± 3.5dB | -4.1dB 0dB 0.2dB Via mapping | Noc: -104.1dBm/15kHz Ês0 / Noc: 10.00dB Ês1 / Noc: 0.2dB CSI-RS#0: SINR_55 to SINR_74 CSI-RS#0: DIFFSINR_5 to DIFFSINR_13 |

Table F.1.3.2-5: Derivation of test requirements for E-UTRA – NR inter-RAT test cases with E-UTRA serving cell RRM tests

| Test | Minimum requirement in TS 38.133 [6] | Test tolerance (TT) | Test requirement in TS 38.533 |
| --- | --- | --- | --- |
| 8.2.1.1 E-UTRA – NR FR1 cell re-selection to higher priority NR target cell | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -4dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -infinity During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: +12dB | During T1: 0dB -1.90dB 0dB 0.35dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 1.55dB 1.55dB | During T1: Noc1: -98dBm/15kHz Noc2: -99.90dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -3.65dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -infinity During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +15.55dB Ês2 / Noc2: +13.55dB |
| 8.2.1.2 E-UTRA – NR FR1 Cell reselection to lower priority NR target Cell in FR1 for UE configured with highSpeedInterRAT-NR-r16 | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -4dB Ês2 / Noc2: 14dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +12dB Ês2 / Noc2: +14dB | During T1: -1.90dB -1.90dB 0.35dB 3.45dB During T2: -1.90dB -1.90dB 3.45dB 0dB | During T1: Noc1: -99.9dBm/15kHz Noc2: -99.9dBm/15kHz Ês1 / Noc1: -3.65dB Ês2 / Noc2: 17.45dB During T2: Noc1: -99.9dBm/15kHz Noc2: -99.9dBm/15kHz Ês1 / Noc1: +15.45dB Ês2 / Noc2: +14dB |
| 8.2.2.1 E-UTRA – NR FR1 Early Measurement Reporting | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -4dB During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -4dB Reported SS-RSRP and SS-RSRQ for NR Cell 2 | FFS | FFS |
| 8.2.2.2 E-UTRA – NR FR2 Early Measurement Reporting | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -4dB During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -4dB Signalled Thresh_X_HighP: 48dB Reported SS-RSRP and SS-RSRQ for NR Cell 2 | FFS | FFS |
| 8.3.1.1 E-UTRA – NR FR1 handover with known target cell | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +7dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +7dB Ês2 / Noc2: 0dB During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +7dB Ês2 / Noc2: 0dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB -1.55dB +1.55dB During T3: 0dB 0dB -1.55dB +1.55dB | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +7dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +5.45dB Ês2 / Noc2: +1.55dB During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +5.45dB Ês2 / Noc2: +1.55dB |
| 8.4.1.1 E-UTRA – NR FR1 SFTD measurement delay in non-DRX | During T1: Noc1: -104dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +4dB | During T1: 0dB 0dB 0dB 0dB | During T1: Noc1: -104dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +4dB |
| 8.4.1.2 E-UTRA – NR FR1 SFTD measurement delay in DRX | Same as 8.4.1.1 | Same as 8.4.1.1 | Same as 8.4.1.1 |
| 8.4.2.1 E-UTRA event-triggered reporting of a NR FR1 neighbour cell without SSB time index detection in non-DRX | During T1: Noc1: -104dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: -infinity During T2: Noc1: -104dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +7dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB -1.65dB +1.65dB | During T1: Noc1: -104dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +15.35dB Ês2 / Noc2: +8.65dB |
| 8.4.2.2 E-UTRA event-triggered reporting of a NR FR1 neighbour cell without SSB time index detection in DRX | Same as 8.4.2.1 | Same as 8.4.2.1 | Same as 8.4.2.1 |
| 8.4.2.3 E-UTRA event-triggered reporting of a NR FR1 neighbour cell with SSB time index detection in non-DRX | Same as 8.4.2.1 | Same as 8.4.2.1 | Same as 8.4.2.1 |
| 8.4.2.4 E-UTRA event-triggered reporting of a NR FR1 neighbour cell with SSB time index detection in DRX | Same as 8.4.2.1 | Same as 8.4.2.1 | Same as 8.4.2.1 |
| 8.4.2.9 E-UTRA – NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection in DRX for UE configured with highSpeedInterRAT-NR-r16 | During T1: Noc1: -104dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: -infinity During T2: Noc1: -104dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +7dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB | During T1: Noc1: -104dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: -infinity During T2: Noc1: -104dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +7dB |
| 8.5.1.1 E-UTRA – NR FR1 SFTD measurement accuracy | Test 1 Ês1 / Noc1: -3dB Ês2 / Noc2: -3dB | Test 1: +0.8dB +0.3dB | Test 1: Ês1 / Noc1: -2.2dB Ês2 / Noc2: -2.7dB |
| 8.5.2.1.1.1 E-UTRA SS-RSRP absolute measurement accuracy of a NR FR1 neighbour cell |  |  |  |
| Test Configuration 1,2,4,5 | Test 1: Noc1: -94.65 dBm/15kHz Ês1 / Noc1: +10.0dB Reported RSRP values: ±8dB (±3.0dB additionally for extreme conditions) Test 2: Noc1: -117dBm/15kHz + ΔBG_offset Ês1 / Noc1: -4.0dB Reported RSRP values: ±4.5dB (±4.5dB additionally for extreme conditions) | Test 1: 0dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping | Test 1: Noc1: -94.65dBm/15kHz Ês1 / Noc1: +10.0dB RSRP_62 to RSRP_82 Test 2: Noc1: -117dBm/15kHz + ΔBG_offset Ês1 / Noc1: -3.2dB RSRP_30 to RSRP_43 RSRP_31 to RSRP_43 RSRP_31 to RSRP_44 RSRP_32 to RSRP_44 RSRP_32 to RSRP_45 RSRP_33 to RSRP_45 RSRP_33 to RSRP_46 RSRP_34 to RSRP_46 depending on operating band |
| Test Configuration 3,6 | Test 1: Noc1: -94.65 dBm/15kHz Ês1 / Noc1: +10.0dB Reported RSRP values: ±8dB (±3.0dB additionally for extreme conditions) Test 2: Noc1: -117dBm/15kHz + ΔBG_offset Ês1 / Noc1: -4.0dB Reported RSRP values: ±4.5dB (±4.5dB additionally for extreme conditions) | Test 1: -1.35dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping | Test 1: Noc1: -96.00dBm/15kHz Ês1 / Noc1: +10.0dB RSRP_64 to RSRP_83 Test 2: Noc1: -117dBm/15kHz + ΔBG_offset Ês1 / Noc1: -3.2dB RSRP_33 to RSRP_46 RSRP_34 to RSRP_46 RSRP_34 to RSRP_47 RSRP_35 to RSRP_47 RSRP_35 to RSRP_48 RSRP_36 to RSRP_48 RSRP_36 to RSRP_49 RSRP_37 to RSRP_49 depending on operating band |
| 8.5.2.1.2 E-UTRA SS-RSRP absolute measurement accuracy of a NR FR2 neighbour cell | Test 1: Noc: -105dBm/15kHz Ês / Noc 11dB Reported RSRP values: ±8dB (±3dB additionally for extreme conditions) Test 2: Ês FR2a PC3: -109.1dBm/SCS Reported RSRP values: ±8dB Ês FR2b PC3: -106.5dBm/SCS Reported RSRP values: ±8dB Ês FR2c PC3: -105.5dBm/SCS Reported RSRP values: ±8dB (±3dB additionally for extreme conditions in all frequency ranges) | Test 1: -0.1dB 0dB Via mapping Test 2: +5.65dB Via mapping +5.65dB Via mapping +5.65dB Via mapping | Test 1: Noc: -105.1dBm/15kHz Ês / Noc: +11.0dB RSRP_48 to RSRP_105 Test 2: Ês: -103.45dBm/SCS RSRP_29 to RSRP_87 Ês: -100.85dBm/SCS RSRP_32 to RSRP_89 Ês: -99.85dBm/SCS RSRP_33 to RSRP_90 |
| 8.5.2.2.1 E-UTRA SS-RSRQ absolute measurement accuracy of a NR FR1 neighbour cell |  |  |  |
| Test Configuration 1,2,4,5 | Test 1: Noc1: -80.18 dBm/15kHz Ês1 / Noc1: -1.75dB Reported RSRQ values: ±2.5dB (±1.5dB additionally for extreme conditions) Test 2: Noc1: -106dBm/15kHz Ês1 / Noc1: -1.75dB Reported RSRQ values: ±2.5dB (±1.5dB additionally for extreme conditions) Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Ês1 / Noc1: -1.75dB Reported RSRQ values: ±2.5dB (±1.5dB additionally for extreme conditions) | Test 1: -1.5dB 0dB Via mapping Test 2: 0dB 0dB Via mapping Test 3: 0dB 0dB Via mapping | Test 1: Noc1: -81.68dBm/15kHz Ês1 / Noc1: -1.75dB RSRQ_51 to RSRQ_63 Test 2: Noc1: -106dBm/15kHz Ês1 / Noc1: -1.75dB RSRQ_51 to RSRQ_63 Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Ês1 / Noc1: -1.75dB RSRQ_51 to RSRQ_63 |
| Test Configuration 3,6 | Test 1: Noc1: -86.27 dBm/15kHz Ês1 / Noc1: -1.75dB Reported RSRQ values: ±2.5dB (±1.5dB additionally for extreme conditions) Test 2: Noc1: -113dBm/15kHz Ês1 / Noc1: -1.75dB Reported RSRQ values: ±2.5dB (±1.5dB additionally for extreme conditions) Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Ês1 / Noc1: -1.75dB Reported RSRQ values: ±2.5dB (±1.5dB additionally for extreme conditions) | Test 1: -1.53dB 0dB Via mapping Test 2: 0dB 0dB Via mapping Test 3: 0dB 0dB Via mapping | Test 1: Noc1: -87.80dBm/15kHz Ês1 / Noc1: -1.75dB RSRQ_51 to RSRQ_63 Test 2: Noc1: -113dBm/15kHz Ês1 / Noc1: -1.75dB RSRQ_51 to RSRQ_63 Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Ês1 / Noc1: -1.75dB RSRQ_51 to RSRQ_63 |
| 8.5.2.2.2 E-UTRA SS-RSRQ absolute measurement accuracy of a NR FR2 neighbour cell | Test 1: Noc1: -105dBm/15kHz Ês1 / Noc1: -0.5dB Reported RSRQ values: ±2.5dB (±1.5dB additionally for extreme conditions) Test 2: Noc1 FR2a PC3: -115.1dBm/15kHz Noc1 FR2b PC3: -112.5dBm/15kHz Noc1 FR2c PC3: -111.5dBm/15kHz Ês1 / Noc1: -1.75dB Reported RSRQ values: ±2.5dB (±1.5dB additionally for extreme conditions in all frequency ranges) | Test 1: 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 5.3dB Via mapping | Test 1: Noc1: -105dBm/15kHz Ês1 / Noc1: -0.5dB RSRQ_23 to RSRQ_95 Test 2: Noc1: -115.1dBm/15kHz Noc1: -112.5dBm/15kHz Noc1: -111.5dBm/SCS Ês1 / Noc1: 3.55dB RSRQ_26 to RSRQ_98 |
| 8.5.2.3.1 E-UTRA SS-SINR absolute measurement accuracy of a NR FR1 neighbour cell | Test 1: Noc1: -88 dBm/15kHz Ês1 / Noc1: -1.75dB Reported SINR values: ±3dB Test 2: Noc1: -108.5dBm/15kHz Ês1 / Noc1: 20dB Reported SINR values: ±3dB Test 3: Noc1: -119.5dBm/15kHz + ΔBG_offset Ês1 / Noc1: -4dB Reported SINR values: ±3.5dB | Test 1: 0dB 0dB Via mapping Test 2: 0dB 0dB Via mapping Test 3: 0dB 0.8dB Via mapping | Test 1: Noc1: -88 dBm/15kHz Ês1 / Noc1: -1.75dB SINR_35 to SINR_51 Test 2: Noc1: -108.5dBm/15kHz Ês1 / Noc1: 20dB SINR_79 to SINR_94 Test 3: Noc1: -119.5dBm/15kHz + ΔBG_offset Ês1 / Noc1: -3.2dB SINR_32 to SINR_49 |
| 8.5.2.3.2 E-UTRA SS-SINR absolute measurement accuracy of a NR FR2 neighbour cell | Test 1: Noc1: -105dBm/15kHz Ês1 / Noc1: -0.5dB Reported SINR values: ±3dB (±1dB additionally for extreme conditions) Test 2: Noc1: -105dBm/15kHz Ês1 / Noc1: 11dB Reported SINR values: ±3dB (±1dB additionally for extreme conditions) Test 3: Noc1 FR2a PC3: -115.1dBm/15kHz Noc1 FR2b PC3: -112.5dBm/15kHz Noc1 FR2c PC3: -111.5dBm/15kHz Ês1 / Noc1: -1dB Reported SINR values: ±3dB (±1dB additionally for extreme conditions in all frequency ranges) | Test 1: 0dB 0dB Via mapping Test 2: -0.1dB 0dB Via mapping Test 3: 0dB 0dB 0dB 4.6dB Via mapping | Test 1: Noc1: -105dBm/15kHz Ês1 / Noc1: -0.5dB SINR_9 to SINR_82 Test 2: Noc1: -105.1dBm/15kHz Ês1 / Noc1: 11dB SINR_32 to SINR_105 Test 3: Noc1: -115.1dBm/15kHz Noc1: -112.5dBm/15kHz Noc1: -111.5dBm/SCS Ês1 / Noc1: 3.6dB SINR_17 to SINR_90 |

Table F.1.3.2-6: Derivation of test requirements for FR1 NR sidelink RRM tests

| Test | Minimum requirement in TS 38.133 [6] | Test tolerance (TT) | Test requirement in TS 38.533 |
| --- | --- | --- | --- |
| 9.1.1.1 NR SA FR1 UE transmit timing accuracy for GNSS as synchronization reference source | Sidelink transmit timing error limit: ±12*64 Tc | ±3*64 Tc | ±15*64 Tc |
| 9.1.1.2 NR SA FR1 UE transmit timing accuracy for SyncRef UE as synchronization reference source | Sidelink transmit timing error limit: ±8*64 Tc Noc = -95 dBm/30 kHz Es1/Noc = 3 dB | ±2*64 Tc 0dB 0dB | ±10*64 Tc Noc = -95 dBm/30 kHz Es1/Noc = 3 dB |
| 9.1.1.3 NR SA FR1 UE transmit timing accuracy for FR1 NR cell as synchronization reference source | Uu Test Configuration 1, 2 |  |  |
|  | Sidelink transmit timing error limit: ±12*64 Tc Noc = -98 dBm/15 kHz Es1/Noc = 3 dB | ±3*64 Tc 0dB 0dB | ±15*64 Tc Noc = -98 dBm/15 kHz Es1/Noc = 3 dB |
|  | Uu Test Configuration 3 |  |  |
|  | Sidelink transmit timing error limit: ±10*64 Tc Noc = -98 dBm/15 kHz Es1/Noc = 3 dB | ±3*64 Tc 0dB 0dB | ±13*64 Tc Noc = -98 dBm/15 kHz Es1/Noc = 3 dB |
| 9.1.2.1 NR SA FR1 initiation/cease of S-SSB transmission for FR1 NR cell as synchronization reference source | Uu Test Configuration 1, 2 |  |  |
|  | During T1: Noc = -110 dBm/15 kHz Es1/Noc = 4.50 dB During T2: Noc = -110 dBm/15 kHz Es1/Noc = -4.50 dB During T3: Noc = -110 dBm/15 kHz Es1/Noc = 4.50 dB In signaling: syncTxThreshIC = -110.00 dBm/SCS | -1.55dB 3.10dB -1.55dB 0dB -1.55dB 3.10dB 0dB | During T1: Noc = -111.55 dBm/15 kHz Es1/Noc = 7.60 dB During T2: Noc = -111.55 dBm/15 kHz Es1/Noc = -4.50 dB During T3: Noc = -111.55 dBm/15 kHz Es1/Noc = 7.60 dB syncTxThreshIC = -110.00 dBm/SCS |
|  | Uu Test Configuration 3 |  |  |
|  | During T1: Noc = -110 dBm/15 kHz Es1/Noc = 4.50 dB During T2: Noc = -110 dBm/15 kHz Es1/Noc = -4.50 dB During T3: Noc = -110 dBm/15 kHz Es1/Noc = 4.50 dB In signaling: syncTxThreshIC = -110.00 dBm/SCS | -4.55dB 3.10dB -4.55dB 0dB -4.55dB 3.10dB 0dB | During T1: Noc = -114.55 dBm/15 kHz Es1/Noc = 7.60 dB During T2: Noc = -114.55 dBm/15 kHz Es1/Noc = -4.50 dB During T3: Noc = -114.55 dBm/15 kHz Es1/Noc = 7.60 dB syncTxThreshIC = -110.00 dBm/SCS |
| 9.1.2.2 NR SA FR1 initiation/cease of S-SSB transmission for SyncRef UE as synchronization reference source | During T1: Noc = -98 dBm/30 kHz Es1/Noc = 5.50 dB During T2: Noc = -98 dBm/30 kHz Es1/Noc = -3.50 dB During T3: Noc = -98 dBm/30 kHz Es1/Noc = 5.50 dB In signaling: syncTxThreshOoC = -100.00 dBm/SCS | -1.35dB 0dB -4.55dB 0dB -1.35dB 0dB 0dB | During T1: Noc = -99.35 dBm/30 kHz Es1/Noc = 5.50 dB During T2: Noc = -102.55 dBm/30 kHz Es1/Noc = -3.50 dB During T3: Noc = -99.35 dBm/30 kHz Es1/Noc = 5.50 dB syncTxThreshOoC = -100.00 dBm/SCS |
| 9.1.3.1 NR SA FR1 synchronization reference selection/reselection for GNSS configured as the highest priority synchronization reference source | During T1: Noc = -95 dBm/30 kHz Es1/Noc = 0 dB Es2/Noc = -infinity Es3/Noc = -infinity During T2: Noc = -95 dBm/30 kHz Es1/Noc = 0 dB Es2/Noc = 0 dB Es3/Noc = -infinity During T3: Noc = -95 dBm/30 kHz Es1/Noc = 0 dB Es2/Noc = 0 dB Es3/Noc = 3 dB | 0dB 0.35dB N/A N/A 0dB 0dB 0.35dB N/A 0dB 0dB 0dB 0.35dB | During T1: Noc = -95 dBm/30 kHz Es1/Noc = 0.35 dB Es2/Noc = -infinity Es3/Noc = -infinity During T2: Noc = -95 dBm/30 kHz Es1/Noc = 0 dB Es2/Noc = 0.35 dB Es3/Noc = -infinity During T3: Noc = -95 dBm/30 kHz Es1/Noc = 0 dB Es2/Noc = 0 dB Es3/Noc = 3.35 dB |
| 9.1.3.2 NR SA FR1 synchronization reference selection/reselection for FR1 NR Cell configured as the highest priority synchronization reference source | During T1: Noc = -95 dBm/30 kHz Es1/Noc = -infinity Es2/Noc = -infinity During T2: Noc = -95 dBm/30 kHz Es1/Noc = 0 dB Es2/Noc = -infinity During T3: Noc = -95 dBm/30 kHz Es1/Noc = 0 dB Es2/Noc = 0 dB | 0dB N/A N/A 0dB 0.35dB N/A 0dB 0dB 0.35dB | During T1: Noc = -95 dBm/30 kHz Es1/Noc = -infinity Es2/Noc = -infinity During T2: Noc = -95 dBm/30 kHz Es1/Noc = 0.35 dB Es2/Noc = -infinity During T3: Noc = -95 dBm/30 kHz Es1/Noc = 0 dB Es2/Noc = 0.35 dB |
| 9.1.4.1 NR SA FR1 L1 SL-RSRP measurement for autonomous resource selection/reselection | During T1: Noc = -110 dBm/30 kHz Esi/Noc = 10 dB, i = 20~29 Esi/Noc = 0 dB, i = 0~19, 30~49 During T2: Noc = -121 dBm/30 kHz Esi/Noc = 10 dB, i = 20~29 Esi/Noc = 20 dB, i = 0~19, 30~49 In signaling: SL-Thres-RSRP = -106 dBm/SCS | 0dB 10.55dB 0.35dB 0dB 0dB 10.55dB 6dB | During T1: Noc = -110 dBm/30 kHz Esi/Noc = 20.55 dB, i = 20~29 Esi/Noc = 0.35 dB, i = 0~19, 30~49 During T2: Noc = -121 dBm/30 kHz Esi/Noc = 10 dB, i = 20~29 Esi/Noc = 30.55 dB, i = 0~19, 30~49 In signaling: SL-Thres-RSRP = -100 dBm/SCS |
| 9.1.4.2 NR SA FR1 L1 SL-RSRP measurement for resource pre-emption | During T1: Noc = -infinity Es1/Noc = -infinity During T2: Noc = -100 dBm/30 kHz Es1/Noc = 5 dB In signaling: SL-Thres-RSRP = -106 dBm/SCS | N/A N/A 0dB 0dB 0dB | During T1: Noc = -infinity Es1/Noc = -infinity During T2: Noc = -100 dBm/30 kHz Es1/Noc = 5 dB In signaling: SL-Thres-RSRP = -106 dBm/SCS |
| 9.1.4.3 NR SA FR1 L1 SL-RSRP measurement for resource re-evaluation | During T1: Noc = -103 dBm/30 kHz Esi/Noc = 12dB, i = 0~29 Esi/Noc = 2 dB, i = 30~49 Esi/Noc = 25.50 dB, i = 50~64, 85~99 Esi/Noc = 12 dB, i = 65~84 Esi/Noc = -infinity, i = 100~129 During T2: Noc = -103 dBm/30 kHz Esi/Noc = -infinity, i = 0~29 Esi/Noc = 2 dB, i = 30~49 Esi/Noc = 25.5 dB, i = 50~64, 85~99 Esi/Noc = 12 dB, i = 65~84 Esi/Noc = 25.5dB, i = 100~129 In signaling: SL-Thres-RSRP = -96 dBm/SCS | -7.05dB 2.1dB 0dB 7.1dB 2.1dB N/A -7.05dB N/A 0dB 7.1dB 2.1dB 7.55dB -6dB | During T1: Noc = -110.05 dBm/30 kHz Esi/Noc = 14.1 dB, i = 0~29 Esi/Noc = 2 dB, i = 30~49 Esi/Noc = 32.60 dB, i = 50~64, 85~99 Esi/Noc = 14.1 dB, i = 65~84 Esi/Noc = -infinity, i = 100~129 During T2: Noc = -110.05 dBm/30 kHz Esi/Noc = -infinity, i = 0~29 Esi/Noc = 2 dB, i = 30~49 Esi/Noc = 32.60 dB, i = 50~64, 85~99 Esi/Noc = 14.1 dB, i = 65~84 Esi/Noc = 33.05dB, i = 100~129 In signaling: SL-Thres-RSRP = -102 dBm/SCS |
| 9.1.5.1 NR SA FR1 congestion control measurement for concurrent operation | During T1: Noc = -103 dBm/30kHz Esi/Noc = 4.35 dB, i = 0,1,2,3 During T2: Noc = -103 dBm/30kHz Esi/Noc = 10.32 dB, i = 0,1,2,3 In signaling: threshS-RSSI-CBR = -74 dBm/subchannel | -1.55dB 0dB -1.55dB 3.30dB 0dB | During T1: Noc = -104.55 dBm/30kHz Esi/Noc = 4.35 dB, i = 0,1,2,3 During T2: Noc = -104.55 dBm/30kHz Esi/Noc = 13.62 dB, i = 0,1,2,3 In signaling: threshS-RSSI-CBR = -74 dBm/subchannel |
| 9.1.5.2 NR SA FR1 congestion control measurement for PC5-only operation | Same as 9.1.5.1 | Same as 9.1.5.1 | Same as 9.1.5.1 |
| 9.1.6.1 NR SA FR1 interruption to WAN due to NR sidelink communication | N/A | N/A | N/A |

Table F.1.3.2-7: Derivation of test requirements for NR SA FR1 RRM tests for RedCap

| Test | Minimum requirement in TS 38.133 [6] | Test tolerance (TT) | Test requirement in TS 38.533 |
| --- | --- | --- | --- |
| 16.3.1.8 NR - E-UTRA handover for 2Rx UE | Same as 6.3.1.4 | Same as 6.3.1.4 | Same as 6.3.1.4 |
| 16.3.1.10 NR - E-UTRA handover with unknown target cell for 2 Rx UE | Same as 6.3.1.5 | Same as 6.3.1.5 | Same as 6.3.1.5 |
| 16.3.2.1.2 NR SA FR1 Intra-frequency RRC Re-establishment in FR1 for 2 Rx UE | Same as 6.3.2.1.1 | Same as 6.3.2.1.1 | Same as 6.3.2.1.1 |
| 16.3.2.1.4 NR SA FR1-FR1 Inter-frequency RRC Re-establishment in FR1 for 2 Rx UE | Same as 6.3.2.1.2 | Same as 6.3.2.1.2 | Same as 6.3.2.1.2 |
| 16.3.2.1.6 NR SA FR1 Intra-frequency RRC Re-establishment in FR1 for 2 Rx UE without serving cell timing | Same as 6.3.2.1.3 | Same as 6.3.2.1.3 | Same as 6.3.2.1.3 |
| 16.3.2.2.2 NR SA FR1 4-step RA type contention based random access test in FR1 for NR standalone for 2 Rx UE | Same as 6.3.2.2.1 | Same as 6.3.2.2.1 | Same as 6.3.2.2.1 |
| 16.3.2.2.4 NR SA FR1 4-step RA type non-contention based random access test in FR1 for NR standalone for 2 Rx UE | Same as 6.3.2.2.1 | Same as 6.3.2.2.1 | Same as 6.3.2.2.1 |
| 16.3.2.2.6 NR SA FR1 2-step RA type contention based random access test in FR1 for NR standalone for 2 Rx UE | Same as 6.3.2.2.1 | Same as 6.3.2.2.1 | Same as 6.3.2.2.1 |
| 16.3.2.2.8 NR SA FR1 2-step RA type non-contention based test in FR1 for NR standalone for 2 RX UE | Same as 6.3.2.2.1 | Same as 6.3.2.2.1 | Same as 6.3.2.2.1 |
| 16.3.2.3.2 NR SA FR1-FR1 Redirection from NR in FR1 to NR in FR1 for 2 Rx UE | Same as 6.3.2.3.1 | Same as 6.3.2.3.1 | Same as 6.3.2.3.1 |
| 16.3.2.3.4 NR SA FR1-FR1 Redirection from NR in FR1 to E-UTRAN for 2 Rx UE | Same as 6.3.2.3.2 | Same as 6.3.2.3.2 | Same as 6.3.2.3.2 |
| 16.4.1.1 NR SA FR1 NR UE Transmit Timing Test for FR1 for 1Rx RedCap UE | Same as 6.4.1.1 | Same as 6.4.1.1 | Same as 6.4.1.1 |
| 16.4.1.2 NR SA FR1 NR UE Transmit Timing Test for FR1 for 2Rx RedCap UE | Same as 6.4.1.1 | Same as 6.4.1.1 | Same as 6.4.1.1 |
| 16.5.1.2 NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 2 Rx UE | Same as 6.5.1.1 with the exception : - 4Rx is not considered | Same as 6.5.1.1 with the exception : - 4Rx is not considered | Same as 6.5.1.1 with the exception : - 4Rx is not considered |
| 16.5.1.4 NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 2 Rx UE | Same as 6.5.1.2 with the exception : - 4Rx is not considered | Same as 6.5.1.2 with the exception : - 4Rx is not considered | Same as 6.5.1.2 with the exception : - 4Rx is not considered |
| 16.5.1.8 NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 2 Rx UE | Same as 6.5.1.2 with the exception : - 4Rx is not considered | Same as 6.5.1.2 with the exception : - 4Rx is not considered | Same as 6.5.1.2 with the exception : - 4Rx is not considered |
| 16.5.2.2 NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode for 2 Rx UE | Same as 6.5.5.1 with the exception : - 4Rx is not considered | Same as 6.5.5.1 with the exception : - 4Rx is not considered | Same as 6.5.5.1 with the exception : - 4Rx is not considered |
| 16.6.1.4 NR SA FR1 Event triggered reporting tests without gap under DRX for 2 Rx UE | Same as 6.6.1.1 | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 16.6.1.8 NR SA FR1 Event triggered reporting tests with per-UE gaps under DRX for 2 Rx UE | Same as 6.6.1.1 | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 16.6.1.12 NR SA FR1 Event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading for 2 Rx UE | Same as 6.6.1.1 | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 16.6.4.2 NR SA FR1 SSB based L1-RSRP measurement when DRX is not used for 2 Rx UE | Same as 6.6.4.1 | Same as 6.6.4.1 | Same as 6.6.4.1 with following excpetions: - Test requirements for configuration 1,2,4,5 also apply to configuration 7,8 |
| 16.6.4.4 NR SA FR1 SSB based L1-RSRP measurement when DRX is used for 2 Rx UE | Same as 6.6.4.1 | Same as 6.6.4.1 | Same as 6.6.4.1 with following excpetions: - Test requirements for configuration 1,2,4,5 also apply to configuration 7,8 |

Table F.1.3.2-8: Derivation of test requirements for NR SA FR2 RRM tests for RedCap

| Test | Minimum requirement in TS 38.133 [6] | Test tolerance (TT) | Test requirement in TS 38.533 |
| --- | --- | --- | --- |
| 17.3.2.1.1 NR SA FR2 Intra-frequency RRC Re-establishment in FR2 | Same as 7.3.2.1.1 | Same as 7.3.2.1.1 | Same as 7.3.2.1.1 |
| 17.3.2.1.2 NR SA FR2-FR2 Inter-frequency RRC Re-establishment in FR2 | Same as 7.3.2.1.2 | Same as 7.3.2.1.2 | Same as 7.3.2.1.2 |
| 17.5.1.1 NR SA FR2 Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode | Same as 7.5.1.1 | Same as 7.5.1.1 | Same as 7.5.1.1 |
| 17.5.1.2 NR SA FR2 Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode | Same as 7.5.1.2 | Same as 7.5.1.2 | Same as 7.5.1.2 |
| 17.5.1.4 NR SA FR2 Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode | Same as 7.5.1.4 | Same as 7.5.1.4 | Same as 7.5.1.4 |
| 17.6.1.1 NR SA FR2 Event triggered reporting test without gap under non-DRX | Same as 7.6.1.1 | Same as 7.6.1.1 | Same as 7.6.1.1 |

Table F.1.3.2-9: Derivation of test requirements for E-UTRA – NR inter-RAT test cases with E-UTRA serving cell RRM tests for RedCap

| Test | Minimum requirement in TS 38.133 [6] | Test tolerance (TT) | Test requirement in TS 38.533 |
| --- | --- | --- | --- |
| 18.2.1.1 E-UTRA - NR SA FR1 E-UTRAN - NR handover in FR1 | Same as 8.3.1.1 | Same as 8.3.1.1 | Same as 8.3.1.1 |
| 18.2.2.1 E-UTRA - NR SA FR1 Redirection from E-UTRA to NR SA FR1 for redcap UE | For 1Rx RedCap: Same as 6.3.2.3.1 For 2Rx RedCap: Same as 6.3.2.3.1 | For 1Rx RedCap: FFS For 2Rx RedCap: Same as 6.3.2.3.1 | For 1Rx RedCap: FFS For 2Rx RedCap: Same as 6.3.2.3.1 |

<< Some Unchanged Sections>>

Table F.1.3.2-3: Derivation of test requirements for EN-DC FR2 RRM tests

| Test | Minimum requirement in TS 38.133 [6] | Test tolerance (TT) | Test requirement in TS 38.533 |
| --- | --- | --- | --- |
| 5.3.2.2.1 EN-DC FR2 contention based random access | Absolute uplink power: Pint ≥ P ≥ Pmin ±14.0dB PUMAX ≥ P > Pint ±12.0dB Relative uplink power step: Pint ≥ P ≥ Pmin ±6.0dB PUMAX ≥ P > Pint ±4.0dB Uplink timing: 120kHz SCS Te ±3.5*64*Tc | Not applicable due to UL calibration FFS dB FFS dB [48]Tc | Absolute uplink power: ±FFSdB Relative uplink power step: ±(6+FFS)dB, either PRACH being compared ≤ (Pmax – 6dB) ±(4+FFS)dB, both PRACHs being compared > (Pmax – 6dB) Uplink timing: 120kHz SCS Te ±224±[48]*Tc |
| 5.3.2.2.2 EN-DC FR2 non-contention based random access | Same as 5.3.2.2.1 | Same as 5.3.2.2.1 | Same as 5.3.2.2.1 |
| 5.4.1.1 EN-DC FR2 UE transmit timing accuracy | Test 1 (no DRX): Uplink timing: ±3*64*Tc for 240 kHz SSB SCS Max step size Tq: 2.5*64*Tc Min adjust rate Tp: 2.5*64*Tc Max adjust rate: 2.5*64*Tc Noc = -112 dBm/15 kHz Ês2 / Noc: +4.0dB Test 2 (with DRX): Uplink timing: ±3*64*Tc for 240 kHz SSB SCS Noc = -112 dBm/15 kHz Ês2 / Noc: +4.0dB | ±0.75*64*Tc +0.625*64Tc -3.725*64*Tc +1.225*64*Tc 0dB 0dB ±0.75*64*Tc 0dB 0dB | Test 1 (no DRX): Uplink timing: ±3.75*64*Tc Max step size Tq: 3.125*64*Tc Min adjust rate: -1.225*64*Tc Max adjust rate: 3.725*64*Tc Noc = -112 dBm/15 kHz Ês2 / Noc: +4.0dB Test 2 (with DRX): Uplink timing: ±3.75*64*Tc Noc = -112 dBm/15 kHz Ês2 / Noc: +4.0dB |
| 5.4.3.1 EN-DC FR2 UE timing advance adjustment accuracy | Noc = -112 dBm/15 kHz Ês1 / Noc = 4 dB UE Timing Advance Adjustment Accuracy for 120kHz SCS = ±32 Tc | 0dB 0dB +/- 40 Tc | Noc = -112 dBm/15 kHz Ês1 / Noc = 4 dB UE TAAA for 120kHz SCS = ±72 Tc |
| 5.5.1.5 EN-DC FR2 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode | During T1: Noc: -92.1 dBm/15kHz Ês1 / Noc: +2 dB Ês2 / Noc: +2 dB During T2: Noc: -92.1dBm/15kHz Ês1 / Noc: -6 dB Ês2 / Noc: -14 dB During T3: Noc: -92.1dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: -15 dB | During T1: -2.7 dB 2.1 dB 2.1 dB During T2: -2.7 dB 2.1 dB 0 dB During T3: -2.7 dB 0 dB 0 dB | During T1: Noc: -94.8 dBm/15kHz Ês1 / Noc: +4.1 dB Ês2 / Noc: +4.1 dB During T2: Noc: -94.8 dBm/15kHz Ês1 / Noc: -3.9 dB Ês2 / Noc: -14 dB During T3: Noc: -94.8 dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: -15 dB |
| 5.5.1.6 EN-DC FR2 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode | During T1: Noc: -92.1 dBm/15kHz Ês1 / Noc: +2 dB Ês2 / Noc: +2 dB During T2: Noc: -92.1dBm/15kHz Ês1 / Noc: -6 dB Ês2 / Noc: -14 dB During T3: Noc: -92.1dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: -15 dB During T4: Noc: -92.1dBm/15kHz Ês1 / Noc: -4.5 dB Ês2 / Noc: -15 dB During T5: Noc: -92.1dBm/15kHz Ês1 / Noc: +2 dB Ês2 / Noc: -14 dB | During T1: -2.7 dB 2.1 dB 2.1 dB During T2: -2.7 dB 2.1 dB 0 dB During T3: -2.7 dB 0 dB 0 dB During T4: -2.7 dB 0 dB 0 dB During T5: -2.7 dB 2.1 dB 0 dB | During T1: Noc: -94.8 dBm/15kHz Ês1 / Noc: +4.1 dB Ês2 / Noc: +4.1 dB During T2: Noc: -94.8 dBm/15kHz Ês1 / Noc: -3.9 dB Ês2 / Noc: -14 dB During T3: Noc: -94.8 dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: -15 dB During T4: Noc: -94.8 dBm/15kHz Ês1 / Noc: -4.5 dB Ês2 / Noc: -15 dB During T5: Noc: -94.8 dBm/15kHz Ês1 / Noc: 4.1 dB Ês2 / Noc: -14 dB |
| 5.5.5.1 EN-DC FR2 SSB-based beam failure detection and link recovery in non-DRX | During T1: SNR_SSB q0: 5 dB SNR_SSB q1: 0.2 dB During T2: SNR_SSB q0: -3 dB SNR_SSB q1: 0.2 dB During T3-T5: SNR_SSB q0: -12 dB SNR_SSB q1: 20.2 dB In signaling: rsrp-ThresholdSSB: -95 dBm/120kHz for Config 1,2 rsrp-ThresholdSSB: -92 dBm/240kHz for Config 3,4 | +8.70 dB 0 dB +8.70 dB 0 dB 0 dB -0.2 dB -14 dB -14 dB | During T1: SNR_SSB q0: 13.7 dB SNR_SSB q1: 0.2 dB During T2: SNR_SSB q0: 5.7 dB SNR_SSB q1: 0.2 dB During T3-T5: SNR_SSB q0: -12 dB SNR_SSB q1: 20 dB In signaling: rsrp-ThresholdSSB: -109 dBm/120kHz for Config 1,2 srp-ThresholdSSB: -106 dBm/240kHz for Config 3,4 |
| 5.5.5.2 EN-DC FR2 SSB-based beam failure detection and link recovery in DRX | Same as 5.5.5.1 | Same as 5.5.5.1 | Same as 5.5.5.1 |
| 5.5.5.3 EN-DC FR2 CSI-RS-based beam failure detection and link recovery in non-DRX | Noc = -104.7 dBm/120 kHz Ês2 / Noc for CSI-RS set q0 T1: 5 dB T2: -3 dB T3: -12 dB T4: -12 dB T5: -12 dB Ês2 / Noc for CSI-RS set q1 T1: 0.2 dB T2: 0.2 dB T3: 20.2 dB T4: 20.2 dB T5: 20.2 dB rsrp-ThresholdSSB: -95 dBm/SCS kHz | 0 dB 8.7 dB 8.7 dB 0 dB 0 dB 0 dB 0 dB 0 dB -0.2 dB -0.2 dB -0.2 dB -14 dB | Noc = -104.7 dBm/120 kHz Ês2 / Noc for CSI-RS set q0 T1: 13.7 dB T2: 5.7 dB T3: -12 dB T4: -12 dB T5: -12 dB Ês2 / Noc for CSI-RS set q1 T1: 0.2 dB T2: 0.2 dB T3: 20 dB T4: 20 dB T5: 20 dB rsrp-ThresholdSSB: -109.0 dBm/SCS kHz |
| 5.5.5.4 EN-DC FR2 CSI-RS-based beam failure detection and link recovery in DRX | Same as 5.5.5.3 | Same as 5.5.5.3 | Same as 5.5.5.3 |
| 5.5.5.5 EN-DC FR2 scheduling available restriction during SSB-based beam failure detection and link recovery in non-DRX | Same as 5.5.5.1 | Same as 5.5.5.1 | Same as 5.5.5.1 |
| 5.5.5.6 EN-DC FR2 SCell CSI-RS-based beam failure detection and link recovery in non-DRX | Noc = -104.7 dBm/120 kHz Ês3 / Noc for CSI-RS set q0 T1: 5 dB T2: -3 dB T3: -12 dB T4: -12 dB T5: -12 dB Ês3 / Noc for CSI-RS set q1 T1: 0.2 dB T2: 0.2 dB T3: 20.2 dB T4: 20.2 dB T5: 20.2 dB rsrp-ThresholdBFR: -95 dBm/SCS kHz | 0 dB 8.7 dB 8.7 dB 0 dB 0 dB 0 dB 0 dB 0 dB -0.2 dB -0.2 dB -0.2 dB -14 dB | Noc = -104.7 dBm/120 kHz Ês3 / Noc for CSI-RS set q0 T1: 13.7 dB T2: 5.7 dB T3: -12 dB T4: -12 dB T5: -12 dB Ês3 / Noc for CSI-RS set q1 T1: 0.2 dB T2: 0.2 dB T3: 20 dB T4: 20 dB T5: 20 dB rsrp-ThresholdBFR: -109.0 dBm/SCS kHz |
| 5.5.5.7 EN-DC FR2 SCell CSI-RS-based beam failure detection and link recovery in DRX | Same as 5.5.5.6 | Same as 5.5.5.6 | Same as 5.5.5.6 |
| 5.6.1.1 EN-DC FR2 event-triggered reporting without gap in non-DRX | During T1: Ês2: -86dBm/120kHz Ês3: -infinity During T2: Ês2: -86dBm/120kHz Ês3: -86dBm/120kHz | During T1: 0dB 0dB During T2: 0dB 0dB | During T1: Ês2: -86dBm/120kHz Ês3: -infinity During T2: Ês2: -86dBm/120kHz Ês3: -86dBm/120kHz |
| 5.6.1.2 EN-DC FR2 event-triggered reporting without gap in DRX | During T1: Noc: -98dBm/15kHz Ês2 / Noc: +4.00dB Ês3 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês2 / Noc: +4.00dB Ês3 / Noc: +4.00dB Signalled A3-offset: -6.0 | During T1: -3.5dB 0dB 0dB During T2: -3.5dB 0dB 0dB Signalled A3-offset: -1.0 | During T1: Noc: -101.5dBm/15kHz Ês2 / Noc: +4.00dB Ês3 / Noc: -infinity During T2: Noc: -101.5dBm/15kHz Ês2 / Noc: +4.00dB Ês3 / Noc: +4.00dB Signalled A3-offset: -7.0 |
| 5.6.1.3 EN-DC FR2 event-triggered reporting with gap in non-DRX | Same as 5.6.1.1 | Same as 5.6.1.1 | Same as 5.6.1.1 |
| 5.6.1.4 EN-DC FR2 event-triggered reporting with gap in DRX | Same as 5.6.1.2 | Same as 5.6.1.2 | Same as 5.6.1.2 |
| 5.6.2.1 EN-DC FR2-FR2 event-triggered reporting in non-DRX | During T1: Ês1: -87 dBm/120kHz Ês2: - infinity During T2: Ês1: -87 dBm/120kHz Ês2: -87 dBm/120kHz | During T1: 0dB 0dB During T2: 0dB 0dB | During T1: Ês1: -87 dBm/120kHz Ês2: -- infinity During T2: Ês1: -87 dBm/120kHz Ês2: -87 dBm/120kHz |
| 5.6.2.2 EN-DC FR2-FR2 event-triggered reporting in DRX | During T1: Noc: -104.70dBm/15kHz Ês1 / Noc: +6.00dB Ês2 / Noc: -infinity During T2: Noc: -104.70dBm/15kHz Ês1 / Noc: +6.00dB Ês2 / Noc: +9.00dB Signalled A3-offset: -6.0 | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB Signalled A3-offset: -6.0 | During T1: Noc: -104.70dBm/15kHz Ês1 / Noc: +6.00dB Ês2 / Noc: -infinity During T2: Noc: -104.70dBm/15kHz Ês1 / Noc: +6.00dB Ês2 / Noc: +9.00dB Signalled A3-offset: -12.0 |
| 5.6.2.3 EN-DC FR2-FR2 event-triggered reporting in non-DRX with SSB time index detection | Same as 5.6.2.1 | Same as 5.6.2.1 | Same as 5.6.2.1 |
| 5.6.2.4 EN-DC FR2-FR2 event-triggered reporting in DRX with SSB time index detection | Same as 5.6.2.2 | Same as 5.6.2.2 | Same as 5.6.2.2 |
| 5.6.2.5 EN-DC FR1-FR2 event-triggered reporting in non-DRX | During T1: Ês2: - Infinity During T2: Ês2: -87 dBm/120kHz | During T1: 0dB During T2: 0dB | During T1: Ês2: - Infinity During T2: Ês2: -87 dBm/120kHz |
| 5.6.2.6 EN-DC FR1-FR2 event-triggered reporting in DRX | During T1: Noc = -104.7 dBm/15 kHz Ês2 / Noc = - Infinity During T2: Noc = -104.7 dBm/15 kHz Ês2 / Noc = 9 dB | During T1: 0dB 0dB During T2: 0dB 0dB | During T1: Noc = -104.7 dBm/15 kHz Ês2 / Noc = - Infinity During T2: Noc = -104.7 dBm/15 kHz Ês2 / Noc = 9 dB |
| 5.6.2.7 EN-DC FR1-FR2 event-triggered reporting in non-DRX with SSB time index detection | Same as 5.6.2.5 | Same as 5.6.2.5 | Same as 5.6.2.5 |
| 5.6.2.8 EN-DC FR1-FR2 event-triggered reporting in DRX with SSB time index detection | Same as 5.6.2.6 | Same as 5.6.2.6 | Same as 5.6.2.6 |
| 5.6.3.1 EN-DC FR2 SSB-based L1-RSRP measurement in non-DRX | During T1: Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: -infinity During T2: Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +9.00dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB | During T1: Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: -infinity During T2: Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +9.00dB |
| 5.6.3.2 EN-DC FR2 SSB-based L1-RSRP measurement in DRX | Same as 5.6.3.1 | Same as 5.6.3.1 | Same as 5.6.3.1 |
| 5.6.3.3 EN-DC FR2 CSI-RS-based L1-RSRP measurement in non-DRX | Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +9.00dB Reported CSI-RSRP values ± 29dB Reported Differential CSI-RSRP values ± 7dB | 0dB 0dB 0dB Via mapping | Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +9.00dB CSI-RS#1: RSRP_40 to RSRP_99 CSI-RS#0: DIFFRSRP_1 to DIFFRSRP_7 |
| 5.6.3.4 EN-DC FR2 CSI-RS-based L1-RSRP measurement in DRX | Same as 5.6.3.3 | Same as 5.6.3.3 | Same as 5.6.3.3 |
| 5.6.4.1 EN-DC FR2 SRS-RSRP measurement in non-DRX | During T1: Noc: -98 dBm/15kHz Io: -70.01 dBm/ 95.04 MHz During T2: Noc: -98 dBm/15kHz Io: -68.82 dBm/ 95.04 MHz | During T1: 0dB 0dB During T2: 0dB 0dB | During T1: Noc: -98 dBm/15kHz Io: -70.01 dBm/ 95.04 MHz During T2: Noc: -98 dBm/15kHz Io: -68.82 dBm/ 95.04 MHz |
| 5.6.6.1 EN-DC FR2 CSI-RS based CMR and no dedicated IMR L1-SINR measurement in DRX | Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +9.00dB Reported CSI-SINR values ± 5.5dB Reported Differential CSI-SINR values ± 4.5dB | 0dB 0dB 0dB Via mapping | Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +9.00dB CSI-RS#1: SINR_51 to SINR_74 CSI-RS#0: DIFFSINR_4 to DIFFSINR_13 |
| 5.6.6.2 EN-DC FR2 SSB based CMR and dedicated IMR L1-SINR measurement in non-DRX | During T1: Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: -infinity During T2: Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +9.00dB Reported SS-SINR values ± 4.0dB Reported Differential SS-SINR values ± 3.0dB | During T1: 0dB 1.5dB 0dB During T2: 0dB 1.5dB 0dB Via mapping | During T1: Noc: -105dBm/15kHz Ês0 / Noc: +1.50dB Ês1 / Noc: -infinity During T2: Noc: -105dBm/15kHz Ês0 / Noc: +1.50dB Ês1 / Noc: +9.00dB SSB#1: SINR_54 to SINR_71 SSB#0: DIFFSINR_4 to DIFFSINR_10 |
| 5.6.6.3 EN-DC FR2 CSI-RS based CMR and dedicated IMR L1-SINR measurement in non-DRX | Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +9.00dB Reported CSI-SINR values ± 4.5dB Reported Differential CSI-SINR values ± 3.5dB | 0dB 0dB 0dB Via mapping | Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +9.00dB CSI-RS#1: SINR_53 to SINR_72 CSI-RS#0: DIFFSINR_5 to DIFFSINR_12 |
| 5.7.1.1 EN-DC FR2 SS-RSRP measurement accuracy | Test 1: Noc: -91.60dBm/15kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +1.0dB Reported absolute RSRP values: ±8dB Reported relative RSRP values: ±6dB For extreme conditions allow ±3dB + FFS MU additionally Test 2: n257 Ês2: -110.0dBm/120kHz n257 Ês3: -110.0dBm/120kHz n260 Ês2: -107.4dBm/120kHz n260 Ês3: -107.4dBm/120kHz Reported absolute RSRP values: ±8dB Reported relative RSRP values: ±6dB For extreme conditions allow ±3dB+ FFS MU additionally | Test 1: -5.80dB 0dB 0.4dB Via Mapping Test 2: 7.70dB 7.70dB 7.70dB 7.70dB Via Mapping | Test 1: Noc: -97.40dBm/15kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +1.4dB Cell 2: RSRP_50 to RSRP_108 Cell 3: RSRP_46 to RSRP_103 Cell 3: RSRP_x-12 to RSRP_X+2 Test 2: n257 Ês2: -102.3dBm/120kHz n257 Ês3: -102.3dBm/120kHz n260 Ês2: -99.7dBm/120kHz n260 Ês3: -99.7dBm/120kHz Band dependent. See test case tables in clause 5.7.1.1.5 |
| 5.7.1.2 EN-DC FR2-FR2 SS-RSRP measurement accuracy | Test 1 Config 1: Noc1: -90.60dBm/15kHz Noc2: -90.60dBm/15kHz Ês1 / Noc1: +6.0dB Ês2 / Noc2: +6.0dB Reported absolute RSRP values: ±8.0dB For extreme conditions allow ±3dB+ FFS MU additionally Reported relative RSRP values: ±6.0dB For extreme conditions allow ±3dB+ FFS MU additionally Test 2 Config 1: n257 Noc1: -108.13dBm/15kHz n257 Noc2: -113.13dBm/15kHz n260 Noc1: -105.53dBm/15kHz n260 Noc2: -110.53dBm/15kHz Ês1 / Noc1: 17.0dB Ês2 / Noc2: -1.0dB Reported absolute RSRP values: ±8.0dB For extreme conditions allow ±3dB+ FFS MU additionally Reported relative RSRP values: ±6.0dB For extreme conditions allow ±3dB+ FFS MU additionally Test 1 Config 2: Noc1: -93.70dBm/15kHz Noc2: -93.70dBm/15kHz Ês1 / Noc1: +6.0dB Ês2 / Noc2: +6.0dB Reported absolute RSRP values: ±8.0dB For extreme conditions allow ±3dB+ FFS MU additionally Reported relative RSRP values: ±6.0dB For extreme conditions allow ±3dB+ FFS MU additionally Test 2 Config 2: n257 Noc1: -108.13dBm/15kHz n257 Noc2: -113.13dBm/15kHz n260 Noc1: -105.53dBm/15kHz n260 Noc2: -110.53dBm/15kHz Ês1 / Noc1: 17.0dB Ês2 / Noc2: -1.0dB Reported absolute RSRP values: ±8.0dB For extreme conditions allow ±3dB+ FFS MU additionally Reported relative RSRP values: ±6.0dB For extreme conditions allow ±3dB+ FFS MU additionally | Test 1 Config 1: -5.70dB -5.70dB 0dB 0dB Via Mapping Via Mapping Test 2 Config 1: -6.60dB 0dB -6.60dB 0dB 0dB 2dB Via Mapping Via Mapping Test 1 Config 2: -5.60dB -5.60dB 0dB 0dB Via Mapping Via Mapping Test 2 Config 2: -6.60dB 0dB -6.60dB 0dB 0dB 2dB Via Mapping Via Mapping | Test 1 Config 1: Noc1: -96.30dBm/15kHz Noc2: -96.30dBm /15kHz Ês1 / Noc1: +6.0dB Ês2 / Noc2: +6.0dB Cell 2 n257: RSRP_41 to RSRP_109 n260: RSRP_39 to RSRP_109 Cell 3: RSRP_52 to RSRP_109 Cell 3: n257: RSRP_x-15 to RSRP_x+25 n260: RSRP_x-15 to RSRP_x+27 Test 2 Config 1: -114.73dBm/15kHz -113.13dBm/15kHz -112.13dBm/15kHz -110.53dBm/15kHz Ês1 / Noc1: 17.0dB Ês2 / Noc2: 1.0dB Cell 2 n257: RSRP_33 to RSRP_101 n260: RSRP_34 to RSRP_104 Cell 3: n257: RSRP_32 to RSRP_87 n260: RSRP_34 to RSRP_90 Cell 3: n257: RSRP_x-29 to RSRP_x+11 n260: RSRP_x-29 to RSRP_x+13 Test 1 Config 2: Noc1: -99.30dBm/15kHz Noc2: -99.30dBm /15kHz Ês1 / Noc1: +6.0dB Ês2 / Noc2: +6.0dB Cell 12 n257: RSRP_41 to RSRP_109 n260: RSRP_39 to RSRP_109 Cell 3: RSRP_52 to RSRP_109 Cell 3: n257: RSRP_x-15 to RSRP_x+25 n260: RSRP_x-15 to RSRP_x+27 Test 2 Config 2: -114.73dBm/15kHz -113.13dBm/15kHz -112.13dBm/15kHz -110.53dBm/15kHz Ês1 / Noc1: 17.0dB Ês2 / Noc2: 1.0dB Cell 2 n257: RSRP_36 to RSRP_104 n260: RSRP_37 to RSRP_107 Cell 3: n257: RSRP_35 to RSRP_90 n260: RSRP_37 to RSRP_93 Cell 3: n257: RSRP_x-29 to RSRP_x+11 n260: RSRP_x-29 to RSRP_x+13 |
| 5.7.2.1 EN-DC FR2 SS-RSRQ measurement accuracy | Test 1: Noc: -95.0dBm/15kHz Ês2 / Noc: +3.0dB Ês3 / Noc: +3.0dB Reported absolute RSRQ values: ±2.5dB For extreme conditions allow ±1.5dB+ FFS MU additionally Test 2: Noc: -95.0dBm/15kHz Ês2 / Noc: -3.0dB Ês3 / Noc: -3.0dB Reported absolute RSRQ values: ±3.5dB For extreme conditions allow ±0.5dB+ FFS MU additionally | Test 1: -5.70dB 0dB 0dB Via Mapping Test 2: -1.70dB 0dB 0dB Via Mapping | Test 1: Noc: -100.70dBm/15kHz Ês2 / Noc: +3.0dB Ês3 / Noc: +3.0dB Cell 3: All bands: RSRQ_41 to RSRQ_73 Test 2: Noc: -96.70dBm/15kHz Ês2 / Noc: -3.0dB Ês3 / Noc: -3.0dB Cell 3: n257, n258, n261: RSRQ_35 to RSRQ_71 n260: RSRQ_34 to RSRQ_71 |
| 5.7.2.2 EN-DC FR2-FR2 SS-RSRQ measurement accuracy | Test 1: Noc1: -94.03dBm/15kHz Noc2: -94.03dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported absolute RSRQ values: ±2.5dB For extreme conditions allow ±1.5dB+ FFS MU additionally Reported relative RSRQ values: ±3.0dB For extreme conditions allow ±1.0dB+ FFS MU additionally Test 2: Noc1: -94.03dBm/15kHz Noc2: -94.03dBm/15kHz Ês1 / Noc1: -3.0dB Ês2 / Noc2: -3.0dB Reported absolute RSRQ values: ±3.5dB For extreme conditions allow ±0.5dB+ FFS MU additionally Reported relative RSRQ values: ±4.0dB For extreme conditions allow ±0.0dB+ FFS MU additionally | Test 1: -1.90dB -1.90dB 0dB 0dB Via Mapping Via Mapping Test 2: -1.41dB -1.41dB 0dB 0dB Via Mapping Via Mapping | Test 1: Noc1: -95.93dBm/15kHz Noc2: -95.93dBm /15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Cell 2 and Cell 3: RSRQ_41 to RSRQ_73 Cell 3: RSRQ_x-7 to RSRQ_x+7 Test 2: Noc1: -95.44dBm/15kHz Noc2: -95.44dBm /15kHz Ês1 / Noc1: -3.0dB Ês2 / Noc2: -3.0dB Cell 2 and Cell 3: RSRQ_37 to RSRQ_74 Cell 3: RSRQ_x-9 to RSRQ_x+9 |
| 5.7.3.1 EN-DC FR2 SS-SINR measurement accuracy | Test 1: Noc: -105.0dBm/15kHz Ês2 / Noc: +4.54dB Ês3 / Noc: +2.66dB Reported absolute SINR values: ±3.5dB For extreme conditions allow ±0.5dB+ FFS MU additionally Test 2: Noc: -105.0dBm/15kHz Ês2 / Noc: -3.0dB Ês3 / Noc: -3.0dB Reported absolute SINR values: ±3.5dB For extreme conditions allow ±0.5dB+ FFS MU additionally | Test 1: 0dB 0dB 0dB Via Mapping Test 2: 0dB 0.2dB 0.2dB Via Mapping | Test 1: Noc: -105.0dBm/15kHz Ês2 / Noc: +4.54dB Ês3 / Noc: +2.66dB Cell 3: n257, n258, n261: SINR_22 to RSRQ_58 n260: SINR_21 to RSRQ_58 Test 2: Noc: -105.0dBm/15kHz Ês2 / Noc: -2.8dB Ês3 / Noc: -2.8dB Cell 3: n257, n258, n261: SINR_18 to RSRQ_55 n260: SINR_18 to RSRQ_54 |
| 5.7.3.2 EN-DC FR2-FR2 SS-SINR measurement accuracy | Test 1: Noc1: -105.0dBm/15kHz Noc2: -105.0dBm/15kHz Ês1 / Noc1: -0.5dB Ês2 / Noc2: -0.5dB Reported absolute SINR values: ±3.0dB For extreme conditions allow ±1.0dB+ FFS MU additionally Reported absolute SINR values: ±3.5dB For extreme conditions allow ±0.5dB+ FFS MU additionally Test 2: Noc1: -105.0dBm/15kHz Noc2: -105.0dBm/15kHz Ês1 / Noc1: +11.0dB Ês2 / Noc2: +11.0dB Reported absolute SINR values: ±3.0dB For extreme conditions allow ±1.0dB+ FFS MU additionally Reported absolute SINR values: ±3.5dB For extreme conditions allow ±0.5dB+ FFS MU additionally Test 3: Noc1: -105.0dBm/15kHz Noc2: -105.0dBm/15kHz Ês1 / Noc1: -3.0dB Ês2 / Noc2: -3.0dB Reported absolute SINR values: ±3.5dB For extreme conditions allow ±0.5dB+ FFS MU additionally Reported absolute SINR values: ±4.0dB For extreme conditions allow ±0.0dB+ FFS MU additionally | Test 1: 0dB 0dB 0dB 0dB Via Mapping Via Mapping Test 2: -0.1dB -0.1dB 0dB 0dB Via Mapping Via Mapping Test 3: 0dB 0dB 0.9dB 0.9dB Via Mapping Via Mapping | Test 1: Noc1: -105.0dBm/15kHz Noc2: -105.0dBm/15kHz Ês1 / Noc1: -0.5dB Ês2 / Noc2: -0.5dB Cell 2 and Cell 3: n257, n258, n261: SINR_27 to SINR_62 n260: SINR_27 to SINR_61 Cell 3: SINR_x-8 to SINR_x+8 Test 2: Noc1: -105.1dBm/15kHz Noc2: -105.1dBm/15kHz Ês1 / Noc1: +11.0dB Ês2 / Noc2: +11.0dB Cell 2 and Cell 3: n257, n258, n261: SINR_48 to SINR_87 n260: SINR_48 to SINR_86 Cell 3: SINR_x-17 to SINR_x+17 Test 3: Noc1: -105.0dBm/15kHz Noc2: -105.0dBm/15kHz Ês1 / Noc1: -2.1dB Ês2 / Noc2: -2.1dB Cell 2 and Cell 3: n257, n258, n261: SINR_23 to SINR_60 n260: SINR_22 to SINR_59 Cell 3: SINR_x-9 to SINR_x+9 |
| 5.7.4.1 EN-DC FR2 SSB based L1-RSRP measurement accuracy | Test 1: Noc: -100dBm/15kHz SSB#0 Ês / Noc: +10.0dB SSB#1 Ês / Noc: -2.0dB Reported absolute RSRP values: ±8.5dB Reported relative RSRP values: ±6.5dB For extreme conditions allow ±3dB + FFS MU additionally Test 2: n257 SSB#0 Ês: -108.1dBm/120kHz n257 SSB#1 Ês: -105.5dBm/120kHz n260 SSB#0 Ês: -108.1dBm/120kHz n260 SSB#1 Ês: -105.5dBm/120kHz Reported absolute RSRP values: -6.5 +8.5dB Reported relative RSRP values: ±6.5dB For extreme conditions allow ±3dB+ FFS MU additionally | Test 1: -4.1dB 0dB 0.4dB Test 2: 5.7dB | Test 1: Noc: -104.1dBm/15kHz SSB#0 Ês / Noc: +10.0dB SSB#1 Ês / Noc: -1.6dB SSB#0: RSRP_42 to RSRP_101 SSB#1: RSRP_31 to RSRP_89 SSB#1-SSB#0: -9 to -2 Test 2: n257 SSB#0 Ês: -102.4dBm/120kHz n257 SSB#1 Ês: -99.8dBm/120kHz n260 SSB#0 Ês: -102.4dBm/120kHz n260 SSB#1 Ês: -99.8dBm/120kHz SSB#0 and SSB#1 n257: RSRP_27 to RSRP_83 n260: RSRP_30 to RSRP_86 SSB#1-SSB#0: -4 to +4 |
| 5.7.4.2 EN-DC FR2 CSI-RS based L1-RSRP measurement accuracy | Test 1: Noc: -100dBm/15kHz CSI-RS#0 Ês / Noc: +10.0dB CSI-RS#1 Ês / Noc: -2.0dB Reported absolute RSRP values: ±8.5dB Reported relative RSRP values: ±6.5dB For extreme conditions allow ±3dB + FFS MU additionally Test 2: n257 CSI-RS#0 Ês: -108.1dBm/120kHz n257 CSI-RS#1 Ês: -105.5dBm/120kHz n260 CSI-RS#0 Ês: -108.1dBm/120kHz n260 CSI-RS#1 Ês: -105.5dBm/120kHz Reported absolute RSRP values: -6.5 +8.5dB Reported relative RSRP values: ±6.5dB For extreme conditions allow ±3dB+ FFS MU additionally | Test 1: -4.1dB 0dB 0.2dB Test 2: 5.7dB | Test 1: Noc: -104.1dBm/15kHz CSI-RS#0 Ês / Noc: +10.0dB CSI-RS#1 Ês / Noc: -1.8dB CSI-RS#0: RSRP_48 to RSRP_95 CSI-RS#1: RSRP_30 to RSRP_89 CSI-RS#1-CSI-RS#0: -9 to -2 Test 2: n257 CSI-RS#0 Ês: -102.4dBm/120kHz n257 CSI-RS#1 Ês: -99.8dBm/120kHz n260 CSI-RS#0 Ês: -102.4dBm/120kHz n260 CSI-RS#1 Ês: -99.8dBm/120kHz CSI-RS#0 and CSI-RS#1 n257: RSRP_27 to RSRP_83 n260: RSRP_30 to RSRP_86 CSI-RS#1-CSI-RS#0: -4 to +4 |
| 5.7.5.1 EN-DC FR2 SRS-RSRP measurement accuracy | During T1: Noc: -98 dBm/15kHz Io: -70.01 dBm/ 95.04 MHz During T2: Noc: -98 dBm/15kHz Io: -68.82 dBm/ 95.04 MHz SRS_RP n257,n258, n261 -110.1 n259 -106.5 n260 -107.5 n257,n258, n261 -99.2 n259 -93.7 n260 -94.9 | During T1: 0dB During T2: 5dB | During T1: Noc: -98 dBm/15kHz Io: -70.01 dBm/ 95.04 MHz During T2: Noc: -98 dBm/15kHz Io: -68.82 dBm/ 95.04 MHz SRS_RP n257,n258, n261 -105.1 n259 -101.5 n260 -102.5 n257,n258, n261 -94.2 n259 -88.7 n260 -89.9 |

Table F.1.3.2-4: Derivation of test requirements for NR SA FR2 RRM tests

| Test | Minimum requirement in TS 38.133 [6] | Test tolerance (TT) | Test requirement in TS 38.533 |
| --- | --- | --- | --- |
| 7.3.1.4 NR SA FR1-FR2 synchronous DAPS handover | During T1: Noc -104.7dBm/15kHz Es2/Noc -infinity During T2-T5: Noc -104.7 dBm/15kHz Es2/Noc 10 dB | During T1: 0 dB 0 dB During T2-T5: 0 dB 0 dB | During T1: Noc -104.7dBm/15kHz Es2/Noc -infinity During T2-T5: Noc -104.7 dBm/15kHz Es2/Noc 10 dB |
| 7.3.1.5 NR SA FR1-FR2 asynchronous DAPS handover | Same as 7.3.1.4 | Same as 7.3.1.4 | Same as 7.3.1.4 |
| 7.3.2.1.1 NR SA FR2 RRC re-establishment | During T1: Noc = -104.7 dBm/15 kHz Ês1 / Noc = 4 dB Ês2 / Noc = 2 dB During T2: Noc = -104.7 dBm/15 kHz Ês1 / Noc = - Infinity Ês2 / Noc = 2 dB During T3: Noc = -104.7 dBm/15 kHz Ês1 / Noc = - Infinity Ês2 / Noc = 2 dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB During T3: 0dB 0dB 0dB | During T1: Noc = -104.7 dBm/15 kHz Ês1 / Noc = 4 dB Ês2 / Noc = 2 dB During T2: Noc = -104.7 dBm/15 kHz Ês1 / Noc = - Infinity Ês2 / Noc = 2 dB During T3: Noc = -104.7 dBm/15 kHz Ês1 / Noc = - Infinity Ês2 / Noc = 2 dB |
| 7.3.2.1.2 NR SA FR2 - FR2 RRC re-establishment | During T1: Noc1 = -92.1 dBm/15 kHz Noc2 = -92.1 dBm/15 kHz Ês1 / Noc1 = 0 dB Ês2 / Noc 2= - Infinity During T2: Noc1 = -92.1 dBm/15 kHz Noc2 = -92.1 dBm/15 kHz Ês1 / Noc1 = - Infinity Ês2 / Noc2 = - Infinity During T3: Noc1 = -92.1 dBm/15 kHz Noc2 = -92.1 dBm/15 kHz Ês1 / Noc1 = - Infinity Ês2 / Noc2 = 0 dB | During T1: -0.2dB -0.2dB 0dB 0dB During T2: -0.2dB -0.2dB 0dB 0dB During T3: -0.2dB -0.2dB 0dB 0dB | During T1: Noc1 = -92.3 dBm/15 kHz Noc2 = -92.3 dBm/15 kHz Ês1 / Noc1 = 0 dB Ês2 / Noc 2= - Infinity During T2: Noc1 = -92.3 dBm/15 kHz Noc2 = -92.3 dBm/15 kHz Ês1 / Noc1 = - Infinity Ês2 / Noc2 = - Infinity During T3: Noc1 = -92.3 dBm/15 kHz Noc2 = -92.3 dBm/15 kHz Ês1 / Noc1 = - Infinity Ês2 / Noc2 = 0 dB |
| 7.3.2.2.1 NR SA FR2 contention based random access | Absolute uplink power: Pint ≥ P ≥ Pmin ±14.0dB PUMAX ≥ P > Pint ±12.0dB Relative uplink power step: Pint ≥ P ≥ Pmin ±6.0dB PUMAX ≥ P > Pint ±4.0dB Uplink timing: 120kHz SCS Te ±3.5*64*Tc | Not applicable due to UL calibration FFS dB FFS dB [48]Tc | Absolute uplink power: ±FFSdB Relative uplink power step: ±(6+FFS)dB, either PRACH being compared ≤ (Pmax – 6dB) ±(4+FFS)dB, both PRACHs being compared > (Pmax – 6dB) Uplink timing: 120kHz SCS Te ±224±[48]*Tc |
| 7.3.2.2.2 NR SA FR2 non-contention based random access | Same as 7.3.2.2.1 | Same as 7.3.2.2.1 | Same as 7.3.2.2.1 |
| 7.3.3.1 NR SA FR2 conditional handover | During T1: Noc -104.7dBm/15kHz Es1/Noc 6 dB Es2/Noc -infinity During T2: Noc -104.7dBm/15kHz Es1/Noc 6 dB Es2/Noc 11dB In signaling: A3-offset = -1dB | During T1 -1.5dB 0dB 0dB During T2 -1.5dB 0dB 0dB In signaling: -1dB | During T1: Noc -106.2 dBm/15kHz Es1/Noc 6 dB Es2/Noc -infinity During T2: Noc -106.2 dBm/15kHz Es1/Noc 6 dB Es2/Noc 11dB In signaling: A3-offset = -2dB |
| 7.4.1.1 SA FR2 UE transmit timing accuracy | Test 1 (no DRX): Uplink timing: ±3*64*Tc for 240 kHz SSB SCS Max step size Tq: 2.5*64*Tc Min adjust rate Tp: 2.5*64*Tc Max adjust rate: 2.5*64*Tc Noc = -112 dBm/15 kHz Ês1 / Noc: +4.0dB Test 2 (with DRX): Uplink timing: ±3*64*Tc for 240 kHz SSB SCS Noc = -112 dBm/15 kHz Ês1 / Noc: +4.0dB | ±0.75*64*Tc +0.625*64Tc -3.725*64*Tc +1.225*64*Tc 0dB 0dB ±0.75*64*Tc 0dB 0dB | Test 1 (no DRX): Uplink timing: ±3.75*64*Tc Max step size Tq: 3.125*64*Tc Min adjust rate: -1.225*64*Tc Max adjust rate: 3.725*64*Tc Noc = -112 dBm/15 kHz Ês1 / Noc: +4.0dB Test 2 (with DRX): Uplink timing: ±3.75*64*Tc Noc = -112 dBm/15 kHz Ês1 / Noc: +4.0dB |
| 7.4.3.1 NR SA FR2 UE timing advance adjustment accuracy | Noc = -112 dBm/15 kHz Ês1 / Noc = 4 dB UE Timing Advance Adjustment Accuracy for 120kHz SCS = ±32 Tc | 0dB 0dB +/- 40 Tc | Noc = -112 dBm/15 kHz Ês1 / Noc = 4 dB UE TAAA for 120kHz SCS = ±72 Tc |
| 7.5.5.1 NR SA FR2 SSB-based beam failure detection and link recovery in non-DRX | During T1: SNR_SSB q0: 5 dB SNR_SSB q1: 0.2 dB During T2: SNR_SSB q0: -3 dB SNR_SSB q1: 0.2 dB During T3-T5: SNR_SSB q0: -12 dB SNR_SSB q1: 20.2 dB In signaling: rsrp-ThresholdSSB: -95 dBm/120kHz for Config 1 rsrp-ThresholdSSB: -92 dBm/240kHz for Config 2 | +8.70 dB 0 dB +8.70 dB 0 dB 0 dB -0.2 dB -14 dB -14 dB | During T1: SNR_SSB q0: 13.7 dB SNR_SSB q1: 0.2 dB During T2: SNR_SSB q0: 5.7 dB SNR_SSB q1: 0.2 dB During T3-T5: SNR_SSB q0: -12 dB SNR_SSB q1: 20 dB In signaling: rsrp-ThresholdSSB: -109 dBm/120kHz for Config 1 srp-ThresholdSSB: -106 dBm/240kHz for Config 2 |
| 7.5.5.2 NR SA FR2 SSB-based beam failure detection and link recovery in DRX | Same as 7.5.5.1 | Same as 7.5.5.1 | Same as 7.5.5.1 |
| 7.5.5.3 NR SA FR2 CSI-RS-based beam failure detection and link recovery in non-DRX | Noc = -104.7 dBm/120 kHz Ês1 / Noc for CSI-RS set q0 T1: 5 dB T2: -3 dB T3: -12 dB T4: -12 dB T5: -12 dB Ês1 / Noc for CSI-RS set q1 T1: 0.2 dB T2: 0.2 dB T3: 20.2 dB T4: 20.2 dB T5: 20.2 dB rsrp-ThresholdSSB: -95 dBm/SCS kHz | 0 dB 8.7 dB 8.7 dB 0 dB 0 dB 0 dB 0 dB 0 dB -0.2 dB -0.2 dB -0.2 dB -14 dB | Noc = -104.7 dBm/120 kHz Ês1 / Noc for CSI-RS set q0 T1: 13.7 dB T2: 5.7 dB T3: -12 dB T4: -12 dB T5: -12 dB Ês1 / Noc for CSI-RS set q1 T1: 0.2 dB T2: 0.2 dB T3: 20 dB T4: 20 dB T5: 20 dB rsrp-ThresholdSSB: -109.0 dBm/SCS kHz |
| 7.5.5.4 NR SA FR2 CSI-RS-based beam failure detection and link recovery in DRX | Same as 7.5.5.3 | Same as 7.5.5.3 | Same as 7.5.5.3 |
| 7.5.5.5 NR SA FR2 scheduling available restriction during SSB-based beam failure detection and link recovery in non-DRX | Same as 7.5.5.1 | Same as 7.5.5.1 | Same as 7.5.5.1 |
| 7.5.5.6 NR SA FR2 SCell CSI-RS-based beam failure detection and link recovery in non-DRX | Noc = -104.7 dBm/120 kHz Ês2 / Noc for CSI-RS set q0 T1: 5 dB T2: -3 dB T3: -12 dB T4: -12 dB T5: -12 dB Ês2 / Noc for CSI-RS set q1 T1: 0.2 dB T2: 0.2 dB T3: 20.2 dB T4: 20.2 dB T5: 20.2 dB rsrp-ThresholdBFR: -95 dBm/SCS kHz | 0 dB 8.7 dB 8.7 dB 0 dB 0 dB 0 dB 0 dB 0 dB -0.2 dB -0.2 dB -0.2 dB -14 dB | Noc = -104.7 dBm/120 kHz Ês2 / Noc for CSI-RS set q0 T1: 13.7 dB T2: 5.7 dB T3: -12 dB T4: -12 dB T5: -12 dB Ês2 / Noc for CSI-RS set q1 T1: 0.2 dB T2: 0.2 dB T3: 20 dB T4: 20 dB T5: 20 dB rsrp-ThresholdBFR: -109.0 dBm/SCS kHz |
| 7.5.5.7 NR SA FR2 SCell CSI-RS-based beam failure detection and link recovery in DRX | Same as 7.5.5.6 | Same as 7.5.5.6 | Same as 7.5.5.6 |
| 7.6.1.1 NR SA FR2 event-triggered reporting without gap in non-DRX | During T1: Ês1: -86dBm/120kHz Ês2: -infinity During T2: Ês1: -86dBm/120kHz Ês2: -86dBm/120kHz | During T1: 0dB 0dB During T2: 0dB 0dB | During T1: Ês1: -86dBm/120kHz Ês2: -infinity During T2: Ês1: -86dBm/120kHz Ês2: -86dBm/120kHz |
| 7.6.1.2 NR SA FR2 event-triggered reporting without gap in DRX | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +4.00dB Signalled A3-offset: -6.0 | During T1: -3.5dB 0dB 0dB During T2: -3.5dB 0dB 0dB Signalled A3-offset: -1.0 | During T1: Noc: -101.5dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: -infinity During T2: Noc: -101.5dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +4.00dB Signalled A3-offset: -7.0 |
| 7.6.1.3 NR SA FR2 event-triggered reporting with gap in non-DRX | Same as 7.6.1.1 | Same as 7.6.1.1 | Same as 7.6.1.1 |
| 7.6.1.4 NR SA FR2 event-triggered reporting with gap in DRX | Same as 7.6.1.2 | Same as 7.6.1.2 | Same as 7.6.1.2 |
| 7.6.2.1 NR SA FR2-FR2 event-triggered reporting in non-DRX | During T1: Ês1: -87 dBm/120kHz Ês2: - infinity During T2: Ês1: -87 dBm/120kHz Ês2: -87 dBm/120kHz | During T1: 0dB 0dB During T2: 0dB 0dB | During T1: Ês1: -87 dBm/120kHz Ês2: - infinity During T2: Ês1: -87 dBm/120kHz Ês2: -87 dBm/120kHz |
| 7.6.2.3 NR SA FR2-FR2 event-triggered reporting in non-DRX with SSB time index detection | Same as 7.6.2.1 | Same as 7.6.2.1 | Same as 7.6.2.1 |
| 7.6.2.5 SA event triggered reporting tests for FR2 without SSB time index detection when DRX is not used (PCell in FR1) | During T1: Ês2: - Infinity During T2: Ês2: -87 dBm/120kHz | During T1: 0dB During T2: 0dB | During T1: Ês2: - Infinity During T2: Ês2: -87 dBm/120kHz |
| 7.6.2.6 SA event triggered reporting tests for FR2 without SSB time index detection when DRX is used (PCell in FR1) | During T1: Noc = -104.7 dBm/15 kHz Ês2 / Noc = - Infinity During T2: Noc = -104.7 dBm/15 kHz Ês2 / Noc = 9 dB | During T1: 0dB 0dB During T2: 0dB 0dB | During T1: Noc = -104.7 dBm/15 kHz Ês2 / Noc = - Infinity During T2: Noc = -104.7 dBm/15 kHz Ês2 / Noc = 9 dB |
| 7.6.2.7 SA event triggered reporting tests for FR2 with SSB time index detection when DRX is not used (PCell in FR1) | Same as 5.6.2.5 | Same as 5.6.2.5 | Same as 5.6.2.5 |
| 7.6.2.8 SA event triggered reporting tests for FR2 with SSB time index detection when DRX is used (PCell in FR1) | Same as 5.6.2.6 | Same as 5.6.2.6 | Same as 5.6.2.6 |
| 7.6.3.1 NR SA FR2 SSB-based L1-RSRP measurement in non-DRX | Same as 5.6.3.1 | Same as 5.6.3.1 | Same as 5.6.3.1 |
| 7.6.3.2 NR SA FR2 SSB-based L1-RSRP measurement in DRX | Same as 5.6.3.1 | Same as 5.6.3.1 | Same as 5.6.3.1 |
| 7.6.3.3 NR SA FR2 CSI-RS-based L1-RSRP measurement in non-DRX | Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +9.00dB Reported CSI-RSRP values ± 29dB Reported Differential CSI-RSRP values ± 7dB | 0dB 0dB 0dB Via mapping | Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +9.00dB CSI-RS#1: RSRP_40 to RSRP_99 CSI-RS#0: DIFFRSRP_1 to DIFFRSRP_7 |
| 7.6.3.4 NR SA FR2 CSI-RS-based L1-RSRP measurement in DRX | Same as 7.6.3.3 | Same as 7.6.3.3 | Same as 7.6.3.3 |
| 7.6.4.1 NR SA FR2 SRS-RSRP measurement in non-DRX | Same as 5.6.4.1 | Same as 5.6.4.1 | Same as 5.6.4.1 |
| 7.6.4.2 NR SA FR2 CLI-RSSI measurement in non-DRX | Same as 5.6.4.2 | Same as 5.6.4.2 | Same as 5.6.4.2 |
| 7.6.6.1 NR SA FR2 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR measurement accuracy | Same as 5.6.6.1 | Same as 5.6.6.1 | Same as 5.6.6.1 |
| 7.6.6.2 NR SA FR2 SSB based CMR and dedicated IMR L1-SINR measurement accuracy | During T1: Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: -infinity During T2: Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +9.00dB Reported SS-SINR values ± 4.5dB Reported Differential SS-SINR values ± 3.5dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB Via mapping | During T1: Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: -infinity During T2: Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +9.00dB SSB#1: SINR_53 to SINR_72 SSB#0: DIFFSINR_5 to DIFFSINR_12 |
| 7.6.6.3 NR SA FR2 CSI-RS based CMR and dedicated IMR L1-SINR measurement accuracy | Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +9.00dB Reported CSI-SINR values ± 4.0dB Reported Differential CSI-SINR values ± 3.0dB | 0dB 1.5dB 0dB Via mapping | Noc: -105dBm/15kHz Ês0 / Noc: +1.50dB Ês1 / Noc: +9.00dB CSI-RS#1: SINR_54 to SINR_71 CSI-RS#0: DIFFSINR_4 to DIFFSINR_10 |
| 7.7.1.1 NR SA FR2 SS-RSRP measurement accuracy | Test 1: Noc: -91.60dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported absolute RSRP values: ±8dB Reported relative RSRP values: ±6dB For extreme conditions allow ±3dB + FFS MU additionally Test 2: n257 Ês1: -110.0dBm/120kHz n257 Ês2: -110.0dBm/120kHz n260 Ês1: -107.4dBm/120kHz n260 Ês2: -107.4dBm/120kHz Reported absolute RSRP values: ±8dB Reported relative RSRP values: ±6dB For extreme conditions allow ±3dB+ FFS MU additionally | Test 1: -5.80dB 0dB 0.4dB Via Mapping Test 2: 7.70dB 7.70dB 7.70dB 7.70dB Via Mapping | Test 1: Noc: -97.40dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.4dB Cell 1: RSRP_50 to RSRP_108 Cell 2: RSRP_46 to RSRP_103 Cell 2: RSRP_x-12 to RSRP_X+2 Test 2: n257 Ês1: -102.3dBm/120kHz n257 Ês2: -102.3dBm/120kHz n260 Ês1: -99.7dBm/120kHz n260 Ês2: -99.7dBm/120kHz Band dependent. See test case tables in clause 5.7.1.1.5 |
| 7.7.1.2 NR SA FR2-FR2 SS-RSRP measurement accuracy | Test 1 Config 1: Noc1: -90.60dBm/15kHz Noc2: -90.60dBm/15kHz Ês1 / Noc1: +6.0dB Ês2 / Noc2: +6.0dB Reported absolute RSRP values: ±8.0dB For extreme conditions allow ±3dB+ FFS MU additionally Reported relative RSRP values: ±6.0dB For extreme conditions allow ±3dB+ FFS MU additionally Test 2 Config 1: n257 Noc1: -108.13dBm/15kHz n257 Noc2: -113.13dBm/15kHz n260 Noc1: -105.53dBm/15kHz n260 Noc2: -110.53dBm/15kHz Ês1 / Noc1: 17.0dB Ês2 / Noc2: -1.0dB Reported absolute RSRP values: ±8.0dB For extreme conditions allow ±3dB+ FFS MU additionally Reported relative RSRP values: ±6.0dB For extreme conditions allow ±3dB+ FFS MU additionally Test 1 Config 2: Noc1: -93.70dBm/15kHz Noc2: -93.70dBm/15kHz Ês1 / Noc1: +6.0dB Ês2 / Noc2: +6.0dB Reported absolute RSRP values: ±8.0dB For extreme conditions allow ±3dB+ FFS MU additionally Reported relative RSRP values: ±6.0dB For extreme conditions allow ±3dB+ FFS MU additionally Test 2 Config 2: n257 Noc1: -108.13dBm/15kHz n257 Noc2: -113.13dBm/15kHz n260 Noc1: -105.53dBm/15kHz n260 Noc2: -110.53dBm/15kHz Ês1 / Noc1: 17.0dB Ês2 / Noc2: -1.0dB Reported absolute RSRP values: ±8.0dB For extreme conditions allow ±3dB+ FFS MU additionally Reported relative RSRP values: ±6.0dB For extreme conditions allow ±3dB+ FFS MU additionally | Test 1 Config 1: -5.70dB -5.70dB 0dB 0dB Via Mapping Via Mapping Test 2 Config 1: -6.60dB 0dB -6.60dB 0dB 0dB 2dB Via Mapping Via Mapping Test 1 Config 2: -5.60dB -5.60dB 0dB 0dB Via Mapping Via Mapping Test 2 Config 2: -6.60dB 0dB -6.60dB 0dB 0dB 2dB Via Mapping Via Mapping | Test 1 Config 1: Noc1: -96.30dBm/15kHz Noc2: -96.30dBm /15kHz Ês1 / Noc1: +6.0dB Ês2 / Noc2: +6.0dB Cell 1 n257: RSRP_41 to RSRP_109 n260: RSRP_39 to RSRP_109 Cell 2: RSRP_52 to RSRP_109 Cell 2: n257: RSRP_x-15 to RSRP_x+25 n260: RSRP_x-15 to RSRP_x+27 Test 2 Config 1: -114.73dBm/15kHz -113.13dBm/15kHz -112.13dBm/15kHz -110.53dBm/15kHz Ês1 / Noc1: 17.0dB Ês2 / Noc2: 1.0dB Cell 1 n257: RSRP_33 to RSRP_101 n260: RSRP_34 to RSRP_104 Cell 2: n257: RSRP_32 to RSRP_87 n260: RSRP_34 to RSRP_90 Cell 2: n257: RSRP_x-29 to RSRP_x+11 n260: RSRP_x-29 to RSRP_x+13 Test 1 Config 2: Noc1: -99.30dBm/15kHz Noc2: -99.30dBm /15kHz Ês1 / Noc1: +6.0dB Ês2 / Noc2: +6.0dB Cell 1 n257: RSRP_41 to RSRP_109 n260: RSRP_39 to RSRP_109 Cell 2: RSRP_52 to RSRP_109 Cell 2: n257: RSRP_x-15 to RSRP_x+25 n260: RSRP_x-15 to RSRP_x+27 Test 2 Config 2: -114.73dBm/15kHz -113.13dBm/15kHz -112.13dBm/15kHz -110.53dBm/15kHz Ês1 / Noc1: 17.0dB Ês2 / Noc2: 1.0dB Cell 1 n257: RSRP_36 to RSRP_104 n260: RSRP_37 to RSRP_107 Cell 2: n257: RSRP_35 to RSRP_90 n260: RSRP_37 to RSRP_93 Cell 2: n257: RSRP_x-29 to RSRP_x+11 n260: RSRP_x-29 to RSRP_x+13 |
| 7.7.2.1 NR SA FR2 SS-RSRQ measurement accuracy | Test 1: Noc: -95.0dBm/15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: +3.0dB Reported absolute RSRQ values: ±2.5dB For extreme conditions allow ±1.5dB+ FFS MU additionally Test 2: Noc: -95.0dBm/15kHz Ês1 / Noc: -3.0dB Ês2 / Noc: -3.0dB Reported absolute RSRQ values: ±3.5dB For extreme conditions allow ±0.5dB+ FFS MU additionally | Test 1: -5.70dB 0dB 0dB Via Mapping Test 2: -1.70dB 0dB 0dB Via Mapping | Test 1: Noc: -100.70dBm/15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: +3.0dB Cell 2: All bands: RSRQ_41 to RSRQ_73 Test 2: Noc: -96.70dBm/15kHz Ês1 / Noc: -3.0dB Ês2 / Noc: -3.0dB Cell 2: n257, n258, n261: RSRQ_35 to RSRQ_71 n260: RSRQ_34 to RSRQ_71 |
| 7.7.2.2 NR SA FR2-FR2 SS-RSRQ measurement accuracy | Test 1: Noc1: -94.03dBm/15kHz Noc2: -94.03dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported absolute RSRQ values: ±2.5dB For extreme conditions allow ±1.5dB+ FFS MU additionally Reported relative RSRQ values: ±3.0dB For extreme conditions allow ±1.0dB+ FFS MU additionally Test 2: Noc1: -94.03dBm/15kHz Noc2: -94.03dBm/15kHz Ês1 / Noc1: -3.0dB Ês2 / Noc2: -3.0dB Reported absolute RSRQ values: ±3.5dB For extreme conditions allow ±0.5dB+ FFS MU additionally Reported relative RSRQ values: ±4.0dB For extreme conditions allow ±0.0dB+ FFS MU additionally | Test 1: -1.90dB -1.90dB 0dB 0dB Via Mapping Via Mapping Test 2: -1.41dB -1.41dB 0dB 0dB Via Mapping Via Mapping | Test 1: Noc1: -95.93dBm/15kHz Noc2: -95.93dBm /15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Cell 1 and Cell 2: RSRQ_41 to RSRQ_73 Cell 2: RSRQ_x-7 to RSRQ_x+7 Test 2: Noc1: -95.44dBm/15kHz Noc2: -95.44dBm /15kHz Ês1 / Noc1: -3.0dB Ês2 / Noc2: -3.0dB Cell 1 and Cell 2: RSRQ_37 to RSRQ_74 Cell 2: RSRQ_x-9 to RSRQ_x+9 |
| 7.7.3.1 NR SA FR2 SS-SINR measurement accuracy | Test 1: Noc: -105.0dBm/15kHz Ês1 / Noc: +4.54dB Ês2 / Noc: +2.66dB Reported absolute SINR values: ±3.5dB For extreme conditions allow ±0.5dB+ FFS MU additionally Test 2: Noc: -105.0dBm/15kHz Ês1 / Noc: -3.0dB Ês2 / Noc: -3.0dB Reported absolute SINR values: ±3.5dB For extreme conditions allow ±0.5dB+ FFS MU additionally | Test 1: 0dB 0dB 0dB Via Mapping Test 2: 0dB 0.2dB 0.2dB Via Mapping | Test 1: Noc: -105.0dBm/15kHz Ês1 / Noc: +4.54dB Ês2 / Noc: +2.66dB Cell 2: n257, n258, n261: SINR_22 to RSRQ_58 n260: SINR_21 to RSRQ_58 Test 2: Noc: -105.0dBm/15kHz Ês1 / Noc: -2.8dB Ês2 / Noc: -2.8dB Cell 2: n257, n258, n261: SINR_18 to RSRQ_55 n260: SINR_18 to RSRQ_54 |
| 7.7.3.2 NR SA FR2-FR2 SS-SINR measurement accuracy | Test 1: Noc1: -105.0dBm/15kHz Noc2: -105.0dBm/15kHz Ês1 / Noc1: -0.5dB Ês2 / Noc2: -0.5dB Reported absolute SINR values: ±3.0dB For extreme conditions allow ±1.0dB+ FFS MU additionally Reported absolute SINR values: ±3.5dB For extreme conditions allow ±0.5dB+ FFS MU additionally Test 2: Noc1: -105.0dBm/15kHz Noc2: -105.0dBm/15kHz Ês1 / Noc1: +11.0dB Ês2 / Noc2: +11.0dB Reported absolute SINR values: ±3.0dB For extreme conditions allow ±1.0dB+ FFS MU additionally Reported absolute SINR values: ±3.5dB For extreme conditions allow ±0.5dB+ FFS MU additionally Test 3: Noc1: -105.0dBm/15kHz Noc2: -105.0dBm/15kHz Ês1 / Noc1: -3.0dB Ês2 / Noc2: -3.0dB Reported absolute SINR values: ±3.5dB For extreme conditions allow ±0.5dB+ FFS MU additionally Reported absolute SINR values: ±4.0dB For extreme conditions allow ±0.0dB+ FFS MU additionally | Test 1: 0dB 0dB 0dB 0dB Via Mapping Via Mapping Test 2: -0.1dB -0.1dB 0dB 0dB Via Mapping Via Mapping Test 3: 0dB 0dB 0.9dB 0.9dB Via Mapping Via Mapping | Test 1: Noc1: -105.0dBm/15kHz Noc2: -105.0dBm/15kHz Ês1 / Noc1: -0.5dB Ês2 / Noc2: -0.5dB Cell 1 and Cell 2: n257, n258, n261: SINR_27 to SINR_62 n260: SINR_27 to SINR_61 Cell 2: SINR_x-8 to SINR_x+8 Test 2: Noc1: -105.1dBm/15kHz Noc2: -105.1dBm/15kHz Ês1 / Noc1: +11.0dB Ês2 / Noc2: +11.0dB Cell 1 and Cell 2: n257, n258, n261: SINR_48 to SINR_87 n260: SINR_48 to SINR_86 Cell 2: SINR_x-17 to SINR_x+17 Test 3: Noc1: -105.0dBm/15kHz Noc2: -105.0dBm/15kHz Ês1 / Noc1: -2.1dB Ês2 / Noc2: -2.1dB Cell 1 and Cell 2: n257, n258, n261: SINR_23 to SINR_60 n260: SINR_22 to SINR_59 Cell 2: SINR_x-9 to SINR_x+9 |
| 7.7.4.1 NR SA FR2 SSB based L1-RSRP measurement accuracy | Test 1: Noc: -100dBm/15kHz SSB#0 Ês / Noc: +10.0dB SSB#1 Ês / Noc: -2.0dB Reported absolute RSRP values: ±8.5dB Reported relative RSRP values: ±6.5dB For extreme conditions allow ±3dB + FFS MU additionally Test 2: n257 SSB#0 Ês: -108.1dBm/120kHz n257 SSB#1 Ês: -105.5dBm/120kHz n260 SSB#0 Ês: -108.1dBm/120kHz n260 SSB#1 Ês: -105.5dBm/120kHz Reported absolute RSRP values: -6.5 +8.5dB Reported relative RSRP values: ±6.5dB For extreme conditions allow ±3dB+ FFS MU additionally | Test 1: -4.1dB 0dB 0.4dB Test 2: 5.7dB | Test 1: Noc: -104.1dBm/15kHz SSB#0 Ês / Noc: +10.0dB SSB#1 Ês / Noc: -1.6dB SSB#0: RSRP_42 to RSRP_101 SSB#1: RSRP_31 to RSRP_89 SSB#1-SSB#0: -9 to -2 Test 2: n257 SSB#0 Ês: -102.4dBm/120kHz n257 SSB#1 Ês: -99.8dBm/120kHz n260 SSB#0 Ês: -102.4dBm/120kHz n260 SSB#1 Ês: -99.8dBm/120kHz SSB#0 and SSB#1 n257: RSRP_27 to RSRP_83 n260: RSRP_30 to RSRP_86 SSB#1-SSB#0: -4 to +4 |
| 7.7.4.2 NR SA FR2 CSI-RS based L1-RSRP measurement accuracy | Test 1: Noc: -100dBm/15kHz CSI-RS#0 Ês / Noc: +10.0dB CSI-RS#1 Ês / Noc: -2.0dB Reported absolute RSRP values: ±8.5dB Reported relative RSRP values: ±6.5dB For extreme conditions allow ±3dB + FFS MU additionally Test 2: n257 CSI-RS#0 Ês: -108.1dBm/120kHz n257 CSI-RS#1 Ês: -105.5dBm/120kHz n260 CSI-RS#0 Ês: -108.1dBm/120kHz n260 CSI-RS#1 Ês: -105.5dBm/120kHz Reported absolute RSRP values: -6.5 +8.5dB Reported relative RSRP values: ±6.5dB For extreme conditions allow ±3dB+ FFS MU additionally | Test 1: -4.1dB 0dB 0.2dB Test 2: 5.7dB | Test 1: Noc: -104.1dBm/15kHz CSI-RS#0 Ês / Noc: +10.0dB CSI-RS#1 Ês / Noc: -1.8dB CSI-RS#0: RSRP_45 to RSRP_104 CSI-RS#1: RSRP_30 to RSRP_89 CSI-RS#1-CSI-RS#0: -9 to -2 Test 2: n257 CSI-RS#0 Ês: -102.4dBm/120kHz n257 CSI-RS#1 Ês: -99.8dBm/120kHz n260 CSI-RS#0 Ês: -102.4dBm/120kHz n260 CSI-RS#1 Ês: -99.8dBm/120kHz CSI-RS#0 and CSI-RS#1 n257: RSRP_27 to RSRP_83 n260: RSRP_30 to RSRP_86 CSI-RS#1-CSI-RS#0: -4 to +4 |
| 7.7.5.1 NR SA FR2 SRS-RSRP measurement accuracy | Same as 5.7.5.1 | Same as 5.7.5.1 | Same as 5.7.5.1 |
| 7.7.5..2 NR SA FR2 CLI-RSSI measurement accuracy | Same as 5.7.5.2 | Same as 5.7.5.2 | Same as 5.7.5.2 |

<<End of changes>>