# Title:	Test Tolerances for FR1 SRS -RSRP measurement accuracy tests

# Source:	Qualcomm

# 1. Introduction

The purpose of this document is to present the methodology and principles used for the development of Test Tolerances for FR1 SRS-RSRP measurement accuracy.

The test cases in question, from TS 38.533, are:

- 4.7.6.1, EN-DC FR1 SRS-RSRP measurement in non-DRX

- 6.7.8.1, NR SA FR1 SRS-RSRP measurement accuracy

This document describes the process to derive the Test Tolerances. The calculations are provided in the accompanying spreadsheet.

# 2. Prototype test case in Annex A of TS 38.133

The Test Requirements are defined in the following extracts from 3GPP TS 38.133 V16.14.0 (2022-12) Annex A.

A standalone test case 6.7.8.1 is chosen, to show only the NR-related parameters.

<< Extract from TS 38.133 >>

#### A.6.7.8.1	SA SRS-RSRP measurement accuracy with FR1 serving cell

##### A.6.7.8.1.1	Test Purpose and Environment

The purpose of this test is to verify that the SRS-RSRP measurement accuracy is within the specified limits. This test will verify the requirements in Clauses 10.1.22.1.1 with the testing configurations for NR cells in Table A.6.7.8.1.1-1.

Table A.6.7.8.1.1-1: Applicable NR configurations for FR1 SRS-RSRP accuracy test

| Config | Description |
| --- | --- |
| 1 | 15kHz SRS SCS, 10 MHz bandwidth, TDD duplex mode |
| 2 | 30kHz SRS SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

##### A.6.7.8.1.2	Test parameters

In this set of test cases there is one cell in the test, FR1 PCell (Cell 1). The test parameters for the Cell 1 are given in Table A.6.7.8.1.2-1 below. The test parameter for the (virtual) neighbor cell UE transmitting SRS are given in Table A.6.7.8.1.2-2.

Before the test UE is configured to perform SRS-RSRP measurement. During the test, the test system transmits SRS resources for measurement in the DL slots according to the SRS configuration in Table A.6.7.8.1.2-3. There is no measurement gap configured in the test. During the test, the test system does not transmit PDCCH/PDSCH/OCNG on SRS symbol to be transmitted and on 1 data symbol before SRS to be transmitted.

Table A.6.7.8.1.2-1: FR1 test parameters for SRS-RSRP accuracy for PCell

| Parameter |  | Config | Unit | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- | --- | --- |
| SSB GSCN |  | 1~2 |  | freq1 | freq1 | freq1 |
| Duplex mode |  | 1~2 |  | TDD | TDD | TDD |
| TDD configuration |  | 1 |  | TDDConf.1.1 | TDDConf.1.1 | TDDConf.1.1 |
|  |  | 2 |  | TDDConf.2.1 | TDDConf.2.1 | TDDConf.2.1 |
| BWchannel |  | 1 | MHz | 10: NRB,c = 52 | 10: NRB,c = 52 | 10: NRB,c = 52 |
|  |  | 2 |  | 40: NRB,c = 106 | 40: NRB,c = 106 | 40: NRB,c = 106 |
| PDSCH Reference measurement channel |  | 1 |  | SR.1.1 TDD | SR.1.1 TDD | SR.1.1 TDD |
|  |  | 2 |  | SR.2.1 TDD | SR.2.1 TDD | SR.2.1 TDD |
| RMSI CORESET Reference Channel |  | 1 |  | CR.1.1 TDD | CR.1.1 TDD | CR.1.1 TDD |
|  |  | 2 |  | CR.2.1 TDD | CR.2.1 TDD | CR.2.1 TDD |
| Dedicated CORESET Reference Channel |  | 1 |  | CCR.1.1 TDD | CCR.1.1 TDD | CCR.1.1 TDD |
|  |  | 2 |  | CCR.2.1 TDD | CCR.2.1 TDD | CCR.2.1 TDD |
| SSB configuration |  | 1 |  | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 |
|  |  | 2 |  | SSB.2 FR1 | SSB.2 FR1 | SSB.2 FR1 |
| OCNG Patterns |  | 1~2 |  | OP.1 | OP.1 | OP.1 |
| TRS configuration |  | 1 |  | TRS.1.1 TDD | TRS.1.1 TDD | TRS.1.1 TDD |
|  |  | 2 |  | TRS.1.2 TDD | TRS.1.2 TDD | TRS.1.2 TDD |
| Initial BWP Configuration |  | 1~2 |  | DLBWP.0.1 ULBWP.0.1 | DLBWP.0.1 ULBWP.0.1 | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration |  | 1~2 |  | DLBWP.1.1 ULBWP.1.1 | DLBWP.1.1 ULBWP.1.1 | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration |  | 1~2 |  | SMTC.1 | SMTC.1 | SMTC.1 |
| Time offset between DL from serving cell and SRS from test system |  | 1~2 | s | 17.67 | 17.67 | 17.67 |
| EPRE ratio of PSS to SSS |  | 1~2 | dB | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |
| Note2 | NR_TDD_FR1_A NOTE 3 | 1 | dBm/15kHz | -106 | -88 | -114 |
|  | NR_TDD_FR1_C |  |  |  |  | -113 |
|  | NR_TDD_FR1_D |  |  |  |  | -112.5 |
|  | NR_TDD_FR1_E |  |  |  |  | -112 |
|  | NR_TDD_FR1_A NOTE 5 | 2 |  | Not applicableNote 4 | -91 | -114 |
|  | NR_TDD_FR1_C |  |  |  |  | -113 |
|  | NR_TDD_FR1_D |  |  |  |  | -112.5 |
|  | NR_TDD_FR1_E |  |  |  |  | -112 |
| Note2 | NR_TDD_FR1_A NOTE 3 | 1 | dBm/SRS SCS | -106 | -88 | -114 |
|  | NR_TDD_FR1_C |  |  |  |  | -113 |
|  | NR_TDD_FR1_D |  |  |  |  | -112.5 |
|  | NR_TDD_FR1_E |  |  |  |  | -112 |
|  | NR_TDD_FR1_A NOTE 3 | 2 |  | Not applicableNote 4 | -88 | -111 |
|  | NR_TDD_FR1_C |  |  |  |  | -110 |
|  | NR_TDD_FR1_D |  |  |  |  | -109.5 |
|  | NR_TDD_FR1_E |  |  |  |  | -109 |
| Note 1: OCNG shall be used such that a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification Note 4: Test 1 is not used when testing with 30kHz SSB SCS |  |  |  |  |  |  |

Table A.6.7.8.1.2-2: FR1 test parameters for SRS-RSRP accuracy for neighbour cell UE

| Parameter |  | Config | Unit | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- | --- | --- |
| Note2 | NR_TDD_FR1_A NOTE 5 | 1 | dBm/15kHz | -106 | -88 | -114 |
|  | NR_TDD_FR1_C |  |  |  |  | -113 |
|  | NR_TDD_FR1_D |  |  |  |  | -112.5 |
|  | NR_TDD_FR1_E |  |  |  |  | -112 |
|  | NR_TDD_FR1_A NOTE 5 | 2 |  | Not applicableNote 6 | -91 | -114 |
|  | NR_TDD_FR1_C |  |  |  |  | -113 |
|  | NR_TDD_FR1_D |  |  |  |  | -112.5 |
|  | NR_TDD_FR1_E |  |  |  |  | -112 |
| Note2 | NR_TDD_FR1_A NOTE 5 | 1 | dBm/SRS SCS | -106 | -88 | -114 |
|  | NR_TDD_FR1_C |  |  |  |  | -113 |
|  | NR_TDD_FR1_D |  |  |  |  | -112.5 |
|  | NR_TDD_FR1_E |  |  |  |  | -112 |
|  | NR_TDD_FR1_A NOTE 5 | 2 |  | Not applicableNote 6 | -88 | -111 |
|  | NR_TDD_FR1_C |  |  |  |  | -110 |
|  | NR_TDD_FR1_D |  |  |  |  | -109.5 |
|  | NR_TDD_FR1_E |  |  |  |  | -109 |
| on SRS |  | 1~2 | dB | 1 | 1 | 1 |
| SRS RSRP Note3 | NR_TDD_FR1_A NOTE 5 | 1 | dBm/SRS SCS | -105 | -87 | -113 |
|  | NR_TDD_FR1_C |  |  |  |  | -112 |
|  | NR_TDD_FR1_D |  |  |  |  | -111.5 |
|  | NR_TDD_FR1_E |  |  |  |  | -111 |
|  | NR_TDD_FR1_A NOTE 5 | 2 |  | Not applicableNote 6 | -87 | -110 |
|  | NR_TDD_FR1_C |  |  |  |  | -109 |
|  | NR_TDD_FR1_D |  |  |  |  | -108.5 |
|  | NR_TDD_FR1_E |  |  |  |  | -108 |
| Io Note3 | NR_TDD_FR1_A NOTE 5 | 1 | dBm/9.36 MHz | -74.51 | -56.51 | -82.51 |
|  | NR_TDD_FR1_C |  |  |  |  | -81.51 |
|  | NR_TDD_FR1_D |  |  |  |  | -81.01 |
|  | NR_TDD_FR1_E |  |  |  |  | -79.51 |
|  | NR_TDD_FR1_A NOTE 5 | 2 | dBm/38.16 MHz | Not applicableNote 6 | -53.42 | -76.42 |
|  | NR_TDD_FR1_C |  |  |  |  | -75.42 |
|  | NR_TDD_FR1_D |  |  |  |  | -74.92 |
|  | NR_TDD_FR1_E |  |  |  |  | -74.42 |
| on SRS |  | 1~2 | dB | 1 | 1 | 1 |
| Propagation condition |  | 1~2 |  | AWGN | AWGN | AWGN |
| Antenna configuration |  | 1~2 |  | 1x2 | 1x2 | 1x2 |
| SRS configuration |  | 1 |  | SRSConf.1 | SRSConf.1 | SRSConf.1 |
|  |  | 2 |  | SRSConf.2 | SRSConf.2 | SRSConf.2 |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of the test. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification Note 6: Test 1 is not used when testing with 30kHz SSB SCS |  |  |  |  |  |  |

Table A.6.7.8.1.2-3: SRS configuration parameters for FR1 SRS-RSRP accuracy

|  | Field | SRSConf.1 | SRSConf.2 |
| --- | --- | --- | --- |
| SRS-ResourceSet | srs-ResourceSetId | 0 | 0 |
|  | srs-ResourceIdList | 0 | 0 |
|  | resourceType | Periodic | Periodic |
|  | Usage | Codebook | Codebook |
| SRS-Resource | SRS-ResourceId | 0 | 0 |
|  | nrofSRS-Ports | Port1 | Port1 |
|  | transmissionComb | n2 | n2 |
|  | combOffset-n2 | 0 | 0 |
|  | cyclicShift-n2 | 0 | 0 |
|  | resourceMapping startPosition | 0 | 0 |
|  | resourceMapping nrofSymbols | n1 | n1 |
|  | resourceMapping repetitionFactor | n1 | n1 |
|  | freqDomainPosition | 0 | 0 |
|  | freqDomainShift | 0 | 0 |
|  | freqHopping c-SRS | 12 | 12 |
|  | freqHopping b-SRS | 0 | 0 |
|  | freqHopping b-hop | 0 | 0 |
|  | groupOrSequenceHopping | Neither | Neither |
|  | resourceType | Periodic | Periodic |
|  | periodicityAndOffset-p | sl20, 9 | sl40, 19 |
|  | sequenceId | 0 | 0 |

#### A.6.7.8.1.3	Test Requirements

The SRS-RSRP measurement accuracy shall fulfil the requirements in clauses 10.1.22.1.1.

# 3. Discussion

The tests consist of one time period with one NR cell. The parameters are chosen to allow absolute accuracy measurement on the same cell.

# 4. Calculation of Test Tolerances

General approach

The general approach is given in the steps below:

- Copy the originally specified key

- parameters from the core requirements

- Where relevant, calculate derived parameters from the core requirements

- Define uncertainties for a minimum set of parameters

- Define controlled parameters (critical to the test verdict), calculate sensitivity factors and uncertainty

- Determine which parameters to offset (apply Test Tolerances to) and by how much

- Recalculate original or derived parameters including Test Tolerances

- Check that the controlled parameters meet requirements to get the correct test verdict

Each step is explained below, and the calculations are given in the accompanying spreadsheet.

a) Original specified key parameters

The key parameters are copied from Tables A.6.7.8.1.2-1, A.6.7.8.1.2-2 and A.6.7.8.1.2-3 in TS 38.133. The key parameters are selected as the minimum set to define the cell power levels, and which are subject to a test system uncertainty which may affect the verdict of the test. The angle of arrival of the radiated signals is also recorded.

A number of UE core requirement values and their associated parameters are also entered in this section, taking into account the angle of arrival of the radiated signals. The requirements are entered because they affect the radiated signal received by the UE, or affect how the UE sees the received signals at baseband.

The key parameters appear in section a) of the accompanying spreadsheet.

b) Derived parameters

A number of derived parameters are calculated, using the base information in a). The reason for deriving each additional parameter is given in the “Comment” column of section b) in the accompanying spreadsheet.

c) Uncertainties

The Test system provides 1 cell on 1 frequency, with AWGN. We propose to control the following parameters:

- AWGN Absolute power, Noc averaged over BWConfig uncertainty, ±1.5

- Ratio of cell 1 signal / AWGN, Ês1 / Noc averaged over BWConfig uncertainty, ±0.3

This value is the same as for signalling test cases in TS 38.508-1 Table 6.2.2.2-4.

UE-related uncertainties are considered, together with selected UE core requirements that have an allowed range, and are therefore handled as a UE uncertainty:

- UE SRS-RSRP absolute accuracy: Value taken from TS 38.133  10.1.22.1.1

<< Extracts from TS 38.133 v16.7.0 >>

#### 10.1.22.1	SRS-RSRP

##### 10.1.22.1.1	SRS-RSRP Accuracy

The SRS-RSRP measurement reported by the UE shall fulfil the accuracy requirements defined in Table 10.1.22.1.1-1 for FR1 and Table 10.1.22.1.1-2 for FR2, provided that the following conditions are met. The accuracy requirements in this clause are derived based on AWGN radio propagation conditions.

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for SRS-RSRP measurements are fulfilled according to Annex B.2.z for a corresponding Band for each relevant SRS resource configured for measurement.

-	The time difference between UE’s DL reference timing in the serving cell and SRS arrival time is no larger than Terror_SRS_RSRP, where

-	Terror_SRS_RSRP = TC × NTA_offset + 4.67us for FR1

-	Terror_SRS_RSRP = TC × NTA_offset + 3.67us for FR2

-	NTA_offset is defined in Table 7.1.2-2

-	TC is 0.509ns

-	The number of SRS ports in the SRS resource configured for measurement is 1,

-	The number of symbols in the SRS resource configured for measurement is 1,

-	The number of repetitions in the SRS resource configured for measurement is 1,

-	Frequency hopping, sequence group hopping or sequence hopping is disabled in the SRS resource configured for measurement,

-	The bandwidth of the SRS resource is 48 PRBs.

-	One of the following conditions is met

-	There is no other SRS resource with the same root sequence and on the same symbol and with same comb as the relevant SRS resource.

-	If multiple SRS resources are on the same symbol and with same comb, the distance between cyclic shifts of any two resources is no less than 6 if transmissionComb = n4, and no less than 4 if transmissionComb = n2.

Table 10.1.22.1.1-1:  SRS-RSRP absolute accuracy in FR1

| Accuracy |  |  |  |  |  | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition |  |  | Extreme condition |  |  | SRS Ês/Iot | Io Note 1 range |  |  |  |  |  |
|  |  |  |  |  |  |  | NR operating band groups Note 2 | Minimum Io |  |  |  | Maximum Io |
| dB |  |  |  |  |  | dB |  | dBm / SCSSRS |  |  | dBm/BW Channel | dBm/BW Channel |
| SCSSRS (kHz) |  |  | SCSSRS (kHz) |  |  |  |  |  |  |  |  |  |
| 15 | 30 | 60 | 15 | 30 | 60 |  |  | SCSSRS = 15 kHz | SCSSRS = 30 kHz | SCSSRS = 60 kHz |  |  |
| 3 | 3.5 | 5 | 7.5 | 8 | 9.5 | 1 | NR_TDD_FR1_A, | -120 | -117 | -114 | N/A | -70 |
|  |  |  |  |  |  |  | NR_TDD_FR1_C | -119 | -116 | -113 | N/A | -70 |
|  |  |  |  |  |  |  | NR_TDD_FR1_D | -118.5 | -115.5 | -112.5 | N/A | -70 |
|  |  |  |  |  |  |  | NR_TDD_FR1_E | -118 | -115 | -112 | N/A | -70 |
| 6.5 | 7 | 8.5 | 9.5 | 10 | 11.5 | 1 | NR_TDD_FR1_A, NR_TDD_FR1_C, NR_TDD_FR1_D, NR_TDD_FR1_E | N/A | N/A | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 10.1.22.1.1-2: SRS-RSRP absolute accuracy in FR2

| Accuracy |  |  |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition |  | Extreme condition |  | SRS Ês/Iot | Io Note 1 range |  |  |  |
|  |  |  |  |  | Minimum Io |  |  | Maximum Io |
| dB |  |  |  | dB | dBm / SCSSRS Note 2 |  | dBm/BWChannel | dBm/BWChannel |
| SCSSRS (kHz) |  | SCSSRS (kHz) |  |  | SCSSRS = | SCSSRS = |  |  |
| 60 | 120 | 60 | 120 |  | 60kHz | 120kHz |  |  |
| 6 | 8.5 | 9 | 11.5 | ≥1 | Same value as SRS_RP in Table B.2.7-2, according to UE Power class, operating band and angle of arrival |  | N/A | -70 |
| 9 | 11.5 | 11 | 13.5 | ≥1 | N/A |  | -70 | -50 |
| NOTE 1: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. NOTE 2: Values based on Refsens and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. NOTE 3: In the test cases, the SSB Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. |  |  |  |  |  |  |  |  |

d) Controlled parameters critical to verdict

In many RRM test cases there is not a simple one-to-one relationship between the parameters that can be set by the test equipment, and their effect on parameters determining the test verdict.

It is therefore essential to identify those parameters determining the test verdict, and the side conditions that should be met for the UE to be tested correctly. The controlled parameters listed in the accompanying spreadsheet, have been derived by study of the test case and by careful reading of the relevant clauses in TS 38.133. The reason for each parameter being critical to the test verdict is given briefly in the “Comment” column of section d) in the accompanying spreadsheet. Information about the value to be achieved is given in the “Requirement” column of section g) in the spreadsheet.

Having identified the parameters critical to the test verdict which need to be controlled, we now need to consider how they are affected by the parameters which can be set by the test equipment. This is done by working out “sensitivity factors”. A sensitivity factor is just the ratio (effect on a critical parameter y / a test equipment uncertainty x) and is usually in dB/dB.

Where there is a simple one-to-one relationship between the parameters that can be set by the test equipment, and their effect on parameters determining the test verdict, the sensitivity factors are derived by inspection as one, or if no effect, as zero.

Where there is not a simple one-to-one relationship between the parameters that can be set by the test equipment, and their effect on parameters determining the test verdict, the calculation of sensitivity factors is done using a method selected from TR 38.903. Cell 1 and Cell 2 Es/Iot at UE baseband is calculated according to TR 38.903 A.2.4, and applied Io calculated according to TR 38.903 A.2.5.

Sensitivity factors for UE-reported SRS-RSRP also follow methods defined in TR 38.903. The UE-related factors are from clause A.2.6, depending on the type of measurement and the the test setup. UE-reported SRS-RSRP is affected by a combination of Test system uncertainties and UE-related uncertainties.

The test system uncertainties are combined root-sum-square where uncorrelated, and arithmetically where correlated. More detailed explanation is given in the accompanying spreadsheet, and in TR 38.903 A.2.7.

Having filled in the matrix of sensitivity factors, the accompanying spreadsheet calculates the overall uncertainty for each controlled parameter, taking into account the uncertainties and sensitivity factors for each parameter that can be set by the test equipment, and the UE-related uncertainties where relevant. This process follows the superposition principle. More details and explanation can be found in section 4 of TR 38.903.

e) Determine parameters to offset

During the test, the variation due to uncertainties does not make the Io greater than the -50dBm side condition, hence AWGN doesn’t need to be adjusted.

f) Parameters modified by Test Tolerances

Based on the decision in e), the set of parameters in a) and b) is reproduced in section f) of the accompanying spreadsheet, modified by the Test Tolerances (applied offsets).

Re-derived parameters are calculated using the same methods as were used in step b).

g) Check controlled parameters Min/Max

Using a format similar to that in step d), the nominal value of each controlled parameter is recalculated, as some will have changed from the original due to the application of the Test Tolerances in step f).

The minimum and maximum values, due to variability from uncertainties, of controlled parameters is then calculated and compared against the requirements (Es/IotBB, Applied SRS_RP and Applied Io). The critical requirement for each parameter is given in the “Requirement” column of section g) in the accompanying spreadsheet. The cases closest to limit are identified by turquoise cells in the spreadsheet. If all the requirements are met, then the chosen offsets are acceptable. It can be seen that with the uncertainty values and Test Tolerances proposed, the requirements are met.

For this test, the verdict is based on the SRS-RSRP values reported by the UE. As stated earlier, the reported values are affected by:

- The uncertainties in the stimulus set by the Test system (Downlink signal)

- The uncertainties in the UE response (SRS-RSRP reports)

Having ensured that the stimulus set by the Test system remains within the side conditions, and knowing its uncertainty, we now need to calculate the range of SRS-RSRP values that a conformant UE could report. The mapping is defined in TS 38.133:

##### 10.1.22.1.2	SRS-RSRP report mapping

The reporting range of SRS-RSRP is defined from -140 dBm to -44 dBm with 1 dB resolution. The mapping of measured quantity is defined in Table 10.1.22.1.2-1. The range in the signalling may be larger than the guaranteed accuracy range.

Table 10.1.22.1.2-1: SRS-RSRP measurement report mapping

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| SRS-RSRP_0 | SRS-RSRP<-140 | dBm |
| SRS-RSRP_1 | -140≤ SRS-RSRP<-139 | dBm |
| SRS-RSRP_2 | -139≤ SRS-RSRP<-138 | dBm |
| SRS-RSRP_3 | -138≤ SRS-RSRP<-137 | dBm |
| SRS-RSRP_4 | -137≤ SRS-RSRP<-136 | dBm |
| .. | .. | … |
| SRS-RSRP_95 | -46≤ SRS-RSRP<-45 | dBm |
| SRS-RSRP_96 | -45≤ SRS-RSRP<-44 | dBm |
| SRS-RSRP_97 | -44≤ SRS-RSRP | dBm |
| SRS-RSRP_98 | Infinity |  |
| Note: ‘Infinity’ means that UE cannot detect SRS due to too strong signal to measure. |  |  |

The acceptable range of reported SRS-RSRP values is calculated taking into account the uncertainties in the stimulus set by the Test system, the variations and uncertainties in the UE response, and the UE measurement report mapping function. The calculation is done in section g) of the accompanying spreadsheet.Absolute SRS-RSRP reported values

The rationale and method is given in TR 38.903 clause A.2.6.2.

# 5. Treatment of other configurations within the same test case

Not applicable, as all the configurations defined have the same NR cell parameters.

# 6. Treatment of related test cases

TS 38.533 also contains a similar EN-DC test case in clause 4.7.6.1, which has the same NR Cell signal levels, so we propose to apply the same level uncertainties and Test Tolerances. There is no need to provide a tab on the spreadsheet as all the numeric values are identical.

# 7. Treatment of extreme conditions

RSRP values calculated are for normal conditions.