3GPP TSG RAN WG5 Meeting #98	R5-230745r1

# Athens, Greece, 27 Feb - 03 Mar, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2715 | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to SIB1 and SIB18 for Rel-17 eNPN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RAN_PRN_enh_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2023-02-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To update the default contents of SIB1 and SIB18 for Rel-17 eNPN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updating the default contents of SIB1 and SIB18 for Rel-17 eNPN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Work Plan will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.2, 4.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN Impact. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: move the text in condition colum for SIB18I and modify the condition 'SNPN' as per MCC suggestion. |  |  |  |  |  |  |  |  |

4.6.2	System information blocks

<Start of modified section>

–	SIB18

Table 4.6.2-18B: SIB18

| Derivation Path: TS 38.331 [6], clause 6.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB18-r17 ::= SEQUENCE { |  |  |  |
| gin-ElementList-r17 SEQUENCE (SIZE (1..maxGIN-r17)) OF GIN-Element-r17 { | 1 entry |  |  |
| GIN-Element-r17[1] SEQUENCE { |  | entry 1 |  |
| plmn-Identity-r17 | PLMN-Identity |  |  |
| nid-List-r17 SEQUENCE (SIZE (1..maxGIN-r17)) OF NID-r16 { | 1 entry |  |  |
| NID-r16[1] | See table 4.4.2-4 | entry 1; NID is coded as a BITSTRING based on TS 38.508-1 Table 4.4.2-4 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| gins-PerSNPN-List-r17 | Not present | Not present if there is only a single SNPN in snpn-AccessInfoList in SIB1 |  |
| lateNonCriticalExtension | Not present |  |  |
| } |  |  |  |

<End of modified section>

4.6.3	Radio resource control information elements

<Start of modified section>

–	CellAccessRelatedInfo

Table 4.6.3-16: CellAccessRelatedInfo

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellAccessRelatedInfo ::= SEQUENCE { |  |  |  |
| plmn-IdentityList | PLMN-IdentityInfoList |  |  |
| cellReservedForOtherUse | Not present |  |  |
|  | true |  | SNPN, CAG |
| cellReservedForFutureUse-r16 | Not present |  |  |
| npn-IdentityInfoList-r16 | Not present |  |  |
|  | NPN-IdentityInfoList-r16 |  | SNPN, CAG |
| snpn-AccessInfoList-r17 SEQUENCE (SIZE (1..maxNPN-r16)) OF SNPN-AccessInfo-r17 { | 1 entry |  | eNPN |
| SNPN-AccessInfo-r17[1] SEQUENCE { |  | entry 1 |  |
| extCH-Supported-r17 | true |  |  |
| extCH-WithoutConfigAllowed-r17 | true |  |  |
| onboardingEnabled-r17 | true |  |  |
| imsEmergencySupportForSNPN-r17 | true |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| eNPN | Rel-17 enhanced NPN |

<End of modified section>