3GPP TSG-RAN5 Meeting #98	R5-230751r1

# Athens, Greece, 27th Feb - 3rd Mar 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2717 | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of test frequencies for n46 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Keysight |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_unlic-UEConTest |  |  |  |  |  | Date: |  |  | 2023-02-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Test frequencies for NR band n46 in clause 4.3.1.1.1.46 needs to be corrected for some channel bandwidths to get the SSB fully inside the carrier. - Test frequencies for CBW=80 MHz in Table 4.3.1.1.1.46-2 and Table 4.3.1.1.1.46-3 are specified using a non-valid NR-ARFCN value (766668) - Editorial correction needed to Table 4.3.1.1.1.46-2 needed to the Range column. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Corrected selection of absoluteFrequencySSB values in Table 4.3.1.1.1.46-Table 4.3.1.1.1.46-2 and Table 4.3.1.1.1.46-3 to fit SSB inside the carrier by chosing the SSref value of the SSB to the value correspondent to the closest higher GSCN value on the synchronisation raster. - Updated test frequencies parameters for CBW=80 MHz in Table 4.3.1.1.1.46-2 anb Table 4.3.1.1.1.46-3 for NR-ARFCN value of 768668. - Corrected editorial cut and paste errors in column Range in Table 4.3.1.1.1.46-2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Testing of UE supporting NR band n46 will fail conformant UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1.1.1.46 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | R5-230751r1: Added change 2 above. Reverted the change of absoluteFrequencySSB values to the current values for the CBWs and frequncy ranges that not caused SSB outside the carrier. Corrected typo under reason for change from n41 to n46. Added Keysight as co-source (mering with R5-230302). |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.1.46	Reference test frequencies for NR operating band n46

Table 4.3.1.1.1.46-1: Test frequencies for NR operating band n46 and SCS 15 kHz (Note 3)

| CBW [MHz] | carrierBandwidth [PR Bs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] Note 4 | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 52 | Downlink | Low | 5730 | 782000 | 5725.32 | 781688 | 0 | 30 | - | 781928 | 31 | - | - | - |
|  |  | & | Mid |  |  |  |  |  |  | - |  |  |  |  |  |
|  |  | Uplink | High | 5830.02 | 788668 | 5734.62 | 782308 | 504 |  | - | 788596 | 31 | - | - | - |
| 20 | 106 | Downlink | Low | 5160 | 744000 | 5150.46 | 743364 | 0 | 30 | - | 743616 | 31 | - | - | - |
|  |  | & | Mid | 5479.98 | 765332 | 5452.08 | 763472 | 102 |  | - | 764936 | 31 | - | - | - |
|  |  | Uplink | High | 5905.02 | 793668 | 5804.76 | 786984 | 504 |  | - | 793272 | 31 | - | - | - |
| 40 | 216 | Downlink | Low | 5170.02 | 744668 | 5150.58 | 743372 | 0 | 30 | - | 743616 | 31 | - | - | - |
|  |  | & | Mid | 5509.98 | 767332 | 5472.18 | 764812 | 102 |  | - | 766276 | 31 | - | - | - |
|  |  | Uplink | High | 5895 | 793000 | 5784.84 | 785656 | 504 |  | - | 791944 | 31 | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-3 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 3: NR band n46 with SCS =15 kHz is restricted for operation when carrier is configured as an SCell part of a DC or CA configuration. As the bandwidth is limited to SCell then absence of CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). Note 4: The selection of Low, Mid and High test frequencies are restricted to the limited allowed ARFCN values as specified in TS 38.101 [7], Table 5.4.2.3-2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.46-2: Test frequencies for NR operating band n46 and SCS 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] Note 4 | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low | 5730 | 782000 | 5725.68 | 781712 | 0 | 30 | - | 782016 | 31 | - | - | - |
| Note 3 |  | & | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Uplink | High | 5830.02 | 788668 | 5644.26 | 776284 | 504 |  | - | 788620 | 31 | - | - | - |
| 20 | 51 | Downlink | Low | 5160 | 744000 | 5150.82 | 743388 | 0 | 30 | 8996 | 743712 | 12 | 0 | 3 (3) | 6 |
|  |  | & | Mid | 5479.98 | 765332 | 5434.08 | 762272 | 102 |  | 9218 | 765024 | 16 | 0 | 2 (2) | 208 |
|  |  | Uplink | High | 5905.02 | 793668 | 5714.4 | 780960 | 504 |  | 9513 | 793344 | 0 | 0 | 2 (2) | 1012 |
| 40 | 106 | Downlink | Low | 5170.02 | 744668 | 5150.94 | 743396 | 0 | 30 | 8996 | 743712 | 4 | 0 | 3 (3) | 6 |
|  |  | & | Mid | 5509.98 | 767332 | 5454.18 | 763612 | 102 |  | 9232 | 766368 | 20 | 0 | 2 (2) | 208 |
|  |  | Uplink | High | 5895 | 793000 | 5694.48 | 779632 | 504 |  | 9499 | 792000 | 8 | 0 | 1 (1) | 1010 |
| 60 | 162 | Downlink | Low | 5200.02 | 746668 Note 5 | 5170.86 | 744724 | 0 | 30 | 9010 | 745056 | 20 | 0 | 3 (3) | 6 |
|  |  | & | Mid | 5500.02 | 766668 | 5434.14 | 762276 | 102 |  | 9218 | 765024 | 12 | 0 | 2 (2) | 208 |
|  |  | Uplink | High | 5884.98 | 792332 | 5674.38 | 778292 | 504 |  | 9485 | 790656 | 4 | 0 | 1 (1) | 1010 |
| 80 | 217 | Downlink | Low | 5190 | 746000 | 5150.94 | 743396 | 0 | 30 | 8996 | 743712 | 4 | 0 | 3 (3) | 6 |
|  |  | & | Mid | 5530,02 | 768668 | 5454,24 | 763616 | 102 |  | 9232 | 766368 | 16 | 0 | 2 (2) | 208 |
|  |  | Uplink | High | 5875.02 | 791668 | 5654.52 | 776968 | 504 |  | 9472 | 789408 | 8 | 1 | 3 (3) | 1016 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-4 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 3: For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as an SCell part of a DC or CA configuration. As the bandwidth is limited to SCell then absence of CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). Note 4: The selection of Low, Mid and High test frequencies are restricted to the limited allowed ARFCN values as specified in TS 38.101 [7], Table 5.4.2.3-2. Note 5: The next lowest allowed ARFCN values has been selected as the lowest ARFCN value in TS 38.101 [7], Table 5.4.2.3-2 for CBW=60 MHz will cause the required guardband to exceed the bands lower edge. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.46-3: Test frequencies for NR operating band n46 and SCS 60 kHz without CORESET#0 (Note 1)

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] Note 2 |  | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Downlink | Low | 5730 | 782000 |  | 5726.04 | 781736 | 0 | 30 | - | 782016 |
|  |  | & | Mid |  |  |  |  |  |  |  |  |  |
|  |  | Uplink | High | 5830.02 | 788668 | 5463.18 |  | 764212 | 504 |  | - | 788644 |
| 20 | 24 | Downlink | Low | 5160 | 744000 | 5151.36 |  | 743424 | 0 | 30 | - | 743712 |
|  |  | & | Mid | 5479.98 | 765332 | 5397.9 |  | 759860 | 102 |  | - | 764996 |
|  |  | Uplink | High | 5905.02 | 793668 | 5533.5 |  | 768900 | 504 |  | - | 793332 |
| 40 | 51 | Downlink | Low | 5170.02 | 744668 | 5151.66 |  | 743444 | 0 | 30 | - | 743684 |
|  |  | & | Mid | 5509.98 | 767332 | 5418.18 |  | 761212 | 102 |  | - | 766348 |
|  |  | Uplink | High | 5895 | 793000 | 5513.76 |  | 767584 | 504 |  | - | 792016 |
| 60 | 79 | Downlink | Low | 5179.98 | 745332 | 5151.54 |  | 743436 | 0 | 30 | - | 743712 |
|  |  | & | Mid | 5500.02 | 766668 | 5398.14 |  | 759876 | 102 |  | - | 765012 |
|  |  | Uplink | High | 5884.98 | 792332 | 5493.66 |  | 766244 | 504 |  | - | 790676 |
| 80 | 107 | Downlink | Low | 5190 | 746000 | 5151.48 |  | 743432 | 0 | 30 | - | 743712 |
|  |  | & | Mid | 5530,02 | 768668 | 5418,06 |  | 761204 | 102 |  | - | 766340 |
|  |  | Uplink | High | 5875.02 | 791668 | 5473.62 |  | 764908 | 504 |  | - | 789340 |
| Note 1: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). Note 2: The selection of Low, Mid and High test frequencies are restricted to the limited allowed ARFCN values as specified in TS 38.101 [7], Table 5.4.2.3-2. |  |  |  |  |  |  |  |  |  |  |  |  |