3GPP TSG-RAN5 Meeting #98                                                  	                  R5-230786r1

# Athens, Greece, 27th February 2023 – 3rd March 2023

Agenda item:	5.4.17

Source: 	Qualcomm Incorporated

Title: 	Discussion on RRM applicability rules and test optimization

Document for:	Discussion and Endorsement

## 1.	Introduction

TS 38.133 [1] section A.3.13 has introduced a principle of testing in which any set of RRM requirements being verified by multiple test cases (SA, EN-DC) can be tested only once, according to certain rules. We think this principle will help optimize the test case execution and reduce testing time.

This paper presents some our views on this principle and how we believe RAN5 should adopt it into the specifications.

## 2.	Discussion

This principle of testing, added in 38.133 [1] Annex A.3.13.2, establishes the following guidelines:

### A.3.13.2 Principle of Testing If test cases are defined in both SA and EN-DC operations to verify the same RRM requirement then the UE capable of both SA and EN-DC operations needs to verify that RRM requirement by performing test case(s) in either SA operation or in EN-DC operation. If test cases are defined in both SA and EN-DC operations to verify at least one common RRM requirement then the UE capable of both SA and EN-DC operations needs to verify RRM requirements by performing test case(s) in either SA operation or in EN-DC operation provided that the performed test case(s): - verifies the largest number of RRM requirements and - verifies at least all RRM requirements covered in the test case(s), which is not performed. Ref. 38.133 [1]

Two observations can be derived from this snippet:

First, when both SA, and its equivalent EN-DC test, verify the same set of requirements, either test variant shall be considered equally good enough to fulfill the requirement. Therefore, if one of these tests is executed, its counterpart can be skipped.

Observation1: When both SA, and its equivalent EN-DC test, verify the same set of requirements, if one of them is executed, its counterpart can be skipped.

Second, when a test (SA or EN-DC) verifies a subset of requirements compared to its (EN-DC or SA) counterpart, if the test variant with largest number of requirements (superset) is tested, then its counterpart can be skipped. However, that is not the case when the subset counterpart is tested, as there would still be requirements pending to be tested. Therefore, the superset would still be required to be tested.

Observation2: When a test verifies a subset of requirements compared to its counterpart, if the test variant with largest number of requirements (superset) is tested, then its counterpart can be skipped.

Based on these two observations, it seems like test cases could be categorized in three separate groups:

- Group1: Tests where both SA and EN-DC counterparts verify the same set of requirements.

- Group2: Tests in which either the SA or the EN-DC counterpart verifies a superset of requirements.

- Group3: Tests that either don’t have a SA/EN-DC counterpart, or in which the counterpart satisfies a different requirement.

According to these groups the following actions shall be performed:

- Group1: If either counterpart is tested, the other can be skipped

- Group2: Tests in this group will require further analysis.

- Group3: No counterpart (if available) can be skipped.

It is therefore our view that a test-by-test case review needs to be performed to understand in which of these groups each RRM test case can be categorized.

Observation3: RRM test cases can be categorized according to the Group1, Group2, Group3 definitions presented in this paper.

With respect to how to implement these changes in the specifications, we think a similar approach to that used in performance test cases:

Applicability specification 38.522 [4] Table 4.1.4-1 lists all performance test cases, along with their corresponding applicability. In the column indicated as ‘Additional Information’ we notice that it is stated when a test can be skipped, and for which reason.

As an example, in the below snippet from 38.522 [4] we can see that test 6.2A.3.1.1 can be skipped if 6.2A.3.1.2 has been tested, and test 6.2A.3.1.2 can be skipped if 6.2A.3.1.3 has been tested.

A similar approach could be performed for RRM test Group1 test cases in 38.522 [4] section 4.2:

- Both Group1 counterparts (SA, EN-DC) can have an entry in the ‘Additional Information’ column indicating that the test can be skipped if their counterpart has been tested.

- Group2 tests will require further analysis.

- No entry shall be added to Group3 test cases.

We have filed R5-231321 [6] to show an implementation example for a Group1 test requirement.

Proposal1: Update the ‘Additional Information’ column of 38.522 [4] section 4.2, according to the approach presented in this paper.

Table 4.1.4-1: Applicability of performance test cases, ref. TS 38.521-4 [4] Clause; TC Title; Release; Applicability; Tested Bands Selection; Additional Information. Condition; Comment. 6.2.3.2.2.2; 4Rx TDD FR1 aperiodic subband CQI reporting under fading conditions for both SA and NSA; Rel-15; C019; UEs supporting 5GS TDD FR1 and 4Rx antenna ports; D010. 6.2A.3.1.1; 2Rx CQI reporting accuracy under AWGN conditions for CA (2DL CA); Rel-15; C031; UEs supporting 5GS FR1 and CA (2DL CA); E016; Test execution not necessary if 6.2A.3.1.2 is executed.. 6.2A.3.1.2; 2Rx CQI reporting accuracy under AWGN conditions for CA (3DL CA); Rel-15; C033; UEs supporting 5GS FR1 and CA (3DL CA); E017; Test execution not necessary if 6.2A.3.1.3 is executed.. 6.2A.3.1.3; 2Rx CQI reporting accuracy under AWGN conditions for CA (4DL CA); Rel-15; C036; UEs supporting 5GS FR1 and CA (4DL CA); E018. Ref. 38.522 [4]

Finally, we also think it would be beneficial to include in 38.533 specification, Annex D, the principle of testing specified in 38.133 [1] Annex A.3.13 along with the proposals presented in this paper. We have filed R5-231320 [5] in this regard.

Proposal2: Update Annex D of 38.533 specification according to R5-231320.

## 3.	Conclusion

In this paper we have presented our views with respect to the principle of testing introduced in 38.133 [1] Annex A.3.13, which can be summarized in the following observations:

Observation1: When both SA, and its equivalent EN-DC test, verify the same set of requirements, if one of them is executed, its counterpart can be skipped.

Observation2: When a test verifies a subset of requirements compared to its counterpart, if the test variant with largest number of requirements (superset) is tested, then its counterpart can be skipped.

Observation3: RRM test cases can be categorized according to the Group1, Group2, Group3 definitions presented in this paper.

In addition, we present our way forward on how this principle should be considered:

Proposal1: Update the ‘Additional Information’ column of 38.522 [4] section 4.2, according to the approach presented in this paper.

Proposal2: Update Annex D of 38.533 specification according to R5-231320.

## 4.		References

[1] 3GPP TS 38.133: “NR; Requirements for support of radio resource management”.

[2] 3GPP TS 38.533: “NR; User Equipment (UE) conformance specification; Radio Resource Management (RRM)”.

[3] 3GPP TS 38.521-4: “NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 4: Performance”.

[4] 3GPP TS 38.522: “NR; User Equipment (UE) conformance specification; Applicability of radio transmission, radio reception and radio resource management test cases”.

[5] R5-231320: “Update to RRM applicability rules and test optimization - 38.533”

[6] R5-231321: “Update to RRM applicability rules and test optimization - 38.522”