3GPP TSG-RAN WG5 Meeting #98                                      R5-230810r1

Athens, Greece, Feburary 27th – March 3rd, 2023

Source: 	Huawei, HiSilicon, Apple Inc.

Title: 	Discussion on testability for beam correspondence in initial access

Agenda Item:	5.2

Document for:	Approval

Introduction

RAN5#98 received the LS R5-230023 from RAN4 on the testability for beam correspondence in initial access. This documents provides some initial thoughts on this topic.

Overall description

Currently RAN4 is discussing beam correspondence requirement and the implications on testability aspects in initial access. It is found that UE beam lock function (UBF) is not available in initial access. Without UBF, it’s unclear how to measure the PRACH transmit power from both polarizations.

RAN4 would like to invite RAN5 to check the testability aspects for beam correspondence in initial access. e.g.  whether it is necessary to introduce the UBF function in initial access for simplification of the test (note that the beam used in initial access may be different from the beam in connected mode). If necessary but not feasible, RAN4 would also like to invite RAN5 to check if there is any other solution on testing the EIRP in initial access. e.g., for testing EIRP of Msg1.

RAN4 would also like to know how to ensure P-MPR is set to 0 during the test.

Actions

To RAN2:

ACTION: 	RAN4 respectfully requests RAN5 feedback on the above question raised by RAN4.

# Overall description

Currently RAN4 is discussing beam correspondence requirement and the implications on testability aspects in initial access. It is found that UE beam lock function (UBF) is not available in initial access. Without UBF, it’s unclear how to measure the PRACH transmit power from both polarizations.

RAN4 would like to invite RAN5 to check the testability aspects for beam correspondence in initial access. e.g.  whether it is necessary to introduce the UBF function in initial access for simplification of the test (note that the beam used in initial access may be different from the beam in connected mode). If necessary but not feasible, RAN4 would also like to invite RAN5 to check if there is any other solution on testing the EIRP in initial access. e.g., for testing EIRP of Msg1.

RAN4 would also like to know how to ensure P-MPR is set to 0 during the test.

# Actions

To RAN2:

ACTION: 	RAN4 respectfully requests RAN5 feedback on the above question raised by RAN4.

Discussion

Background of RAN4 discussion

One of the objectives of the RAN4 leading WI NR_FR2_Fenh is to specify the beam correspondence requirements during RRC_IDLE mode and RRC_INACTIVE mode with SDT configured.

In RRC_IDLE mode, UE detects the SSB sent by network and decides the best Tx beam autonomously to transmit PRACH preamble. In RRC_INACTIVE mode, UE detects the SSB sent by network and decides the best Tx beam autonomously to transmit PRACH preamble for RA-SDT or transmit PUSCH for CG-SDT. At the moment the BC requirement in RRC_IDLE mode is prioritized.

In RRC_IDLE mode, the BC requirement could only be specified based on PRACH transmission. The BC requirement in Rel-15 and Rel-16 is the combination of beam peak EIRP, EIRP spherical coverage and BC tolerance. For IDLE mode, it’s already agreed that spherical coverage of PRACH will be specified. It’s still under discussion whether beam peak EIRP and BC tolerance for PRACH need to be specified.

During RAN4 discussion, some testability issues are identified, and the RAN5 test mode and beam lock function is considered as one potential solution.

Polarization issue

The polarization issue exists when the test equipment could only transmit/receive with single polarization at a time. When the TE switches the antenna polarization, the UE’s corresponding receive/transmit beam might alter as well, which will lead to unstable power measurement. In RRC_CONNECTED mode, this was solved by introducing beam lock function for UE’s test mode in RAN5. However, for idle mode the beam lock function is never specified.

Observation 1: Beam lock function is needed to solve the polarization issue.

Unstable Beam

According to the PRACH procedure specified in RAN1 and RAN2, the UE will measure the SS-RSRP of SSBs, and select an SSB with SS-RSRP above rsrp-ThresholdSSB. The PRACH preamble associated with selected SSB will be transmitted. The Tx beam used by UE is not restricted by the specification.

If the random access procedure is not completed, e.g. RAR is not received, UE could a) retransmit preamble associated with the same SSB through the same Tx beam, with transmit power ramping; b) retransmit preamble associated with the same SSB through a different Tx beam, with transmit power unchanged; c) retransmit preamble associated with a different SSB, with transmit power unchanged.

From the PRACH procedure we could tell:

- It’s not ensured that UE transmits PRACH with the best Tx beam.

- It’s not ensured that UE always transmits PRACH with the same Tx beam

The unpredictable UE behaviour will cause unstable measurement of the PRACH transmit power. To solve this issue, some modification to the beam lock function specified by RAN5 could be an option.

Current beam lock function in RRC_CONNECTED mode is activated assuming the UE has finished the beam management procedure. Once beam lock activation command is sent to the UE, UE shall lock the Tx beam immediately. If similar approach applies to PRACH BC, it’s difficult to decide when to send the beam lock activation command to UE due to the reasons explained above.

The beam lock function could be further modified in following way. If beam lock activation command is sent to UE, UE is not expected to lock its Rx/Tx beam immediately. UE could take its time to do the beam management until the best Rx/Tx beam is identified, then starts to transmit the PRACH preamble. Once the PRACH transmission starts, the beam lock function takes effect, which means UE needs to lock its Tx beam for all the subsequent PRACH transmissions. With this modification, the measurement of PRACH transmit power could stably reflect UE’s best Tx performance based on its beam correspondence performance.

Observation 2: Rel-15 Beam lock function could be modified to enable beam lock during RRC_IDLE mode. In addition the procedure could be designed carefully to allow UE complete the beam management procedure before transmitting PRACH.

How to achieve maximum output power

In Rel-15 and Rel-16, the BC requirements are tested at Pcmax by controlling UE’s transmit power using the power control command. For PRACH transmission, the power control command is not available. The potential methods of enforcing UE to achieve its maximum output power were 1) open loop power control; 2) power ramping. With open loop power control, the absolute power tolerance needs to be take into consideration in the requirement definition which leads to a significant requirement relaxation. With power ramping, it’s difficult to tell when the UE has ramped to its maximum output power.

The beam lock function could be modified to also take care of the power aspect. If the beam lock function is activated, UE shall always transmit PRACH with PCMAX. The P-MPR is also set to 0dB.

Observation 3: Beam lock function could be used to require UE to transmit PRACH with PCMAX.

Another approach to be investigated is in the initial access, the UE measures the RSRP and estimates the PL based on the broadcasted DL RS power at first. Then the required power for PRACH transmission will be calculated based on the estimated PL and the target Rx power at gNB and compares with the Pcmax. Hence the PRACH power is determined by the following formula in 38.213. With this, we can set the UE to transmit at Pcmax for PRACH by well design the signalling parameters so that the min {} is always dominated by Pcmax,f,c.

[dBm],

Observation 4: The maximum output power in initial access is possibly achievable for the first preamble by sufficiently setting/signaling the parameters in the PRACH power formula

How to maintain maximum output power

In the following measurement process, the UE maximum transmission power should be maintained until the test is done. As mentioned in [4], it can be maintained by holding RAR(msg2) and the target received power, preamble power step and number of preamble retransmissions.

Observation 5: the maximum output power can be maintained during the test by holing RAR through parameter setting on preamble power step and number of retransmissions.

How to fix the UE beam direction during the test

Another challenge is the beam change and stability issue. We think the performance verification for beam correspondence in initial access should be based on similar consideration on beam change as for RRC_CONNECTED. Currently, RAN5 only enabled beam lock function for RRC_CONNECTED state in 38.509.

Observation 6: Current UBF is specific to RRC_Connected Mode so a modification to UBF will be needed in the future to handle initial access state.

One aspect that needs to be factored is the difference in approach needed for SA versus NSA when it comes to defining such a beam lock test function in initial access state.

Observation 7: The beam lock for intial access will need to be handled separately for SA and NSA.

- For SA, the updated UBF function will operate during RRC_IDLE mode and must specifically help with beam lock once SSBs have been measured and best beam is identified)

- For NSA, the UBF test function can be send through E-UTRA bearer

Example of modified beam lock function

Based on above observations, the beam lock function could be modified in following way as an example.

1> UE is in RRC_CONNECTED mode, and test mode is activated.

1> If UE receives the Activate_Beamlock_IDLE command, UE should:

2> When UE enters RRC_IDLE state, UE needs to activate Beamlock state

2> UE decides the best Tx beam which generates the highest EIRP toward the direction of the incoming SS/PBCH block signal

2> UE doesn’t start transmitting PRACH preamble before UE decides the best Tx beam

2> When UE starts transmitting PRACH, the Tx beam should not be changed until the Beamlock state is deactivated

2> When UE starts transmitting, the transmit power should be set to Pcmax without considering the PRACHTargetPowerSetting

1> The Beamlock state is deactivated if,

2> UE enters RRC_CONNECTED mode, and Dectivate_Beamlock_IDLE command is received

Conclusion

In this contribution we discussed some possible extension of beam lock function to solve the identified test issues. Following proposals are for RAN4 consideration.

Observation 1: Beam lock function is needed to solve the polarization issue.

Observation 2: Rel-15 Beam lock function could be modified to enable beam lock during RRC_IDLE mode. In addition the procedure could be designed carefully to allow UE complete the beam management procedure before transmitting PRACH.

Observation 3: Beam lock function could be used to require UE to transmit PRACH with PCMAX.

Proposal 1: It’s proposed RAN5 approves the draft LS in Annex of this document.

- References

- RP-220967, “New WID: NR RF requirements enhancement for frequency range 2 (FR2), Phase 3”, Nokia, RAN #95-e, March 2022.

- R4-2214454, “WF on BC in RRC_INACTIVE and initial access”, Nokia, RAN4 #104 Meeting, August 2022.

- R4-2217727, “WF on BC in RRC_INACTIVE and initial access”, Nokia, RAN4 #104bis Meeting, October 2022.

- R4-2220517, “WF on beam correspondence requirements for initial access and RRC_inactive mode”, Nokia, RAN4 #105 Meeting, November 2022.

- Annex

3GPP TSG-RAN WG5 Meeting #98                                      R5-23xxxx

Athens, Greece, Feburary 27th – March 3rd, 2023

Title:	Reply LS on testability for beam correspondence in initial access

Response to:

Release:	Rel-18

Work Item:	NR_RF_FR2_req_Ph3-Core

Source:	RAN5

To:	RAN4

Cc:

#### Contact person:	Name:	Ashwin Mohan

#### E-mail Address:	ashwin_mohan@apple.com

#### Contact person:	Name:	Chunying Gu

#### E-mail Address:	guchunying@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

- 1	Overall description

RAN5 would like to thank RAN4 for providing the background of beam correspondence discussion in initial access scenario and associated open test issues. RAN5 has not begun any work on Release 18 FR2 RF Further Enhancements, therefore the response to the questions from RAN5 is based on an initial assessment based on known behavior of existing UE beamlock function in RRC_Connected mode. RAN5 will work on actual test procedures and definitions as required when the core requirements are defined and RAN5 Release 18 work commences in the future.

With the above background, RAN5 can provide the following responses on the feasibility of introducing UBF function in initial access.

Question 1:  whether it is necessary to introduce the UBF function in initial access for simplification of the test (note that the beam used in initial access may be different from the beam in connected mode)?

Response: Yes, it would be necessary to evaluate and introduce a new UBF-like test function in initial access if the objective of the test is to lock the beam during initial access. The current UBF applies only in RRC Connected Mode. If the core requirements are specified for both Standalone (SA) and Non-Standalone (NSA), the implementation of test function has to be discussed separately.

The UBF function could be designed carefully to ensure UE complete the beam management procedure before transmitting PRACH. The UE is expected to transmit PRACH only with the best Tx beam which generates the highest EIRP toward the direction of the incoming SS/PBCH block signal.

Question 2: RAN4 would also like to invite RAN5 to check if there is any other solution on testing the EIRP in initial access. e.g., for testing EIRP of Msg1.

Response: There are currently no RF requirements in RAN5 that require testing of EIRP in initial access therefore this scope has not been discussed for conformance testing. New test procedure would have to be introduced to establish and maintain maximum power/peak EIRP during initial access.

Question 3: RAN4 would also like to know how to ensure P-MPR is set to 0 during the test.

Response: It’s specified that P-MPR is 0dB in conformance testing in 6.2.4 of TS 38.101-2. There is no other specific action in RAN5 to ensure P-MPR is set to 0.

- 2	Actions

To RAN4:

ACTION: 	RAN5 respectfully requests RAN4 take above information into consideration.

3	Dates of next TSG-RAN WG5 meetings

TSG-RAN WG5 Meeting #99         22nd – 26th May 2023             Incheon, Korea

TSG-RAN WG5 Meeting #100        21st – 25th Aug 2023	            	      	EU