3GPP TSG-RAN5 Meeting #98	R5-230813r1

# Athens, Greece, Feb 27th – Mar 3rd, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.905 | CR | 0731 | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to TP analysis of 6.2.3 NS_27 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_bands_BW_R16-UEConTest |  |  |  |  |  | Date: |  |  | 2023-02-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - The minimum requirement of NS_27 is not aligned with latest 38.101-1. - The proposals are not aligned with existing test configuration table in TS 38.521-1 |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Updating the minimum requirement of NS_27. Relevant test points are not impacted. - Updating proposals to align with existing test configuration tables in TS 38.521-1. The changes have no impact to test case details. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The TP analysis is not aligned with test configurations. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Added zip file: 38.521-1_TPanalysis_6.2.3_AMPR_NS_27_v5.zip Deleted zip files from TR 38.905: 38.521-1_TPanalysis_6.2.3_AMPR_NS_27_v4.zip |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: Adding Year information in coversheet. |  |  |  |  |  |  |  |  |

<Start of Changes>

4.1.2.1	A-MPR, A-SEM and A-SE FR1 test cases for single carrier and UL MIMO

This section contains information on test point selection for single carrier test cases 6.2.3, Additional Maximum Power Reduction (A-MPR), 6.5.2.3 Additional spectrum emission mask (A-SEM) and 6.5.3.3 Additional spurious emissions (A-SE); and for correspondent UL-MIMO test cases in 6.2D.3 and 6.5D.3.3 in [2].

Selection of test points should include some possible worst combinations based on the A-MPR characteristics specified for each NS value and these shall be selected so that they match with corresponding spectrum emission requirements test points. The number of test points should be realistic.

For pi/2 BPSK with Rel-16 DMRS, the correspondent A-MPR requirements are the same as that for Rel-15 DMRS but the PAPR is lower. Given the UE can pass the requirements using Rel-15 DMRS, it can be expected that the UE can pass the requirements using Rel-16 DMRS. Therefore there is no need to additionally test A-MPR, A-SEM and A-SE for Rel-16 DMRS.

Table 4.1.2.1-1 lists number of test points for A-MPR, A-SEM and A-SE single carrier test cases and for different NS values.

Table 4.1.2.1-1: NS value specific test points for A-MPR, A-SEM and A-SE single carrier

| NS label | Number of test points for A-MPR | Number of test points A-SEM | Number of test points A-SE | Justification | Comments |
| --- | --- | --- | --- | --- | --- |
| NS_03 NS_03U | 144 for Non-SUL testing 72 for SUL testing |  |  | “38.521-1_TPanalysis_6.2.3_AMPR_NS_03.zip” | RAN5#85 |
| NS_04 | 6.2.3: 120 6.2D.3: 112 6.5D.3.3:112 |  |  | “38.521-1_TPanalysis_6.2.3_AMPR_NS_04_v3.zip” | RAN5#93-e |
| NS_05 | 6.5.3.3: 432 |  |  | “38.521-1_TP analysis_6.5.3.3_TX_Additional_Spurious_Emission_NS_05.zip” | RAN5#87-e |
| NS_05, NS_05U | 6.2.3: 288 for PC3 6.2.3: 396 for PC2 |  |  | “38.521-1_TPanalysis_6.2.3_AMPR_NS_05_v3.zip” | RAN5#96-e |
| NS_06 | 6.2.3: PC1 160 PC3 144 |  |  | “38.521-1_TPanalysis_6.2.3_AMPR_NS_06_v2.zip” | RAN5#92-e |
| NS_12 | 48 |  |  | “38.521-1_TPanalysis_6.2.3_AMPR_NS_12_13_14_15.zip” | RAN5#90-e |
| NS_13 | 21 |  |  | “38.521-1_TPanalysis_6.2.3_AMPR_NS_12_13_14_15.zip” | RAN5#90-e |
| NS_14 | 6.2.3: 50 6.5.3.3: 25 |  | 25 | “38.521-1_TPanalysis_6.2.3_AMPR_NS_12_13_14_15.zip” “38.521-1_TPanalysis_6.5.3.3_TX_Additional_Spurious_Emission_NS_14.zip | RAN5#90-e RAN5#96-e |
| NS_15 | 102 |  |  | “38.521-1_TPanalysis_6.2.3_AMPR_NS_12_13_14_15.zip” | RAN5#90-e |
| NS_17 | 6.5.3.3: 4 |  |  | “38.521-1_TPanalysis_6.5.3.3_TX_Additional_Spurious_Emission_NS_17_v2.zip” | RAN5#92-e |
| NS_18 | 88 6.2.3: 108 6.5.3.3: 54 |  |  | “38.521-1_TPanalysis_6.2.3_AMPR_6.5.3.3_ASE_NS_18_v3.zip” | RAN5#89-e |
| NS_21 | 180 |  |  | “38.521-1_TPanalysis_6.2.3_AMPR_NS_21.zip” | RAN5#89-e |
| NS_24 | 6.2.3: 300 |  |  | “38.521-1_TPanalysis_6.2.3_AMPR_NS_24.zip” | RAN5#87 |
| NS_27 | 6.2.3: 360 |  |  | “38.521-1_TPanalysis_6.2.3_AMPR_NS_27_v5.zip” | RAN5#98 |
| NS_35 | 6.2.3: 144 6.2.3: 72 |  |  | “38.521-1_TPanalysis_6.2.3_AMPR_NS_35_v2.zip” | RAN5#5-5G-NR-Adhoc |
| NS_37 | 6.2.3: 48 |  |  | “38.521-1_TPanalysis_6.2.3_AMPR_NS_37.zip” | RAN5#86 |
| NS_38 | 6.2.3: 96 |  |  | “38.521-1_TPanalysis_6.2.3_AMPR_NS_38.zip” | RAN5#86 |
| NS_39 | 6.2.3: 54 |  |  | “38.521-1_TPanalysis_6.2.3_AMPR_NS_39.zip” | RAN5#86 |
| NS_40 | 6.2.3: 24 |  |  | “38.521-1_TPanalysis_6.2.3_AMPR_NS_40.zip” | RAN5#87 |
| NS_41 | 6.2.3: 72 |  |  | “38.521-1_TPanalysis_6.2.3_AMPR_NS_41.zip” | RAN5#87 |
| NS_42 | 6.2.3: 108 |  |  | “38.521-1_TPanalysis_6.2.3_AMPR_NS_42.zip” | RAN5#87 |
| NS_43 | 6.2.3: 28 |  |  | “38.521-1_TPanalysis_6.2.3_AMPR_NS_43_v2.zip” | RAN5#86 |
|  | 6.5.3.3: 81 |  |  | “38.521-1_TP analysis_6.5.3.3_TX_Additional_Spurious_Emission_NS_43.zip” | RAN5#87-e |
| NS_43U | 6.2.3: 72 |  |  | “38.521-1_TPanalysis_6.2.3_AMPR_NS_43U.zip” | RAN5#85 |
| NS_44 | 360 | N/A | 180 | “38.521-1_TPanalysis_6.2.3_AMPR_6.5.3.3_ASE_NS_44.zip” | RAN5#90-e |
| NS_45 | 24 |  |  | “38.521-1_TPanalysis_6.2.3_AMPR_NS_45.zip” | RAN5#89-e |
| NS_46 | 176 |  |  | “38.521-1_TPanalysis_6.2.3_AMPR_NS_46.zip” | RAN5#89-e |
| NS_47 | 70 |  |  | “38.521-1_TPanalysis_6.2.3_AMPR_NS_47.zip” | RAN5#87 |
| NS_48 | 240 for PC3 384 for PC2 | N/A | 96 | “38.521-1_TPanalysis_6.2.3_AMPR_6.5.3.3_ASE_NS_48_v3.zip” | RAN5#96-e |
| NS_49 | 280 for PC3 336 for PC2 | N/A | 112 | “38.521-1_TPanalysis_6.2.3_AMPR_6.5.3.3_ASE_NS_49_v1.zip” | RAN5#97-e |
| NS_56 | 175 |  | 175 | 38.521-1_TPanalysis_6.2.3_AMPR_6.5.3.3_ASE_NS_56.zip | RAN5#93-e |
| NS_100 | 144 for Non-SUL testing 72 for SUL testing |  |  | “38.521-1_TPanalysis_6.2.3_AMPR_NS_100.zip” | RAN5#85 |

< End of Changes >

Add attached .zip files to TR 38.905:

“38.521-1_TPanalysis_6.2.3_AMPR_NS_27_v5.zip”

Deleted zip files from TR 38.905:

“38.521-1_TPanalysis_6.2.3_AMPR_NS_27_v4.zip”