3GPP TSG-RAN5 Meeting #98	R5-230860r2

# Athens, Greece, 27th Feb - 3rd March, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 2245 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | add new RRC messages and information elements contents for TS38.533 Annex H.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_feMIMO-UEConTest |  |  |  |  |  | Date: |  |  | 2023-2-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | TRP specific Beam Failure Detection and Link Recovery Test need new message in order to make the test work normally. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add new RadioLinkMonitoringConfig basic on Release 17 content in the TS38.331 for the TRP specific BFD and LRM - Add new BeamFailureRecoverySpCellConfig basic on Release 17 content in the TS38.331 for the TRP specific BFD and LRM - Add new BWP-DownlinkDedicated config basic on Release 17 content in the TS38.331 for the TRP specific BFD and LRM R1: - Updated coversheet issues. - Table H.3.1-8A, removed all ‘purpose’ message and unnecessary text, corrected wrong |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The new messages and information elements are the necessary for the TRP specific beam failure dection and link recovery, if not approved these messages, TRP specific BFD and LRM test cases cannot work normally. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | H.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR … CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from R5-230860r1, to fixed 3GU issue. |  |  |  |  |  |  |  |  |

H.3	RRC message content exceptions

H.3.1	RRC messages and information elements contents exceptions for NR measurement configuration

<< unchanged part skipped>>

RadioLinkMonitoringConfig -DEFAULT

Default configuration for RLM resources.

Table H.3.1-8: RadioLinkMonitoringConfig-DEFAULT: Default configuration for RLM and BFD resources

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-133 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioLinkMonitoringConfig ::= SEQUENCE { |  |  |  |
| failureDetectionResourcesToAddModList SEQUENCE (SIZE(1..maxNrofFailureDetectionResources)) OF { | n entries | n = 1 for FR1, n = 2 for FR2 | SSB RLM OR CSI-RS RLM |
| RadioLinkMonitoringRS[1] SEQUENCE { |  |  |  |
| radioLinkMonitoringRS-Id | 0 |  |  |
| purpose | rlf |  |  |
| detectionResource CHOICE { |  |  |  |
| ssb-Index | 0 | Index of SSB #0 | SSB RLM |
| csi-RS-Index | NZP-CSI-RS-ResourceId for TRS(4) | TS 38.508-1[14], table 7.3.1-7C | CSI-RS RLM |
| } |  |  |  |
| } |  |  |  |
| RadioLinkMonitoringRS[2] SEQUENCE { |  |  | FR2 |
| radioLinkMonitoringRS-Id | 1 |  |  |
| purpose | rlf |  |  |
| detectionResource CHOICE { |  |  |  |
| ssb-Index | 1 | Index of SSB #1 | SSB RLM |
| csi-RS-Index | NZP-CSI-RS-ResourceId for TRS(4) with condition SECOND_SET | TS 38.508-1[14], table 7.3.1-7C | CSI-RS RLM |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| failureDetectionResourcesToAddModList SEQUENCE (SIZE(1..maxNrofFailureDetectionResources)) OF { | 2 entries |  | SSB BFD OR CSI-RS BFD |
| RadioLinkMonitoringRS[1] SEQUENCE { |  |  |  |
| radioLinkMonitoringRS-Id | 0 |  |  |
| purpose | both |  |  |
| detectionResource CHOICE { |  |  |  |
| ssb-Index | 0 | Index of SSB #0 | SSB BFD |
| csi-RS-Index | NZP-CSI-RS-ResourceId for BM(0) | TS 38.508-1[14], Table 7.3.1-7E | CSI-RS BFD |
| } |  |  |  |
| } |  |  |  |
| RadioLinkMonitoringRS[2] SEQUENCE { |  |  |  |
| radioLinkMonitoringRS-Id | 1 |  |  |
| purpose | rlf |  |  |
| detectionResource CHOICE { |  |  |  |
| ssb-Index | 1 | Index of SSB #1 | SSB BFD |
| csi-RS-Index | NZP-CSI-RS-ResourceId for BM(1) | TS 38.508-1[14], Table 7.3.1-7E | CSI-RS BFD |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| beamFailureInstanceMaxCount | n1 |  | SSB BFD OR CSI-RS BFD |
| beamFailureDetectionTimer | pbfd4 |  | SSB BFD OR CSI-RS BFD |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SSB RLM | Used when SSB based RLM is configured in test case |
| CSI-RS RLM | Used when CSI-RS based RLM is configured in test case |
| SSB BFD | Used when SSB based BFD is configured in test case |
| CSI-RS BFD | Used when CSI-RS based BFD is configured in test case |

<< Start of change 1>>

Table H.3.1-8A: RadioLinkMonitoringConfig-Release 17: Release 17 configuration for TRP specific RLM and BFD resources

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-133 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioLinkMonitoringConfig ::= SEQUENCE { |  |  |  |
| BeamFailureDetection-r17::= SEQUENCE { |  |  |  |
| failureDetectionSet1-r17::= SEQUENCE { |  |  |  |
| bfdResourcesToAddModList-r17 (SIZE(1..maxNrofFailureDetectionResources)) OF { | n entries | n = 1 for FR1, n = 2 for FR2 | SSB RLM OR CSI-RS RLM |
| BeamLinkMonitoringRS[1]-r17 ::= SEQUENCE { |  |  |  |
| beamLinkMonitoringRS-Id-r17 | 0 |  |  |
| detectionResource-r17 CHOICE { |  |  |  |
| ssb-Index | 0 | Index of SSB #0 | SSB RLM |
| csi-RS-Index | NZP-CSI-RS-ResourceId for TRS(4) | TS 38.508-1[14], table 7.3.1-7C | CSI-RS RLM |
| } |  |  |  |
| } |  |  |  |
| BeamLinkMonitoringRS[2]-r17 ::= SEQUENCE { |  |  |  |
| beamLinkMonitoringRS-Id-r17 | 1 |  |  |
| detectionResource-r17 CHOICE { |  |  |  |
| ssb-Index | 1 | Index of SSB #1 | SSB RLM |
| csi-RS-Index | NZP-CSI-RS-ResourceId for TRS(4) with condition SECOND_SET | TS 38.508-1[14], table 7.3.1-7C | CSI-RS RLM |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| bfdResourcesToReleaseList-r17 SEQUENCE (SIZE(1..maxNrofFailureDetectionResources)) OF { | 2 entries |  | SSB BFD OR CSI-RS BFD |
| BeamLinkMonitoringRS[1]-r17 ::= SEQUENCE { |  |  |  |
| beamLinkMonitoringRS-Id-r17 | 0 |  |  |
| detectionResource-r17 CHOICE { |  |  |  |
| ssb-Index | 0 | Index of SSB #0 | SSB BFD |
| csi-RS-Index | NZP-CSI-RS-ResourceId for BM(0) | TS 38.508-1[14], Table 7.3.1-7E | CSI-RS BFD |
| } |  |  |  |
| } |  |  |  |
| BeamLinkMonitoringRS[2]-r17 ::= SEQUENCE { |  |  |  |
| beamLinkMonitoringRS-Id-r17 | 1 |  |  |
| detectionResource-r17 CHOICE { |  |  |  |
| ssb-Index | 1 | Index of SSB #1 | SSB BFD |
| csi-RS-Index | NZP-CSI-RS-ResourceId for BM(1) | TS 38.508-1[14], Table 7.3.1-7E | CSI-RS BFD |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| beamFailureInstanceMaxCount-r17 | n1 |  | SSB BFD OR CSI-RS BFD |
| beamFailureDetectionTimer-r17 | pbfd4 |  | SSB BFD OR CSI-RS BFD |
| } |  |  |  |
| failureDetectionSet2-r17::= SEQUENCE { |  |  |  |
| bfdResourcesToAddModList-r17 (SIZE(1..maxNrofFailureDetectionResources)) OF { | n entries | n = 1 for FR1, n = 2 for FR2 | SSB RLM OR CSI-RS RLM |
| BeamLinkMonitoringRS[3]-r17 ::= SEQUENCE { |  |  |  |
| beamLinkMonitoringRS-Id-r17 | 2 |  |  |
| detectionResource-r17 CHOICE { |  |  |  |
| ssb-Index | 2 | Index of SSB #2 | SSB RLM |
| csi-RS-Index | NZP-CSI-RS-ResourceId for TRS(4) | TS 38.508-1[14], table 7.3.1-7C | CSI-RS RLM |
| } |  |  |  |
| } |  |  |  |
| BeamLinkMonitoringRS[4]-r17 ::= SEQUENCE { |  |  |  |
| beamLinkMonitoringRS-Id-r17 | 3 |  |  |
| detectionResource-r17 CHOICE { |  |  |  |
| ssb-Index | 3 | Index of SSB #3 | SSB RLM |
| csi-RS-Index | NZP-CSI-RS-ResourceId for TRS(4) with condition SECOND_SET | TS 38.508-1[14], table 7.3.1-7C | CSI-RS RLM |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| bfdResourcesToReleaseList-r17 SEQUENCE (SIZE(1..maxNrofFailureDetectionResources)) OF { | 2 entries |  | SSB BFD OR CSI-RS BFD |
| BeamLinkMonitoringRS[3]-r17 ::= SEQUENCE { |  |  |  |
| beamLinkMonitoringRS-Id-r17 | 2 |  |  |
| detectionResource-r17 CHOICE { |  |  |  |
| ssb-Index | 2 | Index of SSB #2 | SSB BFD |
| csi-RS-Index | NZP-CSI-RS-ResourceId for BM(0) | TS 38.508-1[14], Table 7.3.1-7E | CSI-RS BFD |
| } |  |  |  |
| } |  |  |  |
| BeamLinkMonitoringRS[4]-r17 ::= SEQUENCE { |  |  |  |
| beamLinkMonitoringRS-Id-r17 | 3 |  |  |
| detectionResource-r17 CHOICE { |  |  |  |
| ssb-Index | 3 | Index of SSB #3 | SSB BFD |
| csi-RS-Index | NZP-CSI-RS-ResourceId for BM(1) | TS 38.508-1[14], Table 7.3.1-7E | CSI-RS BFD |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| beamFailureInstanceMaxCount-r17 | n1 |  | SSB BFD OR CSI-RS BFD |
| beamFailureDetectionTimer-r17 | pbfd4 |  | SSB BFD OR CSI-RS BFD |
| } |  |  |  |
| additionalPCI-r17 | not present |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SSB RLM | Used when SSB based RLM is configured in test case |
| CSI-RS RLM | Used when CSI-RS based RLM is configured in test case |
| SSB BFD | Used when SSB based BFD is configured in test case |
| CSI-RS BFD | Used when CSI-RS based BFD is configured in test case |

<< end of change 1>>

<< unchanged part skipped>>

BeamFailureRecoverySCellConfig-DEFAULT

Default configuration for CBD and contention-free RACH in link recovery.

Table H.3.1-12: BeamFailureRecoverySCellConfig-DEFAULT

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-6AA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BeamFailureRecoverySCellConfig ::= SEQUENCE { |  |  |  |
| rsrp-ThresholdBFR-r16 | 58 | -98dBm | SCS15 |
|  | 61 | -95dBm | SCS30 |
|  | 47 | -109dBm | SCS120 |
| candidateBeamRSSCellList-r16 SEQUENCE (SIZE(1..maxNrofCandidateBeams-r16)) OF SEQUENCE { |  |  |  |
| CandidateBeamRS-r16[1] SEQUENCE { |  |  |  |
| candidateBeamConfig-r16 CHOICE { |  |  |  |
| ssb-r16 | 1 |  | SSB |
| csi-RS-r16 | NZP-CSI-RS-ResourceId of the CSI-RS resource used for CBD |  | CSI-RS |
| } |  |  |  |
| servingCellId | ServCellIndex of the SCell |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

<< start of change 2>>

Table H.3.1-12A: BeamFailureRecoverySpCellConfig-DEFAULT

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BeamFailureRecoverySpCellConfig ::= SEQUENCE { |  |  |  |
| rsrp-ThresholdBFR-r16 | 58 | -98dBm | SCS15 |
|  | 61 | -95dBm | SCS30 |
|  | 47 | -109dBm | SCS120 |
| candidateBeamRSSpCellList2-r17 SEQUENCE (SIZE(1..maxNrofCandidateBeams-r16)) OF SEQUENCE { |  |  |  |
| CandidateBeamRS-r16[1] SEQUENCE { |  |  |  |
| candidateBeamConfig-r16 CHOICE { |  |  |  |
| ssb-r16 | 1 |  | SSB |
| csi-RS-r16 | NZP-CSI-RS-ResourceId of the CSI-RS resource used for CBD |  | CSI-RS |
| } |  |  |  |
| servingCellId | ServCellIndex of the SpCell |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

<< end of change 2>>

<< start of change 3>>

BWP-DownlinkDedicated

Table H.3.1-13: BWP-DownlinkDedicated

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-DownlinkDedicated ::= SEQUENCE { |  |  |  |
| beamFailureRecoverySCellConfig-r16 CHOICE { |  |  |  |
| setup | BeamFailureRecoverySCellConfig-DEFAULT |  | SCell |
| } |  |  |  |
| beamFailureRecoverySpCellConfig-r17 CHOICE { |  |  |  |
| setup | BeamFailureRecoverySpCellConfig-DEFAULT |  | SpCell |
| } |  |  |  |
| } |  |  |  |

<< end of change 3>>