3GPP TSG-RAN WG5 Meeting #98                                         R5-230950r2

Athens, Greece, 27th Feb – 3rd Mar. 2023

Source: 	Huawei, HiSilicon

Title: 	Discussion on handling simultaneous Rx/Tx capability for REFSENS testing

Agenda Item:	5.4.17

Document for:	Discussion

Introduction

Simultaneous Rx/Tx capability for inter-band CA, SUL and EN-DC band combinations were introduced from Rel-15, but actually not all combinations mandatorily support this capability.

For RAN5 REFSENS test case, the exception testing is based on the assumption that UE supports simultaneous Rx/Tx capability regardless of support for simultaneous Rx/Tx, which is apparently not in line with RAN4 core requirements. This document is proposed to analysis how to consider simultaneous Rx/Tx capability in RAN5 REFSENS testing.

Discussion

The applicability of simultaneous Rx/Tx

In RAN4, there is a Rel-17 work item Simultaneous Rx/Tx band combinations for NR CA/DC, NR SUL and LTE/NR DC which aims to define clear principles for inter-band CA, SUL and EN-DC band combinations for which can support or not support simultaneous Rx/Tx. In the WID [1], the description of the applicability of simultaneous Rx/Tx is as follows:

Simultaneous Rx/Tx capability for inter-band CA, SUL and EN-DC band combinations were introduced from Rel-15. Specifically, for inter-band CA and EN-DC combination, the capability is used for TDD-TDD and TDD-FDD band combinations. According to the description of the capability, it is conditional mandatory and the condition is described in the field, i.e. indicated in the RAN4 spec which combinations should mandatorily support simultaneous Rx/Tx. For the combinations which have no such indication, the capability is optional, i.e. for UE supporting simultaneous Rx/Tx, the capability should be reported, otherwise, the capability is absent or not reported. Since the capability is important for network scheduling, it should be reported accurately.

Take inter-band EN-DC as an example, the wording “indication” in the above is like a NOTE in TS 38.101-3 as below:

- 5.5B.4	Inter-band EN-DC within FR1

- 5.5B.4.1	Inter-band EN-DC configurations within FR1 (two bands)

Table 5.5B.4.1-1: Inter-band EN-DC configurations within FR1 (two bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed | DL interruption allowed (Note 14) |
| --- | --- | --- | --- |
| DC_1A_n3A DC_1C_n3A | DC_1A_n3A DC_1C_n3A | DC_1_n3 |  |
| DC_1A_n5A | DC_1A_n5A | No |  |
| DC_1A_n7A DC_1A_n7B | DC_1A_n7A DC_1A_n7B | No |  |
| DC_1A-1A_n7A DC_1A-1A_n7B | DC_1A_n7A | No |  |
| DC_1A_n8A | DC_1A_n8A | No |  |
| DC_1A_n20A | DC_1A_n20A | No |  |
| DC_1A_n28A | DC_1A_n28A | No |  |
| DC_1A-1A_n28A | DC_1A_n28A | No |  |
| DC_1A_n38A DC_1C_n38A | DC_1A_n38A | No |  |
| DC_1A_n40A DC_1A_n40B | DC_1A_n40A | No |  |
| DC_1A_n41A7 | DC_1A_n41A | No |  |
| DC_1A_n50A | DC_1A_n50A | No |  |
| DC_1A_n51A | DC_1A_n51A | No |  |
| DC_1A_n71A DC_1A_n71B | DC_1A_n71A | No |  |
| DC_1A_n77A7 DC_1A_n77C7 | DC_1A_n77A | DC_1_n77 | No |
| DC_1A_n77(2A)7 DC_1A_n77(3A)7 | DC_1A_n77A | DC_1_n77 | No |
| DC_1A_n78A7 DC_1A_n78C7 | DC_1A_n78A | No | No |
| DC_1A_n78(2A)7 | DC_1A_n78A | No | No |
| DC_1A-1A_n78A | DC_1A_n78A | No | No |
| DC_1A_n79A7 DC_1A_n79C7 | DC_1A_n79A | No | No |
| DC_2A_n5A | DC_2A_n5A | No |  |
| DC_2A-2A_n5A | DC_2A_n5A | No |  |
| DC_2A_n7A | DC_2A_n7A | No |  |
| DC_2A_n7(2A) | DC_2A_n7A | No |  |
| DC_2A_n12A | DC_2A_n12A | No |  |
| DC_2A_n25A11, 13, 20 | N/A | N/A |  |
| DC_2A_n28A | DC_2A_n28A | No |  |
| DC_2A_n30A | DC_2A_n30A | No |  |
| DC_2A-2A_n30A | DC_2A_n30A | No |  |
| DC_2A_n38A | DC_2A_n38A | No |  |
| DC_2A-2A_n38A | DC_2A_n38A | No |  |
| DC_2A_n41A DC_2A_n41C DC_2C_n41A | DC_2A_n41A DC_2C_n41A | No |  |
| DC_2A_n41(2A) | DC_2A_n41A | No |  |
| DC_2A-2A_n41A | DC_2A_n41A | No |  |
| DC_2A_n46A | DC_2A_n46A | No |  |
| DC_2A_n48A DC_2A_n48B | DC_2A_n48A | No |  |
| DC_2A_n66A | DC_2A_n66A | DC_2_n66 |  |
| DC_2A_n66(2A) | DC_2A_n66A | DC_2_n66 |  |
| DC_2A-2A_n66A | DC_2A_n66A | DC_2_n66 |  |
| DC_2A_n71A DC_2A_n71B DC_2C_n71A | DC_2A_n71A | No |  |
| DC_2A-2A_n71A | DC_2A_n71A | No |  |
| DC_2A_n77A DC_2A_n77C | DC_2A_n77A | DC_2_n77 |  |
| DC_2A_n77(2A) | DC_2A_n77A | DC_2_n77 |  |
| DC_2A-2A_n77A DC_2A-2A_n77C | DC_2A_n77A | DC_2_n77 |  |
| DC_2A-2A_n77(2A) | DC_2A_n77A | DC_2_n77 |  |
| DC_2A_n78A | DC_2A_n78A | DC_2_n78 |  |
| DC_2A-2A_n78(2A) | DC_2A_n78A | DC_2_n78 |  |
| DC_2A_n78(2A) | DC_2A_n78A | DC_2_n78 |  |
| DC_2A-2A_n78A | DC_2A_n78A | DC_2_n78 |  |
| DC_3A_n1A DC_3C_n1A | DC_3A_n1A DC_3C_n1A | DC_3_n1 |  |
| DC_3A-3A_n1A | DC_3A_n1A | DC_3_n1 |  |
| DC_3A_n5A DC_3C_n5A | DC_3A_n5A | DC_3_n5 |  |
| DC_3A_n7A DC_3A_n7B DC_3C_n7A DC_3C_n7B | DC_3A_n7A DC_3A_n7B DC_3C_n7A | No |  |
| DC_3A-3A_n7A DC_3A-3A_n7B | DC_3A_n7A | No |  |
| DC_3A_n8A | DC_3A_n8A | No |  |
| DC_3A-3A_n8A | DC_3A_n8A | No |  |
| DC_3A_n20A DC_3C_n20A | DC_3A_n20A | No |  |
| DC_3A_n28A DC_3C_n28A | DC_3A_n28A | No |  |
| DC_3A_n34A | DC_3A_n34A | No |  |
| DC_3A_n38A DC_3C_n38A | DC_3A_n38A | No |  |
| DC_3A_n40A DC_3A_n40B | DC_3A_n40A | No |  |
| DC_3A_n41A7 DC_3C_n41A | DC_3A_n41A DC_3C_n41A | DC_3_n41 | No |
| DC_3A_n50A | DC_3A_n50A | No |  |
| DC_3A_n51A | DC_3A_n51A | No |  |
| DC_3A_n71A DC_3A_n71B | DC_3A_n71A | No |  |
| DC_3A_n77A7 DC_3A_n77C7 DC_3C_n77A7 | DC_3A_n77A DC_3C_n77A | DC_3_n77 | No |
| DC_3A_n77(2A)7 DC_3A_n77(3A)7 DC_3C_n77(2A)7 | DC_3A_n77A DC_3C_n77A | DC_3_n77 | No |
| DC_3A-3A_n77A7 | DC_3A_n77A | DC_3_n77 | No |
| DC_3A_n78A7 DC_3A_n78C7 DC_3C_n78A7 | DC_3A_n78A DC_3C_n78A | DC_3_n78 | No |
| DC_3A_n78(2A)7 DC_3C_n78(2A)7 | DC_3A_n78A DC_3C_n78A | DC_3_n78 | No |
| DC_3A-3A_n78A7, 21 | DC_3A_n78A21 | DC_3_n78 | No |
| DC_3A_n79A7 DC_3A_n79C7 DC_3C_n79A7 | DC_3A_n79A DC_3C_n79A | No | No |
| DC_4A_n2A | DC_4A_n2A | No |  |
| DC_4A_n5A | DC_4A_n5A | DC_4_n5 |  |
| DC_4A_n7A | DC_4A_n7A | No |  |
| DC_4A_n28A | DC_4A_n28A | No |  |
| DC_4A_n38A | DC_4A_n38A | No |  |
| DC_4A_n41A | DC_4A_n41A | No |  |
| DC_4A_n78A | DC_4A_n78A | No |  |
| DC_4A_n78(2A) | DC_4A_n78A | No |  |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 2: Restricted to E-UTRA operation when inter-band carrier aggregation is configured. The downlink operating band for Band 46 is paired with the uplink operating band (external E-UTRA band) of the carrier aggregation configuration that is supporting the configured Pcell. NOTE 3: The minimum requirements apply only when there is non-simultaneous Tx/Rx operation between E-UTRA and NR carriers. This restriction applies also for these carriers when applicable EN-DC configuration is part of a higher order EN-DC configuration. NOTE 4: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for intra-band non-contiguous EN-DC apply for the Band 42/48 and Band n77/n78 combination. For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, when UE capability interBandContiguousMRDC is indicated, the minimum requirements for intra-band-contiguous EN-DC also should be met in addtion to intra-band non-contiguous EN-DC. The intra-band requirements also apply for these carriers when applicable EN-DC configuration is a subset of a higher order EN-DC configuration. NOTE 5: The frequency range above 3600 MHz for Band n78 is not used in this combination. NOTE 6: The frequency range below 2506 MHz for Band 41 is not used in this combination. NOTE 7: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability. NOTE 8: The frequency range in band n28 / 28 is restricted for this band combination to 703 - 733 MHz for the UL and 758-788 MHz for the DL. This restriction also apply for any band combinations when DC_20_n28/ DC_28_n20/ CA_20-28/ CA_n20-n28 is a subset of a higher order band combination. NOTE 9: The combination is not used alone as fall back mode of other band combinations in which UL in Band 42 or Band 48 is not used. |  |  |  |

For combinations with Note 7 suffix, UE should mandatorily support simultaneous Rx/Tx capability. For combinations without Note 7 suffix, UE can optionally support simultaneous Rx/Tx depending on the reported UE capability. This criterion is not only for EN-DC combinations but also for inter-band CA and SUL combinations. Therefore, we can summarize the applicability of simultaneous Rx/Tx as follows:

Observation 1: For inter-band CA, SUL and EN-DC band combination with explicit indication in TS 38.101-1 clause 5.2A, TS 38.101-1 clause 5.3A and TS 38.101-3 clause 5.5B, UE should mandatorily support simultaneous Rx/Tx with UE capability indicating support.

Observation 2: For inter-band CA, SUL and EN-DC band combination without indication in TS 38.101-1 clause 5.2A, TS 38.101-1 clause 5.3A and TS 38.101-3 clause 5.5B, UE can optionally support simultaneous Rx/Tx depending on the reported UE capability indicating support or not.

Reference sensitivity tests

According to RAN4 specifications, reference sensitivity requirements for inter-band CA, SUL and EN-DC band combination are defined in TS 38.101-1 and TS 36.101 including requirements with and without sensitivity degradation exception. For those combinations with reference sensitivity exceptions, maximum sensitivity degradation (MSD) will be applied into applicable REFSENS requirement defined in TS 38.101-1 and TS 36.101.

Take inter-band EN-DC as an example, the reference sensitivity exceptions may be caused by UL harmonics, receiver harmonic mixing, cross band isolation and dual uplink intermodulation, where there is uplink transmission in the aggressor band and downlink transmission in the victim band. In other words, Reference sensitivity exceptions only apply when UE performs uplink transmission and downlink transmission at the same time.

Observation 3: Reference sensitivity exceptions are only applicable to UEs with support for simultaneous Rx/Tx capability.

In the current RAN5 specifications, reference sensitivity testing includes both non-exception testing and exception testing. For exception testing, the test configuration is based on the assumption that UE supports simultaneous Rx/Tx capability regardless of support for simultaneous Rx/Tx capability, which is apparently not in line with RAN4 core requirements. For combinations with mandatory support for simultaneous Rx/Tx, the corresponding UE capability must indicate support, the current REFSENS testing structure is applicable. But for those combinations with optional support for simultaneous Rx/Tx, the exception testing is not necessarily applicable based on RAN4 core requirements.

Take DC_1A_n8A as an example, UE can optionally support simultaneous Rx/Tx for this configuration. So for those UEs reporting support of simultaneous Rx/Tx capability, exception testing can be performed well but for those UEs not reporting support of simultaneous Rx/Tx capability, exception testing can't be tested since network is not expected to schedule uplink transmission and downlink transmission at the same time. To handle simultaneous Rx/Tx capability in REFSENS testing, the following proposals can be considered.

Proposal 1: Void.

Proposal 2: For inter-band CA, SUL and EN-DC band combination, whether exception testing can be performed depends on UEs support for simultaneous Rx Tx capability:

- If UEs support simultaneous Rx/Tx capability, both non-exception testing and exception testing should be performed.

- If UEs do not support simultaneous Rx/Tx capability, only non-exception testing needs to be performed.

Based on the above proposals, whether simultaneous Rx/Tx capability is supported or not would decide if REFSENS exception testing can be executed. Hence, the test applicability of exception testing can be updated as below:

Table 7.3B.2.3.4.2.1-6: Test Configuration Table for EN-DC configurations affected by Reference sensitivity exceptions (two bands)

| Test Settings for a DC_1A_n8A Configuration |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 14 | 1 | UL 1965 / DL 2155 |  |  | n8 | UL 887.5 / DL 932.5 |  |  |
|  | QPSK | QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK | CP-OFDM QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 |
| Test Settings for a DC_1A_n77A Configuration |  |  |  |  |  |  |  |  |  |
| 13 | 1 | UL 1950/ DL 2140 |  |  | n77 | UL/DL 3900 |  |  |  |
|  | QPSK | QPSK | 20 MHz | All RBs / REFSENS_ENDC_1 | N/A | CP-OFDM QPSK | 100 MHz | All RBs / 0 |  |
| 23 | 1 | UL 1950/ DL 2140 |  |  | n77 | UL/DL 3870 |  |  |  |
|  | QPSK | QPSK | 20 MHz | All RBs / REFSENS_ENDC_1 | N/A | CP-OFDM QPSK | 20 MHz | All RBs / 0 |  |
| 34 | 1 | UL 1950/ DL 2140 |  |  | n77 | UL/DL 4090.005 |  |  |  |
|  | QPSK | QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 | N/A | CP-OFDM QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 |  |
| 44 | 1 | UL 1950/ DL 2140 |  |  | n77 | UL/DL 3709.005 |  |  |  |
|  | QPSK | QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 | N/A | CP-OFDM QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 |
| NOTE 1: REFSENS_LTE refers to the single carrier Uplink RB allocation for reference sensitivity according to table 7.3.3-2 of TS 36.521-1 [10]. REFSENS_NR refers to the single carrier Uplink RB allocation for reference sensitivity according to table 7.3.2.4.1-3 of TS 38.521-1 [8]. REFSENS_ENDC_1 refers to the Uplink RB allocation for reference sensitivity exceptions due to UL harmonic interference according to table 7.3B.2.0.3.1-2. REFSENS_ENDC_2 refers to the Uplink RB allocation for reference sensitivity exceptions due to receiver harmonic mixing according to table 7.3B.2.0.3.2-2. REFSENS_ENDC_3 refers to the Uplink RB allocation for reference sensitivity exceptions due to cross band isolation according to table 7.3B.2.0.3.4-2. REFSENS_ENDC_4 refers to the Uplink RB allocation for reference sensitivity exceptions due to dual uplink operation for ENDC according to table 7.3B.2.0.3.5.1-1 for PC3 and table 7.3B.2.0.3.5.1-1a for PC2. NOTE 2: Void NOTE 3: Test ID with UL harmonic exception NOTE 4: Test ID with 2UL intermodulation exception NOTE 5: Test ID with UL receiver harmonic mixing NOTE 6: Test ID with UL cross band isolation NOTE 7: Void NOTE 8: Test ID with UL harmonic exception avoided NOTE 9: Test ID with UL receiving harmonic mixing exception avoided NOTE 10: Only applicable to UEs not supporting UE capability singleUL-Transmission. NOTE 11: Test ID with Cross band isolation exception avoided, which is only applicable to DC_1_n3, DC_3_n1 and DC_3_n84. NOTE 12: In an E-UTRA band or FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected. NOTE 13: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 from TS 38.101-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. NOTE 14: If the NR frequency does not match to a valid NR-ARFCN, apply the closest NR frequency with a valid NR-ARFCN. NOTE X: The test points are executed only when UEs support simultaneous Rx/Tx capability as indicated in Table A.4.3.2B.2.3.1-2 of TS 38.508-2. |  |  |  |  |  |  |  |  |

Table A.4.3.2B.2.3.1-2: Supported Inter-band EN-DC configurations within FR1 (two bands)

| EN-DC configuration / Item | Release | Supported | Supported EN-DC Bandwidth Class(es) in UL | Supported ULTxSwitching Band Pair | Supported simultaneousRxTx (Note X) |
| --- | --- | --- | --- | --- | --- |
| DC_1A_n3A | Rel-16 |  |  |  |  |
| DC_1A_n5A | Rel-16 |  |  |  |  |
| DC_1A_n7A | Rel-16 |  |  |  |  |
| DC_1A_n28A | Rel-15 |  |  |  |  |
| DC_1A_n77A | Rel-15 |  |  |  | Yes |
| DC_1A_n78A | Rel-15 |  |  |  | Yes |
| DC_1A_n78C | Rel-15 |  |  |  | Yes |
| DC_1A-1A_n78A | Rel-17 |  |  |  |  |
| DC_1A_n79A | Rel-15 |  |  |  | Yes |
| DC_2A_n5A | Rel-15 |  |  |  |  |
| DC_2A_n41A | Rel-16 |  |  |  |  |
| DC_2C_n41A | Rel-16 |  |  |  |  |
| Note X: For configurations with Note 7 in Table 5.5B.4.1-1 of TS 38.521-3 [5], UE capability simultaneousRxTxInterBandENDC is mandatory. |  |  |  |  |  |

Conclusion

To handle simultaneous Rx/Tx capability, RAN5 can consider the following proposal:

Proposal 1: Void

Proposal 2: For inter-band CA, SUL and EN-DC band combination, whether exception testing can be performed depends on UEs support for simultaneous Rx Tx capability:

- If UEs support simultaneous Rx/Tx capability, both non-exception testing and exception testing should be performed.

- If UEs do not support simultaneous Rx/Tx capability, only non-exception testing needs to be performed.

- 4 References

- RP-211382, “revised WID on simultaneous Rx/Tx band combinations for NR CA/DC, NR SUL and LTE/NR DC”, Huawei, Hisilicon