3GPP TSG-RAN5 Meeting #98	R5-230951r1

# Athens, Greece, 27th Feb – 3rd Mar 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-3 | CR | 1566 | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of EN-DC reference sensitivity to handle simultaneous Rx/Tx capability |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-02-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As proposed in R5-230950, for EN-DC configurations reference sensitivity exceptions can be tested only when UE reports support for simultaneous Rx/Tx capability. The current test configuration for inter-band EN-DC does not take simultaneous Rx/Tx capability into consideration, which is not in line with core requirements defined in TS 38.101-3. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update EN-DC REFSENS exception test configuration table to handle simultaneous Rx/Tx capability |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The reference sensitivity for inter-band EN-DC would not be tested properly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3B.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.508-2 CR XXXX |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Associated discussion paper R5-230950 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | The 1st revision: Re-wording the Note to refer to PICS in TS 38.508-2. |  |  |  |  |  |  |  |  |

<Start of Changes>

7.3B.2.3	Reference sensitivity for Inter-band EN-DC within FR1 (2 CCs)

7.3B.2.3.1	Test purpose

Same as in clause 7.3B.2.1.1.

7.3B.2.3.2	Test applicability

This test applies to all types of NR UE release 15 and forward supporting inter-band EN-DC within FR1 with 2 DL CCs.

7.3B.2.3.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 7.3B.2.0.

LTE anchor agnostic approach is not applied.

7.3B.2.3.4	Test description

7.3B.2.3.4.1	Void

7.3B.2.3.4.2	Test description

7.3B.2.3.4.2.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths based on EN-DC operating bands specified in clause 5.5B, channel bandwidths and sub-carrier spacings for the NR cell specified in TS 38.521-1 [8] clause 5.3 and channel bandwidth for the E-UTRA cell are specified in TS 36.521-1 [10] clause 5.4.2. All of these configurations shall be tested with applicable test parameters for each EN-DC configuration, and are shown in Table 7.3B.2.3.4.2.1-0 to Table 7.3B.2.3.4.2.1-6.

The details of the uplink and downlink reference measurement channels (RMCs) are specified in TS 36.521-1 [10] Annex A.2 and A.3 for E-UTRA RMC, and in TS 38.521-1 [8] Annex A.2 and A.3 for NR RMC respectively. The details of the OCNG patterns used are specified in TS 36.521-1 [10] Annex A.5 and in TS 38.521-1 [8] Annex A.5 for E-UTRA CG and NR CG, respectively. Configurations of PDSCH and PDCCH before measurement are specified in TS 36.521-1 [10] Annex C.2 and in TS 38.521-1 [8] Annex C.2 for E-UTRA CG and NR CG, respectively.

For configurations without any reference sensitivity exception, in a FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected. For configurations with reference sensitivity exception, in an E-UTRA band or FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected.

Table 7.3B.2.3.4.2.1-0: Test configurations table for all EN-DC configurations for FR1 non-exception requirements

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |
| NR Test Frequencies as specified in TS 38.508-1 [6] clause4.3.1, E-UTRA Test Frequencies as specified in TS 36.508 [11] clause 4.3.1 |  |  |  |  |  | Mid range for E-UTRA and Mid range for NR |  |  |  |
| Test EN-DC channel bandwidth as specified in TS 36.508 [11] clause 4.3.1 and TS 38.508-1 [6] clause 4.3.1 |  |  |  |  |  | Refer to "NR NRB "and "E-UTRA NRB " columns |  |  |  |
| NR Test SCS as specified in Table 5.3.5-1 in TS 38.521-1 [8] |  |  |  |  |  | Lowest supported SCS |  |  |  |
| Network signalling value |  |  |  |  |  | NS_01 by default Unless given by Table 7.3.3-3 in TS 36.521-1 [10] for the E-UTRA band and Table 7.3.2.3-4 in TS 38.521-1 [8] for the NR band. |  |  |  |
| Test Parameters for DC Configurations |  |  |  |  |  |  |  |  |  |
| ID | PCC – E-UTRA |  |  |  | SCG -NR |  |  |  |  |
|  | Band | Range | NRB |  | Band |  | Range | NRB |  |
|  | UL MOD | DL MOD | CH BW | DLalloc / UL alloc | UL MOD |  | DL MOD | UL/DL Ch BW | DLalloc / UL alloc |
| Test Settings for a DC_XA_nYA Configuration (Inter-band EN-DC) – Note 2 |  |  |  |  |  |  |  |  |  |
| 1 | X | default |  |  | nY |  | default |  |  |
|  | N/A | N/A | 5 MHz | N/A | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | Highest NRB | All RBs / REFSENS_NR |
| NOTE 1: REFSENS_NR refers to the single carrier Uplink RB allocation for reference sensitivity according to table 7.3.2.4.1-3 of TS 38.521-1 [8]. NOTE 2: LTE anchor agnostic configuration NOTE 3: In a FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected. NOTE 4: The test configuration table only applies to EN-DC only capable devices and devices not supporting NR/5GC mode in the tested band. NOTE 5: Only one EN-DC combination per FR1 band is tested for each EN-DC configuration as defined in clause 5.5B. |  |  |  |  |  |  |  |  |  |

<Unchanged Skipped>

Table 7.3B.2.3.4.2.1-6: Test Configuration Table for EN-DC configurations affected by Reference sensitivity exceptions (two bands)

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NR Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1, E-UTRA Test Frequencies as specified in TS 36.508 [11] clause 4.3.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | For test frequencies refer to “Range” columns. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test EN-DC channel bandwidth as specified in TS 36.508 [11] clause 4.3.1 and TS 38.508-1 [6] clause 4.3.1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Refer to "NR NRB"and "E-UTRA NRB " columns |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NR Test SCS as specified in Table 5.3.5-1 in TS 38.521-1 [8] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Lowest supported SCS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Network signalling value |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | NS_01 by default Unless given by Table 7.3.3-3 in TS 36.521-1 [10] for the E-UTRA band and Table 7.3.2.3-4 in TS 38.521-1 [8] for the NR band. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Parameters for DC Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID | PCC – E-UTRA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SCG -NR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Band |  |  |  |  | Range |  |  |  |  |  | NRB |  |  |  |  |  |  |  |  |  | Band |  |  |  |  |  |  | Range |  |  |  |  | NRB |  |  |  |  |  |
|  | UL MOD |  |  |  |  | DL MOD |  |  |  |  |  | CH BW |  |  |  |  |  |  | DLalloc / UL alloc |  |  | UL MOD |  |  |  |  |  |  | DL MOD |  |  |  |  | UL/DL Ch BW | DLalloc / UL alloc |  |  |  |  |
| Test Settings for a DC_1A_n3A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 12,11 | 1 |  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n3 |  |  |  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_3 |  |  | N/A |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 40 MHz | All RBs / 0 |  |  |  |  |
| 24,10 | 1 |  |  |  |  | UL 1950 / DL 2140 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n3 |  |  |  |  |  |  | UL 1760 / DL 1855 |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |  |
| 36 | 1 |  |  |  |  | Low |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n3 |  |  |  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_3 |  |  | N/A |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 40 MHz | All RBs / 0 |  |  |  |  |
| Test Settings for a DC_1A_n8A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 1 |  |  |  |  | UL 1965 / DL 2155 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n8 |  |  |  |  |  |  | UL 887.5 / DL 932.5 |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |  |
| Test Settings for a DC_1A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 1 |  |  |  |  | UL 1950/ DL 2140 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  |  | UL/DL 3900 |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  | N/A |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 100 MHz | All RBs / 0 |  |  |  |  |  |
| 23 | 1 |  |  |  |  | UL 1950/ DL 2140 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  |  | UL/DL 3870 |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  | N/A |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 20 MHz | All RBs / 0 |  |  |  |  |  |
| 34 | 1 |  |  |  |  | UL 1950/ DL 2140 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  |  | UL/DL 4090.005 |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  | N/A |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |  |  |
| 44 | 1 |  |  |  |  | UL 1950/ DL 2140 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  |  | UL/DL 3709.005 |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  | N/A |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |  |
| Test Settings for a DC_1A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 1 |  |  |  |  | UL 1950 / DL 2140 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  |  | UL/DL 3709.995 |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |  |
| Test Settings for a DC_2A_n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 | 2 |  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n41 |  |  |  |  |  |  | Low |  |  |  |  |  |  |  |  |  |  |  |
|  | N/A |  |  |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  |  |  |  | All RBs / REFSENS_LTE |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 100 MHz | All RBs / REFSENS_ENDC_3 |  |  |  |  |
| Test Settings for a DC_2A_n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 |  | 2 |  |  |  | UL 1855 / DL 1935 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n66 |  |  |  |  |  |  | DL 2175 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | QPSK |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / REFSENS_ENDC_4 |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |  |  |
| 24 |  | 2 |  |  |  | UL 1883.3 /DL1963.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n66 |  |  |  |  |  |  | DL 2150 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | QPSK |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / REFSENS_ENDC_4 |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |  |  |
| Test Settings for a DC_2A_n71A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 2 |  |  |  |  | DL 1980 MHZ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n71 |  |  |  |  |  |  | UL 665.5 MHz |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  |  |  |  | All RBs / 0 |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 5 MHz | All RBs / REFSENS_ENDC_1 |  |  |  |  |  |
| 23 | 2 |  |  |  |  | DL1980 MHZ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n71 |  |  |  |  |  |  | UL 673 MHz |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  |  |  |  | All RBs / 0 |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 10 MHz | All RBs / REFSENS_ENDC_1 |  |  |  |  |  |
| 35 | 2 |  |  |  |  | UL 1881 MHZ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n71 |  |  |  |  |  |  | Low |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_2 |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 20 MHz | All RBs / 0 |  |  |  |  |  |
| Test Settings for a DC_2A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 2 |  |  |  |  | UL 1860 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  |  | UL/DL 3720 |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  | N/A |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 100 MHz | All RBs / 0 |  |  |  |  |  |
| 23 | 2 |  |  |  |  | UL 1860 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  |  | UL/DL 3690.005 |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  | N/A |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 20 MHz | All RBs / 0 |  |  |  |  |  |
| 3 8 | 2 |  |  |  |  | UL 1860 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  |  | UL/DL 3850.005 |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  | N/A |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 10 MHz | All RBs / 0 |  |  |  |  |  |
| 45 | 2 |  |  |  |  | DL Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  |  | UL/DL 3920.01 |  |  |  |  |  |  |  |  |  |  |  |
|  | N/A |  |  |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  |  |  |  | All RBs / 0 |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |  |  |  |  |
| 4a 9 | 2 |  |  |  |  | DL Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  |  | UL/DL 3999.99 |  |  |  |  |  |  |  |  |  |  |  |
|  | N/A |  |  |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  |  |  |  | All RBs / 0 |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |  |  |  |  |
| 54 | 2 |  |  |  |  | UL 1855/DL 1935 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  |  | UL/DL 3790.005 |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |  |  |
| 64 | 2 |  |  |  |  | UL 1900/DL 1980 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  |  | UL/DL 3720 |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |  |  |
| 74 | 2 |  |  |  |  | UL 1885/DL 1965 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  |  | UL/DL 3810 |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |  |  |
| Test Settings for a DC_2A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 2 |  |  |  |  | UL 1870 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  |  | UL/DL 3740.01 |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  | N/A |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 100 MHz | All RBs / 0 |  |  |  |  |  |
| 23 | 2 |  |  |  |  | UL 1885 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  |  | UL/DL 3739.995 |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  | N/A |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 20 MHz | All RBs / 0 |  |  |  |  |  |
| 34 | 2 |  |  |  |  | UL 1855/ DL 1935 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  |  | UL/DL 3790.005 |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |  |  |
| 44 | 2 |  |  |  |  | UL 1885/ DL 1965 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  |  | UL/DL 3690 |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |  |  |
| Test Settings for a DC_3A_n1A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1,11 | 3 |  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n1 |  |  |  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |
|  | N/A |  |  |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  |  |  |  | All RBs / 0 |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 50 MHz | All RBs / REFSENS_ENDC_3 |  |  |  |  |
| 26 | 3 |  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n1 |  |  |  |  |  |  | Low |  |  |  |  |  |  |  |  |  |  |
|  | N/A |  |  |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  |  |  |  | All RBs / 0 |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 50 MHz | All RBs / REFSENS_ENDC_3 |  |  |  |  |
| 34 | 3 |  |  |  |  | UL 1760 / DL 1855 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n1 |  |  |  |  |  |  | UL 1950 / DL 2140 |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |  |
| Test Settings for a DC_3A_n5A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 3 |  |  |  |  | UL 1771 / DL 1866 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n5 |  |  |  |  |  |  | UL 838 / DL 883 |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  |  |  | 10 MHz |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |  |
| 24 | 3 |  |  |  |  | UL 1721 / DL 1816 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n5 |  |  |  |  |  |  | UL 838 / DL 883 |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  |  |  | 10 MHz |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |  |
| Test Settings for a DC_3A_n7A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 3 |  |  |  |  | UL 1730 / DL 1825 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n7 |  |  |  |  |  |  | UL 2535 / DL 2655 |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  |  |  | 5 MHz |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |  |
| Test Settings for a DC_3A_n8A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 3 |  |  |  |  | UL 1755 / DL 1850 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n8 |  |  |  |  |  |  | UL 900 / DL 945 |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  |  |  | 10 MHz |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |  |
| 24 | 3 |  |  |  |  | UL 1747.5 / DL 1842.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n8 |  |  |  |  |  |  | UL 897.5 / DL 942.5 |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  |  |  | 10 MHz |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |  |
| Test Settings for a DC_3A_n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 | 3 |  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n41 |  |  |  |  |  |  | Low |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_3 |  |  | N/A |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 100 MHz | All RBs / 0 |  |  |  |  |
| 26 | 3 |  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n41 |  |  |  |  |  |  | Low |  |  |  |  |  |  |  |  |  |  |
|  | N/A |  |  |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  |  |  |  | All RBs / 0 |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 50 MHz | All RBs / REFSENS_ENDC_3 |  |  |  |  |
| 34, 10 | 3 |  |  |  |  | UL 1740 / DL 1835 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n41 |  |  |  |  |  |  | UL/DL 2657.5 |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |  |
| Test Settings for DC_3A_n77A and DC_3A_n78A Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 3 |  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  |  | UL/DL 3495 |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  | N/A |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 100 MHz | All RBs / 0 |  |  |  |  |
| 23 | 3 |  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  |  | UL/DL 3525 |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  | N/A |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 20 MHz | All RBs / 0 |  |  |  |  |
| 35 | 3 |  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  |  | UL/DL 3685.005 |  |  |  |  |  |  |  |  |  |  |
|  | N/A |  |  |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  |  |  |  | All RBs / 0 |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 20 MHz | All RBs / REFSENS_ENDC_2 |  |  |  |  |
| 39 | 3 |  |  |  |  | Low |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  |  |  |  | All RBs/0 |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 20 MHz | All RBs / REFSENS_ENDC_2 |  |  |  |  |
| 44 | 3 |  |  |  |  | UL 1740 / DL 1835 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  |  | UL/DL 3574.995 |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |  |
| 54 | 3 |  |  |  |  | UL 1765 / DL 1860 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  |  | UL/DL 3435 |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |  |
| Test Settings for a DC_5A_n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 5 |  |  |  |  | UL 838/ DL 883 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n66 |  |  |  |  |  |  | UL 1721/ DL 2121 |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | N/A |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  | N/A |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |  |  |
| Test Settings for a DC_5A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 5 |  |  |  |  | UL 837.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  |  | UL/DL 3350.01 |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 10 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  | N/A |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 100 MHz | All RBs / 0 |  |  |  |  |  |
| 23 | 5 |  |  |  |  | UL 829 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  |  | UL/DL 4145.01 |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 10 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  | N/A |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 100 MHz | All RBs / 0 |  |  |  |  |  |
| 38 | 5 |  |  |  |  | UL Mid (836.5 MHz) 840 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  |  | UL/DL Mid |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 10 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  | N/A |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 100 MHz | All RBs / 0 |  |  |  |  |  |
| 44, 13 | 5 |  |  |  |  | UL 844/DL 889 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  |  | UL/DL 3421.005 |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |  |  |
| 54, 13 | 5 |  |  |  |  | UL 826.5/DL 871.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  |  | UL/DL 4177.5 |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |  |  |
| Test Settings for DC_5A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 5 |  |  |  |  | UL 840 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  |  | UL/DL 3360 |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 10 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  | N/A |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 100 MHz | All RBs / 0 |  |  |  |  |  |
| 28 | 5 |  |  |  |  | UL 840 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  |  | UL/DL 3560.01 |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 10 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  | N/A |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 100 MHz | All RBs / 0 |  |  |  |  |  |
| 34 | 5 |  |  |  |  | UL 844/DL 889 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  |  | UL/DL 3421.005 |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |  |  |
| Test Settings for a DC_7A_n3A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 7 |  |  |  |  | DL 2655 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n3 |  |  |  |  |  |  | DL 1825 |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | N/A |  |  |  |  |  | 10 MHz |  |  |  |  |  |  | N/A |  |  | N/A |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |  |
| Test Settings for a DC_7A_n5A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 7 |  |  |  |  | DL 2667 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n5 |  |  |  |  |  |  | DL 879 |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | N/A |  |  |  |  |  | 10 MHz |  |  |  |  |  |  | N/A |  |  | N/A |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |  |
| Test Settings for a DC_7A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 | 7 |  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  |  | Low |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_3 |  |  | N/A |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 100 MHz | All RBs / 0 |  |  |  |  |
| Test Settings for a DC_8A_n1A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 8 |  |  |  |  | DL 932.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n1 |  |  |  |  |  |  | DL 2155 |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | N/A |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | N/A |  |  | N/A |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |  |
| Test Settings for a DC_8A_n3A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 8 |  |  |  |  | DL 945 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n3 |  |  |  |  |  |  | DL 1850 |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | N/A |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | N/A |  |  | N/A |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |  |
| 24 | 8 |  |  |  |  | DL 942.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n3 |  |  |  |  |  |  | DL 1842.5 |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | N/A |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | N/A |  |  | N/A |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |  |
| Test Settings for a DC_8A_n20A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 8 |  |  |  |  | UL 890.5 / DL 935.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n20 |  |  |  |  |  |  | UL 849.5 / DL 808.5 |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |  |
| 24 | 8 |  |  |  |  | UL 892.5 / DL 937.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n20 |  |  |  |  |  |  | UL 847.5 / DL 806.5 |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |  |
| Test Settings for DC_8A_n77A and DC_8A_n78A Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 8 |  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  |  | UL/DL 3590.01 |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 10 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  | N/A |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 100 MHz | All RBs / 0 |  |  |  |  |
| 2,8 | 8 |  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  |  | UL/DL 3520.005 |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 10 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  | N/A |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 100 MHz | All RBs / 0 |  |  |  |  |
| 34 | 8 |  |  |  |  | UL 897.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  |  | UL/DL 3634.995 |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |  |
| Test Settings for DC_8A_n79A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 8 |  |  |  |  | UL 900 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n79 |  |  |  |  |  |  | UL/DL 4500 |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 10 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  | N/A |  |  |  |  |  |  | QPSK |  |  |  |  | 100 MHz | All RBs / 0 |  |  |  |  |  |
| 2,8 | 8 |  |  |  |  | UL 900 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n79 |  |  |  |  |  |  | UL/DL 4299.99 |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 10 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  | N/A |  |  |  |  |  |  | QPSK |  |  |  |  | 100 MHz | All RBs / 0 |  |  |  |  |  |
| 34 | 8 |  |  |  |  | UL 897.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n79 |  |  |  |  |  |  | UL/DL 4532.505 |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 40 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |  |
| Test Settings for a DC_11A_n79A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | 11 |  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n79 |  |  |  |  |  |  | UL/DL 4457.7 |  |  |  |  |  |  |  |  |  |  |
|  | N/A |  |  |  |  | QPSK |  |  |  |  |  | 10 MHz |  |  |  |  |  |  | All RBs / 0 |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |  |  |  |
| 2,9 | 11 |  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n79 |  |  |  |  |  |  | UL/DL 4512.69 |  |  |  |  |  |  |  |  |  |  |
|  | N/A |  |  |  |  | QPSK |  |  |  |  |  | 10 MHz |  |  |  |  |  |  | All RBs / 0 |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |  |  |  |
| Test Settings for a DC_12A_n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 12 |  |  |  |  | UL 710 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n66 |  |  |  |  |  |  | UL 1730 /DL 2130 |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 10 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  | N/A |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 40 MHz | All RBs / 0 |  |  |  |  |  |
| 28 | 12 |  |  |  |  | UL 710 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n66 |  |  |  |  |  |  | UL 1770 /DL 2170 |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 10 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  | N/A |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 40 MHz | All RBs / 0 |  |  |  |  |  |
| Test Settings for a DC_13A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 13 |  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  |  | UL/DL 3909.99 |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 10 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  | N/A |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 100 MHz | All RBs / 0 |  |  |  |  |  |
| 28 | 13 |  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  |  | UL/DL Mid |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 10 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  | N/A |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 100 MHz | All RBs / 0 |  |  |  |  |  |
| 35 | 13 |  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  |  | UL/DL 3909.99 |  |  |  |  |  |  |  |  |  |  |  |
|  | N/A |  |  |  |  | QPSK |  |  |  |  |  | 10 MHz |  |  |  |  |  |  | All RBs / 0 |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |  |  |  |  |
| 49 | 13 |  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  |  | UL/DL 3834 |  |  |  |  |  |  |  |  |  |  |  |
|  | N/A |  |  |  |  | QPSK |  |  |  |  |  | 10 MHz |  |  |  |  |  |  | All RBs / 0 |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |  |  |  |  |
| 54 | 13 |  |  |  |  | UL 784.5/DL 753.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  |  | UL/DL 3891.495 |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |  |  |
| Test Settings for a DC_19A_n79A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | 19 |  |  |  |  | DL 884 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n79 |  |  |  |  |  |  | Low |  |  |  |  |  |  |  |  |  |  |
|  | N/A |  |  |  |  | QPSK |  |  |  |  |  | 10 MHz |  |  |  |  |  |  | All RBs / 0 |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 40 MHz | All RBs / REFSENS_ENDC_2 |  |  |  |  |
| 2,9 | 19 |  |  |  |  | DL 884 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n79 |  |  |  |  |  |  | UL/DL 4445.025 |  |  |  |  |  |  |  |  |  |  |
|  | N/A |  |  |  |  | QPSK |  |  |  |  |  | 10 MHz |  |  |  |  |  |  | All RBs / 0 |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 40 MHz | All RBs / REFSENS_ENDC_2 |  |  |  |  |
| Test Settings for a DC_20A_n3A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 20 |  |  |  |  | UL 840 / DL 799 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n3 |  |  |  |  |  |  | UL 1775 / DL 1870 |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |  |
| 24 | 20 |  |  |  |  | UL 847 / DL 806 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n3 |  |  |  |  |  |  | UL 1735 / DL 1830 |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |  |
| Test Settings for a DC_20A_n7A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 20 |  |  |  |  | UL 851 / DL 810 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n7 |  |  |  |  |  |  | UL 2512 / DL 2632 |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |  |
| Test Settings for a DC_20A_n8A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 20 |  |  |  |  | UL 849.5 / DL 808.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n8 |  |  |  |  |  |  | UL 892.5 / DL 937.5 |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |  |
| Test Settings for a DC_20A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1,8 | 20 |  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  | N/A |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 100 MHz | All RBs / 0 |  |  |  |  |
| 23 | 20 |  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  |  | UL/DL 3387.99 |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  | N/A |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 100 MHz | All RBs / 0 |  |  |  |  |
| 34 | 20 |  |  |  |  | UL 850 / DL 809 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  |  | UL/DL 3358.995 |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |  |
| Test Settings for a DC_21A_n79A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | 21 |  |  |  |  | DL 1506 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n79 |  |  |  |  |  |  | UL/DL 4518 |  |  |  |  |  |  |  |  |  |  |  |
|  | N/A |  |  |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  |  |  |  | All RBs / 0 |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | QPSK |  |  |  |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |  |  |  |  |
| 29 | 21 |  |  |  |  | DL 1506 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n79 |  |  |  |  |  |  | UL/DL 4800 |  |  |  |  |  |  |  |  |  |  |  |
|  | N/A |  |  |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  |  |  |  | All RBs / 0 |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | QPSK |  |  |  |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |  |  |  |  |
| 34 | 21 |  |  |  |  | UL 1457.5/ DL 1505.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n79 |  |  |  |  |  |  | UL/DL 4420.5 |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 40 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |  |
| Test Settings for a DC_26A_n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 26 |  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n41 |  |  | UL/DL 2524.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  | 15 MHz |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  |  |  | N/A |  |  | CP-OFDM QPSK |  |  |  |  | 50 MHz |  |  |  |  |  | All RBs / 0 |  |  |
| 2,8 | 26 |  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n41 |  |  | UL/DL 2572 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  | 15 MHz |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  | 50 MHz |  |  |  |  |  | All RBs / 0 |  |  |
| 34 | 26 |  |  |  |  | UL 839 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n41 |  |  | UL/DL 2562 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  | 5 MHz |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  |  |  | CP-OFDM QPSK |  |  | CP-OFDM QPSK |  |  |  |  | 10 MHz |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |
| 45 | 26 |  |  |  |  | DL 886.6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n41 |  |  | UL/DL 2660 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  | 15 MHz |  |  |  |  |  | All RBs / REFSENS_LTE |  |  |  |  |  | CP-OFDM QPSK |  |  | CP-OFDM QPSK |  |  |  |  | 100 MHz |  |  |  |  |  | All RBs / REFSENS_ENDC_2 |  |  |
| Test Settings for a DC_26A_n77A/DC_26A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 26 |  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77/n78 |  |  | UL/DL 3366 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  | 15 MHz |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  |  |  | N/A |  |  | CP-OFDM QPSK |  |  |  |  | 100 MHz |  |  |  |  |  | All RBs / 0 |  |  |
| 2,8 | 26 |  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77/n78 |  |  | UL/DL 3446.01 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  | 15 MHz |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  |  |  | N/A |  |  | CP-OFDM QPSK |  |  |  |  | 100 MHz |  |  |  |  |  | All RBs / 0 |  |  |
| 34 | 26 |  |  |  |  | UL 836.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77/n78 |  |  | UL/DL 3391.005 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  | 5 MHz |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  |  |  | CP-OFDM QPSK |  |  | CP-OFDM QPSK |  |  |  |  | 10 MHz |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_26A_n79A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | 26 |  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n79 |  |  | UL/DL 4432.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | N/A |  |  |  |  | QPSK |  |  |  |  | 15 MHz |  |  |  |  |  | All RBs / 0 |  |  |  |  |  | CP-OFDM QPSK |  |  | CP-OFDM QPSK |  |  |  |  | 60 MHz |  |  |  |  |  | All RBs / REFSENS_ENDC_2 |  |  |
| 22,9 | 26 |  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n79 |  |  | UL/DL 4470.51 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | N/A |  |  |  |  | QPSK |  |  |  |  | 15 MHz |  |  |  |  |  | All RBs / 0 |  |  |  |  |  | CP-OFDM QPSK |  |  | CP-OFDM QPSK |  |  |  |  | 60 MHz |  |  |  |  |  | All RBs / REFSENS_ENDC_2 |  |  |
| Test Settings for a DC_28A_n5A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 | 28 |  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n5 |  |  | Low |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  | 20MHz |  |  |  |  |  | All RBs / REFSENS_ENDC_3 |  |  |  |  |  | N/A |  |  | CP-OFDM QPSK |  |  |  |  | 20 MHz |  |  |  |  |  | All RBs / 0 |  |  |
| Test Settings for a DC_28A_n77A/DC_28A_n78A Configuration (Test ID 4 only apply to DC_28A_n77A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 |  |  | 28 |  |  | Low |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77/n78 |  |  |  | UL/DL 3540 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | QPSK |  |  | QPSK |  |  |  | 10 MHz |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  |  | N/A |  |  |  | CP-OFDM QPSK |  |  |  |  | 100 MHz |  |  |  |  |  | All RBs / 0 |  |  |  |  |
| 28 |  |  | 28 |  |  | Low |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77/n78 |  |  |  | Low |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | QPSK |  |  | QPSK |  |  |  | 10 MHz |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  |  | N/A |  |  |  | CP-OFDM QPSK |  |  |  |  | 100 MHz |  |  |  |  |  | All RBs / 0 |  |  |  |  |
| 34 |  |  | 28 |  |  | UL 705.5 / DL 760.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77/n78 |  |  |  | UL/DL 3582.495 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | QPSK |  |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  |  | CP-OFDM QPSK |  |  |  | CP-OFDM QPSK |  |  |  |  | 10 MHz |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  |  |
| 45 |  |  | 28 |  |  | Low |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  | UL/DL 3815.01 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | N/A |  |  | QPSK |  |  |  | 10 MHz |  |  |  |  |  | All RBs / 0 |  |  |  |  | CP-OFDM QPSK |  |  |  | CP-OFDM QPSK |  |  |  |  | 100 MHz |  |  |  |  |  | All RBs / REFSENS_ENDC_2 |  |  |  |  |
| Test Settings for a DC_30A_n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 | 30 |  |  |  |  | Low |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n66 |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | N/A |  |  |  |  | QPSK |  |  |  |  | 10 MHz |  |  |  |  |  | All RBs / 0 |  |  |  |  |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  |  |  | 40 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_3 |  |  |
| Test Settings for a DC_40A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | 40 |  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  | UL/DL 3525 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | N/A |  |  |  |  | QPSK |  |  |  |  | 20 MHz |  |  |  |  |  | All RBs / 0 |  |  |  |  |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  |  |  | 20 MHz |  |  |  |  |  | All RBs / REFSENS_ENDC_2 |  |  |
| 2,9 | 40 |  |  |  |  | Low |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  | 20 MHz |  |  |  |  |  | All RBs / REFSENS_ENDC_2 |  |  |  |  |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  |  |  | 100 MHz |  |  |  |  |  | All RBs / REFSENS_ENDC_2 |  |  |
| Test Settings for a DC_41A_n77A/DC_41A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | 41 |  |  |  |  | Low |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77/n78 |  |  | UL/DL 3750 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | N/A |  |  |  |  | QPSK |  |  |  |  | 20 MHz |  |  |  |  |  | All RBs / 0 |  |  |  |  |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  |  |  | 100 MHz |  |  |  |  |  | All RBs / REFSENS_ENDC_2 |  |  |
| 26 | 41 |  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77/n78 |  |  | Low |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | N/A |  |  |  |  | QPSK |  |  |  |  | 20 MHz |  |  |  |  |  | All RBs / 0 |  |  |  |  |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  |  |  | 100 MHz |  |  |  |  |  | All RBs / REFSENS_ENDC_3 |  |  |
| 36 | 41 |  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77/n78 |  |  | Low |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  | 20MHz |  |  |  |  |  | All RBs / REFSENS_ENDC_3 |  |  |  |  |  | N/A |  |  | CP-OFDM QPSK |  |  |  |  | 100 MHz |  |  |  |  |  | All RBs / 0 |  |  |
| Test Settings for a DC_48A_n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 |  |  |  | 48 |  |  |  |  | UL/DL 3555 |  |  |  |  |  |  |  |  |  |  | n66 |  |  |  |  |  |  |  | UL 1760 MHZ |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | N/A |  |  |  |  | QPSK |  |  |  | 10 MHz |  |  |  | All RBs / 0 |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 40 MHz |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  |
| 23 |  |  |  | 48 |  |  |  |  | UL/DL 3565 |  |  |  |  |  |  |  |  |  |  | n66 |  |  |  |  |  | UL 1770 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | N/A |  |  |  |  | QPSK |  |  |  | 10 MHz |  |  |  | All RBs / 0 |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 40 MHz |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  |
| 38 |  |  |  | 48 |  |  |  |  | UL/DL 3600 |  |  |  |  |  |  |  |  |  |  | n66 |  |  |  |  |  | UL1760 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | N/A |  |  |  |  | QPSK |  |  |  | 10 MHz |  |  |  | All RBs / 0 |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 40 MHz |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  |
| 44 |  |  |  | 48 |  |  |  |  | UL/DL 3630 |  |  |  |  |  |  |  |  |  |  | n66 |  |  |  |  |  | UL 1715 / DL 2115 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | QPSK |  |  |  |  | QPSK |  |  |  | 20 MHz |  |  |  | All RBs / REFSENS_ENDC_4 |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 5 MHz |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  |
| Test Settings for a DC_66A_n2A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14, 10 |  |  |  | 66 |  |  |  |  | UL1775/ DL2175 |  |  |  |  |  |  |  |  |  |  | n2 |  |  |  |  |  | UL 1855 / DL 1935 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | QPSK |  |  |  |  | QPSK |  |  |  | 5 MHz |  |  |  | All RBs / REFSENS_ENDC_4 |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 5 MHz |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  |
| 24, 10 |  |  |  | 66 |  |  |  |  | UL 1750/ DL 2150 |  |  |  |  |  |  |  |  |  |  | n2 |  |  |  |  |  | UL 1883.3 / DL 1963.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | QPSK |  |  |  |  | QPSK |  |  |  | 5 MHz |  |  |  | All RBs / REFSENS_ENDC_4 |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 5 MHz |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  |
| Test Settings for a DC_66A_n5A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 |  |  |  | 66 |  |  |  |  | UL 1721/ DL 2121 |  |  |  |  |  |  |  |  |  |  | n5 |  |  |  |  |  | UL 838/ DL 883 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | QPSK |  |  |  |  | QPSK |  |  |  | 5 MHz |  |  |  | All RBs / REFSENS_ENDC_4 |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 5 MHz |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  |
| Test Settings for a DC_66A_n71A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 |  |  |  | 66 |  |  | UL 1750/ DL 2150 |  |  |  |  |  |  |  |  |  |  |  |  | n71 |  |  |  |  |  | UL 675/ DL 629 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | QPSK |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / REFSENS_ENDC_4 |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 5 MHz |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  |
| Test Settings for a DC_66A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 |  |  |  |  | 66 |  |  | UL 1720 |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  |  | UL/DL 3440.01 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | QPSK |  |  | QPSK |  |  |  |  |  | 20 |  |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 100 MHz |  |  |  |  |  | All RBs / 0 |  |  |
| 23 |  |  |  |  | 66 |  |  | UL 1720 |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  |  | UL/DL 3410 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | QPSK |  |  | QPSK |  |  |  |  |  | 20 |  |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 20 MHz |  |  |  |  |  | All RBs / 0 |  |  |
| 38 |  |  |  |  | 66 |  |  | UL Mid |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  |  | UL/DL Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | QPSK |  |  | QPSK |  |  |  |  |  | 20 |  |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 100 MHz |  |  |  |  |  | All RBs / 0 |  |  |
| 44 |  |  |  |  | 66 |  |  | UL 1775/ DL 2175 |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  |  | UL/DL 3949.995 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | QPSK |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 10 MHz |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |
| 54 |  |  |  |  | 66 |  |  | UL 1760 / DL 2160 |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  |  | UL/DL 3720 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | QPSK |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 10 MHz |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_66A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 |  |  |  |  | 66 |  |  | UL 1755 |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  |  | UL/DL 3510 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | QPSK |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 100 MHz |  |  |  |  |  | All RBs / 0 |  |  |
| 23 |  |  |  |  | 66 |  |  | UL 1755 |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  |  | UL/DL 3480 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | QPSK |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 20 MHz |  |  |  |  |  | All RBs / 0 |  |  |
| 34 |  |  |  |  | 66 |  |  | UL 1730/ DL2130 |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  |  | UL/DL 3660 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | QPSK |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 10 MHz |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |
| NOTE 1: REFSENS_LTE refers to the single carrier Uplink RB allocation for reference sensitivity according to table 7.3.3-2 of TS 36.521-1 [10]. REFSENS_NR refers to the single carrier Uplink RB allocation for reference sensitivity according to table 7.3.2.4.1-3 of TS 38.521-1 [8]. REFSENS_ENDC_1 refers to the Uplink RB allocation for reference sensitivity exceptions due to UL harmonic interference according to table 7.3B.2.0.3.1-2. REFSENS_ENDC_2 refers to the Uplink RB allocation for reference sensitivity exceptions due to receiver harmonic mixing according to table 7.3B.2.0.3.2-2. REFSENS_ENDC_3 refers to the Uplink RB allocation for reference sensitivity exceptions due to cross band isolation according to table 7.3B.2.0.3.4-2. REFSENS_ENDC_4 refers to the Uplink RB allocation for reference sensitivity exceptions due to dual uplink operation for ENDC according to table 7.3B.2.0.3.5.1-1 for PC3 and table 7.3B.2.0.3.5.1-1a for PC2. NOTE 2: Void NOTE 3: Test ID with UL harmonic exception NOTE 4: Test ID with 2UL intermodulation exception NOTE 5: Test ID with UL receiver harmonic mixing NOTE 6: Test ID with UL cross band isolation NOTE 7: Void NOTE 8: Test ID with UL harmonic exception avoided NOTE 9: Test ID with UL receiving harmonic mixing exception avoided NOTE 10: Only applicable to UEs not supporting UE capability singleUL-Transmission. NOTE 11: Test ID with Cross band isolation exception avoided, which is only applicable to DC_1_n3, DC_3_n1 and DC_3_n84. NOTE 12: In an E-UTRA band or FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected. NOTE 13: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 from TS 38.101-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. NOTE 14: If the NR frequency does not match to a valid NR-ARFCN, apply the closest NR frequency with a valid NR-ARFCN. NOTE X: The test points are executed only when UEs support simultaneous Rx/Tx capability as indicated in Table A.4.3.2B.2.3.1-2 of TS 38.508-2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, Figure A.3.1.1.1 for TE diagram and clause A.3.2 for UE diagram.

2.	The parameter settings for NR cell are set up according to TS 38.508-1 [6] clause 4.4.3.

3.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

4.	NR downlink signals are initially set up according to Annex C.0, C.1, C.2, C.3.1, and uplink signals according to Annex G.0, G.1, G.2, and G.3.1 of TS 38.521-1 [8].

5.	E-UTRA downlink signals are initially set up according to Annex C.0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0 of TS 36.521-1 [10].

6.	The DL and UL Reference Measurement channels are set according to Tables 7.3B.2.3.4.2.1-0 to 7.3B.2.3.4.2.1-6 for E-UTRA CG and NR CG.

7.	NR propagation conditions are set according to Annex B.0 of TS 38.521-1 [8]. E-UTRA propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

8.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause7.3B.2.3.4.2.3.

9.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6. Step 9 only applicable to the test configuration in Table 7.3B.2.3.4.2.1-0.

<End of Changes>