3GPP TSG-RAN5 Meeting #98	R5-231041r2

# Athens, Greece, 27th Feb 2023 - 3rd Mar 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5197 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of IoT NTN TC 22.5.23 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN_plus_EPS-UEConTest |  |  |  |  |  | Date: |  |  | 2023-02-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - The conformance requirements need to be updated according to latest TS 24.301 and section 5.5.1.2.6 is not used in this case. - Cell 55 needs to be updated according to the SI combination. - UE postion needs to be pre-configured in the preamble. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Conformance requirements were updated. - Cell 55 replaced by Cell 62. - UE postion was pre-configured in the preamble. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Outdated confrmance requirement and incorrect cell configured. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.5.23 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | r1: description of pre-configured UE location in pre-test condition was updated. To avoid impact to other cases, steps 7-9 were added to remove the “list of "PLMN not allowed to operate at the present UE location". r2: reword step 9 and add a Note to mention the way to achive this. SIB31-NB for Ncell62 was added. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

[Start of change]

22.5.23	NB-IoT / NTN / GNSS position reporting / reject cause #78 "PLMN not allowed to operate at the present UE location"

22.5.23.1	Test Purpose (TP)

(1)

with { UE has sent an ATTACH REQUEST message }

ensure that {

when { UE receives an ATTACH REJECT message with the reject cause set to "PLMN not allowed to operate at the present UE location" }

then { UE sets the EPS update status to EU3 ROAMING NOT ALLOWED, UE deletes the GUTI, last visited registered TAI, TAI list and eKSI, UE resets the registration attempt counter, UE enters the state EMM-DEREGISTERED.PLMN-SEARCH and perform a PLMN selection }

}

22.5.23.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.301 clauses 4.11.2 and 5.5.1.2.5. Unless otherwise stated these are Rel-17 requirements.

[TS 24.301, clause 4.11.2]

The UE may be rejected with EMM cause #78 in ATTACH REJECT message, TRACKING AREA UPDATE REJECT message or DETACH REQUEST message. The EMM cause #78 is applicable for the UE only when the UE is accessing a PLMN using a satellite E-UTRAN access.

For the satellite E-UTRAN access the UE shall store a list of "PLMN not allowed to operate at the present UE location". Each entry in the list consists of:

a)	PLMN identity of the PLMN which sent a message including EMM cause value #78 "PLMN not allowed to operate at the present UE location" via the satellite E-UTRAN access technology;

b)	geographical location, if known by the UE, where the EMM cause value #78 was received over the satellite E-UTRAN access technology; and

c)	if the geographical location exists, a UE implementation specific distance value.

Before storing a new entry in the list, the UE shall delete any existing entry with the same PLMN identity. Upon storing a new entry, the UE starts a timer instance associated with the entry with an implementation specific value that shall not be set to a value smaller than the timer value indicated by the network in the Lower bound timer value IE, if any. If the Lower bound timer value IE was not provided by the network, the value of the timer shall be set based on the UE implementation

.

The UE is allowed to access a PLMN via the satellite E-UTRAN access technology which is part of the list of "PLMNs not allowed to operate at the present UE location" if:

a)	the current UE location is known, a geographical location is stored for the entry of this PLMN, and the distance from location where EMM cause value #78 was received to the current UE location is smaller than a UE implementation specific value:

b)	the timer associated with the entry of this PLMN has expired; or

c)	the access is for emergency services (see 3GPP TS 23.122 [5] for further details).

NOTE:	When the UE is accessing network for emergency services, it is up to operator and regulatory policies whether the network needs to determine if the UE is in a location where network is not allowed to operate.

The list shall accommodate three or more entries. The maximum number of entries is an implementation decision. When the list is full and a new entry has to be inserted, the oldest entry shall be deleted.

Each entry shall be removed if for the entry:

a)	the UE successfully registers via the satellite E-UTRAN access technology to the PLMN stored in the entry except when the UE is attached for emergency bearer services; or

b)	the timer instance associated with the entry expires.

The UE may remove an entry in the list, if the UE's current location is known, a geographical is stored for the entry of this PLMN, and the distance from location where EMM cause value #78 was received to the current UE location is larger than the UE implementation specific value.

If the UE is in EMM-DEREGISTERED.LIMITED-SERVICE state and an entry from the list of "PLMNs not allowed to operate at the present UE location" is removed, the UE shall perform PLMN selection according to 3GPP TS 23.122 [6]

When the UE is switched off, the UE shall keep the list of "PLMNs not allowed to operate at the present UE location" in its non-volatile memory together with the SUPI from the USIM. The UE shall delete the list of "PLMNs not allowed to operate at the present UE location" if the USIM is removed.

If the UE is switched off when the timer instance associated with the entry in the list is running, the UE shall behave as follows when the UE is switched on and the USIM in the UE remains the same:

let t1 be the time remaining for timer instance associated with the entry in the list to timeout at switch off and let t be the time elapsed between switch off and switch on. If t1 is greater than t, then the timer shall be restarted with the value t1 – t. If t1 is equal to or less than t, then the timer need not be restarted and considered expired. If the UE is not capable of determining t, then the UE shall restart the timer with the value t1.

[TS 24.301, clause 5.5.1.2.5]

If the attach request cannot be accepted by the network, the MME shall send an ATTACH REJECT message to the UE including an appropriate EMM cause value.

…

In NB-S1 mode or WB-S1 mode via satellite E-UTRAN access, if the attach request is from a UE via a satellite E-UTRA cell and the network using the User Location Information provided by the eNodeB (see 3GPP TS 36.413 [23]), is able to determine that the UE is in a location where the network is not allowed to operate, the network shall set the EMM cause value in the ATTACH REJECT message to #78 "PLMN not allowed to operate at the present UE location".

…

Furthermore, the UE shall take the following actions depending on the EMM cause value received in the ATTACH REJECT message.

…

#78	(PLMN not allowed to operate at the present UE location).

This cause value received from a non-satellite E-UTRA cell is considered as an abnormal case and the behaviour of the UE is specified in clause 5.5.1.2.6.

The UE shall set the EPS update status to EU3 ROAMING NOT ALLOWED (and shall store it according to clause 5.1.3.3) and shall delete any GUTI, last visited registered TAI, TAI list and eKSI. Additionally, the UE shall reset the registration attempt counter. The UE shall store the PLMN identity and, if it is known, the current geographical location in the list of "PLMNs not allowed to operate at the present UE location", start a corresponding timer instance (see clause 4.11.2) and shall enter state EMM-DEREGISTERED.PLMN-SEARCH and perform a PLMN selection according to 3GPP TS 23.122 [6].

If the UE is operating in single-registration mode, the UE shall in addition handle the 5GMM parameters 5GMM state, 5GS update status and registration attempt counter as specified in 3GPP TS 24.501 [54] for the case when the initial registration procedure performed over 3GPP access is rejected with the 5GMM cause with the same value.

Other values are considered as abnormal cases. The behaviour of the UE in those cases is specified in clause 5.5.1.2.6.

22.5.23.3	Test description

22.5.23.3.1	Pre-test conditions

System Simulator:

-	Ncell 50 (PLMN1, HPLMN) and Ncell 62 (PLMN2, visited PLMN belongs to TAI-7, MCC=001 MNC=02) are configured according to Table 8.1.4.2-3 and Table 8.1.4.2-5 in TS 36.508 [18].

-	System information combination 9 as defined in TS 36.508 [18] clause 8.1.4.3.1.1 is used for both cells.

UE:

-	The UE is in Automatic PLMN selection mode.

-	The UE is previously registered on Ncell 50 and the UE is last authenticated and registered using default message contents according to TS 36.508 [18].

-	The preconfigured UE location is defined in TS 36.508 [18] Clause 4.xx.

Preamble:

-	The UE is in State 1-NB switched off according to TS 36.508 [18].

22.5.23.3.2	Test procedure sequence

Table 22.5.23.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. The configuration marked "T1" is applied at the point indicated in the Main behaviour description in Table 22.5.23.3.2-2. Cell powers are chosen for a serving cell and a non-suitable “Off” cell as defined in TS 36.508 [18].

Table 22.5.23.3.2-1: Ncell configuration changes over time

|  | Parameter | Unit | Ncell 50 | Ncell 62 | Remarks |
| --- | --- | --- | --- | --- | --- |
| T0 | NRS EPRE | dBm/15kHz | -85 | “Off” | Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1 |
| T1 | NRS EPRE | dBm/15kHz | -85 | -73 |  |

Table 22.5.23.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | The following messages are to be observed on Ncell 50 unless explicitly stated otherwise. | - | - | - | - |
| 1 | The UE is switched on. | - | - | - | - |
| 2 | The UE transmits an ATTACH REQUEST message. | --> | ATTACH REQUEST | - | - |
| 3 | The SS transmits an ATTACH REJECT message including EMM cause = "PLMN not allowed to operate at the present UE location". | <-- | ATTACH REJECT | - | - |
| 4 | The SS releases the RRC connection. | - | - | - | - |
| 5 | SS adjusts Ncell levels according to row T1 of table 22.5.23.3.2-1. | - | - | - | - |
| - | The following messages are to be observed on Ncell 62 unless explicitly stated otherwise. | - | - | - | - |
| 6 | Check: Does the UE transmit an ATTACH REQUEST message? | --> | ATTACH REQUEST | 1 | P |
| 7 | Steps 5 to 14b1 of the registration procedure described in TS 36.508 [18] subclause 8.1.5.2.3 are performed on Ncell 62. | - | - | - | - |
| 8 | The SS transmits an RRCConnectionRelease-NB message to move to RRC_IDLE state. | - | - | - | - |
| 9 | Make the UE delete the list of PLMNs not allowed to operate at the present UE location. (Note 1) | - | - | - | - |
| Note 1: This could be done by an MMI/AT command. |  |  |  |  |  |

22.5.23.3.3	Specific message contents

Table 22.5.23.3.3-1: Message ATTACH REJECT (step 3, Table 22.5.23.3.2-2)

| Derivation path: TS 36.508 [18], Table 8.1.7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EMM cause | 01001110 | #78 "PLMN not allowed to operate at the present UE location" |  |
| ESM message container | Not present |  |  |

Table 22.5.23.3.3-2: Message ATTACH REQUEST (step 6, Table 22.5.23.3.2-2)

| Derivation path: TS 36.508 [18], Table 8.1.7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Old GUTI or IMSI | IMSI-1 | GUTI has been deleted after receiving ATTACH REJECT at step 3; only IMSI is available. |  |
| Last visited registered TAI | Not present | TAI has been deleted after receiving ATTACH REJECT at step 3. |  |

Table 22.5.23.3.3-3: SystemInformationBlockType31-NB on Ncell 62 (preamble and all steps, Table Table 22.5.23.3.2-2)

| Derivation Path: TS 36.508 [18], Table 8.1.4.3.3-10 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType31-r17 ::= SEQUENCE { |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |
| orbitalParameters SEQUENCE { |  |  |  |
| semiMajorAxis-r17 | FFS |  |  |
| eccentricity-r17 | FFS |  |  |
| periapsis-r17 | FFS |  |  |
| longitude-r17 | FFS |  |  |
| inclination-r17 | FFS |  |  |
| anomaly-r17 | FFS |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

[End of change]