3GPP TSG-RAN5 Meeting #98	R5-231110r1

Athens, February 27 - March 3 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.522 | CR | 0259 | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Additional information note correction for RRM test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-02-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Note 1 which is used to mark incomplete test cases is not reflecting the complete status of the RRM test cases in TS 38.533. Ambiguity regarding the test case coverage exists (i.e. test cases which are listed in 38.133 with the E-UTRA/FR1 – FR2 carriers testability issue). |  |  |  |  |  |  |  |  |
| Summary of change: |  | To properly reflect the current test case status in TS 38.533 following changes are proposed: For the issue of mix of E-UTRA/FR1 – FR2 carriers testability issue, new Note was added in tables 4.2-2, 4.2-4 and 4.2-5 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguity regarding test case coverage continues to exist. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | R1: “N/A” in the condition column was used instead of the note to indicate the test cases with FR1/LTE – FR2 testability issue. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

///Start of changes

## 4.2	RRM conformance test cases

Table 4.2-1: Applicability of RRM EN-DC FR1 conformance test cases, ref. TS 38.533 [5]

| Clause |  | TC Title |  | Release |  | Applicability |  |  |  |  | Additional Information | Branch |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Condition |  | Comment |  |  |  |  |
| 4.3 |  | RRC_CONNECTED state mobility |  |  |  |  |  |  |  |  |  |  |
| 4.3.2 |  | RRC connection mobility control |  |  |  |  |  |  |  |  |  |  |
| 4.3.2.2 |  | Random access |  |  |  |  |  |  |  |  |  |  |
| 4.3.2.2.1 |  | EN-DC FR1 contention based random access |  | Rel-15 |  | C021 |  | UEs supporting EN-DC FR1 |  |  |  | 2Rx 4Rx |
| 4.3.2.2.2 |  | EN-DC FR1 non-contention based random access |  | Rel-15 |  | C030 |  | UEs supporting EN-DC FR1 and CSI-RS based PRACH |  |  |  | 2Rx 4Rx |
| 4.3.2.2.3 |  | EN-DC FR1 2-step contention based random access |  | Rel-16 |  | C157 |  | UEs supporting EN-DC FR1 and 2-step RACH |  |  |  | 2Rx 4Rx |
| 4.3.2.2.4 |  | EN-DC FR1 2-step non-contention based random access |  | Rel-16 |  | C158 |  | UEs supporting EN-DC FR1 and 2-step RACH |  |  |  | 2Rx 4Rx |
| 4.4 |  | Timing |  |  |  |  |  |  |  |  |  |  |
| 4.4.1 |  | UE Transmit Timing |  |  |  |  |  |  |  |  |  |  |
| 4.4.1.1 |  | EN-DC FR1 UE transmit timing accuracy |  | Rel-15 |  | C021 |  | UEs supporting EN-DC FR1 |  |  |  | 2Rx 4Rx |
| 4.4.2 |  | UE timer accuracy |  |  |  |  |  |  |  |  |  |  |
| 4.4.3 |  | Timing Advance |  |  |  |  |  |  |  |  |  |  |
| 4.4.3.1 |  | EN-DC FR1 timing advance adjustment accuracy |  | Rel-15 |  | C021 |  | UEs supporting EN-DC FR1 |  |  |  | 2Rx 4Rx |
| 4.5 |  | Signalling characteristics |  |  |  |  |  |  |  |  |  |  |
| 4.5.1 |  | Radio link monitoring |  |  |  |  |  |  |  |  |  |  |
| 4.5.1.1 |  | EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode |  | Rel-15 |  | C021 |  | UEs supporting EN-DC FR1 |  |  |  | 2Rx 4Rx |
| 4.5.1.2 |  | EN-DC FR1 radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode |  | Rel-15 |  | C021 |  | UEs supporting EN-DC FR1 |  |  |  | 2Rx 4Rx |
| 4.5.1.3 |  | EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in DRX mode |  | Rel-15 |  | C021a |  | UEs supporting EN-DC FR1 and long DRX cycle |  |  |  | 2Rx 4Rx |
| 4.5.1.4 |  | EN-DC FR1 radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in DRX mode |  | Rel-15 |  | C021a |  | UEs supporting EN-DC FR1 and long DRX cycle |  |  |  | 2Rx 4Rx |
| 4.5.1.5 |  | EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode |  | Rel-15 |  | C038 |  | UEs supporting EN-DC FR1 and CSI-RS-based RLM |  |  |  | 2Rx 4Rx |
| 4.5.1.6 |  | EN-DC FR1 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode |  | Rel-15 |  | C038 |  | UEs supporting EN-DC FR1 and CSI-RS-based RLM |  |  |  | 2Rx 4Rx |
| 4.5.1.7 |  | EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in DRX mode |  | Rel-15 |  | C038a |  | UEs supporting EN-DC FR1, CSI-RS-based RLM and long DRX cycle |  |  |  | 2Rx 4Rx |
| 4.5.1.8 |  | EN-DC FR1 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM RS in DRX mode |  | Rel-15 |  | C038a |  | UEs supporting EN-DC FR1, CSI-RS-based RLM and long DRX cycle |  |  |  | 2Rx 4Rx |
| 4.5.2 |  | Interruption |  |  |  |  |  |  |  |  |  |  |
| 4.5.2.1 |  | EN-DC FR1 interruptions at transitions between active and non-active during DRX in synchronous EN-DC |  | Rel-15 |  | C021a |  | UEs supporting EN-DC FR1 and long DRX cycle |  |  |  | 2Rx 4Rx |
| 4.5.2.2 |  | EN-DC FR1 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC |  | Rel-15 |  | C021a |  | UEs supporting EN-DC FR1 and long DRX cycle |  |  |  | 2Rx 4Rx |
| 4.5.2.3 |  | EN-DC FR1 interruptions during measurements on deactivated NR SCC in synchronous EN-DC |  | Rel-15 |  | C067 |  | UEs supporting EN-DC FR1 and 2DL CA in NR |  |  |  | 2Rx 4Rx |
| 4.5.2.4 |  | EN-DC FR1 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC |  | Rel-15 |  | C067 |  | UEs supporting EN-DC FR1 and 2DL CA in NR |  |  |  | 2Rx 4Rx |
| 4.5.2.5 |  | EN-DC FR1 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC |  | Rel-15 |  | C068 |  | UEs supporting EN-DC FR1 and 2DL CA in E-UTRA |  |  |  | 2Rx 4Rx |
| 4.5.2.6 |  | EN-DC FR1 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC |  | Rel-15 |  | C068 |  | UEs supporting EN-DC FR1 and 2DL CA in E-UTRA |  |  |  | 2Rx 4Rx |
| 4.5.3 |  | SCell activation and deactivation delay |  |  |  |  |  |  |  |  |  |  |
| 4.5.3.1 |  | EN-DC FR1 SCell activation and deactivation of known SCell in non-DRX for 160ms SCell measurement cycle |  | Rel-15 |  | C067 |  | UEs supporting EN-DC FR1 and 2DL CA in NR |  |  |  | 2Rx 4Rx |
| 4.5.3.2 |  | EN-DC FR1 SCell activation and deactivation of known SCell in non-DRX for 640ms SCell measurement cycle |  | Rel-15 |  | C067 |  | UEs supporting EN-DC FR1 and 2DL CA in NR |  |  |  | 2Rx 4Rx |
| 4.5.3.3 |  | EN-DC FR1 SCell activation and deactivation of unknown SCell in non-DRX |  | Rel-15 |  | C067 |  | UEs supporting EN-DC FR1 and 2DL CA in NR |  |  |  | 2Rx 4Rx |
| 4.5.4 |  | UE UL carrier RRC reconfiguration delay |  |  |  |  |  |  |  |  |  |  |
| 4.5.4.1 |  | EN-DC FR1 UE UL carrier RRC reconfiguration delay |  | Rel-15 |  | C032 |  | UEs supporting EN-DC FR1 and SUL |  |  |  | 2Rx 4Rx |
| 4.5.5 |  | Beam failure detection and link recovery procedures |  |  |  |  |  |  |  |  |  |  |
| 4.5.5.1 |  | EN-DC FR1 SSB-based beam failure detection and link recovery in non-DRX |  | Rel-15 |  | C082 |  | UEs supporting EN-DC FR1 and link recovery |  |  |  | 2Rx 4Rx |
| 4.5.5.2 |  | EN-DC FR1 SSB-based beam failure detection and link recovery in DRX |  | Rel-15 |  | C082a |  | UEs supporting EN-DC FR1 and long DRX cycle and link recovery |  |  |  | 2Rx 4Rx |
| 4.5.5.3 |  | EN-DC FR1 CSI-RS-based beam failure detection and link recovery in non-DRX |  | Rel-15 |  | C083 |  | UEs supporting EN-DC FR1 and CSI-RS-based RLM and link recovery |  |  |  | 2Rx 4Rx |
| 4.5.5.4 |  | EN-DC FR1 CSI-RS-based beam failure detection and link recovery in DRX |  | Rel-15 |  | C083a |  | UEs supporting EN-DC FR1 and long DRX cycle and CSI-RS-based RLM and link recovery |  |  |  | 2Rx 4Rx |
| 4.5.5.5 |  | EN-DC FR1 Scell CSI-RS-based beam failure detection and SSB-based link recovery in non-DRX |  | Rel-16 |  | C175 |  | UEs supporting EN-DC FR1 and CSI-RS-based RLM and SSB link recovery |  |  |  | 2Rx 4Rx |
| 4.5.5.6 |  | EN-DC FR1 Scell CSI-RS-based beam failure detection and SSB-based link recovery in DRX |  | Rel-16 |  | C176 |  | UEs supporting EN-DC FR1 and long DRX cycle and CSI-RS-based RLM and SSB link recovery |  |  |  | 2Rx 4Rx |
| 4.5.6 |  | Active BWP switch delay |  |  |  |  |  |  |  |  |  |  |
| 4.5.6.1 |  | DCI-based and timer-based active BWP switch |  |  |  |  |  |  |  |  |  |  |
| 4.5.6.1.1 |  | EN-DC FR1 DCI-based DL active BWP switch in non-DRX in synchronous EN-DC |  | Rel-15 |  | C065 |  | UEs supporting EN-DC FR1 and (DCI and timer based active BWP switching delay Type1 or Type2) and (Support of BWP adaptation upto2 or upto4) |  |  |  | 2Rx 4Rx |
| 4.5.6.1.2 |  | EN-DC FR1 DCI-based DL active BWP switch with SCell in non-DRX in synchronous EN-DC |  | Rel-15 |  | C065a |  | UEs supporting EN-DC FR1 and (DCI and timer based active BWP switching delay Type1 or Type2) and (Support of BWP adaptation upto2 or upto4) and 2DL CA |  |  |  | 2Rx 4Rx |
| 4.5.6.2 |  | RRC-based active BWP switch |  |  |  |  |  |  |  |  |  |  |
| 4.5.6.2.1 |  | EN-DC FR1 RRC-based DL active BWP switch in non-DRX in synchronous EN-DC |  | Rel-15 |  | C065b |  | UEs supporting EN-DC FR1 and (Support of BWP adaptation upto2 or upto4) |  |  |  | 2Rx 4Rx |
| 4.5.7 |  | PSCell addition and release delay |  |  |  |  |  |  |  |  |  |  |
| 4.5.7.1 |  | EN-DC FR1 addition and release delay of known PSCell |  | Rel-15 |  | C021 |  | UEs supporting EN-DC FR1 |  |  |  | 2Rx 4Rx |
| 4.5.8 |  | UL switching |  |  |  |  |  |  |  |  |  |  |
| 4.5.8.1 |  | EN-DC FR1 interruptions at switching between two uplink carriers |  | Rel-16 |  | C126a |  | UEs supporting EN-DC and dynamic UL Tx switching in case of inter-band EN-DC |  |  |  | 2Rx 4Rx |
| 4.6 |  | Measurement procedures |  |  |  |  |  |  |  |  |  |  |
| 4.6.1 |  | Intra-frequency measurements |  |  |  |  |  |  |  |  |  |  |
| 4.6.1.1 |  | EN-DC FR1 event-triggered reporting without gap in non-DRX |  | Rel-15 |  | C021 |  | UEs supporting EN-DC FR1 |  |  |  | 2Rx 4Rx |
| 4.6.1.2 |  | EN-DC FR1 event-triggered reporting without gap in DRX |  | Rel-15 |  | C021a |  | UEs supporting EN-DC FR1 and long DRX cycle |  |  |  | 2Rx 4Rx |
| 4.6.1.3 |  | EN-DC FR1 event-triggered reporting with gap in non-DRX |  | Rel-15 |  | C042 |  | UEs supporting EN-DC FR1 and CSI-RS-based RLM and BWP operation without bandwidth restriction |  |  |  | 2Rx 4Rx |
| 4.6.1.4 |  | EN-DC FR1 event-triggered reporting with gap in DRX |  | Rel-15 |  | C042a |  | UEs supporting EN-DC FR1, CSI-RS-based RLM, BWP operation without bandwidth restriction and long DRX cycle |  |  |  | 2Rx 4Rx |
| 4.6.1.5 |  | EN-DC FR1 event-triggered reporting without gap in non-DRX with SSB time index detection |  | Rel-15 |  | C021b |  | UEs supporting EN-DC FDD FR1 |  |  |  | 2Rx 4Rx |
| 4.6.1.6 |  | EN-DC FR1 event-triggered reporting with gap in non-DRX with SSB time index detection |  | Rel-15 |  | C042b |  | UEs supporting EN-DC FDD FR1 and CSI-RS based RLM and BWP operation without bandwidth restriction |  |  |  | 2Rx 4Rx |
| 4.6.1.7 |  | EN-DC FR1 event-triggered reporting without gap in DRX for UE configured with highSpeedMeasFlag-r16 |  | Rel-15 |  | C097 |  | UEs supporting EN-DC FR1 and long DRX cycle and measurement enhancements in HST |  |  |  | 2Rx 4Rx |
| 4.6.1.8 |  | EN-DC FR1 event triggered reporting cell without SSB time index detection in DRX for UE configured with highSpeedMeasCA-Scell-r17 |  | Rel-16 |  | C021c |  |  | UEs supporting EN-DC FR1 and CA measurement enhancements in HST |  | NOTE 1 | 2Rx 4Rx |
| 4.6.2 |  | Inter-frequency measurements |  |  |  |  |  |  |  |  |  |  |
| 4.6.2.1 |  | EN-DC FR1-FR1 event-triggered reporting in non-DRX |  | Rel-15 |  | C021 |  | UEs supporting EN-DC FR1 |  |  |  | 2Rx 4Rx |
| 4.6.2.2 |  | EN-DC FR1-FR1 event-triggered reporting in DRX |  | Rel-15 |  | C021a |  | UEs supporting EN-DC FR1 and long DRX cycle |  |  |  | 2Rx 4Rx |
| 4.6.2.5 |  | EN-DC FR1-FR1 event-triggered reporting in non-DRX with SSB time index detection |  | Rel-15 |  | C021 |  | UEs supporting EN-DC FR1 |  |  |  | 2Rx 4Rx |
| 4.6.2.6 |  | EN-DC FR1-FR1 event-triggered reporting in DRX with SSB time index detection |  | Rel-15 |  | C021a |  | UEs supporting EN-DC FR1 and long DRX cycle |  |  |  | 2Rx 4Rx |
| 4.6.2.9 |  | EN-DC FR1-FR1 event triggered reporting without SSB time index detection in DRX for UE configured with highSpeedMeasInterFreq-r17 |  | Rel-16 |  | C021d |  |  | UEs supporting EN-DC FR1 and inter-frequency measurement enhancements in HST |  | NOTE 1 | 2Rx 4Rx |
| 4.6.4 |  | L1-RSRP for beam reporting |  |  |  |  |  |  |  |  |  |  |
| 4.6.4.1 |  | EN-DC FR1 SSB-based L1-RSRP measurement in non-DRX |  | Rel-15 |  | C021 |  | UEs supporting EN-DC FR1 |  |  |  | 2Rx 4Rx |
| 4.6.4.2 |  | EN-DC FR1 SSB-based L1-RSRP measurement in DRX |  | Rel-15 |  | C021a |  | UEs supporting EN-DC FR1 and long DRX cycle |  |  |  | 2Rx 4Rx |
| 4.6.4.3 |  | EN-DC FR1 CSI-RS-based L1-RSRP measurement in non-DRX |  | Rel-15 |  | C021 |  | UEs supporting EN-DC FR1 |  |  |  | 2Rx 4Rx |
| 4.6.4.4 |  | EN-DC FR1 CSI-RS-based L1-RSRP measurement in DRX |  | Rel-15 |  | C021a |  | UEs supporting EN-DC FR1 and long DRX cycle |  |  |  | 2Rx 4Rx |
| 4.6.4.5 |  | EN-DC FR1 SSB-based L1-RSRP measurement in DRX for UE configured with highSpeedMeasFlag-r16 |  | Rel-15 |  | C098 |  | UEs supporting EN-DC FR1, long DRX cycle and intra-NR measurement enhancement in HST |  |  |  | 2Rx 4Rx |
| 4.6.7 |  | L1-SINR for beam reporting |  |  |  |  |  |  |  |  |  |  |
| 4.6.7.1 |  | EN-DC FR1 CSI-RS based CMR and no dedicated IMR L1-SINR measurement in non-DRX |  | Rel-16 |  | C141 |  | UEs supporting EN-DC FR1 and L1-SINR measurement based on CSI-RS as CMR without dedicated IMR configured |  |  |  | 2Rx 4Rx |
| 4.6.7.2 |  | EN-DC FR1 SSB based CMR and dedicated IMR L1-SINR measurement in DRX |  | Rel-16 |  | C142 |  | UEs supporting EN-DC FR1 and long DRX cycle and L1-SINR measurement based on SSB as CMR and dedicated CSI-IM as IMR |  |  |  | 2Rx 4Rx |
| 4.6.7.3 |  | EN-DC FR1 CSI-RS based CMR and dedicated IMR L1-SINR measurement in DRX |  | Rel-16 |  | C143 |  | UEs supporting EN-DC FR1 and long DRX cycle and L1-SINR measurement based on CSI-RS as CMR and dedicated CSI-RS as IMR |  |  |  | 2Rx 4Rx |
| 4.7 |  | Measurement performance requirements |  |  |  |  |  |  |  |  |  |  |
| 4.7.1 |  | SS-RSRP |  |  |  |  |  |  |  |  |  |  |
| 4.7.1.1 |  | Intra-frequency measurements |  |  |  |  |  |  |  |  |  |  |
| 4.7.1.1.1 |  | EN-DC FR1 SS-RSRP absolute measurement accuracy |  | Rel-15 |  | C021 |  | UEs supporting EN-DC FR1 |  |  |  | 2Rx 4Rx |
| 4.7.1.1.2 |  | EN-DC FR1 SS-RSRP relative measurement accuracy |  | Rel-15 |  | C021 |  | UEs supporting EN-DC FR1 |  |  |  | 2Rx 4Rx |
| 4.7.1.2 |  | Inter-frequency measurements |  |  |  |  |  |  |  |  |  |  |
| 4.7.1.2.1 |  | EN-DC FR1-FR1 SS-RSRP absolute measurement accuracy |  | Rel-15 |  | C021 |  | UEs supporting EN-DC FR1 |  |  |  | 2Rx 4Rx |
| 4.7.1.2.2 |  | EN-DC FR1-FR1 SS-RSRP relative measurement accuracy |  | Rel-15 |  | C021 |  | UEs supporting EN-DC FR1 |  |  |  | 2Rx 4Rx |
| 4.7.2 |  | SS-RSRQ |  |  |  |  |  |  |  |  |  |  |
| 4.7.2.1 |  | EN-DC FR1 SS-RSRQ measurement accuracy |  | Rel-15 |  | C021 |  | UEs supporting EN-DC FR1 |  |  |  | 2Rx 4Rx |
| 4.7.2.2.1 |  | EN-DC FR1-FR1 SS-RSRQ absolute measurement accuracy |  | Rel-15 |  | C021 |  | UEs supporting EN-DC FR1 |  |  |  | 2Rx 4Rx |
| 4.7.2.2.2 |  | EN-DC FR1-FR1 SS-RSRQ relative measurement accuracy |  | Rel-15 |  | C021 |  | UEs supporting EN-DC FR1 |  |  |  | 2Rx 4Rx |
| 4.7.3 |  | SS-SINR |  |  |  |  |  |  |  |  |  |  |
| 4.7.3.1 |  | EN-DC FR1 SS-SINR measurement accuracy |  | Rel-15 |  | C035 |  | UEs supporting EN-DC FR1 and SS-SINR-meas |  |  |  | 2Rx 4Rx |
| 4.7.3.2.1 |  | EN-DC FR1-FR1 SS-SINR absolute measurement accuracy |  | Rel-15 |  | C035 |  | UEs supporting EN-DC FR1 and SS-SINR-meas |  |  |  | 2Rx 4Rx |
| 4.7.3.2.2 |  | EN-DC FR1-FR1 SS-SINR relative measurement accuracy |  | Rel-15 |  | C035 |  | UEs supporting EN-DC FR1 and SS-SINR-meas |  |  |  | 2Rx 4Rx |
| 4.7.4 |  | L1-RSRP |  |  |  |  |  |  |  |  |  |  |
| 4.7.4.1.1 |  | EN-DC FR1 SSB-based L1-RSRP absolute measurement accuracy |  | Rel-15 |  | C021 |  | UEs supporting EN-DC FR1 |  |  |  | 2Rx 4Rx |
| 4.7.4.1.2 |  | EN-DC FR1 SSB-based L1-RSRP relative measurement accuracy |  | Rel-15 |  | C021 |  | UEs supporting EN-DC FR1 |  |  |  | 2Rx 4Rx |
| 4.7.4.2.1 |  | EN-DC FR1 CSI-RS-based L1-RSRP absolute measurement accuracy |  | Rel-15 |  | C021 |  | UEs supporting EN-DC FR1 |  |  |  | 2Rx 4Rx |
| 4.7.4.2.2 |  | EN-DC FR1 CSI-RS-based L1-RSRP relative measurement accuracy |  | Rel-15 |  | C021 |  | UEs supporting EN-DC FR1 |  |  |  | 2Rx 4Rx |
| 4.7.5 |  | SFTD |  |  |  |  |  |  |  |  |  |  |
| 4.7.5.1 |  | EN-DC FR1 SFTD measurement accuracy |  | Rel-15 |  | C043 |  | UEs supporting EN-DC FR1 and SFTD measurements between E-UTRA PCell and NR PSCell |  |  |  | 2Rx 4Rx |
| 4.7.7 |  | L1-SINR |  |  |  |  |  |  |  |  |  |  |
| 4.7.7.1.1 |  | EN-DC FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR absolute measurement accuracy |  | Rel-16 |  | C135 |  | UEs supporting EN-DC FR1 and L1-SINR-measurement based on CSI-RS as CMR without dedicated IMR configured |  |  |  | 2Rx 4Rx |
| 4.7.7.1.2 |  | EN-DC FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR relative measurement accuracy |  | Rel-16 |  | C135 |  | UEs supporting EN-DC FR1 and L1-SINR-measurement based on CSI-RS as CMR without dedicated IMR configured |  |  |  | 2Rx 4Rx |
| 4.7.7.2 |  | EN-DC FR1 SSB based CMR and dedicated IMR L1-SINR absolute measurement accuracy |  | Rel-16 |  | C136 |  | UEs supporting EN-DC FR1 and L1-SINR-measurement based on SSB as CMR and dedicated CSI-IM as IMR |  |  |  | 2Rx 4Rx |
| 4.7.7.3.1 |  | EN-DC FR1 CSI-RS based CMR and dedicated IMR L1-SINR absolute measurement accuracy |  | Rel-16 |  | C137 |  | UEs supporting 5GS NR SA FR1 and L1-SINR-measurement based on CSI-RS as CMR and dedicated CSI-IM as IMR |  |  |  | 2Rx 4Rx |
| 4.7.7.3.2 |  | EN-DC FR1 CSI-RS based CMR and dedicated IMR L1-SINR relative measurement accuracy |  | Rel-16 |  | C137 |  | UEs supporting 5GS NR SA FR1 and L1-SINR-measurement based on CSI-RS as CMR and dedicated CSI-IM as IMR |  |  |  |  | 2Rx 4Rx |
| 4A |  | NE-DC with all NR cells in FR1 |  |  |  |  |  |  |  |  |  |  |
| 4A.1 |  | Signalling characteristics |  |  |  |  |  |  |  |  |  |  |
| 4A.1.1 |  | E-UTRA PSCell addition |  |  |  |  |  |  |  |  |  |  |
| 4A.1.1.1 |  | NE-DC FR1 addition and release delay of known PSCell |  | Rel-15 |  | FFS |  | FFS |  |  | NOTE 1 | 2Rx 4Rx |
| NOTE 1: The test case/branch is incomplete for any Band/CA/DC Configuration but has basic test configurations already. NOTE 1 can be removed only when the test case/branch is complete for at least one Band / CA/DC Configuration for at least one feature included in the test case/branch. Detailed completion status can be found in the corresponding test case section in 38.533. NOTE 2: Test X refers to the corresponding Sub-Test as defined in TS 38.533 [5]. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.2-1a: Void

Table 4.2-2: Applicability of RRM EN-DC FR2 conformance test cases, ref. TS 38.533 [5]

| Clause | TC Title | Release | Applicability |  | Additional Information | Branch |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Condition | Comment |  |  |
| 5.3 | RRC_CONNECTED state mobility |  |  |  |  |  |
| 5.3.2 | RRC connection mobility control |  |  |  |  |  |
| 5.3.2.2 | Random access |  |  |  |  |  |
| 5.3.2.2.1 | EN-DC FR2 contention based random access | Rel-16 | C022 | UEs supporting EN-DC FR2 |  | 2Rx 4Rx |
| 5.3.2.2.2 | EN-DC FR2 non-contention based random access | Rel-16 | C030a | UEs supporting EN-DC FR2 and CSI-RS based PRACH |  | 2Rx 4Rx |
| 5.3.2.2.3 | EN-DC FR2 2-step contention based random access | Rel-16 | C158 | UEs supporting EN-DC FR2 and 2-step RACH | NOTE 1 | 2Rx 4Rx |
| 5.3.2.2.4 | EN-DC FR2 2-step non-contention based random access | Rel-16 | C158 | UEs supporting EN-DC FR2 and 2-step RACH | NOTE 1 | 2Rx 4Rx |
| 5.4 | Timing |  |  |  |  |  |
| 5.4.1 | UE transmit timing |  |  |  |  |  |
| 5.4.1.1 | EN-DC FR2 UE transmit timing accuracy | Rel-15 | C022 | UEs supporting EN-DC FR2 |  | 2Rx |
| 5.4.2 | UE timer accuracy |  |  |  |  |  |
| 5.4.3 | Timing advance |  |  |  |  |  |
| 5.4.3.1 | EN-DC FR2 timing advance adjustment accuracy | Rel-15 | C022 | UEs supporting EN-DC FR2 |  | 2Rx |
| 5.5 | Signalling characteristics |  |  |  |  |  |
| 5.5.1 | Radio link monitoring |  |  |  |  |  |
| 5.5.1.1 | EN-DC FR2 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 5.5.1.2 | EN-DC FR2 radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 5.5.1.3 | EN-DC FR2 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in DRX mode | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 5.5.1.4 | EN-DC FR2 radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in DRX mode | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 5.5.1.5 | EN-DC FR2 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 5.5.1.6 | EN-DC FR2 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 5.5.1.7 | EN-DC FR2 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in DRX mode | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 5.5.1.8 | EN-DC FR2 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM RS in DRX mode | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 5.5.1.9 | EN-DC FR2 radio link monitoring UE scheduling restrictions | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 5.5.2 | Interruption |  |  |  |  |  |
| 5.5.2.1 | EN-DC FR2 interruptions at transitions between active and non-active during DRX in synchronous EN-DC | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 5.5.2.2 | EN-DC FR2 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 5.5.2.3 | EN-DC FR2 interruptions during measurements on deactivated NR SCC in synchronous EN-DC | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 5.5.2.4 | EN-DC FR2 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 5.5.2.5 | EN-DC FR2 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 5.5.2.6 | EN-DC FR2 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 5.5.3 | SCell activation and deactivation delay |  |  |  |  |  |
| 5.5.3.1 | EN-DC FR2 SCell activation and deactivation intra-band in non-DRX | Rel-15 | C180 | UEs supporting EN-DC FR2 and 2DL CA in NR | NOTE 1 | 2Rx |
| 5.5.4 | UE UL carrier RRC reconfiguration delay |  |  |  |  |  |
| 5.5.5 | Beam failure detection and link recovery procedures |  |  |  |  |  |
| 5.5.5.1 | EN-DC FR2 SSB-based beam failure detection and link recovery in non-DRX | Rel-15 | C022 | UEs supporting EN-DC FR2 | NOTE 1 | 2Rx |
| 5.5.5.2 | EN-DC FR2 SSB-based beam failure detection and link recovery in DRX | Rel-15 | C022a | UEs supporting EN-DC FR2 and long DRX cycle | NOTE 1 | 2Rx |
| 5.5.5.3 | EN-DC FR2 CSI-RS-based beam failure detection and link recovery in non-DRX | Rel-15 | C161 | UEs supporting EN-DC FR2 and CSI-RS-based RLM | NOTE 1 | 2Rx |
| 5.5.5.4 | EN-DC FR2 CSI-RS-based beam failure detection and link recovery in DRX | Rel-15 |  | UEs supporting EN-DC FR2 and long DRX cycle and CSI-RS-based RLM | NOTE 1 | 2Rx |
| 5.5.5.5 | EN-DC FR2 scheduling available restriction during SSB-based beam failure detection and link recovery in non-DRX | Rel-15 | C022 | UEs supporting EN-DC FR2 | NOTE 1 | 2Rx |
| 5.5.5.6 | EN-DC FR2 CSI-RS-based BFD and LR for SCell in non-DRX | Rel-16 | C149 | UEs supporting EN-DC FR2 and CSI-RS based BFR on SCell |  |  |
| 5.5.5.7 | EN-DC FR2 SCell CSI-RS-based beam failure detection and link recovery in DRX | Rel-16 | C150 | UEs supporting EN-DC FR2 and long DRX cycle and CSI-RS based BFR on SCell |  |  |
| 5.5.6 | Active BWP switch delay |  |  |  |  |  |
| 5.5.6.1 | DCI-based and timer-based active BWP switch |  |  |  |  |  |
| 5.5.6.1.1 | EN-DC FR2 DCI-based DL active BWP switch in non-DRX in synchronous EN-DC | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 5.5.6.1.2 | EN-DC FR2 DCI-based DL active BWP switch with SCell in non-DRX in synchronous EN-DC | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 5.5.6.2 | RRC-based active BWP switch |  |  |  |  |  |
| 5.5.6.2.1 | EN-DC FR2 RRC-based DL active BWP switch in non-DRX in synchronous EN-DC | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 5.5.7 | PSCell addition and release delay |  |  |  |  |  |
| 5.5.7.1 | Void |  |  |  |  |  |
| 5.5.8 | Active TCI state switch delay |  |  |  |  |  |
| 5.5.8.1 | EN-DC FR2 MAC-CE based active TCI state switch | Rel-15 | C022 | UEs supporting EN-DC FR2 | NOTE 1 | 2Rx |
| 5.5.8.2 | EN-DC FR2 RRC based active TCI state switch | Rel-15 | C022 | UEs supporting EN-DC FR2 | NOTE 1 | 2Rx |
| 5.6 | Measurement procedures |  |  |  |  |  |
| 5.6.1 | Intra-frequency measurements |  |  |  |  |  |
| 5.6.1.1 | EN-DC FR2 event-triggered reporting without gap in non-DRX | Rel-15 | C022 | UEs supporting EN-DC FR2 | NOTE 1 | 2Rx |
| 5.6.1.2 | EN-DC FR2 event-triggered reporting without gap in DRX | Rel-15 | C022a | UEs supporting EN-DC FR2 and long DRX cycle |  | 2Rx |
| 5.6.1.3 | EN-DC FR2 event-triggered reporting with gap in non-DRX | Rel-15 | C163 | UEs supporting EN-DC FR2, CSI-RS-based RLM and BWP operation without bandwidth restriction | NOTE 1 | 2Rx |
| 5.6.1.4 | EN-DC FR2 event-triggered reporting with gap in DRX | Rel-15 | C043a | UEs supporting EN-DC FR2, long DRX cycle, CSI-RS based RLM and BWP operation without BW restriction |  | 2Rx |
| 5.6.2 | Inter-frequency measurements |  |  |  |  |  |
| 5.6.2.1 | EN-DC FR2-FR2 event-triggered reporting in non-DRX | Rel-15 | C022 | UEs supporting EN-DC FR2 |  | 2Rx |
| 5.6.2.2 | EN-DC FR2-FR2 event-triggered reporting in DRX | Rel-15 | C022a | UEs supporting EN-DC FR2 and long DRX cycle |  | 2Rx |
| 5.6.2.3 | EN-DC FR2-FR2 event-triggered reporting in non-DRX with SSB time index detection | Rel-15 | C022 | UEs supporting EN-DC FR2 |  | 2Rx |
| 5.6.2.4 | EN-DC FR2-FR2 event-triggered reporting in DRX with SSB time index detection | Rel-15 | C022a | UEs supporting EN-DC FR2 and long DRX cycle |  | 2Rx |
| 5.6.2.5 | EN-DC FR1-FR2 event-triggered reporting in non-DRX | Rel-15 | C023 | UEs supporting EN-DC FR1 and FR2 |  | 2Rx |
| 5.6.2.6 | EN-DC FR1-FR2 event-triggered reporting in DRX | Rel-15 | C023a | UEs supporting EN-DC FR1 and FR2 and long DRX cycle |  | 2Rx |
| 5.6.2.7 | EN-DC FR1-FR2 event-triggered reporting in non-DRX with SSB time index detection | Rel-15 | C023 | UEs supporting EN-DC FR1 and FR2 |  | 2Rx |
| 5.6.2.8 | EN-DC FR1-FR2 event-triggered reporting in DRX with SSB time index detection | Rel-15 | C023a | UEs supporting EN-DC FR1 and FR2 and long DRX cycle |  | 2Rx |
| 5.6.3 | L1-RSRP for beam reporting |  |  |  |  |  |
| 5.6.3.1 | EN-DC FR2 SSB-based L1-RSRP measurement in non-DRX | Rel-15 | C022 | UEs supporting EN-DC FR2 |  | 2Rx |
| 5.6.3.2 | EN-DC FR2 SSB-based L1-RSRP measurement in DRX | Rel-15 | C022a | UEs supporting EN-DC FR2 and long DRX cycle |  | 2Rx |
| 5.6.3.3 | EN-DC FR2 CSI-RS-based L1-RSRP measurement in non-DRX | Rel-15 | C022 | UEs supporting EN-DC FR2 |  | 2Rx |
| 5.6.3.4 | EN-DC FR2 CSI-RS-based L1-RSRP measurement in DRX | Rel-15 | C022a | UEs supporting EN-DC FR2 and long DRX cycle |  | 2Rx |
| 5.6.3 | L1-SINR measurement for beam reporting |  |  |  |  |  |
| 5.6.6.1 | EN-DC FR2 CSI-RS based CMR and no dedicated IMR L1-SINR measurement in DRX | Rel-16 | C141a | UEs supporting EN-DC FR2 and long DRX cycle and L1-SINR measurement based on CSI-RS as CMR without dedicated IMR configured |  | 2Rx 4Rx |
| 5.6.6.2 | EN-DC FR2 SSB based CMR and dedicated IMR L1-SINR measurement in non-DRX | Rel-16 | C142a | UEs supporting EN-DC FR2 and L1-SINR measurement based on SSB as CMR and dedicated CSI-RS as IMR |  | 2Rx 4Rx |
| 5.6.6.3 | EN-DC FR2 CSI-RS based CMR and dedicated IMR L1-SINR measurement in non-DRX | Rel-16 | C143a | UEs supporting EN-DC FR2 and L1-SINR measurement based on CSI-RS as CMR and dedicated CSI-IM as IMR |  | 2Rx 4Rx |
| 5.7 | Measurement performance requirements |  |  |  |  |  |
| 5.7.1 | SS-RSRP |  |  |  |  |  |
| 5.7.1.1 | EN-DC FR2 SS-RSRP measurement accuracy | Rel-15 | C022 | UEs supporting EN-DC FR2 |  | 2Rx |
| 5.7.1.2 | EN-DC FR2-FR2 SS-RSRP measurement accuracy | Rel-15 | C022 | UEs supporting EN-DC FR2 |  | 2Rx |
| 5.7.1.3 | EN-DC FR1-FR2 SS-RSRP measurement accuracy | Rel-15 | N/A | not recommended due to E-UTRA/FR1 – FR2 testability issue |  | 2Rx |
| 5.7.2 | SS-RSRQ |  |  |  |  |  |
| 5.7.2.1 | EN-DC FR2 SS-RSRQ measurement accuracy | Rel-15 | C022 | UEs supporting EN-DC FR2 |  | 2Rx |
| 5.7.2.2 | EN-DC FR2-FR2 SS-RSRQ measurement accuracy | Rel-15 | C022 | UEs supporting EN-DC FR2 | NOTE 1 | 2Rx |
| 5.7.3 | SS-SINR |  |  |  |  |  |
| 5.7.3.1 | EN-DC FR2 SS-SINR measurement accuracy | Rel-15 | C069 | UEs supporting EN-DC FR2 and SS-SINR-meas |  | 2Rx |
| 5.7.3.2 | EN-DC FR2-FR2 SS-SINR measurement accuracy | Rel-15 | C069 | UEs supporting EN-DC FR2 and SS-SINR-meas | NOTE 1 | 2Rx |
| 5.7.4 | L1-RSRP |  |  |  |  |  |
| 5.7.4.1 | EN-DC FR2 SSB based L1-RSRP measurement accuracy | Rel-15 | C022 | UEs supporting EN-DC FR2 |  | 2Rx |
| 5.7.4.2 | EN-DC FR2 CSI-RS based L1-RSRP measurement accuracy | Rel-15 | C022 | UEs supporting EN-DC FR2 |  | 2Rx |
| 5.7.6 | L1-SINR measurement for beam reporting |  |  |  |  |  |
| 5.7.6.1 | EN-DC FR2 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR measurement accuracy | Rel-16 | C141b | UEs supporting EN-DC FR2 and L1-SINR measurement based on CSI-RS as CMR without dedicated IMR configured |  | 2Rx |
| 5.7.6.2 | EN-DC FR2 SSB based CMR and dedicated IMR L1-SINR absolute measurement accuracy | Rel-16 | C142a | UEs supporting EN-DC FR2 and L1-SINR measurement based on SSB as CMR and dedicated CSI-RS as IMR |  | 2Rx |
| 5.7.6.3 | EN-DC FR2 CSI-RS based CMR and dedicated IMR L1-SINR measurement accuracy | Rel-16 | C143a | UEs supporting EN-DC FR2 and L1-SINR measurement based on CSI-RS as CMR and dedicated CSI-IM as IMR |  | 2Rx |
| NOTE 1: The test case/branch is incomplete for any Band/CA/DC Configuration but has basic test configurations already. NOTE 1 can be removed only when the test case/branch is complete for at least one Band / CA/DC Configuration for at least one feature included in the test case/branch. Detailed completion status can be found in the corresponding test case section in 38.533. NOTE 2: Void. NOTE 3: Void. |  |  |  |  |  |  |

Table 4.2-2a: Void

Table 4.2-3: Applicability of RRM NR SA FR1 conformance test cases, ref. TS 38.533 [5]

| Clause |  | TC Title |  | Release |  | Applicability |  |  |  | Additional Information | Branch |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Condition |  | Comment |  |  |  |
| 6.1 |  | RRC_IDLE state mobility |  |  |  |  |  |  |  |  |  |
| 6.1.1 |  | NR cell re-selection |  |  |  |  |  |  |  |  |  |
| 6.1.1.1 |  | NR SA FR1 cell re-selection |  | Rel-15 |  | C001 |  | UEs supporting 5GS NR SA FR1 |  |  | 2Rx 4Rx |
| 6.1.1.2 |  | NR SA FR1-FR1 cell re-selection |  | Rel-15 |  | C001 |  | UEs supporting 5GS NR SA FR1 |  |  | 2Rx 4Rx |
| 6.1.1.3 |  | NR SA FR1 cell re-selection for UE fulfilling low mobility relaxed measurement criterion |  | Rel-16 |  | C093 |  | UEs supporting 5GS NR SA FR1 and relaxed RRM measurement |  |  | 2Rx 4Rx |
| 6.1.1.4 |  | NR SA FR1 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion |  | Rel-16 |  | C093 |  | UEs supporting 5GS NR SA FR1 and relaxed RRM measurement |  |  | 2Rx 4Rx |
| 6.1.1.5 |  | NR SA FR1-FR1 cell re-selection for UE fulfilling low mobility relaxed measurement criterion |  | Rel-16 |  | C093 |  | UEs supporting 5GS NR SA FR1 and relaxed RRM measurement |  |  | 2Rx 4Rx |
| 6.1.1.6 |  | NR SA FR1-FR1 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion |  | Rel-16 |  | C093 |  | UEs supporting 5GS NR SA FR1 and relaxed RRM measurement |  |  | 2Rx 4Rx |
| 6.1.1.7 |  | NR SA FR1 cell re-selection for UE configured with highSpeedMeasFlag-r16 |  | Rel-15 |  | C052 |  | UEs supporting 5GS NR SA FR1 and measurement enhancements in HST |  |  | 2Rx 4Rx |
| 6.1.1.8 |  | NR SA FR1-FR1 Cell reselection for UE configured with highSpeedMeasInterFreq-r17 |  | Rel-16 |  | C052a |  | UEs supporting 5GS NR SA FR1 and inter-freq measurement enhancements in HST |  | NOTE 1 | 2Rx 4Rx |
| 6.1.2 |  | NR – E-UTRA cell re-selection |  |  |  |  |  |  |  |  |  |
| 6.1.2.1 |  | NR SA FR1 – E-UTRA cell re-selection to higher priority E-UTRA |  | Rel-15 |  | C025 |  | UEs supporting 5GS NR SA FR1 and E-UTRA |  |  | 2Rx 4Rx |
| 6.1.2.2 |  | NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRA |  | Rel-15 |  | C025 |  | UEs supporting 5GS NR SA FR1 and E-UTRA |  |  | 2Rx 4Rx |
| 6.1.2.3 |  | NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRAN for UE fulfilling low mobility relaxed measurement criterion |  | Rel-16 |  | C094 |  | UEs supporting 5GS NR SA FR1 and E-UTRA and relaxed RRM measurement |  |  | 2Rx 4Rx |
| 6.1.2.4 |  | NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRAN for UE fulfilling not-at-cell edge relaxed measurement criterion |  | Rel-16 |  | C094 |  | UEs supporting 5GS NR SA FR1 and E-UTRA and relaxed RRM measurement |  |  | 2Rx 4Rx |
| 6.1.2.5 |  | NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRA for UE configured with highSpeedMeasFlag-r16 |  | Rel-15 |  | C025b |  | UEs supporting 5GS NR SA FR1 and E-UTRA and E-UTRA inter-RAT measurement enhancements in HST |  |  | 2Rx 4Rx |
| 6.2 |  | RRC_INACTIVE state mobility |  |  |  |  |  |  |  |  |  |
| 6.3 |  | RRC_CONNECTED state mobility |  |  |  |  |  |  |  |  |  |
| 6.3.1 |  | Handover |  |  |  |  |  |  |  |  |  |
| 6.3.1.1 |  | NR SA FR1 handover with known target cell |  | Rel-15 |  | C001 |  | UEs supporting 5GS NR SA FR1 |  |  | 2Rx 4Rx |
| 6.3.1.2 |  | NR SA FR1 handover with unknown target cell |  | Rel-15 |  | C001 |  | UEs supporting 5GS NR SA FR1 |  |  | 2Rx 4Rx |
| 6.3.1.3 |  | NR SA FR1-FR1 handover with unknown target cell |  | Rel-15 |  | C001 |  | UEs supporting 5GS NR SA FR1 |  |  | 2Rx 4Rx |
| 6.3.1.4 |  | NR SA FR1 – E-UTRA handover with known target cell |  | Rel-15 |  | C025 |  | UEs supporting 5GS NR SA FR1 and E-UTRA |  |  | 2Rx 4Rx |
| 6.3.1.5 |  | NR SA FR1 – E-UTRA handover with unknown target cell |  | Rel-15 |  | C025 |  | UEs supporting 5GS NR SA FR1 and E-UTRA |  |  | 2Rx 4Rx |
| 6.3.1.6 |  | NR SA FR1 – UTRAN FDD handover with known target cell |  | Rel-16 |  | C096 |  | UEs supporting 5GS NR SA FR1 and UTRAN FDD |  |  | 2Rx 4Rx |
| 6.3.1.7 |  | NR SA FR1 synchronous DAPS handover |  | Rel-16 |  | C101 |  | UEs supporting 5GS NR SA FR1 and intra-frequency DAPS handover |  |  | 2Rx 4Rx |
| 6.3.1.8 |  | NR SA FR1 asynchronous DAPS handover |  | Rel-16 |  | C102 |  | UEs supporting 5GS NR SA FR1 and intra-frequency async DAPS handover |  |  | 2Rx 4Rx |
| 6.3.1.9 |  | NR SA FR1 Intra-band inter-frequency synchronous DAPS handover |  | Rel-16 |  | C107 |  | UEs supporting 5GS NR SA FR1 and inter-frequency DAPS handover |  |  | 2Rx 4Rx |
| 6.3.1.10 |  | NR SA FR1 Intra-band inter-frequency asynchronous DAPS handover |  | Rel-16 |  | C108 |  | UEs supporting 5GS NR SA FR1 and inter-frequency async DAPS handover |  |  | 2Rx 4Rx |
| 6.3.1.11 |  | NR SA FR1 Inter-band inter-frequency synchronous DAPS handover |  | Rel-16 |  | C107 |  | UEs supporting 5GS NR SA FR1 and inter-frequency DAPS handover |  | For test configuration 1, 2, 4, 5, 9 | 2Rx 4Rx |
|  |  |  |  |  |  | C109 |  | UEs supporting 5GS NR SA FR1 and inter-frequency DAPS handover and supporting different SCSs in source PCell and inter-frequency target PCell |  | For test configuration 3, 6, 7, 8 | 2Rx 4Rx |
| 6.3.1.12 |  | NR SA FR1 Inter-band inter-frequency asynchronous DAPS handover |  | Rel-16 |  | C108 |  | UEs supporting 5GS NR SA FR1 and inter-frequency async DAPS handover |  | For test configuration 1, 2, 4, 5, 9 | 2Rx 4Rx |
|  |  |  |  |  |  | C110 |  | UEs supporting 5GS NR SA FR1 and inter-frequency async DAPS handover and supporting different SCSs in source PCell and inter-frequency target PCell |  | For test configuration 3, 6, 7, 8 | 2Rx 4Rx |
| 6.3.2 |  | RRC connection mobility control |  |  |  |  |  |  |  |  |  |
| 6.3.2.1 |  | RRC re-establishment |  |  |  |  |  |  |  |  |  |
| 6.3.2.1.1 |  | NR SA FR1 RRC re-establishment |  | Rel-15 |  | C001 |  | UEs supporting 5GS NR SA FR1 |  |  | 2Rx 4Rx |
| 6.3.2.1.2 |  | NR SA FR1 - FR1 RRC re-establishment |  | Rel-15 |  | C001 |  | UEs supporting 5GS NR SA FR1 |  |  | 2Rx 4Rx |
| 6.3.2.1.3 |  | NR SA FR1 RRC re-establishment without serving cell timing |  | Rel-15 |  | C001 |  | UEs supporting 5GS NR SA FR1 |  |  | 2Rx 4Rx |
| 6.3.2.2 |  | Random access |  |  |  |  |  |  |  |  |  |
| 6.3.2.2.1 |  | NR SA FR1 contention based random access |  | Rel-15 |  | C001 |  | UEs supporting 5GS NR SA FR1 |  |  | 2Rx 4Rx |
| 6.3.2.2.2 |  | NR SA FR1 non-contention based random access |  | Rel-15 |  | C029 |  | UEs supporting 5GS NR SA FR1 and CSI-RS based PRACH |  |  | 2Rx 4Rx |
| 6.3.2.2.3 |  | NR SA FR1 2-step contention based random access |  | Rel-16 |  | C159 |  | UEs supporting 5GS NR SA FR1and 2-step RACH |  |  | 2Rx 4Rx |
| 6.3.2.2.4 |  | NR SA FR1 2-step non-contention based random access |  | Rel-16 |  | C159 |  | UEs supporting 5GS NR SA FR1and 2-step RACH |  |  | 2Rx 4Rx |
| 6.3.2.3 |  | RRC connection release with redirection |  |  |  |  |  |  |  |  |  |
| 6.3.2.3.1 |  | NR SA FR1 RRC connection release with redirection |  | Rel-15 |  | C001 |  | UEs supporting 5GS NR SA FR1 |  |  | 2Rx 4Rx |
| 6.3.2.3.2 |  | NR SA FR1 - E-UTRA RRC connection release with redirection |  | Rel-15 |  | C025 |  | UEs supporting 5GS NR SA FR1 and E-UTRA |  |  | 2Rx 4Rx |
| 6.3.3 |  | Conditional handover |  |  |  |  |  |  |  |  |  |
| 6.3.3.1 |  | NR SA FR1 conditional handover |  | Rel-16 |  | C105 |  | UEs supporting 5GS NR SA FR1 and Conditional handover |  |  | 2Rx 4Rx |
| 6.3.3.2 |  | NR SA FR1-FR1 conditional handover |  | Rel-16 |  | C105 |  | UEs supporting 5GS NR SA FR1 and Conditional handover |  |  | 2Rx 4Rx |
| 6.4 |  | Timing |  |  |  |  |  |  |  |  |  |
| 6.4.1 |  | UE transmit timing |  |  |  |  |  |  |  |  |  |
| 6.4.1.1 |  | NR SA FR1 UE transmit timing accuracy |  | Rel-15 |  | C001 |  | UEs supporting 5GS NR SA FR1 |  |  | 2Rx 4Rx |
| 6.4.2 |  | UE timer accuracy |  |  |  |  |  |  |  |  |  |
| 6.4.3 |  | Timing advance |  |  |  |  |  |  |  |  |  |
| 6.4.3.1 |  | NR SA FR1 timing advance adjustment accuracy |  | Rel-15 |  | C001 |  | UEs supporting 5GS NR SA FR1 |  |  | 2Rx 4Rx |
| 6.5 |  | Signalling characteristics |  |  |  |  |  |  |  |  |  |
| 6.5.1 |  | Radio Link Monitoring |  |  |  |  |  |  |  |  |  |
| 6.5.1.1 |  | NR SA FR1 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in non-DRX mode |  | Rel-15 |  | C001 |  | UEs supporting 5GS NR SA FR1 |  |  | 2Rx 4Rx |
| 6.5.1.2 |  | NR SA FR1 radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in non-DRX mode |  | Rel-15 |  | C001 |  | UEs supporting 5GS NR SA FR1 |  |  | 2Rx 4Rx |
| 6.5.1.3 |  | NR SA FR1 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in DRX mode |  | Rel-15 |  | C001b |  | UEs supporting 5GS NR SA FR1 and long DRX cycle |  |  | 2Rx 4Rx |
| 6.5.1.4 |  | NR SA FR1 radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in DRX mode |  | Rel-15 |  | C001b |  | UEs supporting 5GS NR SA FR1 and long DRX cycle |  |  | 2Rx 4Rx |
| 6.5.1.5 |  | NR SA FR1 radio link monitoring out-of-sync test for PCell configured with CSI-RS-based RLM RS in non-DRX mode |  | Rel-15 |  | C037 |  | UEs supporting 5GS NR SA FR1 and CSI-RS-based RLM |  |  | 2Rx 4Rx |
| 6.5.1.6 |  | NR SA FR1 radio link monitoring in-sync test for PCell configured with CSI-RS-based RLM RS in non-DRX mode |  | Rel-15 |  | C037 |  | UEs supporting 5GS NR SA FR1 and CSI-RS-based RLM |  |  | 2Rx 4Rx |
| 6.5.1.7 |  | NR SA FR1 radio link monitoring out-of-sync test for PCell configured with CSI-RS-based RLM RS in DRX mode |  | Rel-15 |  | C037a |  | UEs supporting 5GS NR SA FR1, CSI-RS-based RLM and long DRX cycle |  |  | 2Rx 4Rx |
| 6.5.1.8 |  | NR SA FR1 radio link monitoring in-sync test for PCell configured with CSI-RS-based RLM RS in DRX mode |  | Rel-15 |  | C037a |  | UEs supporting 5GS NR SA FR1, CSI-RS-based RLM and long DRX cycle |  |  | 2Rx 4Rx |
| 6.5.2 |  | Interruption |  |  |  |  |  |  |  |  |  |
| 6.5.2.1 |  | NR SA FR1 interruptions during measurements on deactivated NR SCC |  | Rel-15 |  | C031 |  | UEs supporting 5GS NR SA FR1 and CA (2DL CA) |  |  | 2Rx 4Rx |
| 6.5.3 |  | Scell activation and deactivation delay |  |  |  |  |  |  |  |  |  |
| 6.5.3.1 |  | NR SA FR1 SCell activation and deactivation of known SCell in non-DRX for 160ms SCell measurement cycle |  | Rel-15 |  | C031 |  | UEs supporting 5GS NR SA FR1 and CA (2DL CA) |  |  | 2Rx 4Rx |
| 6.5.3.2 |  | NR SA FR1 SCell activation and deactivation of known SCell in non-DRX for 640ms SCell measurement cycle |  | Rel-15 |  | C031 |  | UEs supporting 5GS NR SA FR1 and CA (2DL CA) |  |  | 2Rx 4Rx |
| 6.5.3.3 |  | NR SA FR1 SCell activation and deactivation of unknown SCell in non-DRX |  | Rel-15 |  | C031 |  | UEs supporting 5GS NR SA FR1 and CA (2DL CA) |  |  | 2Rx 4Rx |
| 6.5.4 |  | UE UL carrier RRC reconfiguration delay |  |  |  |  |  |  |  |  |  |
| 6.5.4.1 |  | NR SA FR1 UE UL carrier RRC reconfiguration delay |  | Rel-15 |  | C002 |  | UEs supporting 5GS NR SA FR1 and SUL |  |  | 2Rx 4Rx |
| 6.5.5 |  | Link recovery procedures |  |  |  |  |  |  |  |  |  |
| 6.5.5.1 |  | NR SA FR1 SSB-based beam failure detection and link recovery in non-DRX |  | Rel-15 |  | C084 |  | UEs supporting 5GS NR SA FR1 and link recovery |  |  | 2Rx 4Rx |
| 6.5.5.2 |  | NR SA FR1 SSB-based beam failure detection and link recovery in DRX |  | Rel-15 |  | C084a |  | UEs supporting 5GS NR SA FR1 and long DRX cycle and link recovery |  |  | 2Rx 4Rx |
| 6.5.5.3 |  | NR SA FR1 CSI-RS-based beam failure detection and link recovery in non-DRX |  | Rel-15 |  | C085 |  | UEs supporting 5GS NR SA FR1 and CSI-RS-based RLM and link recovery |  |  | 2Rx 4Rx |
| 6.5.5.4 |  | NR SA FR1 CSI-RS-based beam failure detection and link recovery in DRX |  | Rel-15 |  | C085a |  | UEs supporting 5GS NR SA FR1 and long DRX cycle and CSI-RS-based RLM and link recovery |  |  | 2Rx 4Rx |
| 6.5.5.5 |  | NR SA FR1 Scell CSI-RS-based beam failure detection and SSB-based link recovery in non-DRX |  | Rel-16 |  | C173 |  | UEs supporting 5GS NR SA FR1 and CSI-RS-based RLM and SSB-based link recovery on SCell |  |  | 2Rx 4Rx |
| 6.5.5.6 |  | NR SA FR1 Scell CSI-RS-based beam failure detection and SSB-based link recovery in DRX |  | Rel-16 |  | C174 |  | UEs supporting 5GS NR SA FR1 and CSI-RS-based RLM and SSB-based link recovery on SCell and long DRX cycle |  |  | 2Rx 4Rx |
| 6.5.6 |  | Active BWP switch delay |  |  |  |  |  |  |  |  |  |
| 6.5.6.1 |  | DCI-based and timer-based active BWP switch |  |  |  |  |  |  |  |  |  |
| 6.5.6.1.1 |  | NR SA FR1-FR1 DCI-based DL active BWP switch in non-DRX |  | Rel-15 |  | C066a |  | UEs supporting 5GS NR SA FR1 and (DCI and timer based active BWP switching delay type1 or type2) and (Support of BWP adaptation upto2 or upto4) and 2DL CA |  |  | 2Rx 4Rx |
| 6.5.6.1.2 |  | NR SA FR1 DCI-based DL active BWP switch in non-DRX |  | Rel-15 |  | C066 |  | UEs supporting 5GS NR SA FR1 and (DCI and timer based active BWP switching delay type1 or type2) and (Support of BWP adaptation upto2 or upto4) |  |  | 2Rx 4Rx |
| 6.5.6.2 |  | RRC-based active BWP switch |  |  |  |  |  |  |  |  |  |
| 6.5.6.2.1 |  | NR SA FR1 RRC-based DL active BWP switch in non-DRX |  | Rel-15 |  | C066b |  | UEs supporting 5GS NR SA FR1 and (Support of BWP adaptation upto2 or upto4) |  |  | 2Rx 4Rx |
| 6.5.7 |  | DL interruptions at switching between two uplink carriers |  |  |  |  |  |  |  |  |  |
| 6.5.7.1 |  | NR SA FR1 DL Interruptions at switching between two uplink carriers in FDD-TDD CA |  | Rel-16 |  | C051 |  | UEs supporting 5GS NR SA FR1 and Inter-band CA (2UL CA)and dynamic UL Tx switching |  |  | 2Rx 4Rx |
| 6.5.7.2 |  | NR SA FR1 DL Interruptions at switching between two uplink carriers in TDD-TDD CA |  | Rel-16 |  | C051 |  | UEs supporting 5GS NR SA FR1 and Inter-band CA (2UL CA)and dynamic UL Tx switching |  |  | 2Rx 4Rx |
| 6.6 |  | Measurement procedures |  |  |  |  |  |  |  |  |  |
| 6.6.1 |  | Intra-frequency measurements |  |  |  |  |  |  |  |  |  |
| 6.6.1.1 |  | NR SA FR1 event-triggered reporting without gap in non-DRX |  | Rel-15 |  | C001 |  | UEs supporting 5GS NR SA FR1 |  |  | 2Rx 4Rx |
| 6.6.1.2 |  | NR SA FR1 event-triggered reporting without gap in DRX |  | Rel-15 |  | C001b |  | UEs supporting 5GS NR SA FR1 and long DRX cycle |  |  | 2Rx 4Rx |
| 6.6.1.3 |  | NR SA FR1 event-triggered reporting with gap in non-DRX |  | Rel-15 |  | C041 |  | UEs supporting 5GS NR SA FR1 and CSI-RS-based RLM and BWP operation without bandwidth restriction |  |  | 2Rx 4Rx |
| 6.6.1.4 |  | NR SA FR1 event-triggered reporting with gap in DRX |  | Rel-15 |  | C041a |  | UEs supporting 5GS NR SA FR1, CSI-RS-based RLM, BWP operation without bandwidth restriction and long DRX cycle |  |  | 2Rx 4Rx |
| 6.6.1.5 |  | NR SA FR1 event-triggered reporting without gap in non-DRX with SSB index reading |  | Rel-15 |  | C024 |  | UEs supporting 5GS NR FDD SA FR1 |  |  | 2Rx 4Rx |
| 6.6.1.6 |  | NR SA FR1 event-triggered reporting with gap in non-DRX with SSB index reading |  | Rel-15 |  | C041b |  | UEs supporting 5GS NR FDD SA FR1 and CSI-RS-based RLM and BWP operation without bandwidth restriction |  |  | 2Rx 4Rx |
| 6.6.1.7 |  | NR SA FR1 event-triggered reporting without gap in DRX for UE configured with highSpeedMeasFlag-r16 |  | Rel-15 |  | C052 |  | UEs supporting 5GS NR SA FR1 and measurement enhancements in HST |  |  | 2Rx 4Rx |
| 6.6.1.8 |  | NR SA FR1 event triggered reporting without gap in DRX for UE configured with highSpeedMeasCA-Scell-r17 |  | Rel-16 |  | C052b |  | UEs supporting 5GS NR SA FR1 and CA measurement enhancements in HST |  | NOTE 1 | 2Rx 4Rx |
| 6.6.2 |  | Inter-frequency measurements |  |  |  |  |  |  |  |  |  |
| 6.6.2.1 |  | NR SA FR1-FR1 event-triggered reporting in non-DRX |  | Rel-15 |  | C001 |  | UEs supporting 5GS NR SA FR1 |  |  | 2Rx 4Rx |
| 6.6.2.2 |  | NR SA FR1-FR1 event-triggered reporting in DRX |  | Rel-15 |  | C001b |  | UEs supporting 5GS NR SA FR1 and long DRX cycle |  |  | 2Rx 4Rx |
| 6.6.2.5 |  | NR SA FR1-FR1 event-triggered reporting in non-DRX with SSB time index detection |  | Rel-15 |  | C001 |  | UEs supporting 5GS NR SA FR1 |  |  | 2Rx 4Rx |
| 6.6.2.6 |  | NR SA FR1-FR1 event-triggered reporting in DRX with SSB time index detection |  | Rel-15 |  | C001b |  | UEs supporting 5GS NR SA FR1 and long DRX cycle |  |  | 2Rx 4Rx |
| 6.6.2.12 |  | NR SA FR1-FR1 event triggered reporting tests without SSB time index detection in DRX for UE configured with highSpeedMeasInterFreq-r17 |  | Rel-16 |  | C052c |  | UEs supporting 5GS NR SA FR1 and inter-freq measurement enhancements in HST |  | NOTE 1 | 2Rx 4Rx |
| 6.6.3 |  | Inter-RAT measurements |  |  |  |  |  |  |  |  |  |
| 6.6.3.1 |  | NR SA FR1 – E-UTRAN event-triggered reporting in non-DRX |  | Rel-15 |  | C025 |  | UEs supporting 5GS NR SA FR1 and E-UTRAN |  |  | 2Rx 4Rx |
| 6.6.3.2 |  | NR SA FR1 – E-UTRAN event-triggered reporting in DRX |  | Rel-15 |  | C025a |  | UEs supporting 5GS NR SA FR1, E-UTRAN and long DRX cycle |  |  | 2Rx 4Rx |
| 6.6.3.3 |  | NR SA FR1 – E-UTRAN event-triggered reporting in DRX for UE configured with highSpeedMeasFlag-r16 |  | Rel-15 |  | C025c |  | UEs supporting 5GS NR SA FR1 and E-UTRAN, long DRX cycle and E-UTRA inter-RAT measurement enhancements in HST |  |  | 2Rx 4Rx |
| 6.6.4 |  | L1-RSRP measurement for beam reporting |  |  |  |  |  |  |  |  |  |
| 6.6.4.1 |  | NR SA FR1 SSB-based L1-RSRP measurement in non-DRX |  | Rel-15 |  | C001 |  | UEs supporting 5GS NR SA FR1 |  |  | 2Rx 4Rx |
| 6.6.4.2 |  | NR SA FR1 SSB-based L1-RSRP measurement in DRX |  | Rel-15 |  | C001b |  | UEs supporting 5GS NR SA FR1 and long DRX cycle |  |  | 2Rx 4Rx |
| 6.6.4.3 |  | NR SA FR1 CSI-RS-based L1-RSRP measurement in non-DRX |  | Rel-15 |  | C001 |  | UEs supporting 5GS NR SA FR1 |  |  | 2Rx 4Rx |
| 6.6.4.4 |  | NR SA FR1 CSI-RS-based L1-RSRP measurement in DRX |  | Rel-15 |  | C001b |  | UEs supporting 5GS NR SA FR1 and long DRX cycle |  |  | 2Rx 4Rx |
| 6.6.4.5 |  | NR SA FR1 SSB-based L1-RSRP measurement in DRX for UE configured with highSpeedMeasFlag-r16 |  | Rel-15 |  | C001f |  | UEs supporting 5GS NR SA FR1, long DRX cycle and intra-NR measurement enhancements in HST |  |  | 2Rx 4Rx |
| 6.6.5 |  | FFS |  |  |  |  |  |  |  |  |  |
| 6.6.5.1 |  | NR SA FR1 – UTRAN FDD event triggered reporting in non-DRX |  | Rel-16 |  | C096 |  | UEs supporting 5GS NR SA FR1 and UTRAN FDD |  |  | 2Rx 4Rx |
| 6.6.8 |  | L1-SINR measurement for beam reporting |  |  |  |  |  |  |  |  |  |
| 6.6.8.1 |  | NR SA FR1 CSI-RS based CMR and no dedicated IMR L1-SINR measurement in DRX |  | Rel-16 |  | C144 |  | UEs supporting 5GS NR SA FR1 and long DRX cycle and L1-SINR measurement based on CSI-RS as CMR without dedicated IMR configured |  |  | 2Rx 4Rx |
| 6.6.8.2 |  | NR SA FR1 SSB based CMR and dedicated IMR L1-SINR measurement in non-DRX |  | Rel-16 |  | C145 |  | UEs supporting 5GS NR SA FR1 and L1-SINR measurement based on SSB as CMR and dedicated CSI-RS as IMR |  |  | 2Rx 4Rx |
| 6.6.8.3 |  | NR SA FR1 CSI-RS based CMR and dedicated IMR L1-SINR measurement in non-DRX |  | Rel-16 |  | C146 |  | UEs supporting 5GS NR SA FR1 and L1-SINR measurement based on CSI-RS as CMR and dedicated CSI-IM as IMR |  |  | 2Rx 4Rx |
| 6.6.9 |  | Idle Mode CA/DC Measurements |  |  |  |  |  |  |  |  |  |
| 6.6.9.1 |  | NR SA FR1 SA Idle mode CA/DC measurement for FR1 |  | Rel-16 |  | C031a |  | UEs supporting 5GS NR SA FR1 and CA (2DL CA) and NR SSB measurements in RRC_IDLE/RRC_INACTIVE |  |  |  |
| 6.6.15 |  | Idle Mode inter-RAT CA/DC Measurements |  |  |  |  |  |  |  |  |  |
| 6.6.15.1 |  | NR SA FR1 Idle Mode measurements of inter-RAT CA candidate cells for early reporting |  | Rel-16 |  | C031b |  | UEs supporting NE-DC FR1 and E-UTRA measurements in RRC_IDLE/RRC_INACTIVE |  |  |  |
| 6.7 |  | Measurement performance requirements |  |  |  |  |  |  |  |  |  |
| 6.7.1 |  | SS-RSRP |  |  |  |  |  |  |  |  |  |
| 6.7.1.1 |  | Intra-frequency measurements |  |  |  |  |  |  |  |  |  |
| 6.7.1.1.1 |  | NR SA FR1 SS-RSRP absolute measurement accuracy |  | Rel-15 |  | C001 |  | UEs supporting 5GS NR SA FR1 |  |  | 2Rx 4Rx |
| 6.7.1.1.2 |  | NR SA FR1 SS-RSRP relative measurement accuracy |  | Rel-15 |  | C001 |  | UEs supporting 5GS NR SA FR1 |  |  | 2Rx 4Rx |
| 6.7.1.2 |  | Inter-frequency measurements |  |  |  |  |  |  |  |  |  |
| 6.7.1.2.1 |  | NR SA FR1-FR1 SS-RSRP absolute measurement accuracy |  | Rel-15 |  | C001 |  | UEs supporting 5GS NR SA FR1 |  |  | 2Rx 4Rx |
| 6.7.1.2.2 |  | NR SA FR1-FR1 SS-RSRP relative measurement accuracy |  | Rel-15 |  | C001 |  | UEs supporting 5GS NR SA FR1 |  |  | 2Rx 4Rx |
| 6.7.2 |  | SS-RSRQ |  |  |  |  |  |  |  |  |  |
| 6.7.2.1 |  | NR SA FR1 SS-RSRQ measurement accuracy |  | Rel-15 |  | C001 |  | UEs supporting 5GS NR SA FR1 |  |  | 2Rx 4Rx |
| 6.7.2.2.1 |  | NR SA FR1-FR1 SS-RSRQ absolute measurement accuracy |  | Rel-15 |  | C001 |  | UEs supporting 5GS NR SA FR1 |  |  | 2Rx 4Rx |
| 6.7.2.2.2 |  | NR SA FR1-FR1 SS-RSRQ relative measurement accuracy |  | Rel-15 |  | C001 |  | UEs supporting 5GS NR SA FR1 |  |  | 2Rx 4Rx |
| 6.7.3 |  | SS-SINR |  |  |  |  |  |  |  |  |  |
| 6.7.3.1 |  | NR SA FR1 SS-SINR measurement accuracy |  | Rel-15 |  | C034 |  | UEs supporting 5GS NR SA FR1 and SS-SINR-meas |  |  | 2Rx 4Rx |
| 6.7.3.2.1 |  | NR SA FR1-FR1 SS-SINR absolute measurement accuracy |  | Rel-15 |  | C034 |  | UEs supporting 5GS NR SA FR1 and SS-SINR-meas |  |  | 2Rx 4Rx |
| 6.7.3.2.2 |  | NR SA FR1-FR1 SS-SINR relative measurement accuracy |  | Rel-15 |  | C034 |  | UEs supporting 5GS NR SA FR1 and SS-SINR-meas |  |  | 2Rx 4Rx |
| 6.7.4 |  | L1-RSRP for beam reporting |  |  |  |  |  |  |  |  |  |
| 6.7.4.1.1 |  | NR SA FR1 SSB-based L1-RSRP absolute measurement accuracy |  | Rel-15 |  | C001 |  | UEs supporting 5GS NR SA FR1 |  |  | 2Rx 4Rx |
| 6.7.4.1.2 |  | NR SA FR1 SSB-based L1-RSRP relative measurement accuracy |  | Rel-15 |  | C001 |  | UEs supporting 5GS NR SA FR1 |  |  | 2Rx 4Rx |
| 6.7.4.2.1 |  | NR SA FR1 CSI-RS-based L1-RSRP absolute measurement accuracy |  | Rel-15 |  | C001 |  | UEs supporting 5GS NR SA FR1 |  |  | 2Rx 4Rx |
| 6.7.4.2.2 |  | NR SA FR1 CSI-RS-based L1-RSRP relative measurement accuracy |  | Rel-15 |  | C001 |  | UEs supporting 5GS NR SA FR1 |  |  | 2Rx 4Rx |
| 6.7.5 |  | E-UTRAN RSRP |  |  |  |  |  |  |  |  |  |
| 6.7.5.1 |  | NR SA FR1 – E-UTRAN RSRP absolute measurement accuracy |  | Rel-15 |  | C001 |  | UEs supporting 5GS NR SA FR1 |  |  | 2Rx 4Rx |
| 6.7.6 |  | E-UTRAN RSRQ |  |  |  |  |  |  |  |  |  |
| 6.7.6.1 |  | NR SA FR1 – E-UTRAN RSRQ absolute measurement accuracy |  | Rel-15 |  | C001 |  | UEs supporting 5GS NR SA FR1 |  |  | 2Rx 4Rx |
| 6.7.7 |  | E-UTRAN RS-SINR |  |  |  |  |  |  |  |  |  |
| 6.7.7.1 |  | NR SA FR1 – E-UTRAN RS-SINR absolute measurement accuracy |  | Rel-15 |  | C168 |  | UEs supporting 5GS NR SA FR1 and E-UTRA RS-SINR measurements |  |  | 2Rx 4Rx |
| 6.7.9 |  | L1-SINR |  |  |  |  |  |  |  |  |  |
| 6.7.9.1.1 |  | NR SA FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR absolute measurement accuracy |  | Rel-16 |  | C132 |  | UEs supporting 5GS NR SA FR1 and L1-SINR-measurement based on CSI-RS as CMR without dedicated IMR configured |  |  | 2Rx 4Rx |
| 6.7.9.1.2 |  | NR SA FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR relative measurement accuracy |  | Rel-16 |  | C132 |  | UEs supporting 5GS NR SA FR1 and L1-SINR-measurement based on CSI-RS as CMR without dedicated IMR configured |  |  | 2Rx 4Rx |
| 6.7.9.2 |  | NR SA FR1 SSB based CMR and dedicated IMR L1-SINR absolute measurement accuracy |  | Rel-16 |  | C133 |  | UEs supporting 5GS NR SA FR1 and L1-SINR-measurement based on SSB as CMR and dedicated CSI-IM as IMR |  |  | 2Rx 4Rx |
| 6.7.9.3.1 |  | NR SA FR1 CSI-RS based CMR and dedicated IMR L1-SINR absolute measurement accuracy |  | Rel-16 |  | C134 |  | UEs supporting 5GS NR SA FR1 and L1-SINR-measurement based on CSI-RS as CMR and dedicated CSI-IM as IMR |  |  | 2Rx 4Rx |
| 6.7.9.3.2 |  | NR SA FR1 CSI-RS based CMR and dedicated IMR L1-SINR relative measurement accuracy |  | Rel-16 |  | C134 |  | UEs supporting 5GS NR SA FR1 and L1-SINR-measurement based on CSI-RS as CMR and dedicated CSI-IM as IMR |  |  |  | 2Rx 4Rx |
| NOTE 1: The test case/branch is incomplete for any Band/CA/DC Configuration but has basic test configurations already. NOTE 1 can be removed only when the test case/branch is complete for at least one Band / CA/DC Configuration for at least one feature included in the test case/branch. Detailed completion status can be found in the corresponding test case section in 38.533. NOTE 2: Test X refers to the corresponding Sub-Test as defined in TS 38.533 [5]. |  |  |  |  |  |  |  |  |  |  |  |

Table 4.2-3a: Void

Table 4.2-4: Applicability of RRM NR SA FR2 conformance test cases, ref. TS 38.533 [5]

| Clause | TC Title | Release | Applicability |  | Additional Information | Branch |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Condition | Comment |  |  |
| 7.1 | RRC_IDLE state mobility |  |  |  |  |  |
| 7.1.1 | NR cell re-selection |  |  |  |  |  |
| 7.1.1.1 | NR SA FR2 cell re-selection | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 7.1.1.2 | NR SA FR2-FR2 cell re-selection | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 7.1.1.3 | NR SA FR2 cell re-selection for UE fulfilling low mobility relaxed measurement criterion | Rel-16 | C095 | UEs supporting 5GS NR SA FR2 and relaxed RRM measurement |  | 2Rx |
| 7.1.1.4 | NR SA FR2 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion | Rel-16 | C095 | UEs supporting 5GS NR SA FR2 and relaxed RRM measurement |  | 2Rx |
| 7.1.1.5 | NR SA FR2-FR2 cell re-selection for UE fulfilling low mobility relaxed measurement criterion | Rel-16 | C095 | UEs supporting 5GS NR SA FR2 and relaxed RRM measurement |  | 2Rx |
| 7.1.1.6 | NR SA FR2-FR2 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion | Rel-16 | C095 | UEs supporting 5GS NR SA FR2 and relaxed RRM measurement |  | 2Rx |
| 7.2 | RRC_INACTIVE state mobility |  |  |  |  |  |
| 7.3 | RRC_CONNECTED state mobility |  |  |  |  |  |
| 7.3.1 | Handover |  |  |  |  |  |
| 7.3.1.4 | NR SA FR1-FR2 synchronous DAPS handover | Rel-16 | C103 | UEs supporting 5GS NR SA FR1 and 5GS NR SA FR2 and inter-frequency DAPS handover and supporting different SCSs in source PCell and inter-frequency target PCell | NOTE 1 | 2Rx |
| 7.3.1.5 | NR SA FR1-FR2 asynchronous DAPS handover | Rel-16 | C104 | UEs supporting 5GS NR SA FR1 and 5GS NR SA FR2 and inter-frequency async DAPS handover and supporting different SCSs in source PCell and inter-frequency target PCell | NOTE 1 | 2Rx |
| 7.3.2 | RRC connection mobility control |  |  |  |  |  |
| 7.3.2.1 | RRC re-establishment |  |  |  |  |  |
| 7.3.2.1.1 | NR SA FR2 RRC re-establishment | Rel-15 | C006 | UEs supporting 5GS NR SA FR2 |  | 2Rx |
| 7.3.2.1.2 | NR SA FR2 - FR2 RRC re-establishment | Rel-15 | C006 | UEs supporting 5GS NR SA FR2 |  | 2Rx |
| 7.3.2.1.3 | NR SA FR2 RRC re-establishment without serving cell timing | Rel-15 | C006 | UEs supporting 5GS NR SA FR2 | NOTE 1 | 2Rx |
| 7.3.2.2 | Random access |  |  |  |  |  |
| 7.3.2.2.1 | NR SA FR2 contention based random access | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 7.3.2.2.2 | NR SA FR2 non-contention based random access | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 7.3.2.2.4 | NR SA FR2 2-step non-contention based random access | Rel-16 | C160 | UEs supporting 5GS NR SA FR2 and 2-step RACH | NOTE 1 | 2Rx |
| 7.3.2.3 | RRC connection release with redirection |  |  |  |  |  |
| 7.3.2.3.1 | NR SA FR2 RRC connection release with redirection | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 7.3.3 | Conditional Handover |  |  |  |  |  |
| 7.3.3.1 | NR SA FR2 conditional handover | Rel-16 | C106 | UEs supporting 5GS NR SA FR2 and conditional handover | NOTE 1 | 2Rx |
| 7.3.3.2 | NR SA FR2-FR2 conditional handover | Rel-16 | C106 | UEs supporting 5GS NR SA FR2 and conditional handover | NOTE 1 | 2Rx |
| 7.4 | Timing |  |  |  |  |  |
| 7.4.1 | UE transmit timing |  |  |  |  |  |
| 7.4.1.1 | NR SA FR2 UE transmit timing accuracy | Rel-15 | C006 | UEs supporting 5GS NR SA FR2 |  | 2Rx |
| 7.4.2 | UE timer accuracy |  |  |  |  |  |
| 7.4.3 | Timing advance |  |  |  |  |  |
| 7.4.3.1 | NR SA FR2 timing advance adjustment accuracy | Rel-15 | C006 | UEs supporting 5GS NR SA FR2 |  | 2Rx |
| 7.5 | Signalling characteristics |  |  |  |  |  |
| 7.5.1 | Radio Link Monitoring |  |  |  |  |  |
| 7.5.1.9 | NR SA FR2 radio link monitoring UE scheduling restrictions | Rel-15 | C006 | UEs supporting 5GS NR SA FR2 | NOTE 1 | 2Rx |
| 7.5.2 | Interruption |  |  |  |  |  |
| 7.5.2.1 | NR SA FR2 interruptions during measurements on deactivated NR SCC | Rel-15 | C006b | UEs supporting 5GS NR SA FR2 and 2DL CA in NR |  | 2Rx |
| 7.5.3 | Scell activation and deactivation delay |  |  |  |  |  |
| 7.5.3.1 | NR SA FR2-FR2 intra-band SCell activation and deactivation delay | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 7.5.3.2 | NR SA FR1-FR2 inter-band SCell activation and deactivation delay | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 7.5.4 | UE UL carrier RRC reconfiguration delay |  |  |  |  |  |
| 7.5.5 | Beam failure detection and link recovery procedures |  |  |  |  |  |
| 7.5.5.1 | NR SA FR2 SSB-based beam failure detection and link recovery in non-DRX | Rel-15 | C006 | UEs supporting 5GS NR SA FR2 | NOTE 1 | 2Rx |
| 7.5.5.2 | NR SA FR2 SSB-based beam failure detection and link recovery in DRX | Rel-15 | C006a | UEs supporting 5GS NR SA FR2 and long DRX cycle | NOTE 1 | 2Rx |
| 7.5.5.3 | NR SA FR2 CSI-RS-based beam failure detection and link recovery in non-DRX | Rel-15 | C164 | UEs supporting 5GS NR SA FR2 and CSI-RS based RLM | NOTE 1 | 2Rx |
| 7.5.5.4 | NR SA FR2 CSI-RS-based beam failure detection and link recovery in DRX | Rel-15 | C165 | UEs supporting 5GS NR SA FR2, long DRX cycle and CSI-RS based RLM | NOTE 1 | 2Rx |
| 7.5.5.5 | NR SA FR2 scheduling availability restriction during SSB-based beam failure detection and link recovery in non-DRX | Rel-15 | C006 | UEs supporting 5GS NR SA FR2 | NOTE 1 | 2Rx |
| 7.5.5.6 | NR SA FR2 Scell CSI-RS-based beam failure detection and link recovery in non-DRX | Rel-16 | C147 | UEs supporting 5GS NR SA FR2 and CSI-RS based BFR on SCell |  | 2Rx |
| 7.5.5.7 | NR SA FR2 Scell CSI-RS-based beam failure detection and link recovery in DRX | Rel-16 | C148 | UEs supporting 5GS NR SA FR2 and long DRX cycle and CSI-RS based BFR on SCell |  | 2Rx |
| 7.5.6 | Active BWP switch delay |  |  |  |  |  |
| 7.5.6.1 | Intra-frequency measurements |  |  |  |  |  |
| 7.5.6.1.1 | NR SA FR2 2DL CA DCI-based DL active BWP switch in non-DRX | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 7.5.6.1.2 | NR SA FR1-FR2 DCI-based DL active BWP switch in non-DRX | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 7.5.6.1.3 | NR SA FR2 DCI-based DL active BWP switch in non-DRX | Rel-15 | C006 | UEs supporting 5GS NR SA FR2 | NOTE 1 | 2Rx |
| 7.5.6.2 | RRC-based active BWP switch |  |  |  |  |  |
| 7.5.6.2.1 | NR SA FR2 RRC-based DL active BWP switch in non-DRX | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 7.5.7 | PSCell addition and release delay |  |  |  |  |  |
| 7.5.7.1 | NR SA FR2 addition and release delay of known PSCell | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 7.5.7.2 | NR SA FR2 addition and release delay of unknown PSCell | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 7.6 | Measurement procedures |  |  |  |  |  |
| 7.6.1 | Intra-frequency measurements |  |  |  |  |  |
| 7.6.1.1 | NR SA FR2 event-triggered reporting without gap in non-DRX | Rel-15 | C006 | UEs supporting 5GS NR SA FR2 | NOTE 1 | 2Rx |
| 7.6.1.2 | NR SA FR2 event-triggered reporting without gap in DRX | Rel-15 | C006a | UEs supporting 5GS NR SA FR2 and long DRX cycle | NOTE 1 | 2Rx |
| 7.6.1.3 | NR SA FR2 event-triggered reporting with gap in non-DRX | Rel-15 | C166 | UEs supporting 5GS NR SA FR2, CSI-RS-based RLM and BWP operation without bandwidth restriction | NOTE 1 | 2Rx |
| 7.6.1.4 | NR SA FR2 event-triggered reporting with gap in DRX | Rel-15 | C167 | UEs supporting 5GS NR SA FR2 long DRX cycle, CSI-RS-based RLM and BWP operation without bandwidth restriction | NOTE 1 | 2Rx |
| 7.6.2 | Inter-frequency measurements |  |  |  |  |  |
| 7.6.2.1 | NR SA FR2-FR2 event-triggered reporting in non-DRX | Rel-15 | C006 | UEs supporting 5GS NR SA FR2 |  | 2Rx |
| 7.6.2.2 | NR SA FR2-FR2 event-triggered reporting in DRX | Rel-15 | C006a | UEs supporting 5GS NR SA FR2 and long DRX cycle |  | 2Rx |
| 7.6.2.3 | NR SA FR2-FR2 event-triggered reporting in non-DRX with SSB time index detection | Rel-15 | C006 | UEs supporting 5GS NR SA FR2 |  | 2Rx |
| 7.6.2.4 | NR SA FR2-FR2 event-triggered reporting in DRX with SSB time index detection | Rel-15 | C006a | UEs supporting 5GS NR SA FR2 and long DRX cycle |  | 2Rx |
| 7.6.2.5 | NR SA FR1-FR2 event-triggered reporting in non-DRX | Rel-15 | N/A | not recommended due to FR1 – FR2 testability issue |  | 2Rx |
| 7.6.2.6 | NR SA FR1-FR2 event-triggered reporting in DRX | Rel-15 | N/A | not recommended due to FR1 – FR2 testability issue |  | 2Rx |
| 7.6.2.7 | NR SA FR1-FR2 event-triggered reporting in non-DRX with SSB time index detection | Rel-15 | N/A | not recommended due to FR1 – FR2 testability issue |  | 2Rx |
| 7.6.2.8 | NR SA FR1-FR2 event-triggered reporting in DRX with SSB time index detection | Rel-15 | N/A | not recommended due to FR1 – FR2 testability issue |  | 2Rx |
| 7.6.3 | L1-RSRP for beam reporting |  |  |  |  |  |
| 7.6.3.1 | NR SA FR2 SSB-based L1-RSRP measurement in non-DRX | Rel-15 | C006 | UEs supporting 5GS NR SA FR2 |  | 2Rx |
| 7.6.3.2 | NR SA FR2 SSB-based L1-RSRP measurement in DRX | Rel-15 | C006a | UEs supporting 5GS NR SA FR2 and long DRX cycle |  | 2Rx |
| 7.6.3.3 | NR SA FR2 CSI-RS-based L1-RSRP measurement in non-DRX | Rel-15 | C006 | UEs supporting 5GS NR SA FR2 |  | 2Rx |
| 7.6.3.4 | NR SA FR2 CSI-RS-based L1-RSRP measurement in DRX | Rel-15 | C006a | UEs supporting 5GS NR SA FR2 and long DRX cycle |  | 2Rx |
| 7.6.6 | L1-SINR measurement for beam reporting |  |  |  |  |  |
| 7.6.6.1 | NR SA FR2 CSI-RS based CMR and no dedicated IMR L1-SINR measurement in non-DRX | Rel-16 | C144a | UEs supporting 5GS NR SA FR2 and L1-SINR measurement based on CSI-RS as CMR without dedicated IMR configured |  | 2Rx 4Rx |
| 7.6.6.2 | NR SA FR2 SSB based CMR and dedicated IMR L1-SINR measurement in DRX | Rel-16 | C145a | UEs supporting 5GS NR SA FR2 and long DRX cycle and L1-SINR measurement based on SSB as CMR and dedicated CSI-IM as IMR |  | 2Rx 4Rx |
| 7.6.6.3 | NR SA FR2 CSI-RS based CMR and dedicated IMR L1-SINR measurement in DRX | Rel-16 | C146a | UEs supporting 5GS NR SA FR2 and long DRX cycle and L1-SINR measurement based on CSI-RS as CMR and dedicated CSI-RS as IMR |  | 2Rx 4Rx |
| 7.7 | Measurement performance requirements |  |  |  |  |  |
| 7.7.1 | SS-RSRP |  |  |  |  |  |
| 7.7.1.1 | NR SA FR2 SS-RSRP measurement accuracy | Rel-15 | C006 | UEs supporting 5GS NR SA FR2 |  | 2Rx |
| 7.7.1.2 | NR SA FR2-FR2 SS-RSRP measurement accuracy | Rel-15 | C006 | UEs supporting 5GS NR SA FR2 |  | 2Rx |
| 7.7.1.3 | Inter-frequency measurements between FR1 and FR2 |  |  |  |  |  |
| 7.7.1.3.1 | NR SA FR1-FR2 SS-RSRP measurement accuracy | Rel-15 | C006 | UEs supporting 5GS NR SA FR2 |  | 2Rx |
| 7.7.2 | SS-RSRQ |  |  |  |  |  |
| 7.7.2.1 | NR SA FR2 SS-RSRQ measurement accuracy | Rel-15 | C006 | UEs supporting 5GS NR SA FR2 |  | 2Rx |
| 7.7.2.2 | NR SA FR2-FR2 SS-RSRQ measurement accuracy | Rel-15 | C006 | UEs supporting 5GS NR SA FR2 |  | 2Rx |
| 7.7.3 | SS-SINR |  |  |  |  |  |
| 7.7.3.1 | NR SA FR2 SS-SINR measurement accuracy | Rel-15 | C006 | UEs supporting 5GS NR SA FR2 |  | 2Rx |
| 7.7.3.2 | NR SA FR2-FR2 SS-SINR measurement accuracy | Rel-16 | C006 | UEs supporting 5GS NR SA FR2 |  | 2Rx |
| 7.7.4 | L1-RSRP for beam reporting |  |  |  |  |  |
| 7.7.4.1 | NR SA FR2 SSB based L1-RSRP measurement accuracy | Rel-15 | C006 | UEs supporting 5GS NR SA FR2 |  | 2Rx |
| 7.7.4.2 | NR SA FR2 CSI-RS based L1-RSRP measurement accuracy | Rel-15 | C006 | UEs supporting 5GS NR SA FR2 |  | 2Rx |
| 7.7.6 | L1-SINR |  |  |  |  |  |
| 7.7.6.1 | NR SA FR2 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR measurement accuracy | Rel-16 | C138 | UEs supporting 5GS NR SA FR2 and L1-SINR-measurement based on CSI-RS as CMR without dedicated IMR configured |  | 2Rx |
| 7.7.6.2 | NR SA FR2 SSB based CMR and dedicated IMR L1-SINR measurement accuracy | Rel-16 | C139 | UEs supporting 5GS NR SA FR2 and L1-SINR-measurement based on SSB as CMR and dedicated CSI-IM as IMR |  | 2Rx |
| 7.7.6.3 | NR SA FR2 CSI-RS based CMR and dedicated IMR L1-SINR measurement accuracy | Rel-16 | C140 | UEs supporting 5GS NR SA FR2 and L1-SINR-measurement based on CSI-RS as CMR and dedicated CSI-IM as IMR |  | 2Rx |
| NOTE 1: The test case/branch is incomplete for any Band/CA/DC Configuration but has basic test configurations already. NOTE 1 can be removed only when the test case/branch is complete for at least one Band / CA/DC Configuration for at least one feature included in the test case/branch. Detailed completion status can be found in the corresponding test case section in 38.533. NOTE 2: Void. NOTE 3: Void. |  |  |  |  |  |  |

Table 4.2-4a: Void

Table 4.2-5: Applicability of E-UTRA – NR Inter-RAT conformance test cases, ref. TS 38.533 [5]

| Clause | TC Title | Release | Applicability |  | Additional Information | Branch |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Condition | Comment |  |  |
| 8.2 | RRC_IDLE state mobility |  |  |  |  |  |
| 8.2.1 | Inter-RAT cell re-selection |  |  |  |  |  |
| 8.2.1.1 | E-UTRA – NR FR1 cell re-selection to higher priority NR target cell | Rel-15 | C025 | UEs supporting 5GS NR SA FR1 and E-UTRA |  | 2Rx 4Rx |
| 8.2.1.2 | E-UTRA – NR FR1 Cell reselection to lower priority NR target Cell in FR1 for UE configured with highSpeedInterRAT-NR-r16 | Rel-15 | C025d | UEs supporting 5GS NR SA FR1 and E-UTRAN and NR inter-RAT measurement enhancement in HST |  | 2Rx 4Rx |
| 8.2.2 | E-UTRA - NR Inter-RAT Early Measurement Reporting |  |  |  |  |  |
| 8.2.2.1 | E-UTRA – NR Early Measurement Reporting for NR in FR1 | Rel-16 | C025f | UEs supporting 5GS NR SA FR1 and E-UTRA and NR FR1 SSB measurements in RRC_IDLE/RRC_INACTIVE | NOTE 1 | 2Rx 4Rx |
| 8.2.2.2 | E-UTRA – NR Early Measurement Reporting for NR in FR2 | Rel-16 | N/A | not recommended due to E-UTRA/FR1 – FR2 testability issue | NOTE 1 | 2Rx 4Rx |
| 8.3 | RRC_CONNECTED state mobility |  |  |  |  |  |
| 8.3.1 | Inter-RAT cell handover |  |  |  |  |  |
| 8.3.1.1 | E-UTRA – NR FR1 handover with known target cell | Rel-15 | C025 | UEs supporting 5GS NR SA FR1 and E-UTRA |  | 2Rx 4Rx |
| 8.4 | Measurement procedures |  |  |  |  |  |
| 8.4.1 | SFTD measurement delay |  |  |  |  |  |
| 8.4.1.1 | E-UTRA – NR FR1 SFTD measurement delay in non-DRX | Rel-15 | C081 | UEs supporting EN-DC and E-UTRA and SFTD measurements between E-UTRA PCell and NR neighbour cell |  | 2Rx 4Rx |
| 8.4.1.2 | E-UTRA – NR FR1 SFTD measurement delay in DRX | Rel-15 | C081a | UEs supporting EN-DC and E-UTRA and long DRX cycle and SFTD measurements between E-UTRA PCell and NR neighbour cell |  | 2Rx 4Rx |
| 8.4.2 | Inter-RAT measurements |  |  |  |  |  |
| 8.4.2.1 | E-UTRA event-triggered reporting of a NR FR1 neighbour cell without SSB time index detection in non-DRX | Rel-15 | C086 | UEs supporting E-UTRA and NR FR1 measurement |  | 2Rx 4Rx |
| 8.4.2.2 | E-UTRA event-triggered reporting of a NR FR1 neighbour cell without SSB time index detection in DRX | Rel-15 | C086a | UEs supporting E-UTRA and NR FR1 measurement and long DRX cycle |  | 2Rx 4Rx |
| 8.4.2.3 | E-UTRA event-triggered reporting of a NR FR1 neighbour cell with SSB time index detection in non-DRX | Rel-15 | C086 | UEs supporting E-UTRA and NR FR1 measurement |  | 2Rx 4Rx |
| 8.4.2.4 | E-UTRA event-triggered reporting of a NR FR1 neighbour cell with SSB time index detection in DRX | Rel-15 | C086a | UEs supporting E-UTRA and NR FR1 measurement and long DRX cycle |  | 2Rx 4Rx |
| 8.4.2.5 | E-UTRA event-triggered reporting of a NR FR2 neighbour cell without SSB time index detection in non-DRX | Rel-15 | N/A | not recommended due to E-UTRA – FR2 testability issue | NOTE 1 | 2Rx |
| 8.4.2.6 | E-UTRA event-triggered reporting of a NR FR2 neighbour cell without SSB time index detection in DRX | Rel-15 | N/A | not recommended due to E-UTRA – FR2 testability issue | NOTE 1 | 2Rx |
| 8.4.2.7 | E-UTRA event-triggered reporting of a NR FR2 neighbour cell with SSB time index detection in non-DRX | Rel-15 | N/A | not recommended due to E-UTRA – FR2 testability issue | NOTE 1 | 2Rx |
| 8.4.2.8 | E-UTRA event-triggered reporting of a NR FR2 neighbour cell with SSB time index detection in DRX | Rel-15 | N/A | not recommended due to E-UTRA – FR2 testability issue | NOTE 1 | 2Rx |
| 8.4.2.9 | E-UTRA – NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection in DRX for UE configured with highSpeedInterRAT-NR-r16 | Rel-15 | C025e | UEs supporting 5GS NR SA FR1 and E-UTRAN, long DRX cycle and NR inter-RAT measurement enhancement in HST |  | 2Rx 4Rx |
| 8.5 | Measurement performance requirements |  |  |  |  |  |
| 8.5.1 | SFTD measurement accuracy |  |  |  |  |  |
| 8.5.1.1 | E-UTRA – NR FR1 SFTD measurement accuracy | Rel-15 | C081 | UEs supporting EN-DC and E-UTRA and SFTD measurements between E-UTRA PCell and NR neighbour cell | NOTE 1 | 2Rx 4Rx |
| 8.5.2 | Inter-RAT |  |  |  |  |  |
| 8.5.2.1 | SS-RSRP |  |  |  |  |  |
| 8.5.2.1.1.1 | E-UTRA SS-RSRP absolute measurement accuracy of a NR FR1 neighbour cell | Rel-15 | C086 | UEs supporting E-UTRA and NR FR1 measurement |  | 2Rx 4Rx |
| 8.5.2.1.2 | E-UTRA SS-RSRP measurement accuracy of a NR FR2 neighbour cell | Rel-15 | C080 | UEs supporting E-UTRA and NR FR2 measurement |  | 2Rx |
| 8.5.2.2 | SS-RSRQ |  |  |  |  |  |
| 8.5.2.2.1 | E-UTRA SS-RSRQ measurement accuracy of a NR FR1 neighbour cell | Rel-15 | C086 | UEs supporting E-UTRA and NR FR1 measurement |  | 2Rx 4Rx |
| 8.5.2.2.2 | E-UTRA SS-RSRQ measurement accuracy of a NR FR2 neighbour cell | Rel-15 | C080 | UEs supporting E-UTRA and NR FR2 measurement |  | 2Rx |
| 8.5.2.3 | SS-SINR |  |  |  |  |  |
| 8.5.2.3.1 | E-UTRA SS-SINR measurement accuracy of a NR FR1 neighbour cell | Rel-15 | C086 | UEs supporting E-UTRA and NR FR1 measurement |  | 2Rx 4Rx |
| 8.5.2.3.2 | E-UTRA SS-SINR measurement accuracy of a NR FR2 neighbour cell | Rel-15 | C080 | UEs supporting E-UTRA and NR FR2 measurement |  | 2Rx |
| NOTE 1: The test case/branch is incomplete for any Band/CA/DC Configuration but has basic test configurations already. NOTE 1 can be removed only when the test case/branch is complete for at least one Band / CA/DC Configuration for at least one feature included in the test case/branch. Detailed completion status can be found in the corresponding test case section in 38.533. |  |  |  |  |  |  |

///End of changes