3GPP TSG-RAN5 Meeting #98	R5-231122r1

Athens, February 27 - March 3 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 2297 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of NR SA FR1 Cell reselection RedCap test case 16.1.1.8 including Test Tolerance |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2023-02-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | RRM RedCap test case 16.1.1.8 is incomplete in TS 38.533 |  |  |  |  |  |  |  |  |
| Summary of change: |  | RRM RedCap test case 16.1.1.8 has been updated. Following has beed added/corrected: Minimum conformance requirements has been added, Cell mapping updated, Test procedure added, Message content added.TT added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RRM RedCap test case 16.1.1.8 will remain incomplete in TS 38.533 and WI cannot be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.1.1.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | RAN4 dependent R1: Typo removed (leftover TBD in the connection diagram). |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

///Start of changes

#### 16.1.1.8	NR SA FR1-FR1 Cell reselection for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE

16.1.1.8.1	Test purpose

The purpose of this test is to verify the requirement for the inter frequency NR cell reselection requirements for UE fulfilling stationary relaxed measurement criterion. The requirements are specified in TS 38.133 [6] in clause 4.2B.2.10.2.

16.1.1.8.2	Test applicability

This test applies to all types of NR RedCap UE with 2Rx from release 17 and forward fulfilling stationary relaxed measurement criterion.

16.1.1.8.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 16.1.1.0.4.

The normative reference for this requirement is TS 38.133 [6] clause A.16.1.1.8.

16.1.1.8.4	Test description

16.1.1.8.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 16.1.1.8.4.1-1.

Table 16.1.1.8.4.1-1: Supported test configurations for NR SA FR1-FR1 Cell reselection for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE

| Configuration | Description of serving cell | Description of target cell |
| --- | --- | --- |
| 16.1.1.8.1-1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.1.1.8.1-2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.1.1.8.1-3 | 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode | 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.1.1.8.1-4 | 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode | 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |  |

Configure the test equipment and the DUT according to the parameters in Table 16.1.1.8.4.1-2.

Table 16.1.1.8.4.1-2: Initial conditions for NR SA FR1-FR1 Cell reselection for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.1.1.8.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | Without faders. Without LTE link. |  |  |

1.	The general test parameter settings are set up according to Table 16.1.1.8.4.1-3.

2.	Message contents are defined in clause 16.1.1.8.4.3.

3.	There is 1 NR carrier, and 2 cells specified in the test. Both cell 1 and cell 2 are already identified by the UE prior to the start of the test. Cell 1 and cell 2 belong to different tracking areas. Furthermore, UE has not registered with network for the tracking area containing cell 2.

Table 16.1.1.8.4.1-3: General test parameters for NR SA FR1-FR1 Cell reselection for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2, 3, 4 | Cell2 | The UE camps on cell 2 in the initial phase, it fulfills stationary relaxation measurements criterion, and during T1 period the UE reselects to cell 1 |
|  | Neighbour cells |  | 1, 2, 3, 4 | Cell1 |  |
| T1 end condition | Active cell |  | 1, 2, 3, 4 | Cell1 | The UE shall perform reselection to cell 1 during T1 |
|  | Neighbour cells |  | 1, 2, 3, 4 | Cell2 |  |
| T2 end condition | Active cell |  | 1, 2, 3, 4 | Cell2 | The UE shall perform reselection to cell 2 with higher priority during T2 |
|  | Neighbour cells |  | 1, 2, 3, 4 | Cell1 |  |
| RF Channel Number |  |  | 1, 2, 3, 4 | 1, 2 |  |
| Time offset between cells |  |  | 1, 4 | 3 ms | Asynchronous cells |
|  |  |  | 2 | 3 s | Synchronous cells |
|  |  |  | 3 | 3 s | Synchronous cells |
| Access Barring Information |  | - | 1, 2, 3, 4 | Not Sent | No additional delays in random access procedure. |
| SSB Configuration |  |  | 1, 4 | SSB.1 FR1 |  |
|  |  |  | 2 | SSB.1 FR1 |  |
|  |  |  | 3 | SSB.1 RedCap FR1 |  |
| SMTC configuration |  |  | 1, 4 | SMTC.2 | Configured in SIB4 of Cell 1 |
|  |  |  |  | SMTC.6 | Configured in SIB4 of Cell 2 |
|  |  |  | 2 | SMTC.1 |  |
|  |  |  | 3 | SMTC.1 |  |
| DRX cycle length |  | s | 1, 2, 3, 4 | 0.64 | The value shall be used for all cells in the test. |
| PRACH configuration index |  |  | 1, 2, 3, 4 | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| rangeToBestCell |  |  | 1, 2, 3, 4 | Not configured |  |
| T1 |  | s | 1, 2, 3, 4 | 32 | T1 is defined so that cell re-selection reaction time is taken into account. |
| T2 |  | s | 1, 2, 3, 4 | 32 | T2 is defined so that cell re-selection reaction time is taken into account. |

16.1.1.8.4.2	Test procedure

16.1.1.8.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 16.1.1.8.4.3-1: SIB1 (Cell 1 and Cell 2)

| Derivation Path: TS 38.508-1 [14], Table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |  |
| servingCellConfigCommon SEQUENCE { |  |  |  |  |
| downlinkConfigCommon SEQUENCE { |  |  |  |  |
| pcch-Config SEQUENCE { |  |  |  |  |
| defaultPagingCycle |  | rf64 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 16.1.1.8.4.3-2: SIB2 (Cell 1)

| Derivation Path: Table H.2.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB2 ::= SEQUENCE { |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| s-NonIntraSearchP | Not present |  |  |
| threshServingLowP | 22 | Actual value = 22*2 = 44dB |  |
| } |  |  |  |
| relaxedMeasurement-r16 SEQUENCE { |  |  |  |
| lowMobilityEvaluation-r16 SEQUENCE { |  |  |  |
| s-SearchDeltaP-r16 | dB3 |  |  |
| t-SearchDeltaP-r16 | s5 |  |  |
| } |  |  |  |
| cellEdgeEvaluation-r16 | Not present |  |  |
| combineRelaxedMeasCondition-r16 | Not present |  |  |
| highPriorityMeasRelax-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

Table 16.1.1.8.4.3-3: SIB2 (Cell 2)

| Derivation Path: Table H.2.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB2 ::= SEQUENCE { |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| s-NonIntraSearchP | Not present |  |  |
| threshServingLowP | 23 | Actual value = 23*2 = 46dB | Cell 2 |
| } |  |  |  |
| relaxedMeasurement-r16 SEQUENCE { |  |  |  |
| lowMobilityEvaluation-r16 SEQUENCE { |  |  |  |
| s-SearchDeltaP-r16 | dB15 |  | Cell 2 |
| t-SearchDeltaP-r16 | s5 |  |  |
| } |  |  |  |
| cellEdgeEvaluation-r16 | Not present |  |  |
| combineRelaxedMeasCondition-r16 | Not present |  |  |
| highPriorityMeasRelax-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

Table 16.1.1.8.4.3-4: SIB4 (Cell 1)

| Derivation Path: Table H.2.2-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB4 ::= SEQUENCE { |  |  |  |
| interFreqCarrierFreqList SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo { | 1 entry |  |  |
| InterFreqCarrierFreqInfo[1] SEQUENCE { |  | entry 1 |  |
| dl-CarrierFreq | ARFCN-ValueNR for Cell 2 |  |  |
| smtc | SSB-MTC specified in 38.508-1 [14] Table 7.3.1-3 with condition SMTC.2 |  | 16.1.1.7-1, 16.1.1.7-4 |
|  | SSB-MTC specified in 38.508-1 [14] Table 7.3.1-3 with condition SMTC.1 |  | 16.1.1.7-2, 16.1.1.7-3 |
| deriveSSB-IndexFromCell | false |  | 16.1.1.7-1, 16.1.1.7-4 |
|  | true |  | 16.1.1.7-2, 16.1.1.7-3 |
| threshX-HighP | 24 | Actual value = 24*2 = 48dB |  |
| threshX-LowP | 25 | Actual value = 25*2 = 50dB |  |
| cellReselectionPriority | 5 | Same as the priority in SIB 2 of Cell 2. |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 16.1.1.8.4.3-5: SIB4 (Cell 2)

| Derivation Path: Table H.2.2-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB4 ::= SEQUENCE { |  |  |  |
| interFreqCarrierFreqList SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo { | 1 entry |  |  |
| InterFreqCarrierFreqInfo[1] SEQUENCE { |  | entry 1 |  |
| dl-CarrierFreq | ARFCN-ValueNR for Cell 1 SSB |  | Cell 2 |
| smtc | SSB-MTC specified in 38.508-1 [14] Table 7.3.1-3 with condition SMTC.6 |  | 16.1.1.7-1, 16.1.1.7-4 |
|  | SSB-MTC specified in 38.508-1 [14] Table 7.3.1-3 with condition SMTC.1 |  | 16.1.1.7-2, 16.1.1.7-3 |
| deriveSSB-IndexFromCell | false |  | 16.1.1.7-1, 16.1.1.7-4 |
|  | true |  | 16.1.1.7-2, 16.1.1.7-3 |
| threshX-HighP | 22 | Actual value = 22*2 = 44dB | Cell 2 |
| threshX-LowP | 25 | Actual value = 25*2 = 50dB | Cell 2 |
| cellReselectionPriority | 4 | Same as the priority in SIB 2 of Cell 1. | Cell 2 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

16.1.1.8.5	Test requirement

Table 16.1.1.8.5-1 defines the primary level settings including test tolerances for FR1-FR1 Cell reselection for 2 Rx UE

Table 16.1.1.8.5-1: NR cell specific test parameters for NR SA FR1-FR1 Cell reselection for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 |  | T2 |
| TDD configuration |  | 1, 4 | N/A |  | N/A |  |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |  |
| PDSCH RMC |  | 1, 4 | SR.1.1 FDD |  | SR.1.1 FDD |  |  |
| configuration |  | 2 | SR.1.1 TDD |  | SR.1.1 TDD |  |  |
|  |  | 3 | SR.2.1 TDD |  | SR.2.1 TDD |  |  |
| RMSI CORESET |  | 1, 4 | CR.1.1 FDD |  | CR.1.1 FDD |  |  |
| RMC configuration |  | 2 | CR.1.1 TDD |  | CR.1.1 TDD |  |  |
|  |  | 3 | CR.2.1 TDD |  | CR.2.1 TDD |  |  |
| Dedicated CORESET |  | 1, 4 | CCR.1.1 FDD |  | CCR.1.1 FDD |  |  |
| RMC configuration |  | 2 | CCR.1.1 TDD |  | CCR.1.1 TDD |  |  |
|  |  | 3 | CCR.2.1 TDD |  | CCR.2.1 TDD |  |  |
| OCNG Pattern |  | 1, 2, 3, 4 | OP.1 defined in A.3.2.1 |  | OP.1 defined in A.3.2.1 |  |  |
| Initial DL BWP configuration |  | 1, 2, 3, 4 | DLBWP.0.1 |  | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | 1, 2, 3, 4 | ULBWP.0.1 |  | ULBWP.0.1 |  |  |
| RLM-RS |  | 1, 2, 3, 4 | SSB |  | SSB |  |  |
| Qrxlevmin | dBm/SCS | 1, 2, 4 | -140 |  | -140 |  |  |
|  |  | 3 | -137 |  | -137 |  |  |
| Pcompensation | dB | 1, 2, 3, 4 | 0 |  | 0 |  |  |
| Qhysts | dB | 1, 2, 3, 4 | 0 |  | 0 |  |  |
| Qoffsets, n | dB | 1, 2, 3, 4 | 0 |  | 0 |  |  |
| Cell_selection_and_ reselection_quality_measurement |  | 1, 2, 3, 4 | SS-RSRP |  | SS-RSRP |  |  |
|  | dB | 1, 4 | 14 | 14 | -3.7 | 9.75 |  |
|  |  | 2 |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |
| Note2 | dBm/SCS | 1, 4 | -98 |  |  |  |  |
|  |  | 2 | -98 |  |  |  |  |
|  |  | 3 | -95 |  |  |  |  |
| Note2 | dBm/15 kHz | 1, 4 | -98 |  |  |  |  |
|  |  | 2 |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |
|  | dB | 1, 4 | 14 | 14 | -3.70 |  | 9.75 |
|  |  | 2 |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |
| SS-RSRP Note3 | dBm/SCS | 1, 4 | -84 | -84 | -101.70 |  | -88.25 |
|  |  | 2 | -84 | -84 | -101.70 |  | -88.25 |
|  |  | 3 | -81 | -81 | -98.7 |  | -85.25 |
| Io | dBm/9.36 MHz | 1, 4 | -55.96 | -55.96 | -68.60 |  | -59.95 |
|  | dBm/9.36 MHz | 2 | -55.96 | -55.96 | -68.60 |  | -59.95 |
|  | dBm/18.36 MHz | 3 | -52.95 | -52.95 | -65.58 |  | -56.93 |
| Treselection | s | 1, 2, 3, 4 | 0 | 0 | 0 |  | 0 |
| SnonintrasearchP | dB | 1, 2, 3, 4 | Not sent |  | Not sent |  |  |
| Threshx, highP Note 4 | dB | 1, 2, 3, 4 | 48 |  | 44 |  |  |
| Threshserving, lowP Note 4 | dB | 1, 2, 3, 4 | 44 |  | 46 |  |  |
| Threshx, lowP Note 4 | dB | 1, 2, 3, 4 | 48 |  | 50 |  |  |
| SSearchDeltaP-Stationary Note 4 | dB | 1, 2, 3 | 3 |  | 15 |  |  |
| TSearchDeltaP-Stationary | s | 1, 2, 3 | 5 |  | 5 |  |  |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Thresholds include Test Tolerance applied to fulfil the cell re-selection criterion. |  |  |  |  |  |  |  |

During T1, the cell re-selection delay to a lower priority cell for UE fulfilling stationary relaxed measurements shall be less than 32 s.

The cell reselection delay to an already detected lower priority cell for UE fulfilling stationary relaxed measurements is defined as the time from the beginning of time period T1, to the moment when the UE camps on cell 1, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Tracking Area Update procedure on cell 1.

During T2, the cell re-selection delay to an already detected higher priority cell for UE fulfilling stationary relaxed measurements shall be less than 32 s.

The cell reselection delay to an already detected higher priority cell for UE fulfilling stationary relaxed measurements is defined as the time from the beginning of time period T2, to the moment when the UE camps on cell 2, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Tracking Area Update procedure on cell 2.

The rate of correct events observed during repeated tests shall be at least 90% with the confidence level of 95%.

///End of changes