3GPP TSG-RAN5 Meeting #98	R5-231205r1

# Athens, Greece, 27th Feb 2023 - 3rd Mar 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 3603 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR-U test case 8.1.8.1.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_unlic-UEConTest |  |  |  |  |  | Date: |  |  | 2023-02-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | NR-U test case 8.1.8.1.2 [“Measurement configuration control and reporting for Shared spectrum / RMTC / RSSI measurements / Channel Occupancy reporting / inter-frequency”] needs to be added as per NR-U WP. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added test case 8.1.8.1.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR unlicensed WP will remain incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.8.1.2 (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 38.523-2 CR 0328 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN Impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-231205r1: 1. Fixed typo in comment column of Table 8.1.8.1.2.3.3-1. |  |  |  |  |  |  |  |  |

<Start of modified section>

##### 8.1.8.1.2	Measurement configuration control and reporting for Shared spectrum / RMTC / RSSI measurements / Channel Occupancy reporting / inter-frequency

8.1.8.1.2.1	Test Purpose (TP)

(1)

with { UE in NR RRC_CONNECTED state and measurements configured for RSSI reporting of inter frequency carriers using measurement gaps configured as gapUE }

ensure that {

when { The UE receives signal power for carriers on the frequency where measurements are configured }

then { UE sends MeasurementReport message at regular intervals for these carriers }

}

(2)

with { UE in NR RRC_CONNECTED state and RSSI measurement reporting ongoing }

ensure that {

when { The UE receives a RRCReconfiguration message removing measIds for RSSI reporting }

then { UE stops sending MeasurementReport messages for these measIds }

}

8.1.8.1.2.2	Conformance requirements

Same as test case 8.1.8.1.1 with the following difference:

[TS 38.331, clause 5.5.2.9]

The UE shall:

…

1>	if gapUE is set to setup:

2>	if a per UE measurement gap configuration is already setup, release the per UE measurement gap configuration;

2>	setup the per UE measurement gap configuration indicated by the measGapConfig in accordance with the received gapOffset, i.e., the first subframe of each gap occurs at an SFN and subframe meeting the following condition:

SFN mod T = FLOOR(gapOffset/10);

subframe = gapOffset mod 10;

with T = MGRP/10 as defined in TS 38.133 [14];

2>	apply the specified timing advance mgta to the gap occurrences calculated above (i.e. the UE starts the measurement mgta ms before the gap subframe occurrences);

8.1.8.1.2.3	Test description

8.1.8.1.2.3.1	Pre-test conditions

Same as test case 8.1.8.1.1 with the following differences:

-	Cells configuration: NR Cell 3 replaces NR Cell 2.

-	System information combination: NR-4 replaces NR-2.

8.1.8.1.2.3.2	Test procedure sequence

Same as test case 8.1.8.1.1 with the following differences:

-	Cells configuration: NR Cell 3 replaces NR Cell 2.

8.1.8.1.2.3.3	Specific message contents

Same as test case 8.1.8.1.1 with the following difference:

-	Cells configuration: NR Cell 3 replaces NR Cell 2.

Table 8.1.8.1.2.3.3-1: MeasConfig (Table 8.1.8.1.1.3.3-1)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-69 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasObjectToAddMod { | 2 entries |  |  |
| MeasObjectToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measObjectId | 1 |  |  |
| measObject CHOICE { |  |  |  |
| measObjectNR SEQUENCE { |  |  |  |
| absThreshSS-BlocksConsolidation | Not present |  |  |
| nrofSS-BlocksToAverage | Not present |  |  |
| rmtc-Config-r16 SEQUENCE { |  |  |  |
| rmtc-Periodicity-r16 | ms160 |  |  |
| rmtc-SubframeOffset-r16 | 0 |  |  |
| measDurationSymbols-r16 | sym28or24 |  |  |
| rmtc-Frequency-r16 | Set to the ARFCN for NR Cell 1 |  |  |
| ref-SCS-CP-r16 | kHz30 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| MeasObjectToAddMod[1] SEQUENCE { |  | entry 2 |  |
| measObjectId | 2 |  |  |
| measObject CHOICE { |  |  |  |
| measObjectNR SEQUENCE { |  |  |  |
| absThreshSS-BlocksConsolidation | Not present |  |  |
| nrofSS-BlocksToAverage | Not present |  |  |
| rmtc-Config-r16 SEQUENCE { |  |  |  |
| rmtc-Periodicity-r16 | ms160 |  |  |
| rmtc-SubframeOffset-r16 | 0 |  |  |
| measDurationSymbols-r16 | sym28or24 |  |  |
| rmtc-Frequency-r16 | Set to the ARFCN for NR Cell 3 |  |  |
| ref-SCS-CP-r16 | kHz30 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE(SIZE (1..maxReportConfigId)) OF ReportConfigToAddMod { | 1 entry |  |  |
| ReportConfigToAddMod[1] SEQUENCE { |  | entry 1 |  |
| reportConfigId | 1 |  |  |
| reportConfig CHOICE { |  |  |  |
| reportConfigNR | ReportConfigNR-PERIODICAL | Table 8.1.8.1.1.3.3-3 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasIdToAddMod { | 1 entry |  |  |
| MeasIdToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measId | 1 |  |  |
| measObjectId | 2 |  |  |
| reportConfigId | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| measGapConfig | MeasGapConfig specified in 38.508-1 [4] Table 4.6.3-70 |  |  |
| } |  |  |  |

<End of modifed section>