3GPP TSG-RAN5 Meeting #98	R5-231206r1

# Athens, Greece, 27th Feb 2023 - 3rd Mar 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 3604 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR unlicensed test case 8.1.8.2.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_unlic-UEConTest |  |  |  |  |  | Date: |  |  | 2023-02-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | New NR-U test case 8.1.8.2.2 [“Paging monitoring / multiple PDCCH monitoring occasions / Short message indication / stopPagingMonitoring / RRC inactive”] needs to be added as per NR unlicensed WP. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added test case 8.1.8.2.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR-U WP will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.8.2.2 (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 38.523-2 CR 0328 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN Impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-231206r1: 1. Updated UE state in preamble from Idle to Inactive. |  |  |  |  |  |  |  |  |

<Start of modified section>

##### 8.1.8.2.2	Paging monitoring / multiple PDCCH monitoring occasions / Short message indication / stopPagingMonitoring / RRC inactive

8.1.8.2.2.1	Test Purpose (TP)

(1)

with { UE in NR RRC_INACTIVE state }

ensure that {

when { multiple PDCCH monitoring occasions are configured and UE receives a Paging message to resume RRC connection}

then { UE successfully resumes the RRC connection }

}

(2)

with { UE in NR RRC_INACTIVE state }

ensure that {

when { UE receives a short message transmitted on PDCCH using P-RNTI indicating a stopPagingMonitoring }

then { UE does not resume RRC connection }

}

8.1.8.2.2.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 38.331 clauses 5.3.2.1, 5.3.2.2, 5.3.2.3, 6.3.2, 6.5 and TS 38.304 clause 7.1. Unless otherwise stated these are Rel-16 requirements.

[TS 38.331, clause 5.3.2.1]

Figure 5.3.2.1-1: Paging

The purpose of this procedure is:

-	to transmit paging information to a UE in RRC_IDLE or RRC_INACTIVE.

[TS 38.331, clause 5.3.2.2]

The network initiates the paging procedure by transmitting the Paging message at the UE's paging occasion as specified in TS 38.304 [20]. The network may address multiple UEs within a Paging message by including one PagingRecord for each UE.

[TS 38.331, clause 5.3.2.3]

Upon receiving the Paging message, the UE shall:

…

1>	if in RRC_INACTIVE, for each of the PagingRecord, if any, included in the Paging message:

2>	if the ue-Identity included in the PagingRecord matches the UE's stored fullI-RNTI:

…

3>	else:

4>	initiate the RRC connection resumption procedure according to 5.3.13 with resumeCause set to mt-Access;

[TS 38.331, clause 6.3.2]

nrofPDCCH-MonitoringOccasionPerSSB-InPO

The number of PDCCH monitoring occasions corresponding to an SSB within a Paging Occasion, see TS 38.304 [20], clause 7.1.

[TS 38.331, clause 6.5]

Short Messages can be transmitted on PDCCH using P-RNTI with or without associated Paging message using Short Message field in DCI format 1_0 (see TS 38.212 [17], clause 7.3.1.2.1).

Table 6.5-1 defines Short Messages. Bit 1 is the most significant bit.

Table 6.5-1: Short Messages

| Bit | Short Message |
| --- | --- |
| 1 | systemInfoModification If set to 1: indication of a BCCH modification other than SIB6, SIB7 and SIB8. |
| 2 | etwsAndCmasIndication If set to 1: indication of an ETWS primary notification and/or an ETWS secondary notification and/or a CMAS notification. |
| 3 | stopPagingMonitoring This bit can be used for only operation with shared spectrum channel access and if nrofPDCCH-MonitoringOccasionPerSSB-InPO is present. If set to 1: indication that the UE may stop monitoring PDCCH occasion(s) for paging in this Paging Occasion as specified in TS 38.304 [20], clause 7.1. |
| 4 | systemInfoModification-eDRX If set to 1: indication of a BCCH modification other than SIB6, SIB7 and SIB8. This indication applies only to UEs using eDRX cycle longer than the BCCH modification period. |
| 5 – 8 | Not used in this release of the specification, and shall be ignored by UE if received. |

[TS 38.304, clause 7.1]

The PDCCH monitoring occasions for paging are determined according to pagingSearchSpace as specified in TS 38.213 [4] and firstPDCCH-MonitoringOccasionOfPO and nrofPDCCH-MonitoringOccasionPerSSB-InPO if configured as specified in TS 38.331 [3].

…

When SearchSpaceId other than 0 is configured for pagingSearchSpace, the UE monitors the (i_s + 1)th PO. A PO is a set of 'S*X ' consecutive PDCCH monitoring occasions where 'S' is the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1 and X is the nrofPDCCH-MonitoringOccasionPerSSB-InPO if configured or is equal to 1 otherwise. The [x*S+K]th PDCCH monitoring occasion for paging in the PO corresponds to the Kth transmitted SSB, where x=0,1,…,X-1, K=1,2,…,S. The PDCCH monitoring occasions for paging which do not overlap with UL symbols (determined according to tdd-UL-DL-ConfigurationCommon) are sequentially numbered from zero starting from the first PDCCH monitoring occasion for paging in the PF.

8.1.8.2.2.3	Test description

8.1.8.2.2.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1.

-	System information combination NR-1 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used in NR cell.

UE:

-	None.

Preamble:

-	The UE is in state Registered, Inactive mode (state 2N-A) on NR Cell 1 according to TS 38.508-1 [4] Table 4.4A.2-2.

8.1.8.2.2.3.2	Test procedure sequence

Table 8.1.8.2.2.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits a Paging message in the second PDCCH monitoring occasion of a paging occasion with matched identity. | <-- | NR RRC: Paging | - | - |
| 2 | Check: Does the UE transmit a RRCResumeRequest message? | --> | NR RRC: RRCResumeRequest | 1 | P |
| 3 | The SS transmits an RRCResume message. | <-- | NR RRC: RRCResume | - | - |
| 4 | The UE transmits an RRCResumeComplete message. | --> | NR RRC: RRCResumeComplete | - | - |
| 5 | The SS transmits an RRCRelease message including suspendConfig with NR_RRC_INACTIVE condition. | <-- | NR RRC: RRCRelease | - | - |
| 6 | The SS transmits a Short message on PDCCH using P-RNTI indicating a stopPagingMonitoring in the first PDCCH monitoring occasion of a paging occasion. | <-- | PDCCH (DCI 1_0): Short Message | - | - |
| 7 | The SS transmits a Paging message in all the subsequent PDCCH monitoring occasions of the same paging occasion as step 6 with matched identity. | <-- | NR RRC: Paging | - | - |
| 8 | Check: Does the UE transmit an RRCResumeRequest message within 10s? | --> | NR RRC: RRCResumeRequest | 2 | F |

8.1.8.2.2.3.3	Specific message contents

Table 8.1.8.2.2.3.3-1: SIB1 (All steps, Table 8.1.8.2.2.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |  |
| servingCellConfigCommon |  | ServingCellConfigCommonSIB | Table 8.1.8.2.2.3.3-2 |  |
| } |  |  |  |  |

Table 8.1.8.2.2.3.3-2: ServingCellConfigCommonSIB (Table 8.1.8.2.2.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-169 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommonSIB ::= SEQUENCE { |  |  |  |
| downlinkConfigCommon | DownlinkConfigCommonSIB | Table 8.1.8.2.2.3.3-3 |  |
| } |  |  |  |

Table 8.1.8.2.2.3.3-3: DownlinkConfigCommonSIB (Table 8.1.8.2.2.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-53 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DownlinkConfigCommonSIB ::= SEQUENCE { |  |  |  |
| PCCH-Config SEQUENCE { |  |  |  |
| nrofPDCCH-MonitoringOccasionPerSSB-InPO-r16 | 4 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.8.2.2.3.3-4: Paging (steps 1, 7, Table 8.1.8.2.2.3.3-2)

Derivation Path: TS 38.508-1 [4], Table 4.6.1-9 with condition NR_RRC_RESUME

Table 8.1.8.2.2.3.3-5: RRCRelease (step 5, Table 8.1.8.2.2.3.3-2)

Derivation Path: TS 38.508-1 [4], Table 4.6.1-16 with condition NR_RRC_INACTIVE

<End of modified section>