3GPP TSG-RAN5 Meeting # 98	R5-231254r2

# Athens, Greece, 27th February 2023 – 3rd March 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 2176 | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to A-MPR and A-SEM for NS_21 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_bands_BW_R16-UEConTest |  |  |  |  |  | Date: |  |  | 2023-02-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Additional Maximum power reduction and Additional spectrum emissions mask test cases (test cases 6.2.3 and 6.5.2.3) include an editor’s note indicating that TP Analysis for NS_21 when channel bandwidth is 5 MHz is pending. - Table 6.2.3.3.1-1 for NS-21 does not include applicability of new Additional spectrum emissions mask for NS_21 and reference to A-MPR core requirements is wrong. - Due to a wrong section number, minimum core requirements for A-SEM for NS_21 was defined in test requirement section. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - R5-231241 includes the test point analysis for A-MPR/A-SEM for NS_21 including channel bandwidth 5 MHz. Editor’s notes in A-MPR and A-SEM deleted, Initial conditions table and test requirements section for A-MPR for NS_21 updated accordingly. - R4-2302306 includes applicability of new additional spectrum emissions mask for NS_21. Hence Table 6.2.3.3.1-1 is updated accordingly (reference to A-MPR core requirements also updated). - A-SEM minimum conformance requirements and test requirements updated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Additional spectrum emission mask and additional spurious emissions for band n30 will not have the appropriate test coverage for channel bandwidth 5 MHz. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3, 6.5.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR depends on RAN4 CR R4-2302306 and on RAN5 CR R5-231241. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision 1: Removed overlapping with R5-230565. Revision 2: Spec number corrected in the cover page. Revision 3: TT added to test requirements for Additional spectrum emissions mask Table 6.5.2.3.5.9-1 |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES 1>>>

### 6.2.3	UE additional maximum output power reduction

Editor’s note: The following aspects are either missing or not yet determined:

- Tests for network signalling values NS_07, NS_40, NS_09 not complete.

- The requirements of this test apply in test case 6.5.3.3 Additional Spurious Emissions for network signalling values NS_44, NS_46, NS_47, NS_48, and NS_49 to all types of NR Power Class 2 and 3 UE release 16 forward, and UE release 15 if the corresponding channel bandwidths are supported.

-

## <<< END OF CHANGES 1>>>

## <<< START OF CHANGES 2>>>

6.2.3.3.1	General

Table 6.2.3.3.1-1 specifies the additional requirements with their associated network signalling values and the allowed A-MPR and applicable operating band(s) for each NS value. In case of a power class 3 UE, when IE powerBoostPi2BPSK is set to 1, power class 2 A-MPR values apply. The mapping of NR frequency band numbers and values of the additionalSpectrumEmission to network signalling labels is specified in Table 6.2.3.3.1-1A.

For almost contiguous allocations in CP-OFDM waveforms in power class 3, the allowed A-MPR defined in clause 6.2.3 is increased by CEIL{ 10 log10(1 + NRB_gap / NRB_alloc), 0.5 } dB, where CEIL{x, 0.5} means x rounding upwards to closest 0.5dB, NRB_gap is the total number of unallocated RBs between allocated RBs and NRB_alloc is the total number of allocated RBs, and the parameter LCRB is replaced by NRB_alloc + NRB_gap in specifying the RB allocation regions.

Unless otherwise specified, pi/2 BPSK in following A-MPR tables refers to both variants of pi/2 BPSK referenced in 6.2.2 Table 6.2.2.3-1.

Table 6.2.3.3.1-1: Additional maximum power reduction (A-MPR)

| Network signalling label | Requirements (subclause) | NR Band | Channel bandwidth (MHz) | Resources blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_01 |  | Table 5.2-1 (NOTE 8) | 5, 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | Table 5.3.2-1 | N/A |
| NS_03 | 6.5.2.3.3.3 | n2, n25, n66, n70, n86 |  |  | Clause 6.2.3.3.7 |
| NS_03U | 6.5.2.3.3.3, 6.5.2.4.2.3 | n2, n25, n66, n86 (NOTE 1) |  |  | Clause 6.2.3.3.7 |
| NS_04 | 6.5.2.3.3.2, 6.5.3.3.3.1 | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  | Clause 6.2.3.3.2 |
| NS_05 | 6.5.3.3.3.4 | n1, n65, n84 (NOTE 1) | 5, 10, 15, 20 (Note 2) |  | Clause 6.2.3.3.4 (NOTE 7) |
| NS_05U | 6.5.3.3.3.4, 6.5.2.4.2.3 | n1, n65, n84 | 5, 10, 15, 20 |  | Clause 6.2.3.3.4 (NOTE 7) |
| NS_06 | 6.5.2.3.3.4 | n12 | 5, 10, 15 |  | N/A |
|  |  | n14 | 5, 10 |  |  |
| NS_10 |  | n20 | 15, 20 | Table 6.2.3.3.3-1 | Table 6.2.3.3.3-1 |
| NS_12 | 6.5.3.3.17 | n26 | 5, 10 | Table 6.2.3.3.21-1 | Table 6.2.3.3.21-2 |
| NS_13 | 6.5.3.3.18 | n26 | 5 | Table 6.2.3.3.22-1 | Table 6.2.3.3.22-2 |
| NS_14 | 6.5.3.3.19 | n26 | 10, 15, 20 | Table 6.2.3.3.23-1 | Table 6.2.3.3.23-2 |
| NS_15 | 6.5.3.3.20 | n26 | 5, 10, 15, 20 | Table 6.2.3.3.24-1 | Table 6.2.3.3.24-2 |
| NS_17 | 6.5.3.3.3.2 | n28, n83 | 5, 10 | Table 5.3.2-1 | N/A |
| NS_18 | 6.5.3.3.3.3 | n28, n83 | 5 |  | Table 6.2.3.3.13-1, A1 |
|  |  |  | 10, 15, 20 |  | Table 6.2.3.3.13-1, A2 |
|  |  |  | 30 |  | Table 6.2.3.3.13-1, A3, A4, A5 |
| NS_21 | 6.5.2.3.3.9 6.5.3.3.3.12 | n30 | 5,10 |  | Clause 6.2.3.3.14 |
| NS_24 | 6.5.3.3.3.13 | n65 (Note 4) | 5, 10, 15, 20 | Table 6.2.3.3.15-1 | Subclause 6.2.3.3.15 |
| NS_27 | 6.5.2.3.3.8 6.5.3.3.3.14 | n48 | 5, 10, 15, 20, 30, 40 | Table 6.2.3.3.16-1 | Table 6.2.3.3.16-2 |
| NS_35 | 6.5.2.3.3.1 | n71 | 5, 10, 15, 20 | Table 5.3.2-1 | N/A |
| NS_37 | 6.5.3.3.3.6 | n74 (Note 3) | 10, 15 | Table 6.2.3.3.8-1 | Table 6.2.3.3.8-1 |
| NS_38 | 6.5.3.3.3.7 | n74 | 5, 10, 15, 20 | Table 6.2.3.3.9-1 | Table 6.2.3.3.9-1 |
| NS_39 | 6.5.3.3.3.8 | n74 | 10, 15, 20 | Table 6.2.3.3.10-1 | Table 6.2.3.3.10-1 |
| NS_40 | 6.5.3.3.3.9 | n51 | 5 |  | Table 6.2.3.3.5-1 |
| NS_41 | 6.5.3.3.3.10 | n50 | 5, 10, 15, 20, 40, 50, 60 |  | Table 6.2.3.3.11-1 |
| NS_42 | 6.5.3.3.3.11 | n50 | 5, 10, 15, 20, 40, 50, 60 |  | Table 6.2.3.3.12-1 |
| NS_43 | 6.5.3.3.3.5 | n8, n81 | 5, 10, 15 |  | Clause 6.2.3.3.6 |
| NS_43U | 6.5.3.3.3.5, 6.5.2.4.2.3 | n8, n81 (NOTE 1) | 5, 10, 15 |  | Clause 6.2.3.3.6 |
| NS_44 | 6.5.3.3.24 | n38 | 25,30,40 | Table 6.2.3.3.20-1 | Table 6.2.3.3.20-1 |

| Network signalling label | Requirements (subclause) | NR Band | Channel bandwidth (MHz) | Resources blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_45 | 6.5.3.3.3.21 | n53 | 5, 10 |  | Clause 6.2.3.3.25 |
| NS_46 | 6.5.3.3.3.25 | n7 | 25, 30, 40, 50 | Table 6.2.3.3.17-1 | Table 6.2.3.3.17-2 |
| NS_47 | 6.5.3.3.3.15 | n41 (Note 5) | 30 | Table 6.2.3.3.18-1 | Table 6.2.3.3.18-2 |
| NS_48 | 6.5.3.3.3.22 | n1 and n84 | 10, 15, 20, 25, 30, 40, 45, 50 | Table 6.2.3.3.26-1, Table 6.2.3.3.26-3 | Table 6.2.3.3.26-2, Table 6.2.3.3.26-4 (NOTE 7) |
| NS_49 | 6.5.3.3.3.23 | n1 and n84 | 10, 15, 20, 25, 30, 40, 45, 50 | Table 6.2.3.3.27-1 Table 6.2.3.3.27-3 | Table 6.2.3.3.27-2, Table 6.2.3.3.27-4 (NOTE 7) |
| NS_50 | 6.5.3.3.3.16 | n39 | 10, 15, 20, 25, 30, 40 |  | Clause 6.2.3.3.19 |
| NS_55 | NOTE 6 | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  | N/A |
| NS_56 | 6.5.3.3.3.27 | n24, n99 | 5, 10 | Table 6.2.3.3.30 | Clause 6.2.3.3.30 |
| NS_100 | 6.5.2.4.2.3 | n1, n2, n3, n5, n8, n25, n26, n66, n80, n81, n84, n86 (Note 1) |  |  | Table 6.2.3.3.1-2 |
| NOTE 1: This NS can be signalled for NR bands that have UTRA services deployed. NOTE 2: No A-MPR is applied for 5 MHz BWchannel where the lower channel edge is ≥1930 MHz,10 MHz BWchannel where the lower channel edge is ≥1950 MHz and 15 MHz BWchannel where the lower channel edge is ≥1955 MHz. NOTE 3: Applicable when the NR carrier is within 1447.9 – 1462.9 MHz. NOTE 4: Applicable when the upper edge of the channel bandwidth frequency is greater than 1980 MHz. NOTE 5: Applicable when the NR carrier is within 2545 – 2575 MHz. NOTE 6: This NS value is applicable for cells in the range 3450 – 3550 MHz for operations in the USA. This NS value does not indicate any additional spurious emission and maximum output power reduction requirements. NOTE 7: The 1Tx architecture is assumed. For power class 2 UE indicating txDiversity-r16 [TS 38.306], the additional relaxation of [2] dB is applicable. NOTE 8: The NS_01 label with the field additionalPmax [7] absent is default for all NR bands. |  |  |  |  |  |

Table 6.2.3.3.1-1A: Mapping of Network Signalling label

| NR band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| n1 | NS_01 | NS_100 | NS_05 | NS_05U | NS_48 | NS_49 |  |  |
| n2 | NS_01 | NS_100 | NS_03 | NS_03U |  |  |  |  |
| n3 | NS_01 | NS_100 |  |  |  |  |  |  |
| n5 | NS_01 | NS_100 |  |  |  |  |  |  |
| n7 | NS_01 | NS_46 |  |  |  |  |  |  |
| n8 | NS_01 | NS_100 | NS_43 | NS_43U |  |  |  |  |
| n12 | NS_01 | NS_06 |  |  |  |  |  |  |
| n14 | NS_01 | NS_06 |  |  |  |  |  |  |
| n20 | NS_01 | Void | NS_10 |  |  |  |  |  |
| n24 | NS_01 | NS_56 |  |  |  |  |  |  |
| n25 | NS_01 | NS_100 | NS_03 | NS_03U |  |  |  |  |
| n26 | NS_01 | NS_100 | NS_12 | NS_13 | NS_14 | NS_15 |  |  |
| n28 | NS_01 | NS_17 | NS_18 |  |  |  |  |  |
| n30 | NS_01 | NS_21 |  |  |  |  |  |  |
| n34 | NS_01 |  |  |  |  |  |  |  |
| n38 | NS_01 | NS_44 |  |  |  |  |  |  |
| n39 | NS_01 | NS_50 |  |  |  |  |  |  |
| n40 | NS_01 |  |  |  |  |  |  |  |
| n41 | NS_01 | NS_04 | NS_47 |  |  |  |  |  |
| n48 | NS_01 | NS_27 |  |  |  |  |  |  |
| n50 | NS_01 | NS_41 | NS_42 |  |  |  |  |  |
| n51 | NS_01 | NS_40 |  |  |  |  |  |  |
| n53 | NS_01 | NS_45 |  |  |  |  |  |  |
| n65 | NS_01 | NS_24 | NS_100 | NS_05 | NS_05U | NS_51 |  |  |
| n66 | NS_01 | NS_100 | NS_03 | NS_03U |  |  |  |  |
| n70 | NS_01 | NS_03 |  |  |  |  |  |  |
| n71 | NS_01 | NS_35 |  |  |  |  |  |  |
| n74 | NS_01 | NS_37 | NS_38 | NS_39 |  |  |  |  |
| n77 | NS_01 | NS_55 |  |  |  |  |  |  |
| n78 | NS_01 |  |  |  |  |  |  |  |
| n79 | NS_01 |  |  |  |  |  |  |  |
| n80 | NS_01 | NS_100 |  |  |  |  |  |  |
| n81 | NS_01 | NS_100 | NS_43 | NS_43U |  |  |  |  |
| n82 | NS_01 | Void |  |  |  |  |  |  |
| n83 | NS_01 | NS_17 | NS_18 |  |  |  |  |  |
| n84 | NS_01 | NS_100 | NS_05 | NS_05U |  |  |  |  |
| n86 | NS_01 | NS_100 | NS_03 | NS_03U |  |  |  |  |
| n91 | NS_01 |  |  |  |  |  |  |  |
| n92 | NS_01 |  |  |  |  |  |  |  |
| n93 | NS_01 |  |  |  |  |  |  |  |
| n94 | NS_01 |  |  |  |  |  |  |  |
| n95 | NS_01 |  |  |  |  |  |  |  |
| n99 | NS_01 | NS_56 |  |  |  |  |  |  |
| NOTE: additionalSpectrumEmission corresponds to an information element of the same name defined in clause 6.3.2 of TS 38.331 [6]. |  |  |  |  |  |  |  |  |

Table 6.2.3.3.1-2: A-MPR for NS_100 (UTRA protection)

| Modulation/Waveform |  | Outer (dB) |
| --- | --- | --- |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 2 |
|  | QPSK | ≤ 2 |
|  | 16 QAM | ≤ 2.5 |
|  | 64 QAM | ≤ 3 |
|  | 256 QAM | ≤ 4.5 |
| CP-OFDM | QPSK | ≤ 4 |
|  | 16 QAM | ≤ 4 |
|  | 64 QAM | ≤ 4 |
|  | 256 QAM | ≤ 6.5 |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2.3.1.

## <<< END OF CHANGES 2>>>

## <<< START OF CHANGES 3>>>

Table 6.2.3.4.1-27: Test Configuration table for NS_21

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | Low range, High range |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | Lowest, Highest |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | Lowest, Highest |  |  |  |
| A-MPR test parameters for NS_21 |  |  |  |  |  |  |  |  |  |  |
| Test ID | Freq | ChBw | SCS | Downlink Configuration | Uplink Configuration |  |  |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  |  | RB allocation (Note 1) |  |  |
|  |  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz |  |
| 1 | Low | Default | Default | N/A for A-MPR test cases | DFT-s OFDM | PI/2 BPSK |  | Edge_1RB_Left |  |  |
| 2 | High | Default | Default |  |  | PI/2 BPSK |  | Edge_1RB_Right |  |  |
| 3 | Default | Default | Default |  |  | PI/2 BPSK |  | Outer_Full |  |  |
| 4 | Default | Default | Default |  |  | PI/2 BPSK |  | 4@0 | 2@0 |  |
| 5a | Default | 5 MHz | Default |  |  | PI/2 BPSK |  | 4@21 | 2@7 |  |
| 5b | Default | 10 MHz | Default |  |  | PI/2 BPSK |  | 4@48 | 2@22 |  |
| 6 | Low | Default | Default |  |  | QPSK |  | Edge_1RB_Left |  |  |
| 7 | High | Default | Default |  |  | QPSK |  | Edge_1RB_Right |  |  |
| 8 | Default | Default | Default |  |  | QPSK |  | Outer_Full |  |  |
| 9 | Default | Default | Default |  |  | QPSK |  | 4@0 |  | 2@0 |
| 10a | Default | 5 MHz | Default |  |  | QPSK |  | 4@21 |  | 2@7 |
| 10b | Default | 10 MHz | Default |  |  | QPSK |  | 4@48 |  | 2@22 |
| 11 | Low | Default | Default |  |  | 16 QAM |  | Edge_1RB_Left |  |  |
| 12 | High | Default | Default |  |  | 16 QAM |  | Edge_1RB_Right |  |  |
| 13 | Default | Default | Default |  |  | 16 QAM |  | Outer_Full |  |  |
| 14 | Default | Default | Default |  |  | 16 QAM |  | 4@0 |  | 2@0 |
| 15a | Default | 5 MHz | Default |  |  | 16 QAM |  | 4@21 |  | 2@7 |
| 15b | Default | 10 MHz | Default |  |  | 16 QAM |  | 4@48 |  | 2@22 |
| 16 | Low | Default | Default |  |  | 64 QAM |  | Edge_1RB_Left |  |  |
| 17 | High | Default | Default |  |  | 64 QAM |  | Edge_1RB_Right |  |  |
| 18 | Default | Default | Default |  |  | 64 QAM |  | Outer_Full |  |  |
| 19 | Default | Default | Default |  |  | 64 QAM |  | 4@0 |  | 2@0 |
| 20a | Default | 5 MHz | Default |  |  | 64 QAM |  | 4@21 |  | 2@7 |
| 20b | Default | 10 MHz | Default |  |  | 64 QAM |  | 4@48 |  | 2@22 |
| 21 | Low | Default | Default |  |  | 256 QAM |  | Edge_1RB_Left |  |  |
| 22 | High | Default | Default |  |  | 256 QAM |  | Edge_1RB_Right |  |  |
| 23 | Default | Default | Default |  |  | 256 QAM |  | Outer_Full |  |  |
| 24 | Default | Default | Default |  |  | 256 QAM |  | 4@0 |  | 2@0 |
| 25a | Default | 5 MHz | Default |  |  | 256 QAM |  | 4@21 |  | 2@7 |
| 25b | Default | 10 MHz | Default |  |  | 256 QAM |  | 4@48 |  | 2@22 |
| 26 | Low | Default | Default |  | CP-s OFDM | QPSK |  | Edge_1RB_Left |  |  |
| 27 | High | Default | Default |  |  | QPSK |  | Edge_1RB_Right |  |  |
| 28 | Default | Default | Default |  |  | QPSK |  | Outer_Full |  |  |
| 29 | Default | Default | Default |  |  | QPSK |  | 4@0 |  | 2@0 |
| 30a | Default | 5 MHz | Default |  |  | QPSK |  | 4@21 |  | 2@7 |
| 30b | Default | 10 MHz | Default |  |  | QPSK |  | 4@48 |  | 2@22 |
| 31 | Low | Default | Default |  |  | 16 QAM |  | Edge_1RB_Left |  |  |
| 32 | High | Default | Default |  |  | 16 QAM |  | Edge_1RB_Right |  |  |
| 33 | Default | Default | Default |  |  | 16 QAM |  | Outer_Full |  |  |
| 34 | Default | Default | Default |  |  | 16 QAM |  | 4@0 |  | 2@0 |
| 35a | Default | 5 MHz | Default |  |  | 16 QAM |  | 4@21 |  | 2@7 |
| 35b | Default | 10 MHz | Default |  |  | 16 QAM |  | 4@48 |  | 2@22 |
| 36 | Low | Default | Default |  |  | 64 QAM |  | Edge_1RB_Left |  |  |
| 37 | High | Default | Default |  |  | 64 QAM |  | Edge_1RB_Right |  |  |
| 38 | Default | Default | Default |  |  | 64 QAM |  | Outer_Full |  |  |
| 39 | Default | Default | Default |  |  | 64 QAM |  | 4@0 |  | 2@0 |
| 40a | Default | 5 MHz | Default |  |  | 64 QAM |  | 4@21 |  | 2@7 |
| 40b | Default | 10 MHz | Default |  |  | 64 QAM |  | 4@48 |  | 2@22 |
| 41 | Low | Default | Default |  |  | 256 QAM |  | Edge_1RB_Left |  |  |
| 42 | High | Default | Default |  |  | 256 QAM |  | Edge_1RB_Right |  |  |
| 43 | Default | Default | Default |  |  | 256 QAM |  | Outer_Full |  |  |
| 44 | Default | Default | Default |  |  | 256 QAM |  | 4@0 |  | 2@0 |
| 45a | Default | 5 MHz | Default |  |  | 256 QAM |  | 4@21 |  | 2@7 |
| 45b | Default | 10 MHz | Default |  |  | 256 QAM |  | 4@48 |  | 2@22 |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. |  |  |  |  |  |  |  |  |  |  |

## <<< END OF CHANGES 3>>>

## <<< START OF CHANGES 4>>>

Table 6.2.3.5-32: UE Power Class 3 test requirements (NS_21)

| Test ID | ChBw (MHz) | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) |  | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 2 | 5 | 23 | 0.5 | 4 | 0 |  | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
|  | 10 | 23 | 0.5 | 6 | 0 |  | 17 | 5 | 2 | 25+TT | 12-TT |
| 3 | 5 | 23 | 0.5 | 1.5 | 0 |  | 21.5 | 2 | 2 | 25+TT | 20.5-TT |
|  | 10 | 23 | 0.5 | 4 | 0 |  | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
|  | 5 | 23 | 0.5 | 2 | 0 |  | 21 | 2 | 2 | 25+TT | 19 -TT |
| 4 | 10 | 23 | 0.5 | 3 | 0 |  | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 5a | 5 | 23 | 0.5 | 2 | 0 |  | 21 | 2 | 2 | 25+TT | 19 -TT |
| 5b | 10 | 23 | 0.5 | 3 | 0 |  | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 6, 7 | 5 | 23 | 1 | 4.5 | 0 |  | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
|  | 10 | 23 | 1 | 6 | 0 |  | 17 | 5 | 2 | 25+TT | 12-TT |
| 8 | 5 | 23 | 1 | 2.0 | 0 |  | 21 | 2 | 2 | 25+TT | 19-TT |
|  | 10 | 23 | 1 | 4 | 0 |  | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
|  | 5 | 23 | 1 | 2.5 | 0 |  | 20.5 | 2.5 | 2 | 25+TT | 18 -TT |
| 9 | 10 | 23 | 1 | 3 | 0 |  | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 10a | 5 | 23 | 1 | 2.5 | 0 |  | 20.5 | 2.5 | 2 | 25+TT | 18 -TT |
| 10b | 10 | 23 | 1 | 3 | 0 |  | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 11, 12 | 5 | 23 | 2 | 4.5 | 0 |  | 18.5 | 4 | 2 | 25+TT | 14.5 -TT |
|  | 10 | 23 | 2 | 6 | 0 |  | 17 | 5 | 2 | 25+TT | 12-TT |
| 13 | 5 | 23 | 2 | 2.5 | 0 |  | 20.5 | 2.5 | 2 | 25+TT | 18 -TT |
|  | 10 | 23 | 2 | 4 | 0 |  | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
|  | 5 | 23 | 2 | 2.5 | 0 |  | 20.5 | 2.5 | 2 | 25+TT | 18 -TT |
| 14 | 10 | 23 | 2 | 3 | 0 |  | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 15a | 5 | 23 | 2 | 2.5 | 0 |  | 20.5 | 2.5 | 2 | 25+TT | 18 -TT |
| 15b | 10 | 23 | 2 | 3 | 0 |  | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 16, 17 | 5 | 23 | 2.5 | 4.5 | 0 |  | 18.5 | 4 | 2 | 25+TT | 14.5 -TT |
|  | 10 | 23 | 2.5 | 6 | 0 |  | 17 | 5 | 2 | 25+TT | 12-TT |
| 18 | 5 | 23 | 2.5 | 4.5 | 0 |  | 18.5 | 4 | 2 | 25+TT | 14.5 -TT |
|  | 10 | 23 | 2.5 | 4 | 0 |  | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
|  | 5 | 23 | 2.5 | 2.5 | 0 |  | 20.5 | 2.5 | 2 | 25+TT | 18 -TT |
| 19 | 10 | 23 | 2.5 | 3 | 0 |  | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 20a | 5 | 23 | 2.5 | 2.5 | 0 |  | 20.5 | 2.5 | 2 | 25+TT | 18 -TT |
| 20b | 10 | 23 | 2.5 | 3 | 0 |  | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 21, 22 | 5 | 23 | 4.5 | 4.5 | 0 |  | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
|  | 10 | 23 | 4.5 | 6 | 0 |  | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 23 | 5 | 23 | 4.5 | 0 | 0 |  | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
|  | 10 | 23 | 4.5 | 4 | 0 |  | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
|  | 5 | 23 | 4.5 | 4.5 | 0 |  | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
| 24 | 10 | 23 | 4.5 | 3 | 0 |  | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
| 25a |  | 5 |  | 23 |  | 4.5 |  | 4.5 |  | 0 |  |  | 18.5 | 4.0 |  | 2 |  | 25+TT |  | 14.5-TT |  |
| 25b |  | 10 |  | 23 |  | 4.5 |  | 3 |  | 0 |  |  | 18.5 | 4.0 |  | 2 |  | 25+TT |  | 14.5-TT |  |
| 26, 27 | 5 | 23 | 3 | 4.5 | 0 |  | 18.5 | 4.0 | 2 | 25+TT | 14.5 -TT |
|  | 10 | 23 | 3 | 6 | 0 |  | 17 | 5 | 2 | 25+TT | 12-TT |
| 28 | 5 | 23 | 3 | 4 | 0 |  | 19 | 3.5 | 2 | 25+TT | 15.5 -TT |
|  | 10 | 23 | 3 | 5.5 | 0 |  | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
|  | 5 | 23 | 3 | 4 | 0 |  | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 29 | 10 | 23 | 3 | 4 | 0 |  | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 30a |  | 5 |  | 23 |  | 3 |  | 4 |  | 0 |  |  | 19 | 3.5 |  | 2 |  | 25+TT |  | 15.5-TT |  |
| 30b |  | 10 |  | 23 |  | 3 |  | 4 |  | 0 |  |  | 19 | 3.5 |  | 2 |  | 25+TT |  | 15.5-TT |  |
| 31, 32 | 5 | 23 | 3 | 4.5 | 0 |  | 18.5 | 4 | 2 | 25+TT | 14.5 -TT |
|  | 10 | 23 | 3 | 6 | 0 |  | 17 | 5 | 2 | 25+TT | 12-TT |
| 33 | 5 | 23 | 3 | 4 | 0 |  | 19 | 3.5 | 2 | 25+TT | 15.5 -TT |
|  | 10 | 23 | 3 | 5.5 | 0 |  | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
|  | 5 | 23 | 3 | 4 | 0 |  | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 34 | 10 | 23 | 3 | 4 | 0 |  | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 35a |  | 5 |  | 23 |  | 3 |  | 4 |  | 0 |  |  | 19 | 3.5 |  | 2 |  | 25+TT |  | 15.5-TT |  |
| 35b |  | 10 |  | 23 |  | 3 |  | 4 |  | 0 |  |  | 19 | 3.5 |  | 2 |  | 25+TT |  | 15.5-TT |  |
| 36, 37 | 5 | 23 | 3.5 | 4.5 | 0 |  | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
|  | 10 | 23 | 3.5 | 6 | 0 |  | 17 | 5 | 2 | 25+TT | 12-TT |
| 38 | 5 | 23 | 3.5 | 4 | 0 |  | 19 | 3.5 | 2 | 25+TT | 15.5 -TT |
|  | 10 | 23 | 3.5 | 5.5 | 0 |  | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
|  | 5 | 23 | 3.5 | 4 | 0 |  | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 39 | 10 | 23 | 3.5 | 4 | 0 |  | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 40a |  | 5 |  | 23 |  | 3.5 |  | 4 |  | 0 |  |  | 19 | 3.5 |  | 2 |  | 25+TT |  | 15.5-TT |  |
| 40b |  | 10 |  | 23 |  | 3.5 |  | 4 |  | 0 |  |  | 19 | 3.5 |  | 2 |  | 25+TT |  | 15.5-TT |  |
| 41, 42 | 5 | 23 | 6.5 | 6.5 | 0 |  | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
|  | 10 | 23 | 6.5 | 6 | 0 |  | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 43 | 5 | 23 | 6.5 | 6.5 | 0 |  | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
|  | 10 | 23 | 6.5 | 5.5 | 0 |  | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
|  |  | 5 |  | 23 |  | 6.5 |  | 6.5 |  | 0 |  |  | 16.5 | 5 |  | 2 |  | 25+TT |  | 11.5-TT |  |
| 44 | 10 | 23 | 6.5 | 5.5 | 0 |  | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 45a | 5 | 23 | 6.5 | 6.5 | 0 |  | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 45b | 10 | 23 | 6.5 | 4 | 0 |  | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |  |  |

## <<< END OF CHANGES 4>>>

## <<< START OF CHANGES 5>>>

#### 6.5.2.3	Additional spectrum emission mask

Editor’s Note: The following aspects are either missing or not yet determined:

- NS_07 test requirements is not complete.

## <<< END OF CHANGES 5>>>

## <<< START OF CHANGES 6>>>

6.5.2.3.3.8	Requirements for network signalling value "NS_27"

Additional spectrum emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

When "NS_27" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.2.3.3.8-1.

Table 6.5.2.3.3.8-1: Additional requirements for "NS_27"

| ΔfOOB MHz | Channel bandwidth (MHz) / Spectrum emission limit (dBm) |  |  |  |  |  | Measurement bandwidth |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5 | 10 | 15 | 20 | 30 | 40 |  |
| ± 0 - 1 | -13 |  |  |  |  |  | 1 % channel bandwidth |
| ± 1 - X | -13 |  |  |  |  |  | 1 MHz |
| < – X or > X | -25 |  |  |  |  |  |  |
| NOTE 1: X is occupied channel bandwidth as defined in Table 6.5.1.3-1. NOTE 2: The requirements apply only at the frequency range from 3540 MHz to 3710 MHz. |  |  |  |  |  |  |  |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.5.2.3.8.

6.5.2.3.3.9	Requirements for network signalling value "NS_21"

Additional spectrum emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

When "NS_21" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.2.3.3.9-1.

Table 6.5.2.3.3.9-1: Additional requirements for "NS_21" (applicable to UEs Release 17 and forward indicating modifiedMPR-Behavior)

| ΔfOOB MHz | Channel bandwidth (MHz) / Spectrum emission limit (dBm) |  | Measurement bandwidth |
| --- | --- | --- | --- |
|  | 5 | 10 |  |
| 0-1 | -13 | -13 | 1 MHz |
| 1-6 | -13 | -13 | 1 MHz |
| 6-10 | -25 | -13 | 1 MHz |
| 10-15 |  | -25 | 1 MHz |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

6.5.2.3.4	Test description

6.5.2.3.4.1	Initial conditions

## <<< END OF CHANGES 6>>>

## <<< START OF CHANGES 7>>>

6.5.2.3.5.4	Test requirements (network signalling value "NS_06")

When "NS_06" is indicated in the cell:

-	the measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in table 6.2.3.5-34 as appropriate for a NR UE

and

-	the power of any UE emission shall fulfil requirements in table 6.5.2.3.5.4-1, as applicable.

Table 6.5.2.3.5.4-1: Additional requirements for "NS_06"

| Spectrum emission limit (dBm) / Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 5 MHz | 10 MHz | 15 MHz | Measurement bandwidth |
| ± 0 – 0.1 | -15 + TT | -18 + TT | -20 + TT | 30 kHz |
| ± 0.1 – 1 | -13 + TT | -13 + TT | -13 + TT | 100 kHz |
| ± 1 – 6 | -13 + TT | -13 + TT | -13 + TT | 1 MHz |
| ± 6 – 10 | -25 + TT | -13 + TT | -13 + TT | 1 MHz |
| ± 10 – 15 |  | -25 + TT | -13 + TT | 1 MHz |
| ± 15 – 20 |  |  | -25 + TT | 1 MHz |
| NOTE 1: TT for each frequency and channel bandwidth is specified in Table 6.5.2.3.5-1. |  |  |  |  |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

6.5.2.3.5.5	Void

6.5.2.3.5.6	Void

6.5.2.3.5.7	Void

6.5.2.3.5.8	FFS

6.5.2.3.5.9	Test requirements for network signalling value "NS_21"

When "NS_21" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.2.3.5.9-1.

Table 6.5.2.3.5.9-1: Additional requirements for "NS_21" (applicable to UEs Release 17 and forward indicating modifiedMPR-Behavior)

| ΔfOOB MHz | Channel bandwidth (MHz) / Spectrum emission limit (dBm) |  | Measurement bandwidth |
| --- | --- | --- | --- |
|  | 5 | 10 |  |
| 0-1 | -13 + TT | -13 + TT | 1 MHz |
| 1-6 | -13 + TT | -13 + TT | 1 MHz |
| 6-10 | -25 + TT | -13 + TT | 1 MHz |
| 10-15 |  | -25 + TT | 1 MHz |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

#### 6.5.2.4	Adjacent channel leakage ratio

## <<< END OF CHANGES 7>>>