3GPP TSG-RAN WG5 Meeting # 98	R5-231292r1

# Athens, Greece, 27 Feb – 03 Mar, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-2 | CR | 0906 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on CA MPR definition in FR2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_FR2_req_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2023-02-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The definition for intra-band non-contiguous UL CA MPR is not clear and need to be corrected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | (1) Correct the definition for intra-band non-contiguous UL CA MPR. (3) Some other editorial corrections. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The definition for intra-band non-contiguous UL CA MPR is not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2A.2.0.2, 6.2A.2.0.4, 6.2A.2.1.3, 6.2A.2.1.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-231292r1: Correct the sentence below Table 6.2A.2.0.4-2 the requirement set by the waveform type used for intra-band non-contiguous UL CA to the largest MPRNC_CA. |  |  |  |  |  |  |  |  |

<< Start of changes >>

6.2A.2	UE maximum output power reduction for CA

6.2A.2.0	Minimum conformance requirements

6.2A.2.0.1	General

The UE is defined to be configured for CA operation when it has at least one of UL or DL configured for CA. In CA operation, the UE may reduce its maximum output power due to higher order modulations and transmit bandwidth configurations. This Maximum Power Reduction (MPR) is defined in subclauses below.

When the maximum output power of a UE is modified by MPR, the power limits specified in subclause 6.2A.4.0 apply.

The requirements in the following subclauses are only applicable to the following CA configurations:

-	intra-band contiguous uplink CA, with the aggregated channel bandwidth up to 800 MHz.

-	intra-band non-contiguous uplink CA with UL frequency separation no greater than 1400 MHz, and no more than 3 sub-blocks. A sub-block may consist of single CC or multiple contiguous CCs.

-	In case the CA configuration consists of a single UL CC, MPR for contiguous UL CA applies and where necessary, BWchannel shall be used as BWchannel_CA.

6.2A.2.0.2	Maximum output power reduction for power class 1

For power class 1, MPR for intra-band contiguous UL CA with contiguous allocations within the cumulative aggregated bandwidth is defined as:

MPRC_CA = max(MPRWT_C_CA, MPRnarrow)

Where,

MPRnarrow = 14.4 dB, when BWalloc,RB is less than or equal to 1.44 MHz, MPRnarrow = 10 dB, when 1.44 MHz < BWalloc,RB ≤ 10.8 MHz, where BWalloc,RB is the bandwidth of the RB allocation size.

MPRWT_C_CA is the maximum power reduction due to modulation orders, transmit bandwidth configurations, and waveform types. MPRWT_C_CA is defined in Table 6.2A.2.0.2-1.

Table 6.2A.2.0.2-1: Maximum power reduction (MPRWT_C_CA) for UE power class 1

| Waveform Type |  | Cumulative aggregated channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | < 400 MHz | ≥ 400 MHz and < 800 MHz | ≥ 800 MHz and ≤ 1400 MHz | > 1400 MHz and ≤ 2400 MHz |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 5.5 | 7.7 | 8.2 | ≤ 8.7 |
|  | QPSK | ≤ 6.5 | 8.7 | 9.7 | ≤ 9.7 |
|  | 16 QAM | ≤ 6.5 | 8.7 | 9.2 | ≤ 9.7 |
|  | 64 QAM | ≤ 9.0 | 10.7 | 11.2 | ≤ 11.7 |
| CP-OFDM | QPSK | ≤ 6.5 | 8.7 | 8.7 | ≤ 9.7 |
|  | 16 QAM | ≤ 6.5 | 8.7 | 8.7 | ≤ 9.7 |
|  | 64 QAM | ≤ 9.0 | 10.7 | 11.2 | ≤ 11.7 |
| NOTE 1: Void. |  |  |  |  |  |

In case of a contiguous RB, DFT-s-BPSK or DFT-s-QPSK UL allocation in a single CC of a CA configuration with contiguous CCs, and whose cumulative aggregated BW  400 MHz, MPRWT_C_CA shall be derived instead as MAX(MPR1, MPR2), where:

MPR1 shall be determined from Table 6.2.2.3.1-1 if CABW  200 MHz, from Table 6.2.2.3.1-2 if CABW > 200 MHz.

MPR2 shall be determined from Table 6.2.2.3.1-1 if UL BWchannel_CA  200 MHz, from Table 6.2.2.3.1-2 if UL BWchannel_CA > 200 MHz.

and assume all UL CCs use the same SCS for the purpose of determination of inner and outer RB allocations in Table 6.2.2.3.1-1 and Table 6.2.2.3.1-2:

NRB shall be chosen as the sum of NRB of all constituent UL CCs in the CA configuration.

LCRB shall be chosen as BWalloc,RB

RBstart shall be derived as: RBstart_allocatedCC+NRB_unallocatedCC_low

RBstart_allocatedCC is the index of the first allocated RB in the CC with allocation

NRB_unallocatedCC_low is the sum of NRB in all UL CCs lower in frequency compared to the CC with allocation

When different waveform types exist across CCs, the requirement is set by the waveform type used in the configuration with the largest MPRC_CA.

For intra-band contiguous UL CA with non-contiguous RB allocations, the following rule for MPR applies:

MPR = max(MPRC_CA, -10*A + 14.4)

Where:

A = NRB_alloc / NRB_agg_C.

NRB_alloc is the total number of allocated UL RBs

NRB_agg_C is the number of the aggregated RBs within the fully allocated cumulative aggregated channel bandwidth assuming lowest SCS among all configured CCs

For intra-band non-contiguous UL CA, the following rule for MPR applies:

MPR = max(MPRNC_CA, -10*A +  14.4)

Where:

MPRNC_CA is derived from table 6.2A.2.0.2-2

A = NRB_alloc / NRB_agg_C.

NRB_alloc is the total number of allocated UL RBs

NRB_agg_C is the number of the aggregated RBs within the fully allocated cumulative aggregated channel bandwidth assuming lowest SCS among all configured CCs

Table 6.2A.2.0.2-2: MPRNC_CA for UE power class 1

| Waveform Type |  | Cumulative aggregated channel bandwidth (CABW) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | < 400 MHz | ≥ 400 MHz and < 800 MHz | ≥ 800 MHz and ≤ 1400 MHz | > 1400 MHz and ≤ 2400 MHz |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 6 | ≤ 7.7 | ≤ 8.2 | ≤ 8.7 |
|  | QPSK | ≤ 7 | ≤ 8.7 | ≤ 9.2 | ≤ 9.7 |
|  | 16 QAM | ≤ 7 | ≤ 8.7 | ≤ 9.2 | ≤ 9.7 |
|  | 64 QAM | ≤ 9.0 | ≤ 10.7 | ≤ 11.2 | ≤ 11.7 |
| CP-OFDM | QPSK | ≤ 7 | ≤ 8.7 | ≤ 9.2 | ≤ 9.7 |
|  | 16 QAM | ≤ 7 | ≤ 8.7 | ≤ 9.2 | ≤ 9.7 |
|  | 64 QAM | ≤ 9.0 | ≤ 10.7 | ≤ 11.2 | ≤ 11.7 |

When different waveform types exist across CCs, the requirement is set by the waveform type used in the configuration with the largest MPRC_CA.

6.2A.2.0.3	Maximum output power reduction for power class 2

For power class 2, MPR specified in sub-clause 6.2A.2.0.4 applies.

Table 6.2A.2.0.3-1: (Void)

6.2A.2.0.4	Maximum output power reduction for power class 3

For power class 3, MPR for intra-band contiguous UL CA with contiguous allocations within the cumulative aggregated bandwidth is denoted as MPRC_CA and is defined in Table 6.2A.2.0.4-1.

Table 6.2A.2.0.4-1: Maximum power reduction (MPRC_CA) for UE power class 3

|  |  | Cumulative aggregated bandwidth configuration (CABW) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | ≤ 400 MHz | > 400 MHz and < 800 MHz | ≥ 800 MHz and ≤ 1400 MHz | > 1400 MHz and ≤ 2400 MHz |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 5.0 | ≤ 7.7 | ≤ 8.2 | ≤ 8.7 |
|  | QPSK | ≤ 5.0 | ≤ 7.7 | ≤ 8.2 | ≤ 9.7 |
|  | 16 QAM | ≤ 6.5 | ≤ 8.7 | ≤ 9.3 | ≤ 9.7 |
|  | 64 QAM | ≤ 9.0 | ≤ 10.7 | ≤ 11.2 | ≤ 11.7 |
| CP-OFDM | QPSK | ≤ 5.0 | ≤ 7.5 | ≤ 8.0 | ≤ 9.7 |
|  | 16 QAM | ≤ 6.5 | ≤ 8.7 | ≤ 9.2 | ≤ 9.7 |
|  | 64 QAM | ≤ 9.0 | ≤ 10.7 | ≤ 11.2 | ≤ 11.7 |
| NOTE 1: void. |  |  |  |  |  |

In case of a contiguous RB, DFT-s-BPSK or DFT-s-QPSK UL allocation in a single CC of a CA configuration with contiguous CCs, and whose cumulative aggregated BW  400 MHz, MPRC_CA shall be derived instead as MAX(MPR1, MPR2), where:

MPR1 shall be determined from Table 6.2.2.3.3-1 if CABW  200 MHz, from Table 6.2.2.3.3-2 if CABW > 200 MHz.

MPR2 shall be determined from Table 6.2.2.3.3-1 if UL BWchannel_CA  200 MHz, from Table 6.2.2.3.3-2 if UL BWchannel_CA > 200 MHz.

and assume all UL CCs use the same SCS for the purpose of determination of inner and outer RB allocations in Table 6.2.2.3.3-1 and Table 6.2.2.3.3-2:

NRB shall be chosen as the sum of NRB of all constituent UL CCs in the CA configuration.

LCRB shall be chosen as BWalloc,RB

RBstart shall be derived as: RBstart_allocatedCC+NRB_unallocatedCC_low

RBstart_allocatedCC is the index of the first allocated RB in the CC with allocation

NRB_unallocatedCC_low is the sum of NRB in all UL CCs lower in frequency compared to the CC with allocation

When different waveform types exist across CCs, the requirement is set by the waveform type used in the configuration with the highest contiguous MPR.

For intra-band contiguous UL CA with non-contiguous RB allocations, the following rule for MPR applies:

MPR = max(MPRC_CA, -10*A +7.0)

Where:

A = NRB_alloc / NRB_agg_C.

NRB_alloc is the total number of allocated UL RBs

NRB_agg_C is the number of the aggregated RBs within the fully allocated cumulative aggregated channel bandwidth assuming lowest SCS among all configured CCs

For intra-band non-contiguous UL CA, the following rule for MPR applies:

MPR = max(MPRNC_CA, -8*A +10.0)

Where:

MPRNC_CA is derived from table 6.2A.2.0.4-2

A = NRB_alloc / NRB_agg_C.

NRB_alloc is the total number of allocated UL RBs

NRB_agg_C is the number of the aggregated RBs within the fully allocated cumulative aggregated channel bandwidth assuming lowest SCS among all configured CCs

Table 6.2A.2.0.4-2: MPRNC_CA for UE power class 3

|  |  | Cumulative aggregated channel bandwidth (CABW) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | ≤ 400 MHz | > 400 MHz and < 800 MHz | ≥ 800 MHz and ≤ 1400 MHz | > 1400 MHz and ≤ 2400 MHz |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 5.5 | ≤ 7.7 | ≤ 8.2 | ≤ 8.7 |
|  | QPSK | ≤ 6 | ≤ 7.7 | ≤ 8.2 | ≤ 8.7 |
|  | 16 QAM | ≤ 7 | ≤ 8.7 | ≤ 9.3 | ≤ 9.8 |
|  | 64 QAM | ≤ 9.0 | ≤ 10.7 | ≤ 11.2 | ≤ 11.7 |
| CP-OFDM | QPSK | ≤ 6 | ≤ 7.5 | ≤ 8.0 | ≤ 8.5 |
|  | 16 QAM | ≤ 7 | ≤ 8.7 | ≤ 9.2 | ≤ 9.7 |
|  | 64 QAM | ≤ 9.0 | ≤ 10.7 | ≤ 11.2 | ≤ 11.7 |

When different waveform types exist across CCs, the requirement is set by the waveform type used in the configuration with the largest MPRNC_CA.

6.2A.2.0.5	Maximum output power reduction for power class 4

For power class 4, MPR specified in sub-clause 6.2A.2.0.4 applies.

The normative reference for this requirement is TS 38.101-2 [3] clause 6.2A.2.

6.2A.2.1	UE maximum output power reduction for CA (2UL CA)

Editor’s note: The following aspects are either missing or not yet determined:

-	The UPLF test mode is applicable to UEs Release 16 and forward.

-	This test case is incomplete for Power classes 1, 2, 4 Release 15.

For a transition period until RAN#99, the stability and repeatability of test procedure with PHR (variant b) for Rel-15 UEs is under evaluation.

-	Whether additional check is needed in the test procedure to ensure UE continues transmissions on the SCell is FFS

-	Measurement Uncertainties and Test Tolerances are FFS for power class 1, 2 and 4.

-	Measurement Uncertainties and Test Tolerances for intra-band contiguous CA supporting aggregated BW > 400MHz and intra-band non-contiguous CA are TBD.

-	The test points for higher bandwidth classes with testability problem need an update to decrease the UL bandwidth until they become testable.

-	Test points with more than 3 DL CCs for PC1, 2, 4 are pending removal as already done for PC3

-	Following aspects are pending RAN4

Minimum requirements for cumulative aggregated bandwidth >=800MHz are within brackets

6.2A.2.1.1	Test purpose

The number of RB identified in 6.2.2.3 is based on meeting the requirements for the maximum power reduction (MPR) due to Cubic Metric (CM).

6.2A.2.1.2	Test applicability

The requirements of this test apply to all types of NR UE release 15 and forward supporting 2UL CA.

6.2A.2.1.3	Minimum conformance requirements

The normative reference for this requirement is TS 38.101-2 [3] clause 6.2A.2.

6.2A.2.1.4	Test description

6.2A.2.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and CC combinations based on NR operating bands specified in clause 5.5A. All of these configurations shall be tested with applicable test parameters for each CA configuration and subcarrier spacing, are shown in Table 6.2A.2.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2A.2.1.4.1-1: Intra-band Contiguous UL CA Test Configuration Table (Power Class 1, MPRnarrow)

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1.2.3 for different CA bandwidth classes |  |  |  | Refer to “Test frequency” column |  |  |
| Test CC Combination setting (aggregated BW of the CA configuration) as specified in TS 38.508-1 [10] subclause 4.3.1.2.3 for the CA Configuration across bandwidth combination sets supported by the UE |  |  |  | Highest aggregated channel bandwidth of the CA configuration |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  | 120 kHz |  |  |
| Test Parameters |  |  |  |  |  |  |
| Test ID | CC & Mapping (NOTE 2) | ChBw(MHz) | Test frequency | DL RB allocation | UL Modulation | UL RB allocation (NOTE 1) |
| Default Test Settings for a CA_nXB, CA_nXD, CA_nXG, CA_nXO Configuration |  |  |  |  |  |  |
| 1 | PCC/CC1 | Default | Low | - | CP-OFDM 64QAM | Outer_1RB_Left |
|  | SCC/CC2 |  | Low |  | - | - |
| 2 | PCC/CC1 |  | High |  | CP-OFDM 64QAM | Outer_1RB_Right |
|  | SCC/CC2 |  | High |  | - | - |
| 3 | PCC/CC1 |  | Low |  | CP-OFDM 64QAM | 7@0 |
|  | SCC/CC2 |  | Low |  | - | - |
| 4 | PCC/CC1 |  | High |  | CP-OFDM 64QAM | 7@NRB-7 |
|  | SCC/CC2 |  | High |  | - | - |
| Default Test Settings for a CA_nX(D-G)_UL_nXD, CA_nX(D-G)_UL_nXG, CA_nX(D-O)_UL_nXD, CA_nX(D-O)_UL_nXO Configuration |  |  |  |  |  |  |
| 1 | PCC/CC1 | Default | Low | - | CP-OFDM 64QAM | Outer_1RB_Left |
|  | SCC1/CC2 |  | Low |  | - | - |
|  | Wgap |  | Max Wgap |  | N/A | N/A |
|  | SCC2/CC3 |  | Low |  | N/A | N/A |
|  | SCC3/CC4 |  | Low |  | N/A | N/A |
| 2 | PCC/CC1 |  | High |  | CP-OFDM 64QAM | Outer_1RB_Right |
|  | SCC1/CC2 |  | High |  | - | - |
|  | Wgap |  | Max Wgap |  | N/A | N/A |
|  | SCC2/CC3 |  | High |  | N/A | N/A |
|  | SCC3/CC4 |  | High |  | N/A | N/A |
| 3 | PCC/CC1 |  | Low |  | CP-OFDM 64QAM | 7@0 |
|  | SCC1/CC2 |  | Low |  | - | - |
|  | Wgap |  | Max Wgap |  | N/A | N/A |
|  | SCC2/CC3 |  | Low |  | N/A | N/A |
|  | SCC3/CC4 |  | Low |  | N/A | N/A |
| 4 | PCC/CC1 |  | High |  | CP-OFDM 64QAM | 7@NRB-7 |
|  | SCC1/CC2 |  | High |  | - | - |
|  | Wgap |  | Max Wgap |  | N/A | N/A |
|  | SCC2/CC3 |  | High |  | N/A | N/A |
|  | SCC3/CC4 |  | High |  | N/A | N/A |
| Default Test Settings for a CA_nX(D-H)_UL_nXD, CA_nX(D-P)_UL_nXD, CA_nX(E-O)_UL_nXO Configuration |  |  |  |  |  |  |
| 1 | PCC/CC1 | Default | Low | - | CP-OFDM 64QAM | Outer_1RB_Left |
|  | SCC1/CC2 |  | Low |  | - | - |
|  | Wgap |  | Max Wgap |  | N/A | N/A |
|  | SCC2/CC3 |  | Low |  | N/A | N/A |
|  | SCC3/CC4 |  | Low |  | N/A | N/A |
|  | SCC4/CC5 |  | Low |  | N/A | N/A |
| 2 | PCC/CC1 |  | High |  | CP-OFDM 64QAM | Outer_1RB_Right |
|  | SCC1/CC2 |  | High |  | - | - |
|  | Wgap |  | Max Wgap |  | N/A | N/A |
|  | SCC2/CC3 |  | High |  | N/A | N/A |
|  | SCC3/CC4 |  | High |  | N/A | N/A |
|  | SCC4/CC5 |  | High |  | N/A | N/A |
| 3 | PCC/CC1 |  | Low |  | CP-OFDM 64QAM | 7@0 |
|  | SCC1/CC2 |  | Low |  | - | - |
|  | Wgap |  | Max Wgap |  | N/A | N/A |
|  | SCC2/CC3 |  | Low |  | N/A | N/A |
|  | SCC3/CC4 |  | Low |  | N/A | N/A |
|  | SCC4/CC5 |  | Low |  | N/A | N/A |
| 4 | PCC/CC1 |  | High |  | CP-OFDM 64QAM | 7@NRB-7 |
|  | SCC1/CC2 |  | High |  | - | - |
|  | Wgap |  | Max Wgap |  | N/A | N/A |
|  | SCC2/CC3 |  | High |  | N/A | N/A |
|  | SCC3/CC4 |  | High |  | N/A | N/A |
|  | SCC4/CC5 |  | High |  | N/A | N/A |
| Default Test Settings for a CA_nX(D-I)_UL_nXD, CA_nX(D-Q)_UL_nXD, CA_nX(G-I)_UL_nXG Configuration |  |  |  |  |  |  |
| 1 | PCC/CC1 | Default | Low | - | CP-OFDM 64QAM | Outer_1RB_Left |
|  | SCC1/CC2 |  | Low |  | - | - |
|  | Wgap |  | Max Wgap |  | N/A | N/A |
|  | SCC2/CC3 |  | Low |  | N/A | N/A |
|  | SCC3/CC4 |  | Low |  | N/A | N/A |
|  | SCC4/CC5 |  | Low |  | N/A | N/A |
|  | SCC5/CC6 |  | Low |  | N/A | N/A |
| 2 | PCC/CC1 |  | High |  | CP-OFDM 64QAM | Outer_1RB_Right |
|  | SCC1/CC2 |  | High |  | - | - |
|  | Wgap |  | Max Wgap |  | N/A | N/A |
|  | SCC2/CC3 |  | High |  | N/A | N/A |
|  | SCC3/CC4 |  | High |  | N/A | N/A |
|  | SCC4/CC5 |  | High |  | N/A | N/A |
|  | SCC5/CC6 |  | High |  | N/A | N/A |
| 3 | PCC/CC1 |  | Low |  | CP-OFDM 64QAM | 7@0 |
|  | SCC1/CC2 |  | Low |  | - | - |
|  | Wgap |  | Max Wgap |  | N/A | N/A |
|  | SCC2/CC3 |  | Low |  | N/A | N/A |
|  | SCC3/CC4 |  | Low |  | N/A | N/A |
|  | SCC4/CC5 |  | Low |  | N/A | N/A |
|  | SCC5/CC6 |  | Low |  | N/A | N/A |
| 4 | PCC/CC1 |  | High |  | CP-OFDM 64QAM | 7@NRB-7 |
|  | SCC1/CC2 |  | High |  | - | - |
|  | Wgap |  | Max Wgap |  | N/A | N/A |
|  | SCC2/CC3 |  | High |  | N/A | N/A |
|  | SCC3/CC4 |  | High |  | N/A | N/A |
|  | SCC4/CC5 |  | High |  | N/A | N/A |
|  | SCC5/CC6 |  | High |  | N/A | N/A |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-2. NOTE 2: PCC/CCi and SCC/CCj means PCC is on component carrier CCi and SCC is on component carrier CCj, with CCi or CCj frequencies defined in TS38.508-1 [10]. |  |  |  |  |  |  |

Table 6.2A.2.1.4.1-2: Intra-band Contiguous UL CA Test Configuration Table (Power Class 1, single CC MPR requirement)

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1.2.3 for different CA bandwidth classes |  |  |  | Low range, High range |  |  |
| Test CC Combination setting (aggregated BW of the CA configuration) as specified in TS 38.508-1 [10] subclause 4.3.1.2.3 for the CA Configuration across bandwidth combination sets supported by the UE |  |  |  | Highest aggregated channel bandwidth of the CA configuration |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  | 120 kHz |  |  |
| Test Parameters |  |  |  |  |  |  |
| Test ID | CC & Mapping (NOTE 2) | ChBw(MHz) | Test frequency | DL RB allocation | UL Modulation | UL RB allocation |
| Default Test Settings for a CA_nXG, CA_nXO Configuration (Cumulative aggregated BWchannel <= 200MHz) |  |  |  |  |  |  |
| 1 | PCC/CC1 | Default | Default | - | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
|  | SCC/CC2 |  |  |  | - | - |
| 2 | PCC/CC1 |  |  |  | DFT-s-OFDM QPSK | Inner_Full_Region1 |
|  | SCC/CC2 |  |  |  | - | - |
| Default Test Settings for a CA_nXD Configuration (Cumulative aggregated BWchannel <= 400MHz) |  |  |  |  |  |  |
| 1 | PCC/CC1 | Default | Default | - | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
|  | SCC/CC2 |  |  |  | - | - |
| 2 | PCC/CC1 |  |  |  | DFT-s-OFDM Pi/2 BPSK | Inner_Full_Region1 |
|  | SCC/CC2 |  |  |  | - | - |
| 3 | PCC/CC1 |  |  |  | DFT-s-OFDM QPSK | Inner_Full_Region1 |
|  | SCC/CC2 |  |  |  | - | - |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-2. NOTE 2: PCC/CCi and SCC/CCj means PCC is on component carrier CCi and SCC is on component carrier CCj, with CCi or CCj frequencies defined in TS38.508-1 [10]. NOTE 3: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. |  |  |  |  |  |  |

Table 6.2A.2.1.4.1-3: Intra-band Contiguous UL CA Test Configuration Table (Power Class 1, MPRC_CA)

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1.2.3 and 4.3.1.2.4 for different CA bandwidth classes |  |  |  | For intra-band contiguous CA: Mid range. For intra-band non-contiguous CA: FFS. |  |  |
| Test CC Combination setting (aggregated BW of the CA configuration) as specified in TS 38.508-1 [10] subclause 4.3.1.2.3 and 4.3.1.2.4 for the CA Configuration across bandwidth combination sets supported by the UE |  |  |  | Highest aggregated channel bandwidth of the CA configuration |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  | 120 kHz |  |  |
| Test Parameters |  |  |  |  |  |  |
| Test ID | CC& Mapping (NOTE 2) | ChBw(MHz) | Test frequency | DL RB allocation | UL Modulation | UL RB allocation |
| Default Test Settings for a CA_nXB, CA_nXC_UL_nXB Configuration (800MHz <= Cumulative aggregated BWchannel <= 1400MHz) |  |  |  |  |  |  |
| 1 | PCC/CC1 | Default | Default | - | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
|  | SCC/CC2 |  |  |  | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
| 2 | PCC/CC1 |  |  |  | DFT-s-OFDM QPSK | Outer_Full |
|  | SCC/CC2 |  |  |  | DFT-s-OFDM QPSK | Outer_Full |
| 3 | PCC/CC1 |  |  |  | DFT-s-OFDM 16QAM | Outer_Full |
|  | SCC/CC2 |  |  |  | DFT-s-OFDM 16QAM | Outer_Full |
| 4 | PCC/CC1 |  |  |  | CP-OFDM 16QAM | Outer_Full |
|  | SCC/CC2 |  |  |  | CP-OFDM 16QAM | Outer_Full |
| 5 | PCC/CC1 |  |  |  | CP-OFDM 64QAM | Outer_Full |
|  | SCC/CC2 |  |  |  | CP-OFDM 64QAM | Outer_Full |
| Default Test Settings for a CA_nXD Configuration (400MHz <= Cumulative aggregated BWchannel < 800MHz) |  |  |  |  |  |  |
| 1 | PCC/CC1 | Default | Default | - | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
|  | SCC/CC2 |  |  |  | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
| 2 | PCC/CC1 |  |  |  | CP-OFDM 16QAM | Outer_Full |
|  | SCC/CC2 |  |  |  | CP-OFDM 16QAM | Outer_Full |
| 3 | PCC/CC1 |  |  |  | CP-OFDM 64QAM | Outer_Full |
|  | SCC/CC2 |  |  |  | CP-OFDM 64QAM | Outer_Full |
| Default Test Settings for a CA_nXB Configuration (400MHz <= Cumulative aggregated BWchannel < 800MHz) |  |  |  |  |  |  |
| 1 | PCC/CC1 | 200MHz | Default | - | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
|  | SCC/CC2 | 400MHz |  |  | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
| 2 | PCC/CC1 | 200MHz |  |  | CP-OFDM 16QAM | Outer_Full |
|  | SCC/CC2 | 400MHz |  |  | CP-OFDM 16QAM | Outer_Full |
| 3 | PCC/CC1 | 200MHz |  |  | CP-OFDM 64QAM | Outer_Full |
|  | SCC/CC2 | 400MHz |  |  | CP-OFDM 64QAM | Outer_Full |
| Default Test Settings for a CA_nXG, CA_nXO Configuration (Cumulative aggregated BWchannel < 400MHz) |  |  |  |  |  |  |
| 1 | PCC/CC1 | Default | Default | - | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
|  | SCC/CC2 |  |  |  | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
| 2 | PCC/CC1 |  |  |  | CP-OFDM 16QAM | Outer_Full |
|  | SCC/CC2 |  |  |  | CP-OFDM 16QAM | Outer_Full |
| 3 | PCC/CC1 |  |  |  | CP-OFDM 64QAM | Outer_Full |
|  | SCC/CC2 |  |  |  | CP-OFDM 64QAM | Outer_Full |
| Default Test Settings for a CA_nXD Configuration (Cumulative aggregated BWchannel < 400MHz) |  |  |  |  |  |  |
| 1 | PCC/CC1 | 100MHz | Default | - | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
|  | SCC/CC2 | 200MHz |  |  | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
| 2 | PCC/CC1 | 100MHz |  |  | CP-OFDM 16QAM | Outer_Full |
|  | SCC/CC2 | 200MHz |  |  | CP-OFDM 16QAM | Outer_Full |
| 3 | PCC/CC1 | 100MHz |  |  | CP-OFDM 64QAM | Outer_Full |
|  | SCC/CC2 | 200MHz |  |  | CP-OFDM 64QAM | Outer_Full |
| Default Test Settings for a CA_nX(D-G)_UL_nXD, CA_nX(D-G)_UL_nXG, CA_nX(D-O)_UL_nXD, CA_nX(D-O)_UL_nXO Configuration (800MHz <= Cumulative aggregated BWchannel <= 1400MHz) |  |  |  |  |  |  |
| 1 | PCC/CC1 | Default | Default | - | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
|  | SCC1/CC2 |  |  |  | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
|  | Wgap |  |  |  | N/A | N/A |
|  | SCC2/CC3 |  |  |  | N/A | N/A |
|  | SCC3/CC4 |  |  |  | N/A | N/A |
| 2 | PCC/CC1 |  |  |  | DFT-s-OFDM QPSK | Outer_Full |
|  | SCC1/CC2 |  |  |  | DFT-s-OFDM QPSK | Outer_Full |
|  | Wgap |  |  |  | N/A | N/A |
|  | SCC2/CC3 |  |  |  | N/A | N/A |
|  | SCC3/CC4 |  |  |  | N/A | N/A |
| 3 | PCC/CC1 |  |  |  | DFT-s-OFDM 16QAM | Outer_Full |
|  | SCC1/CC2 |  |  |  | DFT-s-OFDM 16QAM | Outer_Full |
|  | Wgap |  |  |  | N/A | N/A |
|  | SCC2/CC3 |  |  |  | N/A | N/A |
|  | SCC3/CC4 |  |  |  | N/A | N/A |
| 4 | PCC/CC1 |  |  |  | CP-OFDM 16QAM | Outer_Full |
|  | SCC1/CC2 |  |  |  | CP-OFDM 16QAM | Outer_Full |
|  | Wgap |  |  |  | N/A | N/A |
|  | SCC2/CC3 |  |  |  | N/A | N/A |
|  | SCC3/CC4 |  |  |  | N/A | N/A |
| 5 | PCC/CC1 |  |  |  | CP-OFDM 64QAM | Outer_Full |
|  | SCC1/CC2 |  |  |  | CP-OFDM 64QAM | Outer_Full |
|  | Wgap |  |  |  | N/A | N/A |
|  | SCC2/CC3 |  |  |  | N/A | N/A |
|  | SCC3/CC4 |  |  |  | N/A | N/A |
| Default Test Settings for a CA_nX(D-H)_UL_nXD, CA_nX(D-P)_UL_nXD, CA_nX(E-O)_UL_nXO Configuration (800MHz <= Cumulative aggregated BWchannel <= 1400MHz) |  |  |  |  |  |  |
| 1 | PCC/CC1 | Default | Default | - | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
|  | SCC1/CC2 |  |  |  | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
|  | Wgap |  |  |  | N/A | N/A |
|  | SCC2/CC3 |  |  |  | N/A | N/A |
|  | SCC3/CC4 |  |  |  | N/A | N/A |
|  | SCC4/CC5 |  |  |  | N/A | N/A |
| 2 | PCC/CC1 |  |  |  | DFT-s-OFDM QPSK | Outer_Full |
|  | SCC1/CC2 |  |  |  | DFT-s-OFDM QPSK | Outer_Full |
|  | Wgap |  |  |  | N/A | N/A |
|  | SCC2/CC3 |  |  |  | N/A | N/A |
|  | SCC3/CC4 |  |  |  | N/A | N/A |
|  | SCC4/CC5 |  |  |  | N/A | N/A |
| 3 | PCC/CC1 |  |  |  | DFT-s-OFDM 16QAM | Outer_Full |
|  | SCC1/CC2 |  |  |  | DFT-s-OFDM 16QAM | Outer_Full |
|  | Wgap |  |  |  | N/A | N/A |
|  | SCC2/CC3 |  |  |  | N/A | N/A |
|  | SCC3/CC4 |  |  |  | N/A | N/A |
|  | SCC4/CC5 |  |  |  | N/A | N/A |
| 4 | PCC/CC1 |  |  |  | CP-OFDM 16QAM | Outer_Full |
|  | SCC1/CC2 |  |  |  | CP-OFDM 16QAM | Outer_Full |
|  | Wgap |  |  |  | N/A | N/A |
|  | SCC2/CC3 |  |  |  | N/A | N/A |
|  | SCC3/CC4 |  |  |  | N/A | N/A |
|  | SCC4/CC5 |  |  |  | N/A | N/A |
| 5 | PCC/CC1 |  |  |  | CP-OFDM 64QAM | Outer_Full |
|  | SCC1/CC2 |  |  |  | CP-OFDM 64QAM | Outer_Full |
|  | Wgap |  |  |  | N/A | N/A |
|  | SCC2/CC3 |  |  |  | N/A | N/A |
|  | SCC3/CC4 |  |  |  | N/A | N/A |
|  | SCC4/CC5 |  |  |  | N/A | N/A |
| Default Test Settings for a CA_nX(D-I)_UL_nXD, CA_nX(D-Q)_UL_nXD, CA_nX(G-I)_UL_nXG Configuration (800MHz <= Cumulative aggregated BWchannel <= 1400MHz) |  |  |  |  |  |  |
| 1 | PCC/CC1 | Default | Default | - | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
|  | SCC1/CC2 |  |  |  | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
|  | Wgap |  |  |  | N/A | N/A |
|  | SCC2/CC3 |  |  |  | N/A | N/A |
|  | SCC3/CC4 |  |  |  | N/A | N/A |
|  | SCC4/CC5 |  |  |  | N/A | N/A |
|  | SCC5/CC6 |  |  |  | N/A | N/A |
| 2 | PCC/CC1 |  |  |  | DFT-s-OFDM QPSK | Outer_Full |
|  | SCC1/CC2 |  |  |  | DFT-s-OFDM QPSK | Outer_Full |
|  | Wgap |  |  |  | N/A | N/A |
|  | SCC2/CC3 |  |  |  | N/A | N/A |
|  | SCC3/CC4 |  |  |  | N/A | N/A |
|  | SCC4/CC5 |  |  |  | N/A | N/A |
|  | SCC5/CC6 |  |  |  | N/A | N/A |
| 3 | PCC/CC1 |  |  |  | DFT-s-OFDM 16QAM | Outer_Full |
|  | SCC1/CC2 |  |  |  | DFT-s-OFDM 16QAM | Outer_Full |
|  | Wgap |  |  |  | N/A | N/A |
|  | SCC2/CC3 |  |  |  | N/A | N/A |
|  | SCC3/CC4 |  |  |  | N/A | N/A |
|  | SCC4/CC5 |  |  |  | N/A | N/A |
|  | SCC5/CC6 |  |  |  | N/A | N/A |
| 4 | PCC/CC1 |  |  |  | CP-OFDM 16QAM | Outer_Full |
|  | SCC1/CC2 |  |  |  | CP-OFDM 16QAM | Outer_Full |
|  | Wgap |  |  |  | N/A | N/A |
|  | SCC2/CC3 |  |  |  | N/A | N/A |
|  | SCC3/CC4 |  |  |  | N/A | N/A |
|  | SCC4/CC5 |  |  |  | N/A | N/A |
|  | SCC5/CC6 |  |  |  | N/A | N/A |
| 5 | PCC/CC1 |  |  |  | CP-OFDM 64QAM | Outer_Full |
|  | SCC1/CC2 |  |  |  | CP-OFDM 64QAM | Outer_Full |
|  | Wgap |  |  |  | N/A | N/A |
|  | SCC2/CC3 |  |  |  | N/A | N/A |
|  | SCC3/CC4 |  |  |  | N/A | N/A |
|  | SCC4/CC5 |  |  |  | N/A | N/A |
|  | SCC5/CC6 |  |  |  | N/A | N/A |
| Default Test Settings for a CA_nX(D-G)_UL_nXD, CA_nX(D-O)_UL_nXD Configuration (400MHz <= Cumulative aggregated BWchannel <800MHz) |  |  |  |  |  |  |
| 1 | PCC/CC1 | 200MHz | Default | - | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
|  | SCC1/CC2 | 200MHz |  |  | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
|  | Wgap | 190MHz |  |  | N/A | N/A |
|  | SCC2/CC3 | 100MHz |  |  | N/A | N/A |
|  | SCC3/CC4 | 100MHz |  |  | N/A | N/A |
| 2 | PCC/CC1 | 200MHz |  |  | CP-OFDM 16QAM | Outer_Full |
|  | SCC1/CC2 | 200MHz |  |  | CP-OFDM 16QAM | Outer_Full |
|  | Wgap | 190MHz |  |  | N/A | N/A |
|  | SCC2/CC3 | 100MHz |  |  | N/A | N/A |
|  | SCC3/CC4 | 100MHz |  |  | N/A | N/A |
| 3 | PCC/CC1 | 200MHz |  |  | CP-OFDM 64QAM | Outer_Full |
|  | SCC1/CC2 | 200MHz |  |  | CP-OFDM 64QAM | Outer_Full |
|  | Wgap | 190MHz |  |  | N/A | N/A |
|  | SCC2/CC3 | 100MHz |  |  | N/A | N/A |
|  | SCC3/CC4 | 100MHz |  |  | N/A | N/A |
| Default Test Settings for a CA_nX(D-G)_UL_nXG, CA_nX(D-O)_UL_nXO Configuration (400MHz <= Cumulative aggregated BWchannel <800MHz) |  |  |  |  |  |  |
| 1 | PCC/CC1 | 100MHz | Default | - | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
|  | SCC1/CC2 | 100MHz |  |  | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
|  | Wgap | 190MHz |  |  | N/A | N/A |
|  | SCC2/CC3 | 200MHz |  |  | N/A | N/A |
|  | SCC3/CC4 | 200MHz |  |  | N/A | N/A |
| 2 | PCC/CC1 | 100MHz |  |  | CP-OFDM 16QAM | Outer_Full |
|  | SCC1/CC2 | 100MHz |  |  | CP-OFDM 16QAM | Outer_Full |
|  | Wgap | 190MHz |  |  | N/A | N/A |
|  | SCC2/CC3 | 200MHz |  |  | N/A | N/A |
|  | SCC3/CC4 | 200MHz |  |  | N/A | N/A |
| 3 | PCC/CC1 | 100MHz |  |  | CP-OFDM 64QAM | Outer_Full |
|  | SCC1/CC2 | 100MHz |  |  | CP-OFDM 64QAM | Outer_Full |
|  | Wgap | 190MHz |  |  | N/A | N/A |
|  | SCC2/CC3 | 200MHz |  |  | N/A | N/A |
|  | SCC3/CC4 | 200MHz |  |  | N/A | N/A |
| Default Test Settings for a CA_nX(D-H)_UL_nXD, CA_nX(D-P)_UL_nXD, Configuration (400MHz <= Cumulative aggregated BWchannel < 800MHz) |  |  |  |  |  |  |
| 1 | PCC/CC1 | 200MHz | Default | - | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
|  | SCC1/CC2 | 200MHz |  |  | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
|  | Wgap | 90MHz |  |  | N/A | N/A |
|  | SCC2/CC3 | 100MHz |  |  | N/A | N/A |
|  | SCC3/CC4 | 100MHz |  |  | N/A | N/A |
|  | SCC4/CC5 | 100MHz |  |  | N/A | N/A |
| 2 | PCC/CC1 | 200MHz |  |  | CP-OFDM 16QAM | Outer_Full |
|  | SCC1/CC2 | 200MHz |  |  | CP-OFDM 16QAM | Outer_Full |
|  | Wgap | 90MHz |  |  | N/A | N/A |
|  | SCC2/CC3 | 100MHz |  |  | N/A | N/A |
|  | SCC3/CC4 | 100MHz |  |  | N/A | N/A |
|  | SCC4/CC5 | 100MHz |  |  | N/A | N/A |
| 3 | PCC/CC1 | 200MHz |  |  | CP-OFDM 64QAM | Outer_Full |
|  | SCC1/CC2 | 200MHz |  |  | CP-OFDM 64QAM | Outer_Full |
|  | Wgap | 90MHz |  |  | N/A | N/A |
|  | SCC2/CC3 | 100MHz |  |  | N/A | N/A |
|  | SCC3/CC4 | 100MHz |  |  | N/A | N/A |
|  | SCC4/CC5 | 100MHz |  |  | N/A | N/A |
| Default Test Settings for a CA_nX(O-E)_UL_nXO Configuration (400MHz <= Cumulative aggregated BWchannel < 800MHz) |  |  |  |  |  |  |
| 1 | PCC/CC1 | 100MHz | Default | - | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
|  | SCC1/CC2 | 100MHz |  |  | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
|  | Wgap | 90MHz |  |  | N/A | N/A |
|  | SCC2/CC3 | 100MHz |  |  | N/A | N/A |
|  | SCC3/CC4 | 200MHz |  |  | N/A | N/A |
|  | SCC4/CC5 | 200MHz |  |  | N/A | N/A |
| 2 | PCC/CC1 | 100MHz |  |  | CP-OFDM 16QAM | Outer_Full |
|  | SCC1/CC2 | 100MHz |  |  | CP-OFDM 16QAM | Outer_Full |
|  | Wgap | 90MHz |  |  | N/A | N/A |
|  | SCC2/CC3 | 100MHz |  |  | N/A | N/A |
|  | SCC3/CC4 | 200MHz |  |  | N/A | N/A |
|  | SCC4/CC5 | 200MHz |  |  | N/A | N/A |
| 3 | PCC/CC1 | 100MHz |  |  | CP-OFDM 64QAM | Outer_Full |
|  | SCC1/CC2 | 100MHz |  |  | CP-OFDM 64QAM | Outer_Full |
|  | Wgap | 90MHz |  |  | N/A | N/A |
|  | SCC2/CC3 | 100MHz |  |  | N/A | N/A |
|  | SCC3/CC4 | 200MHz |  |  | N/A | N/A |
|  | SCC4/CC5 | 200MHz |  |  | N/A | N/A |
| Default Test Settings for a CA_nX(D-I)_UL_nXD, CA_nX(D-Q)_UL_nXD Configuration (400MHz <= Cumulative aggregated BWchannel < 800MHz) |  |  |  |  |  |  |
| 1 | PCC/CC1 | 100MHz | Default | - | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
|  | SCC1/CC2 | 200MHz |  |  | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
|  | Wgap | 90MHz |  |  | N/A | N/A |
|  | SCC2/CC3 | 100MHz |  |  | N/A | N/A |
|  | SCC3/CC4 | 100MHz |  |  | N/A | N/A |
|  | SCC4/CC5 | 100MHz |  |  | N/A | N/A |
|  | SCC5/CC6 | 100MHz |  |  | N/A | N/A |
| 2 | PCC/CC1 | 100MHz |  |  | CP-OFDM 16QAM | Outer_Full |
|  | SCC1/CC2 | 200MHz |  |  | CP-OFDM 16QAM | Outer_Full |
|  | Wgap | 90MHz |  |  | N/A | N/A |
|  | SCC2/CC3 | 100MHz |  |  | N/A | N/A |
|  | SCC3/CC4 | 100MHz |  |  | N/A | N/A |
|  | SCC4/CC5 | 100MHz |  |  | N/A | N/A |
|  | SCC5/CC6 | 100MHz |  |  | N/A | N/A |
| 3 | PCC/CC1 | 100MHz |  |  | CP-OFDM 64QAM | Outer_Full |
|  | SCC1/CC2 | 200MHz |  |  | CP-OFDM 64QAM | Outer_Full |
|  | Wgap | 90MHz |  |  | N/A | N/A |
|  | SCC2/CC3 | 100MHz |  |  | N/A | N/A |
|  | SCC3/CC4 | 100MHz |  |  | N/A | N/A |
|  | SCC4/CC5 | 100MHz |  |  | N/A | N/A |
|  | SCC5/CC6 | 100MHz |  |  | N/A | N/A |
| Default Test Settings for a CA_nX(G-I)_UL_nXG Configuration (400MHz <= Cumulative aggregated BWchannel < 800MHz) |  |  |  |  |  |  |
| 1 | PCC/CC1 | 100MHz | Default | - | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
|  | SCC1/CC2 | 100MHz |  |  | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
|  | Wgap | 190MHz |  |  | N/A | N/A |
|  | SCC2/CC3 | 100MHz |  |  | N/A | N/A |
|  | SCC3/CC4 | 100MHz |  |  | N/A | N/A |
|  | SCC4/CC5 | 100MHz |  |  | N/A | N/A |
|  | SCC5/CC6 | 100MHz |  |  | N/A | N/A |
| 2 | PCC/CC1 | 100MHz |  |  | CP-OFDM 16QAM | Outer_Full |
|  | SCC1/CC2 | 100MHz |  |  | CP-OFDM 16QAM | Outer_Full |
|  | Wgap | 190MHz |  |  | N/A | N/A |
|  | SCC2/CC3 | 100MHz |  |  | N/A | N/A |
|  | SCC3/CC4 | 100MHz |  |  | N/A | N/A |
|  | SCC4/CC5 | 100MHz |  |  | N/A | N/A |
|  | SCC5/CC6 | 100MHz |  |  | N/A | N/A |
| 3 | PCC/CC1 | 100MHz |  |  | CP-OFDM 64QAM | Outer_Full |
|  | SCC1/CC2 | 100MHz |  |  | CP-OFDM 64QAM | Outer_Full |
|  | Wgap | 190MHz |  |  | N/A | N/A |
|  | SCC2/CC3 | 100MHz |  |  | N/A | N/A |
|  | SCC3/CC4 | 100MHz |  |  | N/A | N/A |
|  | SCC4/CC5 | 100MHz |  |  | N/A | N/A |
|  | SCC5/CC6 | 100MHz |  |  | N/A | N/A |
| Default Test Settings for a CA_nX(D-O)_UL_nXD Configuration (Cumulative aggregated BWchannel <400MHz) |  |  |  |  |  |  |
| 1 | PCC/CC1 | 50MHz | Default | - | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
|  | SCC1/CC2 | 200MHz |  |  | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
|  | Wgap | 40MHz |  |  | N/A | N/A |
|  | SCC2/CC3 | 50MHz |  |  | N/A | N/A |
|  | SCC3/CC4 | 50MHz |  |  | N/A | N/A |
| 2 | PCC/CC1 | 50MHz |  |  | CP-OFDM 16QAM | Outer_Full |
|  | SCC1/CC2 | 200MHz |  |  | CP-OFDM 16QAM | Outer_Full |
|  | Wgap | 40MHz |  |  | N/A | N/A |
|  | SCC2/CC3 | 50MHz |  |  | N/A | N/A |
|  | SCC3/CC4 | 50MHz |  |  | N/A | N/A |
| 3 | PCC/CC1 | 50MHz |  |  | CP-OFDM 64QAM | Outer_Full |
|  | SCC1/CC2 | 200MHz |  |  | CP-OFDM 64QAM | Outer_Full |
|  | Wgap | 40MHz |  |  | N/A | N/A |
|  | SCC2/CC3 | 50MHz |  |  | N/A | N/A |
|  | SCC3/CC4 | 50MHz |  |  | N/A | N/A |
| Default Test Settings for a CA_nX(D-O)_UL_nXO Configuration (Cumulative aggregated BWchannel <400MHz) |  |  |  |  |  |  |
| 1 | PCC/CC1 | 50MHz | Default | - | N/A | N/A |
|  | SCC1/CC2 | 200MHz |  |  | N/A | N/A |
|  | Wgap | 40MHz |  |  | N/A | N/A |
|  | SCC2/CC3 | 50MHz |  |  | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
|  | SCC3/CC4 | 50MHz |  |  | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
| 2 | PCC/CC1 | 50MHz |  |  | N/A | N/A |
|  | SCC1/CC2 | 200MHz |  |  | N/A | N/A |
|  | Wgap | 40MHz |  |  | N/A | N/A |
|  | SCC2/CC3 | 50MHz |  |  | CP-OFDM 16QAM | Outer_Full |
|  | SCC3/CC4 | 50MHz |  |  | CP-OFDM 16QAM | Outer_Full |
| 3 | PCC/CC1 | 50MHz |  |  | N/A | N/A |
|  | SCC1/CC2 | 200MHz |  |  | N/A | N/A |
|  | Wgap | 40MHz |  |  | N/A | N/A |
|  | SCC2/CC3 | 50MHz |  |  | CP-OFDM 64QAM | Outer_Full |
|  | SCC3/CC4 | 50MHz |  |  | CP-OFDM 64QAM | Outer_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-2. NOTE 2: PCC/CCi and SCC/CCj means PCC is on component carrier CCi and SCC is on component carrier CCj, with CCi or CCj frequencies defined in TS38.508-1 [10]. |  |  |  |  |  |  |

Table 6.2A.2.1.4.1-4: Intra-band Contiguous UL CA Test Configuration Table (Power Class 1, Non-contiguous allocation)

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1.2.3 and 4.3.1.2.4 for different CA bandwidth classes |  |  |  | For intra-band contiguous CA: Mid range. For intra-band non-contiguous CA: FFS. |  |  |
| Test CC Combination setting (aggregated BW of the CA configuration) as specified in TS 38.508-1 [10] subclause 4.3.1.2.3 and 4.3.1.2.4 for the CA Configuration across bandwidth combination sets supported by the UE |  |  |  | Highest aggregated channel bandwidth of the CA configuration |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  | 120 kHz |  |  |
| Test Parameters |  |  |  |  |  |  |
| Test ID | CC & Mapping (NOTE 2) | ChBw(MHz) | Test frequency | DL RB allocation | UL Modulation | UL RB allocation |
| Default Test Settings for a CA_nXB, CA_nXD, CA_XG, CA_nXO Configuration |  |  |  |  |  |  |
| 1 | PCC/CC1 | Default | Default | - | CP-OFDM 64QAM | Outer_1RB_Left |
|  | SCC/CC2 |  |  |  | CP-OFDM 64QAM | Outer_1RB_Right |
| 2 | PCC/CC1 |  |  |  | DFT-s-OFDM Pi/2 BPSK | [Outer_0.9_Left] |
|  | SCC/CC2 |  |  |  | DFT-s-OFDM Pi/2 BPSK | [Outer_0.9_Right] |
| 3 | PCC/CC1 |  |  |  | DFT-s-OFDM Pi/2 QPSK | [Outer_0.9_Left] |
|  | SCC/CC2 |  |  |  | DFT-s-OFDM Pi/2 QPSK | [Outer_0.9_Right] |
| Default Test Settings for a CA_nX(D-G), CA_nX(D-O) Configuration |  |  |  |  |  |  |
| 1 | PCC/CC1 | Default | Default | - | DFT-s-OFDM QPSK | Outer_1RB_Left |
|  | SCC1/CC2 |  |  |  | DFT-s-OFDM QPSK | Outer_1RB_Right |
|  | Wgap |  |  |  | N/A | N/A |
|  | SCC2/CC3 |  |  |  | N/A | N/A |
|  | SCC3/CC4 |  |  |  | N/A | N/A |
| 2 | PCC/CC1 |  |  |  | DFT-s-OFDM Pi/2 BPSK | [Outer_0.9_Left] |
|  | SCC1/CC2 |  |  |  | DFT-s-OFDM Pi/2 BPSK | [Outer_0.9_Right] |
|  | Wgap |  |  |  | N/A | N/A |
|  | SCC2/CC3 |  |  |  | N/A | N/A |
|  | SCC3/CC4 |  |  |  | N/A | N/A |
| 3 | PCC/CC1 |  |  |  | DFT-s-OFDM Pi/2 QPSK | [Outer_0.9_Left] |
|  | SCC1/CC2 |  |  |  | DFT-s-OFDM Pi/2 QPSK | [Outer_0.9_Right] |
|  | Wgap |  |  |  | N/A | N/A |
|  | SCC2/CC3 |  |  |  | N/A | N/A |
|  | SCC3/CC4 |  |  |  | N/A | N/A |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-2. NOTE 2: PCC/CCi and SCC/CCj means PCC is on component carrier CCi and SCC is on component carrier CCj, with CCi or CCj frequencies defined in TS38.508-1 [10]. |  |  |  |  |  |  |

<< End of changes >>