3GPP TSG-RAN5 Meeting #98	R5-231385r1

# Athens, Greece, Feb 27th - Mar 3rd, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.508 | CR | 1415 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of eMTC NTN SIG test freqs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN_req-UEConTest |  |  |  |  |  | Date: |  |  | 2023-02-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Test freqs for eMTC NTN SIG testing have been missing and need to be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Test freqs for eMTC NTN SIG testing have been updated in TC 6.2.3 and 6.2.3.1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The work item cannot be finalized. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3, 6.2.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1:Modify eMTC NTN SIG test freqs f2, f3 and f4 as per MCC suggestion. |  |  |  |  |  |  |  |  |

<<< START OF CHANGES >>>

6.2.3	Default test frequencies

The default channel bandwidth of 1.4/5/10/20 MHz is applied to the signalling test. The test frequencies are defined so that no frequency overlapping takes place, in order to avoid unnecessary inter-frequency interference.

For sidelink direct discovery, the default channel bandwidth of 5/10/15/20 MHz is applied to the signalling test for all ProSe bands except band 14 where default bandwidth is 5/10 MHz and bands 31, 72 where default bandwidth is 5 MHz. For sidelink direct communication the default channel bandwidth of 10 MHz is applied to the signalling test except bands 31, 72 where bandwidth is 5 MHz.

For V2X communication, the default channel bandwidth of 10/20 MHz is applied to the signalling test for all V2X bands.

For eMTC NTN, the default channel bandwidth of 1.4 MHz is applied to the signalling test for all eMTC NTN bands.

For Band 13, Band 18,  Band 31 and Band 72, only one test frequency f1 is defined. All operating Bands except Band 13, Band 18, Band 31 and Band 72 can accommodate at least two test frequencies f1 and f2 (f1<f2). An additional test frequency f3 can be defined for the operating Bands with at least triple of the default bandwidth. The fourth test frequency f4 (f3<f1<f4<f2) is applicable to the operating Bands which have at least quadruple of the default bandwidth.

To the single cell signalling test with channel bandwidth different from the default bandwidths of the operating bands, Mid Range defined in clause 4.3.1 is applied.

6.2.3.1	Test frequencies for signalling test

Test frequencies for signalling test are specified in table 6.2.3.1-1, 6.2.3.1-1a and 6.2.3.1-1d for FDD and table 6.2.3.1-2 and 6.2.3.1-2a for TDD. Except f4 and a few f1, f5 which are specified according to EARFCN of the concerned operating Bands, the majority of the test frequencies in table 6.2.3.1-1, 6.2.3.1-1a, 6.2.3.1-1d, 6.2.3.1-2 and 6.2.3.1-2a are specified in terms of Low, Mid and High which are referred to the Low Range, Mid Range and High Range in clause 4.3.1.

Test frequencies for signalling test of MFBI are specified in table 6.2.3.1-1b and 6.2.3.1-1c for FDD and table 6.2.3.1-2b and 6.2.3.1-2c for TDD. Except f4 and a few f1, f5 which are specified according to EARFCN of the concerned operating Bands, the majority of the test frequencies in table 6.2.3.1-1b, 6.2.3.1-1c, 6.2.3.1-2b and 6.2.3.1-2c are specified in terms of Low, Mid and High which are referred to the Low Range, Mid Range and High Range in clause 4.3.1.

Table 6.2.3.1-1: Test frequencies for E-UTRA FDD(5MHz)

| E-UTRA Operating Band | Bandwidth [MHz] | f1, f5 |  | f2, f6 |  | f3, f7 |  | f4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | NUL | NDL | NUL | NDL | NUL | NDL | NUL | NDL |
| 1 | 60 | Mid | Mid | High | High | Low | Low | 18350 | 350 |
| 2 | 60 | Mid | Mid | High | High | Low | Low | 18950 | 950 |
| 3 | 75 | Mid | Mid | High | High | Low | Low | 19625 | 1625 |
| 4 | 45 | Mid | Mid | High | High | Low | Low | 20225 | 2225 |
| 5 | 25 | Mid | Mid | High | High | Low | Low | 20575 | 2575 |
| 6 | 10 | Low | Low | High | High | N/A | N/A | N/A | N/A |
| 7 | 70 | Mid | Mid | High | High | Low | Low | 21150 | 3150 |
| 8 | 35 | Mid | Mid | High | High | Low | Low | 21675 | 3675 |
| 9 | 35 | Mid | Mid | High | High | Low | Low | 22025 | 4025 |
| 10 | 60 | Mid | Mid | High | High | Low | Low | 22500 | 4500 |
| 12 | 17 | Low | Low | High | High | Mid | Mid | N/A | N/A |
| 14 | 10 | Low | Low | High | High | N/A | N/A | N/A | N/A |
| … |  |  |  |  |  |  |  |  |  |
| 17 | 12 | Low | Low | High | High | N/A | N/A | N/A | N/A |
| 19 | 15 | Mid | Mid | High | High | Low | Low | N/A | N/A |
| 21 | 15 | Mid | Mid | High | High | Low | Low | N/A | N/A |
| 22 | 80 | Mid | Mid | High | High | Low | Low | 24950 | 6950 |
| ... |  |  |  |  |  |  |  |  |  |
| 23 | 20 | 25575 | 7575 | High | High | Low | Low | 25625 | 7625 |
| 24 | 342 | Low | Low | High | High | N/A | N/A | N/A | N/A |
| 25 | 65 | Mid | Mid | High | High | Low | Low | 26415 | 8415 |
| 26 | 35 | Mid | Mid | High | High | Low | Low | 26915 | 8915 |
| 27 | 17 | Mid | Mid | High | High | Low | Low | N/A | N/A |
| 28 | 45 | Mid | Mid | High | High | Low | Low | 27560 | 9560 |
| 30 | 10 | Low | Low | High | High | N/A | N/A | N/A | N/A |
| 31 | 5 | Mid | Mid | N/A | N/A | N/A | N/A | N/A | N/A |
| … |  |  |  |  |  |  |  |  |  |
| 65 | 90 | Mid | Mid | High | High | Low | Low | 131422 | 65886 |
| 66 | 70+901 | Mid | Mid | High | High | Low | Low | 132272 | 66736 |
| 68 | 30 | Mid | Mid | High | High | Low | Low | 132867 | 67731 |
| … |  |  |  |  |  |  |  |  |  |
| 70 | 15+251 | Mid | Mid | High | High | Low | Low | N/A | N/A |
| 71 | 35 | Mid | Mid | High | High | Low | Low | 133247 | 68711 |
| 72 | 5 | Mid | Mid | N/A | N/A | N/A | N/A | N/A | N/A |
| 73 | 5 | Mid | Mid | N/A | N/A | N/A | N/A | N/A | N/A |
| 74 | 43 | Mid | Mid | High | High | Low | Low | 133882 | 69336 |
| 85 | 18 | Low | Low | High | High | Mid | Mid | N/A | N/A |
| 87 | 5 | Mid | Mid | N/A | N/A | N/A | N/A | N/A | N/A |
| 88 | 5 | Mid | Mid | N/A | N/A | N/A | N/A | N/A | N/A |
| NOTE 1: Asymmetric operating band (UL + DL) NOTE 2: DL operation in this band is restricted to 1526 – 1536 MHz and UL operation is restricted to 1627.5 – 1637.5 MHz and 1646.5 – 1656.5 MHz. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.1-1a: Test frequencies for E-UTRA FDD(10MHz)

| E-UTRA Operating Band | Bandwidth [MHz] | f1, f5 |  | f2, f6 |  | f3, f7 |  | f4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | NUL | NDL | NUL | NDL | NUL | NDL | NUL | NDL |
| 11 | 20 | Low | Low | High | High | N/A | N/A | N/A | N/A |
| 13 | 10 | Mid | Mid | N/A | N/A | N/A | N/A | N/A | N/A |
| 18 | 15 | Low | Low | N/A | N/A | N/A | N/A | N/A | N/A |
| 20 | 30 | Mid | Mid | High | High | Low | Low | N/A | N/A |
| 23 | 20 | Low | Low | High | High | N/A | N/A | N/A | N/A |

Table 6.2.3.1-1b: Test frequencies for E-UTRA FDD(5MHz) MFBI

| E-UTRA Operating Band | MFBI Overlapping Band | Bandwidth [MHz] | f1, f5 |  | f2, f6 |  | f3, f7 |  | f4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | NUL | NDL | NUL | NDL | NUL | NDL | NUL | NDL |
| 2 | 25 | 60 | Mid | Mid | High | High | Low | Low | 18950 | 950 |
| 3 | 9 | 35 | Mid | Mid | High | High | Low | Low | 19824 | 1824 |
| 4 | 10 | 45 | Mid | Mid | High | High | Low | Low | 20225 | 2225 |
| 4 | 66 | 45 | Mid | Mid | High | High | Low | Low | 20225 | 2225 |
| 5 | 18 | 6 | Mid | Mid | N/A | N/A | N/A | N/A | N/A | N/A |
| 5 | 19 | 25 | Mid | Mid | High | High | Low | Low | 20585 | 2585 |
| 5 | 26 | 25 | Mid | Mid | High | High | Low | Low | 20575 | 2575 |
| 9 | 3 | 35 | Mid | Mid | High | High | Low | Low | 22025 | 4025 |
| 10 | 4 | 45 | Mid | Mid | High | High | Low | Low | 22425 | 4425 |
| 10 | 66 | 60 | Mid | Mid | High | High | Low | Low | 22425 | 4425 |
| 12 | 17 | 12 | Low | Low | High | High | N/A | N/A | N/A | N/A |
| 17 | 12 | 12 | Low | Low | High | High | N/A | N/A | N/A | N/A |
| 19 | 5 | 15 | Mid | Mid | High | High | Low | Low | N/A | N/A |
| 19 | 26 | 15 | Mid | Mid | High | High | Low | Low | N/A | N/A |
| 25 | 2 | 60 | Mid | Mid | High | High | Low | Low | 26415 | 8415 |
| 26 | 5 | 25 | Mid | Mid | High | High | Low | Low | 26915 | 8915 |
| 26 | 18 | 15 | Mid | Mid | High | High | Low | Low | N/A | N/A |
| 26 | 19 | 15 | Low | Low | High | High | N/A | N/A | N/A | N/A |
| 26 | 27 | 10 | Mid | Mid | High | High | Low | Low | N/A | N/A |
| 27 | 18 | 9 | Low | Low | High | High | N/A | N/A | N/A | N/A |
| 27 | 26 | 10 | Low | Low | High | High | N/A | N/A | N/A | N/A |
| 66 | 4 | 45 | Mid | Mid | High | High | Low | Low | 132247 | 66711 |
| 66 | 10 | 60 | Mid | Mid | High | High | Low | Low | 132247 | 66711 |

Table 6.2.3.1-1c: Test frequencies for E-UTRA FDD(10MHz) MFBI

| E-UTRA Operating Band | MFBI Overlapping Band | Bandwidth [MHz] | f1, f5 |  | f2, f6 |  | f3, f7 |  | f4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | NUL | NDL | NUL | NDL | NUL | NDL | NUL | NDL |
| 18 | 5 | 6 | Low | Low | N/A | N/A | N/A | N/A | N/A | N/A |
| 18 | 26 | 15 | Low | Low | N/A | N/A | N/A | N/A | N/A | N/A |
| 18 | 27 | 9 | Low | Low | N/A | N/A | N/A | N/A | N/A | N/A |

Table 6.2.3.1-1d: Test frequencies for E-UTRA FDD(1.4MHz)

| E-UTRA Operating Band | Bandwidth [MHz] | f1, f5 |  | f2, f6 |  | f3, f7 |  | f4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | NUL | NDL | NUL | NDL | NUL | NDL | NUL | NDL |
| 255 | 34 | Mid | Mid | High | High | Low | Low | 261724 | 228956 |
| 256 | 30 | Mid | Mid | High | High | Low | Low | 262039 | 229271 |

Table 6.2.3.1-2: Test frequencies for E-UTRA TDD (5MHz)

| E-UTRA Operating Band | Bandwidth [MHz] | f1, f5 | f2, f6 | F3, f7 | f4 |
| --- | --- | --- | --- | --- | --- |
| 33 | 20 | 36075 | High | Low | 36125 |
| 34 | 15 | Mid | High | Low | N/A |
| 35 | 60 | Mid | High | Low | 36700 |
| 36 | 60 | Mid | High | Low | 37300 |
| 37 | 20 | 37625 | High | Low | 37675 |
| … |  |  |  |  |  |
| 43 | 100 | Mid | High | Low | 44640 |
| 44 | 50 | Mid | High | Low | 46115 |
| 45 | 20 | 46665 | High | Low | 46715 |
| … |  |  |  |  |  |
| 53 | 11.5 | Low | High | N/A | N/A |

Table 6.2.3.1-2a: Test frequencies for E-UTRA TDD(20MHz)

| E-UTRA Operating Band | Bandwidth [MHz] | f1, f5 | f2, f6 | f3, f7 | f4 |
| --- | --- | --- | --- | --- | --- |
| 38 | 50 | Low | High | N/A | N/A |
| 39 | 40 | Low | High | N/A | N/A |
| 40 | 100 | Mid | High | Low | 39350 |
| 41 | 194 | Mid | High | Low | 40970 |
| 42 | 200 | Mid | High | Low | 43040 |
| … |  |  |  |  |  |
| 48 | 150 | Mid | High | Low | 56315 |

Table 6.2.3.1-2b: Test frequencies for E-UTRA TDD (5MHz) MFBI

| E-UTRA Operating Band | MFBI Overlapping Band | Bandwidth [MHz] | f1, f5 | f2, f6 | F3, f7 | f4 |
| --- | --- | --- | --- | --- | --- | --- |
| 33 | 39 | 20 | 36075 | High | Low | 36125 |

Table 6.2.3.1-2c: Test frequencies for E-UTRA TDD(20MHz) MFBI

| E-UTRA Operating Band | MFBI Overlapping Band | Bandwidth [MHz] | f1, f5 | f2, f6 | f3, f7 | f4 |
| --- | --- | --- | --- | --- | --- | --- |
| 38 | 41 | 50 | Low | High | N/A | N/A |
| 39 | 33 | 20 | Low | High | N/A | N/A |
| 41 | 38 | 50 | High | Low | Mid | N/A |

<< END OF CHANGES >>