3GPP TSG-RAN5 Meeting #98	R5-231386r1

# Athens, Greece, Feb 27th - Mar 3rd, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.508 | CR | 1416 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NB-IoT NTN SIG test freqs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN_req-UEConTest |  |  |  |  |  | Date: |  |  | 2023-02-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Test freqs for NB-IoT NTN SIG testing have been missing and need to be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Test freqs for NB-IoT NTN SIG testing have been updated in TC 8.3.2.3 and 8.3.2.3.1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The work item cannot be finalized. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.2.3 and 8.3.2.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | - This CR is related to R5-230397. TS 36.102 reference number 74 mentioned in clause 8.3.2.3 general part is introduced in R5-230397. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: Revise some editorial errors as per TF160 suggestion. |  |  |  |  |  |  |  |  |

<<< START OF CHANGES >>>

8.3.2.3	NB-IoT Default test frequencies

The default channel bandwidth of 200 kHz is applied to the signalling test. The test frequencies are defined so that no frequency overlapping takes place, in order to avoid unnecessary inter-frequency interference.

All operating bands can accommodate at least three test frequencies f1, f2 and f3 (f3<f1<f2).

For NB-IoT in-band and guard-band signalling testing, the NB-IoT frequency to be tested are low/mid/high range and are associated to the LTE frequency low/mid/high range as specified in tables 4.3.1.1.x. The E-UTRA channel bandwidth to be tested is 10MHz and NB-IoT PRB 30 for in-band and 5 MHz for guard-band for all operating bands for all test cases; unless the specific channel bandwidth is specified for the operating band below:

For Band 31 in-band testing, the E-UTRA channel bandwidth to be tested is 5 MHz.

NB-IoT NTN in-band and guard-band signalling testing for Band 255 and Band 256 is not supported in this version as per TS 36.102 [74] clause 5.4B.2.

8.3.2.3.1	NB-IoT FDD Mode Test frequencies for signalling test

Test frequencies for signalling test are specified in table 8.3.2.3.1-1 for FDD in terms of Low, Mid and High which are referred to the Low Range, Mid Range and High Range in clause 8.1.3.1.1.

Table 8.3.2.3.1-1: Test frequencies for NB-IoT FDD (200 kHz)

| NB-IoT Operating Band |  | Bandwidth [MHz] |  | f1, f5 |  |  |  | f2 |  |  |  | f3 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | NUL |  | NDL |  | NUL |  | NDL |  | NUL |  | NDL |  |
| 1 |  | 60 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| 2 |  | 60 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| 3 |  | 75 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| 4 |  | 45 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| 5 |  | 25 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7 |  | 70 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| 8 |  | 35 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 11 |  | 20 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| 12 |  | 17 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| 13 |  | 10 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| 14 |  | 10 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 17 |  | 12 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| 18 |  | 15 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| 19 |  | 15 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| 20 |  | 30 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| 21 |  | 15 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 25 |  | 65 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| 26 |  | 35 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 28 |  | 45 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 31 |  | 5 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 65 |  | 90 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| 66 |  | 70+901 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| 70 |  | 15 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| 71 |  | 35 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 73 |  | 5 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 85 |  | 18 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 87 |  | 5 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| 88 |  | 5 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| 103 |  | 1 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 255 |  | 34 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| 256 |  | 30 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| NOTE 1: Asymmetric operating band (UL + DL). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Test frequencies for signalling test of MFBI are specified in Table 8.3.2.3.1-1a for FDD in terms of Low, Mid and High which are referred to as Low Range, Mid Range and High range in clause 8.1.3.1.1.

Table 8.3.2.3.1-1a: Test frequencies for NB-IoT FDD MFBI (200 kHz)

| NB-IoT Operating Band |  | MFBI Overlapping Band |  | Bandwidth [MHz] |  | f1, f5 |  |  |  | f2 |  |  |  | f3 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | NUL |  | NDL |  | NUL |  | NDL |  | NUL |  | NDL |  |
| 2 |  | 25 |  | 60 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| 4 |  | 10 |  | 45 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| 4 |  | 66 |  | 45 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| 5 |  | 18 |  | 15 |  | Mid |  | Mid |  | 23999 |  | 5999 |  | Low |  | Low |  |
| 5 |  | 19 |  | 15 |  | Mid |  | Mid |  | 20609 |  | 2609 |  | Low |  | Low |  |
| 5 |  | 26 |  | 25 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| 12 |  | 17 |  | 12 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| 17 |  | 12 |  | 12 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| 19 |  | 5 |  | 15 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| 19 |  | 26 |  | 15 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| 25 |  | 2 |  | 60 |  | Mid |  | Mid |  | 26638 |  | 8638 |  | Low |  | Low |  |
| 26 |  | 5 |  | 25 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| 26 |  | 18 |  | 15 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| 26 |  | 19 |  | 15 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| 66 |  | 4 |  | 45 |  | Mid |  | Mid |  | 132421 |  | 66885 |  | Low |  | Low |  |
| 66 |  | 10 |  | 60 |  | Mid |  | Mid |  | 132571 |  | 67035 |  | Low |  | Low |  |

Test frequencies for signalling test are specified in table 8.3.2.3.1-2 for TDD in terms of Low, Mid and High which are referred to the Low Range, Mid Range and High Range in clause 8.1.3.1.

Table 8.3.2.3.1-2: Test frequencies for NB-IoT TDD (200 kHz)

| NB-IoT Operating Band | Bandwidth [MHz] | f1, f5 |  | f2 |  | f3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 41 | 60 | Mid | Mid | High | High | Low | Low |

8.3.2.3.2	NB-IoT TDD Mode Test frequencies for signalling test

Test frequencies for signalling test are specified in table 8.3.2.3.2-1 in terms of Low, Mid and High which are referred to the Low Range, Mid Range and High Range in clause 8.1.3.1.2

Table 8.3.2.3.2-1: Test frequencies for NB-IoT TDD (200 kHz)

| NB-IoT Operating Band | Bandwidth [MHz] | f1, f5 |  | f2 |  | f3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | NUL | NDL | NUL | NDL | NUL | NDL |
| 41 | 194 | Mid | Mid | High | High | Low | Low |
| 42 | 200 | Mid | Mid | High | High | Low | Low |
| 43 | 200 | Mid | Mid | High | High | Low | Low |

Test frequencies for signalling test of MFBI are specified in Table 8.3.2.3.2-1a for TDD in terms of Low, Mid and High which are referred to as Low Range, Mid Range and High range in clause 8.1.3.1.2.

Table 8.3.2.3.2-1a: Test frequencies for NB-IoT TDD MFBI (200 kHz)

| NB-IoT Operating Band | MFBI Overlapping Band | Bandwidth [MHz] | f1, f5 |  | f2 |  | f3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | NUL | NDL | NUL | NDL | NUL | NDL |
| 41 | 38 | 50 | Mid | Mid | High | High | Low | Low |

8.3.3	NB-IoT Reference system configurations

<< END OF CHANGES >>