3GPP TSG-RAN5 Meeting #98	R5-231910r1

# Athens, Greece, 27th Feb 2023 - 3rd Mar 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-1 | CR | 1520 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to PIDF Location object contents |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test |  |  |  |  |  | Date: |  |  | 2023-02-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The Rel-15 core specification (TS 24.229) requirement PIDF-LO structure is in accordance to RFC 4119 while in later releases (Rel-16 and beyond) RFC 5491 is added along with RFC 4119 for PIDF-LO structure. RFC 5491 includes optional “device” and “person” types as containers for geopriv-info alongside “tuple” defined in RFC 4119 & 3863. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A note added in UPDATE (Step 7) specific message contents in test case 19.3.1 and in clause A.2.1 to clarify that UE may send PIDF-LO element using the RFC 4119 or 5491 In addition Content-Type of the PIDF document ("application/pidf+xml") is added to the MIME body, more then one location element may be contained in a <geopriv> element and presence of the <usage-rules> is not needed as it is mandatory according to the XML schema. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant device mail fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 19.3.1, A.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | r1-> Please refer to highlighted text in Summary of Change |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### <Start of modified section 1>

### 19.3.1	Non-UE detectable emergency call / IM CN sends a 1xx response / UE geographical location information available or not

#### 19.3.1.1	Definition

Test to verify that the UE acts correctly when it receives a 1xx response to an initial request for a dialog from the IM CN, the response containing a P-Asserted-Identity header field set to an emergency number that is recognisable by the UE and the UE sends an UPDATE request with:

- Geolocation header and information if the UE is capable of obtaining location information; and

- Contact header set appropriately

#### 19.3.1.2	Conformance requirement

If the UE receives a 1xx or 200 (OK) response to an initial request for a dialog, the response containing a P-Asserted-Identity header field set to an emergency number as specified in 3GPP TS 22.101, and:

-	if a public GRUU value (pub-gruu) has been saved associated with the public user identity, the public GRUU value has not been included in the Contact header field of the initial request for a dialog as specified in RFC 5627;

-	if a public GRUU value (pub-gruu) has not been saved and a protected server port was not included in the address in the Contact header field of the initial request for a dialog; or

-	if the UE has its geographical location information available and the geographical location information has not been included in the initial request for a dialog;

then the UE shall send an UPDATE request according to RFC 3311; and

1)	if available to the UE, and if defined for the access type as specified in subclause 7.2A.4, the UE shall include in the UPDATE request a P-Access-Network-Info header field and it shall contain a location identifier such as the cell id or the identity of the I-WLAN access node;

2)	if the UE has its geographical location information available, then the UE shall include it in the UPDATE request in the following way:

I)	if the UE is aware of the URI that points to where the UE's location is stored, include the URI in the Geolocation header field and set the "inserted-by" parameter to indicate its hostport, all in accordance with RFC 6442 and set the "inserted-by" parameter to indicate its hostport, all in accordance with draft-ietf-sipcore-loca; or

II)	if the geographical location information of the UE is available to the UE, include its geographical location information as PIDF location object in accordance with RFC 4119 and include the location object in a message body with the content type application/pidf+xml in accordance with RFC 6442. The Geolocation header field is set to a Content ID and set the "inserted-by" parameter to indicate its hostport, all in accordance with RFC 6442;

3)	if the UE has no geographical location information available, the UE shall not include any geographical location information as specified in RFC 6442; and

4)	if a public GRUU value ("pub-gruu" header field parameter) has been saved associated with the public user identity, then the UE shall insert the public GRUU ("pub-gruu" header field parameter) value in the Contact header field of the UPDATE request as specified in RFC 5627; otherwise the UE shall include the address in the Contact header field set in accordance with subclause 5.1.6.8.4, item 8.

NOTE 1:	The IMS emergency specification in 3GPP TS 23.167 describes several methods how the UE can get its location information from the access network or from a server. Such methods are not in the scope of this specification.

Reference(s)

3GPP TS 24.229 [10], clauses 5.1.6.10

#### 19.3.1.3	Test purpose

To verify that if the UE is not able to detect that an emergency number has been dialled:

-	in the event the UE receives a 1xx response to an INVITE request the response containing a P-Asserted-Identity header field set to an emergency number, the UE:

-	If the UE is able to obtain its geolocation and the geographical location information has not been included in the initial request for a dialog; then the UE shall include its geolocation information in the UPDATE message

-	If the UE is not able to obtain its geolocation the UE does not include it in the UPDATE message

-	includes a Contact header in the UPDATE message with the correct contents

#### 19.3.1.4	Method of test

Initial conditions

UE contains ISIM and USIM applications or only USIM application on UICC.

Test environment shall be set up to provide the needed input to the UE, in order for the UE to derive its location, if the UE is capable of obtaining location information. This shall be done by use of the test function Update UE Location Information defined in TS 34.109 [117] or in TS 36.509 [118] depending on the RAT being used in the test case, if supported by the UE according to pc_UpdateUE_LocationInformation. Otherwise, or in addition any other suitable method may also be used.

UE has discovered P-CSCF and registered to IMS services, by executing the generic test procedure in Annex C.2 up to the last step.

SS is configured with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) configured on the UICC card equipped into the UE. SS has performed AKAv1-MD5 authentication with the UE and accepted the registration.

Test procedure

- A non-emergency MO call is initiated up following the generic procedure in Annex C.21.

Expected sequence

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1-13 |  |  |  | Steps 1-13 of Annex C.21. The UE initiates a non-emergency call. |

Specific Message Contents INVITE (Step 2)

Use the default message “INVITE” in annex A.2.1 without options A6, A7 and A8.

183 Session in Progress (Step 4)

Use the default message “183 Session in Progress” in annex A.2.3 with option A5.

UPDATE (Step 7)

Use the default message "UPDATE" in annex A.2.5 with the following exceptions:

| Header/param |  | Cond |  | Value/remark |  | Rel |  | Reference |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Geolocation |  | A1 |  |  |  | Rel-9 |  | RFC 6442 [98] |  |
| locationURI |  |  |  | cid-url indicating the Content-Id of the PIDF-LO within the multipart MIME body of INVITE request. (Note that location-by-reference URI is not allowed as the SS does not provide any external storage for location info for the UE to refer.) |  |  |  |  |  |
| Geolocation-Routing |  | A1 |  | “yes” |  | Rel-9 |  | RFC 6442 |  |
| Contact |  |  |  |  |  |  |  | RFC 3261 [15] RFC 5627 [61] |  |
| addr-spec |  | A2 |  | Public GRUU as obtained during registration as pub-gruu contact parameter of the 200 OK for REGISTER response |  |  |  |  |  |
| addr-spec |  | A3 |  | SIP URI with IP address or FQDN and protected server port of UE |  |  |  |  |  |
| addr-spec |  | A4 |  | SIP URI with IP address or FQDN and unprotected server port of UE |  |  |  |  |  |
| Content-Type |  |  |  |  |  |  |  |  |  |
| media-type |  | A1 not A1 |  | multipart/mixed application/sdp |  |  |  | TS 24.229 [10] RFC 3261 [15] |  |
| Message-body |  |  |  | If condition A1 applies, the "application/pidf+xml" multipart-mime body shall contain a PIDF-LO element mapped to the same Content-ID which can be found from the Geolocation header The PIDF-LO shall contain at least the following elements: - One or more ‘geopriv’ elements (Note), each containing: - At least one ‘location-info’ element describing the location of the UE. Note: PIDF-LO shall contain at least one <geopriv> element in a <tuple> element according to RFC 4119 or in a <device> or <person> element according to RFC 4119 and RFC 5491. |  |  |  | RFC 6442 [98] |  |

| Condition | Explanation |
| --- | --- |
| A1 | UE is capable of obtaining location information |
| A2 | obtaining and using GRUUs in the Session Initiation Protocol (SIP) (A.4/53 3GPP TS 34.229-2 [5]) |
| A3 | Not A2 and (IMS security, A.6a/2 3GPP TS 34.229-2 [5]) |
| A4 | Not A2 and (GIBA, A.6a/1 3GPP TS 34.229-2 [5]) |

180 Ringing (Step 9)

Use the default message “180 Ringing” in annex A.2.6 with options A4 and A8.

200 OK (Step 12)

Use the default message "200 OK" in annex A.3.1 with option A6.

#### 19.3.1.5	Test requirements

SS must check that in:

- Step 2 the UE sends a non-emergency INVITE with the correct contents

- Step 7 the UE sends the UPDATE message with:

  - the Geolocation header set appropriately, if the UE is capable of obtaining location information

  - Contact header set appropriately

## <End of modified section 1>

SKIPPED TEXT

<Start of modified section 2>

## A.2.1	INVITE for MO Call Setup

| Header/param | Cond | Value/remark | Rel | Reference |
| --- | --- | --- | --- | --- |
| Request-Line |  |  |  |  |
| Method |  | INVITE |  | RFC 3261 [15] |
| Request-URI | NOT A5 AND NOT A32 | px_IMS_CalleeUri px_IMS_CalleeURI may be either SIP or Tel URI. It may contain a dialstring and phone-context parameter, when calling to dialstring. When calling to dialstring SIP URI must also contain user=phone or user=dialstring parameter. The dialstring, if used, may be global, home local number or geo-local number. For home local numbers the value of phone-context parameter must equal the home domain name i.e. px_IMS_HomeDomainName. For geo-local numbers the home domain name must be prefixed by string “geo-local.” or access technology specific prefix, if the UE supports that option. Note: The way how the UE determines whether numbers in a non-international format are geo-local, home-local or relating to another network, is UE implementation specific. For instance the UE might have a UI setting |  | TS 24.229 [10] cl 5.1.2A.1.3, 5.1.2A.1.5, 7.2A.10 |
|  | A5 OR A32 | MO call has been established: px_IMS_CalleeContactUri MT call has been established: address of the remote UE as sent in the Contact header field of the dialog creating MT INVITE in the preamble |  |  |
|  | A6, A7 AND NOT (A20 OR A21) | emergency service URN beginning with urn:service:sos |  | RFC 5031 [97] |
|  | A20 AND (NOT A25) | urn:service:sos.ecall.manual | Rel-14 | RFC 8147 [149] |
|  | A21 AND (NOT A25) | urn:service:sos:ecall.automatic | Rel-14 | RFC 8147 [149] |
|  | A25 | The Test URI as per the generic “eCall test URI” which uses EFSDNURI from table 4.9.3.5-1 for “eCall capable” UEs or EFFDNURI from table 4.9.3.5-2 for “eCall only” UEs as specified in 3GPP TS 36.508 [94] | Rel-14 | RFC 8147 [149] |
| SIP-Version |  | SIP/2.0 |  |  |
| Via |  |  |  | RFC 3261 [15] |
| sent-protocol |  | SIP/2.0/UDP (when using UDP) or SIP/2.0/TCP (when using TCP) |  |  |
| sent-by | A1,A7 | IP address or FQDN and protected server port of the UE |  |  |
|  | A2,A19 | IP address or FQDN, port (optional) and not checked |  |  |
|  | A6 | IP address and, when using UDP, unprotected server port of the UE |  |  |
|  | A17 | IP address and unprotected server port of the UE |  |  |
| response-port | A6 | rport (when using UDP) |  | RFC 3581 [96] |
| via-branch |  | value starting with ‘z9hG4bK’ |  |  |
| Route |  | order of the parameters in this header must be like in the respective rows |  | RFC 3261 [15] |
| route-param | A1 | <sip:SS P-CSCF address: protected server port of SS;lr>, <sip:scscf.3gpp.org;lr> |  |  |
|  | A2,A17 | <sip:SS P-CSCF address: unprotected server port of SS (optional);lr>, <sip:scscf.3gpp.orgf;lr |  |  |
|  | A5 OR A32 | MO call has been established: URIs of the Record-Route header of 183 response in reverse order (or any other response creating the dialog according to RFC 3261 clause 12.1 [15]) MT call has been established: same value as defined for the Record-Route header in A.2.9 |  |  |
|  | A6,A19 | <sip:SS P-CSCF address: unprotected server port of SS;lr> |  |  |
|  | A7 | <sip:SS P-CSCF address: protected server port of SS;lr> |  |  |
| From |  |  |  | RFC 3261 [15] |
| addr-spec | A6 | Any SIP URI with display name as “Anonymous” or anonymous |  |  |
|  | A7,A19 | emergency public user identity (NOTE 3) |  |  |
|  | A4 | any SIP URI being subscribed and registered as listed in the XML body of the NOTIFY request; additionally when there is a P-Preferred-Identity header within the INVITE request the SIP URI shall match the URI within the P-Preferred-Identity header |  |  |
| tag | A4 | must be present, value not checked |  |  |
| addr-spec | A5 OR A32 | local SIP URI of the UE as used in any previous request in the same dialog (In the earlier requests within the same dialog this URI appears in From header within requests sent by the UE and in To header within requests sent by the SS) |  |  |
| tag | A5 OR A32 | local tag of the dialog ID (In the earlier requests within the same dialog this tag appears in From header within requests sent by the UE and in To header within requests sent by the SS) |  |  |
| To |  |  |  | RFC 3261 [15] |
| addr-spec | A6, A7 AND NOT (A20 OR A21) | emergency service URN beginning as urn:service:sos |  | RFC 5031 [97] |
|  | A20 AND (NOT A25) | urn:service:sos.ecall.manual | Rel-14 | RFC 8147 [149] |
|  | A21 AND (NOT A25) | urn:service:sos:ecall:automatic | Rel-14 | RFC 8147 [149] |
|  | A25 | The Test URI as per the generic “eCall test URI” which uses EFSDNURI from table 4.9.3.5-1 for “eCall capable” UEs or EFFDNURI from table 4.9.3.5-2 for “eCall only” UEs as specified in 3GPP TS 36.508 [94] | Rel-14 | RFC 8147 [149] |
|  | A4 | px_IMS_CalleeUri |  |  |
| tag | A4 | not present |  |  |
| addr-spec | A5 OR A32 | remote SIP URI of SS (i.e. the remote UE) as used in any previous request in the same dialog (In the earlier requests within the same dialog this URI appears in To header within requests sent by the UE and in From header within requests sent by the SS) |  |  |
| tag | A5 OR A32 | remote tag of the dialog ID (In the earlier requests within the same dialog this tag appears in To header within requests sent by the UE and in From header within requests sent by the SS) |  |  |
| Call-ID |  |  |  | RFC 3261 [15] |
| callid | A4 | value different to that received in REGISTER message |  |  |
|  | A5 OR A32 | value of Call-ID as in any previous request in the same dialog |  |  |
| Call-Info | A20,A21 |  | Rel-14 | RFC 8147 [149] |
| cid URL |  | any URL |  |  |
| purpose |  | EmergencyCallData.eCall.MSD |  |  |
| CSeq |  |  |  | RFC 3261 [15] |
| value | A4 | must be present, value not checked |  |  |
|  | A5 OR A32 | value of CSeq sent by the UE within its previous request in the same dialog but increased by one |  |  |
| method |  | INVITE |  |  |
| Supported |  | The option tags defined below shall be included additionally to any option tags defined in any specific message content, unless specified otherwise in this specific message content. |  | RFC 3261 [15] |
| option-tag | A4 OR A5 | 100rel |  |  |
|  | A13 OR A14 | norefersub | Rel-11 | RFC 4488 [126] |
|  | A4 AND A26 AND NOT (A6 OR A7 OR A19 OR A20 OR A21) | timer |  | RFC 4028 [146] |
| P-Early-Media | A16 AND NOT (A5 OR A32) |  |  | RFC 5009 [138] IR.92 [133] |
| em-param |  | supported |  |  |
| Geolocation | A8 |  | Rel-9 | RFC 6442 [98] |
| locationURI |  | cid-url indicating the Content-Id of the PIDF-LO within the multipart MIME body of INVITE request. (Note that location-by-reference URI is not allowed as the SS does not provide any external storage for location info for the UE to refer.) |  |  |
| Geolocation | NOT A8 | not present | Rel-9 | RFC 6442 [98] |
| Geolocation-Routing | A8 | “yes” | Rel-9 | RFC 6442 [98] |
| Geolocation-Routing | NOT A8 | not present | Rel-9 | RFC 6442 [98] |
| Require | A1,A7 |  |  | RFC 3261 [15] |
|  | A6 | not present |  |  |
| option-tag | A1,A7 | sec-agree |  | RFC 3329 [21] |
| Proxy-Require | A1,A7 |  |  | RFC 3261 [15] RFC 3329 [21] |
|  | A6 | not present |  |  |
| option-tag | A1,A7 | sec-agree |  |  |
| Security-Verify | A1,A7 |  |  | RFC 3329 [21] |
|  | A2,A6 | not present |  |  |
| sec-mechanism | A1,A7 | same value as Security-Server header sent by SS |  |  |
| Contact |  |  |  | RFC 3261 [15] |
| addr-spec | (A1 OR A7) AND NOT A15 | SIP URI with IP address or FQDN and protected server port of UE |  |  |
|  | (A2 OR A19) AND NOT A15 | SIP URI with IP address or FQDN and unprotected server port of UE |  |  |
|  | A15 AND NOT A6 | Public GRUU as obtained during registration as pub-gruu contact parameter of the 200 OK for REGISTER response |  | RFC 5627 [61] |
|  | A6 | SIP URI with IP address and unprotected server port of UE |  |  |
| c-p-instance | A6 | +sip.instance="<urn:gsma:imei: (gsma-specifier-defined-substring)>” where gsma-specifier-defined-substring shall be the IMEI code of the UE, coded as specified in RFC 7254 [122], without optional parameters | Rel-10 | RFC 5626 [109] RFC 7254 [122] |
|  | A3,A17,A29 | +g.3gpp.icsi-ref="urn%3Aurn-7%3A3gpp-service.ims.icsi.mmtel" (see NOTE 2, 4) |  |  |
| feature-param | A10,A31 | video |  | RFC 3840 [63] |
|  | A12 | +g.3gpp.srvcc-alerting |  | RFC 3840 [63] |
|  | A30 OR (A22 AND (A23 OR A24)) | audio |  | RFC 3840 [63] |
|  | A18 AND NOT (A5 OR A32) | +g.3gpp.ps2cs-srvcc-orig-pre-alerting |  | RFC 3840 [63] |
| Max-Forwards |  |  |  | RFC 3261 [15] |
| value |  | non-zero value |  |  |
| P-Access-Network-Info | NOT A2 |  |  | RFC 7315 [132] RFC 7913 [154] |
| access-net-spec | A1 AND A27 | access network information and, if applicable, the cell ID |  |  |
|  | A28 | access network information for NR, containing access-class parameter with value "3GPP-NR" or access-type parameter with value "3GPP-NR-FDD" or "3GPP-NR-TDD", and also containing the cell ID | Rel-15 |  |
| P-Access-Network-Info | A2(o) | header optional |  |  |
| access-net-spec |  | access network information and, if applicable, the cell ID |  |  |
| Accept | NOT (A5 OR A32) |  | Rel-7 | RFC 3261 [15] |
|  | A5(o) , A32(o) | header optional |  |  |
| media-range | A4 | application/sdp,application/3gpp-ims+xml (additional medias can be added in any order) |  |  |
|  | A13 | application/vnd.3gpp.mid-call+xml | Rel-11 |  |
|  | A14 | application/vnd.3gpp.state-and-event-info+xml | Rel-11 |  |
|  | A20,A21,A25 | application/EmergencyCallData.Control+xml | Rel-14 | RFC 8147 [149] |
| P-Preferred-Service |  |  |  | RFC 6050 [68] |
| Service-ID | A3 AND A4 | urn:urn-7:3gpp-service.ims.icsi.mmtel |  |  |
| P-Preferred-Identity |  |  |  | RFC 3325 [89] |
| PPreferredID-value | A7 | emergency public user identity (NOTE 3) |  |  |
| Recv-Info |  |  |  | RFC 6086 [139] |
| Info-package-type | A14 | g.3gpp.state-and-event |  |  |
|  | A20,A21 | EmergencyCallData.eCall.MSD | Rel-14 | RFC 8147 [149] |
| Accept-Contact |  |  |  | RFC 3841 [64] |
| ac-value | A3 AND A4 | +g.3gpp.icsi-ref="urn%3Aurn-7%3A3gpp-service.ims.icsi.mmtel" (see NOTE 2, 4) |  | TS 24.173 [65] |
|  | A10 AND A11 | video |  | IR.94 [134] NG.114 [157] |
| Proxy-Authorization | A17 |  |  | RFC 2617 [16] RFC 3310 [17] |
| username | A17 | preconfigured in the UE |  |  |
| realm | A17 | same value as received in the realm directive in the Proxy-Authorization header sent by SS |  |  |
| nonce | A17 | same value as in Proxy-Authorization header sent by SS |  |  |
| digest-uri | A17 | preconfigured in the UE |  |  |
| qop-value | A17 | auth |  |  |
| cnonce-value | A17 | value assigned by UE affecting the response calculation |  |  |
| nonce-count | A17 | counter to indicate how many times UE has sent the same value of nonce within successive INVITESs, initial value shall be 1 |  |  |
| response | A17 | response calculated by UE |  |  |
| algorithm | A17 | MD5 |  |  |
| Content-Type |  |  |  | RFC 3261 [15] |
| media-type | NOT A8 AND NOT A20 AND NOT A21 AND NOT A25 | application/sdp |  |  |
|  | A8, A20, A21, A25 | multipart/mixed;boundary=any value |  | RFC 6442 [98] RFC 8147 [149] |
| Content-Length |  | header shall be present if UE uses TCP to send this message and if there is a message body |  | RFC 3261 [15] |
| Value |  | length of message-body |  |  |
| Message-body |  | consists of one or several parts as indicated by Content-Type, and each part having actual contents as follows (SDP contents, if any, is specified in dedicated sections) |  |  |
|  | A8 | "application/pidf+xml" MIME body part containing a PIDF-LO element mapped to the same Content-ID which can be found from the Geolocation header The PIDF-LO shall contain at least the following elements: - One or more 'geopriv' elements (Note), each containing: - At least one 'location-info' element describing the location of the UE. Note: PIDF-LO shall contain at least one <geopriv> element in a <tuple> element according to RFC 4119 or in a <device> or <person> element according to RFC 4119 and RFC 5491. |  |  |
|  | A20 OR A21 OR A25 | --boundary value (as provided in SIP hdr Content-Type) Content-Type: application/EmergencyCallData.eCall.MSD Content-ID: same URL as in Call-Info header Content-Disposition: by-reference;handling=optional MSD in ASN.1 PER encoding --boundary value (as provided in SIP hdr Content-Type) | Rel-14 | RFC 8147 [149] |

| Condition |  |  | Explanation |  |  |
| --- | --- | --- | --- | --- | --- |
| A1 |  |  | IMS security (A.6a/2 3GPP TS 34.229-2 [5]) |  |  |
| A2 |  |  | GIBA (A.6a/1 3GPP TS 34.229-2 [5]) |  |  |
| A3 |  |  | UE supports MTSI (A.3A/50 3GPP TS 34.229-2 [5]) |  |  |
| A4 |  |  | INVITE creating a dialog |  |  |
| A5 |  |  | re-INVITE within a dialog for session modification |  |  |
| A6 |  |  | INVITE for creating an emergency session in case of no registration |  |  |
| A7 |  |  | INVITE for creating an emergency session within an emergency registration using IMS security |  |  |
| A8 |  |  | UE is capable of obtaining location information, has obtained its location and is setting up an emergency session |  |  |
| A9 |  |  | Void |  |  |
| A10 |  |  | UE supports video feature tag (A.12/32 3GPP TS 34.229-2 [5]) |  |  |
| A11 |  |  | INVITE for creating a video call |  |  |
| A12 |  |  | INVITE for creating a voice or video call and UE supports g.3gpp.srvcc-alerting media feature tag (A.12/34 3GPP TS 34.229-2 [5]) |  |  |
| A13 |  |  | INVITE for creating a voice call during rSRVCC and UE CS to PS SRVCC with the MSC assisted mid-call feature (A.12/42 3GPP TS 34.229-2 [5]) |  |  |
| A14 |  |  | INVITE for creating a voice call and UE supports CS to PS SRVCC for calls in alerting phase (A.12/41 3GPP TS 34.229-2 [5]) |  |  |
| A15 |  |  | obtaining and using GRUUs in the Session Initiation Protocol (SIP) (A.4/53 3GPP TS 34.229-2 [5]) |  |  |
| A16 |  |  | UE supports early media (A.12/45 3GPP TS 34.229-2 [5]) |  |  |
| A17 |  |  | SIP Digest without TLS for Fixed Broadband Access (SIP Digest without TLS, A.6a/5 3GPP TS 34.229-2 [5]) |  |  |
| A18 |  |  | UE indicates g.3gpp.ps2cs-srvcc-orig-pre-alerting media feature tag in INVITE request (A.12/36 3GPP TS 34.229-2 [5]) |  |  |
| A19 |  |  | INVITE for creating an emergency session within an emergency registration using GIBA |  |  |
| A20 |  |  | INVITE for creating an eCall over IMS session manually |  |  |
| A21 |  |  | INVITE for creating an eCall over IMS session automatically |  |  |
| A22 |  |  | UE supports audio media feature tag (A.12/56 3GPP TS 34.229-2 [5]) |  |  |
| A23 |  |  | UE uses E-UTRAN access and has received IMS voice over PS Session Supported Indication in the NAS ATTACH ACCEPT message as described in TS 24.301 [150], clauses 8.2.1 and 9.9.3.12A |  |  |
| A24 |  |  | UE uses UTRAN/GERAN access and has received IMS voice over PS Session Supported Indication in the NAS ATTACH ACCEPT message as described in TS 24.008 [12], clauses 9.4.2 and 10.5.5.23 |  |  |
| A25 |  |  | INVITE for creating a test eCall over IMS session |  |  |
| A26 |  |  | UE supports Session Timer (A.12/57 3GPP TS 34.229-2 [5]) |  |  |
| A27 |  |  | UE uses E-UTRAN access (A.18/1 3GPP TS 34.229-2 [5]) |  |  |
| A28 |  |  | UE uses NR access (A.18/5 3GPP TS 34.229-2 [5]) |  |  |
| A29 |  |  | UE includes mmtel ICSI value as per NG.114 V1.0 [157] (A.22/1 3GPP TS 34.229-2 [5]) |  |  |
| A30 |  |  | UE includes audio feature tag as per NG.114 V1.0 [157] (A.22/2 3GPP TS 34.229-2 [5]) |  |  |
| A31 |  |  | UE includes video feature tag as per NG.114 V1.0 [157] (A.22/3 3GPP TS 34.229-2 [5]) |  |  |
| A32 |  |  | re-INVITE within a dialog for purposes other than session modification |  |  |

NOTE 1:	All choices for applicable conditions are described for each header.

NOTE 2:	The “=” may include optional linear white spaces according to the EQUAL definition in chapter 25.1, RFC 3261 [15].

NOTE 3:	According to TS 24.229 clause 5.1.1.1A and 5.1.6.2 [10] when the UE is using ISIM the emergency public user identity is the first public user identity in the list stored in the ISIM; when there is no ISIM it is the default public user id if the UE registered or the temporary user id (derived from IMSI) else.

NOTE 4:	URN is the outcome of URL encoding (“Percent-Encoding” according to RFC 3986 [129]) of urn:urn-7:3gpp-service.ims.icsi.mmtel.

<End of modified section 2>