3GPP TSG-RAN5 Meeting #98	R5-231959r1

# Athens, Greece, Feburary 27th - March 3rd, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 2192 | rev | 1 | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of RedCap RRM TC 16.6.1.1 - gapless non-DRX 1Rx |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2023-02-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - To add RedCap RRM TC 16.6.1.1 - NR SA FR1 Event triggered reporting tests without gap under non-DRX for 1 Rx UE |  |  |  |  |  |  |  |  |
| Summary of change: |  | - RedCap RRM TC 16.6.1.1 - NR SA FR1 Event triggered reporting tests without gap under non-DRX for 1 Rx UE UE is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Workplan is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.6.1.1 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.522 CR 0244 TS 38.533 CR 2225,2226 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | RAN4 dependency |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | 1st revision: The IE type configured by servingCellMO-r17 should be MeasObjectId rather than MeasId. |  |  |  |  |  |  |  |  |

<Start of modified section 1>

16.6.1.1	NR SA FR1 Event triggered reporting tests without gap under non-DRX for 1 Rx UE

Editor’s note: This test case is incomplete in following aspects:

-	TT analysis is still missing.

16.6.1.1.1	Test purpose

To verify that the UE makes correct reporting of an event. This test will partly verify the intra-frequency cell search requirements in 38.133 [6] clauses 9.2B.5.1 and 9.2B.5.2.

16.6.1.1.2	Test applicability

This test applies to all types of NR RedCap UE with 1 Rx from Release 17 onwards.

16.6.1.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.6.1.0.1 and 16.6.1.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.16.6.1.1.

16.6.1.1.4	Test description

Two cells are deployed in the test, which are FR1 PCell (NR Cell 1) and a FR1 neighbour cell (NR Cell 2) on the same frequency as the PCell. The general and cell specific test parameters for PCell and neighbour cell are given in Table 16.6.1.1.4.1-3 and Table 16.6.1.1.5-1, respectively. In the measurement configuration a measurement object is configured for the frequency of the PCell, and it indicates to the UE that event-triggered reporting with Event A3 is used.

16.6.1.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 16.6.1.1.4.1-1.

Table 16.6.1.1.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 16.6.1.1-1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.6.1.1-2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.6.1.1-3 | 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.6.1.1-4 | 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test requirement and the DUT according to the parameters in Table 16.6.1.1.4.1-2.

Table 16.6.1.1.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.6.1.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.2.1 |  |
| Exceptions to connection diagram |  |  |  |

1. The test parameters for PCell and neighbour cell are given in Table 16.6.1.1.4.1-3 below.

2. Message contents are defined in clause 16.6.1.1.4.3.

3. There is one carrier and two cells specified in the test. NR Cell 1 is the cell used for connection setup with the power level set according to Annex C.1.1 and C.1.2 for this test.

Table 16.6.1.1.4.1-3: General test parameters

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  |  | Cell 1 |  |
| Neighbour cell |  |  | Cell 2 | Cell to be identified. |
| RF Channel Number |  |  | 1: Cell 1 and Cell 2 |  |
| CD-SSB configuration | Config 1,2,4 |  | SSB.1 FR1 |  |
|  | Config 3 |  | SSB.1 RedCap FR1 |  |
| NCD-SSB configuration | Config 1,2,4 |  | SSB.6 RedCap FR1 Note 1 |  |
|  | Config 3 |  | SSB.7 RedCap FR1 Note 1 |  |
| SMTC configuration for NCD-SSB | Config 1,4 |  | SMTC.4 RedCap |  |
|  | Config 2,3 |  | SMTC.2 RedCap |  |
| A3-Offset |  | dB | -4.5 |  |
| CP length |  |  | Normal |  |
| Hysteresis |  | dB | 0 |  |
| Time To Trigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| DRX |  |  | N/A | OFF |
| Time offset between serving and neighbour cells | Config 1,4 | ms | 3 | Asynchronous cells.The timing of Cell 2 is 3 ms later than the timing of Cell 1. |
|  | Config 2,3 | s | 3 | Synchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |
| Note 1: NCD-SSB is configured within dedicated RedCap DL BWP. |  |  |  |  |

16.6.1.1.4.2	Test procedure

The test consists of two successive time periods with time duration of T1 and T2 respectively. During time duration T1, the UE shall not have any timing information of NR Cell 2.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 16.6.1.1.5-1.

3. Set NR Cell 2 physical cell identity = (current NR cell 2 physical cell identity + 1) mod 1008 for current iteration of the test procedure loop

4. SS shall transmit an RRCReconfiguration message to configure RedCap-specific dedicated BWP and Event A3 triggered reporting.

5. The UE shall transmit an RRCReconfigurationComplete message. T1 starts upon the SS receives the RRCReconfigurationComplete message.

6. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 16.6.1.1.5-1. T2 starts.

7. The SS increases the number of successful tests by one if the UE transmits a MeasurementReport message triggered by event A3 satisfiying the overall delay requirement specified in clause 16.6.1.1.5, otherwise the SS increases the number of failure tests by one.

8. After the SS receive the MeasurementReport message in step 7) or when T2 expires, the SS shall:

-	transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources

or

-	switch the UE off.

9. Depending on the choice in step 8, the SS shall,

-	transmit in NR Cell 1 a Paging message including matched ng-5G-S-TMSI for the UE if an RRCRelease has been sent in step 8. If the paging fails, switches off and on the UE,

or

-	switch on the UE if the UE is switched off in step 8.

10. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

11. Repeat step 2-10 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

16.6.1.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 16.6.1.1.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 Table H.3.1-4 with A3-offset = -4.5dB Table H.3.1-5 Table H.3.1-7 with Condition INTRA-FREQ Table H.3.1-8 with Condition SSB RLM |

Table 16.6.1.1.4.3-2: MeasObjectNR (step 4, test procedure)

| Derivation Path: Table H.3.1-3 with condition INTRA-FREQ MO |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectNR ::= SEQUENCE { |  |  |  |
| ssbFrequency | ARFCN-ValueNR of NCD-SSB |  |  |
| smtc1 | SSB-MTC specified in 38.508-1 [14] Table 7.3.1-3 with condition SMTC.2 RedCap | SMTC.2 RedCap | Config 1,4 |
|  | SSB-MTC specified in 38.508-1 [14] Table 7.3.1-3 with condition SMTC.4 RedCap | SMTC.4 RedCap | Config 2,3 |
| } |  |  |  |

Table 16.6.1.1.4.3-2: ServingCellConfig (step 4, test procedure)

| Derivation Path: TS 38.508-1 [14] Table 4.6.3-167 with condition MEAS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| tdd-UL-DL-ConfigurationDedicated | Not present |  |  |
| downlinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF BWP-Downlink { | 1 entry |  |  |
| BWP-Downlink[1] | BWP-Downlink | entry 1 Table 16.6.1.1.4.3-3 |  |
| } |  |  |  |
| firstActiveDownlinkBWP-Id | 1 | Activate the RedCap-specific dedicated DL BWP |  |
| uplinkConfig SEQUENCE { |  |  |  |
| initialUplinkBWP | Not present |  |  |
| uplinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF BWP-Uplink { | 1 entry |  |  |
| BWP-Uplink[1] | BWP-Uplink | entry 1 Table 16.6.1.1.4.3-4 |  |
| } |  |  |  |
| firstActiveUplinkBWP-Id | 1 | Activate the RedCap-specific dedicated UL BWP |  |
| pusch-ServingCellConfig | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| servingCellMO | Not present |  |  |
| } |  |  |  |

Table 16.6.1.1.4.3-3: BWP-Downlink (Table 16.6.1.1.4.3-2)

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-Downlink ::= SEQUENCE { |  |  |  |
| bwp-Id | 1 |  |  |
| bwp-Common SEQUENCE { |  |  |  |
| genericParameters SEQUENCE { |  |  |  |
| locationAndBandwidth | The RIV coprresponds to DLBWP.1.3 RedCap |  |  |
| } |  |  |  |
| } |  |  |  |
| bwp-Dedicated SEQUENCE { |  |  |  |
| nonCellDefiningSSB-r17 SEQUENCE { |  |  |  |
| absoluteFrequencySSB-r17 | ARFCN-ValueNR of NCD-SSB |  |  |
| ssb-Periodicity-r17 | ms80 |  |  |
| ssb-TimeOffset-r17 | ms5 |  |  |
| } |  |  |  |
| servingCellMO-r17 | 1 | MeasObjectId associated to NCD-SSB |  |
| } |  |  |  |
| } |  |  |  |

Table 16.6.1.1.4.3-4: BWP-Uplink (Table 16.6.1.1.4.3-2)

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-Uplink ::= SEQUENCE { |  |  |  |
| bwp-Id | 1 |  |  |
| bwp-Common SEQUENCE { | BWP-UplinkCommon |  |  |
| genericParameters SEQUENCE { |  |  |  |
| locationAndBandwidth | The RIV coprresponds to ULBWP.1.3 RedCap |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

16.6.1.1.5	Test requirement

Table 16.6.1.1.4.1-3 and Table 16.6.1.1.5-1 define the primary level settings including test tolerances.

Table 16.6.1.1.5-1: NR Cell specific test parameters for SA intra-frequency event triggered reporting tests without gap under non-DRX

| Parameter |  | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | Config 1,4 |  |  | N/A |  | N/A |  |
|  |  | Config 2 |  |  | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | Config 3 |  |  | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration | Config 1,4 |  | SR.1.1 FDD |  | N/A |  |
|  | Config 2 |  | SR.1.1 TDD |  |  |  |
|  | Config 3 |  | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration | Config 1,4 |  | CR.1.1 FDD |  | N/A |  |
|  | Config 2 |  | CR.1.1 TDD |  |  |  |
|  | Config 3 |  | CR.2.1 TDD |  |  |  |
| Dedicated CORESET RMC configuration | Config 1,4 |  | CCR.1.1 FDD |  | N/A |  |
|  | Config 2 |  | CCR.1.1 TDD |  |  |  |
|  | Config 3 |  | CCR.2.1 TDD |  |  |  |
| OCNG Patterns |  |  | OP.1 |  | OP.1 |  |
| TRS Configuration | Config 1,4 |  | TRS.1.1 FDD |  | N/A |  |
|  | Config 2 |  | TRS.1.1 TDD |  |  |  |
|  | Config 3 |  | TRS.1.2 TDD |  |  |  |
| Initial DL BWP configuration |  |  | DLBWP.0.1 |  | DLBWP.0.1 |  |
| Initial UL BWP configuration |  |  | ULBWP.0.1 |  | ULBWP.0.1 |  |
| Active DL BWP configuration |  |  | DLBWP.1.3 RedCap |  | DLBWP.1.3 RedCap |  |
| Active UL BWP configuration |  |  | ULBWP.1.3 RedCap |  | ULBWP.1.3 RedCap |  |
| RLM-RS |  |  | SSB |  | SSB |  |
| Noc Note 2 | Config 1,2,4 | dBm/SCS | -98 |  |  |  |
|  | Config 3 |  | -95 |  |  |  |
| Noc Note 2 |  | dBm/15 KHz | -98 |  |  |  |
| Ȇs/Iot |  | dB | 4 | -1.46 | -Infinity | -1.46 |
| Ȇs/Noc |  | dB | 4 | 4 | -Infinity | 4 |
| SS-RSRP Note 3 | Config 1,2,4 | dBm/SCS | -94 | -94 | -Infinity | -94 |
|  | Config 3 |  | -91 | -91 | -Infinity | -91 |
| Io | Config 1,2,4 | dBm/9.36 MHz | -64.60 | -62.25 | -64.60 | -62.25 |
|  | Config 3 | dBm/18.36 MHz | -61.68 | -59.34 | -61.68 | -59.34 |
| Propagation Condition |  |  | AWGN |  |  |  |
| Antenna Configuration |  |  | 1x1 |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

The overall delay is measured from the beginning of time period T2 to the time that the SS receives the first MeasurementReport message triggered by event A3. It is allowed to be up to 2xTTIDCCH higher than the measurement reporting delay specified in 38.133[6] clause 9.2B.4 due to the TTI insertion uncertainty of the measurement report in DCCH.

Then the overall delay shall be no larger than: Tidentify_intra_without_index + 2xTTIDCCH, where

-	Tidentify_intra_without_index = 1000 ms, is the time for intra-frequency cell identification specified in 38.133 [6] clause 9.2B.5.1,

-	TTIDCCH = 1 ms.

Which gives a total of 1002 ms.

The UE is not required to read the neighbour cell SSB index in this test.

For the test to pass, the total number of successful tests shall be more than 90% of the cases with a confidence level of 95%.

<End of modified section 1>