3GPP TSG-RAN5 Meeting #99	R5-232055

# Incheon, Korea (Republic Of), 22nd May 2023 - 26th May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.508 | CR | 1417 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to generic procedure 4.5.2E |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_MUSIM_plus_CT1-UEConTest |  |  |  |  |  | Date: |  |  | 2023-04-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To align the message content in Table 4.5.2E.4-1 with Table 4.7.2.4. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Message content of ATTACH REQUEST was updated with editorial corrections. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Message content will not align with default message. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.5.2E |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

[Start of change]

4.5.2E	MUSIM UE Registration

The same assumptions and definitions apply as in clause 4.5.2.

4.5.2E.1	Initial conditions

System Simulator:

-	Cell A (home PLMN1) and Cell G (home PLMN2) are configured according to table 6.3.2.2-1 and table 6.3.2.2-3. Any change in the cells and their configurations shall be explicitly specified in the TC which calls the procedure in its entirety or refers to parts of it.

-	The procedure shall be performed under ideal radio conditions as defined in clause 5.

User Equipment:

-	The UE is a MUSIM UE.

-	The UE is equipped with two USIMs with configuration as defined in below Tables 4.5.2E.1-1 and 4.5.2E.1-2.

Table 4.5.2E.1-1: USIM A configuration

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EFIMSI |  | The HPLMN (MCC+MNC) of the IMSI is set to 001-01. |  |
| EFFPLMN |  | 001-02 |  |

Table 4.5.2E.1-2: USIM B configuration

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EFIMSI |  | The HPLMN (MCC+MNC) of the IMSI is set to 001-02. |  |
| EFFPLMN |  | 001-01 |  |

4.5.2E.2	Definition of system information messages

The default system information messages are used.

4.5.2E.3	Procedure

Table 4.5.2E.3-1: MUSIM UE registration procedure

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS configures: - Cell A as the "Serving cell". - Cell G as a "Non-Suitable Off cell". | - | - | - | - |
| 2 | The UE is switched on. | - | - | - | - |
| - | The following messages are to be observed on Cell A unless explicitly stated otherwise. | - | - | - | - |
| 3 | Check: Does UE with USIM A configuration transmit ATTACH REQUEST message including UE network capability with at least one MUSIM feature set to “supported”? | --> | ATTACH REQUEST | - | P |
| 4-16 | Steps 5 to 17 of the registration procedure described in TS 36.508 subclause 4.5.2.3 are performed. NOTE: The UE performs registration and the RRC connection is released. | - | - | - | - |
| 17 | The SS configures: - Cell G as the "Serving cell". | - | - | - | - |
| - | The following messages are to be observed on Cell G unless explicitly stated otherwise. | - | - | - | - |
| 18 | Check: Does UE with USIM B configuration transmit ATTACH REQUEST message including UE network capability with at least one MUSIM feature set to “supported”? | --> | ATTACH REQUEST | - | P |
| 19-31 | Steps 5 to 17 of the registration procedure described in TS 36.508 subclause 4.5.2.3 are performed. NOTE: The UE performs registration and the RRC connection is released. | - | - | - | - |

4.5.2E.4	Specific message contents

All specific message contents shall be referred to clause 4.6 and 4.7 with the exceptions below.

Table 4.5.2E.4-1: ATTACH REQUEST (Steps 3 and 18, Table 4.5.2E.3-1)

| Derivation path: Table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| UE network capability |  |  | pc_EPC_MUSIM |
| All octets with the exception of octet 10, bit 6 to bit 8 and octet 11, bit 1 to 2 | Any allowed value |  |  |
| NAS signalling connection release (NCR) (octet 10, bit 6) | ‘1’B | NAS signalling connection release supported | pc_EPC_MUSIM_NCR |
| Paging indication for voice services (PIV) (octet 10, bit 7) | ‘1’B | paging indication for voice services supported | pc_EPC_MUSIM_PIV |
| Reject paging request (RPR) (octet 10, bit 8) | ‘1’B | reject paging request supported | pc_EPC_MUSIM_RPR |
| Paging restriction (PR) (octet 11, bit 1) | ‘1’B | paging restriction supported | pc_EPC_MUSIM_PR |
| Paging timing collision control (PTCC) (octet 11, bit 2) | ‘1’B | paging timing collision control supported | pc_EPC_MUSIM_PTCC |

Table 4.5.2E.4-2: ATTACH ACCEPT (Steps 13 and 28, Table 4.5.2E.3-1)

| Derivation path: Table 4.7.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS network feature support | '0000 0011 0000 0000 0001 1111'B |  |  |

[End of change]