3GPP TSG-RAN5 Meeting #99	R5-232056

# Incheon, Korea (Republic Of), 22nd May 2023 - 26th May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2752 | rev | - | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to test procedure 4.9.9 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-04-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The IE length of 5GS registration type is 1/2 Octet in REGISTRATION REQUEST message as defined in TS 24.501. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 5GS registration type was updated in specific message content tables. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | IE value will mismatch with core specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.9.9.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

[Start of change]

4.9.9	Test procedure for UE for Tracking area updating / Inter-system change from S1 mode to N1 mode in 5GMM/EMM-IDLE mode

4.9.9.1	Scope

This procedure aims at verifying that the UE performs a Mobility and periodic registration update procedure when it performs inter-system change from S1 mode to N1 mode in 5GMM/EMM-IDLE.

4.9.9.2	Procedure description

4.9.9.2.1	Initial conditions

System Simulator:

-	1 NGC Cell connected to 5GC, default parameters, system information combination NR-6 as defined in subclause 4.4.3.1.2.

NOTE:	Details about the E-UTRA cell from which the UE will move to the NGC cell are to be specified in the test.

User Equipment:

-	The Test UICC shall be inserted. It shall provide relevant details on the EPC and 5GC.

All details required shall be explicitly specified in the TC which calls the procedure in its entirety or refers to parts of it.

4.9.9.2.2	Procedure sequence

Table 4.9.9.2.2-1: Test procedure sequence UE Tracking area updating / inter-system change from S1 mode to N1 mode in 5GMM/EMM-IDLE mode

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message/PDU/SDU |  |  |
| - | EXCEPTION: Unless otherwise stated all the messages below are transmitted on the cell specified in the test case. | - | - | - | - |
| 1-3 | Steps 1-3 from the mobility and periodic registration update procedure as described in Table 4.9.5.2.2-1 are performed. For some consequences related to the content of the REGISTRATION REQUEST message sent in step 1 see the Notes in Table 4.9.9.2.3-1. | - | - | - | - |
| 4 | The SS transmits a DLInformationTransfer message and an AUTHENTICATION REQUEST message. | <-- | NR RRC: DLInformationTransfer 5GMM: AUTHENTICATION REQUEST | - | - |
| 5 | The UE transmits an ULInformationTransfer message and an AUTHENTICATION RESPONSE message. | --> | NR RRC: ULInformationTransfer 5GMM: AUTHENTICATION RESPONSE | - | - |
| 6 | The SS transmits a DLInformationTransfer message and a SECURITY MODE COMMAND message. | <-- | NR RRC: DLInformationTransfer 5GMM: SECURITY MODE COMMAND | - | - |
| 7 | The UE transmits an ULInformationTransfer message and a SECURITY MODE COMPLETE message. | --> | NR RRC: ULInformationTransfer 5GMM: SECURITY MODE COMPLETE | - | - |
| 8 | The SS transmits a SecurityModeCommand message. | <-- | NR RRC: SecurityModeCommand | - | - |
| 9 | The UE transmits a SecurityModeComplete message. | --> | NR RRC: SecurityModeComplete | - | - |
| 10-11 | Steps 4-5 of Table 4.9.5.2.2-1 of the test procedure are performed. | - | - | - | - |
| 12a1-13a1 | Void | - | - | - | - |
| - | EXCEPTION: Steps 14a1 to 14b2b1 describe a step sequence depending on test case scenario; the "lower case letter" identifies a step sequence that take place if the test procedure is called in a particular scenario. (NOTE 1) | - | - | - | - |
| 14a1 | IF Interworking without N26 interface supported THEN The generic procedure for UE-requested PDU session establishment, specified in subclause 4.5A.2, takes place performing establishment of UE-requested PDU session(s) with ExpectedNumberOfNewPDUSessions = pc_noOf_PDUsSameConnection. The UE may attempt to transfer some of the existing in S1 PDN connection(s) in which case in the PDU SESSION ESTABLISHMENT REQUEST message the request type shall be set to "existing PDU session" (NOTE 3). | - | - | - | - |
| - | EXCEPTION: Steps 14a2a1 to 14a2b1 describe a step sequence depending on test case scenario; the "lower case letter" identifies a step sequence that take place if the UE performs a specific action. | - | - | - | - |
| 14a2a1 | IF pc_noOf_PDUsNewConnection > 0 THEN the SS transmits an RRCRelease message to release RRC connection and moves the UE to RRC_IDLE. | <-- | NR RRC: RRCRelease | - | - |
| 14a2a2 | The procedure NR RRC_IDLE Extension as specified in table 4.5.2.2-4 takes place. | - | - | - | - |
| 14a2b1 | ELSE IF connected without release is not present THEN, the SS transmits an RRCConnectionRelease message to release RRC connection and move the UE to RRC_IDLE. | <-- | NR RRC: RRCRelease | - | - |
| - | EXCEPTION: In parallel to the events described in step 14b1 below, the steps specified in Table 4.9.9.2.2-2 may take place. | - | - | - | - |
| 14b1 | ELSE (i.e. 'Interworking without N26 interface not supported') The generic procedure for UE-requested PDU session establishment, specified in subclause 4.5A.2, takes place performing establishment of UE-requested PDU session(s) with ExpectedNumberOfNewPDUSessions = pc_noOf_PDUsSameConnection with the exception that IF step 2b1, Table 4.5A.2.2.2-2 takes place THEN the SS shall not assign Fail but continue with the next step in the test sequence not expecting any additional session establishment to take place (NOTE 2). | - | - | - | - |
| - | EXCEPTION: Steps 14b2a1 to 14b2b1 describe a step sequence depending on test case scenario; the "lower case letter" identifies a step sequence that take place if the UE performs a specific action. | - | - | - | - |
| 14b2a1 | IF pc_noOf_PDUsNewConnection> 0 THEN the SS transmits an RRCRelease message to release RRC connection and move the UE to RRC_IDLE. | <-- | NR RRC: RRCRelease | - | - |
| 14b2a2 | The procedure NR RRC_IDLE Extension as specified in Table 4.5.2.2-4 takes place. For the referred in step 8, Table 4.5.2.2-4, generic procedure for UE-requested PDU session establishment, specified in subclause 4.5A.2, IF step 2b1, Table 4.5A.2.2.2-2 takes place THEN the SS shall not assign Fail but continue with the next step in the test sequence not expecting any additional session establishment to take place (NOTE 2). | - | - | - | - |
| 14b2b1 | ELSE IF connected without release is not present THEN, the SS transmits an RRCRelease message to release RRC connection and move the UE to RRC_IDLE. | <-- | NR RRC: RRCRelease | - | - |
| NOTE 1: The NWK will indicated whether Interworking without N26 interface is supported in the REGISTRATION ACCEPT message, IE '5GS network feature support', IWK N26 bit. Consequently which branch would the procedure sequence go through will depend on the content of the REGISTRATION ACCEPT message applicable to e.g. the test case which calls the present test procedure. NOTE 2: Depending on UE implementation and/or NWK behaviour, the UE may transfer some PDN connections into PDU sessions without re-establishing them with the relevant mapping provided in the ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message. This will result in the number of connection modifications, if any, being lower than the pc_noOf_PDUsSameConnection or the pc_noOf_PDUsNewConnection which the UE will establish upon initial attach to the 5GS. NOTE 3: Since the MME does not provide the UE with the mapped PDU session for a PDN connection, the UE does not know whether interworking to 5GS is supported for a PDN connection for which the UE assigned a PDU Session identity before attempting to transfer the PDN connection from S1 mode to N1 mode. It is up to UE implementation to decide which PDN connection(s) to be attempted to transfer from S1 mode to N1 mode, e.g. based on UE policy or UE local configuration. (see TS 24.501 [22], subclause 6.1.4.2) |  |  |  |  |  |

Table 4.9.9.2.2-2: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: Step 1 describe a step sequence depending on test case scenario; the "lower case letter" identifies a step sequence that take place if the UE performs a specific action. NOTE 2, NOTE 3 | - | - | - | - |
| 1 | IF this is the first time in a test case that the UE moves from S1 to N1 THEN the generic procedure for Procedure for UE-requested PDU session modification after the first S1 to N1 mode change / Single-registration mode with N26, specified in subclause 4.5A.2C, takes place with ExpectedNumberOfPDUSessionModifications=(pc_noOf_PDUsSameConnection+pc_noOf_PDUsNewConnection), with the exception that IF step 2b1, Table 4.5A.2C.2.2-2 takes place THEN the SS shall not assign Fail but continue with the next step in the test sequence not expecting any additional session modifications to take place (NOTE 1). |  |  |  |  |
| NOTE 1: Depending on UE implementation and/or NWK behaviour, the UE may transfer with modification some PDN connections into PDU sessions without re-establishing them with the relevant mapping provided in the ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message. This will result in the number of session modifications, if any, being lower than the pc_noOf_PDUsSameConnection+pc_noOf_PDUsNewConnection. NOTE 2: Whether this is the first time in a test case that the UE moves from S1 to N1 depends on the test scenario (including what happens in the preamble of the test). NOTE 3: It is assumed that the PDU session modification for all transferred PDUs will happen on the same connection with the mobility and periodic registration update procedure. NOTE 4: For PDN connections which will be transferred, tests calling the present procedure shall ensure that: - For each PDN connection established during the UE registration to the EPS, the ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message which corresponds to the default EPS bearer of the PDN connectivity being activated, contains the Protocol configuration options IE or the Extended protocol configuration options IE with mapped 5GS PDU a session-AMBR and QoS rule(s). |  |  |  |  |  |

4.9.9.2.3	Specific Message content

Table 4.9.9.2.3-1: REGISTRATION REQUEST (step 1, Table 4.9.9.2.2-1; step 3, Table 4.9.5.2.2-1)

| Derivation Path: Table 4.7.1-6. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| 5GS registration type | 'x010'B | mobility registration updating x - not checked |  |
| ngKSI |  |  |  |
| NAS key set identifier | KSIAMF that was created when the UE last registered to 5GCN |  | Registered_Previously_on_5GCN |
|  | ‘111’B | No key | Not_Registered_Previously_on_5GCN |
| TSC | '0'B | native security context (for KSIAMF) | Registered_Previously_on_5GCN |
|  | Not applicable | TSC does not apply for NAS key set identifier value "111" | Not_Registered_Previously_on_5GCN |
| 5GS mobile identity | 5G-GUTI mapped from the 4G-GUTI assigned when the UE last registered to EPC E-UTRA |  |  |
| Non-current native NAS key set identifier | Not present |  |  |
| 5GMM capability | '1' | S1 mode supported |  |
| Last visited registered TAI | The TAI the last visited NGC Cell belonged to, if any. Not included if the UE does not have last stored 5GC TAI. |  |  |
| S1 UE network capability |  |  |  |
| All octets with the exception of octet 8, bit 8 and octet 9, bit 6 | Not checked |  |  |
| Extended protocol configuration options (ePCO) (octet 8, bit 8) | '1' | Extended protocol configuration options IE supported |  |
| N1 mode supported (N1mode) (octet 9, bit 6) | '1' | N1 mode supported |  |
| PDU session status | Any allowed value | (NOTE 1) |  |
| UE status | "UE is in EMM-REGISTERED state" |  |  |
| Additional GUTI | 5G-GUTI assigned when the UE last registered to 5GC N |  | Registered_Previously_on_5GCN |
|  | Not present |  | Not_Registered_Previously_on_5GCN |
| EPS NAS message container | TRACKING AREA UPDATE REQUEST message | See Table 4.9.9.2.3-2 |  |
| EPS bearer context status | Not present | (NOTE 2) |  |
| NOTE 1: The UE includes the PDU session status IE indicating the status of the PDU session(s) mapped during the inter-system change from S1 mode to N1 mode from the PDN connection(s) for which the EPS indicated that interworking to 5GS is supported, if any. This means that the UE has created locally the default bearer context and the dedicated bearer context(s) based on the parameters of the mapped bearer contexts or the associations between QoS flow and mapped bearer in the PDN connection. Although the contents of the IE is not required to be verified for PASS/FAIL purposes, the provided information shall be taken into account for building any subsequent RRC Reconfiguration message, and can be used e.g. for SS configuration purposes as well. NOTE 2: The UE is assumed NOT to have locally deactivated EPS bearer context(s) for which interworking to 5GS is supported while the UE was in S1 mode without notifying the network. |  |  |  |

| Condition | Explanation |
| --- | --- |
| Not_Registered_Previously_on_5GCN | UE has not_registered_previously_on_5GCN. UE does not have valid 5G NAS security context and 5G-GUTI. |
| Registered_Previously_on_5GCN | UE has registered_previously_on_5GCN. UE have valid 5G NAS security context and 5G-GUTI |

Table 4.9.9.2.3-2: TRACKING AREA UPDATE REQUEST (Table 4.9.9.2.3-1)

| Derivation Path: TS 36.508 [2], Table 4.7.2-27. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EPS update type |  |  |  |
| EPS update type Value | '000'B | TA updating |  |
| "Active" flag | '0'B | No Bearer Establishment requested |  |
| NAS key set identifier | the eKSI for the current EPS security context |  |  |
| TSC | '0'B |  |  |
|  | '1'B |  | Mapped EPS security context |
| Old GUTI | 4G-GUTI assigned when the UE last registered to EPC E-UTRA |  |  |
| UE network capability | Not present |  |  |
| Last visited registered TAI | Not present |  |  |
| DRX parameter | Not present |  |  |
| UE radio capability information update needed | Not present |  |  |
| EPS bearer context status | Not present |  |  |
| MS network capability | Not present |  |  |
| Old location area identification | Not present |  |  |
| TMSI status | Not present |  |  |
| Mobile station classmark 2 | Not present |  |  |
| Mobile station classmark 3 | Not present |  |  |
| Supported Codecs | Not present |  |  |
| Additional update type | Not present |  |  |
| Voice domain preference and UE's usage setting | Not present |  |  |
| Old GUTI type | Not present |  |  |
| Device properties | Not present |  |  |
| MS network feature support | Not present |  |  |
| TMSI based NRI container | Not present |  |  |
| T3324 value | Not present |  |  |
| T3412 extended value | Not present |  |  |
| Extended DRX parameters | Not present |  |  |
| UE additional security capability | Not present |  |  |
| UE status | Not present |  |  |
| Additional information requested | Not present |  |  |
| NOTE: The message shall be integrity protected using the current EPS security context. |  |  |  |

| Condition | Explanation |
| --- | --- |
| Mapped EPS security context | When explicitly specified by the test case in which the message is used. |

Table 4.9.9.2.3-3: REGISTRATION ACCEPT (step 10, Table 4.9.9.2.2-1; step 4, Table 4.9.5.2.2-1)

| Derivation Path: Table 4.7.1-7. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| 5GS network feature support | The IWK N26 (octet 4, bit 7) set to '1' |  | Interworking without N26 interface supported |

[End of change]