3GPP TSG-RAN5 Meeting #99	R5-232068

# Incheon, Korea (Republic Of), 22nd May 2023 - 26th May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 3640 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to 5GC TC 9.1.1.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-04-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The IE length of 5GS registration type is 1/2 Octet in REGISTRATION REQUEST message, it shall align with TS 24.501. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 5GS registration type was updated in specific message content tables with editorial corrections. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | IE value will mismatch with core specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.1.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

[Start of change]

9.1.1.2	EAP based primary authentication and key agreement / Reject

9.1.1.2.1	Test Purpose (TP)

(1)

with {the UE in 5GMM-REGISTERED-INITIATED state }

ensure that {

when { the SS sends the EAP-request/AKA'-challenge message within AUTHENTICATION REQUEST with incorrect SNN }

then { the UE sends an EAP-response/AKA'-authentication-reject message within AUTHENTICATION RESPONSE}

}

(2)

with {the UE in 5GMM-REGISTERED-INITIATED state }

ensure that {

when { the SS sends an EAP-Request/AKA'-notification message within AUTHENTICATION REQUEST }

then { the UE sends an EAP-Response/AKA'-notification message within AUTHENTICATION RESPONSE }

}

(3)

with {the UE in 5GMM-REGISTERED-INITIATED state and SS initiates an EAP based primary authentication and key agreement procedure}

ensure that {

when { the SS sends an EAP-failure message within AUTHENTICATION REJECT }

then { the UE deletes the stored 5G-GUTI, TAI list, last visited registered TAI and ngKSI and enter state 5GMM-DEREGISTERED, the USIM is considered invalid until switching off the UE }

}

9.1.1.2.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.501, clauses 5.4.1.2.2.4, 5.4.1.2.2.6, and 5.4.1.2.2.11.

[TS 24.501, clause 5.4.1.2.2.4]

If a USIM is present, the SNN check fails or the UE does not accept AUTN during handling of the EAP-request/AKA'-challenge message as specified in IETF RFC 5448 [40], the UE shall send an EAP-response/AKA'-authentication-reject message as specified in IETF RFC 5448 [40].

If a USIM is present, the SNN check is successful but the UE detects that the sequence number in AUTN is not correct during handling of the EAP-request/AKA'-challenge message as specified in IETF RFC 5448 [40], the UE shall send an EAP-response/AKA'-synchronization-failure message as specified in IETF RFC 5448 [40].

If a USIM is present, the SNN check is successful, the sequence number in AUTN is correct and the UE detects another error during handling of the EAP-request/AKA'-challenge message as specified in IETF RFC 5448 [40], the UE shall send an EAP-response/AKA'-client-error message as specified in IETF RFC 5448 [40].

If a USIM is not present, the UE shall send an EAP-response/AKA'-client-error message as specified in IETF RFC 5448 [40].

For any of the above, the UE shall start timer T3520 when the AUTHENTICATION RESPONSE message containing the EAP-response message is sent. Furthermore, the UE shall stop any of the retransmission timers that are running (e.g. T3510, T3517 or T3521). Upon receiving an AUTHENTICATION REQUEST message with the EAP message IE containing an EAP-request/AKA'-challenge from the network, the UE shall stop timer T3520, if running, and then process the EAP-request/AKA'-challenge information as normal.

[TS 24.501, clause 5.4.1.2.2.6]

Upon receiving an EAP-request/AKA'-notification message, the UE shall send an EAP-response/AKA'-notification message as specified in IETF RFC 5448 [40].

[TS 24.501, clause 5.4.1.2.2.11]

Upon receiving an EAP-failure message, the UE shall delete the partial native 5G NAS security context if any was created as described in subclause 5.4.1.2.2.3.

The UE shall consider the procedure complete.

If the EAP-failure message is received in an AUTHENTICATION REJECT message:

-	the UE shall set the update status to 5U3 ROAMING NOT ALLOWED, delete the stored 5G-GUTI, TAI list, last visited registered TAI and ngKSI. The USIM shall be considered invalid until switching off the UE or the UICC containing the USIM is removed; and

-	if the UE is operating in single-registration mode, the UE shall handle 4G-GUTI, last visited registered TAI, TAI list and eKSI as specified in 3GPP TS 24.301 [15] for the case when the authentication procedure is not accepted by the network. The USIM shall be considered as invalid also for non-EPS services until switching off or the UICC containing the USIM is removed.

If the AUTHENTICATION REJECT message is received by the UE, the UE shall abort any 5GMM signalling procedure, stop any of the timers T3510, T3517 or T3521 (if they were running) and enter state 5GMM-DEREGISTERED.

9.1.1.2.3	Test description

9.1.1.2.3.1	Pre-test conditions

System Simulator:

-	NGC Cell A "Serving cell" TS 38.508-1 [4] Table 6.2.2.1-3

UE:

-	None

Preamble:

-	The UE is in state Switched OFF (state 0N-B) according to TS 38.508-1 [4].

9.1.1.2.3.2	Test procedure sequence

Table 9.1.1.2.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U – S | Message |  |  |
| 1 | The UE is switched on. | - | - | - | - |
| 2-4 | The UE executes steps 2-4 of Table 4.5.2.2-2 in TS38.508-1 [4]. | - | - | - | - |
| 5 | The SS transmits "EAP-request/AKA'-challenge" message in AUTHENTICATION REQUEST with incorrect SNN. | <-- | 5GMM: AUTHENTICATION REQUEST | - | - |
| 6 | Check: Does the UE transmit an “EAP-response/AKA'-authentication-reject” message in AUTHENTICATION RESPONSE? | --> | 5GMM: AUTHENTICATION RESPONSE | 1 | P |
| 6A | The SS transmits "EAP-failure" message within AUTHENTICATION RESULT message. | <-- | 5GMM: AUTHENTICATION RESULT | - | - |
| 7 | The SS transmits “EAP-request/AKA'-challenge” message in AUTHENTICATION REQUEST. | <-- | 5GMM: AUTHENTICATION REQUEST | - | - |
| 8 | The UE transmits an “EAP-response/AKA'- challenge” message in AUTHENTICATION RESPONSE. | --> | 5GMM: AUTHENTICATION RESPONSE | - | - |
| 9 | The SS transmits “EAP- request /AKA'-notification“message in AUTHENTICATION REQUEST. | <-- | 5GMM: AUTHENTICATION REQUEST | - | - |
| 10 | Check: Does the UE transmit an “EAP-response/AKA'-notification” message in AUTHENTICATION RESPONSE? | --> | 5GMM: AUTHENTICATION RESPONSE | 2 | P |
| 11 | The SS transmits an “EAP-failure” message within AUTHENTICATION REJECT | <-- | 5GMM: AUTHENTICATION REJECT | - | - |
| 12 | The SS releases the RRC connection. | - | - | - | - |
| 13 | Check: Does the UE transmit an RRCSetupRequest message for initial registration procedure within the next 30 seconds? | --> | NR RRC: RRCSetupRequest | 3 | F |
| 14 | The UE is switched off by executing generic procedure in Table 4.9.6.4-1 in TS 38.508-1 [4]. | - | - | - | - |
| 15 | The UE is switched on. | - | - | - | - |
| 16 | Check: Does the UE transmit a REGISTRATION REQUEST message? | --> | 5GMM: REGISTRATION REQUEST | 3 | P |
| 17 | The UE executes steps 5-20a1 of Table 4.5.2.2-2 in TS38.508-1 [4] complete registration procedure. | - | - | - | - |

9.1.1.2.3.3	Specific message contents

Table 9.1.1.2.3.3-1: AUTHENTICATION REQUEST (step 5, Table 9.1.1.2.3.2-1)

| Derivation path: TS 38.508-1 [4], table 4.7.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Extended protocol discriminator | 5GMM |  |  |
| Security header type | ’0000’B | Plain 5GS NAS message, not security protected |  |
| Spare half octet | '0000'B |  |  |
| EAP message | “EAP-request/AKA'-challenge” | SNN in EAP message is incorrect or does not match with the PLMN identity saved in the UE. |  |
| NOTE: This message is sent within SECURITY PROTECTED 5GS NAS MESSAGE message with Integrity protected and ciphered. |  |  |  |

Table 9.1.1.2.3.3-2: AUTHENTICATION RESPONSE (step 6, Table 9.1.1.2.3.2-1)

| Derivation path: TS 38.508-1 [4], table 4.7.1-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Extended protocol discriminator | 5GMM |  |  |
| Security header type | ’0000’B | Plain 5GS NAS message, not security protected |  |
| Spare half octet | '0000'B |  |  |
| EAP message | “EAP-response/AKA'-authentication-reject “ |  |  |
| NOTE: This message is sent within SECURITY PROTECTED 5GS NAS MESSAGE message with Integrity protected and ciphered. |  |  |  |

Table 9.1.1.2.3.3-2A: AUTHENTICATION RESULT (step 6A, Table 9.1.1.2.3.2-1)

| Derivation path: TS 38.508-1 [4], table 4.7.1-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Extended protocol discriminator | 5GMM |  |  |
| Security header type | ’0000’B | Plain 5GS NAS message, not security protected |  |
| Spare half octet | '0000'B |  |  |
| ngKSI | The same value as the last AUTHENTICATION REQUEST message |  |  |
| Spare half octet | '0000'B |  |  |
| EAP message | EAP-failure | EAP-failure |  |
| ABBA | ‘0000 0000 0000 0000’B |  |  |
| NOTE: This message is sent within SECURITY PROTECTED 5GS NAS MESSAGE message with Integrity protected and ciphered. |  |  |  |

Table 9.1.1.2.3.3-3: AUTHENTICATION RESPONSE (step 8, Table 9.1.1.2.3.2-1)

| Derivation path: TS 38.508-1 [4], table 4.7.1-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Extended protocol discriminator | 5GMM |  |  |
| Security header type | ’0000’B | Plain 5GS NAS message, not security protected |  |
| Spare half octet | '0000'B |  |  |
| EAP message | “EAP-request/AKA'-challenge” |  |  |
| NOTE: This message is sent within SECURITY PROTECTED 5GS NAS MESSAGE message with Integrity protected and ciphered. |  |  |  |

Table 9.1.1.2.3.3-4: AUTHENTICATION REQUEST (step 9, Table 9.1.1.2.3.2-1)

| Derivation path: TS 38.508-1 [4], table 4.7.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Extended protocol discriminator | 5GMM |  |  |
| Security header type | ’0000’B | Plain 5GS NAS message, not security protected |  |
| Spare half octet | '0000'B |  |  |
| EAP message | “EAP-request /AKA'-notification“ | See Table 9.1.1.2.3.3-8 |  |
| NOTE: This message is sent within SECURITY PROTECTED 5GS NAS MESSAGE message with Integrity protected and ciphered. |  |  |  |

Table 9.1.1.2.3.3-5: AUTHENTICATION RESPONSE (step 10, Table 9.1.1.2.3.2-1)

| Derivation path: TS 38.508-1 [4], table 4.7.1-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Extended protocol discriminator | 5GMM |  |  |
| Security header type | ’0000’B | Plain 5GS NAS message, not security protected |  |
| Spare half octet | '0000'B |  |  |
| EAP message | “EAP-response/AKA'-notification“ | See Table 9.1.1.2.3.3-9 |  |
| NOTE: This message is sent within SECURITY PROTECTED 5GS NAS MESSAGE message with Integrity protected and ciphered. |  |  |  |

Table 9.1.1.2.3.3-6: AUTHENTICATION REJECT (step 11, Table 9.1.1.2.3.2-1)

| Derivation path: TS 38.508-1 [4], table 4.7.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Extended protocol discriminator | 5GMM |  |  |
| Security header type | ’0000’B | Plain 5GS NAS message, not security protected |  |
| Spare half octet | '0000'B |  |  |
| EAP message | EAP-failure | EAP-failure |  |
| NOTE: This message is sent within SECURITY PROTECTED 5GS NAS MESSAGE message with Integrity protected and ciphered. |  |  |  |

Table 9.1.1.2.3.3-7: REGISTRATION REQUEST (step16, Table 9.1.1.2.3.2-1)

| Derivation path: TS 38.508-1 [4], table 4.7.1-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| 5GS registration type | '010'B | Initial registration |  |
| ngKSI |  |  |  |
| NAS key set identifier | '111'B | no key is available |  |
| TSC | Any allowed value | TSC does not apply for NAS key set identifier value "111" |  |
| Last visited registered TAI | Not present |  |  |
| 5GS mobile identity | SUCI of the UE |  |  |

Table 9.1.1.2.3.3-8: EAP-Request/AKA'-Notification (Table 9.1.1.2.3.3-4)

| Derivation Path: IETF RFC 4187 [30] clause 9.10, RFC 3748 [32] clause 4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Code | 1 | Request |  |
| Length | Set to length of EAP packet |  |  |
| Data |  |  |  |
| AT_NOTIFICATION | AT_NOTIFICATION_Def | See Table 9.1.1.2.3.3-10 |  |

Table 9.1.1.2.3.3-9: EAP-Response/AKA'-Notification (Table 9.1.1.2.3.2-5)

| Derivation Path: IETF RFC 4187 [30] clause 9.11, RFC 3748 [32] clause 4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Code | 2 | Response |  |
| Length | Set to length of EAP packet |  |  |
| Data | Not present |  |  |

Table 9.1.1.2.3.3-10: AT_NOTIFICATION_Def (Table 9.1.1.2.3.3-8)

| Derivation Path: IETF RFC 4187 [30] clause 10.19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |  |
| AT_NOTIFICATION | '0000 1100'B | 12 |  |  |
| Length | '0000 0001'B | 1 |  |  |
| Notification Code | '0100 0000 0000 0000'B | Set to “General failure” |  |  |

[End of change]