3GPP TSG-RAN5 Meeting #99	R5-232113

# Incheon, South Korea, 22 – 26 May, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 2194 | rev | - | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of Spurious emissions for UE co-existence test requirements for CA_n28A-n78A |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_NR_LTE_DC_R16-UEConTest |  |  |  |  |  | Date: |  |  | 2023-05-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Missing Spurious emissions for UE co-existence test requirements for CA_n28A-n78A |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added Spurious emissions for UE co-existence test requirements for CA_n28A-n78A into Table 6.5A.3.2.1.4.1-1 and Table 6.5A.3.2.1.5-1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Spurious emissions for UE co-existence test requirements for CA_n28A-n78A will not be specified |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5A.3.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TR 38.905... CR 0756 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-------- Start of changes --------

##### 6.5A.3.2.1	Spurious emissions for UE co-existence for CA (2UL CA)

6.5A.3.2.1.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to co-existing systems for the specified bands which has specific requirements in terms of transmitter spurious emissions for 2UL CA.

6.5A.3.2.1.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that support 2UL CA.

6.5A.3.2.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in subclause 6.5A.3.2.0.

6.5A.3.2.1.4	Test description

6.5A.3.2.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR CA configuration specified in clause 5.5A. All of these configurations shall be tested with applicable test parameters for each CA configuration, are shown in Table 6.5A.3.2.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5A.3.2.1.4.1-1: Inter band CA Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  |  |  | Normal |  |  |  |  |  |  |  |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1.1.3 for inter band CA in FR1 |  |  |  |  |  |  |  |  | For test frequencies refer to “Range” columns. |  |  |  |  |  |  |  |  |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  |  | Refer to “PCC NRB@SCS”and “SCC NRB@SCS ” columns |  |  |  |  |  |  |  |  |  |  |  |
| Test SCS as specified in Table 5.5A.3-1 |  |  |  |  |  |  |  |  | Lowest SCS for default test points. CA configuration specific test points: Refer to “PCC NRB@SCS”and “SCC NRB@SCS ” columns |  |  |  |  |  |  |  |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID | CA Configuration / NRB_agg |  |  |  |  |  |  |  | DL Allocation |  |  |  |  |  | UL Allocation |  |  |  |  |  |
|  | CA Configuration |  |  |  |  |  | PCC NRB @SCS | SCC NRB@SCS | CC MOD | PCC & SCC RB allocation |  |  |  |  | CC MOD |  | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |
|  | PCC |  | SCC |  |  |  |  |  |  | PCC |  |  | SCC |  |  |  |  |  |  |  |
|  | Band | Range | Band | Range |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Settings for CA_n1A-n8A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n1 | Low | n8 | Low |  | 25@15kHz |  | 25@15kHz | CP-OFDM QPSK | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  | 1@0 |
| Test Settings for CA_n1A-n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n1 | Low | n78 | Low | 106@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  |  | NA |  |  | CP-OFDM QPSK |  |  | 1@105 |  | 1@272 |
| 2 | n1 | Low | n78 | 3421,14 MHz (NOTE 2) | 106@15kHz |  |  | 52@15kHz | CP-OFDM QPSK |  |  | NA |  |  | CP-OFDM QPSK |  |  | 1@0 |  | 1@0 |
| 3 | n1 | Low | n78 | High | 106@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  |  | NA |  |  | CP-OFDM QPSK |  |  | 1@0 |  | 1@0 |
| Test Settings for CA_n1A-n79A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n1 | LOW | n79 | MID | 106@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  |  | NA |  |  | CP-OFDM QPSK |  |  | 1@0 |  | 1@181 |
| 2 | n1 | LOW | n79 | 4618,41 MHz (NOTE 3) | 79@15kHz |  |  | 216@15kHz | CP-OFDM QPSK |  |  | NA |  |  | CP-OFDM QPSK |  |  | 1@0 |  | 1@0 |
| 3 | n1 | LOW | n79 | MID | 106@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  |  | NA |  |  | CP-OFDM QPSK |  |  | 1@105 |  | 1@245 |
| 4 | n1 | LOW | n79 | 4785,54 MHz (NOTE 4) | 106@15kHz |  |  | 216@15kHz | CP-OFDM QPSK |  |  | NA |  |  | CP-OFDM QPSK |  |  | 1@0 |  | 1@0 |
| Test Settings for CA_n2A-n5A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n2 | Low | n5 | low | 20/106 |  |  | 20/106 | CP-OFDM QPSK |  |  | NA |  |  | CP-OFDM QPSK |  |  | 1@0 |  | 1@0 |
| 2 | n2 | High | n5 | High | 20/106 |  |  | 20/106 | CP-OFDM QPSK |  |  | NA |  |  | CP-OFDM QPSK |  |  | 1@105 |  | 1@105 |
| Test Settings for CA_n2A-n48A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| N/A (NOTE 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Settings for CA_n2A-n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n2 | Low | n77 | Mid | 106@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  |  | NA |  |  | CP-OFDM QPSK |  |  | 1@0 |  | 1@272 |
| 2 | n2 | High | n77 | 3850,23 MHz (NOTE 5) | 106@15kHz |  |  | 52@15kHz | CP-OFDM QPSK |  |  | NA |  |  | CP-OFDM QPSK |  |  | 1@0 |  | 1@0 |
| 3 | n2 | High | n77 | 3965,25 MHz (NOTE 6) | 106@15kHz |  |  | 52@15kHz | CP-OFDM QPSK |  |  | NA |  |  | - CP-OFDM QPSK |  |  | 1@0 |  | 1@0 |
| Test Settings for CA_n3A-n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n3 | HIGH | n41 | HIGH | 160@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  |  | NA |  |  | - CP-OFDM QPSK |  |  | 1@0 |  | 1@151 |
| 2 | n3 | LOW | n41 | HIGH | 160@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  |  | NA |  |  | - CP-OFDM QPSK |  |  | 1@149 |  | 1@18 |
| 3 | n3 | LOW | n41 | HIGH | 133@15kHz |  |  | 217@30kHz | CP-OFDM QPSK |  |  | NA |  |  | - CP-OFDM QPSK |  |  | 1@132 |  | 1@155 |
| 4 | n3 | HIGH | n41 | LOW | 160@15kHz |  |  | 217@30kHz | CP-OFDM QPSK |  |  | NA |  |  | - CP-OFDM QPSK |  |  | 1@0 |  | 1@151 |
| 5 (NOTE 7) | n3 | HIGH | n41 | HIGH | 160@15kHz |  |  | 217@30kHz | CP-OFDM QPSK |  |  | NA |  |  | - CP-OFDM QPSK |  |  | 1@159 |  | 1@105 |
| 6 (NOTE 7) | n3 | LOW | n41 | HIGH | 79@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  |  | NA |  |  | - CP-OFDM QPSK |  |  | 1@78 |  | 1@153 |
| Test Settings for CA_n3A-n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n3 | LOW | n78 | HIGH | 160@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@159 |  |  | 1@272 |  |
| 2 | n3 | HIGH | n78 | LOW | 160@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@159 |  |  | 1@0 |  |
| 3 | n3 | LOW | n78 | MID | 133@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@0 |  |  | 1@262 |  |
| 4 | n3 | LOW | n78 | MID | 160@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@0 |  |  | 1@41 |  |
| Test Settings for CA_n5A-n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n5 | HIGH | n77 | MID | 79@15kHz |  |  | 162@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@0 |  |  | 1@161 |  |
| 2 | n5 | HIGH | n77 | MID | 106@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@105 |  |  | 1@17 |  |
| 3 | n5 | LOW | n77 | 3458,79 MHz (NOTE 8) | 106@15kHz |  |  | 52@15kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@0 |  |  | 1@0 |  |
| 4 | n5 | HIGH | n77 | 3593,79 MHz (NOTE 9) | 106@15kHz |  |  | 52@15kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@0 |  |  | 1@0 |  |
| 5 | n5 | LOW | n77 | 4021,89 MHz (NOTE 10) | 52@15kHz |  |  | 52@15kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@51 |  |  | 1@0 |  |
| 6 | n5 | HIGH | n77 | HIGH | 106@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@105 |  |  | 1@272 |  |
| 7 | n5 | LOW | n77 | 3538,29 MHz (NOTE 11) | 106@15kHz |  |  | 52@15kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@0 |  |  | 1@0 |  |
| Test Settings for CA_n8A-n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n8 | HIGH | n78 | LOW | 52@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@0 |  |  | 1@0 |  |
| 2 | n8 | LOW | n78 | MID | 79@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@0 |  |  | 1@192 |  |
| 3 | n8 | HIGH | n78 | HIGH | 106@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@105 |  |  | 1@35 |  |
| 4 | n8 | LOW | n78 | LOW | 106@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@105 |  |  | 1@0 |  |
| 5 | n8 | LOW | n78 | HIGH | 106@15kHz |  |  | 217@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@0 |  |  | 1@139 |  |
| Test Settings for CA_n24A-n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n24 | High | n41 | Low | 52@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@0 |  |  | 1@136 |  |
| 2 | n24 | Low | n41 | Low | 52@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@52 |  |  | 1@108 |  |
| 3 | n24 | High | n41 | Low | 52@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@0 |  |  | 1@136 |  |
| 4 | n24 | Low | n41 | Low | 52@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@0 |  |  | 1@94 |  |
| 5 | n24 | High | n41 | High | 52@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@0 |  |  | 1@192 |  |
| Test Settings for CA_n24A-n48A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n24 | High | n48 | Mid | 52@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@0 |  |  | 1@136 |  |
| 2 | n24 | High | n48 | High | 52@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@0 |  |  | 1@0 |  |
| Test Settings for CA_n24A-n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n24 | Low | n77 | Mid | 52@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@0 |  |  | 1@136 |  |
| 2 | n24 | Low | n77 | 3930 MHz | 52@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@0 |  |  | 1@273 |  |
| 3 | n24 | Low | n77 | 3930 MHz | 52@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@0 |  |  | 1@262 |  |
| 4 | n24 | Low | n77 | 3930 MHz | 52@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@52 |  |  | 1@196 |  |
| Test Settings for CA_n26A-n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n26 | High | n66 | High | 106@15kHz |  |  | 216@15kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@105 |  |  | 1@0 |  |
| Test Settings for CA_n26A-n70A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n26 | Low | n70 | Low | 106@15kHz |  |  | 79@15kHz | QPSK/CP-OFDM QPSK |  | NA |  |  | QPSK / CP-OFDM QPSK |  | 1@0 |  |  | 1@0 |  |
| Test Settings for CA_n28A-n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n28 | Low | n78 | Low | 106@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@0 |  |  | 1@0 |  |
| 2 | n28 | High | n78 | High | 106@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@105 |  |  | 1@272 |  |
| 3 | n28 | High | n78 | Low | 106@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@105 |  |  | 1@0 |  |
| 4 | n28 | Low | n78 | Low | 106@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@0 |  |  | 1@100 |  |
| 5 | n28 | High | n78 | Mid | 106@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@105 |  |  | 1@0 |  |
| 6 | n28 | Mid | n78 | Mid | 106@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@0 |  |  | 1@272 |  |
| Test Settings for CA_n28A-n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n28 | LOW | n41 | HIGH | 79@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@78 |  |  | 1@157 |  |
| 2 | n28 | HIGH | n41 | HIGH | 79@15kHz |  |  | 217@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@78 |  |  | 1@115 |  |
| 3 | n28 | HIGH | n41 | LOW | 79@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@0 |  |  | 1@194 |  |
| Test Settings for CA_n39A-n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n39 | LOW | n41 | HIGH | 216@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@0 |  |  | 1@137 |  |
| Test Settings for CA_n41A-n79A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n41 | LOW | n79 | LOW | 273@30kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@272 |  |  | 1@0 |  |
| 2 | n41 | HIGH | n79 | LOW | 273@30kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@272 |  |  | 1@259 |  |
| 3 | n41 | LOW | n79 | MID | 273@30kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@272 |  |  | 1@150 |  |
| 4 | n41 | HIGH | n79 | LOW | 273@30kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@0 |  |  | 1@0 |  |
| 5 | n41 | HIGH | n79 | LOW | 273@30kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@129 |  |  | 1@0 |  |
| 6 | n41 | HIGH | n79 | LOW | 106@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@0 |  |  | 1@0 |  |
| 7 | n41 | HIGH | n79 | HIGH | 273@30kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@272 |  |  | 1@272 |  |
| 8 | n41 | HIGH | n79 | MID | 217@30kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@216 |  |  | 1@133 |  |
| Test Settings for CA_n48A-n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n48 | High | n66 | Low | 216@15kHz |  |  | 216@15kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@0 |  |  | 1@0 |  |
| Test Settings for CA_n48A-n70A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n48 | High | n70 | High | 216@15kHz |  |  | 79@15kHz | QPSK |  | NA |  |  | QPSK / CP-OFDM QPSK |  | 1@215 |  |  | 1@78 |  |
| 2 | n48 | High | n70 | Low | 216@15kHz |  |  | 79@15kHz | QPSK |  | NA |  |  | QPSK / CP-OFDM QPSK |  | 1@215 |  |  | 1@0 |  |
| Test Settings for CA_n48A-n71A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n48 | HIGH | n71 | LOW | 106@15kHz |  |  | 106@15kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@0 |  |  | 1@0 |  |
| 2 | n48 | LOW | n71 | HIGH | 216@15kHz |  |  | 106@15kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@0 |  |  | 1@105 |  |
| Test Settings for CA_n66A-n71A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n66 | Low | n71 | Low | 216@15kHz |  |  | 216@15kHz | QPSK/CP-OFDM QPSK |  | NA |  |  | QPSK / CP-OFDM QPSK |  | 1@0 |  |  | 1@0 |  |
| Test Settings for CA_n66A-n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n66 | LOW | n77 | 3825,24 MHz (NOTE 12) | 106@15kHz |  |  | 52@15kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@0 |  |  | 1@0 |  |
| 2 | n66 | LOW | n77 | HIGH | 216@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@0 |  |  | 1@151 |  |
| 3 | n66 | LOW | n77 | 4065,18 MHz (NOTE 13) | 79@15kHz |  |  | 52@15kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@0 |  |  | 1@0 |  |
| 4 | n66 | LOW | n77 | HIGH | 216@15kHz |  |  | 217@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@0 |  |  | 1@48 |  |
| 5 | n66 | LOW | n77 | HIGH | 106@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@0 |  |  | 1@217 |  |
| 6 (NOTE 14) | n66 | LOW | n77 | HIGH | 79@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@0 |  |  | 1@214 |  |
| 7 | n66 | LOW | n77 | HIGH | 216@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@0 |  |  | 1@211 |  |
| 8 | n66 | LOW | n77 | MID | 106@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@0 |  |  | 1@13 |  |
| 9 | n66 | HIGH | n77 | MID | 106@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@105 |  |  | 1@24 |  |
| Test Settings for CA_n70A-n71A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n70 | Low | n71 | Low | 79@15kHz |  |  | 106@15kHz | QPSK/CP-OFDM QPSK |  | NA |  |  | QPSK / CP-OFDM QPSK |  | 1@0 |  |  | 1@0 |  |
| NOTE 1: No impact on the protected bands by 2nd and 3rd order IM products caused by the NR CA band combination. NOTE 2: Test Point ID 2 for CA_n1A-n78A and Band n78 is defined as a carrier with CBW=10MHz and centre frequency at 3421.14MHz (NR ARFCN=628067). NOTE 3: Test Point ID 2 for CA_n1A-n79A and Band n79 is defined as a carrier with CBW=40MHz and centre frequency at 4618.41MHz (NR ARFCN=707867). NOTE 4: Test Point ID 4 for CA_n1A-n79A and Band n79 is defined as a carrier with CBW=40MHz and centre frequency at 4785.54MHz (NR ARFCN=719000). NOTE 5: Test Point ID 2 for CA_n2A-n77A and Band n77 is defined as a carrier with CBW=10MHz and centre frequency at 3850.23MHz (NR ARFCN=656667). NOTE 6: Test Point ID 3 for CA_n2A-n77A and Band n77 is defined as a carrier with CBW=10MHz and centre frequency at 3965.25MHz (NR ARFCN=664333). NOTE 7: Test point ID 5 and 6 for CA_n3A-n41A test protected NR band 100 introduced in Rel-17. NOTE 8: Test Point ID 3 for CA_n5A-n77A and Band n77 is defined as a carrier with CBW=10MHz and centre frequency at 3458.79MHz (NR ARFCN=630533). NOTE 9: Test Point ID 4 for CA_n5A-n77A and Band n77 is defined as a carrier with CBW=10MHz and centre frequency at 3593.79MHz (NR ARFCN=639533). NOTE 10: Test Point ID 5 for CA_n5A-n77A and Band n77 is defined as a carrier with CBW=10MHz and centre frequency at 4021.89MHz (NR ARFCN=668067). NOTE 11: Test Point ID 7 for CA_n5A-n77A and Band n77 is defined as a carrier with CBW=10MHz and centre frequency at 3538.29MHz (NR ARFCN=635867). NOTE 12: Test Point ID 1 for CA_n66A-n77A and Band n77 is defined as a carrier with CBW=10MHz and centre frequency at 3825.24MHz (NR ARFCN=655000). NOTE 13: Test Point ID 3 for CA_n66A-n77A and Band n77 is defined as a carrier with CBW=10MHz and centre frequency at 4065.18MHz (NR ARFCN=671000). NOTE 14: Test point ID 6 for CA_n66A-n77A tests protected E-UTRA band 103 introduced in Rel-17. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, in Figure A.3.1.1.3 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [6] subclause4.4.3.

3.	Downlink signals for PCC are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.5A.3.2.1.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.5A.3.2.1.4.3.

6.5A.3.2.1.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1, and Annex C.2 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 38.508-1 [5] clause 5.5.1. Message contents are defined in clause 6.5A.3.2.1.4.3.

3.	SS activates SCC by sending the activation MAC CE (Refer TS 38.321 [18], clauses 5.9, 6.1.3.10). Wait for at least 2 seconds (Refer TS 38.133[19], clause9.3).

4.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.5A.3.2.1.4.1-1 on both PCC and SCC. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

5.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE until the UE transmits at PUMAX level.

6.	Measure the power of the transmitted signal with a measurement filter of bandwidths according to Table 6.5A.3.2.1.5-1. The centre frequency of the filter shall be stepped in contiguous steps according to Table 6.5A.3.2.1.5-1. The measured power shall be verified for each step. The measurement period shall capture the active time slots.

6.5A.3.2.1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 with following exception.

Table 6.5A.3.2.1.4.3-1: FrequencyInfoUL-SIB

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 16 |  | Power class 3 and Inter-band CA |

6.5A.3.2.1.5	Test requirement

Test requirements for Spurious Emissions UE Co-existence are release specific and can be found in Table 6.5A.3.2.1.5-1. If the UE support a band combination from a later release, the requirements for this band combination are taken from the table of earliest release where requirements for this band combination are defined. The test requirement of configurations for CA operating band including Band n41 also apply for the corresponding CA operating bands with Band n90 replacing Band n41.

Table 6.5A.3.2.1.5-1: Requirements for uplink inter-band carrier aggregation according to UE NR release (two bands)

| NR CA | Release | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Configuration |  | Protected Band | Frequency range (Mhz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n1-n8 | Rel-16 Rel-17 | E-UTRA Band 43 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50+TT | 1 | 2 |
| CA_n1-n78 | Rel-16 Rel-17 | E-UTRA Band 3, 11, 21, 74 |  |  |  | -50+TT | 1 |  |
| CA_n1-n79 | Rel-16 Rel-17 | E-UTRA Band 5, 7, 8, 18, 19, 26, 28, 41 | FDL_low | - | FDL_high | -50+TT | 1 |  |
| CA_n2-n5 | Rel-16 | E-UTRA Band 42 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Rel-17 | E-UTRA Band 41, 43, 48 NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| CA_n2-n48 | Rel-16 Rel-17 | N/A (NOTE 16) |  |  |  |  |  |  |
| CA_n2-n77 | Rel-16 | E-UTRA Band 4, 65, 66, 70 | FDL_low | - | FDL_high | -50 +TT | 1 |  |
|  | Rel-17 | E-UTRA Band 2, 25 | FDL_low | - | FDL_high | -50 +TT | 1 | 2 |
| CA_n3-n41 | Rel-16 | E-UTRA Band 5, 8, 18, 19, 20, 26, 27, 28, 44 | FDL_low | - | FDL_high | -50+TT | 1 |  |
|  |  | E-UTRA Band 42 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50+TT | 1 | 2 |
|  | Rel-17 | E-UTRA Band 5, 8, 18, 19, 20, 26, 27, 28, 44 NR Band n100 | FDL_low | - | FDL_high | -50+TT | 1 |  |
|  |  | E-UTRA Band 42 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50+TT | 1 | 2 |
| CA_n3-n78 | Rel-15 | E-UTRA Band 3, 34, 39, 74 | FDL_low | - | FDL_high | -50+TT | 1 |  |
|  | Rel-16 Rel-17 | Frequency range | 1884.5 | - | 1915.7 | -41+TT | 0.3 | 3 |
| CA_n5-n77 | Rel-16 Rel-17 | E-UTRA Band 1, 2, 3, 4, 25, 30, 34, 40, 65, 66, 70 | FDL_low | - | FDL_high | -50+TT | 1 |  |
|  |  | E-UTRA Band 41 | FDL_low | - | FDL_high | -50+TT | 1 | 2 |
|  |  | Frequency range | 1884.5 | - | 1915.7 | -41+TT | 0.3 | 3 |
| CA_n8-n78 | Rel-15 | E-UTRA Band 8, 20, 28, 34, 39, 40, 74 | FDL_low | - | FDL_high | -50+TT | 1 |  |
|  | Rel-16 | E-UTRA Band 3, 7,41 | FDL_low | - | FDL_high | -50+TT | 1 | 2 |
|  | Rel-17 | Frequency range | 1884.5 | - | 1915.7 | -41+TT | 0.3 | 3 |
| CA_n24-n41 | Rel-17 | E-UTRA Band 5, 12, 13, 14, 17, 26, 29, 48, 71, 85 | FDL_low | - | FDL_high | -50+TT | 1 |  |
|  |  | NR Band n77 | FDL_low | - | FDL_high | -50+TT | 1 | 2 |
| CA_n24-n48 | Rel-17 | E-UTRA Band 2, 25, 70 | FDL_low | - | FDL_high | -50+TT | 1 |  |
| CA_n24-n77 | Rel-17 | E-UTRA Band 4, 10, 29, 30, 66, 71 | FDL_low | - | FDL_high | -50+TT | 1 |  |
| CA_n26-n66 | Rel-17 | E-UTRA Band 5, 26 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | E-UTRA Band 41, 42 NR band 77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| CA_n26-n70 | Rel-17 | E-UTRA Band 5 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| CA_n28-n41 | Rel-16 | E-UTRA Band 2, 3, 25 | FDL_low | - | FDL_high | -50+TT | 1 |  |
|  | Rel-17 | E-UTRA Band 42, 52 | FDL_low | - | FDL_high | -50+TT | 1 | 2 |
|  |  | NR Band n77, n78, n79 |  |  |  | +TT |  |  |
|  |  | Frequency range | 1884.5 | - | 1915.7 | -41+TT | 0.3 | 3, 11 |
| CA_n28-n78 | Rel-16 Rel-17 | E-UTRA Band 3, 7, 34, 39, 40, 41 | FDL_low | - | FDL_high | -50+TT | 1 |  |
|  |  | E-UTRA Band 65 | FDL_low | - | FDL_high | -50+TT | 1 | 2 |
|  |  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50+TT | 1 | 11,15 |
|  |  | Frequency range | 1884.5 | - | 1915.7 | -41+TT | 0.3 | 3,11 |
| CA_n39-n41 | Rel-16 | E-UTRA Band 28, 44 | FDL_low | - | FDL_high | -50+TT | 1 |  |
|  | Rel-17 | NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50+TT | 1 | 2 |
| CA_n41-n79 | Rel-16 Rel-17 | E-UTRA Band 1, 3, 5, 8, 18, 19, 21, 28, 34, 44, 65 | FDL_low | - | FDL_high | -50+TT | 1 |  |
|  |  | E-UTRA Band 40 | FDL_low | - | FDL_high | -40+TT | 1 |  |
|  |  | Frequency range | 1884.5 | - | 1915.7 | -41+TT | 0.3 | 3 |
| CA_n48-n66 | Rel-16 Rel-17 | E-UTRA Band 2, 25 | FDL_low | - | FDL_high | -50+TT | 1 |  |
| CA_n48-n70 | Rel-17 | E-UTRA Band 2, 25 | FDL_low | - | FDL_high | -50+TT | 1 | 16 |
| CA_n48-n71 | Rel-17 | E-UTRA Band30, 66 | FDL_low | - | FDL_high | -50 | 1 |  |
| CA_n66-n71 | Rel-16 Rel-17 | NR Band n77 | FDL_low | - | FDL_high | -50+TT | 1 | 2 |
| CA_n66-n77 | Rel-16 | E-UTRA Band 2, 4, 12, 13, 14, 17, 29, 30, 65, 66, 70, 71 | FDL_low | - | FDL_high | -50+TT | 1 |  |
|  | Rel-17 | E-UTRA Band 2, 4, 12, 13, 14, 17, 29, 30, 65, 66, 70, 71,103 | FDL_low | - | FDL_high | -50 +TT | 1 |  |
| CA_n70-n71 | Rel-16 | E-UTRA Band 30 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Rel-17 | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| CA_n78-n79 | Rel-15 Rel-16 | E-UTRA Band 1, 3, 5, 8, 11, 18, 19, 21, 28, 34, 40, 41, 65, 74 | FDL_low | - | FDL_high | -50+TT | 1 |  |
|  | Rel-17 | Frequency range | 1884.5 | - | 1915.7 | -41+TT | 0.3 | 3 |
| NOTE 1: FDL_low and FDL_high refer to each frequency band specified in Table 5.2-1 or Table 5.5-1 in TS 36.101 NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 and Table 6.5A.3.1-1 from the edge of the channel bandwidth. NOTE 5: This requirement is applicable only for the following cases: - for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 902.5 MHz ≤ Fc < 907.5 MHz with an uplink transmission bandwidth less than or equal to 20 RB - for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 907.5 MHz ≤ Fc ≤ 912.5 MHz without any restriction on uplink transmission bandwidth. - for carriers of 10 MHz channel bandwidth when carrier centre frequency (Fc) is Fc = 910 MHz with an uplink transmission bandwidth less than or equal to 32 RB with RBstart> 3. NOTE 6: This requirement is applicable for any channel bandwidths within the range 1920 – 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 – 1938 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 7: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 8: This requirement is only applicable for carriers with bandwidth confined within 1885-1920 MHz (requirement for carriers with at least 1RB confined within 1880 - 1885 MHz is not specified). This requirement applies for an uplink transmission bandwidth less than or equal to 54 RB for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1892.5 - 1894.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1895 - 1903 MHz. NOTE 9: Void. NOTE 10: Void. NOTE 11: Applicable when the assigned NR carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 12: As exceptions, measurements with a level up to the applicable requirement of -38 dBm/MHz is permitted for each assigned NR carrier used in the measurement due to 2nd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.3.1-1) for which the 2nd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 13: This requirement is applicable for 5 and 10 MHz NR channel bandwidth allocated within 718 - 728 MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and Rbstart < 48. NOTE 14: This requirement is applicable in the case of a 10 MHz NR carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. NOTE 15: As exceptions, measurements with a level up to the applicable requirement of -36 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 3rd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 3rd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 16: No impact on the protected bands by 2nd and 3rd order IM products caused by the NR CA band combination. NOTE 17: No impact on the protected bands by 2nd and 3rd order IM products caused by the NR CA band combination. |  |  |  |  |  |  |  |  |

Table 6.5A.3.2.1.5-1a: Void

Table 6.5A.3.2.1.5-1b: Void

Table 6.5A.3.2.1.5-2: Test Tolerance for uplink inter-band carrier aggregation (two bands)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 0 | 0 |
| 40MHz < BW ≤ 100MHz | 0 | 0 |

Unless otherwise stated, the spurious emission limits apply for the frequency ranges that are more than ΔfOOB (MHz) in Table 6.5.3.2.3-1 from the edge of the channel bandwidth. The spurious emission limits in Table 6.5.3.2.3-1 apply for all transmitter band configurations (NRB) and channel bandwidths for all CC combinations.

-------- End of changes --------