3GPP TSG-RAN5 Meeting #99	R5-232177

# Incheon, South Korea, 22nd – 26th May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 2356 | rev | - | Current version: | 17.6.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification to test procedure for SA active BWP switch TCs: 6.5.6.1.1, 6.5.6.1.2, 6.5.6.2.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Rohde & Schwarz |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Currently the test procedure is creating confusion by including “before the test starts” section, which contains same configuration as test step 3 of the test procedure itself |  |  |  |  |  |  |  |  |
| Summary of change: |  | “before the test starts” has been removed from test procedure and bwp-InactivityTimer timer configuration added to step 3 of test procedure |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test procedure will remain confusing whether BWP configuration is to be made before the procedure starts, or during step 3 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.6.1.1, 6.5.6.1.2, 6.5.6.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# < Unchanged sections omitted >

##### 6.5.6.1.1	NR SA FR1-FR1 DCI-based DL active BWP switch in non-DRX

6.5.6.1.1.1	Test purpose

The purpose of this test is to verify the DL BWP switch delay requirement defined in TS 38.133 [6] clause 8.6, and interruption requirement on other active serving cell defined in TS 38.133 [6] clause 8.2.2.2.5.

6.5.6.1.1.2	Test applicability

This test applies to all types of NR UE release 15 onwards supporting BWP adaptation of at least 2 BWPs, DCI and timer-based active BWP switching delay Type1 or Type2 and 2DL CA.

6.5.6.1.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.5.6.1.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.6.5.6.1.1.

6.5.6.1.1.4	Test description

6.5.6.1.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in this clause. The supported test configurations for PCell are shown in Table 6.5.6.1.1.4.1-1. Supported test configurations for NR SCell are shown in Table 6.5.6.1.1.4.1-1A below. Test configuration for NR PCell and test configuration for NR SCell are chosen independently.

Table 6.5.6.1.1.4.1-1: Supported test configurations for NR PCell for NR SA FR1-FR1 DCI-based DL active BWP switch with SCell in non-DRX

| Config | Description |
| --- | --- |
| 6.5.6.1.1-1 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, FDD duplex mode |
| 6.5.6.1.1-2 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, TDD duplex mode |
| 6.5.6.1.1-3 | NR 30 kHz SSB SCS, ≥40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: The UE is only required to be tested in one with smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration, |  |

Table 6.5.6.1.1.4.1-1A: Supported test configurations for NR SCell for NR SA FR1-FR1 DCI-based DL active BWP switch with SCell in non-DRX

| ConfigSCell | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, ≥40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: The UE is only required to be tested in one with smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration, |  |

Configure the test equipment and the DUT according to the parameters in Table 6.5.6.1.1.4.1-2.

Table 6.5.6.1.1.4.1-2: Initial conditions for NR SA FR1-FR1 DCI-based DL active BWP switch in non-DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.5.6.1.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE part. |  |  |

1. The general test parameter settings are set up according to Table 6.5.6.1.1.4.1-3.

2. Message contents are defined in clause 6.5.6.1.1.4.3.

3. The test scenario comprises of one NR PCell (Cell 1) and one NR SCell (Cell 2). Cell 1 and Cell 2 are configured according to Annex C.1.2 and C.1.3.

Table 6.5.6.1.1.4.1-3: General test parameters for DL BWP switch in SA

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1, 2 | Two NR radio channels are used for this test |
| Active PCell |  | Cell 1 | PCell on RF channel number 1. |
| Active SCell |  | Cell 2 | SCell on RF channel number 2. |
| CP length |  | Normal |  |
| DRX |  | OFF | For both PCell and SCell |
| bwp-InactivityTimer | ms | 200 |  |
| Cell-individual offset for cells on RF channel number 1 | dB | 0 | Individual offset for cells on PCC. |
| Cell-individual offset for cells on RF channel number 2 | dB | 0 | Individual offset for cells on SCC. |
| Cell2 timing offset to cell1 | s | 3 | Time alignment error as specified in TS 38.104 [28] clause 6.5.3.1. |
| T1 | s | 0.2 |  |
| T2 | s | 0.2 |  |
| T3 | s | 0.2 |  |

6.5.6.1.1.4.2	Test procedure

The test consists of 3 successive time periods, with durations of T1, T2, and T3, respectively.

PDCCHs indicating new transmissions shall be sent continuously on SCell (Cell 2) to ensure that the UE would have ACK/NACK sending except for the time duration when BWP is switching on Cell 2 and the time duration of T2.

PDCCHs indicating new transmissions shall be sent continuously on PCell (Cell 1) to ensure that the UE will have ACK/NACK sending.

All cells have constant signal levels throughout the test.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	The SS shall configure SCell (Cell 2) on the SCC as per TS 38.508-1 [14] clause 7.5.1. Set the parameters according to Tables 6.5.6.1.1.4.1-3 and 6.5.6.1.1.5-1. Propagation conditions are set according to Annex C clauses C.2.2.

3. 	The SS shall transmit an RRCReconfiguration message releasing the dedicated configuration of the initialDownlinkBWP and the initialUplinkBWP of Cell 2. This message also configures 2 different UE-specific bandwidth parts for Cell 2, BWP-1 and BWP-2, which always include the bandwidth of the initial DL BWP and SSB. The SS indicates BWP-1 as the active DL BWP using firstActiveDownlinkBWP-Id, according to Table 6.5.6.1.1.4.3-2. UE is configured with a bwp-InactivityTimer timer value for SCell.

4.	The SS shall send a DCI format 1_1 command for SCell DL BWP switch.

5.	The UE shall receive the DCI format 1_1 command in SCell’s slot # denoted i, then T1 starts and the UE switch its bandwidth part from BWP-1 to BWP-2:

a) If the UE starts to report valid ACK/NACK for SCell on PCell from the first UL slot that occurs after the beginning of DL slot (i+TBWPswitchDelay+k1)

and

b) If the UE starts to report valid ACK/NACK for PCell from the first UL slot that occurs after the beginning of DL slot (i+TBWPswitchDelay+ Interruption length + k1)

and

c) If the number of consecutive missing ACK/NACK for PCell is no more than 1

Then, the number of successful subtests is increased by one. Otherwise, count a fail for the test, switch off/on the UE and go to step 1.

6.	If the UE sends valid ACK/NACK for the SCell on BWP-2, T2 starts. During T2, the SS shall not transmit DCI format for PDSCH reception on SCell.

7.	T3 starts from the first slot #j of the DL subframe immediately after the slot wherein bwp-InactivityTimer timer expires and the SS restarts to send DCI format for PDSCH reception on SCell. Then, the UE shall switch its bandwidth part from BWP-2 back to the default bandwidth part – BWP-1:

a) If the UE starts to report valid ACK/NACK for SCell on PCell from the first UL slot that occurs after the beginning of DL slot (j+TBWPswitchDelay+k1)

and

b) If the UE starts to report valid ACK/NACK for PCell from the first UL slot that occurs after the beginning of DL slot (j+TBWPswitchDelay+ Interruption length + k1)

and

c) If the number of consecutive missing ACK/NACK for PCell is no more than 1.

Then, the number of successful subtests is increased by one. Otherwise, count a fail for the test, switch off/on the UE and go to step 1.

8.	Repeat steps 4-7 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

The SS verifies the DL BWP switch time in SCell by counting the slots from the time when the BWP switch command is received or bwp-InactivityTimer timer expires till an ACK/NACK is received.

The SS verifies that potential interruption to PCell is carried out in the correct time span by monitoring ACK/NACK sent in PCell during BWP switch of SCell, respectively.

Interruption length is defined in TS 38.133 [6] Table 8.2.2.2.5-1-1.

If all subtests pass, the test passes. If one subtest fails, the test fails.

6.5.6.1.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.5.6.1.1.4.3-1: Common Exception messages for NR SA FR1-FR1 DCI-based DL active BWP switch in non-DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |

Table 6.5.6.1.1.4.3-1A: RRCReconfiguration (Step 3)

| Derivation Path: TS 38.508-1 [14], Table 4.8.1-1B |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| masterCellGroup |  | CellGroupConfig | Table 6.5.6.1.1.4.3-1B |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 6.5.6.1.1.4.3-1B: CellGroupConfig (Table 6.5.6.1.1.4.3-1A)

| Derivation Path: TS 38.508-1 [14], Table 4.8.1-2 with condition SCell_add |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| sCellToAddModList SEQUENCE (SIZE (1..maxNrofSCells)) OF SCellConfig { | 1 entry |  |  |
| SCellConfig[1] SEQUENCE { |  | entry 1 |  |
| sCellConfigDedicated | ServingCellConfig | Table 6.5.6.1.1.4.3-2 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.5.6.1.1.4.3-2: ServingCellConfig (Table 6.5.6.1.1.4.3-1B)

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-167 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| initialDownlinkBWP SEQUENCE { |  |  |  |
| pdcch-Config CHOICE { |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| pdsch-Config CHOICE { |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| radioLinkMonitoringConfig CHOICE { |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| downlinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF SEQUENCE { | 2 entries |  |  |
| BWP-Downlink[1] | BWP-Downlink with condition BWP1 | entry 1 Table 6.5.6.1.1.4.3-3 |  |
| BWP-Downlink[2] | BWP-Downlink with condition BWP2 | entry 2 Table 6.5.6.1.1.4.3-3 |  |
| } |  |  |  |
| firstActiveDownlinkBWP-Id | 1 | According to BWP-1 |  |
| bwp-InactivityTimer | ms200 |  |  |
| defaultDownlinkBWP-Id | 1 | According to BWP-1 |  |
| } |  |  |  |

Table 6.5.6.1.1.4.3-3: BWP-Downlink (Table 6.5.6.1.1.4.3-2)

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-Downlink ::= SEQUENCE { |  |  |  |
| bwp-Id | 1 | BWP-1 | BWP1 |
|  | 2 | BWP-2 | BWP2 |
| bwp-Common SEQUENCE { |  |  |  |
| genericParameters | RIV defined in TS 38.214 [9] that corresponds to DLBWP.1.1 |  | BWP1 |
|  | RIV defined in TS 38.214 [9] that corresponds to DLBWP.1.3 |  | BWP2 |
| } |  |  |  |
| } |  |  |  |

Table 6.5.6.1.1.4.3-4: Void

Table 6.5.6.1.1.4.3-5: Void

Table 6.5.6.1.1.4.3-6: PDSCH-TimeDomainResourceAllocationList

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-103 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-TimeDomainResourceAllocationList ::= SEQUENCE(SIZE(1..maxNrofDL-Allocations)) OF PDSCH-TimeDomainResourceAllocation { | 4 entries |  |  |
| PDSCH-TimeDomainResourceAllocation[1] SEQUENCE { |  | entry 1 |  |
| k0 | Not present |  |  |
| mappingType | typeA |  |  |
| startSymbolAndLength | 53 | Start symbol(S)=2, Length(L)=12 |  |
| } |  |  |  |
| PDSCH-TimeDomainResourceAllocation[2] SEQUENCE { |  | entry 2 |  |
| k0 | Not present |  |  |
| mappingType | typeA |  |  |
| startSymbolAndLength | 72 | S=2, L=6 |  |
| } |  |  |  |
| PDSCH-TimeDomainResourceAllocation[3] SEQUENCE { |  | entry 3 |  |
| k0 | TBWPswitchDelay | Defined in Table 6.5.6.1.0.1-1 | The DCI indicating BWP switch |
| mappingType | typeA |  |  |
| startSymbolAndLength | 53 | Start symbol(S)=2, Length(L)=12 |  |
| } |  |  |  |
| PDSCH-TimeDomainResourceAllocation[4] SEQUENCE { |  | entry 4 |  |
| k0 | 1 |  | First DCI right after DCI-based BWP switch |
| mappingType | typeA |  |  |
| startSymbolAndLength | 53 | Start symbol(S)=2, Length(L)=12 |  |
| } |  |  |  |
| } |  |  |  |

Table 6.5.6.1.1.4.3-7: Void

Table 6.5.6.1.1.4.3-8: PDSCH-Config

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-100 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-Config ::= SEQUENCE { |  |  |  |
| pdsch-TimeDomainAllocationList | PDSCH-TimeDomainResourceAllocationList |  |  |
| } |  |  |  |

6.5.6.1.1.5	Test requirements

Tables 6.5.6.1.1.4.1-3 and 6.5.6.1.1.5-1 and 6.5.6.1.1.5-2 define the primary level settings including test tolerances.

Table 6.5.6.1.1.5-1: NR Cell specific test parameters for NR PCell for DL BWP switch in SA

| Parameter |  | Unit | Cell 1 |
| --- | --- | --- | --- |
| Frequency Range |  |  | FR1 |
| Duplex mode | Config 1 |  | FDD |
|  | Config 2,3 |  | TDD |
| TDD configuration | Config 1 |  | Not Applicable |
|  | Config 2 |  | TDDConf.1.1 |
|  | Config 3 |  | TDDConf.1.2 |
| BWchannel |  |  | Note 7 |
| BWoccupied | Config 1,2 | RB | 52 Note 5 |
|  | Config 3 |  | 106 Note 6 |
| Active BWP ID |  |  | 0 |
| Initial DL BWP Configuration |  |  | DLBWP.0.2Note4 |
| Initial UL BWP Configuration |  |  | ULBWP.0.2Note4 |
| Active DL BWP-0 Configuration |  |  | DLBWP.0.2Note4 |
| Active DL BWP-1 Configuration |  |  | N.A. |
| Active DL BWP-2 Configuration |  |  | N.A. |
| Active UL BWP-0 Configuration |  |  | ULBWP.0.2Note4 |
| Active UL BWP-1 Configuration |  |  | N.A. |
| Active UL BWP-2 Configuration |  |  | N.A. |
| PDSCH Reference | Config 1 |  | SR.1.1 FDD |
| measurement channel | Config 2 |  | SR.1.1 TDD |
|  | Config 3 |  | SR.2.1 TDD |
| RMSI CORESET | Config 1 |  | CR.1.1 FDD |
| parameters | Config 2 |  | CR.1.1 TDD |
|  | Config 3 |  | CR.2.1 TDD |
| Dedicated CORESET | Config 1 |  | CCR.1.2 FDD |
| parameters | Config 2 |  | CCR.1.2 TDD |
|  | Config 3 |  | CCR.2.4 TDD |
| TRS Configuration | Config 1 |  | TRS.1.1 FDD |
|  | Config 2 |  | TRS.1.1 TDD |
|  | Config 3 |  | TRS.1.2 TDD |
| OCNG Patterns | Config 1,2 |  | OP.1 Note 5 |
|  | Config 3 |  | OP.1 Note 6 |
| SSB Configuration | Config 1,2 |  | SSB.1 FR1 |
|  | Config 3 |  | SSB.2 FR1 |
| SMTC Configuration |  |  | SMTC.1 |
| Correlation Matrix and Antenna Configuration |  |  | 1x2 Low |
| EPRE ratio of PSS to SSS |  | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |
| NocNote 2 | Config 1,2 | dBm/SCS | -104 |
|  | Config 3 |  | -101 |
| NocNote 2 |  | dBm/15KHz | -104 |
| SS-RSRP Note 3 | Config 1,2 | dBm/SCS | -87 |
|  | Config 3 |  | -84 |
| Ês/Iot |  | dB | 17 |
| Ês/Noc |  | dB | 17 |
| IoNote3 | Config 1,2 | dBm/ 9.36MHz | -58.96 |
|  | Config 3 | dBm/ 38.16MHz | -52.86 |
| Propagation Condition |  |  | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled within BWoccupied. Note 3 SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For unpaired spectrum, a DL BWP is linked with an UL BWP. DLBWP.0.2 is linked with ULBWP.0.2; DLBWP.1.1 is linked with ULBWP.1.1; DLBWP.1.3 is linked with ULBWP.1.3 defined in clause 12 of TS 38.213 [3]. Note 5: All UL/DL transmission shall be confined within BWoccupied (i.e. 10 MHz, 52 RBs) from FC,low, and Io is independent of the BWchannel configured. Note 6: All UL/DL transmission shall be confined within BWoccupied (i.e. 40 MHz, 106 RBs) from FC,low, and Io is independent of the BWchannel configured. Note 7: NRB,c. is derived from Table 5.3.2-1 in TS38.101-1[2] with configured BWchannel. |  |  |  |

Table 6.5.6.1.1.5-2: NR Cell specific test parameters for NR SCell for DL BWP switch in SA

| Parameter |  | Unit | Cell2 |
| --- | --- | --- | --- |
| Frequency Range |  |  | FR1 |
| Duplex mode | ConfigSCell 1 |  | FDD |
|  | ConfigSCell 2,3 |  | TDD |
| TDD configuration | ConfigSCell 1 |  | Not Applicable |
|  | ConfigSCell 2 |  | TDDConf.1.1 |
|  | ConfigSCell 3 |  | TDDConf.1.2 |
| BWchannel |  |  | Note 7 |
| BWoccupied | ConfigSCell 1,2 | RB | 52 Note 5 |
|  | ConfigSCell 3 |  | 106 Note 6 |
| Active BWP ID |  |  | 1, 2 |
| Initial DL BWP Configuration |  |  | DLBWP.0.2Note4 |
| Initial UL BWP Configuration |  |  | N.A. |
| Active DL BWP-0 Configuration |  |  | N.A. |
| Active DL BWP-1 Configuration |  |  | DLBWP.1.1Note4 |
| Active DL BWP-2 Configuration |  |  | DLBWP.1.3Note4 |
| Active UL BWP-0 Configuration |  |  | N.A. |
| Active UL BWP-1 Configuration |  |  | N.A. |
| Active UL BWP-2 Configuration |  |  | N.A. |
| PDSCH Reference | ConfigSCell 1 |  | SR.1.1 FDD |
| measurement channel | ConfigSCell 2 |  | SR.1.1 TDD |
|  | ConfigSCell 3 |  | SR.2.1 TDD |
| RMSI CORESET | ConfigSCell 1 |  | CR.1.1 FDD |
| parameters | ConfigSCell 2 |  | CR.1.1 TDD |
|  | ConfigSCell 3 |  | CR.2.1 TDD |
| Dedicated CORESET | ConfigSCell 1 |  | CCR.1.2 FDD |
| parameters | ConfigSCell 2 |  | CCR.1.2 TDD |
|  | ConfigSCell 3 |  | CCR.2.4 TDD |
| TRS Configuration | ConfigSCell 1 |  | TRS.1.1 FDD |
|  | ConfigSCell 2 |  | TRS.1.1 TDD |
|  | ConfigSCell 3 |  | TRS.1.2 TDD |
| OCNG Patterns | ConfigSCell 1,2 |  | OP.1 Note 5 |
|  | ConfigSCell 3 |  | OP.1 Note 6 |
| SSB Configuration | ConfigSCell 1,2 |  | SSB.1 FR1 |
|  | ConfigSCell 3 |  | SSB.2 FR1 |
| SMTC Configuration |  |  | SMTC.1 |
| Correlation Matrix and Antenna Configuration |  |  | 1x2 Low |
| EPRE ratio of PSS to SSS |  | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |
| EPRE ratio of OCNG DMRS to SSS Note 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| NocNote 2 | ConfigSCell 1,2 | dBm/SCS | -104 |
|  | ConfigSCell 3 |  | -101 |
| NocNote 2 |  | dBm/15KHz | -104 |
| SS-RSRP Note 3 | ConfigSCell 1,2 | dBm/SCS | -87 |
|  | ConfigSCell 3 |  | -84 |
| Ês/Iot |  | dB | 17 |
| Ês/Noc |  | dB | 17 |
| IoNote3 | ConfigSCell 1,2 | dBm/ 9.36MHz | -58.96 |
|  | ConfigSCell 3 | dBm/ 38.16MHz | -52.86 |
| Propagation Condition |  |  | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled within BWoccupied. Note 3 SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For unpaired spectrum, a DL BWP is linked with an UL BWP. DLBWP.0.2 is linked with ULBWP.0.2; DLBWP.1.1 is linked with ULBWP.1.1; DLBWP.1.3 is linked with ULBWP.1.3 defined in clause 12 of TS 38.213 [3]. Note 5: All UL/DL transmission shall be confined within BWoccupied (i.e. 10 MHz, 52 RBs) from FC,low, and Io is independent of the BWchannel configured. Note 6: All UL/DL transmission shall be confined within BWoccupied (i.e. 40 MHz, 106 RBs) from FC,low, and Io is independent of the BWchannel configured. Note 7: NRB,c. is derived from Table 5.3.2-1 in TS38.101-1[2] with configured BWchannel. |  |  |  |

During T1, the UE shall start to send the ACK/NACK for SCell on PCell from the first UL slot that occurs after the beginning of DL slot (i+TBWPswitchDelay+k1).

During T3, the UE shall start to send the ACK/NACK for SCell on PCell from the first UL slot that occurs after the beginning of DL slot (j+TBWPswitchDelay+k1).

Where, k1 is the timing between DL data receiving and acknowledgement as specified in [9].

Depending on UE capability bwp-SwitchingDelay [13], UE shall finish BWP switch within the time duration TBWPswitchDelay defined in TS 38.133 [6] Table 8.6.2-1.

All of the above test requirements shall be fulfilled in order for the observed SCell active BWP switch delay to be counted as correct.

The rate of correct events observed during repeated tests shall be at least 90%.

During T1 and T3, the start time of PCell interruption during SCell active BWP switch shall not happen outside the BWP switch delay.

The interruption of PCell shall not be longer than the interruption duration specified for active BWP switch in TS 38.133 [6] clause 8.2.2.2.5.

All of the above test requirements shall be fulfilled in order for the observed SCell active BWP switch interruption to be counted as correct.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	During T1, T3 if there are no uplink resources for reporting the ACK/NACK in the first UL slot that occurs after the beginning of DL slot (i+ TBWPswitchDelay+k1), (j+ TBWPswitchDelay+k1), then the UE shall use the next available uplink resource for reporting the corresponding ACK/NACK.

##### 6.5.6.1.2	NR SA FR1 DCI-based DL active BWP switch in non-DRX

6.5.6.1.2.1	Test purpose

The purpose of this test is to verify the DL BWP switch delay requirement defined in TS 38.133 [6] clause 8.6.

6.5.6.1.2.2	Test applicability

This test applies to all types of NR UE release 15 onwards supporting BWP adaptation of at least 2 BWPs, DCI and timer-based active BWP switching delay Type1 or Type2.

6.5.6.1.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.5.6.1.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.6.5.6.1.2.

6.5.6.1.2.4	Test description

6.5.6.1.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.5.6.1.2.4.1-1.

Table 6.5.6.1.2.4.1-1: Supported test configurations for NR SA FR1 DCI-based DL active BWP switch in non-DRX

| Config | Description |
| --- | --- |
| 6.5.6.1.2-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 6.5.6.1.2-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6.5.6.1.2-3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: A UE which fulfils the requirements in test case A.6.5.6.1.1 can skip the test cases in A.6.5.6.1.2 |  |

Configure the test equipment and the DUT according to the parameters in Table 6.5.6.1.2.4.1-2.

Table 6.5.6.1.2.4.1-2: Initial conditions for NR SA FR1 DCI-based DL active BWP switch in non-DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.5.6.1.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE part. |  |  |

1. The general test parameter settings are set up according to Table 6.5.6.1.2.4.1-3.

2. Message contents are defined in clause 6.5.6.1.2.4.3.

3. The test scenario comprises of one NR PCell (Cell 1). Cell 1 is configured according to Annex C.1.2 and C.1.3.

Table 6.5.6.1.2.4.1-3: General test parameters for DL BWP switch in SA

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1 | One NR radio channel is used for this test |
| Active Cell |  | Cell 1 | Cell1 on RF channel number 1. |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| bwp-InactivityTimer | ms | 200 |  |
| T1 | s | 0.2 |  |
| T2 | s | 0.2 |  |
| T3 | s | 0.2 |  |

6.5.6.1.2.4.2	Test procedure

The test consists of 3 successive time periods, with durations of T1, T2, and T3, respectively.

PDCCHs indicating new transmissions shall be sent continuously on Cell 1 to ensure that the UE would have ACK/NACK sending except for the time duration when BWP is switching on Cell 1 and the time duration of T2.

The Cell 1 has constant signal levels throughout the test.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to Tables 6.5.6.1.2.4.1-3 and 6.5.6.1.2.5-1. Propagation conditions are set according to Annex C clauses C.2.2.

3. 	The SS shall transmit an RRCReconfiguration message releasing the dedicated configuration of the initialDownlinkBWP and the initialUplinkBWP. This message also configures 2 different UE-specific bandwidth parts, BWP-1 and BWP-2, which always include the bandwidth of the initial DL BWP and SSB. The SS indicates BWP-1 as the active DL BWP using firstActiveDownlinkBWP-Id, according to Table 6.5.6.1.2.4.3-2. UE is configured with a bwp-InactivityTimer timer value for PCell.

4.	The SS shall send a DCI format 1_1 command for Cell 1 DL BWP switch.

5.	The UE shall receive the DCI format 1_1 command in slot # denoted i, then T1 starts and the UE switch its bandwidth part from BWP-1 to BWP-2:

If the UE starts to report valid ACK/NACK for Cell 1 from the first UL slot that occurs after the beginning of DL slot (i+TBWPswitchDelay+k1). The number of successful subtests is increased by one. Otherwise, count a fail for the test, switch off/on the UE and go to step 1.

6.	If the UE sends valid ACK/NACK for the Cell 1 on BWP-2, T2 starts. During T2, the SS shall not transmit DCI format for PDSCH reception on Cell 1.

7.	T3 starts from the first slot #j of the DL subframe immediately after the slot wherein bwp-InactivityTimer timer expires and the SS restarts to send DCI format for PDSCH reception on PCell. Then, the UE shall switch its bandwidth part from BWP-2 back to the default bandwidth part, BWP-1:

If the UE starts to report valid ACK/NACK for PCell from the first UL slot that occurs after the beginning of DL slot (j+TBWPswitchDelay+k1). The number of successful subtests is increased by one and go to step 8 after T3 expires. Otherwise, count a fail for the test, switch off/on the UE and go to step 1.

8.	Repeat steps 4-7 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

The SS verifies the DL BWP switch time by counting the slots from the time when the BWP switch command is received or bwp-InactivityTimer timer expires till an ACK/NACK is received.

If all subtests pass, the test passes. If one subtest fails, the test fails.

6.5.6.1.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.5.6.1.2.4.3-1: Common Exception messages for NR SA FR1 DCI-based DL active BWP switch in non-DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |

Table 6.5.6.1.2.4.3-1A: Void

Table 6.5.6.1.2.4.3-1B: Void

Table 6.5.6.1.2.4.3-1C: RRCReconfiguration (Step 3)

| Derivation Path: TS 38.508-1 [14], Table 4.8.1-1B |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| masterCellGroup |  | CellGroupConfig | Table 6.5.6.1.2.4.3-1D |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 6.5.6.1.2.4.3-1D: CellGroupConfig (Table 6.5.6.1.2.4.3-1C)

| Derivation Path: TS 38.508-1 [14], Table 4.8.1-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| spCellConfigDedicated | ServingCellConfig | Table 6.5.6.1.2.4.3-2 |  |
| } |  |  |  |
| } |  |  |  |

Table 6.5.6.1.2.4.3-2: ServingCellConfig (Table 6.5.6.1.2.4.3-1D)

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-167 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| initialDownlinkBWP SEQUENCE { |  |  |  |
| pdcch-Config CHOICE { |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| pdsch-Config CHOICE { |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| radioLinkMonitoringConfig CHOICE { |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| downlinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF SEQUENCE { | 2 entries |  |  |
| BWP-Downlink[1] | BWP-Downlink with condition BWP1 | entry 1 Table 6.5.6.1.2.4.3-3 |  |
| BWP-Downlink[2] | BWP-Downlink with condition BWP2 | entry 2 Table 6.5.6.1.2.4.3-3 |  |
| } |  |  |  |
| firstActiveDownlinkBWP-Id | 1 | According to BWP-1 |  |
| bwp-InactivityTimer | ms200 |  |  |
| defaultDownlinkBWP-Id | 1 | According to BWP-1 |  |
| uplinkConfig SEQUENCE { |  |  |  |
| initialUplinkBWP SEQUENCE { |  |  |  |
| pucch-Config CHOICE { |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| pusch-Config CHOICE { |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| srs-Config CHOICE { |  |  |  |
| release | NULL |  |  |
| uplinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF SEQUENCE { |  |  |  |
| BWP-Uplink[1] | BWP-Uplink with condition BWP1 | entry 1 Table 6.5.6.1.2.4.3-4 |  |
| BWP-Uplink[2] | BWP-Uplink with condition BWP2 | entry 2 Table 6.5.6.1.2.4.3-4 |  |
| firstActiveUplinkBWP-Id | 1 | According to BWP-1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.5.6.1.2.4.3-3: BWP-Downlink (Table 6.5.6.1.2.4.3-2)

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-Downlink ::= SEQUENCE { |  |  |  |
| bwp-Id | 1 | BWP-1 | BWP1 |
|  | 2 | BWP-2 | BWP2 |
| bwp-Common SEQUENCE { |  |  |  |
| genericParameters | RIV defined in TS 38.214 [9] that corresponds to DLBWP.1.1 |  | BWP1 |
|  | RIV defined in TS 38.214 [9] that corresponds to DLBWP.1.3 |  | BWP2 |
| } |  |  |  |
| } |  |  |  |

Table 6.5.6.1.2.4.3-4: BWP-Uplink (Table 6.5.6.1.2.4.3-2)

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-Uplink ::= SEQUENCE { |  |  |  |
| bwp-Id | 1 | BWP-1 | BWP1 |
|  | 2 | BWP-2 | BWP2 |
| bwp-Common SEQUENCE { |  |  |  |
| genericParameters | RIV defined in TS 38.214 [9] that corresponds to ULBWP.1.1 | BWP-1 | BWP1 |
|  | RIV defined in TS 38.214 [9] that corresponds to ULBWP.1.3 | BWP-2 | BWP2 |
| } |  |  |  |
| } |  |  |  |

Table 6.5.6.1.2.4.3-5: Void

Table 6.5.6.1.2.4.3-6: PDSCH-TimeDomainResourceAllocationList

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-103 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-TimeDomainResourceAllocationList ::= SEQUENCE(SIZE(1..maxNrofDL-Allocations)) OF PDSCH-TimeDomainResourceAllocation { | 4 entries |  |  |
| PDSCH-TimeDomainResourceAllocation[1] SEQUENCE { |  | entry 1 |  |
| k0 | Not present |  |  |
| mappingType | typeA |  |  |
| startSymbolAndLength | 53 | Start symbol(S)=2, Length(L)=12 |  |
| } |  |  |  |
| PDSCH-TimeDomainResourceAllocation[2] SEQUENCE { |  | entry 2 |  |
| k0 | Not present |  |  |
| mappingType | typeA |  |  |
| startSymbolAndLength | 72 | S=2, L=6 |  |
| } |  |  |  |
| PDSCH-TimeDomainResourceAllocation[3] SEQUENCE { |  | entry 3 |  |
| k0 | TBWPswitchDelay | Defined in Table 6.5.6.1.0.1-1 | The DCI indicating BWP switch |
| mappingType | typeA |  |  |
| startSymbolAndLength | 53 | Start symbol(S)=2, Length(L)=12 |  |
| } |  |  |  |
| PDSCH-TimeDomainResourceAllocation[4] SEQUENCE { |  | entry 4 |  |
| k0 | 1 |  | First DCI right after DCI-based BWP switch |
| mappingType | typeA |  |  |
| startSymbolAndLength | 53 | Start symbol(S)=2, Length(L)=12 |  |
| } |  |  |  |
| } |  |  |  |

Table 6.5.6.1.2.4.3-7: PDSCH-Config

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-100 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-Config ::= SEQUENCE { |  |  |  |
| pdsch-TimeDomainAllocationList | PDSCH-TimeDomainResourceAllocationList |  |  |
| } |  |  |  |

6.5.6.1.2.5	Test requirements

Tables 6.5.6.1.2.4.1-3 and 6.5.6.1.2.5-1 define the primary level settings including test tolerances.

Table 6.5.6.1.2.5-1: NR Cell specific test parameters for DL BWP switch in SA

| Parameter |  |  |  | Unit | Cell 1 |
| --- | --- | --- | --- | --- | --- |
| Frequency Range |  |  |  |  | FR1 |
| Duplex mode |  |  | Config 1 |  | FDD |
|  |  |  | Config 2,3 |  | TDD |
| TDD configuration |  |  | Config 1 |  | Not Applicable |
|  |  |  | Config 2 |  | TDDConf.1.1 |
|  |  |  | Config 3 |  | TDDConf.2.1 |
| BWchannel |  |  | Config 1 |  | 10 MHz: NRB,c = 52 |
|  |  |  | Config 2 |  | 10 MHz: NRB,c = 52 |
|  |  |  | Config 3 |  | 40 MHz: NRB,c = 106 |
| Active BWP ID |  |  |  |  | 1, 2 |
| Initial DL BWP Configuration |  |  | Config 1,2,3 |  | DLBWP.0.2 Note 4 |
| Active DL BWP-1 Configuration |  |  | Config 1,2,3 |  | DLBWP.1.1 Note 4 |
| Active DL BWP-2 Configuration |  |  | Config 1,2,3 |  | DLBWP.1.3 Note 4 |
| Initial UL BWP Configuration |  |  | Config 1,2,3 |  | ULBWP.0.2 Note 4 |
| Active UL BWP-1 Configuration |  |  | Config 1,2,3 |  | ULBWP.1.1 Note 4 |
| Active UL BWP-2 Configuration |  |  | Config 1 |  | N/A |
|  |  |  | Config 2,3 |  | ULBWP.1.3 Note 4 |
| PDSCH Reference measurement channel |  |  | Config 1 |  | SR.1.1 FDD |
|  |  |  | Config 2 |  | SR.1.1 TDD |
|  |  |  | Config 3 |  | SR.2.1 TDD |
| RMSI CORESET parameters |  |  | Config 1 |  | CR.1.1 FDD |
|  |  |  | Config 2 |  | CR.1.1 TDD |
|  |  |  | Config 3 |  | CR.2.1 TDD |
| Dedicated CORESET parameters |  |  | Config 1 |  | CCR.1.2 FDD |
|  |  |  | Config 2 |  | CCR.1.2 TDD |
|  |  |  | Config 3 |  | CCR.2.4 TDD |
| OCNG Patterns |  |  |  |  | OP.1 |
| SSB Configuration |  |  | Config 1,2 |  | SSB.1 FR1 |
|  |  |  | Config 3 |  | SSB.2 FR1 |
| SMTC Configuration |  |  |  |  | SMTC.1 |
| Correlation Matrix and Antenna Configuration |  |  |  |  | 1x2 Low |
| TRS Configuration |  | Config 1,4 |  |  | TRS.1.1 FDD |
|  |  | Config 2,5 |  |  | TRS.1.1 TDD |
|  |  | Config 3,6 |  |  | TRS.1.2 TDD |
| EPRE ratio of PSS to SSS |  |  |  | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |
| NocNote 2 | Config 1,2 |  |  | dBm/SCS | -104 |
|  | Config 3 |  |  |  | -101 |
| NocNote 2 |  |  |  | dBm/15kHz | -104 |
| SS-RSRP Note 3 | Config 1,2 |  |  | dBm/SCS | -87 |
|  | Config 3 |  |  |  | -84 |
| Ês/Iot |  |  |  | dB | 17 |
| Ês/Noc |  |  |  | dB | 17 |
| IoNote3 |  |  | Config 1,2 | dBm/ 9.36MHz | -58.96 |
|  |  |  | Config 3 | dBm/ 38.16MHz | -52.86 |
| Propagation Condition |  |  |  |  | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For unpaired spectrum, a DL BWP is linked with an UL BWP. DLBWP.0.2 is linked with ULBWP.0.2; DLBWP.1.1 is linked with ULBWP.1.1; DLBWP.1.3 is linked with ULBWP.1.3 defined in clause 12 of TS 38.213 [8]. |  |  |  |  |  |

During T1, the UE shall start to send the ACK/NACK for PCell from the first UL slot that occurs after the beginning of DL slot (i+TBWPswitchDelay+k1).

During T3, the UE shall start to send the ACK/NACK for PCell from the first UL slot that occurs right after the beginning of DL slot (j+TBWPswitchDelay+k1).

Where, k1 is the timing between DL data receiving and acknowledgement as specified in 38.214[9].

Depending on UE capability bwp-SwitchingDelay [13], UE shall finish BWP switch within the time duration TBWPswitchDelay defined in TS 38.133 [6] Table 8.6.2-1.

All of the above test requirements shall be fulfilled in order for the observed PCell active BWP switch delay to be counted as correct.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	During T1, T3 if there are no uplink resources for reporting the ACK/NACK in the first UL slot that occurs after beginning of DL slot (i+ TBWPswitchDelay+k1), (j+ TBWPswitchDelay+k1), then the UE shall use the next available uplink resource for reporting the corresponding ACK/NACK.

# < Unchanged sections omitted >

##### 6.5.6.2.1	NR SA FR1 RRC-based DL active BWP switch in non-DRX

6.5.6.2.1.1	Test purpose

The purpose of this test is to verify the DL BWP switch delay requirement for RRC-based BWP switch defined in TS 38.133 [6] clause 8.6.

6.5.6.2.1.2	Test applicability

This test applies to all types of NR UE release 15 onwards supporting BWP adaptation of at least 2 BWPs.

6.5.6.2.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.5.6.2.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.6.5.6.2.1.

6.5.6.2.1.4	Test description

6.5.6.2.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.5.6.2.1.4.1-1.

Table 6.5.6.2.1.4.1-1: Supported test configurations for NR SA FR1 RRC-based DL active BWP switch in non-DRX

| Config | Description |
| --- | --- |
| 6.5.6.2.1-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 6.5.6.2.1-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6.5.6.2.1-3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 6.5.6.2.1.4.1-2.

Table 6.5.6.2.1.4.1-2: Initial conditions for NR SA FR1 RRC-based DL active BWP switch in non-DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.5.6.2.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE part. |  |  |

1. The general test parameter settings are set up according to Table 6.5.6.2.1.4.1-3.

2. Message contents are defined in clause 6.5.6.2.1.4.3.

3. The test scenario comprises of one NR Cell (Cell 1). Cell 1 is configured according to Annex C.1.2 and C.1.3.

Table 6.5.6.2.1.4.1-3: General test parameters for DL BWP switch in SA

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1 | One NR radio channel is used for this test |
| Active Cell |  | Cell 1 | PCell on RF channel number 1. |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| T1 | s | 0.2 |  |

6.5.6.2.1.4.2	Test procedure

The test consists of 1 time period, with duration of T1.

PDCCHs indicating new transmissions shall be sent continuously on Cell 1 to ensure that the UE will have ACK/NACK sending.

Cell 1 has constant signal level throughout the test.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to Tables 6.5.6.2.1.4.1-3 and 6.5.6.2.1.5-1. Propagation conditions are set according to Annex C clauses C.2.2.

3.	The SS shall send an RRCReconfiguration message releasing the dedicated configuration of the initialDownlinkBWP and the initialUplinkBWP. This message also configures another UE-specific bandwidth part, BWP-1 and indicates BWP-1 as the active DL BWP using firstActiveDownlinkBWP-Id, according to the initial condition of Active BWP-1 in Table 6.5.6.2.1.5-1.

4.	The UE shall transmit an RRCReconfigurationComplete message.

5.	The SS shall send an RRCReconfiguration message with updated bandwidth part configuration for DL BWP switch, change the BWP according to the final condition of Active BWP-1 in Table 6.5.6.2.1.5-1. T1 starts.

6.	The UE shall receive the RRCReconfiguration in PCell’s slot # denoted i and reconfigure its bandwidth part with the updated bandwidth part configuration.

7	If the UE starts to report valid ACK/NACK for PCell from the first UL slot that occurs after the beginning of DL slot i + X + k1 then the number of successful tests is increased by one. Otherwise, the number of failure tests is increased by one. Where:

- X = 16 for test configuration 6.5.6.2.1-1 and 6.5.6.2.1-2

- X = 32 for test configuration 6.5.6.2.1-3.

8.	After the SS receives the ACK/NACK in step 7) or when T1 expires, the SS shall transmit RRCRelease message to release the RRC connection.

9.	After the RRC connection release, the SS:

-transmits in NR Cell 1 a Paging message for the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5 (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5),
or
- switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5.

10.	Repeat steps 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

The SS verifies the DL BWP switch time in PCell by counting the slots from the time when the RRC Reconfiguration message including updated BWP configuration is sent till a valid ACK/NACK is received.

6.5.6.2.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.5.6.2.1.4.3-1: Common Exception messages for NR SA FR1 RRC-based DL active BWP switch in non-DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |

Table 6.5.6.2.1.4.3-1A: Void

Table 6.5.6.2.1.4.3-1B: Void

Table 6.5.6.2.1.4.3-1C: RRCReconfiguration (1, Step 3, Step 5)

| Derivation Path: TS 38.508-1 [14], Table 4.6.1-13 with condition NR |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |  |
| radioBearerConfig |  | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| masterCellGroup |  | CellGroupConfig | Table 6.5.6.2.1.4.3-1D |  |
| dedicatedNAS-MessageList |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 6.5.6.2.1.4.3-1D: CellGroupConfig (Table 6.5.6.2.1.4.3-1C)

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| spCellConfigDedicated | ServingCellConfig | Table 6.5.6.2.1.4.3-1E |  |
| } |  |  |  |
| } |  |  |  |

Table 6.5.6.2.1.4.3-1E: ServingCellConfig (Table 6.5.6.2.1.4.3-1D)

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-167 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| initialDownlinkBWP SEQUENCE { |  |  |  |
| pdcch-Config CHOICE { |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| pdsch-Config CHOICE { |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| radioLinkMonitoringConfig CHOICE { |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| downlinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF BWP-Downlink { | 1 entry |  |  |
| BWP-Downlink[1] SEQUENCE { | BWP-Downlink | entry 1 Table 6.5.6.2.1.4.3-1F |  |
| } |  |  |  |
| firstActiveDownlinkBWP-Id | 1 | BWP-1 |  |
| defaultDownlinkBWP-Id | 1 | BWP-1 |  |
| uplinkConfig SEQUENCE { |  |  |  |
| initialUplinkBWP SEQUENCE { |  |  |  |
| pucch-Config CHOICE { |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| pusch-Config CHOICE { |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| srs-Config CHOICE { |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| uplinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF BWP-Uplink { | 1 entry |  |  |
| BWP-Uplink[1] | BWP-Uplink | entry 1 Table 6.5.6.2.1.4.3-1G |  |
| } |  |  |  |
| firstActiveUplinkBWP-Id | 1 | BWP-1 |  |
| } |  |  |  |
| } |  |  |  |

Table 6.5.6.2.1.4.3-1F: BWP-Downlink (Table 6.5.6.2.1.4.3-1E)

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-Downlink ::= SEQUENCE { |  |  |  |
| bwp-Id | 1 |  |  |
| bwp-Common SEQUENCE { |  |  |  |
| genericParameters | RIV defined in TS 38.214 [9] that corresponds to DLBWP.1.3 |  | Step 3 |
|  | RIV defined in TS 38.214 [9] that corresponds to DLBWP.1.1 |  | Step 5 |
| } |  |  |  |
| } |  |  |  |

Table 6.5.6.2.1.4.3-1G: BWP-Uplink (Table 6.5.6.2.1.4.3-1E)

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-Uplink ::= SEQUENCE { |  |  |  |
| bwp-Id | 1 |  |  |
| bwp-Common SEQUENCE { |  |  |  |
| genericParameters | RIV defined in TS 38.214 [9] that corresponds to ULBWP.1.3 |  | Step 3 |
|  | RIV defined in TS 38.214 [9] that corresponds to ULBWP.1.1 |  | Step 5 |
| } |  |  |  |
| } |  |  |  |

Table 6.5.6.2.1.4.3-2: Void

6.5.6.2.1.5	Test requirements

Tables 6.5.6.2.1.4.1-3 and 6.5.6.2.1.5-1 define the primary level settings including test tolerances.

Table 6.5.6.2.1.5-1: NR Cell specific test parameters for DL BWP switch

| Parameter |  |  |  |  | Unit | Cell 1 |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency Range |  |  |  |  |  | FR1 |
| Duplex mode |  |  | Config 1 |  |  | FDD |
|  |  |  | Config 2,3 |  |  | TDD |
| TDD configuration |  |  | Config 1 |  |  | Not Applicable |
|  |  |  | Config 2 |  |  | TDDConf.1.1 |
|  |  |  | Config 3 |  |  | TDDConf.2.1 |
| BWchannel |  |  | Config 1 |  |  | 10 MHz: NRB,c = 52 |
|  |  |  | Config 2 |  |  | 10 MHz: NRB,c = 52 |
|  |  |  | Config 3 |  |  | 40 MHz: NRB,c = 106 |
| Active BWP ID |  |  |  |  |  | 1 |
| Initial DL BWP Configuration |  |  | Config 1,2,3 |  |  | DLBWP.0.2 |
| Initial UL BWP Configuration |  |  | Config 1,2,3 |  |  | ULBWP.0.2 |
| Initial Condition | Active DL BWP-1 Configuration |  | Config 1,2,3 |  |  | DLBWP.1.3 |
|  | Active UL BWP-1 Configuration |  | Config 1,2,3 |  |  | ULBWP.1.3 |
| Final Condition | Active DL BWP-1 Configuration |  | Config 1,2,3 |  |  | DLBWP.1.1 |
|  | Active UL BWP-1 Configuration |  | Config 1,2,3 |  |  | ULBWP.1.1 |
| PDSCH Reference measurement channel |  |  | Config 1 |  |  | SR.1.1 FDD |
|  |  |  | Config 2 |  |  | SR.1.1 TDD |
|  |  |  | Config 3 |  |  | SR.2.1 TDD |
| RMSI CORESET parameters |  |  | Config 1 |  |  | CR.1.1 FDD |
|  |  |  | Config 2 |  |  | CR.1.1 TDD |
|  |  |  | Config 3 |  |  | CR.2.1 TDD |
| Dedicated CORESET parameters |  |  | Config 1 |  |  | CCR.1.1 FDD |
|  |  |  | Config 2 |  |  | CCR.1.1 TDD |
|  |  |  | Config 3 |  |  | CCR.2.3 TDD |
| OCNG Patterns |  |  |  |  |  | OP.1 |
| SSB Configuration |  |  | Config 1,2 |  |  | SSB.1 FR1 |
|  |  |  | Config 3 |  |  | SSB.2 FR1 |
| SMTC Configuration |  |  |  |  |  | SMTC.1 |
| TRS Configuration |  |  |  | Config 1 |  | TRS.1.1 FDD |
|  |  |  |  | Config 2 |  | TRS.1.1 TDD |
|  |  |  |  | Config 3 |  | TRS.1.2 TDD |
| Antenna Configuration |  |  |  |  |  | 1x2 Low |
| Propagation Condition |  |  |  |  |  | AWGN |
| EPRE ratio of PSS to SSS |  |  |  |  | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS(Note 1) |  |  |  |  |  |  |
| NocNote 2 |  | Config 1,2 |  |  | dBm/SCS | -104 |
|  |  | Config 3 |  |  |  | -101 |
| SS-RSRP Note 3 |  | Config 1,2 |  |  | dBm/SCS | -87 |
|  |  | Config 3 |  |  |  | -84 |
| Ês/Iot |  |  |  |  | dB | 17 |
| Ês/Noc |  |  |  |  | dB | 17 |
| IoNote3 |  |  | Config 1,2 |  | dBm/ 9.36MHz | -58.96 |
|  |  |  | Config 3 |  | dBm/ 38.16MHz | -52.86 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Note 4: For unpaired spectrum, a DL BWP is linked with an UL BWP. DLBWP.0.2 is linked with ULBWP.0.2; DLBWP.1.1 is linked with ULBWP.1.1; DLBWP.1.3 is linked with ULBWP.1.3 defined in clause 12 of TS 38.213 [8]. |  |  |  |  |  |  |

During T1, the UE shall be ready for the reception of uplink grant for Cell from the first DL slot that occurs right after the beginning of slot i+TRRCprocessingDelay+TBWPswitchDelayRRCNR Slot length and starts to report valid ACK/NACK for PCell from the first UL slot that occurs after the beginning of DL slot  i+TRRCprocessingDelay+TBWPswitchDelayRRCNR Slot length+k1.

Where, k1 is the timing between DL data receiving and acknowledgement as specified in [12].

All of the above test requirements shall be fulfilled in order for the observed Cell active BWP switch delay to be counted as correct.

The rate of correct events observed during repeated tests shall be at least 90%.

< End of changes >