3GPP TSG-RAN5 Meeting # 99	R5-232193

# Incheon, Korea,  22nd - 26th May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.508-3 | CR | 4733 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NTN-IoT: NB-IoT Test Model updates |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN_plus_EPS-UEConTest |  |  |  |  |  | Date: |  |  | 2023.05.09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The NB-IoT NTN test model description has to be extended with explanations on handling of timing advance & scheduling offsets, needed for synchronisation between the UE and the serving satellite. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added new clause 7A.14.2 defining NTN timing considerations. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The description of NTN timing aspects will be missing in the test model. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7A.14.1, 7A.14.2(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 7A.14	Non-Terrestrial Network (NTN)

### 7A.14.1	NTN cell configuration

An NB-IoT cell to be used in a Non-Terrestrial Network is an NB-IoT cell that is broadcasting a system block combination 8 or 9 as defined in clause 8.1.4.3.1.1 of TS 36.508 [3], i.e. it shall at least broadcast SystemInformationBlockType31-NB (SIB31-NB). In addition to that, the NB-IoT NTN cell shall be configured as "barred" in cellAccessRelatedInfo-r13 IE and as "notBarred" in cellAccessRelatedInfo-NTN-r17 IE of SystemInformationBlockType1-NB.

TTCN implementation is responsible for maintaining the system information up-to-date and to ensure that the updated system information is being broadcast in the NB-IoT NTN cell before the timer T317 expires at UE.

### 7A.14.2	NTN timing considerations

#### 7A.14.2.1	General

DL and UL are frame aligned at the UL time synchronization reference point (RP). To accommodate the long propagation delays in NTN, the timing relationships are defined using a Common Timing Advance (Common TA) and two scheduling offsets: Koffset and Kmac, as illustrated in Figure 7A.14.2.1-1.

Figure 7A.14.2.1-1: NTN timing relationship parameters

#### 7A.14.2.2	NTN timing advance

The Timing Advance (TA) formula, specified in TS 36.211 [35] clause 8.1, can be summarized as follows:

TTA = TTA,legacy + TTA,NTN

TTA,NTN = TTA,common + TTA,UE-specific

with:

-	TTA,legacy = (NTA + NTA,offset) * Ts: the (legacy) part of the TA formula that applies also to Terrestrial Networks,

-	TTA,common = NTA,adjcommon * Ts: the Common TA between the RP and the NTN payload,

-	TTA,UE-specific = NTA,adjUE * Ts: the TA corresponding to the service link delay.

The GSO test environment in TS 36.508 [3] specifies that both the serving satellite (NTN payload) and the UE under test have a static position during the lifetime of a test case execution. Therefore the corresponding TTA is also static for a given NTN cell. TTCN provides to the SS the following inputs to enable the SS to calculate TTA:

Table 7A.14.2.2-1: NTN timing advance - input parameters

| TTA element | Associated parameter(s) |  | Relationship |
| --- | --- | --- | --- |
| (in sec) | Protocol parameter | ASP field | (TS 36.213 [30] clause 16.1.2) |
| TTA,legacy |  | InitialValue integer (0..2047) | NTA = InitialValue * 16 |
| TTA,common | NTA,adjcommon | NtnCommonTA integer (0..8316827) | See below. |
| TTA,UE-specific | NTA,adjUE | NtnUeSpecificTA integer (0..8316827) | NTA,adjUE= NtnUeSpecificTA See below. |
| NOTE: The time unit TS is defined in TS 36.211 [35] clause 4. |  |  |  |

From the nta-CommonParameters-17 specified in TS 36.331 [19], only the value of nta-Common-r17 is defined in TS 36.508 [3] SIB31-NB for the GSO scenario. TTCN will forward this value to the SS as NtnCommonTA for the calculation by the SS of NTA,adjcommon according to the formula given in TS 36.213 [30] clause 16.1.2.

NOTE:	For the GSO scenario it is assumed that the SS uses the calculation: NTA,adjcommon = NtnCommonTA.

NTA,adjUE is calculated using the GSO serving satellite static position specified in TS 36.508 [3] SIB31-NB's ephemeris data and the UE static position specified in TS 36.508 [3] clause 4.13. The end result is provided to the SS by TTCN. The calculation is provided hereafter:

The two-way propagation delay in sec between the GSO serving satellite and the non-moving UE is assumed to be constant at any time and is calculated according to the formula:

t=2*d/c

where c is the speed of light (299792458 m/sec) and d is the distance in meters between the serving satellite and the UE.

For two points in Euclidian geomery d in meters is derived by:

d=∆x+∆y+∆z

with

∆x=(xsat-xUE)2

∆y= (ysat-yUE)2

∆z=(zsat-zUE)2

and

xsat, ysat, zsat being serving satellite coordinates, in m

xUE, yUE, zUE being UE coordinates, in m

To use above formulas the posisioning data provided in TS 36.508 [3] is first converted to the coordinates accordingly.

Example of the calculation of ∆x:

given UE latitude 25.08439333 and positionX-r17 of -26044672:

xsat= -26044672 * 1.3 m = -33858073.6m

xUE=-3025296.94 m

∆x=(-33858073.6 m-(-3025296.94 m))2=(-30832776,66 m)2= 950660116565440.7556 m2

Following the above calculations, the final rounded value t results in 0.2475 sec that yields NTA,adjUE to have a value of 7603200.

#### 7A.14.2.3	NTN scheduling offsets

TTCN configures the NTN scheduling offsets Koffset and Kmac in the SS as part of the NTN cell configuration. The SS shall use those parameters to adjust the timing & scheduling of DL & UL transmissions specified in clauses 7A.4, 7A.5 and 7A.6.