3GPP TSG-RAN5 Meeting #99	R5-232206

# Incheon, Korea (Republic Of), 22nd May 2023 - 26th May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4734 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Routine maintenance for TS 36.523-3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160, Anritsu EMEA Ltd. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2023-05-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In case of UECapabilityInformation, the RRC message size can be more than 1000 bits. In such case, additional UL grants should be scheduled by the SS to enable the UE to transmit the message. To handle this case of CAT-M1, one additional UL grant is sufficient. However, to find a generic solution, the number of UL grants that is required should be calculated from the received RRC message size. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following is proposed: - Schedule 4 periodic UL grants. - Upon reception of the RRC message, based on the size of the RRC message, calculate how many UL grants are actually required. - Determine allowed length of latency. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RRC latency procedure would be incomplete in case the RRC message size is greater than the UL grant size. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.12.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 7.12	Test method for RRC signalling latency

### 7.12.0	General

Test cases testing RRC signalling latency require a special test method. The PUCCH synchronisation state of UE influences the test method. Following 2 different ways in which the UE's completeness of procedure can be probed are considered:

1.	UE is still PUCCH synchronized and can respond to uplink grants.

2.	UE needs a RACH procedure and hence RACH procedural delays add upon the actual procedure delay.

### 7.12.1	Procedure delays in PUCCH synchronized state

For latency tests there may be up to 4 HARQ retransmissions in DL (corresponding to the default configuration of the SS) but HARQ retransmissions in UL cannot be compensated , i.e. any HARQ error in UL shall result in an inconclusive verdict for the test case (otherwise a UE may get fail due to a HARQ error).

Figure 7.12.1-1 demonstrates the latency check procedure that will be applied when UE is in PUCCH synchronized state and can respond to uplink grants.

SS is configured to report ACK/NACK received from UE, to TTCN.

NOTE:	Due to L2 signalling (e.g. RLC STATUS PDUs) it is necessary to limit the reporting of UL HARQ ACK/NACK to the time between sending of the RRC message and receiving the ACK.

By default SS is configured to retransmit any DL MAC PDU max 4 times.

To avoid unexpected side effects the Time Alignment timer needs to be set to infinity and the SS shall be configured to not send any Timing Advance MAC control elements during the latency tests (since this may result in additional ACK/NACK)

The SS shall be configured to report HARQ errors and in the case of an UL HARQ error, an inconclusive verdict is assigned.

In the case of HARQ retransmissions in DL the HARQ RTT Timer according to TS 36.321 clause 7.7 [16] is

-	8 for FDD

-	10 for TDD configuration 1 in case the DL PDU is sent in subframe 4 (as per default; see Table 7.12.1-1).

For type B FDD half-duplex, the restrictions provided in clause 7.26 shall be respected.  This results in an RTT of 10.

For Cat M1, explicit timing is applied with T0 pointing to PDSCH.

The SS shall schedule DL retransmission at 4th TTI for FDD and TDD since reception of the NACK.

Let N be the max allowed delay for procedure.

TTCN schedules at time T1 a DL message to the UE.

TTCN schedules UL grants at

T2(k) = T1 + N + 1 + ∆1 + k * RTT;

with 
k = 0..4; number of HARQ retransmission in DL
RTT = 8 (FDD)
RTT = 10 (TDD) 
∆1 = 0 (FDD)
∆1 = 0..3 (TDD; possible UL subframe uncertainty since not all subframes can be used for UL)

Example: 
given TDD; DL PDU sent at subframe 4; N=10
 ∆1 = 1 since UL grant cannot be scheduled for subframe 5 but needs subframe 6 (36.213 [30] cl.8.0)

After first grant scheduled at T2(K), 3 additional grants are scheduled starting with periodicity (P)

with

P = 8ms for FDD

= 10ms for TDD

= 10ms for Half-Duplex FDD

RRC message is received at T3(K).

RrcMsgSize = Size of received RRC message

GrantSize = Size of scheduled UL Grant

X = Ceiling(RrcMsgSize / GrantSize)

Y= GrantSize  - ((RrcMsgSize modulo GrantSize) + PdcpHeaderSize(1B) + X*RlcHeaderSize(2B) + X*MacHeaderSize(2B) + (X-1)*BufferStatusReport(2B) + MAC-I(= 4 if RRC security is active ; = 0 otherwise))

If Y < 0, then, X = X +1

NumOfAdditionalGrantRequired(NAG) set to X – 1

The latency requirements are fulfilled when

T3(K) - T1 = N + 5 + ∆1 + ∆2 + K * RTT + NAG * P

Looking at TDD configuration 1 in detail it can be shown that ∆ = ∆1 + ∆2 = 0 .. 3

T3(K) - T1 = N + 5 + ∆ + k * RTT + NAG * P; with ∆ = 0 .. 3

NOTE:
as long as N is a multiple of 5ms, given T1 is at 4th TTI for TDD configuration 1 we get ∆ = 3 (∆1 = 1 and ∆2 = 2, 36.213 [30] cl.8.0)

<End of modified section>