3GPP TSG-RAN5 Meeting #99	R5-232223

# Incheon, Korea, 22nd - 26th May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-6 | CR | 0095 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of test case 6.3.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, MCenhUEConTest |  |  |  |  |  | Date: |  |  | 2023-05-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Longitude and latitude in Table 6.3.2.3.3-2A are encoded acording to TS 23.032 clause 6.1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarification added that longitude and latitude in Table 6.3.2.3.3-2A are encoded acording to TS 23.032 clause 6.1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent test specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 6.3.2	On-network / Emergency Alert / Emergency alert origination / Emergency alert cancellation / Client Terminated (CT)

6.3.2.1	Test Purpose (TP)

(1)

with { UE (MCVideo Client) registered and authorized for MCVideo Service, including authorized to initiate an emergency alert }

ensure that {
  when { UE (MCVideo Client) receives an emergency alert via s SIP MESSAGE message with the <alert-ind> element in the application/vnd.3gpp.mcvideo-info+xml MIME body set to a value of either "true" or "false" }

then { UE (MCVideo Client) responds by sending a SIP 200 (OK) message }

}

6.3.2.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.281, clauses 11.2.1.1, 11.2.1.3. The following represents a copy/paste extraction of the requirements relevant to the test purpose; any references within the copy/paste text should be understood within the scope of the core spec they have been copied from. Unless otherwise stated, these are Rel-15 requirements.

[TS 24.281 clause 11.2.1.3]

Upon receipt of a "SIP MESSAGE request for emergency notification", the MCVideo client:

1)	if the received SIP MESSAGE request contains an application/vnd.3gpp.mcvideo-info+xml MIME body with the <alert-ind> element set to a value of "true", should display to the MCVideo user an indication of the MCVideo emergency alert and associated information, including:

a)	the MCVideo group identity contained in <mcvideo-calling-group-id> element application/vnd.3gpp.mcvideo-info+xml MIME body;

b)	the originator of the MCVideo emergency alert contained in the <mcvideo-calling-user-id> element of the application/vnd.3gpp.mcvideo-info+xml MIME body; and

c)	the mission critical organization of the MCVideo emergency alert originator contained in the <mc-org> element of the application/vnd.3gpp.mcvideo-info+xml MIME body;

NOTE 1:	This is the case of the MCVideo client receiving the notification of another MCVideo user's emergency alert.

2)	if the received SIP MESSAGE request contains an application/vnd.3gpp.mcvideo-info+xml MIME body with the <alert-ind> element set to a value of "false":

a)	should display to the MCVideo user an indication of the MCVideo emergency alert cancellation and associated information, including:

i)	the MCVideo group identity contained in the <mcvideo-calling-group-id> element application/vnd.3gpp.mcvideo-info+xml MIME body;

ii)	the originator of the MCVideo emergency alert contained in:

A)	if present, the <originated-by> element of the application/vnd.3gpp.mcvideo-info+xml MIME body; or

B)	the <mcvideo-calling-user-id> element of the application/vnd.3gpp.mcvideo-info+xml MIME body; and

b)	if the MCVideo ID contained in the <originated-by> element is the MCVideo ID of the receiving MCVideo user, shall set the MCVideo emergency alert state to "MVEA 1: no-alert"; and

c)	if the received SIP MESSAGE request contains an application/vnd.3gpp.mcvideo-info+xml MIME body with the <emergency-ind> element is set to a value of "false":

i)	shall set the MCVideo emergency group state to "MVEG 1: no-emergency"; and

ii)	shall set the MCVideo emergency group call state to "MVEGC 1: emergency-gc-capable";

NOTE 2:	This is the case of the MCVideo client receiving the notification of the cancellation by a third party of an MCVideo emergency alert. This can be the MCVideo emergency alert of another MCVideo user or the MCVideo emergency alert of the recipient, as determined by the contents of the <originated-by> element. Optionally, notification of the cancellation of the in-progress emergency state of the MCVideo group can be included.

3)	if the received SIP MESSAGE request contains an application/vnd.3gpp.mcvideo-info+xml MIME body with the <emergency-ind> element set to a value of "true":

a)	should display to the MCVideo user an indication of the additional emergency MCVideo user participating in the MCVideo emergency group call including the following if not already displayed as part of step 1):

i)	the MCVideo group identity contained in the <mcvideo-calling-group-id> element application/vnd.3gpp.mcvideo-info+xml MIME body; and

ii)	the <mcvideo-calling-user-id> element of the application/vnd.3gpp.mcvideo-info+xml MIME body;

b)	shall set the MCVideo emergency group state to "MVEG 2: in-progress" if not already set to that value;

NOTE 3:	This is the case of the MCVideo client receiving notification of an additional MCVideo user in an MCVideo emergency state (i.e., not the MCVideo user that originally triggered the in-progress emergency state of the group) joining the in-progress emergency group call. An emergency alert indication, if included, is handled in step 1).

4)	if the received SIP MESSAGE request contains an application/vnd.3gpp.mcvideo-info+xml MIME body with the <emergency-ind> element set to a value of "false":

a)	should display to the MCVideo user an indication of the cancellation of the in-progress emergency state of the MCVideo group call including the following if not already displayed as part of step 2):

i)	the MCVideo group identity contained in the <mcvideo-calling-group-id> element application/vnd.3gpp.mcvideo-info+xml MIME body; and

ii)	the <mcvideo-calling-user-id> element of the application/vnd.3gpp.mcvideo-info+xml MIME body;

b)	shall set the MCVideo emergency group state to "MVEG 1: no-emergency"; and

c)	shall set the MCVideo emergency group call state to "MVEGC 1: emergency-gc-capable";

NOTE 4:	This is the case of the MCVideo client receiving the notification of the cancellation of the in-progress emergency state of the MCVideo group. In this case, the receiving MCVideo client is affiliated with the MCVideo group but not participating in the session. An emergency alert cancellation, if included, is handled in step 2).

5)	if the received SIP MESSAGE request contains an application/vnd.3gpp.mcvideo-info+xml MIME body with the <imminentperil-ind> element set to a value of "true":

a)	should display to the MCVideo user an indication of the MCVideo user participating in the MCVideo imminent peril group call including the following if not already displayed as part of step 1):

i)	the MCVideo group identity contained in the <mcvideo-calling-group-id> element application/vnd.3gpp.mcvideo-info+xml MIME body; and

ii)	the <mcvideo-calling-user-id> element of the application/vnd.3gpp.mcvideo-info+xml MIME body; and

b)	shall set the MCVideo imminent peril group state to "MVIG 2: in-progress" if not already set to that value;

NOTE 5:	This is the case of the MCVideo client receiving notification of an additional MCVideo user initiating an imminent peril group call when there is already an in-progress imminent peril state in effect on the group.

6)	if the received SIP MESSAGE request contains an application/vnd.3gpp.mcvideo-info+xml MIME body with the <imminentperil-ind> element set to a value of "false":

a)	should display to the MCVideo user an indication of the cancellation of the in-progress imminent peril state of the MCVideo group including the following if not already displayed as part of step 2):

i)	the MCVideo group identity contained in the <mcvideo-calling-group-id> element application/vnd.3gpp.mcvideo-info+xml MIME body; and

ii)	the <mcvideo-calling-user-id> element of the application/vnd.3gpp.mcvideo-info+xml MIME body;

b)	shall set the MCVideo imminent peril group state to "MVIG 1: no-imminent-peril"; and

c)	shall set the MCVideo imminent peril group call state to "MVIGC 1: imminent-peril-gc-capable";

NOTE 6:	This is the case of the MCVideo client receiving notification of the cancellation of the in-progress imminent peril state of the group.

7)	shall generate a SIP 200 (OK) response according to rules and procedures of 3GPP TS 24.229 [11]; and

8)	shall send the SIP 200 (OK) response towards the MCVideo server according to rules and procedures of 3GPP TS 24.229 [11].

6.3.2.3	Test description

6.3.2.3.1	Pre-test conditions

System Simulator:

-	SS (MCVideo server)

-	For the underlying "transport bearer" over which the SS and the UE will communicate Parameters are set to the default parameters for the basic E-UTRA Single cell network scenarios, as defined in TS 36.508 [24] clause 4.4. The simulated Cell 1 shall belong to PLMN1 (the PLMN specified for MCVideo operation in the MCVideo configuration document).

IUT:

-	UE (MCVideo client)

-	The test USIM set as defined in TS 36.579-1 [2] clause 5.5.10 is inserted.

Preamble:

-	The UE has performed procedure 'MCVideo UE registration' as specified in TS 36.579-1 [2] clause 5.4.2A.

-	The UE has performed procedure 'MCX Authorization/Configuration and Key Generation' as specified in TS 36.579-1 [2] clause 5.3.2.

-	UE States at the end of the preamble

-	The UE is in E-UTRA Registered, Idle Mode state.

-	The MCVideo Client Application has been activated and User has registered-in as the MCVideo User with the Server as active user at the Client.

6.3.2.3.2	Test procedure sequence

Table 6.3.2.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Check: Does the UE (MCVideo client) correctly perform procedure 'MCX SIP MESSAGE CT' as described in TS 36.579-1 [2] Table 5.3.33.3-1 to receive an emergency alert? | - | - | 1 | P |
| - | EXCEPTION: Step 1Aa1 describes behaviour that depends on the UE implementation; the "lower case letter" identifies a step sequence that take place if the UE displays an indication and associated information incase of an emergency alert. | - | - | - | - |
| 1Aa1 | IF pc_MCX_DisplayInfoEmergencyAlert THEN Check: Does the UE (MCVideo client) notify the user of the emergency alert? (NOTE 1) | - | - | 1 | P |
| 2 | Check: Does the UE (MCVideo client) correctly perform procedure 'MCX SIP MESSAGE CT' as described in TS 36.579-1 [2] Table 5.3.33.3-1 to receive an emergency alert cancellation? | - | - | 1 | P |
| - | EXCEPTION: Step 3a1 describes behaviour that depends on the UE implementation; the "lower case letter" identifies a step sequence that take place if the UE displays an indication and associated information incase of an emergency alert. | - | - | - | - |
| 3a1 | Check: Does the UE (MCVideo client) notify the user of the emergency alert cancellation? (NOTE 1) | - | - | 2 | P |
| NOTE 1: This is expected to be done via a suitable implementation dependent MMI. |  |  |  |  |  |

6.3.2.3.3	Specific message contents

Table 6.3.2.3.3-1: SIP MESSAGE from the SS (step 1, Table 6.3.2.3.2-1;
step 2, TS 36.579-1 [2] Table 5.3.33.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.2-1, condition LOCATION-INFO |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCVideo-Info |  |  |
| MIME-part-body | MCVideo-Info as described in Table 6.3.2.3.3-2 |  |  |  |
| MIME body part |  | Location-info |  |  |
| MIME-part-body | Location-info as described in Table 6.3.2.3.3-2A |  |  |  |

Table 6.3.2.3.3-2: MCVideo-Info in SIP MESSAGE (Table 6.3.2.3.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.2.2-2, condition GROUP-CALL |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcvideoinfo |  |  |  |  |
| mcvideo-Params |  |  |  |  |
| alert-ind | Encrypted <alert-ind> with mcvideoBoolean set to true | Encryption according to NOTE 1 in TS 36.579-1 [2] Table 5.5.3.2.2-2 |  |  |

Table 6.3.2.3.3-2A: Location-Info in SIP MESSAGE (Table 6.3.2.3.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.4.4-2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| location-info |  |  |  |  |
| Report |  |  |  |  |
| CurrentLocation |  |  |  |  |
| CurrentCoordinate |  |  |  |  |
| longitude | px_MCX_CoordinateLongitude_Client_B (NOTE 1, 2) |  |  |  |
| latitude | px_MCX_CoordinateLatitude_Client_B (NOTE 1, 3) |  |  |  |
| NOTE 1: Shall be encrypted as described in TS 36.579-1[2] Table 5.5.3.4.4-2A. NOTE 2: Longitude of px_MCX_CoordinateLongitude_Client_B degrees encoded according to TS 23.032 [36] clause 6.1. NOTE 3: Latitude of px_MCX_CoordinateLatitude_Client_B degrees encoded according to TS 23.032 [36] clause 6.1. |  |  |  |  |

Table 6.3.2.3.3-3: SIP MESSAGE from the SS (step 2, Table 6.3.2.3.2-1;
step 2, TS 36.579-1 [2] Table 5.3.33.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.2-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCVideo-Info |  |  |
| MIME-part-body | MCVideo-Info as described in Table 6.3.2.3.3-4 |  |  |  |

Table 6.3.2.3.3-4: MCVideo-Info in SIP MESSAGE (Table 6.3.2.3.3-3)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.2.2-2, condition GROUP-CALL |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcvideoinfo |  |  |  |  |
| mcvideo-Params |  |  |  |  |
| alert-ind | Encrypted <alert-ind> with mcvideoBoolean set to false | Encryption according to NOTE 1 in TS 36.579-1 [2] Table 5.5.3.2.2-2 |  |  |