3GPP TSG-RAN5 Meeting #99	R5-232243

# Incheon, South Korea, May 22nd – May 26th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 2199 | rev |  | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update inter-band NR CA PC3 reference sensitivity test configuration and test requirement tables |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Verizon, Qualcomm, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_NR_LTE_DC_R16-UEConTest |  |  |  |  |  | Date: |  |  | 2023-05-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Update inter-band NR CA PC3 reference sensitivity test configuration and test requirement tables to be consistent with the corresponding minimum requirement table changes |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update inter-band NR CA PC3 reference sensitivity test configuration and test requirement tables to be consistent with the corresponding minimum requirement table changes for Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC3 aggressor NR UL band n77 for DL NR CA FR1 of CA_n2A-n77A and CA_n5A-n77A |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The corresponding information in test configuration and test requirement tables are not consistent with the updated minimum requirement table. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3A.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of changes>

### 7.3A.1_1	Reference sensitivity power level for 2DL CA exceptions

Editor’s Note: The following aspects are either missing or not yet determined:

- Test point analysis for CA_n3A-n5A IMD2 and IMD4 is currently missing in TR 38.905.

7.3A.1_1.1	Test purpose

To verify the ability of UE that support CA to receive data with a given average throughput for a specified reference measurement channel, under conditions of low signal level, ideal propagation and no added noise when CA exceptions are allowed.

A UE unable to meet the throughput requirement under these conditions will decrease the effective coverage area.

7.3A.1_1.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that support NR 2DL CA

7.3A.1_1.3	Minimum requirements

The minimum conformance requirements are defined in clause 7.3A.0.

7.3A.1_1.4	Test description

7.3A.1_1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on NR operating bands specified in Table 5.2-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in Table 7.3A.1_1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexe A2.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 7.3A.1_1.4.1-1: Test Configuration Table for inter-band 2DL CA exceptions

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | For test frequencies refer to “Range” columns. |  |  |  |  |  |  |
| Test CC Combination setting (CBW) as specified in subclause Table 5.5A.3.1-1 for the CA Configuration across bandwidth combination sets supported by the UE |  |  |  |  |  |  | Refer to “PCC NRB”and “SCC NRB ” columns |  |  |  |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | Lowest |  |  |  |  |  |  |
| Network signalling value |  |  |  |  |  |  | NS_01 Unless given by Table 7.3.2.3-4 for the band with active uplink carrier |  |  |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID | CA Configuration / CBW |  |  |  |  |  | DL Allocation |  |  | UL Allocation (Note 2) |  |  |  |
|  | CA Configuration |  |  |  | PCC | SCC | CC MOD | PCC & SCC RB allocation |  | CC MOD | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |
|  | PCC |  | SCC |  |  |  |  | PCC | SCC |  |  |  |  |
|  | Band | Range | Band | Range |  |  |  |  |  |  |  |  |  |
| Test Settings for CA_n1A-n3A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n1 | 1950 MHz (UL) | n3 | 1760 MHz | 5MHz | 5MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |  |
|  | n1 | Low | n3 | High | 5MHz | Highest | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_4 |  |  |
| Test Settings for CA_n1A-n8A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n1 | 1965 MHz (UL) | n8 | 887,5 MHz | 5MHz | 5MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |  |
| Test Settings for CA_n1A-n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n1 | Mid | n77 | 3900 MHz | 20 MHz | Highest | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 |  | - |
| 2 | n1 | Mid | n77 | 3870 MHz | 20 MHz | 20 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 |  | - |

| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID | CA Configuration / CBW |  |  |  |  |  | DL Allocation |  |  | UL Allocation (Note 2) |  |  |
|  | CA Configuration |  |  |  | PCC | SCC | CC MOD | PCC & SCC RB allocation |  | CC MOD | PCC & SCC RB allocations (LCRB @ RBstart) |  |
|  | PCC |  | SCC |  |  |  |  | PCC | SCC |  |  |  |
|  | Band | Range | Band | Range |  |  |  |  |  |  |  |  |
| Test Settings for CA_n1A-n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n1 | 1950 MHz (UL) | n78 | 3710 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Test Settings for CA_n2A-n48A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n2 | 1860 MHz (UL) | n48 | 3690 MHz | 20 MHz | 20 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 2 | n2 | UL 1852.5/DL 1932.5 | n48 | 3625 MHz | 5 MHz | 20 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Test Settings for CA_n2A-n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n2 | UL 1855/DL 1935 | n66 | UL 1775/DL 2175 | 5 MHz | 5 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 2 | n2 | UL 1883.3/DL 1963.3 | n66 | UL 1750/DL 2150 | 5 MHz | 5 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Test Settings for CA_n2A-n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n2 | 1860 MHz (UL) | n77 | 3720 MHz | 20 MHz | 100 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 2 | n2 | 1860 MHz (UL) | n77 | 3690 MHz | 20 MHz | 20 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 3 | n2 | DL Mid | n77 | 3920 MHz | 20 MHz | 20 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | - | REFSENS_CA_2 |
| 4 | n2 | DL Mid | n77 | 3920 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | - | REFSENS_CA_2 |
| 5 | n2 | UL 1855/DL 1935 | n77 | 3790 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 5 | n2 | UL 1900/DL 1980 | n77 | 3720 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 7 | n2 | UL 1885/DL 1965 | n77 | 3810 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Test Settings for CA_n3A-n5A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n3 | TBD | n77 | TBD | Highest | Highest | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 |  |
| 2 | n3 | 1721 MHz (UL) | n5 | 838 MHz | 10 MHz | 5 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Test Settings for CA_n3A-n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n3 | Mid | n78 | 3495 MHz | Highest | Highest | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 2 | n3 | Mid | n78 | 3465 MHz | 20 MHz | 20 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 3 | n3 | 1740 MHz (UL) | n78 | 3575 ;MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 4 | n3 | 1765 MHz (UL) | n78 | 3435 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Test Settings for a CA_n5A-n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n5 | UL 838/DL 883 | n66 | UL 1721/DL 2121 | 5 MHz | 5 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Test Settings for a CA_n5A-n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n5 | 834 MHz (UL) | n77 | 3336 MHz | 20 MHz | 100 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 2 | n5 | 834 MHz (UL) | n77 | 4170 MHz | 20 MHz | 20 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 3 | n5 | DL Mid | n77 | 3526 MHz | 20 MHz | 20 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | - | REFSENS_CA_2 |
| 4 | n5 | DL Mid | n77 | 3526 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | - | REFSENS_CA_2 |
| 5 | n5 | UL 844/ DL 889 | n77 | 3421 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 6 | n5 | UL 829/ DL 874 | n77 | 4190 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |

| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID | CA Configuration / CBW |  |  |  |  |  | DL Allocation |  |  | UL Allocation (Note 2) |  |  |
|  | CA Configuration |  |  |  | PCC | SCC | CC MOD | PCC & SCC RB allocation |  | CC MOD | PCC & SCC RB allocations (LCRB @ RBstart) |  |
|  | PCC |  | SCC |  |  |  |  | PCC | SCC |  |  |  |
|  | Band | Range | Band | Range |  |  |  |  |  |  |  |  |
| Test Settings for CA_n5A-n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n5 | Mid | n78 | 3346 MHz | Highest | Highest | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 2 | n5 | Mid | n78 | 3316 MHz | 20 MHz | 20 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| Test Settings for CA_n7A-n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n7 | High | n78 | Low | 50 MHz | Highest | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_4 | - |
| Test Settings for CA_n8A-n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n8 | Mid | n78 | 3590 MHz | Highest | Highest | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 2 | n8 | 897.5 MHz (UL) | n78 | 3635 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Test Settings for CA_n26A-n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n66 | 1721 MHz (UL) | n26 | 838 MHz (UL) | 5 MHz | 5 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Test Settings for CA_n29A-n71A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n71 | High | n29 | Low | Highest | 5 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_4 |  |
| 2 | n71 | High | n29 | Low | Highest | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_4 |  |

| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID | CA Configuration / CBW |  |  |  |  |  | DL Allocation |  |  | UL Allocation (Note 2) |  |  |
|  | CA Configuration |  |  |  | PCC | SCC | CC MOD | PCC & SCC RB allocation |  | CC MOD | PCC & SCC RB allocations (LCRB @ RBstart) |  |
|  | PCC |  | SCC |  |  |  |  | PCC | SCC |  |  |  |
|  | Band | Range | Band | Range |  |  |  |  |  |  |  |  |
| Test Settings for CA_n26A-n70A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n70 | 1707.5 MHz (UL) | n26 | 838 MHz (UL) | 5 MHz UL / 25 MHz DL | 5 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Test Settings for a CA_n41A-n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n41 | 2521 MHz | n66 | 2197.5 MHz | 50 MHz | 5 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_4 | - |
| Test Settings for CA_n41A-n71A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n71 | Low | n41 | 2662 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 2 | n71 | Low | n41 | High | 5 MHz | 100 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| Test Settings for CA_n48A-n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n48 | 3660 MHz (UL) | n66 | 1750 MHz (UL) | 5 MHz | 5 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 2 | n66 | High | n48 | Low | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 3 | n66 | High | n48 | Low | 5 MHz | 60 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 4 | n66 | High | n48 | Mid | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| Test Settings for CA_n48A-n70A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n48 | 3695 MHz (UL) | n70 | 1697.5 MHz (UL) | 10 MHz | 15 MHz UL / 25 MHz DL | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Test Settings for CA_n66A-n71A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 | n66 | 1750 MHz (UL) | n71 | 675 MHz (UL) | 5 MHz | 5 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Test Settings for CA_n66A-n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n66 | 1750 MHz (UL) | n77 | 3500 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 2 | n66 | 1750 MHz (UL) | n77 | 3500 MHz | 20 MHz | 100 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 3 | n66 | 1712.5 MHz (UL) | n77 | 3400 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 4 | n66 | UL 1775/ DL 2175 | n77 | 3950 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 5 | n66 | UL 1760/ DL 2160 | n77 | 3720 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |

| Test Settings for CA_n70A-n71A Configuration |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | n71 | Low | n70 | Low | 10 MHz | Highest | CP-OFDM QPSK | Full RB | DFT-s-OFDM QPSK | REFSENS_CA_1 with RB start 10 | - |
| 2 | n71 | Low | n70 | Low | 5 MHz | 5 MHz | CP-OFDM QPSK | Full RB | DFT-s-OFDM QPSK | REFSENS_CA_1 with RB start 10 | - |
| 3 | n70 | 1697.5 MHz (UL) | n71 | 695.5 MHz (UL) | 5 MHz | 5 MHz | CP-OFDM QPSK | Full RB | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Test Settings for CA_n71A-n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n71 | 671 MHz (UL) | n77 | 3309 MHz (UL) | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Note 1: CA Configuration Test CC Combination test settings are checked separately for each CA Configuration. Note 2: REFSENS refers to the PCC bands and PCC NRB ‘s single carrier Uplink RB allocation for reference sensitivity according to table 7.3.2.4.1-3. REFSENS_CA_1 refers to the Uplink RB allocation for reference sensitivity exceptions due to UL harmonic interference according to table 7.3A.0.4-2. REFSENS_CA_2 refers to the Uplink RB allocation for reference sensitivity exceptions due to receiver harmonic mixing according to table 7.3A.0.4-4a. REFSENS_CA_3 refers to the Uplink RB allocation for reference sensitivity exceptions due to intermodulation interference due to 2UL CA according to table 7.3A.0.5-1 for PC3 and table 7.3A.0.5-1a for PC2. REFSENS_CA_4 refers to the Uplink RB allocation for reference sensitivity exceptions due to cross band isolation for NR CA FR1 according to table 7.3A.0.6-2. Note 3: In a band where UE supports 4Rx, the test needs to be performed only with 4Rx antennas connected. Note 4: The test points are executed only when UEs support simultaneous Rx/Tx capability as indicated in Table A.4.3.2A.4.1-3 of TS 38.508-2. |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3A.1_1.4.1-2: Test Configuration Table for intra-band non-contiguous 2DL CA exceptions

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  |  | For test frequencies refer to “Range” columns. |  |  |  |  |  |  |
| Test CC Combination setting (CBW) as specified in subclause Table 5.5A.3.1-1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  |  |  |  | Refer to “PCC NRB”and “SCC NRB ” columns |  |  |  |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  |  |  | Lowest |  |  |  |  |  |  |
| Network signalling value |  |  |  |  |  |  |  |  | NS_01 Unless given by Table 7.3.2.3-4 for the band with active uplink carrier |  |  |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID |  |  | CA Configuration / CBW |  |  |  |  |  | DL Allocation |  |  |  | UL Allocation (Note 2,3) |  |  |
|  | CA Configuration |  |  |  |  | PCC | Wgap / [MHz] | SCC |  | CC MOD | PCC & SCC RB allocation |  | CC MOD | PCC & SCC RB allocations (LCRB @ RBstart) |  |
|  | PCC |  |  | SCC |  |  |  |  |  |  | PCC | SCC |  |  |  |
|  | Band | Range |  | Band | Range |  |  |  |  |  |  |  |  |  |  |
| Test Settings for a CA_n71(2A) Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n71 | CC1 |  | n71 | CC2 | 5MHz | 25.0 | 5MHz |  | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | 5@0 | - |
| 2 | n71 | CC1 |  | n71 | CC2 | 15MHz | 10.0 | 10MHz |  | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | 5@2 | - |
| 3 | n71 | CC1 |  | n71 | CC2 | 15MHz | 5.0 | 10MHz |  | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | 20@19 | - |
| Note 1: CA Configuration Test CC Combination test settings are checked separately for each CA Configuration. Note 2: Use CA Configuration – specific test points if present in the table, otherwise use test points from matching Group Test Settings, if present in the table. Otherwise use the Default Test Settings test points. Note 3: REFSENS_CA_1 refers to the Uplink RB allocation for reference sensitivity exceptions according to table 7.3A.0.2.2-1 Note 4: The Wgap is defined to be widest possible on band based on the PCC and SCC configuration Note 5: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg, only the combination with the highest NRB_PCC is tested Note 6: In a band where UE supports 4Rx, the test needs to be performed only with 4Rx antennas connected. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.3 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals for PCC are initially set up according to Annex C.0, C.1, C.2, and C.3.1, and uplink signals according to Annex G.0, G.1, G.2, and G.3.1.

4.	The UL and Reference Measurement Channel is set according to Tables 7.3A.1_1.4.1-1 and 7.3A.1_1.4.1-2.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 7.3A.1_1.4.3.

7.3A.1_1.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1, C.2 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 38.508-1 [5] clause 5.5.1.  Message contents are defined in clause 7.3A.1_1.1.4.3.

3.	SS activates SCC by sending the activation MAC CE (Refer TS 38.321 [18], clauses 5.9, 6.1.3.10). Wait for at least 2 seconds (Refer TS 38.133[19], clause 9.3).

4.	SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Tables 7.3A.1_1.4.1-1 and 7.3A.1_1.4.1-2. on both PCC and SCC.  The SS sends downlink MAC padding bits on the DL RMC

5.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.2A.1.1.4.1-1 on both PCC and SCC. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

6.	Set the Downlink signal level to the appropriate REFSENS value defined in Table 7.3A.1_1.5-1 and 7.3A.1_1.5-2 for PC3 CA, and in Table 7.3A.1_1.5-1a for PC2 CA. Send continuously uplink power control "up" commands in every uplink scheduling information to the UE to ensure the UE transmits PUMAX level for at least the duration of the throughput measurement.  Allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level.

7.	Measure the average throughput for each component carrier for a duration sufficient to achieve statistical significance according to Annex H.2A.

7.3A.1_1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED and following exception:

For test points with “REFSENS_CA_3” UL configuration in table 7.3A.1_1.4.1-1, message exception in table 7.3A.1_1.4.3-1 applies.

Table 7.3A.1_1.4.3-1: FrequencyInfoUL-SIB

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 20 |  | Power class 3 and Inter-band 2UL CA |
|  | 23 |  | Power class 2 and Inter-band 2UL CA |

7.3A.1_1.5	Test requirement

For inter-band carrier aggregation the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.2.2 with parameters specified in Table 7.3A.1_1.5-1 for PC3, and in Table 7.3A.1_1.5-1a for PC2. The test requirement of configurations for CA operating band including Band n41 also apply for the corresponding CA operating bands with Band n90 replacing Band n41.

Table 7.3A.1_1.5-1: Reference sensitivity requirement for inter band PC3 CA

| CA configuration | Test ID | NR band | SCS kHz | Channel Bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 70 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) |
| CA_n1A-n3A | all | n1 | 15 | -100+23+TT |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | -100 -2.7 +23+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n3 | 15 | -97+TT |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | --97-2.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 | n3 | 15 |  |  |  |  |  |  | -82.3+1.4 +TT |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | -82.3-2.7+1.4+TT4 |  |  |  |  |  |  |
| CA_n1A-n8A | 1 | n1 | 15 | -100+6+TT |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | -100 -2.7 +6+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n8 | 15 | -97+TT |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | -97-2.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n1A-n77A | all | n1 | 15 |  |  |  | -93.8 +TT |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | -93.8 -2.7+TT4 |  |  |  |  |  |  |  |  |  |
|  | 1 | n77 | 30 |  |  |  |  |  |  |  |  |  |  |  |  | -85.1 +13.8+TT |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | -85.1 -2.2+13.8+TT4 |
|  | 2 | n77 | 15 |  |  |  | -92.2 +0.3+TT |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | -92.2 -2.2+0.3+TT4 |  |  |  |  |  |  |  |  |  |
| CA_n1A-n78A | 1 | n1 | 15 | -100 +8+TT |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | -100 -2.7 +10.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n78 | 15 |  | -95.8 +TT |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -95.8 -2.2 +TT4 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n2A-n48A | 1 | n2 | 15 |  |  |  | -91.8+TT |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | -94.5+TT4 |  |  |  |  |  |  |  |  |  |
|  | 1 | n48 | 15 |  |  |  | -92.7 +0.3+TT |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | -94.9 +0.3+TT4 |  |  |  |  |  |  |  |  |  |
|  | 2 | n2 | 15 | -98.0 +12+TT |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | -100.7 +12+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 | n48 | 15 |  |  |  | -92.7 +TT |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | -94.9 +TT4 |  |  |  |  |  |  |  |  |  |
| CA_n2A-n66A | 1 | n2 | 15 | -98.0 +20+TT |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | -100.7 +20+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n66 | 15 | -99.5+TT |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | -102.2+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 | n2 | 15 | -98.0 +TT |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | -100.7 +TT4 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 | n66 | 15 | -99.5 +4+TT |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | -102.2 +4+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n2A-n77A | 1 | n2 | 15 |  |  |  | -91.8+TT |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | -94.5+TT4 |  |  |  |  |  |  |  |  |  |
|  | 1 | n77 | 30 |  |  |  |  |  |  |  |  |  |  |  |  | -85.1 +13.8+TT |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | -87.3 +13.8+TT4 |
|  | 2 | n2 | 15 |  |  |  | -91.8+TT |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | -94.5+TT4 |  |  |  |  |  |  |  |  |  |
|  | 2 | n77 | 15 |  |  |  | -92.2 +0.3+TT |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | -94.4 +0.3+TT4 |  |  |  |  |  |  |  |  |  |
|  | 3 | n2 | 15 |  |  |  | -91.8 +3.7+TT |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | -94.5+ 3.7+TT4 |  |  |  |  |  |  |  |  |  |
|  | 3 | n77 | 15 |  |  |  | -92.2+TT |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | -94.4+TT4 |  |  |  |  |  |  |  |  |  |
|  | 4 | n2 | 15 | -98.0 +6.7+TT |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | -100.7 +6.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 4 | n77 | 15 |  | -95.3+TT |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -97.5+TT4 |  |  |  |  |  |  |  |  |  |  |  |
|  | 5 | n2 | 15 | -98.0 +26+TT |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | -100.7 +28.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 5 | n77 | 15 |  | -95.3+TT |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -97.5+TT4 |  |  |  |  |  |  |  |  |  |  |  |
|  | 6 | n2 | 15 | -98.0 +8+TT |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | -100.7 +10.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 6 | n77 | 15 |  | -95.3+TT |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -97.5+TT4 |  |  |  |  |  |  |  |  |  |  |  |
|  | 7 | n2 | 15 | -98.0 +5+TT |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | -100.7 +5+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 7 | n77 | 15 |  | -95.3+TT |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -97.5+TT4 |  |  |  |  |  |  |  |  |  |  |  |

| CA configuration | Test ID | NR band | SCS kHz | Channel Bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 70 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) |
| CA_n3A-n78A | 1, 2 | n3 | 15 |  |  |  | -90.8 +TT |  | -88.9 +TT |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | -90.8 -2.7 +TT4 |  | -88.9 -2.7+ TT4 |  |  |  |  |  |  |  |
|  | 1 | n78 | 30 |  |  |  |  |  |  |  |  |  |  |  |  | -85.6 +13.8 +TT |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | -85.6 -2.2 +13.8 +TT |
|  | 2 | n78 | 15 |  |  |  | -92.7 +0.3 +TT |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | -92.7 -2.2 +0.3 +TT4 |  |  |  |  |  |  |  |  |  |
|  | 3 | n3 | 15 | -97.0 +[26] +TT |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | -97.0 -2.7 +[28.7] +TT4 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 4 | n3 | 15 | -97.0 +[8] +TT |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | -97.0 -2.7 +[10.7] +TT4 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 3, 4 | n78 | 15 |  | -95.8 +TT |  |  |  |  |  |  |  |  |  |  | -85.1 +TT |
| CA_n5A-n66A | 1 | n5 | 15 | -98.0 +30+TT |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n66 | 15 | -99.5+TT |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | -102.2+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n5A-n77A | 1 | n5 | 15 |  |  |  | -90.8+TT |  |  |  |  |  |  |  |  |  |
|  | 1 | n77 | 30 |  |  |  |  |  |  |  |  |  |  |  |  | -85.1 +13.8+TT |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | -87.3 +13.8+TT4 |
|  | 2 | n5 | 15 |  |  |  | -90.8+TT |  |  |  |  |  |  |  |  |  |
|  | 2 | n77 | 15 |  |  |  | -92.2 +0.3+TT |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | -94.4 +0.3+TT4 |  |  |  |  |  |  |  |  |  |
|  | 3 | n5 | 15 |  |  |  | -90.8 +2.7+TT |  |  |  |  |  |  |  |  |  |
|  | 3 | n77 | 15 |  |  |  | -92.2+TT |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | -94.4+TT4 |  |  |  |  |  |  |  |  |  |
|  | 4 | n5 | 15 | -98.0 +5.7+TT |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 4 | n77 | 15 |  | -95.3+TT |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -97.5+TT4 |  |  |  |  |  |  |  |  |  |  |  |
|  | 5 | n5 | 15 | -98.0 +8.3+TT |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 5 | n77 | 15 |  | -95.3+TT |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -97.5+TT4 |  |  |  |  |  |  |  |  |  |  |  |
|  | 6 | n5 | 15 | -98.0 +5.5+TT |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 6 | n77 | 15 |  | -95.3+TT |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -97.5+TT4 |  |  |  |  |  |  |  |  |  |  |  |

| CA configuration | Test ID | NR band | SCS kHz | Channel Bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 70 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) |
| CA_n5A-n78A | 1, 2 | n5 | 15 |  |  |  | -86.8 +TT |  |  |  |  |  |  |  |  |  |
|  | 1 | n78 | 30 |  |  |  |  |  |  |  |  |  |  |  |  | -85.6 +1.4 +TT |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | -85.6 - 2.2 +1.4 +TT4 |
|  | 2 | n78 | 15 |  |  |  | -92.7 +7.8 +TT |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | -92.7 -2.2 +7.8 +TT4 |  |  |  |  |  |  |  |  |  |
| CA_n7A-n78A | 1 | n7 | 30 |  |  |  |  |  |  |  | -81.5 +4.5 +TT |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | -81.5 -2.7+4.5 +TT4 |  |  |  |  |  |
|  | 1 | n78 | 30 |  |  |  |  |  |  |  |  |  |  |  |  | -85.6 +1.4 +TT |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | -85.6 - 2.2 +1.4 +TT4 |
| CA_n8A-n78A | 1 | n8 | 15 |  |  |  | -85.8 + TT |  |  |  |  |  |  |  |  |  |
|  | 1 | n78 | 30 |  |  |  |  |  |  |  |  |  |  |  |  | -85.6 +1.4 +TT |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | -85.6 -2.2 +1.4 +TT4 |
|  | 2 | n8 | 15 | -97.0 + 8.3 +TT |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 | n78 | 15 |  | -95.8 +TT |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -95.8 -2.2 +TT4 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n26A-n66A | 1 | n26 | 15 | -97.55 + 30 +TT |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n66 | 15 | -99.5 +TT |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n26A-n70A | 1 | n26 | 15 | -97.55 + 30 +TT |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n70 | 15 |  |  |  |  | -92.7 + TT |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -92.7 - 2.7 +TT4 |  |  |  |  |  |  |  |  |
| CA_n29A-n71A | 1 | n29 | 15 | -97.0 + 17.5 + TT |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n71 | 15 |  |  |  | -86.0 + TT |  |  |  |  |  |  |  |  |  |

| CA configuration | Test ID | NR band | SCS kHz | Channel Bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 70 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) |
| CA_n41A-n66A | 1 | n41 | 30 |  |  |  |  |  |  |  | -88.9 + TT |  |  |  |  |  |
|  | 1 | n66 | 30 | -96.6 + 3.5 +TT |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n41A-n71A | 1 | n41 | 15 |  | -94,8 + 10.8 + TT |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n71 | 15 | -97.2 + TT |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 | n41 | 30 |  |  |  |  |  |  |  |  |  |  |  |  | -94.0 + 1.4 + TT |
|  | 2 | n71 | 15 | -97.2 +TT |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n48A-n66A | 1 | n48 | 15 | -99.0 + TT |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n66 | 15 | -99.5 + 5.0 + TT |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | -99.5 +5.0 -2.7 +TT |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 | n48 | 15 |  | -95.8 + 23.9 + TT |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 | n66 | 15 | -99.5 +TT |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | -99.5 -2.7 +TT |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 3 | n48 | 30 |  |  |  |  |  |  |  |  | -96.1 + 8.3 +TT |  |  |  |  |
|  | 3 | n66 | 15 | -99.5 +TT |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | -99.5 -2.7 +TT |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 4 | n48 | 15 |  | -95.8 + 1.1. + TT |  |  |  |  |  |  |  |  |  |  |  |
|  | 4 | n66 | 15 | -99.5 +TT |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | -99.5 -2.7 +TT |  |  |  |  |  |  |  |  |  |  |  |  |

| CA configuration | Test ID | NR band | SCS kHz | Channel Bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 70 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) |
| CA_n48A-n70A | 1 | n48 | 15 |  | -95.8 + TT |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n70 | 15 |  |  |  |  | -92.7 +26 +TT |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -92.7 - 2.7 +28.4 +TT4 |  |  |  |  |  |  |  |  |
| CA_n66A-n71A | 1 | n66 | 15 | -99.5 +5 +TT |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | -99.5 -2.7 +5 +TT4 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n71 | 15 | -97.2 +TT |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n70A-n71A | 1 | n70 | 15 |  |  |  |  | -92.7 +4.1 +TT |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -92.7 -2.7 +4.1 +TT4 |  |  |  |  |  |  |  |  |
|  | 2 | n70 | 15 | -100.0 +9.9 +TT |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | -100.0 -2.7 +9.9 +TT4 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 3 | n70 | 15 | -100.0 +5 +TT |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | -100.0 -2.7 +5 +TT4 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | All | n71 | 15 | -97.2 +TT | -94.0 +TT |  |  |  |  |  |  |  |  |  |  |  |
| CA_n71A-n77A | 1 | n71 | 15 | -97.2 +5.5 +TT |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n77 | 15 |  |  |  | -95.3 +TT |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | -95.3 -2.2 +TT4 |  |  |  |  |  |  |  |  |  |
|  | 1 | n66 | 15 | -99.5+TT |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | -102.2+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n77 | 15 |  | -95.3 +23.9+TT |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -97.5 +23.9+TT4 |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 | n66 | 15 |  |  |  | -93.3+TT |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | -96.0+TT4 |  |  |  |  |  |  |  |  |  |
|  | 2 | n77 | 30 |  |  |  |  |  |  |  |  |  |  |  |  | -85.1 +13.8+TT |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | -87.3 +13.8+TT4 |
| CA_n66A-n77A | 3 | n66 | 15 | -99.5+TT |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | -102.2+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 3 | n77 | 15 |  | -95.3 +1.1+TT |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -97.5 +1.1+TT4 |  |  |  |  |  |  |  |  |  |  |  |
|  | 4 | n66 | 15 | -99.5 +31 +TT |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | -102.2 +31 +TT4 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 4 | n77 | 15 |  | -95.3 +TT |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -97.5+TT4 |  |  |  |  |  |  |  |  |  |  |  |
|  | 5 | n66 | 15 | -99.5 +5 +TT |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | -102.2 +5 +TT4 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 5 | n77 | 15 |  | -95.3 +TT |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -97.5+TT4 |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: The transmitter shall be set to maximum output power level (Table 7.3A.3.5-2) Note 2: The reference measurement channel is specified in Annexe A2.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2. Note 3: TT for each frequency and channel bandwidth is specified in Table 7.3.2.5-3. Note 4: Applicable only if operation with 4 antenna ports is supported in the band with carrier aggregation configured. Note 5: Values are modified by -0.5dB when carrier channel BW is between 865MHz and 894MHz. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3A.1_1.5-1a: Reference sensitivity requirement for inter band PC2 CA

| CA configuration | Test ID | NR band | SCS kHz | Channel Bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 70 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) |
| CA_n1A-n78A | 1 | n1 | 15 | -100 +17.8+TT |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | -100 -2.7 +17.8+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n78 | 15 |  | -95.8 +TT |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -95.8 -2.2 +TT4 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n78A | 3 | n3 | 15 | -97.0 +31.9 +TT |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | -97.0 -2.7 +31.9 +TT4 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 4 | n3 | 15 | -97.0 +18.5 +TT |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | -97.0 -2.7 +18.5 +TT4 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 3, 4 | n78 | 15 |  | -95.8 +TT |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -95.8 -2.2 +TT4 |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: The transmitter shall be set to maximum output power level (Table 7.3A.3.5-2) Note 2: The reference measurement channel is specified in Annexe A2.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2. Note 3: TT for each frequency and channel bandwidth is specified in Table 7.3.2.5-3. Note 4: Applicable only if operation with 4 antenna ports is supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3A.1_1.5-2: Reference sensitivity requirement for intraband non-contiguous CA

| CA configuration | Test ID | NR band | SCS kHz | Channel Bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 70 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) |
| CA_n71(2A) | 1 | n71 | 15 | -97.2 +TT for PCC -97.2 + 4.0 +TT for SCC |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 | n71 | 15 |  | -94.0 +22.2 +TT | -91.6 +TT |  |  |  |  |  |  |  |  |  |  |
|  | 3 | n71 | 15 |  | -94.0 +5.2 +TT | -91.6 +TT |  |  |  |  |  |  |  |  |  |  |
| Note 1: The transmitter shall be set to maximum output power level (Table 7.3A.3.5-2) Note 2: The reference measurement channel is specified in Annexe A2.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2. Note 3: TT for each frequency and channel bandwidth is specified in Table 7.3.2.5-3. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

<End of changes>