3GPP TSG-RAN WG5 Meeting #99	R5-232264

Incheon, South Korea, 22nd – 26th May, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-1 | CR | 0175 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new positioning test case for pre-configured PRS processing window procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2023-05-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Pre-configured PRS processing window procedures were introduced in Rel-17 positioning WI. A new test case need to be introduced in TS 37.271-2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new test case 9.4.5 Pre-configured PRS processing window procedures was added |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test spec will not applicable to Release-17 NR positioning methods signal test |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.4.5(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 37.571-3 CR 0161 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

9.4.5	Pre-configured PRS processing window procedures

9.4.5.1	Test Purpose (TP)

(1)

with { a NAS signalling connection existing }

ensure that {

when { UE has no assistance data stored and PRS processing window was configured with associated ID via RRCReconfiguration message}}

then { the UE sends a RRCReconfigurationComplete message to confirm the reception of pre-configured PRS processing window configuration }

}

(2)

with { a NAS signalling connection existing }

ensure that {

when { UE was configured with PRS processing window and receives PPW Activation Command and then UE receives a location request from LMF }

then { the UE sends a PROVIDE LOCATION INFORMATION message containing a location estimate }

}

9.4.5.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.305 clause 7.8, TS 38.321 clauses 5.18.21and 5.24, TS 38.214 clause 5.1.6.5 and TS 38.133 clause9.9.1.2. Unless otherwise stated these are Rel-17 requirements.

[TS 38.305, clause 7.8]

The pre-configured PRS processing window procedure is used by the network to provide PRS processing window for NR DL-PRS measurements to the UE without measurement gap. The gNB may activate the pre-configurated PRS processing window upon receiving the request from LMF.

…

Figure 7.8.2-1 shows the general positioning procedure for Pre-configured PRS processing window.

Figure 7.8.2-1: Pre-configured PRS processing window configuration procedure

0.	LMF obtains the TRP information required for positioning services from the gNBs.

1.	The LMF provides the PRS information of the neighbour TRPs to the serving gNB and requests the serving gNBs to pre-configure PRS processing window configuration(s) via NRPPa MEASUREMENT PRECONFIGURATION REQUIRED message.

2.	Based on the assistance information from the LMF and the UE capability, the serving gNB provides pre-configured PRS processing window configuration(s) with associated ID(s) to the UE by sending RRC Reconfiguration message specified in TS 38.331 [14].

3.	The UE sends RRC Reconfiguration complete message to the gNB to confirm the reception of pre-configured PRS processing window configuration(s).

4.	The gNB sends the confirmation message to the LMF to indicate the success of the pre-configuration via NRPPa MEASUREMENT PRECONFIGURATION CONFIRM message.

5.	The LMF sends the NRPPa MEASUREMENT ACTIVATION message to request the gNB to (de)activate the preconfigured PRS processing window.

6.	Based on the request from the LMF in step 5, the gNB sends DL MAC CE PPW Activation/Deactivation Command containing an ID to activate/deactivate the associated PRS processing window.

[TS 38.321, clause 5.18.21]

If the UE is configured with pre-configured PPW, the network may send DL MAC CE for PPW Activation/Deactivation Command to the UE as in clause 6.1.3.42. For the activated PPW, the UE shall follow the specified UE behaviour in clause 5.24.

Upon activation of DL BWP, the PPW(s) configured for that BWP are considered deactivated. Upon reconfiguration of PPW(s) of the active DL BWP, all the PPW(s) for that BWP are considered deactivated.

Upon the reception of the MAC CE for PPW Activation/Deactivation Command, the MAC entity shall:

1>	if the DL MAC CE for PPW Activation/Deactivation Command indicates the deactivation of a pre-configured PPW:

2>	deactivate the PPW.

1>	else if the DL MAC CE for PPW Activation/Deactivation Command indicates the activation of a pre-configured PPW:

2>	activate the PPW according to the procedure specified in clause 5.24.

[TS 38.321, clause 5.24]

When PPW is activated and PRS has higher priority than DL channel and signals, for the affected symbols within the PPW according to clause 5.1.6.5 in TS 38.214 [7], the MAC entity shall:

1>	if the ra-ResponseWindow or the ra-ContentionResolutionTimer or the msgB-ResponseWindow is running:

2>	monitor the PDCCH as specified in clauses 5.1.4 and 5.1.5.

1>	else:

2>	not receive DL-SCH;

2>	not receive PDCCH.

[TS 38.214, clause 5.1.6.5]

The UE is expected to measure the DL PRS outside the measurement gap, subject to UE capability, if the DL PRS is inside the active DL BWP and has the same numerology as the active DL BWP and is within the DL PRS processing window indicated by higher layer parameter DL-PPW-PreConfig. The UE is not expected to measure the DL PRS outside the measurement gap if the expected received timing difference between the DL PRS from the non-serving cell and that from the serving cell, determined by the higher layer parameters nr-DL-PRS-ExpectedRSTD and nr-DL-PRS-ExpectedRSTD-Uncertainty, is larger than maximum Rx timing difference provided by UE capability. For receiving the DL PRS outside the measurement gap and within the DL PRS processing window, the UE determines the DL PRS priority as indicated by higher layer parameter priority subject to UE capability or as implied by UE capability:

-	with value 'st1' where the DL PRS is higher priority than all the DL signal/channels except SSB, or

-	with value 'st2' where the DL PRS is lower priority than PDCCH and the PDSCH scheduled by DCI formats 1_1 or 1_2 with the priority indicator field in the corresponding DCI format set to 1, and is higher priority than other DL signals/channels except SSB, or

-	with value 'st3' where the DL PRS is lower priority than all the DL signals/channels except SSB.

Inside one DL-PPW-PreConfig the UE is only expected to measure a single DL PRS positioning frequency layer.

When the UE is expected to measure the DL PRS outside the measurement gap in a configured PRS processing window with type1A and if the DL PRS is determined to be higher priority than the DL signals and channels inside the PRS processing window, those DL signals and channels are not expected to be measured by the UE. When the UE is expected to measure the DL PRS outside the measurement gap in a configured PRS processing window with type1B and if the DL PRS is determined to be higher priority than the DL signals and channels inside the PRS processing window, those DL signals and channels in the same band as the DL PRS are not expected to be measured by the UE. When the UE is expected to measure the DL PRS outside the measurement gap in a configured PRS processing window with type2 if the DL PRS is determined to be higher priority than the DL signals and channels inside the PRS processing window, those DL signals and channels from the impacted serving cells are not expected to be measured by the UE on the overlapped symbols with the DL PRS, where impacted serving cells refer to the serving cell on which the DL-PPW-PreConfig is configured for a frequency range 1 band, and all the serving cells in the same band as the DL PRS for a frequency range 2 band.

…

For a UE configured with PRS Processing Window(s), when the UE receives an activation/deactivation command, as described in clause 6.1.3.42 of [10, TS 38.321], for a PRS processing window activation, and when the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the command, the corresponding actions in [10, TS 38.321] and the UE assumptions shall be applied starting from the first slot that is after slot n+3Nslotsubframe,µ where  is the SCS configuration for the PUCCH. The UE is not expected to be indicated with more than 4 activated PRS processing windows by higher layer parameter DL-PPW-PreConfig across all active DL BWPs and is not expected to be indicated with the activated PRS processing windows DL-PPW-PreConfig that overlap in time.

[TS 38.133, clause 9.9.1.2]

The requirements for RSTD, PRS-RSRP, UE Rx-Tx time difference, and PRS-RSRPP measurement without measurement gaps specified in clauses 9.9.2.7, 9.9.3.6, 9.9.4.6 and 9.9.6.6 shall apply provided that:

UE is configured with PPW,

No active BWP switching occurs during PPW,

PRS is within PPW and does not overlap with other signals/channels of higher priority,

-	for PPW type 1A/1B, the PPW does not overlap with any symbol for SSB-based RLM/BFD/CBD/L1-RSRP/L1-SINR measurement on any CC or for SSB based RRM measurement on any MOs that are measured outside measurement gaps,

-	for PPW type 2, PRS does not overlap with any symbol for SSB-based RLM/BFD/CBD/L1-RSRP/L1-SINR measurement on any CC or for SSB based RRM measurement on any MOs that are measured outside measurement gaps,

max∣ΔT∣≤ THR, where

∆T is the time difference between the start of a slot containing PRS from the neighbor cell/TRP and the start of the closest slot from the serving cell;

the range of ∆T is determined by the expected RSTD and expected RSTD uncertainty in the assistance data;

THR is the threshold as reported in UE capability prs-MeasurementWithoutMG-r17.

SCS of PRS within PPW and SCS of DL active BWP are the same.

9.4.5.3	Test description

9.4.5.3.1	Pre-test conditions

System Simulator:

-	Sub-test 26: NR Cell 1 as specified in 8.2.9

-	Sub-test 27: NR Cell 1 and NR Cell 2 as specified in 8.2.10.

-	Sub-test 28: NR Cell 1, NR Cell 2 and NR Cell 3 as specified in 8.2.11.

UE:

- The UE shall begin the test with no assistance data stored.

Preamble:

-	The UE is in state 3N-A as defined in TS 38.508-1 [30], subclause 4.4A on NR Cell 1.

-	Then the SS sends the RESET UE POSITIONING STORED INFORMATION message to the UE to clear the stored assistance data in the UE.

Related PICS/PIXIT Statements:

-

9.4.5.3.2	Test procedure sequence

This test case includes sub-test cases dependent on the positioning method(s) supported by the UE. Each sub-test case is identified by a sub-test case number as defined in Table 9.4.5.3.2-1 below:

Table 9.4.5.3.2-1: Sub-test case numbers

| Sub-Test Case Number | Supported Positioning Methods |
| --- | --- |
| 26 | UE supporting Multi-RTT (Rel-17 onwards) |
| 27 | UE supporting DL-AoD (Rel-17 onwards) |
| 28 | UE supporting DL-TDOA (Rel-17 onwards) |

Table 9.4.5.3.2-2: Test Configuration

| Test Configuration | Network Deployment Type | Test Implementation |
| --- | --- | --- |
| B | NG-RAN NR |  |

Table 9.4.5.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS sends an LPP message of type Request Capabilities. | <-- | DLInformationTransfer (LPP REQUEST CAPABILITIES) | - | - |
| 2 | The UE sends an LPP message of type Provide Capabilities including the UE positioning capabilities. | --> | ULInformationTransfer (LPP PROVIDE CAPABILITIES) | - | - |
| 3 | IF the UE LPP message at step 2 includes an acknowledgment request THEN SS sends a LPP Acknowledgement response. | <-- | DLInformationTransfer (LPP ACKNOWLEDGEMENT) | - | - |
| 4 | The SS sends an LPP message of type Provide Assistance Data. | <-- | DLInformationTransfer (LPP PROVIDE ASSISTANCE DATA) | - | - |
| 5 | The SS sends an RRCReconfiguration message to configure PRS processing window with associated ID. | <-- | RRC RRCReconfiguration |  |  |
| 6 | Check: Does the UE send an RRCReconfigurationComplete message to confirm the reception of pre-configured PRS processing window configuration? | --> | RRCReconfigurationComplete | 1 | P |
| 7 | The SS sends a PPW Activation Command containing a PPW ID to activate the associated PRS processing window. | <-- | MAC CE (PPW Activation Command) |  |  |
| 8 | The SS sends an LPP message of type Request Location Information including a request for location estimate and location measurements. | <-- | DLInformationTransfer (LPP REQUEST LOCATION INFORMATION) | - | - |
| 9 | Check: Does the UE send an LPP message of type Provide Location Information including location estimate or location measurements? | --> | ULInformationTransfer (LPP PROVIDE LOCATION INFORMATION) | 2 | P |
| 10 | IF the UE LPP message at step 9 includes an acknowledgment request THEN SS sends a LPP Acknowledgement response. | <-- | DLInformationTransfer (LPP ACKNOWLEDGEMENT) | - | - |

9.4.5.3.3	Specific message contents

Table 9.4.5.3.3-1: RESET UE POSITIONING STORED INFORMATION (Preamble)

| Derivation Path: 38.509 clause 6.6 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UE Positioning Technology |  | Sub-test 26: 0 0 0 0 0 1 1 0 Sub-test 27: 0 0 0 0 1 0 0 0 Sub-test 28: 0 0 0 0 0 1 1 1 | Sub-test 26: Multi-RTT Sub-test 27: DL-AoD Sub-test 28: DL-TDOA |  |

Table 9.4.5.3.3-2: DLInformationTransfer (step 1, step 3, step 4, step 8 and step 10, Table 9.4.5.3.2-3)

| Derivation Path: 38.331 clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| DLInformationTransfer ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| dlInformationTransfer SEQUENCE { |  |  |  |  |
| dedicatedNAS-Message OCTET STRING |  | Set according to Table 9.4.5.3.3-3 | DL NAS TRANSPORT |  |
| nonCriticalExtension SEQUENCE {} |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 9.4.5.3.3-3: DL NAS TRANSPORT (DLInformationTransfer, Table 9.4.5.3.3-2)

| Derivation Path: 24.501 Table 8.2.11.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended Protocol discriminator |  | 01111110 | 5GS mobility management messages |  |
| Security header type |  | 0000 | Plain 5GS NAS message |  |
| Spare half octet |  | 0000 | Downlink generic NAS transport |  |
| DL NAS TRANSPORT message identity |  | 01101000 | DL NAS transport |  |
| Payload container type |  | 0011 | LTE Positioning Protocol (LPP) message container |  |
| Spare half octet |  | 0000 |  |  |
| Payload container |  | Step 1: Set according to Table 8.4-2 | LPP Request Capabilities. |  |
|  |  | Steps 3 and 10: Set according to Table 9.4.5.3.3-4 | LPP Acknowledgement |  |
|  |  | Step 4: Set according to Table 9.4.5.3.3-5 | LPP Provide Assistance Data |  |
|  |  | Step 8: Set according to Table 9.4.5.3.3-7 | LPP Request Location Information |  |
| Additional information |  | Present | Routing Identifier/Correlation ID |  |

Table 9.4.5.3.3-4: LPP Acknowledgement (DL NAS TRANSPORT, Table 9.4.5.3.3-3)

| Derivation Path: 37.355 clause 6.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| LPP-Message ::= SEQUENCE { |  |  |  |  |
| transactionID |  | Not present |  |  |
| endTransaction |  | TRUE |  |  |
| sequenceNumber |  | Not present |  |  |
| acknowledgement SEQUENCE { |  |  |  |  |
| ackRequested |  | FALSE |  |  |
| ackIndicator |  | (0..255) | Contains the same value of the sequenceNumber field in steps 2 or 9, Table 9.4.5.3.2-3 |  |
| } |  |  |  |  |
| lpp-MessageBody |  | Not present. |  |  |
| } |  |  |  |  |

Table 9.4.5.3.3-5: LPP Provide Assistance data (DL NAS TRANSPORT, Table 9.4.5.3.3-3)

| Derivation Path: Table 8.4-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| As defined in Table 8.4-1 with the following exceptions: |  |  |  |
| LPP-Message ::= SEQUENCE { |  |  |  |
| transactionID SEQUENCE { |  |  |  |
| initiator | locationServer |  |  |
| transactionNumber | (0..255) |  |  |
| } |  |  |  |
| lpp-MessageBody CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| provideAssistanceData SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| provideAssistanceData-r9 SEQUENCE { |  |  |  |
| nr-Multi-RTT-ProvideAssistanceData-r16 SEQUENCE { |  |  |  |
| nr-DL-PRS-AssistanceData-r16 | As defined in Table 9.4.5.3.3-6 |  |  |
| } |  |  |  |
| nr-DL-AoD-ProvideAssistanceData-r16 SEQUENCE { |  |  |  |
| nr-DL-PRS-AssistanceData-r16 | As defined in Table 9.4.5.3.3-6 |  |  |
| } |  |  |  |
| nr-DL-TDOA-ProvideAssistanceData-r16 SEQUENCE { |  |  |  |
| nr-DL-PRS-AssistanceData-r16 | As defined in Table 9.4.5.3.3-6 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 9.4.5.3.3-6: NR-DL-PRS-AssistanceData

| Derivation Path: Table 8.4.1.6-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| As defined in Table 8.4.1.6-1 with the following exceptions: |  |  |  |
| NR-DL-PRS-AssistanceData-r16 ::= SEQUENCE { |  |  |  |
| nr-DL-PRS-AssistanceDataList-r16 SEQUENCE (SIZE (1..nrMaxFreqLayers-r16)) OF NR-DL-PRS-AssistanceDataPerFreq-r16 { | 2 entries |  |  |
| NR-DL-PRS-AssistanceDataPerFreq-r16[1] SEQUENCE { |  | entry 1 |  |
| nr-DL-PRS-AssistanceDataPerFreq-r16 SEQUENCE (SIZE (1..nrMaxTRPsPerFreq-r16)) OF NR-DL-PRS-AssistanceDataPerTRP-r16{ | 2 entries |  |  |
| NR-DL-PRS-AssistanceDataPerTRP-r16[1] SEQUENCE { |  | entry 1 |  |
| nr-DL-PRS-ExpectedRSTD-r16 | 0 |  |  |
| nr-DL-PRS-ExpectedRSTD-Uncertainty-r16 | 0 |  |  |
| } |  |  |  |
| NR-DL-PRS-AssistanceDataPerTRP-r16[2] SEQUENCE { |  | entry 2 |  |
| nr-DL-PRS-ExpectedRSTD-r16 | 0 |  |  |
| nr-DL-PRS-ExpectedRSTD-Uncertainty-r16 | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NR-DL-PRS-AssistanceDataPerFreq-r16[2] SEQUENCE { |  | entry 2 |  |
| nr-DL-PRS-AssistanceDataPerFreq-r16 SEQUENCE (SIZE (1..nrMaxTRPsPerFreq-r16)) OF NR-DL-PRS-AssistanceDataPerTRP-r16{ | 1 entry |  |  |
| NR-DL-PRS-AssistanceDataPerTRP-r16[1] SEQUENCE { |  | entry 1 |  |
| nr-DL-PRS-ExpectedRSTD-r16 | 0 |  |  |
| nr-DL-PRS-ExpectedRSTD-Uncertainty-r16 | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 9.4.5.3.3-7: LPP Request Location Information (DL NAS TRANSPORT, Table 9.4.5.3.3-3)

| Derivation Path: Table 8.4-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| As defined in Table 8.4-3 with the following exceptions: |  |  |  |  |
| locationInformationType |  | locationMeasurementsPreferred |  |  |

Table 9.4.5.3.3-8: RRCReconfiguration (step 5, Table 9.4.5.3.2-3)

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| masterCellGroup |  | OCTET STRING (CONTAINING CellGroupConfig) |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 9.4.5.3.3-9: CellGroupConfig (RRCReconfiguration, Table 9.4.5.3.3-8)

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| spCellConfigDedicated SEQUENCE { |  |  |  |
| initialDownlinkBWP SEQUENCE { |  |  |  |
| dl-PPW-PreConfigToAddModList-r17 SEQUENCE (SIZE (1..maxNrofPPW-Config-r17)) OF DL-PPW-PreConfig-r17 { | 1 entry |  |  |
| DL-PPW-PreConfig-r17[1] SEQUENCE { |  | entry 1 |  |
| dl-PPW-ID-r17 | 0 |  |  |
| dl-PPW-PeriodicityAndStartSlot-r17 | The periodicity is 160ms and the starting slot offset is 11 ms for any SCS configuration |  |  |
| length-r17 | The length of DL-PRS processing window is 11 ms for any SCS configuration |  |  |
| type-r17 | type1A or type1B or type2 | Set depending on UE capabilities. Set to any supported type when UE supporting multiple PPW types |  |
| priority-r17 | st1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 9.4.5.3.3-10: ULInformationTransfer (step 2 and step 9, Table 9.4.5.3.2-3)

| Derivation Path: 38.331 clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| ULInformationTransfer ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| ulInformationTransfer SEQUENCE { |  |  |  |  |
| dedicatedNAS-Message OCTET STRING |  | Set according to Table 9.4.5.3.3-11 | UL NAS TRANSPORT |  |
| nonCriticalExtension SEQUENCE {} |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 9.4.5.3.3-11: UL NAS TRANSPORT (ULInformationTransfer, Table 9.4.5.3.3-10)

| Derivation Path: 24.501 Table 8.2.10.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended Protocol discriminator |  | 01111110 | 5GS mobility management messages |  |
| Security header type |  | 0000 | Plain 5GS NAS message |  |
| Spare half octet |  | 0000 |  |  |
| UL NAS TRANSPORT message identity |  | 01100111 | UL NAS TRANSPORT |  |
| Payload container type |  | 0011 | LTE Positioning Protocol (LPP) message container |  |
| Spare half octet |  | 0000 |  |  |
| Payload container |  | Step 2: Set according to Table 9.4.5.3.3-12 | LPP Provide Capabilities |  |
|  |  | Step 9: Set according to Table 9.4.5.3.3-13 | LPP Provide Location Information |  |
| Additional information |  | Present | The UE includes the Routing Identifier received in the Additional Information IE of the DOWNLINK GENERIC NAS TRANSPORT message (step 1 Table 9.4.5.3.2-3) |  |

Table 9.4.5.3.3-12: LPP Provide Capabilities (UL NAS TRANSPORT, Table 9.4.5.3.3-11)

| Derivation Path: 37.355 clause 6.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| LPP-Message ::= SEQUENCE { |  |  |  |  |
| transactionID SEQUENCE { |  |  |  |  |
| initiator |  | locationServer |  |  |
| transactionNumber |  | (0..255) | Contains the same value as the corresponding field in the LPP Request Capabilities message in step 1, Table 9.4.5.3.2-3 |  |
| } |  |  |  |  |
| endTransaction |  | TRUE |  |  |
| sequenceNumber |  | (0..255) |  |  |
| acknowledgement SEQUENCE { |  | Present, or not present |  |  |
| ackRequested |  | TRUE |  |  |
| ackIndicator |  | Not present |  |  |
| } |  |  |  |  |
| lpp-MessageBody CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| provideCapabilities SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| provideCapabilities-r9 SEQUENCE { |  |  |  |  |
| commonIEsProvideCapabilities |  | Dependent on UE capabilities |  |  |
| nr-Multi-RTT-ProvideCapabilities-r16 |  | Dependent on UE capabilities |  |  |
| nr-DL-AoD-ProvideCapabilities-r16 |  | Dependent on UE capabilities |  |  |
| nr-DL-TDOA-ProvideCapabilities-r16 |  | Dependent on UE capabilities |  |  |
| nr-UL-ProvideCapabilities-r16 |  | Dependent on UE capabilities |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 9.4.5.3.3-13: LPP Provide Location Information (UL NAS TRANSPORT, Table 9.4.5.3.3-11)

| Derivation Path: 37.355 clause 6.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| LPP-Message ::= SEQUENCE { |  |  |  |  |
| transactionID SEQUENCE { |  |  |  |  |
| initiator |  | locationServer |  |  |
| transactionNumber |  | (0..255) | Contains the same value as the corresponding field in LPP Request Location Information message in step 8, Table 9.4.5.3.2-3 |  |
| } |  |  |  |  |
| endTransaction |  | TRUE |  |  |
| sequenceNumber |  | (0..255) |  |  |
| acknowledgement SEQUENCE { |  | present, or not present |  |  |
| ackRequested |  | TRUE |  |  |
| ackIndicator |  | Not present |  |  |
| } |  |  |  |  |
| lpp-MessageBody CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| provideLocationInformation SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| provideLocationInformation-r9 SEQUENCE { |  |  |  |  |
| commonIEsProvideLocationInformation |  | May be present. Any value acceptable |  |  |
| nr-Multi-RTT-ProvideLocationInformation-r16 SEQUENCE { |  | Present for sub-test 26 | Rel-17 onwards |  |
| nr-Multi-RTT-SignalMeasurementInformation-r16 |  | Present. Any value acceptable |  |  |
| nr-Multi-RTT-Error-r16 |  | Not present |  |  |
| nr-Multi-RTT-SignalMeasurementInstances-r17 |  | Not present |  |  |
| } |  |  |  |  |
| nr-DL-AoD-ProvideLocationInformation-r16 SEQUENCE { |  | Present for sub-test 27 | Rel-17 onwards |  |
| nr-DL-AoD-SignalMeasurementInformation-r16 |  | Present. Any value acceptable |  |  |
| nr-dl-AoD-LocationInformation-r16 |  | Not checked |  |  |
| nr-DL-AoD-Error-r16 |  | Not present |  |  |
| nr-DL-AoD-SignalMeasurementInstances-r17 |  | Not present |  |  |
| nr-DL-AoD-LocationInformationInstances-r17 |  | Not present |  |  |
| } |  |  |  |  |
| nr-DL-TDOA-ProvideLocationInformation-r16 SEQUENCE { |  | Present for sub-test 28 | Rel-17 onwards |  |
| nr-DL-TDOA-SignalMeasurementInformation-r16 |  | Present. Any value acceptable |  |  |
| nr-dl-tdoa-LocationInformation-r16 |  | Not checked |  |  |
| nr-DL-TDOA-Error-r16 |  | Not present |  |  |
| nr-DL-TDOA-SignalMeasurementInstances-r17 |  | Not present |  |  |
| nr-DL-TDOA-LocationInformationInstances-r17 |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |