3GPP TSG-RAN5 Meeting #99	R5-232314

# Incheon, Korea (Republic Of), 22nd May 2023 – 26th May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 5446 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to MOP and MPR test cases as part of introduction of LTE Band 54 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ligado Networks |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_TDD_1670_1675MHz-UEConTest |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Introduction of LTE TDD Band 54 to conformance specifications |  |  |  |  |  |  |  |  |
| Summary of change: |  | Relevant MOP and MPR clauses have been updated to introduce Band 54 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | LTE TDD Band 54 will be unspecified for UE conformance testing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2.3, 6.2.2.5, 6.2.2B.3, 6.2.2B.5, 6.2.2_s.3, 6.2.3.5, 6.2.3_2.5, 6.2.3_3.5, 6.2.3_4.5, 6.2.3_5.5, 6.2.3_6.5, 6.2.3B.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 6.2.2.3	Minimum conformance requirements

The following UE Power Classes defines the maximum output power for any transmission bandwidth within the channel bandwidth. The period of measurement shall be at least one sub frame (1ms).

Table 6.2.2.3-1: UE Power Class

| EUTRA band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  | 23 | ±2 |  |  |
| 2 |  |  |  |  | 23 | ±22 |  |  |
| 3 |  |  |  |  | 23 | ±22 |  |  |
| 4 |  |  |  |  | 23 | ±2 |  |  |
| 5 |  |  |  |  | 23 | ±2 |  |  |
| 6 |  |  |  |  | 23 | ±2 |  |  |
| 7 |  |  |  |  | 23 | ±22 |  |  |
| 8 |  |  |  |  | 23 | ±22 |  |  |
| 9 |  |  |  |  | 23 | ±2 |  |  |
| 10 |  |  |  |  | 23 | ±2 |  |  |
| 11 |  |  |  |  | 23 | ±2 |  |  |
| 12 |  |  |  |  | 23 | ±22 |  |  |
| 13 |  |  |  |  | 23 | ±2 |  |  |
| 14 |  |  |  |  | 23 | ±2 |  |  |
| … |  |  |  |  |  |  |  |  |
| 17 |  |  |  |  | 23 | ±2 |  |  |
| 18 |  |  |  |  | 23 | ±26 |  |  |
| 19 |  |  |  |  | 23 | ±2 |  |  |
| 20 |  |  |  |  | 23 | ±22 |  |  |
| 21 |  |  |  |  | 23 | ±2 |  |  |
| 22 |  |  |  |  | 23 | +2/-3.52 |  |  |
| 23 |  |  |  |  | 235 | ±25 |  |  |
| 24 |  |  |  |  | 23 | +2/-32 |  |  |
| 25 |  |  |  |  | 23 | ±22 |  |  |
| 26 |  |  |  |  | 23 | ±22 |  |  |
| 27 |  |  |  |  | 23 | ±2 |  |  |
| 28 |  |  |  |  | 23 | +2/-2.5 |  |  |
| 30 |  |  |  |  | 23 | ±2 |  |  |
| 31 |  |  |  |  | 23 | ±2 |  |  |
| ... |  |  |  |  |  |  |  |  |
| 33 |  |  |  |  | 23 | ±2 |  |  |
| 34 |  |  |  |  | 23 | ±2 |  |  |
| 35 |  |  |  |  | 23 | ±2 |  |  |
| 36 |  |  |  |  | 23 | ±2 |  |  |
| 37 |  |  |  |  | 23 | ±2 |  |  |
| 38 |  |  |  |  | 23 | ±2 |  |  |
| 39 |  |  |  |  | 23 | ±2 |  |  |
| 40 |  |  |  |  | 23 | ±2 |  |  |
| 41 |  |  |  |  | 23 | ±22 |  |  |
| 42 |  |  |  |  | 23 | +2/-3 |  |  |
| 43 |  |  |  |  | 23 | +2/-3 |  |  |
| 44 |  |  |  |  | 23 | +2/[-3] |  |  |
| 45 |  |  |  |  | 23 | ±2 |  |  |
| 47 |  |  | 26 | ±2 | 23 | ±2 |  |  |
| 48 |  |  |  |  | 23 | +2/-3 |  |  |
| … |  |  |  |  |  |  |  |  |
| 53 |  |  |  |  | 23 | ±2 |  |  |
| 54 |  |  |  |  | 23 | ±2 |  |  |
| … |  |  |  |  |  |  |  |  |
| 65 |  |  |  |  | 23 | ±2 |  |  |
| 66 |  |  |  |  | 23 | ±2 |  |  |
| … |  |  |  |  |  |  |  |  |
| 68 |  |  |  |  | 23 | ±2 |  |  |
| … |  |  |  |  |  |  |  |  |
| 70 |  |  |  |  | 23 | ±2 |  |  |
| 71 |  |  |  |  | 23 | +2/-2.5 |  |  |
| 72 |  |  |  |  | 23 | ±2 |  |  |
| 73 |  |  |  |  | 23 | ±2 |  |  |
| 74 |  |  |  |  | 23 | ±2 |  |  |
| 85 |  |  |  |  | 23 | ±22 |  |  |
| 87 |  |  |  |  | 23 | ±2 |  |  |
| 88 |  |  |  |  | 23 | ±2 |  |  |
| Note 1: Void Note 2: For transmission bandwidths (Figure 5.4.2-1) confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB Note 3: PPowerClass is the maximum UE power specified without taking into account the tolerance Note 4: For the UE which supports both Band 11 and Band 21 operating frequencies, the tolerance is FFS. Note 5: When NS_20 is signalled, the total output power within 2000-2005 MHz shall be limited to 7 dBm. Note 6: For a UE that supports both Band 18 and Band 26, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5dB for transmission bandwidths confined within 815 MHz and 818 MHz |  |  |  |  |  |  |  |  |

The default power class PPowerClass_Default for an operating band is Power Class 3 unless otherwise stated.

For a power class 2 capable UE operating on Band 41, when an IE P-max as defined in TS 36.331 [7] of 23 dBm or lower is indicated in the cell or if the uplink/downlink configuration is 0 or 6, the requirements for power class 2 are not applicable, and the corresponding requirements for a power class 3 UE shall apply.

For each supported frequency band other than Band 14 and Band 41, the UE shall:

-	if the UE supports a different power class than the default UE power class for the band and the supported power class enables the higher maximum output power than that of the default power class:

-	if the band is a TDD band whose frame configuration is 0 or 6; or

-	if the IE P-Max as defined in TS 36.331 [7] is not provided; or

-	if the IE P-Max as defined in TS 36.331 [7] is provided and set to the maximum output power of the default power class or lower;

-	meet all requirements for the default power class of the operating band in which the UE is operating and set its configured transmitted power as specified in sub-clause 6.2.5;

-	else (i.e the IE P-Max as defined in TS 36.331 [7] is provided and set to the higher value than the maximum output power of the default power class):

-	meet all requirements for the supported power class and set its configured transmitted power class as specified in sub-clause 6.2.5;

The normative reference for this requirement is TS 36.101 clause 6.2.2.

------------------------------------------------------------- NEXT CHANGE---- ------------------------------------------------------

#### 6.2.2.5	Test requirements

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2.2.5-1.

Table 6.2.2.5-1: UE Power Class test requirements

| EUTRA band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  | 23 | ±2.7 |  |  |
| 2 |  |  |  |  | 23 | ±2.72 |  |  |
| 3 |  |  |  |  | 23 | ±2.72 |  |  |
| 4 |  |  |  |  | 23 | ±2.7 |  |  |
| 5 |  |  |  |  | 23 | ±2.7 |  |  |
| 6 |  |  |  |  | 23 | ±2.7 |  |  |
| 7 |  |  |  |  | 23 | ±2.72 |  |  |
| 8 |  |  |  |  | 23 | ±2.72 |  |  |
| 9 |  |  |  |  | 23 | ±2.7 |  |  |
| 10 |  |  |  |  | 23 | ±2.7 |  |  |
| 11 |  |  |  |  | 23 | ±2.7 |  |  |
| 12 |  |  |  |  | 23 | ±2.72 |  |  |
| 13 |  |  |  |  | 23 | ±2.7 |  |  |
| 14 |  |  |  |  | 23 | ±2.7 |  |  |
| … |  |  |  |  |  |  |  |  |
| 17 |  |  |  |  | 23 | ±2.7 |  |  |
| 18 |  |  |  |  | 23 | ±2.76 |  |  |
| 19 |  |  |  |  | 23 | ±2.7 |  |  |
| 20 |  |  |  |  | 23 | ±2.72 |  |  |
| 21 |  |  |  |  | 23 | ±2.7 |  |  |
| 22 |  |  |  |  | 23 | +3.0/-4.52 |  |  |
| 23 |  |  |  |  | 235 | ±2.75 |  |  |
| 24 |  |  |  |  | 23 | +2.7/-3.72 |  |  |
| 25 |  |  |  |  | 23 | ±2.72 |  |  |
| 26 |  |  |  |  | 23 | ±2.72 |  |  |
| 27 |  |  |  |  | 23 | ±2.7 |  |  |
| 28 |  |  |  |  | 23 | +2.7/-3.2 |  |  |
| 30 |  |  |  |  | 23 | ±2.7 |  |  |
| 31 |  |  |  |  | 23 | ±2.7 |  |  |
| ... |  |  |  |  |  |  |  |  |
| 33 |  |  |  |  | 23 | ±2.7 |  |  |
| 34 |  |  |  |  | 23 | ±2.7 |  |  |
| 35 |  |  |  |  | 23 | ±2.7 |  |  |
| 36 |  |  |  |  | 23 | ±2.7 |  |  |
| 37 |  |  |  |  | 23 | ±2.7 |  |  |
| 38 |  |  |  |  | 23 | ±2.7 |  |  |
| 39 |  |  |  |  | 23 | ±2.7 |  |  |
| 40 |  |  |  |  | 23 | ±2.7 |  |  |
| 41 |  |  |  |  | 23 | ±2.72 |  |  |
| 42 |  |  |  |  | 23 | +3.0/-4.0 |  |  |
| 43 |  |  |  |  | 23 | +3.0/-4.0 |  |  |
| 44 |  |  |  |  | 23 | +2.7/[-3.7] |  |  |
| 45 |  |  |  |  | 23 | ±2.7 |  |  |
| 47 |  |  | 26 | ±3.3 | 23 | ±3.3 |  |  |
| 48 |  |  |  |  | 23 | +3.0/-4.0 |  |  |
| … |  |  |  |  |  |  |  |  |
| 53 |  |  |  |  | 23 | +2.7/-2.7 |  |  |
| 54 |  |  |  |  | 23 | +2.7/-2.7 |  |  |
| … |  |  |  |  |  |  |  |  |
| 65 |  |  |  |  | 23 | +2.7/-2.7 |  |  |
| 66 |  |  |  |  | 23 | +2.7/-2.7 |  |  |
| … |  |  |  |  |  |  |  |  |
| 68 |  |  |  |  | 23 | +2.7/-2.7 |  |  |
| … |  |  |  |  |  |  |  |  |
| 70 |  |  |  |  | 23 | +2.7/-2.7 |  |  |
| 71 |  |  |  |  | 23 | +2.7/-3.2 |  |  |
| 72 |  |  |  |  | 23 | ±2.7 |  |  |
| 73 |  |  |  |  | 23 | ±2.7 |  |  |
| 74 |  |  |  |  | 23 | ±2.7 |  |  |
| 85 |  |  |  |  | 23 | ±2.72 |  |  |
| 87 |  |  |  |  | 23 | ±2.7 |  |  |
| 88 |  |  |  |  | 23 | ±2.7 |  |  |
| Note 1: Void Note 2: For transmission bandwidths (Figure 5.4.2-1, Table 5.4.4-1) confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB Note 3: PPowerClass is the maximum UE power specified without taking into account the tolerance. Note 4: For the UE which supports both Band 11 and Band 21 operating frequencies, the tolerance is FFS. Note 5: When NS_20 is signalled, the total output power within 2000-2005 MHz shall be limited to 7 dBm. Note 6: For a UE that supports both Band 18 and Band 26, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5dB for transmission bandwidths confined within 815 MHz and 818 MHz. |  |  |  |  |  |  |  |  |

For the UE which supports inter-band carrier aggregation configurations with uplink assigned to one E-UTRA band the ΔTIB,c in Tables 6.2.5.3-2, 6.2.5.3-3 and 6.2.5.3-4 shall be applied for applicable bands.

In case the UE supports more than one of the above inter-band carrier aggregation configurations and a E-UTRA operating band belongs to more than one inter-band carrier aggregation configurations then:

-	When the E-UTRA operating band frequency range is ≤ 1GHz, the applicable additional tolerance shall be the average of the tolerances in Table 6.2.5.3-2, truncated to one decimal place for that operating band among the supported CA configurations. In case there is a harmonic relation between low band UL and high band DL, then the maximum tolerance among the different supported carrier aggregation configurations involving such band shall be applied

-	When the E-UTRA operating band frequency range is >1GHz, the applicable additional tolerance shall be the maximum tolerance in Table 6.2.5.3-2 that applies for that operating band among the supported CA configurations

------------------------------------------------------------- NEXT CHANGE---- ------------------------------------------------------

#### 6.2.2B.3	Minimum conformance requirements

For UE with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the maximum output power for any transmission bandwidth within the channel bandwidth is specified in Table 6.2.2B.3-1 with the UL-MIMO configurations specified in Table 6.2.2B.3-2. The maximum output power is measured as the sum of the maximum output power at each UE antenna connector. The period of measurement shall be at least one sub frame (1ms).

Table 6.2.2B.3-1: UE Power Class for UL-MIMO in closed loop spatial multiplexing scheme

| EUTRA band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  | 23 | +2/-3 |  |  |
| 2 |  |  |  |  | 23 | +2/-32 |  |  |
| 3 |  |  |  |  | 23 | +2/-32 |  |  |
| 4 |  |  |  |  | 23 | +2/-3 |  |  |
| 5 |  |  |  |  | 23 | +2/-3 |  |  |
| 6 |  |  |  |  | 23 | +2/-3 |  |  |
| 7 |  |  |  |  | 23 | +2/-32 |  |  |
| 8 |  |  |  |  | 23 | +2/-32 |  |  |
| 9 |  |  |  |  | 23 | +2/-3 |  |  |
| 10 |  |  |  |  | 23 | +2/-3 |  |  |
| 11 |  |  |  |  | 23 | +2/-3 |  |  |
| 12 |  |  |  |  | 23 | +2/-32 |  |  |
| 13 |  |  |  |  | 23 | +2/-3 |  |  |
| 14 |  |  |  |  | 23 | +2/-3 |  |  |
| … |  |  |  |  |  |  |  |  |
| 17 |  |  |  |  | 23 | +2/-3 |  |  |
| 18 |  |  |  |  | 23 | +2/-3 |  |  |
| 19 |  |  |  |  | 23 | +2/-3 |  |  |
| 20 |  |  |  |  | 23 | +2/-32 |  |  |
| 21 |  |  |  |  | 23 | +2/-3 |  |  |
| 22 |  |  |  |  | 23 | +2/-4.52 |  |  |
| 23 |  |  |  |  | 23 | +2/-3 |  |  |
| 24 |  |  |  |  | 23 | +2/-42 |  |  |
| 25 |  |  |  |  | 23 | +2/-32 |  |  |
| 26 |  |  |  |  | 23 | +2/-32 |  |  |
| 27 |  |  |  |  | 23 | +2/-3 |  |  |
| 28 |  |  |  |  | 23 | +2/[-3] |  |  |
| 30 |  |  |  |  | 23 | +2/-3 |  |  |
| 31 |  |  |  |  | 23 | +2/-3 |  |  |
| … |  |  |  |  |  |  |  |  |
| 33 |  |  |  |  | 23 | +2/-3 |  |  |
| 34 |  |  |  |  | 23 | +2/-3 |  |  |
| 35 |  |  |  |  | 23 | +2/-3 |  |  |
| 36 |  |  |  |  | 23 | +2/-3 |  |  |
| 37 |  |  |  |  | 23 | +2/-3 |  |  |
| 38 |  |  |  |  | 23 | +2/-3 |  |  |
| 39 |  |  |  |  | 23 | +2/-3 |  |  |
| 40 |  |  |  |  | 23 | +2/-3 |  |  |
| 41 |  |  |  |  | 23 | +2/-32 |  |  |
| 42 |  |  |  |  | 23 | +2/-4 |  |  |
| 43 |  |  |  |  | 23 | +2/-4 |  |  |
| 44 |  |  |  |  | 23 | +2/[-3] |  |  |
| 45 |  |  |  |  | 23 | +2/-3 |  |  |
| … |  |  |  |  |  |  |  |  |
| 48 |  |  |  |  | 23 | +2/-3 |  |  |
| … |  |  |  |  |  |  |  |  |
| 53 |  |  |  |  | 23 | +2/-3 |  |  |
| 54 |  |  |  |  | 23 | +2/-3 |  |  |
| … |  |  |  |  |  |  |  |  |
| 65 |  |  |  |  | 23 | +2/-3 |  |  |
| 66 |  |  |  |  | 23 | +2/-3 |  |  |
| … |  |  |  |  |  |  |  |  |
| 68 |  |  |  |  | 23 | +2/-3 |  |  |
| … |  |  |  |  |  |  |  |  |
| 70 |  |  |  |  | 23 | +2/-3 |  |  |
| 72 |  |  |  |  | 23 | +2/-3 |  |  |
| 73 |  |  |  |  | 23 | +2/-3 |  |  |
| 74 |  |  |  |  | 23 | +2/-3 |  |  |
| Note 1: Void Note 2: For transmission bandwidths (Figure 5.4.2-1) confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB Note 3: For the UE which supports both Band 11 and Band 21 operating frequencies, the tolerance is FFS. Note 4: PPowerClass is the maximum UE power specified without taking into account the tolerance |  |  |  |  |  |  |  |  |

Table 6.2.2B.3-2: UL-MIMO configuration in closed-loop spatial multiplexing scheme

| Transmission mode | DCI format | Codebook Index |
| --- | --- | --- |
| Mode 2 | DCI format 4 | Codebook index 0 |

The normative reference for this requirement is TS 36.101 clause 6.2.2B.

------------------------------------------------------------- NEXT CHANGE---- ------------------------------------------------------

#### 6.2.2B.5	Test requirements

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2.2B.5-1.

Table 6.2.2B.5-1: UE Power Class for UL-MIMO in closed loop spatial multiplexing scheme

| EUTRA band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |  | Class 4 (dBm) |  | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  | 23 | +2.7/-3.7 |  |  |  |  |
| 2 |  |  |  |  | 23 | +2.7/-3.72 |  |  |  |  |
| 3 |  |  |  |  | 23 | +2.7/-3.72 |  |  |  |  |
| 4 |  |  |  |  | 23 | +2.7/-3.7 |  |  |  |  |
| 5 |  |  |  |  | 23 | +2.7/-3.7 |  |  |  |  |
| 6 |  |  |  |  | 23 | +2.7/-3.7 |  |  |  |  |
| 7 |  |  |  |  | 23 | +2.7/-3.72 |  |  |  |  |
| 8 |  |  |  |  | 23 | +2.7/-3.72 |  |  |  |  |
| 9 |  |  |  |  | 23 | +2.7/-3.7 |  |  |  |  |
| 10 |  |  |  |  | 23 | +2.7/-3.7 |  |  |  |  |
| 11 |  |  |  |  | 23 | +2.7/-3.7 |  |  |  |  |
| 12 |  |  |  |  | 23 | +2.7/-3.72 |  |  |  |  |
| 13 |  |  |  |  | 23 | +2.7/-3.7 |  |  |  |  |
| 14 |  |  |  |  | 23 | +2.7/-3.7 |  |  |  |  |
| … |  |  |  |  |  |  |  |  |  |  |
| 17 |  |  |  |  | 23 | +2.7/-3.7 |  |  |  |  |
| 18 |  |  |  |  | 23 | +2.7/-3.7 |  |  |  |  |
| 19 |  |  |  |  | 23 | +2.7/-3.7 |  |  |  |  |
| 20 |  |  |  |  | 23 | +2.7/-3.72 |  |  |  |  |
| 21 |  |  |  |  | 23 | +2.7/-3.7 |  |  |  |  |
| 22 |  |  |  |  | 23 | +3/-5.52 |  |  |  |  |
| 23 |  |  |  |  | 23 | +2.7/-3.7 |  |  |  |  |
| 24 |  |  |  |  | 23 | +2.7/-4.72 |  |  |  |  |
| 25 |  |  |  |  | 23 | +2.7/-3.72 |  |  |  |  |
| 26 |  |  |  |  | 23 | +2.7/-3.7 |  |  |  |  |
| 27 |  |  |  |  | 23 | +2.7/-3.7 |  |  |  |  |
| 28 |  |  |  |  | 23 | +2.7/[-3.7] |  |  |  |  |
| 30 |  |  |  |  | 23 | +2.7/-3.7 |  |  |  |  |
| 31 |  |  |  |  | 23 | +2.7/-3.7 |  |  |  |  |
| … |  |  |  |  |  |  |  |  |  |  |
| 33 |  |  |  |  | 23 | +2.7/-3.7 |  |  |  |  |
| 34 |  |  |  |  | 23 | +2.7/-3.7 |  |  |  |  |
| 35 |  |  |  |  | 23 | +2.7/-3.7 |  |  |  |  |
| 36 |  |  |  |  | 23 | +2.7/-3.7 |  |  |  |  |
| 37 |  |  |  |  | 23 | +2.7/-3.7 |  |  |  |  |
| 38 |  |  |  |  | 23 | +2.7/-3.7 |  |  |  |  |
| 39 |  |  |  |  | 23 | +2.7/-3.7 |  |  |  |  |
| 40 |  |  |  |  | 23 | +2.7/-3.7 |  |  |  |  |
| 41 |  |  |  |  | 23 | +2.7/-3.72 |  |  |  |  |
| 42 |  |  |  |  | 23 | +3/-5 |  |  |  |  |
| 43 |  |  |  |  | 23 | +3/-5 |  |  |  |  |
| 44 |  |  |  |  | 23 | +2.7/[-3.7] |  |  |  |  |
| 45 |  |  |  |  | 23 | +2.7/-3.7 |  |  |  |  |
| … |  |  |  |  |  |  |  |  |  |  |
| 53 |  |  |  |  | 23 | +2.7/-3.7 |  |  |  |  |
| 54 |  |  |  |  | 23 | +2.7/-3.7 |  |  |  |  |
| .. |  |  |  |  |  |  |  |  |  |  |
| 65 |  |  |  |  | 23 | +2.7/-3.7 |  |  |  |  |
| 66 |  |  |  |  | 23 | +2/-3 |  |  |  |  |
| … |  |  |  |  |  |  |  |  |  |  |
| 68 |  |  |  |  | 23 | +2.7/-3.7 |  |  |  |  |
| … |  |  |  |  |  |  |  |  |  |  |
| 70 |  |  |  |  | 23 | +2.7/-3.7 |  |  |  |  |
| 72 |  |  |  |  | 23 | +2.7/-3.7 |  |  |  |  |
| 73 |  |  |  |  | 23 | +2.7/-3.7 |  |  |  |  |
| 74 |  |  |  |  | 23 | +2.7/-3.7 |  |  |  |  |
| Note 1: Void Note 2: For transmission bandwidths (Figure 5.4.2-1) confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB Note 3: For the UE which supports both Band 11 and Band 21 operating frequencies, the tolerance is FFS. Note 4: PPowerClass is the maximum UE power specified without taking into account the tolerance |  |  |  |  |  |  |  |  |  |  |

For the UE which supports inter-band carrier aggregation configurations with uplink assigned to one E-UTRA band the ΔTIB,c in Tables 6.2.5.3-2, 6.2.5.3-3 and 6.2.5.3-4 shall be applied for applicable bands.

In case the UE supports more than one of the above inter-band carrier aggregation configurations and an E-UTRA operating band belongs to more than one inter-band carrier aggregation configurations then:

-	When the E-UTRA operating band frequency range is ≤ 1GHz, the applicable additional tolerance shall be the average of the tolerances in Table 6.2.5.3-2, truncated to one decimal place for that operating band among the supported CA configurations. In case there is a harmonic relation between low band UL and high band DL, then the maximum tolerance among the different supported carrier aggregation configurations involving such band shall be applied

-	When the E-UTRA operating band frequency range is >1GHz, the applicable additional tolerance shall be the maximum tolerance in Table 6.2.5.3-2 that applies for that operating band among the supported CA configurations

------------------------------------------------------------- NEXT CHANGE---- ------------------------------------------------------

6.2.2_s.3	Minimum conformance requirements

The following UE Power Classes defines the maximum output power for any transmission bandwidth within the channel bandwidth. The period of measurement shall be at least 7 OS for slot TTI and 2/3 OS for subslot TTI.

Table 6.2.2_s.3-1: UE Power Class

| EUTRA band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  | 23 | ±2 |  |  |
| 2 |  |  |  |  | 23 | ±22 |  |  |
| 3 |  |  |  |  | 23 | ±22 |  |  |
| 4 |  |  |  |  | 23 | ±2 |  |  |
| 5 |  |  |  |  | 23 | ±2 |  |  |
| 6 |  |  |  |  | 23 | ±2 |  |  |
| 7 |  |  |  |  | 23 | ±22 |  |  |
| 8 |  |  |  |  | 23 | ±22 |  |  |
| 9 |  |  |  |  | 23 | ±2 |  |  |
| 10 |  |  |  |  | 23 | ±2 |  |  |
| 11 |  |  |  |  | 23 | ±2 |  |  |
| 12 |  |  |  |  | 23 | ±22 |  |  |
| 13 |  |  |  |  | 23 | ±2 |  |  |
| 14 |  |  |  |  | 23 | ±2 |  |  |
| … |  |  |  |  |  |  |  |  |
| 17 |  |  |  |  | 23 | ±2 |  |  |
| 18 |  |  |  |  | 23 | ±26 |  |  |
| 19 |  |  |  |  | 23 | ±2 |  |  |
| 20 |  |  |  |  | 23 | ±22 |  |  |
| 21 |  |  |  |  | 23 | ±2 |  |  |
| 22 |  |  |  |  | 23 | +2/-3.52 |  |  |
| 23 |  |  |  |  | 235 | ±25 |  |  |
| 24 |  |  |  |  | 23 | +2/-32 |  |  |
| 25 |  |  |  |  | 23 | ±22 |  |  |
| 26 |  |  |  |  | 23 | ±22 |  |  |
| 27 |  |  |  |  | 23 | ±2 |  |  |
| 28 |  |  |  |  | 23 | +2/-2.5 |  |  |
| 30 |  |  |  |  | 23 | ±2 |  |  |
| 31 |  |  |  |  | 23 | ±2 |  |  |
| ... |  |  |  |  |  |  |  |  |
| 33 |  |  |  |  | 23 | ±2 |  |  |
| 34 |  |  |  |  | 23 | ±2 |  |  |
| 35 |  |  |  |  | 23 | ±2 |  |  |
| 36 |  |  |  |  | 23 | ±2 |  |  |
| 37 |  |  |  |  | 23 | ±2 |  |  |
| 38 |  |  |  |  | 23 | ±2 |  |  |
| 39 |  |  |  |  | 23 | ±2 |  |  |
| 40 |  |  |  |  | 23 | ±2 |  |  |
| 41 |  |  |  |  | 23 | ±22 |  |  |
| 42 |  |  |  |  | 23 | +2/-3 |  |  |
| 43 |  |  |  |  | 23 | +2/-3 |  |  |
| 44 |  |  |  |  | 23 | +2/[-3] |  |  |
| 45 |  |  |  |  | 23 | ±2 |  |  |
| 47 |  |  | 26 | ±2 | 23 | ±2 |  |  |
| 48 |  |  |  |  | 23 | +2/-3 |  |  |
| … |  |  |  |  |  |  |  |  |
| 53 |  |  |  |  | 23 | ±2 |  |  |
| 54 |  |  |  |  | 23 | ±2 |  |  |
| … |  |  |  |  |  |  |  |  |
| 65 |  |  |  |  | 23 | ±2 |  |  |
| 66 |  |  |  |  | 23 | ±2 |  |  |
| … |  |  |  |  |  |  |  |  |
| 68 |  |  |  |  | 23 | ±2 |  |  |
| … |  |  |  |  |  |  |  |  |
| 70 |  |  |  |  | 23 | ±2 |  |  |
| 71 |  |  |  |  | 23 | +2/-2.5 |  |  |
| 72 |  |  |  |  | 23 | ±2 |  |  |
| 73 |  |  |  |  | 23 | ±2 |  |  |
| 74 |  |  |  |  | 23 | ±2 |  |  |
| 85 |  |  |  |  | 23 | ±22 |  |  |
| Note 1: Void Note 2: For transmission bandwidths (Figure 5.4.2-1) confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB Note 3: PPowerClass is the maximum UE power specified without taking into account the tolerance Note 4: For the UE which supports both Band 11 and Band 21 operating frequencies, the tolerance is FFS. Note 5: When NS_20 is signalled, the total output power within 2000-2005 MHz shall be limited to 7 dBm. Note 6: For a UE that supports both Band 18 and Band 26, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5dB for transmission bandwidths confined within 815 MHz and 818 MHz |  |  |  |  |  |  |  |  |

The normative reference for this requirement is TS 36.101 clause 6.2.2.

------------------------------------------------------------- NEXT CHANGE---- ------------------------------------------------------

#### 6.2.3.5	Test requirements

The maximum output power, derived in step 2 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2.3.5-1.

Table 6.2.3.5-1: UE Power Class test requirements

| E-UTRA Band | Class 1 (dBm) | Tol. (Db) | Class 2 (dBm) | Tol. (dB) | Class 3 (dBm) | QPSK partial RB allocation Tol. (dB)) | QPSK full RB allocation Tol. (dB)) | 16QAM partial RB allocation Tol. (dB) | 16QAM full RB allocation Tol. (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  | 23 | ±2.7 | +2.7 / -3.7 | +2.7 / -3.7 | +2.7 / -4.7 |
| 2 |  |  |  |  | 23 | ±2.71 | +2.7 / 1,2 -3.7 | +2.7 / 1,2 -3.7 | +2.7 / 1,2 -4.7 |
| 3 |  |  |  |  | 23 | ±2.71 | +2.7 / 1,2 -3.7 | +2.7 / 1,2 -3.7 | +2.7 / 1,2 -4.7 |
| 4 |  |  |  |  | 23 | ±2.7 | +2.7 / -3.7 | +2.7 / -3.7 | +2.7 / -4.7 |
| 5 |  |  |  |  | 23 | ±2.7 | +2.7 / -3.7 | +2.7 / -3.7 | +2.7 / -4.7 |
| 6 |  |  |  |  | 23 | ±2.7 | +2.7 / -3.7 | +2.7 / -3.7 | +2.7 / -4.7 |
| 7 |  |  |  |  | 23 | ±2.71 | +2.7 / 1,2 -3.7 | +2.7 / 1,2 -3.7 | +2.7 / 1,2 -4.7 |
| 8 |  |  |  |  | 23 | ±2.71 | +2.7 / 1,2 -3.7 | +2.7 / 1,2 -3.7 | +2.7 / 1,2 -4.7 |
| 9 |  |  |  |  | 23 | ±2.7 | +2.7 / -3.7 | +2.7 / -3.7 | +2.7 / -4.7 |
| 10 |  |  |  |  | 23 | ±2.7 | +2.7 / -3.7 | +2.7 / -3.7 | +2.7 / -4.7 |
| 11 |  |  |  |  | 23 | ±2.7 | +2.7 / -3.7 | +2.7 / -3.7 | +2.7 / -4.7 |
| 12 |  |  |  |  | 23 | ±2.71 | +2.7 / 1,2 -3.7 | +2.7 / 1,2 -3.7 | +2.7 / 1,2 -4.7 |
| 13 |  |  |  |  | 23 | ±2.7 | +2.7 / -3.7 | +2.7 / -3.7 | +2.7 / -4.7 |
| 14 |  |  |  |  | 23 | ±2.7 | +2.7 / -3.7 | +2.7 / -3.7 | +2.7 / -4.7 |
| … |  |  |  |  |  |  |  |  |  |
| 17 |  |  |  |  | 23 | ±2.7 | +2.7 / -3.7 | +2.7 / -3.7 | +2.7 / -4.7 |
| 18 |  |  |  |  | 23 | ±2.73 | +2.7 / 3 -3.7 | +2.7 / 3 -3.7 | +2.7 / 3 -4.7 |
| 19 |  |  |  |  | 23 | ±2.7 | +2.7 / -3.7 | +2.7 / -3.7 | +2.7 / -4.7 |
| 20 |  |  |  |  | 23 | ±2.71 | +2.7 / 1,2 -3.7 | +2.7 / 1,2 -3.7 | +2.7 / 1,2 -4.7 |
| 21 |  |  |  |  | 23 | ±2.7 | +2.7 / -3.7 | +2.7 / -3.7 | +2.7 / -4.7 |
| 22 |  |  |  |  | 23 | +3.0/-4.5 | +3.0/-5.5 | +3.0/-5.5 | +3.0/-6.5 |
| 23 |  |  |  |  | 23 | ±2.7 | +2.7 / -3.7 | +2.7 / -3.7 | +2.7 / -4.7 |
| 24 |  |  |  |  | 23 | ±2.7 | +2.7 / 1,2 -4.7 | +2.7 / 1,2 -4.7 | +2.7 / 1,2 -5.7 |
| 25 |  |  |  |  | 23 | ±2.71 | +2.7 / 1,2 -3.7 | +2.7 / 1,2 -3.7 | +2.7 / 1,2 -4.7 |
| 26 |  |  |  |  | 23 | ±2.71 | +2.7 / 1,2 -3.7 | +2.7 / 1,2 -3.7 | +2.7 / 1,2 -4.7 |
| 27 |  |  |  |  | 23 | ±2.7 | +2.7 / -3.7 | +2.7 / -3.7 | +2.7 / -4.7 |
| 28 |  |  |  |  | 23 | +2.7/-3.2 | +2.7/-4.2 | +2.7/-4.2 | +2.7/-5.2 |
| 30 |  |  |  |  | 23 | ±2.7 | +2.7 / -3.7 | +2.7 / -3.7 | +2.7 / -4.7 |
| 31 |  |  |  |  | 23 | ±2.7 | +2.7 / -3.7 | +2.7 / -3.7 | +2.7 / -4.7 |
| ... |  |  |  |  |  |  |  |  |  |
| 33 |  |  |  |  | 23 | ±2.7 | +2.7 / -3.7 | +2.7 / -3.7 | +2.7 / -4.7 |
| 34 |  |  |  |  | 23 | ±2.7 | +2.7 / -3.7 | +2.7 / -3.7 | +2.7 / -4.7 |
| 35 |  |  |  |  | 23 | ±2.7 | +2.7 / -3.7 | +2.7 / -3.7 | +2.7 / -4.7 |
| 36 |  |  |  |  | 23 | ±2.7 | +2.7 / -3.7 | +2.7 / -3.7 | +2.7 / -4.7 |
| 37 |  |  |  |  | 23 | ±2.7 | +2.7 / -3.7 | +2.7 / -3.7 | +2.7 / -4.7 |
| 38 |  |  |  |  | 23 | ±2.7 | +2.7 / -3.7 | +2.7 / -3.7 | +2.7 / -4.7 |
| 39 |  |  |  |  | 23 | ±2.7 | +2.7 / -3.7 | +2.7 / -3.7 | +2.7 / -4.7 |
| 40 |  |  |  |  | 23 | ±2.7 | +2.7 / -3.7 | +2.7 / -3.7 | +2.7 / -4.7 |
| 41 |  |  |  |  | 23 | ±2.71 | +2.7 / 1,2 -3.7 | +2.7 / 1,2 -3.7 | +2.7 / 1,2 -4.7 |
| 42 |  |  |  |  | 23 | +3.0 / -4.0 | +3.0 / -5.0 | +3.0 / -5.0 | +3.0 / -6.0 |
| 43 |  |  |  |  | 23 | +3.0 / -4.0 | +3.0 / -5.0 | +3.0 / -5.0 | +3.0 / -6.0 |
| 44 |  |  |  |  | 23 | +2.7/ [-3.7] | +2.7 / [-4.7] | +2.7 / [-4.7] | +2.7 / [-5.7] |
| 45 |  |  |  |  | 23 | ±2.7 | +2.7 / -3.7 | +2.7 / -3.7 | +2.7 / -4.7 |
| … |  |  |  |  |  |  |  |  |  |
| 54 |  |  |  |  | 23 | ±2.7 | +2.7 / -3.7 | +2.7 / -3.7 | +2.7 / -4.7 |
| … |  |  |  |  |  |  |  |  |  |
| 65 |  |  |  |  | 23 | ±2.7 | +2.7 / -3.7 | +2.7 / -3.7 | +2.7 / -4.7 |
| 66 |  |  |  |  | 23 | ±2.7 | +2.7 / -3.7 | +2.7 / -3.7 | +2.7 / -4.7 |
| … |  |  |  |  |  |  |  |  |  |
| 68 |  |  |  |  | 23 | ±2.7 | +2.7 / -3.7 | +2.7 / -3.7 | +2.7 / -4.7 |
| … |  |  |  |  |  |  |  |  |  |
| 70 |  |  |  |  | 23 | ±2.7 | +2.7 / -3.7 | +2.7 / -3.7 | +2.7 / -4.7 |
| 72 |  |  |  |  | 23 | ±2.7 | +2.7 / -3.7 | +2.7 / -3.7 | +2.7 / -4.7 |
| … |  |  |  |  |  |  |  |  |  |
| 74 |  |  |  |  | 23 | ±2.7 | +2.7 / -3.7 | +2.7 / -3.7 | +2.7 / -4.7 |
| 85 |  |  |  |  | 23 | ±2.71 | +2.7 / 1,2 -3.7 | +2.7 / 1,2 -3.7 | +2.7 / 1,2 -4.7 |
| 87 |  |  |  |  | 23 | ±2.7 | +2.7 / -3.7 | +2.7 / -3.7 | +2.7 / -4.7 |
| 88 |  |  |  |  | 23 | ±2.7 | +2.7 / -3.7 | +2.7 / -3.7 | +2.7 / -4.7 |
| Note 1: For transmission bandwidths (Figure 5.4.2-1) confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB. Note 2: For the UE maximum output power modified by MPR, the power limits specified in Table 6.2.5.3-1 apply Note 3: For a UE that supports both Band 18 and Band 26, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5dB for transmission bandwidths confined within 815 MHz and 818 MHz |  |  |  |  |  |  |  |  |  |

------------------------------------------------------------- NEXT CHANGE---- ------------------------------------------------------

#### 6.2.3_2.5	Test requirements

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2.3_2.5-1.

Table 6.2.3_2.5-1: UE Power Class test requirements for Multi-Cluster PUSCH

| Configuration ID | EUTRA band | Class 1 (dBm) | Tol. (dB) | Class 2 (dBm) | Tol. (dB) | Class 3 (dBm) | Tol. (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 31, 51, 54, 72 |  |  |  |  | 23 | +2.7 / -12.7 |
| 2 | 31, 51, 54, 72 |  |  |  |  | 23 | +2.7 / -7.7 |
| 3 | 31, 51, 54, 72 |  |  |  |  | 23 | +2.7 / -7.7 |
| 4 | 5, 6, 8, 11, 12, 13, 14, 17, 24, 27, 30 |  |  |  |  | 23 | +2.7 / -12.7 |
| 5 | 5, 6, 8, 11, 12, 13, 14, 17, 24, 27, 30 |  |  |  |  | 23 | +2.7 / -7.7 |
| 5 | 24 |  |  |  |  | 23 | +2.7 / 1,2 -7.7 |
| 6 | 5, 6, 8, 11, 12, 13, 14, 17, 27, 30 |  |  |  |  | 23 | +2.7 / -7.7 |
| 6 | 24 |  |  |  |  | 23 | +2.7 / 1,2 -7.71,2 |
| 7 | 18, 19, 21, 26, 34, 68 |  |  |  |  | 23 | +2.7 / -11.7 |
| 8 | 18, 19, 21, 26, 34, 68 |  |  |  |  | 23 | +2.7 / -7.7 |
| 9 | 18, 19, 21, 26, 34, 68 |  |  |  |  | 23 | +2.7 / -7.7 |
| 10 | 1, 2, 3, 4, 7, 9, 10, 20, 23, 25, 28, 33, 35, 36, 37, 38, 39, 40, 41,44, 45, 50, 65, 66, 70, 71 |  |  |  |  | 23 | +2.7 / -13.2 |
|  | 22, 42, 43, 48 |  |  |  |  | 23 | +3.0 / -13.5 |
| 11 | 1, 2, 3, 4, 7, 9, 10, 20, 23, 25, 28, 33, 35, 36, 37, 38, 39, 40, 41, 44, 45, 50, 65, 66, 70, 71 |  |  |  |  | 23 | +2.7 / -7.7 |
|  | 22, 42, 43 |  |  |  |  | 23 | +3.0 / -8.0 |
| 12 | 1, 2, 3, 4, 7, 9, 10, 20, 23, 25, 28,33, 35, 36, 37, 38, 39, 40, 41, 44, 45, 50, 65, 66, 70, 71 |  |  |  |  | 23 | +2.7 / -7.7 |
|  | 22, 42, 43, 48 |  |  |  |  | 23 | +3.0 / -8.0 |
| Note 1: For transmission bandwidths (Figure 5.4.2-1) confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB. Note 2: For the UE maximum output power modified by MPR, the power limits specified in Table 6.2.5.3-1 apply |  |  |  |  |  |  |  |

------------------------------------------------------------- NEXT CHANGE---- ------------------------------------------------------

#### 6.2.3_3.5	Test requirements

The maximum output power, derived in step 2 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2.3_3.5-1.

Table 6.2.3_3.5-1: UE Power Class test requirements

| E-UTRA Band | Class 1 (dBm) | Tol. (Db) | Class 2 (dBm) | Tol. (dB) | Class 3 (dBm) | 64QAM partial RB allocation Tol. (dB) | 64QAM full RB allocation Tol. (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  | 23 | +2.7 / -4.7 | +2.7 / -6.2 |
| 2 |  |  |  |  | 23 | +2.7 / 1,2 -4.7 | +2.7 / 1,2 -6.2 |
| 3 |  |  |  |  | 23 | +2.7 / 1,2 -4.7 | +2.7 / 1,2 -6.2 |
| 4 |  |  |  |  | 23 | +2.7 / -4.7 | +2.7 / -6.2 |
| 5 |  |  |  |  | 23 | +2.7 / -4.7 | +2.7 / -6.2 |
| 6 |  |  |  |  | 23 | +2.7 / -4.7 | +2.7 / -6.2 |
| 7 |  |  |  |  | 23 | +2.7 / 1,2 -4.7 | +2.7 / 1,2 -6.2 |
| 8 |  |  |  |  | 23 | +2.7 / 1,2 -4.7 | +2.7 / 1,2 -6.2 |
| 9 |  |  |  |  | 23 | +2.7 / -4.7 | +2.7 / -6.2 |
| 10 |  |  |  |  | 23 | +2.7 / -4.7 | +2.7 / -6.2 |
| 11 |  |  |  |  | 23 | +2.7 / -4.7 | +2.7 / -6.2 |
| 12 |  |  |  |  | 23 | +2.7 / 1,2 -4.7 | +2.7 / 1,2 -6.2 |
| 13 |  |  |  |  | 23 | +2.7 / -4.7 | +2.7 / -6.2 |
| 14 |  |  |  |  | 23 | +2.7 / -4.7 | +2.7 / -6.2 |
| … |  |  |  |  |  |  |  |
| 17 |  |  |  |  | 23 | +2.7 / -4.7 | +2.7 / -6.2 |
| 18 |  |  |  |  | 23 | +2.7 / 3 -4.7 | +2.7 / 3 -6.2 |
| 19 |  |  |  |  | 23 | +2.7 / -4.7 | +2.7 / -6.2 |
| 20 |  |  |  |  | 23 | +2.7 / 1,2 -4.7 | +2.7 / 1,2 -6.2 |
| 21 |  |  |  |  | 23 | +2.7 / -4.7 | +2.7 / -6.2 |
| 22 |  |  |  |  | 23 | +3.0/-6.5 | +3.0/-8 |
| 23 |  |  |  |  | 23 | +2.7 / -4.7 | +2.7 / -6.2 |
| 24 |  |  |  |  | 23 | +2.7 / 1,2 -5.7 | +2.7 / 1,2 -6.7 |
| 25 |  |  |  |  | 23 | +2.7 / 1,2 -4.7 | +2.7 / 1,2 -6.2 |
| 26 |  |  |  |  | 23 | +2.7 / 1,2 -4.7 | +2.7 / 1,2 -6.2 |
| 27 |  |  |  |  | 23 | +2.7 / -4.7 | +2.7 / -6.2 |
| 28 |  |  |  |  | 23 | +2.7/-5.2 | +2.7/-6.7 |
| 31 |  |  |  |  | 23 | +2.7 / -4.7 | +2.7 / -6.2 |
| ... |  |  |  |  |  |  |  |
| 33 |  |  |  |  | 23 | +2.7 / -4.7 | +2.7 / -6.2 |
| 34 |  |  |  |  | 23 | +2.7 / -4.7 | +2.7 / -6.2 |
| 35 |  |  |  |  | 23 | +2.7 / -4.7 | +2.7 / -6.2 |
| 36 |  |  |  |  | 23 | +2.7 / -4.7 | +2.7 / -6.2 |
| 37 |  |  |  |  | 23 | +2.7 / -4.7 | +2.7 / -6.2 |
| 38 |  |  |  |  | 23 | +2.7 / -4.7 | +2.7 / -6.2 |
| 39 |  |  |  |  | 23 | +2.7 / -4.7 | +2.7 / -6.2 |
| 40 |  |  |  |  | 23 | +2.7 / -4.7 | +2.7 / -6.2 |
| 41 |  |  |  |  | 23 | +2.7 / 1,2 -4.7 | +2.7 / 1,2 -6.2 |
| 42 |  |  |  |  | 23 | +3.0 / -6.0 | +3.0 / -7.5 |
| 43 |  |  |  |  | 23 | +3.0 / -6.0 | +3.0 / -7.5 |
| 44 |  |  |  |  | 23 | +2.7 / [-5.7] | +2.7 / [-7.2] |
| 45 |  |  |  |  | 23 | +2.7 / -4.7 | +2.7 / -6.2 |
| … |  |  |  |  |  |  |  |
| 54 |  |  |  |  | 23 | +2.7 / -4.7 | +2.7 / -6.2 |
| … |  |  |  |  |  |  |  |
| 65 |  |  |  |  | 23 | +2.7 / -4.7 | +2.7 / -6.2 |
| 66 |  |  |  |  | 23 | +2.7 / -4.7 | +2.7 / -6.2 |
| … |  |  |  |  |  |  |  |
| 68 |  |  |  |  | 23 | +2.7 / -4.7 | +2.7 / -6.2 |
| … |  |  |  |  |  |  |  |
| 70 |  |  |  |  | 23 | +2.7 / -4.7 | +2.7 / -6.2 |
| 72 |  |  |  |  | 23 | +2.7 / -4.7 | +2.7 / -6.2 |
| … |  |  |  |  |  |  |  |
| 74 |  |  |  |  | 23 | +2.7 / -4.7 | +2.7 / -6.2 |
| 85 |  |  |  |  | 23 | +2.7 / 1,2 -4.7 | +2.7 / 1,2 -6.2 |
| 87 |  |  |  |  | 23 | +2.7 / -4.7 | +2.7 / -6.2 |
| 88 |  |  |  |  | 23 | +2.7 / -4.7 | +2.7 / -6.2 |
| Note 1: For transmission bandwidths (Figure 5.4.2-1) confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB. Note 2: For the UE maximum output power modified by MPR, the power limits specified in Table 6.2.5.3-1 apply Note 3: For a UE that supports both Band 18 and Band 26, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5dB for transmission bandwidths confined within 815 MHz and 818 MHz |  |  |  |  |  |  |  |

------------------------------------------------------------- NEXT CHANGE---- ------------------------------------------------------

#### 6.2.3_4.5	Test requirements

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2.3_4.5-1.

Table 6.2.3_4.5-1: UE Power Class test requirements for Multi-Cluster PUSCH with UL 64QAM

| Configuration ID | EUTRA band | Class 1 (dBm) | Tol. (dB) | Class 2 (dBm) | Tol. (dB) | Class 3 (dBm) | Tol. (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 31, 51, 54, 72 |  |  |  |  | 23 | +2.7 / -12.7 |
| 2 | 31, 51, 54, 72 |  |  |  |  | 23 | +2.7 / -7.7 |
| 3 | 31, 51, 54, 72 |  |  |  |  | 23 | +2.7 / -7.7 |
| 4 | 5, 6, 8, 11, 12, 13, 14, 17, 24, 27, 30 |  |  |  |  | 23 | +2.7 / -12.7 |
| 5 | 5, 6, 8, 11, 12, 13, 14, 17, 27, 30 |  |  |  |  | 23 | +2.7 / -7.7 |
| 5 | 24 |  |  |  |  | 23 | +2.7 / 1,2 -7.7 |
| 6 | 11, 12, 13, 14, 17, 27, 30 |  |  |  |  | 23 | +2.7 / -7.7 |
| 6 | 24 |  |  |  |  | 23 | +2.7 / 1,2 -7.7 |
| 7 | 18, 19, 21, 26, 34, 68 |  |  |  |  | 23 | +2.7 / -11.7 |
| 8 | 18, 19, 21, 26, 34, 68 |  |  |  |  | 23 | +2.7 / -7.7 |
| 9 | 18, 19, 21, 26, 34, 68 |  |  |  |  | 23 | +2.7 / -7.7 |
| 10 | 1, 2, 3, 4, 7, 9, 10, 20, 23, 25, 28, 33, 35, 36, 37, 38, 39, 40, 41, 44, 45, 50, 65, 66, 70, 71 |  |  |  |  | 23 | +2.7 / -13.2 |
| 10 | 22, 42, 43, 48 |  |  |  |  | 23 | +2.7 / -13.5 |
| 11 | 1, 2, 3, 4, 7, 9, 10, 20, 23, 25, 28,33, 35, 36, 37, 38, 39, 40, 41, 44, 45, 50, 65, 66, 70, 71 |  |  |  |  | 23 | +2.7 / -7.7 |
| 11 | 22, 42, 43, 48 |  |  |  |  | 23 | +2.7 / -8.0 |
| 12 | 1, 2, 3, 4, 7, 9, 10, 20, 23, 25, 28,33, 35, 36, 37, 38, 39, 40, 41, 44, 45, 50, 65, 66, 70, 71 |  |  |  |  | 23 | +2.7 / -7.7 |
| 12 | 22, 42, 43, 48 |  |  |  |  | 23 | +2.7 / -8.0 |
| Note 1: For transmission bandwidths (Figure 5.4.2-1) confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB. Note 2: For the UE maximum output power modified by MPR, the power limits specified in Table 6.2.5.3-1 apply |  |  |  |  |  |  |  |

------------------------------------------------------------- NEXT CHANGE---- ------------------------------------------------------

#### 6.2.3_5.5	Test requirements

The maximum output power, derived in step 2 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2.3_5.5-1.

Table 6.2.3_5.5-1: UE Power Class test requirements

| E-UTRA Band | Class 1 (dBm) | Tol. (Db) | Class 2 (dBm) | Tol. (dB) | Class 3 (dBm) | 256QAM partial RB allocation Tol. (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  | 23 | +2.7 / -7.7 |
| 2 |  |  |  |  | 23 | +2.7 / 1,2 -7.7 |
| 3 |  |  |  |  | 23 | +2.7 / 1,2 -7.7 |
| 4 |  |  |  |  | 23 | +2.7 / -7.7 |
| 5 |  |  |  |  | 23 | +2.7 / -7.7 |
| 6 |  |  |  |  | 23 | +2.7 / -7.7 |
| 7 |  |  |  |  | 23 | +2.7 / 1,2 -7.7 |
| 8 |  |  |  |  | 23 | +2.7 / 1,2 -7.7 |
| 9 |  |  |  |  | 23 | +2.7 / -7.7 |
| 10 |  |  |  |  | 23 | +2.7 / -7.7 |
| 11 |  |  |  |  | 23 | +2.7 / -7.7 |
| 12 |  |  |  |  | 23 | +2.7 / 1,2 -7.7 |
| 13 |  |  |  |  | 23 | +2.7 / -7.7 |
| 14 |  |  |  |  | 23 | +2.7 / -7.7 |
| … |  |  |  |  |  |  |
| 17 |  |  |  |  | 23 | +2.7 / -7.7 |
| 18 |  |  |  |  | 23 | +2.7 / 3 -7.7 |
| 19 |  |  |  |  | 23 | +2.7 / -7.7 |
| 20 |  |  |  |  | 23 | +2.7 / 1,2 -7.7 |
| 21 |  |  |  |  | 23 | +2.7 / -7.7 |
| 22 |  |  |  |  | 23 | +3.0/-6.5 |
| 23 |  |  |  |  | 23 | +2.7 / -7.7 |
| 24 |  |  |  |  | 23 | +2.7 / 1,2 -8.7 |
| 25 |  |  |  |  | 23 | +2.7 / 1,2 -7.7 |
| 26 |  |  |  |  | 23 | +2.7 / 1,2 -7.7 |
| 27 |  |  |  |  | 23 | +2.7 / -7.7 |
| 28 |  |  |  |  | 23 | +2.7/-5.2 |
| 31 |  |  |  |  | 23 | +2.7 / -7.7 |
| ... |  |  |  |  |  |  |
| 33 |  |  |  |  | 23 | +2.7 / -7.7 |
| 34 |  |  |  |  | 23 | +2.7 / -7.7 |
| 35 |  |  |  |  | 23 | +2.7 / -7.7 |
| 36 |  |  |  |  | 23 | +2.7 / -7.7 |
| 37 |  |  |  |  | 23 | +2.7 / -7.7 |
| 38 |  |  |  |  | 23 | +2.7 / -7.7 |
| 39 |  |  |  |  | 23 | +2.7 / -7.7 |
| 40 |  |  |  |  | 23 | +2.7 / -7.7 |
| 41 |  |  |  |  | 23 | +2.7 / 1,2 -7.7 |
| 42 |  |  |  |  | 23 | +3.0 / -9.0 |
| 43 |  |  |  |  | 23 | +3.0 / -9.0 |
| 44 |  |  |  |  | 23 | +2.7 / [-8.7] |
| 45 |  |  |  |  | 23 | +2.7 / -7.7 |
| … |  |  |  |  |  |  |
| 54 |  |  |  |  | 23 | +2.7 / -7.7 |
| … |  |  |  |  |  |  |
| 65 |  |  |  |  | 23 | +2.7 / -7.7 |
| 66 |  |  |  |  | 23 | +2.7 / -7.7 |
| … |  |  |  |  |  |  |
| 70 |  |  |  |  | 23 | +2.7 / -7.7 |
| 72 |  |  |  |  | 23 | +2.7 / -7.7 |
| … |  |  |  |  |  |  |
| 74 |  |  |  |  | 23 | +2.7 / -7.7 |
| 85 |  |  |  |  | 23 | +2.7 / 1,2 -7.7 |
| 87 |  |  |  |  | 23 | +2.7 / -7.7 |
| 88 |  |  |  |  | 23 | +2.7 / -7.7 |
| Note 1: For transmission bandwidths (Figure 5.4.2-1) confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB. Note 2: For the UE maximum output power modified by MPR, the power limits specified in Table 6.2.5.3-1 apply Note 3: For a UE that supports both Band 18 and Band 26, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5dB for transmission bandwidths confined within 815 MHz and 818 MHz |  |  |  |  |  |  |

------------------------------------------------------------- NEXT CHANGE---- ------------------------------------------------------

#### 6.2.3_6.5	Test requirements

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2.3_6.5-1.

Table 6.2.3_6.5-1: UE Power Class test requirements for Multi-Cluster PUSCH with UL 256QAM

| Configuration ID | EUTRA band | Class 1 (dBm) | Tol. (dB) | Class 2 (dBm) | Tol. (dB) | Class 3 (dBm) | Tol. (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 31, 51, 54, 72, 73 |  |  |  |  | 23 | +2,7 / -12,7 |
| 2 | 31, 51, 54, 72, 73 |  |  |  |  | 23 | +2,7 / -11,2 |
| 3 | 31, 51, 54, 72, 73 |  |  |  |  | 23 | +2,7 / -11,2 |
| 4 | 5, 6, 8, 11, 12, 13, 14, 17, 24, 27, 30 |  |  |  |  | 23 | +2,7 / -12,7 |
| 5 | 5, 6, 8, 11, 12, 13, 14, 17, 24, 27, 30 |  |  |  |  | 23 | +2,7 / -11,2 |
| 6 | 5, 6, 8, 11, 12, 13, 14, 17, 24, 27, 30 |  |  |  |  | 23 | +2,7 / -11,2 |
| 7 | 18, 19, 21, 26, 34, 68 |  |  |  |  | 23 | +2,7 / -11,7 |
| 8 | 18, 19, 21, 26, 34, 68 |  |  |  |  | 23 | +2,7 / -11,2 |
| 9 | 18, 19, 21, 26, 34, 68 |  |  |  |  | 23 | +2,7 / -11,2 |
| 10 | 1, 2, 3, 4, 7, 9, 10, 20, 23, 25, 28, 33, 35, 36, 37, 38, 39, 40, 41, 44, 45, 50, 65, 66, 70, 71, 74 |  |  |  |  | 23 | +2,7 / -13,2 |
|  | 22, 42, 43, 48 |  |  |  |  | 23 | +3,0 / -13,5 |
| 11 | 1, 2, 3, 4, 7, 9, 10, 20, 23, 25, 28, 33, 35, 36, 37, 38, 39, 40, 41, 44, 45, 50, 65, 66, 70, 71, 74 |  |  |  |  | 23 | +2,7 / -11,2 |
|  | 22, 42, 43, 48 |  |  |  |  | 23 | +3,0 / -11,5 |
| 12 | 1, 2, 3, 4, 7, 9, 10, 20, 23, 25, 28, 33, 35, 36, 37, 38, 39, 40, 41, 44, 45, 50, 65, 66, 70, 71, 74 |  |  |  |  | 23 | +2,7 / -11,2 |
|  | 22, 42, 43, 48 |  |  |  |  | 23 | +3,0 / -11,5 |

------------------------------------------------------------- NEXT CHANGE---- ------------------------------------------------------

#### 6.2.3B.5	Test requirements

The maximum output power, derived in step 2 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2.3B.5-1.

Table 6.2.3B.5-1: UE Power Class test requirements

| E-UTRA Band | Class 1 (dBm) | Tol. (Db) | Class 2 (dBm) | Tol. (dB) | Class 3 (dBm) | QPSK partial RB allocation Tol. (dB)) | QPSK full RB allocation Tol. (dB)) | 16QAM partial RB allocation Tol. (dB) | 16QAM full RB allocation Tol. (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  | 23 | +2.7/-3.7 | +2.7/-4.7 | +2.7/-4.7 | +2.7/-5.7 |
| 2 |  |  |  |  | 23 | +2.7/-3.71 | +2.7/-4.71,2 | +2.7/-4.71,2 | +2.7/-5.71,2 |
| 3 |  |  |  |  | 23 | +2.7/-3.71 | +2.7/-4.71,2 | +2.7/-4.71,2 | +2.7/-5.71,2 |
| 4 |  |  |  |  | 23 | +2.7/-3.7 | +2.7/-4.7 | +2.7/-4.7 | +2.7/-5.7 |
| 5 |  |  |  |  | 23 | +2.7/-3.7 | +2.7/-4.7 | +2.7/-4.7 | +2.7/-5.7 |
| 6 |  |  |  |  | 23 | +2.7/-3.7 | +2.7/-4.7 | +2.7/-4.7 | +2.7/-5.7 |
| 7 |  |  |  |  | 23 | +2.7/-3.71 | +2.7/-4.71,2 | +2.7/-4.71,2 | +2.7/-5.71,2 |
| 8 |  |  |  |  | 23 | +2.7/-3.71 | +2.7/-4.71,2 | +2.7/-4.71,2 | +2.7/-5.71,2 |
| 9 |  |  |  |  | 23 | +2.7/-3.7 | +2.7/-4.7 | +2.7/-4.7 | +2.7/-5.7 |
| 10 |  |  |  |  | 23 | +2.7/-3.7 | +2.7/-4.7 | +2.7/-4.7 | +2.7/-5.7 |
| 11 |  |  |  |  | 23 | +2.7/-3.7 | +2.7/-4.7 | +2.7/-4.7 | +2.7/-5.7 |
| 12 |  |  |  |  | 23 | +2.7/-3.71 | +2.7/-4.71,2 | +2.7/-4.71,2 | +2.7/-5.71,2 |
| 13 |  |  |  |  | 23 | +2.7/-3.7 | +2.7/-4.7 | +2.7/-4.7 | +2.7/-5.7 |
| 14 |  |  |  |  | 23 | +2.7/-3.7 | +2.7/-4.7 | +2.7/-4.7 | +2.7/-5.7 |
| … |  |  |  |  |  |  |  |  |  |
| 17 |  |  |  |  | 23 | +2.7/-3.7 | +2.7/-4.7 | +2.7/-4.7 | +2.7/-5.7 |
| 18 |  |  |  |  | 23 | +2.7/-3.7 | +2.7/-4.7 | +2.7/-4.7 | +2.7/-5.7 |
| 19 |  |  |  |  | 23 | +2.7/-3.7 | +2.7/-4.7 | +2.7/-4.7 | +2.7/-5.7 |
| 20 |  |  |  |  | 23 | +2.7/-3.71 | +2.7/-4.71,2 | +2.7/-4.71,2 | +2.7/-5.71,2 |
| 21 |  |  |  |  | 23 | +2.7/-3.7 | +2.7/-4.7 | +2.7/-4.7 | +2.7/-5.7 |
| 22 |  |  |  |  | 23 | +3.0/-5.5 | +3.0/-6.5 | +3.0/-6.5 | +3.0/-7.5 |
| … |  |  |  |  |  |  |  |  |  |
| 23 |  |  |  |  | 23 | +2.7/-3.7 | +2.7/-4.7 | +2.7/-4.7 | +2.7/-5.7 |
| 24 |  |  |  |  | 23 | +2.7/-3.7 | +2.7/-5.71,2 | +2.7/-5.71,2 | +2.7/-6.71,2 |
| 25 |  |  |  |  | 23 | +2.7/-3.71 | +2.7/-[4.7]1,2 | +2.7/-[4.7]1,2 | +2.7/-[5.7]1,2 |
| 26 |  |  |  |  | 23 | +2.7/-3.7 | +2.7/-4.7 | +2.7/-4.7 | +2.7/-5.7 |
| 27 |  |  |  |  | 23 | +2.7/-3.7 | +2.7/-4.7 | +2.7/-4.7 | +2.7/-5.7 |
| 28 |  |  |  |  | 23 | +2.7/[-3.7] | +2.7/[-4.7] | +2.7/[-4.7] | +2.7/[-5.7] |
| 30 |  |  |  |  | 23 | +2.7/-3.7 | +2.7/-4.7 | +2.7/-4.7 | +2.7/-5.7 |
| 31 |  |  |  |  | 23 | +2.7/-3.7 | +2.7/-4.7 | +2.7/-4.7 | +2.7/-5.7 |
| ... |  |  |  |  |  |  |  |  |  |
| 33 |  |  |  |  | 23 | +2.7/-3.7 | +2.7/-4.7 | +2.7/-4.7 | +2.7/-5.7 |
| 34 |  |  |  |  | 23 | +2.7/-3.7 | +2.7/-4.7 | +2.7/-4.7 | +2.7/-5.7 |
| 35 |  |  |  |  | 23 | +2.7/-3.7 | +2.7/-4.7 | +2.7/-4.7 | +2.7/-5.7 |
| 36 |  |  |  |  | 23 | +2.7/-3.7 | +2.7/-4.7 | +2.7/-4.7 | +2.7/-5.7 |
| 37 |  |  |  |  | 23 | +2.7/-3.7 | +2.7/-4.7 | +2.7/-4.7 | +2.7/-5.7 |
| 38 |  |  |  |  | 23 | +2.7/-3.7 | +2.7/-4.7 | +2.7/-4.7 | +2.7/-5.7 |
| 39 |  |  |  |  | 23 | +2.7/-3.7 | +2.7/-4.7 | +2.7/-4.7 | +2.7/-5.7 |
| 40 |  |  |  |  | 23 | +2.7/-3.7 | +2.7/-4.7 | +2.7/-4.7 | +2.7/-5.7 |
| 41 |  |  |  |  | 23 | +2.7/-3.71 | +2.7/-4.7 | +2.7/-4.7 | +2.7/-5.7 |
| 42 |  |  |  |  | 23 | +3.0/-5.0 | +3.0/-6.01,2 | +3.0/-6.01,2 | +3.0/-7.01,2 |
| 43 |  |  |  |  | 23 | +3.0/-5.0 | +3.0/-6.0 | +3.0/-6.0 | +3.0/-7.01,2 |
| 44 |  |  |  |  | 23 | +2.7/[-3.7] | +2.7/[-4.7] | +2.7/[-4.7] | +2.7/[-5.7] |
| 45 |  |  |  |  | 23 | +2.7/-3.7 | +2.7/-4.7 | +2.7/-4.7 | +2.7/-5.7 |
| … |  |  |  |  |  |  |  |  |  |
| 54 |  |  |  |  | 23 | +2.7/-3.7 | +2.7/-4.7 | +2.7/-4.7 | +2.7/-5.7 |
| … |  |  |  |  |  |  |  |  |  |
| 65 |  |  |  |  | 23 | +2.7/-3.7 | +2.7/-4.7 | +2.7/-4.7 | +2.7/-5.7 |
| 66 |  |  |  |  | 23 | +2.7/-3.7 | +2.7/-4.7 | +2.7/-4.7 | +2.7/-5.7 |
| … |  |  |  |  |  |  |  |  |  |
| 68 |  |  |  |  | 23 | +2.7/-3.7 | +2.7/-4.7 | +2.7/-4.7 | +2.7/-5.7 |
| … |  |  |  |  |  |  |  |  |  |
| 70 |  |  |  |  | 23 | +2.7/-3.7 | +2.7/-4.7 | +2.7/-4.7 | +2.7/-5.7 |
| 72 |  |  |  |  | 23 | +2.7/-3.7 | +2.7/-4.7 | +2.7/-4.7 | +2.7/-5.7 |
| … |  |  |  |  |  |  |  |  |  |
| 74 |  |  |  |  | 23 | +2.7/-3.7 | +2.7/-4.7 | +2.7/-4.7 | +2.7/-5.7 |
| Note 1: For transmission configurations (Figure 5.4.2-1) confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB. Note 2: For the UE maximum output power modified by MPR, the power limits specified in Table 6.2.5B.3-1 apply |  |  |  |  |  |  |  |  |  |

------------------------------------------------------------- END OF CHANGES ------------------------------------------------------