3GPP TSG-RAN5 Meeting #99	R5-232315

# Incheon, Korea (Republic Of), 22nd May 2023 – 26th May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 5447 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to A-MPR test cases as part of introduction of LTE Band 54 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ligado Networks |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_TDD_1670_1675MHz-UEConTest |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Introduction of LTE TDD Band 54 to conformance specifications; Updates to show that A-MPR is not utilized to meet the additional spurious emissions requirements associated with NS_62 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Relevant clauses have been updated to introduce Band 54 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It cannot be determined whether UE is meeting the additional spurious emissions requirements without utilizing A-MPR for NS_62. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.4.2, 6.2.4.3, 6.2.4.4.1, 6.2.4.4.3, 6.2.4.5, 6.2.4_2.2, 6.2.4_2.4.1, 6.2.4_2.5, 6.2.4_4.2, 6.2.4_4.4.1, 6.2.4_4.5, 6.2.4B.2, 6.2.4B.4.1, 6.2.4B.5, 6.2.4_s.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 6.2.4.2	Test applicability

The requirements of this test apply in test case 6.6.2.2 Additional Spectrum Emission Mask for network signalled values NS_03, NS_04, NS_06, NS_07, NS_11, NS_20, NS_21, NS_27 and NS_35 to all types of E-UTRA Power Class 3 UE release 8 and forward.

The requirements of this test apply in test case 6.6.3.3 Additional Spurious Emissions for network signalled values NS_05, NS_07, NS_08, NS_09, NS_11, NS_12, NS_13, NS_14, NS_15, NS_16, NS_17, NS_18, NS_19, NS_20, NS_21, NS_22, NS_23, NS_24, NS_25, NS_27, NS_28, NS_29, NS_30, NS 31, NS_36, NS_38, NS_39, NS_45, NS_56, NS_62 to all types of E-UTRA Power Class 3 UE release 8 and forward.

NOTE:	As a result TC 6.2.4 has not been included in the test case applicability table 4.1-1, TS 36.521-2. This does not preclude the test from being used for R&D or other purposes if deemed useful.

#### 6.2.4.3	Minimum conformance requirements

For UE Power Class 3 the specific requirements and identified sub-clauses are specified in Table 6.2.4.3-1 along with the allowed A-MPR values that may be used to meet these requirements. The allowed A-MPR values specified below in Table 6.2.4.3-1 to 6.2.4.3-31 are in addition to the allowed MPR requirements specified in clause 6.2.3. For the UE maximum output power modified by A-MPR, the power limits specified in clause 6.2.5 apply.

Table 6.2.4.3-1: Additional Maximum Power Reduction (A-MPR) / Spectrum Emission requirements

| Network Signalling value | Requirements (sub-clause) | E-UTRA Band | Channel bandwidth (MHz) | Resources Blocks (NRB) | A-MPR (dB) |  |
| --- | --- | --- | --- | --- | --- | --- |
| NS_01 | 6.6.2.1.1 | Table 5.2-1 | 1.4,3,5,10,15,20 | Table 5.4.2-1 | N/A |  |
| NS_03 | 6.6.2.2.3.1 | 2,4,10, 23, 25,35,36, 66, 70 | 3 | >5 | ≤ 1 |  |
|  |  |  | 5 | >6 | ≤ 1 |  |
|  |  |  | 10 | >6 | ≤ 1 |  |
|  |  |  | 15 | >8 | ≤ 1 |  |
|  |  |  | 20 | >10 | ≤ 1 |  |
| NS_04 | 6.6.2.2.3.2 | 41 | 5 | >6 | ≤1 |  |
|  |  |  | 10, 15, 20 | Table 6.2.4.3-2 |  |  |
| NS_04 | 6.6.2.2.3.2, 6.6.3.3.3.1 | 41 | 5,10, 15, 20 | Table 6.2.4.3-3, Table 6.2.4.3-4 |  |  |
| NS_05 | 6.6.3.3.3.2 | 1 | 10,15,20 | ≥ 50 1 | ≤ 1 1 |  |
|  |  |  | 15, 20 | Table 6.2.4.3-5 2 |  |  |
|  |  | 65 3 | 10,15,20 | ≥ 50 | ≤ 1 1 |  |
|  |  |  | 15,20 | Table 6.2.4.3-5 2 |  |  |
| NS_06 | 6.6.2.2.3.3 | 12, 13, 14, 17 | 1.4, 3, 5, 10 | Table 5.4.2-1 | N/A |  |
|  |  | 85 | 5,10 | Table 5.4.2-1 | N/A |  |
| NS_07 | 6.6.2.2.3.3 6.6.3.3.3.3 | 13 | 10 | Table 6.2.4.3-6 | Table 6.2.4.3-6 |  |
| NS_08 | 6.6.3.3.3.4 | 19 | 10, 15 | > 44 | ≤ 3 |  |
| NS_09 | 6.6.3.3.3.5 | 21 | 10, 15 | > 40 | ≤ 1 |  |
|  |  |  |  | > 55 | ≤ 2 |  |
|  |  | 74 6 | 10, 15 | > 40 | ≤ 1 |  |
|  |  |  |  | > 55 | ≤ 2 |  |
| NS_10 |  | 20 | 15, 20 | Table 6.2.4.3-7 | Table 6.2.4.3-7 |  |
| NS_11 | 6.6.2.2.1 6.6.3.3.3.6 | 231 | 1.4, 3, 5, 10,15,20 | Table 6.2.4.3-8 | Table 6.2.4.3-8 |  |
| NS_12 | 6.6.3.3.3.7 | 26 | 1.4, 3, 5, 10, 15 | Table 6.2.4.3-9, Table 6.2.4.3-10 | Table 6.2.4.3-9, Table 6.2.4.3-10 |  |
| NS_13 | 6.6.3.3.3.8 | 26 | 5 | Table 6.2.4.3-11 | Table 6.2.4.3-11 |  |
| NS_14 | 6.6.3.3.3.9 | 26 | 10, 15 | Table 6.2.4.3-12 | Table 6.2.4.3-12 |  |
| NS_15 | 6.6.3.3.3.10 | 26 | 1.4, 3, 5, 10, 15 | Table 6.2.4.3-13 Table 6.2.4.3-14 | Table 6.2.4.3-13, Table 6.2.4.3-14 |  |
| NS_16 | 6.6.3.3.3.11 | 27 | 3, 5, 10 | Table 6.2.4.3-15, Table 6.2.4.3-16, Table 6.2.4.3-17 |  |  |
| NS_17 | 6.6.3.3.3.12 | 28 | 5, 10 | Table 5.4.2-1 | N/A |  |
| NS_18 | 6.6.3.3.3.13 | 28 | 5 | ≥ 2 | ≤ 1 |  |
|  |  |  | 10, 15, 20 | ≥ 1 | ≤ 4 |  |
| NS_19 | 6.6.3.3.3.14 | 44 | 10, 15, 20 | Table 6.2.4.3-18 | Table 6.2.4.3-18 |  |
| NS_20 | 6.2.2 6.6.2.2.1 6.6.3.3.3.15 | 23 | 5, 10, 15, 20 | Table 6.2.4.3-19 | Table 6.2.4.3-19 |  |
| NS_21 | 6.6.2.2.3.1 6.6.3.3.3.16 | 30 | 5, 10 | Table 6.2.4.3-20 | Table 6.2.4.3-20 |  |
| NS_22 | 6.6.3.3.3.17 | 42, 43 | 5, 10, 15, 20 | Table 6.2.4.3-21 |  |  |
| NS_23 | 6.6.3.3.3.18 | 42, 43 | 5, 10, 15, 20 | N/A |  |  |
| NS_24 | 6.6.3.3.3.19 | 65 4 | 5, 10, 15, 20 | Table 6.2.4.3-22 |  |  |
| NS_25 | 6.6.3.3.3.20 | 65 4 | 5, 10, 15, 20 | Table 6.2.4.3-23 |  |  |
| … |  |  |  |  |  |  |
| NS_27 | 6.6.2.2.3.4, 6.6.3.3.3.21 | 48 | 5, 10, 15, 20 | Table 6.2.4.3-24 |  |  |
| NS_28 | 6.2.2A, 6.6.3.3.22 | 46 (NOTE 5) | 20 | Table 6.2.4-25 | NS_28 |  |
| NS_29 | 6.2.2A, 6.6.2.3.1a, 6.6.3.3.23 | 46 (NOTE 5) | 20 | Table 6.2.4-26 | NS_29 |  |
| NS_30 | 6.2.2A, 6.6.3.3.24 | 46 (NOTE 5) | 20 | Table 6.2.4-27 | NS_30 |  |
| NS_31 | 6.2.2A, 6.6.3.3.25 | 46 (NOTE 5) | 20 | Table 6.2.4-28 | NS_31 |  |
| NS_32 | - | - | - | - | - |  |
| NS_35 | 6.6.2.2.3.5 | 71 | 5, 10, 15, 20 | N/A |  |  |
| NS_36 |  | 68 | 5, 10, 15 | Table 6.2.4.3-29 |  |  |
| … |  |  |  |  |  |  |
| NS_38 | 6.6.3.3.3.27 | 74 | 3, 5, 10, 15, 20 | Table 6.2.4.3-30 |  |  |
| NS_39 | 6.6.3.3.3.28 | 74 | 10, 15, 20 | Table 6.2.4.3-31 |  |  |
| … |  |  |  |  |  |  |
| NS_45 | 6.6.3.3.3.29 | 53 | 1.4, 3, 5, 10 | N/A |  |  |
| .. |  |  |  |  |  |  |
| NS_56 | 6.6.3.3.3.31 | 24 | 5, 10 | Table 6.2.4.3-32 |  |  |
| … |  |  |  |  |  |  |
| NS_62 | 6.6.3.3.3.32 | 54 | 1.4, 3, 5 | N/A |  |  |
| Note 1 Applicable when the lower edge of the assigned E-UTRA UL channel bandwidth frequency is larger than or equal to the upper edge of PHS band (1915.7 MHz) + 4 MHz + the channel BW assigned, where channel BW is as defined in subclause 5.6. A-MPR for operations below this frequency is not covered in this version of specifications except for the channel assignments in Note2 as the emissions requirement in 6.6.3.3.3.2 may not be met. For 10MHz channel bandwidth whose carrier frequency is larger than or equal to 1945 MHz or 15 MHz channel bandwidth whose carrier frequency is larger than or equal to 1947.5 MHz, no A-MPR applies. Note 2 Applicable when carrier frequency is 1932.5 MHz for 15MHz channel bandwidth or 1930 MHz for 20MHz channel bandwidth case. Note 3: Applicable when the E-UTRA carrier is within 1920-1980 MHz. Note 4: Applicable when the upper edge of the channel bandwidth frequency is greater than 1980MHz. Note 5: Applicable only for an LAA Scell configured in Band 46. |  |  |  |  |  |  |

<<unchanged text skipped>>

------------------------------------------------------------- NEXT CHANGE---- ------------------------------------------------------

##### 6.2.4.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA operating bands specified in table 5.4.2.1-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.2.4.4.1-1 through table 6.2.4.4.1-26. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

<<unchanged text skipped>>

Table 6.2.4.4.1-27: Test Configuration Table (network signalled value "NS_56")

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  |  | Normal |  |  |
| Test Frequencies (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  |  | Refer to FUL in test parameters with following: 5 MHz CH BW: 1630 MHz (low range) 10 MHz CH BW: 1632.5 MHz (low range) |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  |  | 5MHz, 10MHz |  |  |
| Test Parameters for NS_56 A-MPR |  |  |  |  |  |  |  |
|  |  |  | Downlink Configuration |  | Uplink Configuration |  |  |
| Configuration ID | FUL | Ch BW | Mod'n | RB allocation FDD | Mod'n | RB allocation FDD | RBstart FDD |
| 1 | 1630 | 5MHz | N/A for A-MPR testing |  | QPSK | 1 | 0 |
| 2 | 1630 | 5MHz |  |  | QPSK | 25 | 0 |
| 3 (Notes 2,3) | 1630 | 5MHz |  |  | 16QAM | 25 | 0 |
| 4 | 1632.5 | 10MHz |  |  | QPSK | 1 | 0 |
| 5 | 1632.5 | 10MHz |  |  | QPSK | 12 | 0 |
| 6 | 1632.5 | 10MHz |  |  | QPSK | 50 | 0 |
| 7 (Notes 2,3) | 1632.5 | 10MHz |  |  | 16QAM | 50 | 0 |
| 8 | 1632.5 | 10MHz |  |  | QPSK | 1 | 6 |
| 9 | 1632.5 | 10MHz |  |  | QPSK | 1 | 35 |
| 10 | 1632.5 | 10MHz |  |  | QPSK | 1 | 40 |
| 11 | 1632.5 | 10MHz |  |  | QPSK | 1 | 49 |
| 12 | 1632.5 | 10MHz |  |  | QPSK | 15 | 35 |
| 13 | 1632.5 | 10MHz |  |  | QPSK | 10 | 40 |
| 14 (Notes 2,3) | 1632.5 | 10MHz |  |  | 16QAM | 10 | 40 |
| Note 1: The Configuration ID will be used to map the applicable Test Configuration to the corresponding Test Requirement in subclause 6.2.4.5 as not all combinations are necessarily required based on the applicability of the UE. Note 2: Applies only for UE-Categories ≥2. Note 3: The 16QAM configurations shall not apply to UEs which support UL 64QAM. |  |  |  |  |  |  |  |

Table 6.2.4.4.1-28: Test Configuration Table (network signalled value "NS_62")

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  | Normal |  |
| Test Frequencies (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | Low range, Mid range, High range |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | 1.4MHz, 3MHz, 5MHz |  |
| Test Parameters for NS_62 A-MPR |  |  |  |  |  |
|  |  | Downlink Configuration |  | Uplink Configuration |  |
| Configura-tion ID | Ch BW | Mod'n | RB allocation TDD | Mod'n | RB allocation TDD |
| 1 | 1.4MHz | N/A for A-MPR testing |  | QPSK | 1 |
| 2 | 1.4MHz |  |  | QPSK | 6 |
| 3 | 1.4MHz |  |  | 16QAM (Note 3) | 6 |
| 4 | 3MHz |  |  | QPSK | 1 |
| 5 | 3MHz |  |  | QPSK | 15 |
| 6 | 3MHz |  |  | 16QAM (Note 3) | 15 |
| 7 | 5MHz |  |  | QPSK | 1 |
| 8 | 5MHz |  |  | QPSK | 25 |
| 9 | 5MHz |  |  | 16QAM (Note 3) | 25 |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 2: The Configuration ID will be used to map the applicable Test Configuration to the corresponding Test Requirement in subclause 6.2.4.5 as not all combinations are necessarily required based on the applicability of the UE. Note 3: The 16QAM configurations shall not apply to UEs which support UL 64QAM. |  |  |  |  |  |

Editor’s note: The following lines belong at the end of section 6.2.4.4.1. As new tables are added to this section, these lines should always follow the tables.

1.	Connect the SS to the UE antenna connectors as shown in Figure TS 36.508 [7] Annex A, Figure A.3.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] clause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channels are set according to the applicable table from Table 6.2.4.4.1-1 to Table 6.2.4.4.1-26.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.2.4.4.3.

------------------------------------------------------------- NEXT CHANGE---- ------------------------------------------------------

##### 6.2.4.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6, with the following exceptions for each network signalled value.

<<unchanged text skipped>>

6.2.4.4.3.31	Message contents exceptions (network signalled value "NS_56")

1.	Information element additionalSpectrumEmission is set to NS_56. This can be set in the SystemInformationblockType2 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.4.4.3.31-1: SystemInformationBlockType2: Additional spurious emissions requirement

| Derivation Path: TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 56 (NS_56) |  |  |

6.2.4.4.3.32	Message contents exceptions (network signalled value "NS_62")

1.	Information element additionalSpectrumEmission is set to NS_62. This can be set in the SystemInformationblockType2 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.4.4.3.32-1: SystemInformationBlockType2: Additional spurious emissions requirement

| Derivation Path: TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 62 (NS_62) |  |  |

------------------------------------------------------------- NEXT CHANGE---- ------------------------------------------------------

#### 6.2.4.5	Test requirements

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in the applicable table from Table 6.2.4.5-1 to Table 6.2.4.5-29. The allowed A-MPR values specified in Table 6.2.4.3-1 are in addition to the allowed MPR requirements specified in clause 6.2.3. For the UE maximum output power modified by MPR and/or A-MPR, the power limits specified in Table 6.2.5.3-1 apply.

<<unchanged text skipped>>

Table 6.2.4.5-30: UE Power Class 3 test requirements (network signalled value "NS_56")

| Configuration ID | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | TL,c (dB) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 8 | 0 | 15 | 3 | 5 | 25.7 | 9.3 |
| 2 | 1 | 2 | 0 | 20 | 3 | 2.5 | 25.7 | 16.3 |
| 3 | 2 | 2 | 0 | 19 | 3 | 3.5 | 25.7 | 14.8 |
| 4 | 0 | 7 | 0 | 16 | 3 | 5 | 25.7 | 10.3 |
| 5 | 0 | 5 | 0 | 18 | 3 | 4 | 25.7 | 13.3 |
| 6 | 1 | 7 | 0 | 15 | 3 | 5 | 25.7 | 9.3 |
| 7 | 2 | 7 | 0 | 14 | 3 | 5 | 25.7 | 8.3 |
| 8 | 0 | 5 | 0 | 18 | 3 | 4 | 25.7 | 13.3 |
| 9 | 0 | 4 | 0 | 19 | 3 | 3.5 | 25.7 | 14.8 |
| 10 | 0 | 5 | 0 | 18 | 3 | 4 | 25.7 | 13.3 |
| 11 | 0 | 5 | 0 | 18 | 3 | 4 | 25.7 | 13.3 |
| 12 | 1 | 2 | 0 | 20 | 3 | 2.5 | 25.7 | 16.3 |
| 13 | 0 | 3 | 0 | 20 | 3 | 2.5 | 25.7 | 16.3 |
| 14 | 1 | 3 | 0 | 19 | 3 | 3.5 | 25.7 | 14.8 |
| Note 1: Lower limit is assuming ΔTIB,c is zero. If non-zero, PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5.3-1. |  |  |  |  |  |  |  |  |

Table 6.2.4.5-31: UE Power Class 3 test requirements (network signalled value "NS_62")

| Configuration ID | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | TL,c (dB) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 0 | 0 | 23 | 2 | 2 | 25.7 | 20.3 |
| 2 | 1 | 0 | 0 | 22 | 2 | 2 | 25.7 | 19.3 |
| 3 | 2 | 0 | 0 | 21 | 2 | 2 | 25.7 | 18.3 |
| 4 | 0 | 0 | 0 | 23 | 2 | 2 | 25.7 | 20.3 |
| 5 | 1 | 0 | 0 | 22 | 2 | 2 | 25.7 | 19.3 |
| 6 | 2 | 0 | 0 | 21 | 2 | 2 | 25.7 | 18.3 |
| 7 | 0 | 0 | 0 | 23 | 2 | 2 | 25.7 | 20.3 |
| 8 | 1 | 0 | 0 | 22 | 2 | 2 | 25.7 | 19.3 |
| 9 | 2 | 0 | 0 | 21 | 2 | 2 | 25.7 | 18.3 |
| Note 1: Lower limit is assuming ΔTIB,c is zero. If non-zero, PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5.3-1. |  |  |  |  |  |  |  |  |

------------------------------------------------------------- NEXT CHANGE---- ------------------------------------------------------

#### 6.2.4_2.2	Test applicability

The requirements of this test apply in test case 6.6.2.2_1 Additional Spectrum Emission Mask for network signalled values NS_03, NS_04, NS_06, NS_07, NS_11, NS_20, NS_21, NS_27 and NS_35 to all types of E-UTRA UE from release 13 and forward that support UL 64QAM.

The requirements of this test apply in test case 6.6.3.3_1 Additional Spurious Emissions for network signalled values NS_04, NS_05, NS_07, NS_08, NS_09, NS_11, NS_12, NS_13, NS_14, NS_15, NS_16, NS_17, NS_18, NS_19, NS_20, NS_21, NS_22, NS_23, NS_24, NS_25, NS_27, NS_36, NS_38, NS_39, NS_56, NS_62 to all types of E-UTRA UE from release 13 and forward that support UL 64QAM.

NOTE:	As a result TC 6.2.4_2 has not been included in the test case applicability table 4.1-1, TS 36.521-2. This does not preclude the test from being used for R&D or other purposes if deemed useful.

------------------------------------------------------------- NEXT CHANGE---- ------------------------------------------------------

##### 6.2.4_2.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA operating bands specified in table 5.4.2.1-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.2.4_2.4.1-1 through table 6.2.4_2.4.1-27. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

<<unchanged text skipped>>

Table 6.2.4_2.4.1-28: Test Configuration Table (network signalled value "NS_56")

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  |  | Normal |  |  |
| Test Frequencies (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  |  | Refer to FUL in test parameters with following: 5 MHz CH BW: 1630 MHz (low range) 10 MHz CH BW: 1632.5 MHz (low range) |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  |  | 5MHz, 10MHz |  |  |
| Test Parameters for NS_56 A-MPR |  |  |  |  |  |  |  |
|  |  |  | Downlink Configuration |  | Uplink Configuration |  |  |
| Configuration ID | FUL | Ch BW | Mod'n | RB allocation FDD | Mod'n | RB allocation FDD | RBstart FDD |
| 1 | 1630 | 5MHz | N/A for A-MPR testing |  | 64QAM | 1 | 0 |
| 2 | 1630 | 5MHz |  |  | 64QAM | 25 | 0 |
| 3 | 1632.5 | 10MHz |  |  | 64QAM | 1 | 0 |
| 4 | 1632.5 | 10MHz |  |  | 64QAM | 50 | 0 |
| Note 1: The 64QAM configurations shall not apply to UEs for which TC 6.2.4_4 is executed with network signalled value NS_56. |  |  |  |  |  |  |  |

Table 6.2.4_2.4.1-29: Test Configuration Table (network signalled value "NS_62")

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  | Normal |  |
| Test Frequencies (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | Low range, Mid range, High range |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | 1.4MHz, 3MHz, 5MHz |  |
| Test Parameters for NS_62 A-MPR |  |  |  |  |  |
|  |  | Downlink Configuration |  | Uplink Configuration |  |
| Configura-tion ID | Ch BW | Mod'n | RB allocation TDD | Mod'n | RB allocation TDD |
| 1 | 1.4MHz | N/A for A-MPR testing |  | 64QAM | 1 |
| 2 | 1.4MHz |  |  | 64QAM | 6 |
| 3 | 3MHz |  |  | 64QAM | 1 |
| 4 | 3MHz |  |  | 64QAM | 15 |
| 5 | 5MHz |  |  | 64QAM | 1 |
| 6 | 5MHz |  |  | 64QAM | 25 |
| Note 1: The 64QAM configurations shall not apply to UEs for which TC 6.2.4_4 is executed with network signalled value NS_62. |  |  |  |  |  |

------------------------------------------------------------- NEXT CHANGE---- ------------------------------------------------------

#### 6.2.4_2.5	Test requirements

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in the applicable table from Table 6.2.4_2.5-1 to Table 6.2.4_2.5-29. The allowed A-MPR values specified in Table 6.2.4_2.3-1 are in addition to the allowed MPR requirements specified in clause 6.2.3. For the UE maximum output power modified by MPR and/or A-MPR, the power limits specified in Table 6.2.5.3-1 apply.

<<unchanged text skipped>>

Table 6.2.4_2.5-30: UE Power Class 3 test requirements (network signalled value "NS_56")

| Configuration ID | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 8 | 0 | 13 | 5 | 25.7 | 7.3 |
| 2 | 3 | 2 | 0 | 18 | 4 | 25.7 | 13.3 |
| 3 | 2 | 7 | 0 | 14 | 5 | 25.7 | 8.3 |
| 4 | 3 | 7 | 0 | 13 | 5 | 25.7 | 7.3 |
| Note 1: Lower limit is assuming ΔTIB,c is zero. If non-zero, PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5.3-1. |  |  |  |  |  |  |  |

Table 6.2.4_2.5-31: UE Power Class 3 test requirements (network signalled value "NS_62")

| Configuration ID | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 0 | 0 | 21 | 2 | 25.7 | 18.3 |
| 2 | 3 | 0 | 0 | 20 | 2 | 25.7 | 17.3 |
| 3 | 2 | 0 | 0 | 21 | 2 | 25.7 | 18.3 |
| 4 | 3 | 0 | 0 | 20 | 2 | 25.7 | 17.3 |
| 5 | 2 | 0 | 0 | 21 | 2 | 25.7 | 18.3 |
| 6 | 3 | 0 | 0 | 20 | 2 | 25.7 | 17.3 |
| Note 1: Lower limit is assuming ΔTIB,c is zero. If non-zero, PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5.3-1. |  |  |  |  |  |  |  |

------------------------------------------------------------- NEXT CHANGE---- ------------------------------------------------------

#### 6.2.4_4.2	Test applicability

The requirements of this test apply in test case 6.6.2.2_2 Additional Spectrum Emission Mask for network signalled values NS_03, NS_04, NS_06, NS_07, NS_11, NS_20, NS_21, NS_27 and NS_35 to all types of E-UTRA UE from release 14 and forward that support UL 256QAM.

The requirements of this test apply in test case 6.6.3.3_2 Additional Spurious Emissions for network signalled values NS_04, NS_05, NS_07, NS_08, NS_09, NS_11, NS_12, NS_13, NS_14, NS_15, NS_16, NS_17, NS_18, NS_19, NS_20, NS_21, NS_22, NS_23, NS_24, NS_25, NS_27, NS_36, NS_56, NS_62 to all types of E-UTRA UE from release 14 and forward that support UL 256QAM.

NOTE:	As a result, TC 6.2.4_4 has not been included in the test case applicability table 4.1-1, TS 36.521-2. This does not preclude the test from being used for R&D or other purposes if deemed useful.

------------------------------------------------------------- NEXT CHANGE---- ------------------------------------------------------

##### 6.2.4_4.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA operating bands specified in table 5.4.2.1-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.2.4_4.4.1-1 through table 6.2.4_4.4.1-27. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

<<unchanged text skipped>>

Table 6.2.4_4.4.1-28: Test Configuration Table (network signalled value "NS_56”)

|  | Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  |  |  | Normal |  |  |
| Test Frequencies (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  |  |  | Refer to FUL in test parameters with following: 5 MHz CH BW: 1630 MHz (low range) 10 MHz CH BW: 1632.5 MHz (low range) |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  |  |  | 5MHz, 10MHz |  |  |
| Test Parameters for NS_56 A-MPR |  |  |  |  |  |  |  |  |
|  |  |  |  | Downlink Configuration |  | Uplink Configuration |  |  |
| Configuration ID |  | FUL | Ch BW | Mod'n | RB allocation FDD | Mod'n | RB allocation FDD | RBstart FDD |
| 1 |  | 1630 | 5MHz | N/A for A-MPR testing |  | 256QAM | 1 | 0 |
| 2 |  | 1630 | 5MHz |  |  | 256QAM | 25 | 0 |
| 3 |  | 1632.5 | 10MHz |  |  | 256QAM | 1 | 0 |
| 4 |  | 1632.5 | 10MHz |  |  | 256QAM | 50 | 0 |
| Note 1: The Configuration ID will be used to map the applicable Test Configuration to the corresponding Test Requirement in subclause 6.2.4_4.5 as not all combinations are necessarily required based on the applicability of the UE. |  |  |  |  |  |  |  |  |

Table 6.2.4_4.4.1-29: Test Configuration Table (network signalled value "NS_62")

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  | Normal |  |
| Test Frequencies (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | Low range, Mid range, High range |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | 1.4MHz, 3MHz, 5MHz |  |
| Test Parameters for NS_62 A-MPR |  |  |  |  |  |
|  |  | Downlink Configuration |  | Uplink Configuration |  |
| Configura-tion ID | Ch BW | Mod'n | RB allocation TDD | Mod'n | RB allocation TDD |
| 1 | 1.4MHz | N/A for A-MPR testing |  | 256QAM | 1 |
| 2 | 1.4MHz |  |  | 256QAM | 6 |
| 3 | 3MHz |  |  | 256QAM | 1 |
| 4 | 3MHz |  |  | 256QAM | 15 |
| 5 | 5MHz |  |  | 256QAM | 1 |
| 6 | 5MHz |  |  | 256QAM | 25 |
| Note 1: The 64QAM configurations shall not apply to UEs for which TC 6.2.4_4 is executed with network signalled value NS_62. |  |  |  |  |  |

Editor’s note: The following lines belong at the end of section 6.2.4_4.4.1. As new tables are added to this section, these lines should always follow the tables.

1.	Connect the SS to the UE antenna connectors as shown in Figure TS 36.508 [7] Annex A, Figure A.3.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] clause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channels are set according to the applicable table from Table 6.2.4_4.4.1-1 to Table 6.2.4_4.4.1-27.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.2.4_4.4.3.

------------------------------------------------------------- NEXT CHANGE---- ------------------------------------------------------

#### 6.2.4_4.5	Test requirements

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in the applicable table from Table 6.2.4_4.5-1 to Table 6.2.4_4.5-29. The allowed A-MPR values specified in Table 6.2.4.3-1 are in addition to the allowed MPR requirements specified in clause 6.2.3. For the UE maximum output power modified by MPR and/or A-MPR, the power limits specified in Table 6.2.5.3-1 apply.

<<unchanged text skipped>>

Table 6.2.4_4.5-30: UE Power Class 3 test requirements (network signalled value "NS_56")

| Configuration ID | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 5 | 8 | 0 | 10.0 | 6.0 | 25.7 | 3.3 |
| 2 | 5 | 2 | 0 | 16.0 | 5.0 | 25.7 | 10.3 |
| 3 | 5 | 7 | 0 | 11.0 | 6.0 | 25.7 | 4.3 |
| 4 | 5 | 7 | 0 | 11.0 | 6.0 | 25.7 | 4.3 |
| NOTE: Lower limit is assuming ΔTIB,c is zero. If non-zero, PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5.3-1. |  |  |  |  |  |  |  |

Table 6.2.4_5.5-31: UE Power Class 3 test requirements (network signalled value "NS_62")

| Configuration ID | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 5 | 0 | 0 | 18 | 4 | 25.7 | 13.3 |
| 2 | 5 | 0 | 0 | 18 | 4 | 25.7 | 13.3 |
| 3 | 5 | 0 | 0 | 18 | 4 | 25.7 | 13.3 |
| 4 | 5 | 0 | 0 | 18 | 4 | 25.7 | 13.3 |
| 5 | 5 | 0 | 0 | 18 | 4 | 25.7 | 13.3 |
| 6 | 5 | 0 | 0 | 18 | 4 | 25.7 | 13.3 |
| Note 1: Lower limit is assuming ΔTIB,c is zero. If non-zero, PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5.3-1. |  |  |  |  |  |  |  |

------------------------------------------------------------- NEXT CHANGE---- ------------------------------------------------------

#### 6.2.4B.2	Test applicability

The requirements of this test apply in test case 6.6.2.2 B Additional Spectrum Emission Mask for network signalled values NS_03, NS_04, NS_06, NS_07, NS_11, NS_20, NS_21, NS_27 and NS_35 to all types of E-UTRA UE release 10 and forward that support UL MIMO.

The requirements of this test apply in test case 6.6.3.3 B Additional Spurious Emissions for network signalled values NS_04, NS_05, NS_07, NS_08, NS_09, NS_12, NS_13, NS_14, NS_15, NS_16, NS_17, NS_18, NS_19, NS_21, NS_22, NS_23, NS_24, NS_25, NS_27, NS_36, NS_38, NS_39, NS_56, NS_62 to all types of E-UTRA UE release 10 and forward that support UL MIMO.

NOTE:	As a result TC 6.2.4B has not been included in the test case applicability table 4.1-1, TS 36.521-2. This does not preclude the test from being used for R&D or other purposes if deemed useful.

------------------------------------------------------------- NEXT CHANGE---- ------------------------------------------------------

##### 6.2.4B.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA operating bands specified in table 5.4.2.1-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.2.4B.4.1-1 through table 6.2.4B.4.1-23. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

<<unchanged text skipped>>

Table 6.2.4B.4.1-24: Test Configuration Table (network signalled value "NS_56")

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  |  | Normal |  |  |
| Test Frequencies (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  |  | Refer to FUL in test parameters with following: 5 MHz CH BW: 1630 MHz (low range) 10 MHz CH BW: 1632.5 MHz (low range) |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  |  | 5MHz, 10MHz |  |  |
| Test Parameters for NS_56 A-MPR |  |  |  |  |  |  |  |
|  |  |  | Downlink Configuration |  | Uplink Configuration |  |  |
| Configuration ID | FUL | Ch BW | Mod'n | RB allocation FDD | Mod'n | RB allocation FDD | RBstart FDD |
| 1 | 1630 | 5MHz | N/A for A-MPR testing |  | QPSK | 1 | 0 |
| 2 | 1630 | 5MHz |  |  | QPSK | 25 | 0 |
| 3 (Notes 2,3) | 1630 | 5MHz |  |  | 16QAM | 25 | 0 |
| 4 | 1632.5 | 10MHz |  |  | QPSK | 1 | 0 |
| 5 | 1632.5 | 10MHz |  |  | QPSK | 12 | 0 |
| 6 | 1632.5 | 10MHz |  |  | QPSK | 50 | 0 |
| 7 (Notes 2,3) | 1632.5 | 10MHz |  |  | 16QAM | 50 | 0 |
| 8 | 1632.5 | 10MHz |  |  | QPSK | 1 | 6 |
| 9 | 1632.5 | 10MHz |  |  | QPSK | 1 | 35 |
| 10 | 1632.5 | 10MHz |  |  | QPSK | 1 | 40 |
| 11 | 1632.5 | 10MHz |  |  | QPSK | 1 | 49 |
| 12 | 1632.5 | 10MHz |  |  | QPSK | 15 | 35 |
| 13 | 1632.5 | 10MHz |  |  | QPSK | 10 | 40 |
| 14 (Notes 2,3) | 1632.5 | 10MHz |  |  | 16QAM | 10 | 40 |
| Note 1: The Configuration ID will be used to map the applicable Test Configuration to the corresponding Test Requirement in subclause 6.2.4B.5 as not all combinations are necessarily required based on the applicability of the UE. Note 2: Applies only for UE-Categories ≥2. Note 3: The 16QAM configurations shall not apply to UEs which support UL 64QAM. |  |  |  |  |  |  |  |

Table 6.2.4B.4.1-25: Test Configuration Table (network signalled value "NS_62")

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  | Normal |  |
| Test Frequencies (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | Low range, Mid range, High range |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | 1.4MHz, 3MHz, 5MHz |  |
| Test Parameters for NS_62 A-MPR |  |  |  |  |  |
|  |  | Downlink Configuration |  | Uplink Configuration |  |
| Configura-tion ID | Ch BW | Mod'n | RB allocation TDD | Mod'n | RB allocation TDD |
| 1 | 1.4MHz | N/A for A-MPR testing |  | QPSK | 1 |
| 2 | 1.4MHz |  |  | QPSK | 6 |
| 3 | 1.4MHz |  |  | 16QAM (Note 3) | 6 |
| 4 | 3MHz |  |  | QPSK | 1 |
| 5 | 3MHz |  |  | QPSK | 15 |
| 6 | 3MHz |  |  | 16QAM (Note 3) | 15 |
| 7 | 5MHz |  |  | QPSK | 1 |
| 8 | 5MHz |  |  | QPSK | 25 |
| 9 | 5MHz |  |  | 16QAM (Note 3) | 25 |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 2: The Configuration ID will be used to map the applicable Test Configuration to the corresponding Test Requirement in subclause 6.2.4.5 as not all combinations are necessarily required based on the applicability of the UE. Note 3: The 16QAM configurations shall not apply to UEs which support UL 64QAM. |  |  |  |  |  |

Editor’s note: The following lines belong at the end of section 6.2.4B.4.1. As new tables are added to this section, these lines should always follow the tables.

1.	Connect the SS to the UE antenna connectors as shown in Figure TS 36.508 [7] Annex A, Figure A.3.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] clause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channels are set according to the applicable table from Table 6.2.4B.4.1-1 to 6.2.4B.4.1-23.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.2.4B.4.3.

------------------------------------------------------------- NEXT CHANGE---- ------------------------------------------------------

#### 6.2.4B.5	Test requirements

The maximum output power, derived in step 2 shall be within the range prescribed by the nominal maximum output power and tolerance in the applicable table from Table 6.2.4B.5-1 to Table 6.2.4B.5-27. The allowed A-MPR values specified in Table 6.2.4B.3-1 are in addition to the allowed MPR requirements specified in clause 6.2.3B. For the UE maximum output power modified by MPR and/or A-MPR, the power limits specified in Table 6.2.5B.3-1 apply.

<<unchanged text skipped>>

Table 6.2.4B.5-28: UE Power Class 3 test requirements (network signalled value "NS_56")

| Configuration ID | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 8 | 0 | 15 | 6 | 25.7 | 8.3 |
| 2 | 1 | 2 | 0 | 20 | 6 | 25.7 | 13.3 |
| 3 | 2 | 2 | 0 | 19 | 5 | 25.7 | 13.3 |
| 4 | 0 | 7 | 0 | 16 | 5 | 25.7 | 10.3 |
| 5 | 0 | 5 | 0 | 18 | 5 | 25.7 | 12.3 |
| 6 | 1 | 7 | 0 | 15 | 6 | 25.7 | 8.3 |
| 7 | 2 | 7 | 0 | 14 | 6 | 25.7 | 7.3 |
| 8 | 0 | 5 | 0 | 18 | 5 | 25.7 | 12.3 |
| 9 | 0 | 4 | 0 | 19 | 5 | 25.7 | 13.3 |
| 10 | 0 | 5 | 0 | 18 | 5 | 25.7 | 12.3 |
| 11 | 0 | 5 | 0 | 18 | 5 | 25.7 | 12.3 |
| 12 | 1 | 2 | 0 | 20 | 6 | 25.7 | 13.3 |
| 13 | 0 | 3 | 0 | 20 | 6 | 25.7 | 13.3 |
| 14 | 1 | 3 | 0 | 19 | 5 | 25.7 | 13.3 |
| Note 1: Lower limit is assuming ΔTIB,c is zero. If non-zero, PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5.3-1. |  |  |  |  |  |  |  |

Table 6.2.4B.5-29: UE Power Class 3 test requirements (network signalled value "NS_62")

| Configuration ID | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | TL,c (dB) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 0 | 0 | 23 | 2 | 2 | 25.7 | 20.3 |
| 2 | 1 | 0 | 0 | 22 | 2 | 2 | 25.7 | 19.3 |
| 3 | 2 | 0 | 0 | 21 | 2 | 2 | 25.7 | 18.3 |
| 4 | 0 | 0 | 0 | 23 | 2 | 2 | 25.7 | 20.3 |
| 5 | 1 | 0 | 0 | 22 | 2 | 2 | 25.7 | 19.3 |
| 6 | 2 | 0 | 0 | 21 | 2 | 2 | 25.7 | 18.3 |
| 7 | 0 | 0 | 0 | 23 | 2 | 2 | 25.7 | 20.3 |
| 8 | 1 | 0 | 0 | 22 | 2 | 2 | 25.7 | 19.3 |
| 9 | 2 | 0 | 0 | 21 | 2 | 2 | 25.7 | 18.3 |
| Note 1: Lower limit is assuming ΔTIB,c is zero. If non-zero, PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5.3-1. |  |  |  |  |  |  |  |  |

------------------------------------------------------------- NEXT CHANGE---- ------------------------------------------------------

6.2.4_s.2	Test applicability

The requirements of this test apply in test case 6.6.2.2_s Additional Spectrum Emission Mask for network signalled values NS_03, NS_04, NS_06, NS_07, NS_11, NS_20, NS_21, NS_27 and NS_35 to all types of E-UTRA Power Class 3 UE release 15 and forward that support subslot/slot TTI.

The requirements of this test apply in test case 6.6.3.3 Additional Spurious Emissions for network signalled values NS_05, NS_07, NS_08, NS_09, NS_11, NS_12, NS_13, NS_14, NS_15, NS_16, NS_17, NS_18, NS_19, NS_20, NS_21, NS_22, NS_23, NS_24, NS_25, NS_27, NS_28, NS_29, NS_30, NS 31, NS_36, NS_38. NS_39, NS_56, NS_62 to all types of E-UTRA Power Class 3 UE release 15 and forward that support subslot/slot TTI.

NOTE:	As a result TC 6.2.4_s has not been included in the test case applicability Table 4.1-1, TS 36.521-2. This does not preclude the test from being used for R&D or other purposes if deemed useful.

------------------------------------------------------------- END OF CHANGES ------------------------------------------------------