Title: 	Discussion on test point selection for MPR for CA in FR2

Source: 	Huawei

# Introduction

At RAN5#78, it was stated that each test case would require specific analysis to conform its test configuration table based on the study for Frequency, Channel BW, SCS, RB allocation (including modulations). This analysis will be captured in TR 38.905.

The purpose of this contribution is to provide the complete analysis for each parameter included in the Test Configuration Table and propose test points selection for NR MPR for CA test case in FR2.

# Discussion

Following subclauses introduce study for test environment, test frequencies, uplink waveform and modulation, test channel bandwidth, uplink RB allocation and test subcarrier spacing.

## CA configurations covered

In LTE, when testing UL CA requirements, the configured DL CCs are not more than UL CCs. However according to TS 38.101-2, MPR requirement depends on accumulative aggregated bandwidth. In this case the existing of DL CCs would have impact on UL MPR performance. It is the aggregated DL bandwidth that affects the requirements and not the number of DL CCs, meaning it is sufficient to add one extra DL CC (3DL/2UL configuration). Therefore it is proposed to test UL CA MPR with maximum accumulative aggregated bandwidth by configuring one extra DL CC with as high Wgap as possible in non-contiguous CA mode, in addition to the case of same number of DL CCs and UL CCs.

Proposal 1: Test UL CA MPR with maximum accumulative aggregated bandwidth by configuring one extra DL CC with as high Wgap as possible in non-contiguous CA mode, in addition to the case of same number of DL CCs and UL CCs..

## Test Environment

In NR FR2 MPR single carrier testing, Test Environment is Normal Conditions. This can be leveraged for CA testing.

Proposal 2: Define Test Environment as Normal Conditions (NC) for MPR for CA measurement in FR2.

  - Test Frequencies

For NR FR2 MPR for intra-band contiguous DL and UL CA, test frequencies can be selected as Low range and High range, which is following the same approach in LTE.

Proposal 3: Select Low range and High range test frequencies for MPR for intra-band contiguous DL and UL CA in FR2.

For NR FR2 MPR for intra-band non-contiguous DL CA and contiguous UL CA, since the cumulative aggregated channel bandwidth is maximum 1400MHz and less than the width of FR2 bands, it’s proposed to also test Low range and High range.

Proposal 4: Select Low range and High range test frequencies for MPR for intra-band non-contiguous DL CA and contiguous UL CA in FR2.

## Test Subcarrier Spacing

In NR FR2 MPR single carrier testing, only 120 kHz subcarrier spacing is tested. This can be leveraged for CA testing.

Proposal 5: Select 120 kHz subcarrier spacing for MPR for CA in FR2.

## Test channel bandwidth

In LTE the test channel bandwidth is selected as Highest NRB_agg and Lowest NRB_agg. In order to simplify the FR2 testing, only Highest NRB_agg is selected in FR2 MPR CA testing unless there is a testability problem so that this BW cannot be tested in MPR or ACLR test cases. In such cases, a lower NRB_agg is chosen.

Proposal 6: Select Highest NRB_agg for MPR for CA in FR2. In the case this NRB_agg cannot be tested due to testability problems, then the maximum testable NRB_agg is selected

## Uplink Waveform, Modulation and Uplink RB allocation

The selection of test points for Uplink waveform, Modulation and RB allocation should be based on the Minimum conformance requirements for MPR in [1], which are extracted as bellow:

### 6.2A.2	UE maximum output power reduction for CA

#### 6.2A.2.1	General

The UE is defined to be configured for CA operation when it has at least one of UL or DL configured for CA. In CA operation, the UE may reduce its maximum output power due to higher order modulations and transmit bandwidth configurations. This Maximum Power Reduction (MPR) is defined in subclauses below.

The cumulative aggregated channel bandwidth is defined as the frequency band from the lowest edge of the lowest CC to the upper edge of the highest CC of all UL and DL configured CCs. When the maximum output power of a UE is modified by MPR, the power limits specified in subclause 6.2A.4 apply.

The requirements in the following subclauses are only applicable to intra-band contiguous uplink CA, with the aggregated channelbandwidth up to 800 MHz.

#### 6.2A.2.2	Maximum output power reduction for power class 1

For power class 1, MPR for UL contiguous allocations within the cumulative aggregated bandwidth is defined as:

MPRC_CA = max(MPRWT_C_CA, MPRnarrow)

Where,

MPRnarrow = 14.4 dB, when BWalloc,RB is less than or equal to 1.44 MHz, MPRnarrow = 10 dB, when 1.44 MHz < BWalloc,RB ≤ 10.8 MHz, where BWalloc,RB is the bandwidth of the RB allocation size.

MPRWT_C_CA is the maximum power reduction due to modulation orders, transmit bandwidth configurations, and waveform types. MPRWT_contiguous is defined in Table 6.2A.2.2-1.

Table 6.2A.2.2-1: Maximum power reduction (MPRWT_C_CA) for UE power class 1

| Waveform Type |  | Cumulative aggregated channel bandwidth |  |  |
| --- | --- | --- | --- | --- |
|  |  | < 400 MHz | ≥ 400 MHz and < 800 MHz | ≥ 800 MHz and ≤ 1400 MHz |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 5.51 | 7.71 | [8.2] |
|  | QPSK | ≤ 6.51 | 8.71 | [9.7] |
|  | 16 QAM | ≤ 6.5 | 8.7 | [9.2] |
|  | 64 QAM | ≤ 9.0 | 10.7 | [11.2] |
| CP-OFDM | QPSK | ≤ 6.5 | 8.7 | [8.7] |
|  | 16 QAM | ≤ 6.5 | 8.7 | [8.7] |
|  | 64 QAM | ≤ 9.0 | 10.7 | [11.2] |
| NOTE 1: The following condition applies only when the cumulative aggregated BW of the CA configuration 400MHz. For a contiguous RB allocation in a single CC of the CA configuration, the single CC MPR of subclause 6.2.2.1 applies. The cumulative aggregated bandwidth shall be used as BWchannel in Tables 6.2.2.1-1 and 6.2.2.1-2. The applicable column in Tables 6.2.2.1-1 and 6.2.2.1-2 shall be determined based on the transmission bandwidth of the CA allocation in relation to the allocation regions defined in the tables. |  |  |  |  |

When different waveform types exist across CCs, the requirement is set by the waveform type used in the configuration with the largest MPRC_CA.

For non-contiguous RB allocations, the following rule for MPR applies:

MPR = max(MPRC_CA, -10*A +  [14.4])

Where:

A = NRB_alloc / NRB_agg_C.

NRB_alloc is the total number of allocated UL RBs

NRB_agg_C is the number of the aggregated RBs within the fully allocated cumulative aggregated channel bandwidth

#### 6.2A.2.3	Maximum output power reduction for power class 2

For power class 2, MPR specified in sub-clause 6.2A.2.4 applies.

Table 6.2A.2.3-1: (Void)

#### 6.2A.2.4	Maximum output power reduction for power class 3

For power class 3, MPR for UL contiguous allocations within the cumulative aggregated bandwidth is denoted as MPRC_CA and is defined in Table 6.2A.2.4-1.

Table 6.2A.2.4-1: Maximum power reduction (MPRC_CA) for UE power class 3

|  |  | Cumulative aggregated bandwidth configuration |  |  |
| --- | --- | --- | --- | --- |
|  |  | < 400 MHz | ≥ 400 MHz and < 800 MHz | ≥ 800 MHz and ≤ 1400 MHz |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 5.01 | ≤ 7.71 | ≤ [8.2] |
|  | QPSK | ≤ 5.01 | ≤ 7.71 | ≤ [8.2] |
|  | 16 QAM | ≤ 6.5 | ≤ 8.7 | ≤ [9.3] |
|  | 64 QAM | ≤ 9.0 | ≤ 10.7 | ≤ [11.2] |
| CP-OFDM | QPSK | ≤ 5.0 | ≤ 7.5 | ≤ [8.0] |
|  | 16 QAM | ≤ 6.5 | ≤ 8.7 | ≤ [9.2] |
|  | 64 QAM | ≤ 9.0 | ≤ 10.7 | ≤ [11.2] |
| NOTE 1: The following condition applies only when the cumulative aggregated BW of the CA configuration ≤ 400 MHz. For a contiguous RB allocation in a single CC of the CA configuration, the single CC MPR of subclause 6.2.2.1 applies. The cumulative aggregated bandwidth shall be used as BWchannel in Tables 6.2.2.1-1 and 6.2.2.1-2. The applicable column in Tables 6.2.2.1-1 and 6.2.2.1-2 shall be determined based on the transmission bandwidth of the CA allocation in relation to the allocation regions defined in the tables. |  |  |  |  |

When different waveform types exist across CCs, the requirement is set by the waveform type used in the configuration with the highest contiguous MPR.

For non-contiguous RB allocations, the following rule for MPR applies:

MPR = max(MPRC_CA, -10*A +7.0)

Where:

A = NRB_alloc / NRB_agg_C.

NRB_alloc is the total number of allocated UL RBs

NRB_agg_C is the number of the aggregated RBs within the fully allocated cumulative aggregated channel bandwidth

#### 6.2A.2.5	Maximum output power reduction for power class 4

For power class 4, MPR specified in sub-clause 6.2A.2.4 applies.

According to the above MPR requirement and the consideration of Crest Factors, when the waveform is same, the highest modulation order is the most critical case, and when modulation order is same, CP-OFDM is more critical than DFT-s-OFDM for MPR in FR2. Since FR2 testing is more time consuming it is proposed to only select the most critical waveform and modulation order for the same MPR requirement.

Proposal 7: Select the most critical waveform and modulation order for the test of the same MPR requirement, which is, if the waveform is same, select the max modulation order, if the modulation order is same, select CP-OFDM waveform. For DFT-s-OFDM with high modulation order and CP-OFDM with low modulation order, since it is hard to say which is more critical, both should be tested.

The specific Test points for Power class 1 and Power class 2/3/4 are analysed as below.

### 2.6.1 Power class 1

The MPR requirements for Power class 1 can be summarized as below table. In order to ensure the testing coverage, the test points are selected for each test requirement ID.

| Test requirement ID | Allocation type | BW alloc | Cumulative aggregated channel bandwidth | RB allocated in single CC or multiple CCs | MPR requirement |
| --- | --- | --- | --- | --- | --- |
| 1 | contiguous allocation: MPRC_CA | BW alloc <= 1.44MHz: MPRnarrow |  |  | MPR=14.4dB |
| 2 |  | BW alloc <=10.8MHz: MPRnarrow |  |  | MPR=10dB |
| 3 |  | BW alloc > 10.8MHz: MPRWT_contiguous | Cumulative aggregated channel bandwidth <= 400MHz | contiguous RB allocation in a single CC (DFT-s-OFDM BPSK or QPSK) | single CC MPR of subclause 6.2.2.1 |
| 4 |  |  |  | Others | MPR in 6.2A.2.2-1 |
| 5 |  |  | Cumulative aggregated channel bandwidth < 800MHz |  | MPR in 6.2A.2.2-1 |
| 6 |  |  | Cumulative aggregated channel bandwidth < 1400MHz |  | MPR in 6.2A.2.2-1 |
| 7 | Non-contiguous allocation |  |  |  | Max(MPRC_CA, -10*(NRB_alloc / NRB_agg_C) + [14.4]) |

Test requirement ID 1 and 2 depends on BW alloc <= 1.44MHz or BW alloc <=10.8MHz. Since the only SCS tested is 120 kHz, the corresponding transmission bandwidth are 1 RB or 7 RB respectively. For either intra-band contiguous DL CA and intra-band contiguous UL CA, or intra-band non-contiguous DL CA and intra-band contiguous UL CA, it is proposed to schedule the UL RBs at the very edges of the cumulative aggregated channel bandwidth. This principle can apply to UL CA configurations of any UL CC number. For each CA configuration 4 test points are verified.

Proposal 8: To verify Power Class 1 test requirement ID 1 and 2, for either intra-band contiguous DL CA and intra-band contiguous UL CA, or intra-band non-contiguous DL CA and intra-band contiguous UL CA, it is proposed to schedule the UL RBs at the very edges of the cumulative aggregated channel bandwidth. This principle can apply to UL CA configurations of any UL CC number.

Below table lists proposed test points for examples of CA configuration with 2 UL CC.

| Test ID | CC | Band | ChBw(MHz) | Test frequency | DL RB allocation | UL Modulation | UL RB allocation |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Default Test Settings for a CA_nXB, CA_nXD, CA_nXG, CA_nXO Configuration |  |  |  |  |  |  |  |
| 1 | PCC | nX | Default | Low/CC1 | N/A for this test | CP-OFDM 64QAM | Outer_1RB_Left |
|  | SCC | nX |  | Low/CC2 |  | - | - |
| 2 | PCC | nX |  | High/CC2 |  | CP-OFDM 64QAM | Outer_1RB_Right |
|  | SCC | nX |  | High/CC1 |  | - | - |
| 3 | PCC | nX |  | Low/CC1 |  | CP-OFDM 64QAM | 7@0 |
|  | SCC | nX |  | Low/CC2 |  | - | - |
| 4 | PCC | nX |  | High/CC2 |  | CP-OFDM 64QAM | 7@NRB-7 |
|  | SCC | nX |  | High/CC1 |  | - | - |
| Default Test Settings for a CA_nX(D-G), CA_nX(D-O) Configuration |  |  |  |  |  |  |  |
| 1 | PCC | nX | Default | Low/CC1 | N/A for this test | CP-OFDM 64QAM | Outer_1RB_Left |
|  | SCC1 | nX |  | Low/CC2 |  | - | - |
|  | Wgap | - |  | Max Wgap |  | N/A | N/A |
|  | SCC2 | nX |  | Low/CC3 |  | N/A | N/A |
|  | SCC3 | nX |  | Low/CC4 |  | N/A | N/A |
| 2 | PCC | nX |  | High/CC4 |  | CP-OFDM 64QAM | Outer_1RB_Right |
|  | SCC1 | nX |  | High/CC3 |  | - | - |
|  | Wgap | - |  | Max Wgap |  | N/A | N/A |
|  | SCC2 | nX |  | High/CC2 |  | N/A | N/A |
|  | SCC3 | nX |  | High/CC1 |  | N/A | N/A |
| 3 | PCC | nX |  | Low/CC1 |  | CP-OFDM 64QAM | 7@0 |
|  | SCC1 | nX |  | Low/CC2 |  | - | - |
|  | Wgap | - |  | Max Wgap |  | N/A | N/A |
|  | SCC2 | nX |  | Low/CC3 |  | N/A | N/A |
|  | SCC3 | nX |  | Low/CC4 |  | N/A | N/A |
| 4 | PCC | nX |  | High/CC4 |  | CP-OFDM 64QAM | 7@NRB-7 |
|  | SCC1 | nX |  | High/CC3 |  | - | - |
|  | Wgap | - |  | Max Wgap |  | N/A | N/A |
|  | SCC2 | nX |  | High/CC2 |  | N/A | N/A |
|  | SCC3 | nX |  | High/CC1 |  | N/A | N/A |

Test requirement ID 3 depends on BW alloc > 10.8MHz, cumulative aggregated channel bandwidth <= 400Hz, and UL RBs are allocated only in single CC. The MPR requirement of test requirement ID 3 refers to TS 38.101-1 6.2.2.1. It’s proposed to reuse the test points for single CC MPR testing in PCC only. Since only maximum NRB,agg is tested, there is no applicable intra-band non-contiguous CA configuration. This principle can apply to UL CA configurations of any UL CC number. For each applicable intra-band contiguous CA configuration 4~6 test points are verified.

Proposal 9: To verify Power Class 1 test requirement ID 3, it is proposed to reuse test points for single CC MPR testing in PCC only. This principle can apply to UL CA configurations of any UL CC number. The test requirement ID 3 only applies to intra-band contiguous CA configuration.

Below table lists proposed test points for examples of CA configuration with 2 UL CC and accumulative aggregated channel bandwidth <= 200MHz.

| Test ID | CC | Band | ChBw(MHz) | Test frequency | DL RB allocation | UL Modulation | UL RB allocation |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Default Test Settings for a CA_nXG, CA_nXO Configuration |  |  |  |  |  |  |  |
| 1 | PCC | nX | Default | Low/CC1 | N/A for this test | DFT-s-OFDM PI/2 BPSK | Outer_Full |
|  | SCC | nX |  | Low/CC2 |  | - | - |
| 2 | PCC | nX |  | High/CC2 |  | DFT-s-OFDM PI/2 BPSK | Outer_Full |
|  | SCC | nX |  | High/CC1 |  | - | - |
| 3 | PCC | nX |  | Low/CC1 |  | DFT-s-OFDM QPSK | Inner_Full |
|  | SCC | nX |  | Low/CC2 |  | - | - |
| 4 | PCC | nX |  | High/CC2 |  | DFT-s-OFDM QPSK | Inner_Full |
|  | SCC | nX |  | High/CC1 |  | - | - |

Below table lists proposed test points for examples of CA configuration with 2 UL CC and accumulative aggregated channel bandwidth <= 400MHz.

| Test ID | CC | Band | ChBw(MHz) | Test frequency | DL RB allocation | UL Modulation | UL RB allocation |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Default Test Settings for a CA_nXD Configuration |  |  |  |  |  |  |  |
| 1 | PCC | nX | Default | Low/CC1 | N/A for this test | DFT-s-OFDM PI/2 BPSK | Outer_Full |
|  | SCC | nX |  | Low/CC2 |  | - | - |
| 2 | PCC | nX |  | High/CC2 |  | DFT-s-OFDM PI/2 BPSK | Outer_Full |
|  | SCC | nX |  | High/CC1 |  | - | - |
| 3 | PCC | nX |  | Low/CC1 |  | DFT-s-OFDM PI/2 BPSK | Inner_Full |
|  | SCC | nX |  | Low/CC2 |  | - | - |
| 4 | PCC | nX |  | High/CC2 |  | DFT-s-OFDM PI/2 BPSK | Inner_Full |
|  | SCC | nX |  | High/CC1 |  | - | - |
| 5 | PCC | nX |  | Low/CC1 |  | DFT-s-OFDM QPSK | Inner_Full |
|  | SCC | nX |  | Low/CC2 |  | - | - |
| 6 | PCC | nX |  | High/CC2 |  | DFT-s-OFDM QPSK | Inner_Full |
|  | SCC | nX |  | High/CC1 |  | - | - |

Test requirement ID 4, 5 and 6 depends on BW alloc > 10.8MHz, and variable cumulative aggregated channel bandwidth, and UL RBs are contiguously allocated in multiple CCs. The most critical test point would be full allocation in all the UL CCs. The modulation type refers to TS 38.101-1 Table 6.2A.2.4-1. For intra-band contiguous DL CA and intra-band contiguous UL CA, it’s proposed to test only Mid frequency. For intra-band non-contiguous DL CA and intra-band contiguous UL CA, both High and Low range are tested. This principle can apply to UL CA configurations of any UL CC number. For each applicable CA configuration 3~10 test points are verified.

Proposal 10: To verify Power Class 1 test requirement ID 4, 5 and 6, it is proposed to schedule full allocation in all the UL CCs with modulation types in Table 6.2A.2.4-1. For intra-band contiguous DL CA and intra-band contiguous UL CA, it’s proposed to test only Mid frequency. For intra-band non-contiguous DL CA and intra-band contiguous UL CA, both High and Low range are tested. This principle can apply to UL CA configurations of any UL CC number.

Below table lists proposed test points for examples of CA configuration with 2 UL CC and accumulative aggregated channel bandwidth < 400MHz.

| Test ID | CC | Band | ChBw(MHz) | Test frequency | DL RB allocation | UL Modulation | UL RB allocation |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Default Test Settings for a CA_nXG, CA_nXO Configuration |  |  |  |  |  |  |  |
| 1 | PCC | nX | Default | Mid/CC1 | N/A for this test | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
|  | SCC | nX |  | Mid/CC2 |  | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
| 2 | PCC | nX |  | Mid/CC1 |  | CP-OFDM 16QAM | Outer_Full |
|  | SCC | nX |  | Mid/CC2 |  | CP-OFDM 16QAM | Outer_Full |
| 3 | PCC | nX |  | Mid/CC1 |  | CP-OFDM 64QAM | Outer_Full |
|  | SCC | nX |  | Mid/CC2 |  | CP-OFDM 64QAM | Outer_Full |

Below table lists proposed test points for examples of CA configuration with 2 UL CC and accumulative aggregated channel bandwidth < 800MHz.

| Test ID | CC | Band | ChBw(MHz) | Test frequency | DL RB allocation | UL Modulation | UL RB allocation |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Default Test Settings for a CA_nXD Configuration |  |  |  |  |  |  |  |
| 1 | PCC | nX | Default | Mid/CC1 | N/A for this test | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
|  | SCC | nX |  | Mid/CC2 |  | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
| 2 | PCC | nX |  | Mid/CC1 |  | CP-OFDM 16QAM | Outer_Full |
|  | SCC | nX |  | Mid/CC2 |  | CP-OFDM 16QAM | Outer_Full |
| 3 | PCC | nX |  | Mid/CC1 |  | CP-OFDM 64QAM | Outer_Full |
|  | SCC | nX |  | Mid/CC2 |  | CP-OFDM 64QAM | Outer_Full |

Below table lists proposed test points for examples of CA configuration with 2 UL CC and accumulative aggregated channel bandwidth <= 1400MHz.

| Test ID | CC | Band | ChBw(MHz) | Test frequency | DL RB allocation | UL Modulation | UL RB allocation |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Default Test Settings for a CA_nXB Configuration |  |  |  |  |  |  |  |
| 1 | PCC | nX | Default | Mid/CC1 | N/A for this test | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
|  | SCC | nX |  | Mid/CC2 |  | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
| 2 | PCC | nX |  | Mid/CC1 |  | DFT-s-OFDM QPSK | Outer_Full |
|  | SCC | nX |  | Mid/CC2 |  | DFT-s-OFDM QPSK | Outer_Full |
| 3 | PCC | nX |  | Mid/CC1 |  | DFT-s-OFDM 16QAM | Outer_Full |
|  | SCC | nX |  | Mid/CC2 |  | DFT-s-OFDM 16QAM | Outer_Full |
| 4 | PCC | nX |  | Mid/CC1 |  | CP-OFDM 16QAM | Outer_Full |
|  | SCC | nX |  | Mid/CC2 |  | CP-OFDM 16QAM | Outer_Full |
| 5 | PCC | nX |  | Mid/CC1 |  | CP-OFDM 64QAM | Outer_Full |
|  | SCC | nX |  | Mid/CC2 |  | CP-OFDM 64QAM | Outer_Full |
| Default Test Settings for a CA_nX(D-G), CA_nX(D-O) Configuration |  |  |  |  |  |  |  |
| 1 | PCC | nX | Default | Low/CC1 | N/A for this test | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
|  | SCC1 | nX |  | Low/CC2 |  | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
|  | Wgap | - |  | Max Wgap |  | N/A | N/A |
|  | SCC2 | nX |  | Low/CC3 |  | N/A | N/A |
| 2 | PCC | nX |  | High/CC4 |  | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
|  | SCC1 | nX |  | High/CC3 |  | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
|  | Wgap | - |  | Max Wgap |  | N/A | N/A |
|  | SCC2 | nX |  | High/CC2 |  | N/A | N/A |
| 3 | PCC | nX |  | Low/CC1 |  | DFT-s-OFDM QPSK | Outer_Full |
|  | SCC1 | nX |  | Low/CC2 |  | DFT-s-OFDM QPSK | Outer_Full |
|  | Wgap | - |  | Max Wgap |  | N/A | N/A |
|  | SCC2 | nX |  | Low/CC3 |  | N/A | N/A |
| 4 | PCC | nX |  | High/CC4 |  | DFT-s-OFDM QPSK | Outer_Full |
|  | SCC1 | nX |  | High/CC3 |  | DFT-s-OFDM QPSK | Outer_Full |
|  | Wgap | - |  | Max Wgap |  | N/A | N/A |
|  | SCC2 | nX |  | High/CC2 |  | N/A | N/A |
| 5 | PCC | nX |  | Low/CC1 |  | DFT-s-OFDM 16QAM | Outer_Full |
|  | SCC1 | nX |  | Low/CC2 |  | DFT-s-OFDM 16QAM | Outer_Full |
|  | Wgap | - |  | Max Wgap |  | N/A | N/A |
|  | SCC2 | nX |  | Low/CC3 |  | N/A | N/A |
| 6 | PCC | nX |  | High/CC4 |  | DFT-s-OFDM 16QAM | Outer_Full |
|  | SCC1 | nX |  | High/CC3 |  | DFT-s-OFDM 16QAM | Outer_Full |
|  | Wgap | - |  | Max Wgap |  | N/A | N/A |
|  | SCC2 | nX |  | High/CC2 |  | N/A | N/A |
| 7 | PCC | nX |  | Low/CC1 |  | CP-OFDM 16QAM | Outer_Full |
|  | SCC1 | nX |  | Low/CC2 |  | CP-OFDM 16QAM | Outer_Full |
|  | Wgap | - |  | Max Wgap |  | N/A | N/A |
|  | SCC2 | nX |  | Low/CC3 |  | N/A | N/A |
| 8 | PCC | nX |  | High/CC4 |  | CP-OFDM 16QAM | Outer_Full |
|  | SCC1 | nX |  | High/CC3 |  | CP-OFDM 16QAM | Outer_Full |
|  | Wgap | - |  | Max Wgap |  | N/A | N/A |
|  | SCC2 | nX |  | High/CC2 |  | N/A | N/A |
| 9 | PCC | nX |  | Low/CC1 |  | CP-OFDM 64QAM | Outer_Full |
|  | SCC1 | nX |  | Low/CC2 |  | CP-OFDM 64QAM | Outer_Full |
|  | Wgap | - |  | Max Wgap |  | N/A | N/A |
|  | SCC2 | nX |  | Low/CC3 |  | N/A | N/A |
| 10 | PCC | nX |  | High/CC4 |  | CP-OFDM 64QAM | Outer_Full |
|  | SCC1 | nX |  | High/CC3 |  | CP-OFDM 64QAM | Outer_Full |
|  | Wgap | - |  | Max Wgap |  | N/A | N/A |
|  | SCC2 | nX |  | High/CC2 |  | N/A | N/A |

Test requirement ID 7 depends on non-contiguous allocation. The most critical test point would be only 1 RB at both ends of the UL aggregated channel bandwidth. In this case the value of 10*(NRB_alloc / NRB_agg_C) +  [14.4] would be higher than MPRC_CA , therefore CP-OFDM 64QAM can be used. For intra-band contiguous DL CA and intra-band contiguous UL CA, it’s proposed to test only Mid frequency. For intra-band non-contiguous DL CA and intra-band contiguous UL CA, both High and Low range are tested. This principle can apply to UL CA configurations of any UL CC number. For each applicable CA configuration 1~2 test points are verified.

In addition there is a need to verify the minimum MPR for non-contiguous allocation. If the RB allocation satisfies NRB_alloc / NRB_agg_C =0.9, then MPRC_CA would apply. DFT-s-OFDM Pi/2 BPSK and DFT-s-OFDM QPSK are selected to get the minimum MPRC_CA value.

Proposal 11: To verify test requirement ID 7, it is proposed to schedule only 1 RB with CP-OFDM 64QAM at both ends of the UL aggregated channel bandwidth, and schedule non-contiguous RB allocation satisfying 1 > NRB_alloc / NRB_agg_C >=0.9 with DFT-s-OFDM Pi/2 BPSK and DFT-s-OFDM QPSK. For intra-band contiguous DL CA and intra-band contiguous UL CA, it’s proposed to test only Mid frequency. For intra-band non-contiguous DL CA and intra-band contiguous UL CA, both High and Low range are tested. This principle can apply to UL CA configurations of any UL CC number.

Below table lists proposed test points for examples of CA configuration with 2 UL CC.

| Test ID | CC | Band | ChBw(MHz) | Test frequency | DL RB allocation | UL Modulation | UL RB allocation |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Default Test Settings for a CA_nXB, CA_nXD, CA_XG, CA_nXO Configuration |  |  |  |  |  |  |  |
| 1 | PCC | nX | Default | Mid/CC1 | N/A for this test | CP-OFDM 64QAM | Outer_1RB_Left |
|  | SCC | nX |  | Mid/CC2 |  | CP-OFDM 64QAM | Outer_1RB_Right |
| 2 | PCC | nX |  | Mid/CC1 |  | DFT-s-OFDM Pi/2 BPSK | [Outer_0.9_Left] |
|  | SCC | nX |  | Mid/CC2 |  | DFT-s-OFDM Pi/2 BPSK | [Outer_0.9_Right] |
| Default Test Settings for a CA_nX(D-G), CA_nX(D-O) Configuration |  |  |  |  |  |  |  |
| 1 | PCC | nX | Default | Low/CC1 | N/A for this test | CP-OFDM 64QAM | Outer_1RB_Left |
|  | SCC1 | nX |  | Low/CC2 |  | CP-OFDM 64QAM | Outer_1RB_Right |
|  | Wgap | - |  | Max Wgap |  | N/A | N/A |
|  | SCC2 | nX |  | Low/CC3 |  | N/A | N/A |
| 2 | PCC | nX |  | High/CC4 |  | CP-OFDM 64QAM | Outer_1RB_Right |
|  | SCC1 | nX |  | High/CC3 |  | CP-OFDM 64QAM | Outer_1RB_Left |
|  | Wgap | - |  | Max Wgap |  | N/A | N/A |
|  | SCC2 | nX |  | High/CC2 |  | N/A | N/A |
| 3 | PCC | nX |  | Low/CC1 |  | DFT-s-OFDM Pi/2 BPSK | [Outer_0.9_Left] |
|  | SCC1 | nX |  | Low/CC2 |  | DFT-s-OFDM Pi/2 BPSK | [Outer_0.9_Right] |
|  | Wgap | - |  | Max Wgap |  | N/A | N/A |
|  | SCC2 | nX |  | Low/CC3 |  | N/A | N/A |
| 4 | PCC | nX |  | High/CC4 |  | DFT-s-OFDM Pi/2 QPSK | [Outer_0.9_Left] |
|  | SCC1 | nX |  | High/CC3 |  | DFT-s-OFDM Pi/2 QPSK | [Outer_0.9_Right] |
|  | Wgap | - |  | Max Wgap |  | N/A | N/A |
|  | SCC2 | nX |  | High/CC2 |  | N/A | N/A |

### 2.6.2 Power class 2, 3 and 4

The MPR requirements for Power class 2,3 and 4 can be summarized as below table. In order to ensure the testing coverage, the test points are selected for each test requirement ID. Compared with Power class 1, the requirements of MPRnarrow is not defined. The principle of test points selection for other requirements can be reused. The only exception is test requirement ID 5. In order to make sure the non-contiguous MPR requirement apply instead of MPRC_CA, the modulation type is selected as DFT-s-OFDM QPSK.

Proposal 12: To verify MPR for Power Class 2, 3 and 4, the principle of test points selection of Power Class 1 can be reused, except the modulation type of non-contiguous allocation testing is replaced by DFT-s-OFDM QPSK.

| Test requirement ID | Allocation type | Cumulative aggregated channel bandwidth | RB allocated in single CC or multiple CCs | MPR requirement |
| --- | --- | --- | --- | --- |
| 1 | contiguous allocation: MPRC_CA | Cumulative aggregated channel bandwidth <= 400MHz | contiguous RB allocation in a single CC (DFT-s-OFDM BPSK or QPSK) | single CC MPR of subclause 6.2.2.3 |
| 2 |  |  | RB allocation in multiple CCs | MPR in 6.2A.2.4-1 |
| 3 |  | Cumulative aggregated channel bandwidth < 800MHz |  | MPR in 6.2A.2.4-1 |
| 4 |  | Cumulative aggregated channel bandwidth < 1400MHz |  | MPR in 6.2A.2.4-1 |
| 5 | Non-contiguous allocation |  |  | Max(MPRC_CA, -10*(NRB_alloc / NRB_agg_C) + 7.0]) |

### 2.6.3 Exceptions to support PHR method

As discussed in [2], PHR cannot be received for the test IDs using just 1RB allocation and modulation equal or smaller than QPSK. In order to resolve the issue, for the cases of 2UL CA, in case of PI/2 BPSK it is proposed to use 3RBs instead of 1RB and, in case of QPSK, it is proposed to use 2RBs instead of 1RB.

Proposal 13: For 2 UL CA, for Rel-15 UEs, for the 1RB allocations proposed in previous tables, in case of PI/2 BPSK use 3RBs instead of 1RB and, in case of QPSK, use 2RBs instead of 1RB. Higher orders of UL CA are FFS.

# Conclusion

It is proposed that RAN5 discusses and agrees the following proposals for open areas described in section 2 of this document to progress on SA FR2 MPR for CA test case definition:

Proposal 1: Test UL CA MPR with maximum accumulative aggregated bandwidth by configuring one extra DL CC with as high Wgap as possible in non-contiguous CA mode, in addition to the case of same number of DL CCs and UL CCs.

Proposal 2: Define Test Environment as Normal Conditions (NC) for MPR for CA measurement in FR2.

Proposal 3: Select Low range and High range test frequencies for MPR for intra-band contiguous DL and UL CA in FR2.

Proposal 4: Select Low range and High range test frequencies for MPR for intra-band non-contiguous DL CA and contiguous UL CA in FR2.

Proposal 5: Select 120 kHz subcarrier spacing for MPR for CA in FR2.

Proposal 6: Select Highest NRB_agg for MPR for CA in FR2. In the case this NRB_agg cannot be tested due to testability problems, then the maximum testable NRB_agg is selected

Proposal 7: Select the most critical waveform and modulation order for the test of the same MPR requirement, which is, if the waveform is same, select the max modulation order, if the modulation order is same, select CP-OFDM waveform. For DFT-s-OFDM with high modulation order and CP-OFDM with low modulation order, since it is hard to say which is more critical, both should be tested.

Proposal 8: To verify Power Class 1 test requirement ID 1 and 2, for either intra-band contiguous DL CA and intra-band contiguous UL CA, or intra-band non-contiguous DL CA and intra-band contiguous UL CA, it is proposed to schedule the UL RBs at the very edges of the cumulative aggregated channel bandwidth. This principle can apply to UL CA configurations of any UL CC number.

Proposal 9: To verify Power Class 1 test requirement ID 3, it is proposed to reuse test points for single CC MPR testing in PCC only. This principle can apply to UL CA configurations of any UL CC number. The test requirement ID 3 only applies to intra-band contiguous CA configuration.

Proposal 10: To verify Power Class 1 test requirement ID 4, 5 and 6, it is proposed to schedule full allocation in all the UL CCs with modulation types in Table 6.2A.2.4-1. For intra-band contiguous DL CA and intra-band contiguous UL CA, it’s proposed to test only Mid frequency. For intra-band non-contiguous DL CA and intra-band contiguous UL CA, both High and Low range are tested. This principle can apply to UL CA configurations of any UL CC number.

Proposal 11: To verify test requirement ID 7, it is proposed to schedule only 1 RB with CP-OFDM 64QAM at both ends of the UL aggregated channel bandwidth, and schedule non-contiguous RB allocation satisfying 1 > NRB_alloc / NRB_agg_C >=0.9 with DFT-s-OFDM Pi/2 BPSK and DFT-s-OFDM QPSK. For intra-band contiguous DL CA and intra-band contiguous UL CA, it’s proposed to test only Mid frequency. For intra-band non-contiguous DL CA and intra-band contiguous UL CA, both High and Low range are tested. This principle can apply to UL CA configurations of any UL CC number.

Proposal 12: To verify MPR for Power Class 2, 3 and 4, the principle of test points selection of Power Class 1 can be reused, except the modulation type of non-contiguous allocation testing is replaced by DFT-s-OFDM QPSK.

Proposal 13: For 2 UL CA, for Rel-15 UEs, for the 1RB allocations proposed in previous tables, in case of PI/2 BPSK use 3RBs instead of 1RB and, in case of QPSK, use 2RBs instead of 1RB. Higher orders of UL CA are FFS.

# References

[1] 3GPP TS 38.101-1 v15.6.0.

[2] R5-R5-231322, “Limitation on PHR method to avoid Scell drop”, Keysight Technologies, RAN5#98