3GPP TSG-RAN WG5 Meeting #89-e

# Electronic Meeting, 9th Nov – 20th Nov 2020

Source: 	KTL, NTT DOCOMO INC.

Title: 	Discussion on test point selection for ACLR for CA

# Introduction

The purpose of this contribution is to provide the complete analysis for each parameter included in the Test Configuration Table and propose test points selection for FR2 test case Adjacent channel leakage ratio for intra-band contiguous CA (Aggregated BW ≤ 400MHz).

# Discussion

Test Configuration for ACLR for FR2 single carrier is defined section in 38.521-2 [1] as follows:

Table 6.5.2.3.4.1-1: Test Configuration Table

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Lowest, Mid, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | Lowest, Highest |  |
| Test Parameters |  |  |  |  |  |  |
| Test ID | Freq | ChBw | SCS | Downlink Configuration | Uplink Configuration |  |
|  |  | Default | Default | N/A for Adjacent Channel Leakage Ratio test case | Modulation | RB allocation (NOTE 1) |
| 1 | Low |  |  |  | DFT-s-OFDM PI/2 BPSK | Outer_1RB_Left |
| 2 | High |  |  |  | DFT-s-OFDM PI/2 BPSK | Outer_1RB_Right |
| 3 | Default |  |  |  | DFT-s-OFDM PI/2 BPSK | Outer_Full |
| 4 | Low |  |  |  | DFT-s-OFDM QPSK | Outer_1RB_Left |
| 5 | High |  |  |  | DFT-s-OFDM QPSK | Outer_1RB_Right |
| 6 | Default |  |  |  | DFT-s-OFDM QPSK | Outer_Full |
| 7 | Low |  |  |  | DFT-s-OFDM 16 QAM | Outer_1RB_Left |
| 8 | High |  |  |  | DFT-s-OFDM 16 QAM | Outer_1RB_Right |
| 9 | Default |  |  |  | DFT-s-OFDM 16 QAM | Outer_Full |
| 10 | Default |  |  |  | DFT-s-OFDM 64 QAM | Outer_Full |
| 11 | Low |  |  |  | CP-OFDM QPSK | Outer_1RB_Left |
| 12 | High |  |  |  | CP-OFDM QPSK | Outer_1RB_Right |
| 13 | Default |  |  |  | CP-OFDM QPSK | Outer_Full |
| NOTE 1: The specific configuration of each RF allocation is defined in Table 6.1-1 for PC2, PC3 and PC4 or Table 6.1-2 for PC1. NOTE 2: Following Test IDs shall be skipped for FR2b - All Test IDs for 400MHz Channel Bandwidth - Test ID 10-13 for FR2b 200MHz Channel Bandwidth - Test ID 10 for FR2b 100MHz Channel Bandwidth |  |  |  |  |  |  |

Test Configuration for ACLR for UL intra-band contiguous CA in LTE is defined section in 36.521-1 [2] as follows:

Table 6.6.2.3A.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  |  | Low range, High range |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration |  |  |  |  | Lowest NRB_agg, Highest NRB_agg (Note 2) |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |
| CA Configuration / NRB_agg |  |  | DL Allocation | CC MOD | UL Allocation |  |  |  |  |
| ID | PCC NRB | SCCs NRB | PCC & SCC RB allocation |  | NRB_alloc | PCC & SCC RB allocations(LCRB @ RBstart) |  |  |  |
| 1 | 100 | 25 | N/A for this test | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 2 | 100 | 25 |  | QPSK | 25 | P_25@0 | S_0@0 | - | - |
| 3 | 100 | 25 |  | QPSK | 125 | P_100@0 | S_25@0 | - | - |
| 4 | 100 | 25 |  | 16QAM | 8 | P_8@0 | S_0@0 | - | - |
| 5 | 100 | 25 |  | 16QAM | 25 | P_25@0 | S_0@0 | - | - |
| 6 | 100 | 25 |  | 16QAM | 125 | P_100@0 | S_25@0 | - | - |
| 7 | 100 | 25 |  | QPSK | 2 | P_1@0 | S_1@24 | - | - |
| 8 | 75 | 25 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 9 | 75 | 25 |  | QPSK | 25 | P_25@0 | S_0@0 | - | - |
| 10 | 75 | 25 |  | QPSK | 100 | P_75@0 | S_25@0 | - | - |
| 11 | 75 | 25 |  | 16QAM | 8 | P_8@0 | S_0@0 | - | - |
| 12 | 75 | 25 |  | 16QAM | 25 | P_25@0 | S_0@0 | - | - |
| 13 | 75 | 25 |  | 16QAM | 100 | P_75@0 | S_25@0 | - | - |
| 14 | 75 | 25 |  | QPSK | 2 | P_1@0 | S_1@24 | - | - |
| 15 | 75 | 75 |  | QPSK | 16 | P_16@0 | S_0@0 | - | - |
| 16 | 75 | 75 |  | QPSK | 75 | P_75@0 | S_0@0 | - | - |
| 17 | 75 | 75 |  | QPSK | 150 | P_75@0 | S_75@0 | - | - |
| 18 | 75 | 75 |  | 16QAM | 16 | P_16@0 | S_0@0 | - | - |
| 19 | 75 | 75 |  | 16QAM | 75 | P_75@0 | S_0@0 | - | - |
| 20 | 75 | 75 |  | 16QAM | 150 | P_75@0 | S_75@0 | - | - |
| 21 | 75 | 75 |  | QPSK | 2 | P_1@0 | S_1@74 | - | - |
| 22 | 100 | 50 |  | QPSK | 12 | P_12@0 | S_0@0 | - | - |
| 23 | 100 | 50 |  | QPSK | 50 | P_50@0 | S_0@0 | - | - |
| 24 | 100 | 50 |  | QPSK | 150 | P_100@0 | S_50@0 | - | - |
| 25 | 100 | 50 |  | 16QAM | 12 | P_12@0 | S_0@0 | - | - |
| 26 | 100 | 50 |  | 16QAM | 50 | P_50@0 | S_0@0 | - | - |
| 27 | 100 | 50 |  | 16QAM | 150 | P_100@0 | S_50@0 | - | - |
| 28 | 100 | 50 |  | QPSK | 2 | P_1@0 | S_1@49 | - | - |
| 29 | 100 | 75 |  | QPSK | 16 | P_16@0 | S_0@0 | - | - |
| 30 | 100 | 75 |  | QPSK | 75 | P_75@0 | S_0@0 | - | - |
| 31 | 100 | 75 |  | QPSK | 175 | P_100@0 | S_75@0 | - | - |
| 32 | 100 | 75 |  | 16QAM | 16 | P_16@0 | S_0@0 | - | - |
| 33 | 100 | 75 |  | 16QAM | 75 | P_75@0 | S_0@0 | - | - |
| 34 | 100 | 75 |  | 16QAM | 175 | P_100@0 | S_75@0 | - | - |
| 35 | 100 | 75 |  | QPSK | 2 | P_1@0 | S_1@74 | - | - |
| 36 | 100 | 100 |  | QPSK | 18 | P_18@0 | S_0@0 | - | - |
| 37 | 100 | 100 |  | QPSK | 100 | P_100@0 | S_0@0 | - | - |
| 38 | 100 | 100 |  | QPSK | 200 | P_100@0 | S_100@0 | - | - |
| 39 | 100 | 100 |  | 16QAM | 18 | P_18@0 | S_0@0 | - | - |
| 40 | 100 | 100 |  | 16QAM | 100 | P_100@0 | S_0@0 | - | - |
| 41 | 100 | 100 |  | 16QAM | 200 | P_100@0 | S_100@0 | - | - |
| 42 | 100 | 100 |  | QPSK | 2 | P_1@0 | S_1@99 | - | - |
| 43 | 25 | 25 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 44 | 25 | 25 |  | QPSK | 25 | P_25@0 | S_0@0 | - | - |
| 45 | 25 | 25 |  | QPSK | 50 | P_25@0 | S_25@0 | - | - |
| 46 | 25 | 25 |  | 16QAM | 8 | P_8@0 | S_0@0 | - | - |
| 47 | 25 | 25 |  | 16QAM | 25 | P_25@0 | S_0@0 | - | - |
| 48 | 25 | 25 |  | 16QAM | 50 | P_25@0 | S_25@0 | - | - |
| 49 | 25 | 25 |  | QPSK | 2 | P_1@0 | S_1@24 | - | - |
| 50 | 50 | 25 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 51 | 50 | 25 |  | QPSK | 25 | P_25@0 | S_0@0 | - | - |
| 52 | 50 | 25 |  | QPSK | 75 | P_25@0 | S_50@0 | - | - |
| 53 | 50 | 25 |  | 16QAM | 8 | P_8@0 | S_0@0 | - | - |
| 54 | 50 | 25 |  | 16QAM | 25 | P_25@0 | S_0@0 | - | - |
| 55 | 50 | 25 |  | 16QAM | 75 | P_25@0 | S_50@0 | - | - |
| 56 | 50 | 25 |  | QPSK | 2 | P_1@0 | S_1@49 | - | - |
| 57 | 50 | 50 |  | QPSK | 12 | P_12@0 | S_0@0 | - | - |
| 58 | 50 | 50 |  | QPSK | 50 | P_50@0 | S_0@0 | - | - |
| 59 | 50 | 50 |  | QPSK | 100 | P_50@0 | S_50@0 | - | - |
| 60 | 50 | 50 |  | 16QAM | 12 | P_12@0 | S_0@0 | - | - |
| 61 | 50 | 50 |  | 16QAM | 50 | P_50@0 | S_0@0 | - | - |
| 62 | 50 | 50 |  | 16QAM | 100 | P_50@0 | S_50@0 | - | - |
| 63 | 50 | 50 |  | QPSK | 2 | P_1@0 | S_1@49 | - | - |
| 64 | 50 | 25 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 65 | 50 | 25 |  | QPSK | 50 | P_50@0 | S_0@0 | - | - |
| 66 | 50 | 25 |  | QPSK | 75 | P_50@0 | S_25@0 | - | - |
| 67 | 50 | 25 |  | 16QAM | 8 | P_8@0 | S_0@0 | - | - |
| 68 | 50 | 25 |  | 16QAM | 50 | P_50@0 | S_0@0 | - | - |
| 69 | 50 | 25 |  | 16QAM | 75 | P_50@0 | S_25@0 | - | - |
| 70 | 50 | 25 |  | QPSK | 2 | P_1@0 | S_1@24 | - | - |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1. Note 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg , only the combination with the highest NRB_PCC is tested. |  |  |  |  |  |  |  |  |  |

## Test Environment

Test environment Adjacent channel leakage ratio measurement for FR2 Single Carrier in [1] is Normal Conditions. This test environment can be also leveraged for FR2 CA.

Proposal 1: Define Test Environment as Normal Conditions (NC) for Adjacent channel leakage ratio measurement for intra-band contiguous CA.

## Test Frequencies

As mentioned in [3], Low and High frequencies can cover the most critical situations with the frequency variation obtaining a timing reduction in test as well. This criteria is even applied for this kind of bands which are really wide.

This section pretends to analyse the Test Frequencies test points which should be tested in Adjacent Channel Leakage Ratio to evaluate the effects of a possible non-linearity in the channel bandwidth response for different frequencies inside the band.

Therefore, in ACLR for CA FR2, similar test frequencies shall be adopted as for FR2 single carrier.

Proposal 2: Test Low and High Frequencies for FR2 Adjacent Channel Leakage Ratio CA test.

## Test Channel Bandwidth

According to [3], testing on Lowest and Highest channel bandwidth would be enough for test coverage at the most critical situations for FR2 Adjacent Channel Leakage Ratio single carrier. LTE djacent Channel Leakage Ratio in 36.521-1[2] for intra-band contiguous CA is tested on the Lowest and Highest NRB_agg.

In FR2, since the lowest BW is considered tested in single carrier configuration. Hence the precedence is to test only Maximum aggregated BW.

Proposal 3: Test Highest channel bandwidths for ACLR intra-band contiguous CA.

## Test Subcarrier Spacing

According to [3], the worst situation is test in conditions at high frequency and lower SCS and it seems to be reasonable to test the Highest SCS supported by UE Based on the number of secondary lobs in guardband.

Proposal 4: Select the Lowest and Highest Subcarrier Spacing to test for FR2 Adjacent Channel Leakage Ratio intra-band contiguous CA.

## Modulations

According to [3], the testing for both types of modulations would be required due to the different behaviour between DFT-s-OFDM and CP-OFDM modulations based on out of band emission requirement dependency.

Proposal 5: Test the following UL modulations supported by UE: DFT-s-OFDM PI/2 BPSK, DFT-s-OFDM QPSK, DFT-s-OFDM 16 QAM, DFT-s-OFDM 64 QAM, CP-OFDM QPSK,) for FR2 Adjacent Channel Leakage Ratio intra-band contiguous CA.

## Uplink configuration

According to [3], Focused on Outer resource allocations: Only the full allocation is considered. Based on baseline defined in LTE for the UL CA test points, some of most critical situations are related to allocate partial resource blocks in the edge of the transmitter bandwidth. It would seem to be necessary to consider some critical test points to test, for NR Spectrum Emission Mask in FR2, which would cover either 1 RB allocation with start 0 or 1 RB allocation with start LCRBmax – 1.

Proposal 6: Test Outer full RB allocations for NR Adjacent Channel Leakage Ratio in FR2 but adding new test points to cover 1 RB allocation with start 0 and 1 RB allocation with start LCRBmax – 1.

### Exceptions to support PHR method

As discussed in [2], PHR cannot be received for the test IDs using just 1RB allocation and modulation equal or smaller than QPSK. In order to resolve the issue, for the cases of 2UL CA, in case of PI/2 BPSK it is proposed to use 3RBs instead of 1RB and, in case of QPSK, it is proposed to use 2RBs instead of 1RB.

Proposal 7: For 2 UL CA, for Rel-15 UEs, for the 1RB allocations proposed in previous sections, in case of PI/2 BPSK use 3RBs instead of 1RB and, in case of QPSK, use 2RBs instead of 1RB. Higher orders of UL CA are FFS.

# Conclusion

In conclusion, the following selection of test points are proposed SEM for CA FR2 :

Proposal 1: Define Test Environment as Normal Conditions (NC) for Adjacent channel leakage ratio measurement for intra-band contiguous CA.

Proposal 2: Test Low and High Frequencies for FR2 Adjacent Channel Leakage Ratio CA test.

Proposal 3: Test Highest channel bandwidths for ACLR intra-band contiguous CA.

Proposal 4: Select the Lowest and Highest Subcarrier Spacing to test for FR2 Adjacent Channel Leakage Ratio intra-band contiguous CA.

Proposal 5: Test the following UL modulations supported by UE: DFT-s-OFDM PI/2 BPSK, DFT-s-OFDM QPSK, DFT-s-OFDM 16 QAM, DFT-s-OFDM 64 QAM, CP-OFDM QPSK,) for FR2 Adjacent Channel Leakage Ratio intra-band contiguous CA.

Proposal 6: Test Outer full RB allocations for NR Adjacent Channel Leakage Ratio in FR2 but adding new test points to cover 1 RB allocation with start 0 and 1 RB allocation with start LCRBmax – 1.

Proposal 7: For 2 UL CA, for Rel-15 UEs, for the 1RB allocations proposed in previous sections, in case of PI/2 BPSK use 3RBs instead of 1RB and, in case of QPSK, use 2RBs instead of 1RB. Higher orders of UL CA are FFS.

# Number of test points

Along this document several proposals have been stated for different configuration parameters. In all of cases the analysis has been focused on reducing testing time without losing coverage in UE testing for NR.

| Environmental conditions | Maximum Number of Frequencies | Maximum Number of ChBW | Number SCS | Number of steps (mod and RB) | Maximum Number of Test Steps |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 1 | 2 | 13 | 52 |

# References

[1] TS 38.521-2	: “3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 2: Range 2 standalone (Release 16) (v16.5.0)”

[2] TS 36.521-1	: “Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment UE) conformance specification; Radio transmission and reception; Part 1: Conformance Testing”

[3] TR 38.905	: “NR; Derivation of test points for radio transmission and reception conformance test cases”, R5-182030, 38.521-2_TPanalysis_6.5.2.3_ACLR.zip, “Discussion on test point selection for NR ACLR in FR2”, Keysight Technologies (Release 16)

[4] R5-R5-231322, “Limitation on PHR method to avoid Scell drop”, Keysight Technologies, RAN5#98