Title:	Discussion on test point selection for FR2 Spurious emission coex CA test case

Document for:	Discussion and approval

# Introduction

This contribution provides test point analysis for FR2 Spurious emission coex CA test case.

# Discussion

In NR specification TS 38.521-2, the Test configuration for spurious emission coex for single carrier mode is defined as table below.

Table 6.5A.3.1.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  | Low range, High range (NOTE 2) |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  | Highest aggregated BW of the CA configuration |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  | 120kHz |  |
| Test Parameters |  |  |  |  |  |
| Test ID | CC | Downlink Configuration | UL Modulation |  | UL RB allocation (NOTE 1) |
| 1 | PCC | N/A for Spurious Emissions testing | DFT-s-OFDM QPSK |  | Outer_Full |
|  | SCCs |  | DFT-s-OFDM QPSK |  | Outer_Full |
| 2 | PCC |  | DFT-s-OFDM QPSK |  | Inner_1RB for PC2, PC3 and PC4 Inner_Partial for PC1 (NOTE 3) |
|  | SCCs |  | DFT-s-OFDM QPSK |  | Inner_1RB for PC2, PC3 and PC4 Inner_Partial for PC1 |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 for PC2, PC3 and PC4 or Table 6.1-2 for PC1. NOTE 2: When testing Low range test only in Frequency Range lower than (FUL_low – ΔfOOB) and when testing High range test only in Frequency Range higher than (FUL_high + ΔfOOB). NOTE 3: When testing Low range configure uplink RB to Inner_1RB_Left for PC2, PC3 and PC4 or Inner_Partial_Left_Region1 for PC1 and when testing High range configure uplink RB to Inner_1RB_Right for PC2, PC3 and PC4 or Inner_Partial_Right_Region1 for PC1. |  |  |  |  |  |

In following sub-clauses, study for test environment, frequencies, bandwidth and subcarrier spacing and DL and UL allocation is provided based on above baseline.

## Test Environment

It is proposed to reuse the FR2 spurious emission coex single carrier test environment for FR2 spurious emission coex CA test cases.

Proposal 1: Define test environment as NC for FR2 spurious emission coex CA test cases.

## Test Subcarrier Spacing

It is proposed to reuse the FR2 spurious emission coex single carrier SCS for FR2 spurious emission CA test cases. The SCS has the same effect on spurious emission profile irrespective signal carrier or CA being tested. Testing only 120kHz SCS provides necessary coverage.

Proposal 2: Select 120kHz SCS for FR2 spurious emission coex CA test cases.

## Test Frequencies

Some bands only require to be tested for High range.  So, depending on test config choose to test FR2 coex CA for both Low range and High range or just for High range. This can be captured as a note (see NOTE 4 in the proposed test config table below) in the test config table.

Proposal 3: Test for Low range and/or High range frequencies as applicable for FR2 spurious emission coex CA test cases.

## Test Channel Bandwidth

It is proposed to test only highest aggregated bandwidth as this is worst case and in line with configuration chosen for all other FR2 coex CA test cases.

Proposal 4: Test for Highest aggregated bandwidth for the coex CA configuration for FR2 spurious emission coex CA test cases.

## Test Modulations and waveforms

It is proposed to reuse the FR2 spurious emission coex single carrier modulation and waveforms for FR2 spurious emission coex CA test cases as they remain the worst case irrespective of single carrier or CA.

Proposal 5: Test for DFT-s-OFDM QPSK in for FR2 spurious emission coex CA test cases.

## RB allocation

It is proposed to choose the RB allocation in line with approach taken for FR1 spurious emission CA test case as well as approach taken for FR2 MOP CA as well as for FR2 spurious emission single carrier test cases to ensure lowest MPR is tested.  Snippet below from 38.521-2 -> 6.2A.2 defining MPR values is provided below for reference:

### 38.521-2 -> 6.2A.2	UE maximum output power reduction for CA

#### 6.2A.2.0	Minimum conformance requirements

##### 6.2A.2.0.1	General

The UE is defined to be configured for CA operation when it has at least one of UL or DL configured for CA. In CA operation, the UE may reduce its maximum output power due to higher order modulations and transmit bandwidth configurations. This Maximum Power Reduction (MPR) is defined in subclauses below.

The cumulative aggregated channel bandwidth is defined as the frequency band from the lowest edge of the lowest CC to the upper edge of the highest CC of all UL and DL configured CCs. When the maximum output power of a UE is modified by MPR, the power limits specified in subclause 6.2A.4.0 apply.

The requirements in the following subclauses are only applicable to intra-band contiguous uplink CA, with the aggregated channel bandwidth up to 800 MHz.

##### 6.2A.2.0.2	Maximum output power reduction for power class 1

For power class 1, MPR for UL contiguous allocations within the cumulative aggregated bandwidth is defined as:

MPRC_CA = max(MPRWT_C_CA, MPRnarrow)

Where,

MPRnarrow = 14.4 dB, when BWalloc,RB is less than or equal to 1.44 MHz, MPRnarrow = 10 dB, when 1.44 MHz < BWalloc,RB ≤ 10.8 MHz, where BWalloc,RB is the bandwidth of the RB allocation size.

MPRWT_C_CA is the maximum power reduction due to modulation orders, transmit bandwidth configurations, and waveform types. MPRWT_contiguous is defined in Table 6.2A.2.2-1.

Table 6.2A.2.0-1: Maximum power reduction (MPRWT_C_CA) for UE power class 1

| Waveform Type |  | Cumulative aggregated channel bandwidth |  |  |
| --- | --- | --- | --- | --- |
|  |  | < 400 MHz | ≥ 400 MHz and < 800 MHz | ≥ 800 MHz and ≤ 1400 MHz |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 5.51 | 7.71 | [8.2] |
|  | QPSK | ≤ 6.51 | 8.71 | [9.7] |
|  | 16 QAM | ≤ 6.5 | 8.7 | [9.2] |
|  | 64 QAM | ≤ 9.0 | 10.7 | [11.2] |
| CP-OFDM | QPSK | ≤ 6.5 | 8.7 | [8.7] |
|  | 16 QAM | ≤ 6.5 | 8.7 | [8.7] |
|  | 64 QAM | ≤ 9.0 | 10.7 | [11.2] |
| NOTE 1: The following condition applies only when the cumulative aggregated BW of the CA configuration 400MHz. For a contiguous RB allocation in a single CC of the CA configuration, the single CC MPR of subclause 6.2.2.1 applies. The cumulative aggregated bandwidth shall be used as BWchannel in Tables 6.2.2.1-1 and 6.2.2.1-2. The applicable column in Tables 6.2.2.1-1 and 6.2.2.1-2 shall be determined based on the transmission bandwidth of the CA allocation in relation to the allocation regions defined in the tables. |  |  |  |  |

When different waveform types exist across CCs, the requirement is set by the waveform type used in the configuration with the largest MPRC_CA.

For non-contiguous RB allocations, the following rule for MPR applies:

MPR = max(MPRC_CA, -10*A + [14.4])

Where:

A = NRB_alloc / NRB_agg_C.

NRB_alloc is the total number of allocated UL RBs

NRB_agg_C is the number of the aggregated RBs within the fully allocated cumulative aggregated channel bandwidth

##### 6.2A.2.0.3	Maximum output power reduction for power class 2

For power class 2, MPR specified in sub-clause 6.2A.2.0.4 applies.

Table 6.2A.2.0.3-1: (Void)

##### 6.2A.2.0.4	Maximum output power reduction for power class 3

For power class 3, MPR for UL contiguous allocations within the cumulative aggregated bandwidth is denoted as MPRC_CA and is defined in Table 6.2A.2.0.4-1.

Table 6.2A.2.0.4-1: Maximum power reduction (MPRC_CA) for UE power class 3

|  |  | Cumulative aggregated bandwidth configuration |  |  |
| --- | --- | --- | --- | --- |
|  |  | ≤ 400 MHz | > 400 MHz and < 800 MHz | ≥ 800 MHz and ≤ 1400 MHz |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 5.01 | ≤ 7.71 | ≤ [8.2] |
|  | QPSK | ≤ 5.01 | ≤ 7.71 | ≤ [8.2] |
|  | 16 QAM | ≤ 6.5 | ≤ 8.7 | ≤ [9.3] |
|  | 64 QAM | ≤ 9.0 | ≤ 10.7 | ≤ [11.2] |
| CP-OFDM | QPSK | ≤ 5.0 | ≤ 7.5 | ≤ [8.0] |
|  | 16 QAM | ≤ 6.5 | ≤ 8.7 | ≤ [9.2] |
|  | 64 QAM | ≤ 9.0 | ≤ 10.7 | ≤ [11.2] |

When different waveform types exist across CCs, the requirement is set by the waveform type used in the configuration with the highest contiguous MPR.

For non-contiguous RB allocations, the following rule for MPR applies:

MPR = max(MPRC_CA, -10*A +7.0)

Where:

A = NRB_alloc / NRB_agg_C.

NRB_alloc is the total number of allocated UL RBs

NRB_agg_C is the number of the aggregated RBs within the fully allocated cumulative aggregated channel bandwidth

##### 6.2A.2.0.5	Maximum output power reduction for power class 4

For power class 4, MPR specified in sub-clause 6.2A.2.0.4 applies.

The normative reference for this requirement is TS 38.101-2 [3] clause 6.2A.2.

In general, the RB allocation chosen for MOP from 38.521-2 -> 6.2A.2 is also applicable for spurious emissions coex as well.  Based on the MPR equations below, it is observed that the lowest MPR for all power class is achieved when value of A = 1, which implies that Outer Full allocation. The choice of Outer Full allocation also aligns with the choice made for FR2 MOP CA from 38.521-2 -> 6.2A.2 for DFT-s-OFDM QPSK waveform and hence is chosen even for FR2 spurious coex CA.  For high PSD test point choose Inner 1RB for PC2/3/4 and choose Inner Partial RB for PC1 FR2 spurious coex CA testing as these result in lowest MPR and hence highest PSD.

Proposal 6: Test using Outer Full (for both PCC and SCC) and Inner 1RB/Partial for FR2 spurious emission coex CA test cases.

Based on the above proposals the test configuration table for FR2 spurious emission CA test case is as shown below:

Proposed -> Table 6.5A.3.1.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  | Low range and/or High range (NOTE 2, NOTE 4) |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  | Highest aggregated BW of the CA configuration |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  | 120kHz |  |
| Test Parameters |  |  |  |  |  |
| Test ID | CC | Downlink Configuration | UL Modulation |  | UL RB allocation (NOTE 1) |
| 1 | PCC | N/A for Spurious Emissions testing | DFT-s-OFDM QPSK |  | Outer_Full |
|  | SCCs |  | DFT-s-OFDM QPSK |  | Outer_Full |
| 2 | PCC |  | DFT-s-OFDM QPSK |  | Inner_1RB for PC2, PC3 and PC4 Inner_Partial for PC1 (NOTE 3) |
|  | SCCs |  | DFT-s-OFDM QPSK |  | Inner_1RB for PC2, PC3 and PC4 Inner_Partial for PC1 |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 for PC2, PC3 and PC4 or Table 6.1-2 for PC1. NOTE 2: When testing Low range test only in Frequency Range lower than (FUL_low – ΔfOOB) and when testing High range test only in Frequency Range higher than (FUL_high + ΔfOOB). NOTE 3: When testing Low range configure uplink RB to Inner_1RB_Left for PC2, PC3 and PC4 or Inner_Partial_Left_Region1 for PC1 and when testing High range configure uplink RB to Inner_1RB_Right for PC2, PC3 and PC4 or Inner_Partial_Right_Region1 for PC1. NOTE 4: For a FR2 band under test, if the protected band frequency range in Table 6.5A.3.2.0-1 is only lower or higher than the FR2 band under test then it is sufficient to test only Low range or High range, otherwise test at both Low range and High range. |  |  |  |  |  |

### 2.6.1 Exceptions to support PHR method

As discussed in [2], PHR cannot be received for the test IDs using just 1RB allocation and modulation equal or smaller than QPSK. In order to resolve the issue, for the cases of 2UL CA, in case of PI/2 BPSK it is proposed to use 3RBs instead of 1RB and, in case of QPSK, it is proposed to use 2RBs instead of 1RB.

Proposal 7: For 2 UL CA, for Rel-15 UEs, for the 1RB allocations proposed in previous sections, in case of PI/2 BPSK use 3RBs instead of 1RB and, in case of QPSK, use 2RBs instead of 1RB. Higher orders of UL CA are FFS.

# 2.7 Number of test points for CA spurious emission

| Environmental conditions | Maximum Number of Frequencies | Maximum Number of ChBW | Number SCS | Number of steps (mod and RB) | Maximum Number of Test points |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 1 | 1 | 2 | 4 |

# Conclusion

For FR2 spurious emission coex UL CA requirement, the test configuration proposals are as stated below:

Proposal 1: Define test environment as NC for FR2 spurious emission coex CA test cases.

Proposal 2: Select 120kHz SCS for FR2 spurious emission coex CA test cases.

Proposal 3: Test for Low range and/or High range frequencies as applicable for FR2 spurious emission coex CA test cases.

Proposal 4: Test for Highest aggregated bandwidth for the coex CA configuration for FR2 spurious emission coex CA test cases.

Proposal 5: Test for DFT-s-OFDM QPSK in for FR2 spurious emission coex CA test cases.

Proposal 6: Test using Outer Full (for both PCC and SCC) and Inner 1RB/Partial for FR2 spurious emission coex CA test cases.

Proposal 7: For 2 UL CA, for Rel-15 UEs, for the 1RB allocations proposed in previous sections, in case of PI/2 BPSK use 3RBs instead of 1RB and, in case of QPSK, use 2RBs instead of 1RB. Higher orders of UL CA are FFS.

# References

[2] R5-R5-231322, “Limitation on PHR method to avoid Scell drop”, Keysight Technologies, RAN5#98