Title: 	Discussion on test point selection for MPR, NR ACLR and SEM in FR1

Source: 	Keysight Technologies, Huawei, CAICT, Ericsson

# Introduction

A default baseline for test point selection for TRx test cases was presented in [1] at RAN5#1 5G NR Adhoc meeting, where some proposals for environmental conditions, test frequencies, channel bandwidths, subcarrier spacing and RB allocation for FR1 were based on LTE. Finally, it was stated that each test case would require specific analysis to conform its test configuration table based on the study for Frequency, Channel BW, SCS, RB allocation (including modulations).

The purpose of this contribution is to provide the complete analysis for each parameter included in the Test Configuration Table and propose test points selection for Maximum Power Reduction (MPR), Spectrum Emission mask (SEM) and Adjacent Channel Leakage Ratio (ACLR) tests in FR1. Firstly, single transmission case is done, then ACLR UL-MIMO case is provided with exception test points analysis based on the outcome from comparing the minimum requirements and waveforms allowed.

This paper tries to follow endorsed proposals in R5-197586 [6] in RAN5#84:

The baseline for this combined MPR, ACLR and SEM analysis is, as per 38.905 v16.3.0 [7]:

38.521-1_TPanalysis_6.2.2_MPR_v3.zip

38.521-1_TPanalysis_6.5.2.2_SEM_v3.zip

38.521-1_TPanalysis_6.5.2.4_ACLR_v3.zip

The above 3 papers will be replaced by this one, 38.521-1_TPanalysis_6.2.2_MPR_6.5.2.2_SEM_6.5.2.4.1_NR_ACLR_v1.zip

# Discussion

Following subclauses introduce study for test environment, test frequencies, test bandwidth, test subcarrier spacing and uplink configuration aspects.

## Test Environment

Based on the baseline agreed in [1], the test environment for the measurement for FR1 can be defined as in LTE, see table 2.1-1 below.

Table 2.1-1 Test environment configuration summary

| Test Case | Test Environment |
| --- | --- |
| MPR | Normal, TL/VL, TL/VH, TH/VL and TH/VH |
| ACLR | Normal,TL/VL, TL/VH, TH/VL and TH/VH |
| SEM | NC |

Proposal 1: Define Test Environment as per LTE.

## Test Subcarrier Spacing

The goal of this section is to expose the technical reasons to select the SCS to be tested. This study is focused to detect the most critical use cases, based on SCS selection, when the nominal operation mode of the UE is compromised in this test.

The following aspect have been considered:

Spectrum utilization and Minimum GuardBand

The maximum transmission bandwidth configuration NRB for each channel bandwidth and subcarrier spacing is specified in Table 2.2-1 (obtained from Table 5.3.2-1 in [2]).

Table 2.2-1 Maximum transmission bandwidth configuration NRB

| SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50MHz | 60 MHz | 80 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB |
| 15 | 25 | 52 | 79 | 106 | 133 | 160 | 216 | 270 | N/A | N/A | N/A |
| 30 | 11 | 24 | 38 | 51 | 65 | 78 | 106 | 133 | 162 | 217 | 273 |
| 60 | N/A | 11 | 18 | 24 | 31 | 38 | 51 | 65 | 79 | 107 | 135 |

The maximum transmission bandwidth (in MHz) per each channel bandwidth and SCS is calculated by using the values of the maximum number of Resource block and subcarrier spacing information (assuming 12 subcarriers per resource block), these values are shown in Table 2.2-2.

Table 2.2-2 Maximum transmission bandwidth configuration in MHz

| SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50MHz | 60 MHz | 80 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | 4.5 | 9.36 | 14.22 | 19.08 | 23.94 | 28.8 | 38.88 | 48.6 | N/A | N/A | N/A |
| 30 | 3.96 | 8.64 | 13.68 | 18.36 | 23.4 | 28.08 | 38.16 | 47.88 | 58.32 | 78.12 | 98.28 |
| 60 | N/A | 7.92 | 12.96 | 17.28 | 22.32 | 27.36 | 36.72 | 46.8 | 56.88 | 77.04 | 97.2 |

Besides Table 2.2-3 (taken from Table 5.3.3-1 in [2]), shows the minimum guardband for a UE channel bandwidth in each subcarrier spacing. This guardband is the frequency separation between the channel edge and the transmission bandwidth edge.

Table 2.2-3 Minimum Guardband for each UE channel bandwidth and SCS (kHz)

| SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50MHz | 60 MHz | 80 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | 242.5 | 312.5 | 382.5 | 452.5 | 522.5 | [592.5] | 552.5 | 692.5 | N/A | N/A | N/A |
| 30 | 505 | 665 | 645 | 805 | 785 | [945] | 905 | 1045 | 825 | 925 | 845 |
| 60 | N/A | 1010 | 990 | 1330 | 1310 | [1290] | 1610 | 1570 | 1530 | 1450 | 1370 |

Analyzing values in Table 2.2-3, the minimum guardband values are obtained for the lowest SCS (for each channel bandwidth).

Observation 1: Considering spectrum utilization and minimum guardband, it is reasonable to test the Lowest SCS.

Secondary lobes from subcarriers

Continuing the purpose to find the most critical situations for the UE related to the SCS selection other observation might be done.

In Table 2.2-3 the frequency range for the guardband is shown for each SCS and channel bandwith. However, if frequency response of each subcarrier is assumed as non-ideal, then this guardband might be studied in terms of how many secondary lobs would keep inside this guardband. This information gives us an estimation on how much power might go out the channel bandwidth due to the non-linearity of the transmitter.

Table 2.2-4 Number of secondary lobs are included in Guardband for each UE channel bandwidth and SCS (kHz) from last subcarrier

| SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50MHz | 60 MHz | 80 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | 16.16 | 20.83 | 25.5 | 30.16 | 34.83 | 39.5 | 36.83 | 46.16 | N/A | N/A | N/A |
| 30 | 16.83 | 22.16 | 21.5 | 26.83 | 26.16 | 31.5 | 30.16 | 34.83 | 27.5 | 30.83 | 28.16 |
| 60 | N/A | 16.83 | 16.5 | 22.16 | 21.83 | 21.5 | 26.83 | 26.16 | 25.5 | 24.16 | 22.83 |

Table 2.2-4 depicts the number of secondary lobs inside the guardband. The most of cases show that the highest SCS will have lower number of secondary lobs inside the guardband so, if they are not filtered by channel bandwidth filter, this power would be an unwanted spurious emission out of the channel.

Observation 2: Based on the number of secondary lobs in guardband, it seems reasonable to test the Highest SCS supported by UE for ACLR, SEM and MPR.

Proposal 2: Select the Lowest and Highest SCS supported for MPR, ACLR and SEM in FR1.

Test Subcarrier Spacing summary

Table 2.2-5 Test subcarrier spacing

| Test Case | Test Subcarrier Spacing |
| --- | --- |
| MPR | Lowest, Highest SCS |
| ACLR | Lowest, Highest SCS |
| SEM | Lowest, Highest SCS |

  - Test Frequencies

Adjacent Channel Leakage Ratio and Spectrum Emmission Mask are included inside the section Out of band emissions where it is pretended to evaluate the unwanted emissions immediately outside the nominal channel resulting from the modulation process and non-linearity in the transmitter.

This section pretends to analyse the Test Frequencies test points which should be tested in Adjacent Channel Leakage Ratio to evaluate the effects of a possible non-linearity in the channel bandwidth response for different frequencies inside the band.

In LTE, MPR, ACLR and SEM measurement is tested for Low, Mid and High frequencies. This criteria is reasonable to verify different behaviours of the UE transmitter for different frequencies due to non-linearity based on the frequency.

With the purpose to reduce testing time the following analysis is based on detecting the most critical situations, related with the non-linearity with frequency.

The UE filter design, which operates in a specific band, usually is based on a reference frequency to design the active components in its transmitter. In general, the most critical situations to operate are when the transmitter filter in UE is centered in frequencies far from the reference frequency.

The most critical cases based on the reference frequency for UE filters and the frequency to be tested are:

- UE implementation based on the Mid Frequency  The most far frequency would be Low and High, so this option would be covered in case to test Low or High frequency.

- UE implementation based on any extreme frequency (Low or High)  If testing is indicated for both Low and High, then the worst case would be tested as well. For instance: if the reference frequency is Low then the testing on High would be the most far frequency distant.

Observation 3: Low and High frequencies can cover the most critical situations with the frequency variation obtaining a timing reduction in test as well.

Proposal 3: Test Low and High Frequencies for MPR, ACLR and SEM in FR1.

Table 2.3-1 Test frequencies summary

| Test Case | Test Frequencies |
| --- | --- |
| MPR | Low, High |
| ACLR | Low, High |
| SEM | Low, High |

## Test Channel Bandwidth

### Contiguous RB allocation

In LTE, MPR, ACLR and SEM is tested for Lowest, 5 MHz, 10 MHz and Highest channel bandwidths. This assumption seems to be reasonable to test different behaviours of the UE transmitter related to the non-linearity with different channel bandwidth.

Based on it, MPR, ACLR and SEM for FR1 might be tested for Lower, 15 MHz, 20 MHz (more common bandwidth across all NR FR1 bands) and Highest.

Observation 4: Test the Lowest, 15 MHz, 20 MHz and Highest would cover all situations defined in LTE.

Considering Table 2.2-3 of this document, for a specific SCS, the channel bandwidth with narrowest guardband corresponds with the Lowest channel bandwidth. This situation makes the UE to conform its filter with narrow band-pass and with a very high slope, which it is difficult to obtain without adding losses in the bandpass of the filter. Based on this, it would make reasonable to test the Lowest channel bandwidths.

However, the channel bandwidth with Higher band pass are especially sensible to obtain ramp-down with high slope filters whose size is small, so it is interesting to test this option as well.

Observation 5: Test the Lowest and Highest seems to cover the most critical situations.

Furthermore, due to that MPR, ACLR and SEM, will cover the test of all possible non linearities (frequency, modulation, channel bandwidth and SCS) the number of test points increases considerably if the LTE guideline is followed. Thus, a sensible test point reduction should be required.

Proposal 4: Test the Lowest and Highest Channel Bandwidth.

Table 2.4.1-1 Test channel bandwidth

| Test Case | Test Frequencies |
| --- | --- |
| MPR | Lowest, Highest |
| ACLR | Lowest, Highest |
| SEM | Lowest, Highest |

### Almost contiguous RB allocation

In LTE, Test Channel Bandwidth for non-contiguous resource allocation is Highest, basically FR1 MPR testing can follow LTE and select Highest Test Channel Bandwidth for almost contiguous resource allocation. Besides, in the Minimum conformance requirements for MPR in [4], it is mentioned that for almost contiguous allocation, NRB_alloc + NRB_gap is larger than 106, 51 or 24 RBs for 15 kHz, 30 kHz or 60 kHz respectively, which means Test Channel Bandwidth for non-contiguous resource allocation should be greater than 20MHz.

Proposal 5: Select Highest Test Channel Bandwidth (greater than 20MHz) for almost contiguous allocation MPR test.

## UL Modulation and RB Allocation

### Contiguous RB allocation

As it has been introduced this test cases needs to evaluate the UE behaviour in MPR, ACLR and SEM considering modulation and RB allocation.

In LTE, ACLR and MPR test cases use the same values for the configuration table related to modulations, frequencies, channel bandwidth and allocations. However, as per R5-197586, ACLR/SEM could be tested on a subset of MPR test points. Additionally, as per R5-197586, the aim is to cover RAN4 MPR table as much as possible in respect to modulation and RB allocation. The MPR minimum conformance requirements from 38.101-1 [2] are extracted as below:

In LTE, MPR is tested only for the combinations of modulation and the largest RB allocation for each MPR minimum conformance requirement. Basically, FR1 MPR testing should follow what is being tested for LTE, except for Edge RB allocations. Because based on the baseline agreed in [3], which is:

Edge_Full allocation is not tested in MPR since Edge_1RB can be considered as more critical due to higher PSD. Edge_1RB_Left could be selected for Low range test frequency, and Edge_1RB_Right for High range test frequency.

Observation 6: If the same LTE philosophy is used for MPR and ACLR, then the modulations to be tested in this test case would be defined from the table 6.2.2-1 and table 6.2.2-2 in 38.101-1.

For SEM testing, it is estimated that the impact of Inner allocations on the unwanted power emissions out of band might be low. With the purpose to reduce testing time, only Outer and Edge RB allocations are recommended to be tested.

Observation 7: The configuration table in NR Spectrum Emission Mask can include only the Outer and Edge RB allocations which can contribute to the unwanted emissions and not include the Inner allocations.

Proposal 6: Select the following combinations of modulation and RB allocation from Table 2.5.1-1 and Table 2.5.1-2 for MPR and ACLR in FR1. Select the Outer and Edge RB allocations from Table 2.5.1-1 and Table 2.5.1-2 for SEM in FR1.

Table 2.5.1-1 Combinations of modulation and RB allocation for power class 3

| Modulation | RB allocation |
| --- | --- |
| DFT-s-OFDM PI/2 BPSK1 | Inner Full |
| DFT-s-OFDM PI/2 BPSK1 | Edge_1RB_Left |
| DFT-s-OFDM PI/2 BPSK1 | Edge_1RB_Right |
| DFT-s-OFDM PI/2 BPSK1 | Outer Full |
| DFT-s-OFDM QPSK | Inner Full |
| DFT-s-OFDM QPSK | Edge_1RB_Left |
| DFT-s-OFDM QPSK | Edge_1RB_Right |
| DFT-s-OFDM QPSK | Outer Full |
| DFT-s-OFDM 16 QAM | Inner Full |
| DFT-s-OFDM 16 QAM | Edge_1RB_Left |
| DFT-s-OFDM 16 QAM | Edge_1RB_Right |
| DFT-s-OFDM 16 QAM | Outer Full |
| DFT-s-OFDM 64 QAM | Edge_1RB_Left |
| DFT-s-OFDM 64 QAM | Edge_1RB_Right |
| DFT-s-OFDM 64 QAM | Outer Full |
| DFT-s-OFDM 256 QAM | Edge_1RB_Left |
| DFT-s-OFDM 256 QAM | Edge_1RB_Right |
| DFT-s-OFDM 256 QAM | Outer Full |
| CP-OFDM QPSK | Inner Full |
| CP-OFDM QPSK | Edge_1RB_Left |
| CP-OFDM QPSK | Edge_1RB_Right |
| CP-OFDM QPSK | Outer Full |
| CP-OFDM 16 QAM | Inner Full |
| CP-OFDM 16 QAM | Edge_1RB_Left |
| CP-OFDM 16 QAM | Edge_1RB_Right |
| CP-OFDM 16 QAM | Outer Full |
| CP-OFDM 64 QAM | Edge_1RB_Left |
| CP-OFDM 64 QAM | Edge_1RB_Right |
| CP-OFDM 64 QAM | Outer Full |
| CP-OFDM 256 QAM | Edge_1RB_Left |
| CP-OFDM 256 QAM | Edge_1RB_Right |
| CP-OFDM 256 QAM | Outer Full |
| DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS3,4 | Edge_1RB_Left |
| DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS3,4 | Edge_1RB_Right |
| DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS3,4 | Outer Full |
| NOTE 1: Include two steps for UE operating in bands n40, n41, n77, n78 and n79, with IE powerBoostPi2BPSK set to 1 and 0 separately. NOTE 2: Edge_1RB_Left is only for Low range test frequency, and Edge_1RB_Right is only for High range test frequency. NOTE 3: UE operating in FDD mode, or in TDD mode in bands other than n40, n41, n77, n78 and n79, or in TDD mode the IE powerBoostPi2BPSK is set to 0 for bands n40, n41, n77, n78 and n79. NOTE 4: Applicable to UEs indicating support for UE capability lowPAPR-DMRS-PUSCHwithPrecoding-r16. |  |

Table 2.5.1-2 Combinations of modulation and RB allocation for power class 2

| Modulation | RB allocation |
| --- | --- |
| DFT-s-OFDM PI/2 BPSK | Inner Full |
| DFT-s-OFDM PI/2 BPSK | Edge_1RB_Left |
| DFT-s-OFDM PI/2 BPSK | Edge_1RB_Right |
| DFT-s-OFDM PI/2 BPSK | Outer Full |
| DFT-s-OFDM QPSK | Inner Full |
| DFT-s-OFDM QPSK | Edge_1RB_Left |
| DFT-s-OFDM QPSK | Edge_1RB_Right |
| DFT-s-OFDM QPSK | Outer Full |
| DFT-s-OFDM 16 QAM | Inner Full |
| DFT-s-OFDM 16 QAM | Edge_1RB_Left |
| DFT-s-OFDM 16 QAM | Edge_1RB_Right |
| DFT-s-OFDM 16 QAM | Outer Full |
| DFT-s-OFDM 64 QAM | Edge_1RB_Left |
| DFT-s-OFDM 64 QAM | Edge_1RB_Right |
| DFT-s-OFDM 64 QAM | Outer Full |
| DFT-s-OFDM 256 QAM | Edge_1RB_Left |
| DFT-s-OFDM 256 QAM | Edge_1RB_Right |
| DFT-s-OFDM 256 QAM | Outer Full |
| CP-OFDM QPSK | Inner Full |
| CP-OFDM QPSK | Edge_1RB_Left |
| CP-OFDM QPSK | Edge_1RB_Right |
| CP-OFDM QPSK | Outer Full |
| CP-OFDM 16 QAM | Inner Full |
| CP-OFDM 16 QAM | Edge_1RB_Left |
| CP-OFDM 16 QAM | Edge_1RB_Right |
| CP-OFDM 16 QAM | Outer Full |
| CP-OFDM 64 QAM | Edge_1RB_Left |
| CP-OFDM 64 QAM | Edge_1RB_Right |
| CP-OFDM 64 QAM | Outer Full |
| CP-OFDM 256 QAM | Edge_1RB_Left |
| CP-OFDM 256 QAM | Edge_1RB_Right |
| CP-OFDM 256 QAM | Outer Full |
| NOTE 1: Edge_1RB_Left is only for Low range test frequency，and Edge_1RB_Right is only for High range test frequency. |  |

### Almost Contiguous RB allocation

The selection of test points for uplink modulation and RB allocation for almost contiguous allocation should be based on the Minimum conformance requirements for MPR in [2], which are extracted as bellow:

If CP-OFDM allocation satisfies following conditions, it is considered as almost contiguous allocation

NRB_gap / (NRB_alloc + NRB_gap ) ≤ 0.25

and NRB_alloc + NRB_gap is larger than 106, 51 or 24 RBs for 15 kHz, 30 kHz or 60 kHz respectively where NRB_gap is the total number of unallocated RBs between allocated RBs and NRB_alloc is the total number of allocated RBs. The size and location of allocated and unallocated RBs are restricted by RBG parameters specified in sub-clause 6.1.2.2 of TS 38.214 [12]. For these almost contiguous signals in power class 3, the allowed maximum power reduction defined in Table 6.2.2.3-1 is increased by

CEIL{ 10 log10(1 + NRB_gap / NRB_alloc), 0.5 } dB,

where CEIL{x,0.5} means x rounding upwards to closest 0.5dB.

According to the above Minimum conformance requirements, almost contiguous allocation is only for CP-OFDM, so just select CP-OFDM for almost contiguous allocation testing.

Besides, in the above Minimum conformance requirements, it is mentioned that for almost contiguous allocation, NRB_alloc + NRB_gap is larger than 106, 51 or 24 RBs for 15 kHz, 30 kHz or 60 kHz respectively, which means only Outer_Full withTest Channel Bandwidth greater than 20MHz and Inner_Full with Test Channel Bandwidth greater than 30MHz can be selected for almost contiguous allocation testing.

Almost contiguous allocation with the smallest NRB_alloc and largest NRB_gap is supposed to be the most critical case for MPR testing. So just select almost contiguous allocation with the smallest NRB_alloc and largest NRB_gap and separate NRB_alloc to two identical or almost identical parts for each MPR Minimum conformance requirement. To align the test points, almost contiguous allocation is also selected for ACLR and SEM.

It must be noted that when almost contiguous RB allocation (RB allocation Type 0 as defined in 38.214) is used, the size of the first and last resource block groups (RBG) depend on the parameter RBstart that, according to 38.213, depends on the parameter offsetToCarrier.

The parameter offsetToCarrier is defined differently in 38.508-1 depending on frequency Low, Mid or High and depending on the duplex mode FDD or TDD. Only in case of Low, it is set to 0.

Hence, RB allocations need to be defined for all the possible combinations: Low frequency (FDD or TDD), High frequency FDD and High frequency TDD. Mid frequency not needed to be analysed as per proposal in section 2.3 to test only Low and High.

In addition, observed that some RB allocations for 45MHz CBW are not even correct for Low frequency.

Proposal 7: Select the following combinations of modulation and RB allocation from Table 2.5.2-1 to Table 2.5.2-4 for almost contiguous allocation MPR, ACLR and SEM test.

Table 2.5.2-1 Combinations of modulation and RB allocation for almost contiguous allocation

| Modulation | RB allocation |
| --- | --- |
| CP-OFDM QPSK | Inner Full |
| CP-OFDM QPSK | Outer Full |
| CP-OFDM 16 QAM | Inner Full |
| CP-OFDM 16 QAM | Outer Full |
| CP-OFDM 64 QAM | Outer Full |
| CP-OFDM 256 QAM | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 2.7.2-2/3/4. |  |

Table 2.5.2-2: Uplink configuration for almost contiguous allocation for low frequency range

| Channel Bandwidth(MHz) | SCS(kHz) | OFDM | Outer Full |  | Inner Full |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cluster1 RB allocations | Cluster2 RB allocations | Cluster1 RB allocations | Cluster2 RB allocations |
|  |  |  | (LCRB @ RBstart) | (LCRB @ RBstart) | (LCRB @ RBstart) | (LCRB @ RBstart) |
| 25 | 15 | CP | 48@0 | 53@80 | N/A | N/A |
|  | 30 | CP | 24@0 | 25@40 | N/A | N/A |
|  | 60 | CP | 12@0 | 13@18 | N/A | N/A |
| 30 | 15 | CP | 64@0 | 64@96 | N/A | N/A |
|  | 30 | CP | 32@0 | 30@48 | N/A | N/A |
|  | 60 | CP | 16@0 | 14@24 | N/A | N/A |
| 35 | 15 | CP | 80@0 | 76@112 | N/A | N/A |
|  | 30 | CP | 40@0 | 36@56 | N/A | N/A |
|  | 60 | CP | 20@0 | 16@28 | N/A | N/A |
| 40 | 15 | CP | 80@0 | 88@128 | N/A | N/A |
|  | 30 | CP | 40@0 | 42@64 | N/A | N/A |
|  | 60 | CP | 20@0 | 19@32 | 12@12 | 8@28 |
| 45 | 15 | CP | 80@0 | 114@128 | 48@64 | 48@128 |
|  | 30 | CP | 40@0 | 55@64 | 24@32 | 24@64 |
|  | 60 | CP | 20@0 | 26@32 | 12@16 | 12@32 |
| 50 | 15 | CP | 96@0 | 110@160 | 48@64 | 48@144 |
|  | 30 | CP | 48@0 | 53@80 | 24@32 | 24@72 |
|  | 60 | CP | 24@0 | 25@40 | 12@16 | 12@36 |
| 60 | 15 | CP | N/A | N/A | N/A | N/A |
|  | 30 | CP | 64@0 | 66@96 | 32@32 | 16@80 |
|  | 60 | CP | 32@0 | 31@48 | 16@16 | 8@40 |
| 70 | 15 | CP | N/A | N/A | N/A | N/A |
|  | 30 | CP | 80@0 | 77@112 | 32@32 | 16@80 |
|  | 60 | CP | 40@0 | 37@56 | 16@16 | 8@40 |
| 80 | 15 | CP | N/A | N/A | N/A | N/A |
|  | 30 | CP | 80@0 | 89@128 | 32@32 | 16@80 |
|  | 60 | CP | 40@0 | 43@64 | 16@16 | 8@40 |
| 90 | 15 | CP | N/A | N/A | N/A | N/A |
|  | 30 | CP | 96@0 | 101@144 | 32@32 | 16@80 |
|  | 60 | CP | 48@0 | 49@72 | 16@16 | 8@40 |
| 100 | 15 | CP | N/A | N/A | N/A | N/A |
|  | 30 | CP | 112@0 | 97@176 | 48@64 | 48@144 |
|  | 60 | CP | 48@0 | 55@80 | 24@32 | 24@72 |

Table 2.5.2-3: Uplink configuration for almost contiguous allocation for TDD high frequency range

| Channel Bandwidth(MHz) | SCS(kHz) | OFDM | Outer Full |  | Inner Full |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cluster1 RB allocations | Cluster2 RB allocations | Cluster1 RB allocations | Cluster2 RB allocations |
|  |  |  | (LCRB @ RBstart) | (LCRB @ RBstart) | (LCRB @ RBstart) | (LCRB @ RBstart) |
| 25 | 15 | CP | 48@0 | 53@80 | N/A | N/A |
|  | 30 | CP | 24@0 | 25@40 | N/A | N/A |
|  | 60 | CP | 12@0 | 13@18 | N/A | N/A |
| 30 | 15 | CP | 56@0 | 64@88 | N/A | N/A |
|  | 30 | CP | 32@0 | 30@48 | N/A | N/A |
|  | 60 | CP | 16@0 | 14@24 | N/A | N/A |
| 35 | 15 | CP | 72@0 | 84@104 | N/A | N/A |
|  | 30 | CP | 40@0 | 36@56 | N/A | N/A |
|  | 60 | CP | 20@0 | 16@28 | N/A | N/A |
| 40 | 15 | CP | 72@0 | 96@120 | N/A | N/A |
|  | 30 | CP | 40@0 | 42@64 | N/A | N/A |
|  | 60 | CP | 20@0 | 19@32 | 12@12 | 8@28 |
| 45 | 15 | CP | 72@0 | 122@120 | 48@56 | 48@120 |
|  | 30 | CP | 40@0 | 55@64 | 24@32 | 24@64 |
|  | 60 | CP | 20@0 | 26@32 | 12@16 | 12@32 |
| 50 | 15 | CP | 88@0 | 112@152 | 48@72 | 48@152 |
|  | 30 | CP | 48@0 | 53@80 | 24@32 | 24@72 |
|  | 60 | CP | 24@0 | 25@40 | 12@16 | 12@36 |
| 60 | 15 | CP | N/A | N/A | N/A | N/A |
|  | 30 | CP | 56@0 | 74@88 | 32@40 | 16@88 |
|  | 60 | CP | 32@0 | 31@48 | 16@16 | 8@40 |
| 70 | 15 | CP | N/A | N/A | N/A | N/A |
|  | 30 | CP | 72@0 | 80@104 | 32@40 | 16@88 |
|  | 60 | CP | 40@0 | 37@56 | 16@16 | 8@40 |
| 80 | 15 | CP | N/A | N/A | N/A | N/A |
|  | 30 | CP | 72@0 | 96@120 | 32@40 | 16@88 |
|  | 60 | CP | 40@0 | 43@64 | 16@16 | 8@40 |
| 90 | 15 | CP | N/A | N/A | N/A | N/A |
|  | 30 | CP | 88@0 | 109@136 | 32@40 | 16@88 |
|  | 60 | CP | 48@0 | 49@72 | 16@16 | 8@40 |
| 100 | 15 | CP | N/A | N/A | N/A | N/A |
|  | 30 | CP | 104@0 | 105@168 | 48@72 | 48@152 |
|  | 60 | CP | 48@0 | 55@80 | 24@32 | 24@72 |

Table 2.5.2-3: Uplink configuration for almost contiguous allocation for FDD high frequency range

| Channel Bandwidth(MHz) | SCS(kHz) | OFDM | Outer Full |  | Inner Full |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cluster1 RB allocations | Cluster2 RB allocations | Cluster1 RB allocations | Cluster2 RB allocations |
|  |  |  | (LCRB @ RBstart) | (LCRB @ RBstart) | (LCRB @ RBstart) | (LCRB @ RBstart) |
| 25 | 15 | CP | 42@0 | 56@74 | N/A | N/A |
|  | 30 | CP | 22@0 | 27@38 | N/A | N/A |
|  | 60 | CP | 12@0 | 13@18 | N/A | N/A |
| 30 | 15 | CP | 58@0 | 64@90 | N/A | N/A |
|  | 30 | CP | 26@0 | 32@42 | N/A | N/A |
|  | 60 | CP | 14@0 | 16@22 | N/A | N/A |
| 35 | 15 | CP | 74@0 | 82@106 | N/A | N/A |
|  | 30 | CP | 42@0 | 34@58 | N/A | N/A |
|  | 60 | CP | 22@0 | 18@26 | N/A | N/A |
| 40 | 15 | CP | 74@0 | 94@122 | N/A | N/A |
|  | 30 | CP | 34@0 | 48@58 | N/A | N/A |
|  | 60 | CP | 18@0 | 20@30 | 12@14 | 8@30 |
| 45 | 15 | CP | 74@0 | 120@122 | 48@58 | 48@122 |
|  | 30 | CP | 34@0 | 56@58 | 24@34 | 24@66 |
|  | 60 | CP | 18@0 | 28@30 | 12@14 | 12@30 |
| 50 | 15 | CP | 90@0 | 112@154 | 48@74 | 48@154 |
|  | 30 | CP | 42@0 | 56@74 | 24@34 | 24@74 |
|  | 60 | CP | 22@0 | 27@38 | N/A | N/A |
| 60 | 15 | CP | N/A | N/A | N/A | N/A |
|  | 30 | CP | 58@0 | 72@90 | 32@42 | 16@90 |
|  | 60 | CP | 26@0 | 32@42 | 16@18 | 8@42 |
| 70 | 15 | CP | N/A | N/A | N/A | N/A |
|  | 30 | CP | 74@0 | 80@106 | 32@42 | 16@90 |
|  | 60 | CP | 34@0 | 40@50 | 16@18 | 8@42 |
| 80 | 15 | CP | N/A | N/A | N/A | N/A |
|  | 30 | CP | 74@0 | 95@122 | 32@42 | 16@90 |
|  | 60 | CP | 34@0 | 48@58 | 16@18 | 8@42 |
| 90 | 15 | CP | N/A | N/A | N/A | N/A |
|  | 30 | CP | 90@0 | 107@138 | 32@42 | 16@90 |
|  | 60 | CP | 42@0 | 55@66 | 16@18 | 8@42 |
| 100 | 15 | CP | N/A | N/A | N/A | N/A |
|  | 30 | CP | 106@0 | 103@170 | 48@74 | 48@154 |
|  | 60 | CP | 42@0 | 56@74 | 24@34 | 24@74 |

## Number of Test Points for single transmission

Table 2.6-1 Maximum Number of Test Steps for MPR and ACLR (contiguous allocation)

|  | Number of Enviromental conditions | Number SCS | Maximum Number of ChBW | Maximum Number of Frequencies | Number of steps (mod and RB) | Maximum Number of Test Steps |
| --- | --- | --- | --- | --- | --- | --- |
| Power Class 3 | 5 | 2 | 2 | 2 for Outer/Inner allocation 1 for Edge allocation | 14 (153,4) for Outer/Inner allocation 18 (203,4) for Edge allocation | 920 (1040210003,4) |
| Power Class 2 | 5 | 2 | 2 | 2 for Outer/Inner allocation 1 for Edge allocation | 14 for Outer/Inner allocation 18 for Edge allocation | 920 |
| NOTE 1: DFT-s-OFDM PI/2 BPSK should be tested with IE powerBoostPi2BPSK set to 1 and 0 separately for power class 3 UE operating in bands n40, n41, n77, n78 and n79. NOTE 2: For power class 3 UE operating in bands n40, n41, n77, n78 and n79. NOTE 3: UE operating in FDD mode, or in TDD mode in bands other than n40, n41, n77, n78 and n79, or in TDD mode the IE powerBoostPi2BPSK is set to 0 for bands n40, n41, n77, n78 and n79. NOTE 4: Applicable to UEs indicating support for UE capability lowPAPR-DMRS-PUSCHwithPrecoding-r16. |  |  |  |  |  |  |

Table 2.6-2 Maximum Number of Test Steps for MPR and ACLR (almost contiguous allocation)

| Environmental conditions | Maximum Number of Frequencies | Maximum Number of ChBW | Number SCS | Number of steps (mod and RB) 1 | Maximum Number of Test Steps |
| --- | --- | --- | --- | --- | --- |
| 5 | 2 | 1 | 2 | 6 | 120 |

Table 2.6-3 Maximum Number of Test Steps for SEM (contiguous allocation)

|  | Number of Enviromental conditions | Number SCS | Maximum Number of ChBW | Maximum Number of Frequencies | Number of steps (mod and RB) | Maximum Number of Test Steps |
| --- | --- | --- | --- | --- | --- | --- |
| Power Class 3 | 1 | 2 | 2 | 2 for Outer allocation 1 for Edge allocation | 9 (103,4) for Outer allocation 18 (203,4) for Edge allocation | 144 (1682,1603,4) |
| Power Class 2 | 1 | 2 | 2 | 2 for Outer allocation 1 for Edge allocation | 9 for Outer allocation 18 for Edge allocation | 144 |
| NOTE 1: DFT-s-OFDM PI/2 BPSK should be tested with IE powerBoostPi2BPSK set to 1 and 0 separately for power class 3 UE operating in bands n40, n41, n77, n78 and n79. NOTE 2: For power class 3 UE operating in bands n40, n41, n77, n78 and n79. NOTE 3: UE operating in FDD mode, or in TDD mode in bands other than n40, n41, n77, n78 and n79, or in TDD mode the IE powerBoostPi2BPSK is set to 0 for bands n40, n41, n77, n78 and n79. NOTE 4: Applicable to UEs indicating support for UE capability lowPAPR-DMRS-PUSCHwithPrecoding-r16. |  |  |  |  |  |  |

Table 2.6-4 Maximum Number of Test Steps for SEM (almost contiguous allocation)

| Environmental conditions | Maximum Number of Frequencies | Maximum Number of ChBW | Number SCS | Number of steps (mod and RB) 1 | Maximum Number of Test Steps |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 1 | 2 | 6 | 24 |

## Exceptions for UL MIMO test

Extracted from Clause 6.5D.2 [2]

6.5D.2	Out of band emission for UL-MIMO

For UE supporting UL-MIMO, the requirements for Out of band emissions resulting from the modulation process and non-linearity in the transmitters is defined as the sum of the emissions from both UE transmit antenna connectors.

For UEs with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the requirements in subclasuse 6.5.2 apply. The requirements shall be met with UL-MIMO configurations described in sub-clause 6.2D.1.

For UE support uplink full power transmission (ULFPTx) for UL MIMO, the requirements in clause 6.5.2 shall apply. The requirements shall be met with the PUSCH configurations specified in Table 6.2D.1-3, based upon UE’s support of uplink full power transmission mode.

If UE is configured for single-antenna port PUSCH transmission by DCI format 0_0 or by DCI format 0_1 for single antenna port codebook based transmission, the requirements in subclause 6.5.2 apply.

Observation 8: Same out of band emission requirements as specified for single transmission apply for UE supporting UL-MIMO.

According to [5], CP-OFDM is mandatory to UL-MIMO transmission with two layer data, therefore DFT-s-OFDM is precluded from the test points for UL-MIMO measurement with 2-layer.

Proposal 8: Test CP-OFDM QPSK, CP-OFDM 16 QAM, CP-OFDM 64 QAM and CP-OFDM 256 QAM for NR MPR, ACLR and SEM UL-MIMO measurement with 2-layer in FR1, while the other test parameters are the same as single transmission case.

For Rel-16 UEs supporting uplink full power transmission (ULFPTx) for UL MIMO, the requirements specified in TS 38.101-1 clause 6.5D.2 shall be met with the PUSCH configurations specified in Table 6.2D.1-3, based upon UE’s support of uplink full power transmission mode. Table 6.2D.1-3 includes both DFT-s-OFDM and CP-OFDM.

Proposal 9: Test points for Rel-16 ULFPTx UE can be the same as single transmission case without exception.

## Number of Test Points for UL MIMO test

Table 2.8-1 Maximum Number of Test Steps for MPR and ACLR for UL MIMO

|  | Number of Enviromental conditions | Number SCS | Maximum Number of ChBW | Maximum Number of Frequencies | Number of steps (mod and RB) | Maximum Number of Test Steps |
| --- | --- | --- | --- | --- | --- | --- |
| UL MIMO with 2-layer | 5 | 2 | 2 | 2 for Outer/Inner allocation 1 for Edge allocation | 6 for Outer/Inner allocation 8 for Edge allocation | 400 |
| ULFPTx | 5 | 2 | 2 | 2 for Outer/Inner allocation 1 for Edge allocation | 14 for Outer/Inner allocation 18 for Edge allocation | 920 |

Table 2.8-2 Maximum Number of Test Steps for SEM for UL MIMO

|  | Number of Enviromental conditions | Number SCS | Maximum Number of ChBW | Maximum Number of Frequencies | Number of steps (mod and RB) | Maximum Number of Test Steps |
| --- | --- | --- | --- | --- | --- | --- |
| UL MIMO with 2-layer | 1 | 2 | 2 | 2 for Outer allocation 1 for Edge allocation | 4 for Outer allocation 8 for Edge allocation | 64 |
| ULFPTx | 1 | 2 | 2 | 2 for Outer allocation 1 for Edge allocation | 9 for Outer allocation 18 for Edge allocation | 144 |
| NOTE 1: DFT-s-OFDM PI/2 BPSK should be tested with IE powerBoostPi2BPSK set to 1 and 0 separately for power class 3 UE operating in bands n40, n41, n77, n78 and n79. NOTE 2: For power class 3 UE operating in bands n40, n41, n77, n78 and n79. NOTE 3: UE supporting ULFPTx for UL MIMO. |  |  |  |  |  |  |

# Conclusion

It is proposed that RAN5 discusses and agrees the following proposals for open areas described in section 2 of this document to progress on SA FR1 Adjacent Channel Leakage Ratio test case definition:

Proposal 1: Define Test Environment as per LTE

| Test Case | Test Environment |
| --- | --- |
| MPR | Normal, TL/VL, TL/VH, TH/VL and TH/VH |
| ACLR | NormalTL/VL, TL/VH, TH/VL and TH/VH |
| SEM | NC |

Proposal 2: Select the Lowest and Highest supported SCS

| Test Case | Test Subcarrier Spacing |
| --- | --- |
| MPR | Lowest, Highest SCS |
| ACLR | Lowest, Highest SCS |
| SEM | Lowest, Highest SCS |

Proposal 3: Test Low and High Frequencies.

| Test Case | Test Frequencies |
| --- | --- |
| MPR | Low, High |
| ACLR | Low, High |
| SEM | Low, High |

Proposal 4: Test the Lowest and Highest Channel Bandwidth for contiguous allocation.

| Test Case | Test Frequencies |
| --- | --- |
| MPR | Lowest, Highest |
| ACLR | Lowest, Highest |
| SEM | Lowest, Highest |

Proposal 5: Select Highest Test Channel Bandwidth (greater than 20MHz) for almost contiguous allocation MPR test.

Proposal 6: Select the combinations of modulation and RB allocation from Table 2.5.1-1 and Table 2.5.1-2 for MPR, ACLR and SEM for contiguous allocation. Select the Outer and Edge RB allocations from Table 2.5.1-1 and Table 2.5.1-2 for SEM in FR1.

Proposal 7: Select the following combinations of modulation and RB allocation in Table 2.5.2-1 and Table 2.5.2-2 for almost contiguous allocation MPR, ACLR and SEM test.

Proposal 8: Test CP-OFDM QPSK, CP-OFDM 16 QAM, CP-OFDM 64 QAM and CP-OFDM 256 QAM for NR MPR, ACLR and SEM UL-MIMO measurement, while the other test parameters are the same as single transmission case.

Proposal 9: Test points for Rel-16 ULFPTx UE can be the same as single transmission case without exception.

# References

[1] R5-180065, “Discussion about Test parameters for NR TRx TCs“, NTT DOCOMO, RAN5 Ad hoc, Munich, Jan. 2018.

[2] 3GPP TS 38.101-1 v16.6.0 User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone (Release 15)

[3] R5-180115 3GPP TS 38.521-1 v0.1.0 User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Range 1 Standalone (Release 15)

[4] 3GPP TR 38.817-01 v0.3.0 General aspects for UE RF for NR (Release 15)

[5] R5-192024,” Discussion on the waveform selection for UL MIMO tests”, Huawei, RAN5#82, Athens, Feb.2019

[6] R5-197586, “Discussion on the relation between MPR and SEM/ACLR test cases”, Ericsson. RAN5#84, Ljubljana Aug. 2020

[7] 3GPP TR 38.905 v16.3.0, Derivation of test points for radio transmission and reception User Equipment (UE) conformance test cases