3GPP TSG-RAN5 Meeting #99	R5-232480

# Incheon, Korea, May 22nd - May 26th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 2394 | rev | - | Current version: | 17.6.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to Annex A for RedCap RRM TCs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2023-05-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - To align DLBWP.1.3 RedCap/ULBWP.1.2 RedCap with 38.133 - DLBWP.1.2/ULBWP.1.2 is used in gap-based intra-frequency measurement TCs. For 30kHz test configuration the BW is 51RB, i.e. the whole CBW. Which means CD-SSB always falls in the BWP. Then UE can’t perform gap-based intra-frequency measurements. To fix this issue we suggest: 1) change BWP RMC to DLBWP.1.2 RedCap/ULBWP.1.2 RedCap to avoid impacting legacy TCs and, 2) change BWP BW of DLBWP.1.2 RedCap/ULBWP.1.2 RedCap for 30kHz to 25RBs. - DLBWP.1.1 RedCap/ULBWP.1.1 RedCap is used in NCD-based handover TCs. For 30kHz test configuration, BWP BW is 51 RBs, which conflicts with Table A.16.3.1.3.2-3/4 Note 1/2 as shown in following figure. We suggest change BWP BW of DLBWP.1.1 RedCap/ ULBWP.1.1 RedCap for 30kHz to 25RBs, depicted as follows. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - RedCap RMCs are updated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE may fail the test. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.8A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | RAN4 dependency |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

A.8A	Reference BWP configurations for RedCap

This clause provides the typical BWP configurations used for RedCap RRM test cases defined in Annex A. RedCap downlink dedicated BWP configurations are specified in clause A.8A.1 and RedCap uplink dedicated BWP configurations are specified in clause A.8A.2. Other parameters not listed in this clause can be derived from the set up of each test.

A.8A.1	Downlink BWP configurations

Table A.8A.1-1: Downlink BWP patterns for RedCap dedicated BWP configuration

| BWP Parameters | Unit | Values |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DL BWP |  | DLBWP.1.1 RedCap |  |  |  | DLBWP.1.2 RedCap |  |  |  | DLBWP.1.3 RedCap |  |  |  |
| Starting PRB index |  | RBa Note 1 |  |  |  | RBb Note 2 |  |  |  | RBc Note 3 |  |  |  |
| SSB SCS | kHz | 15 | 30 | 120 | 240 | 15 | 30 | 120 | 240 | 15 | 30 | 120 | 240 |
| Bandwidth | RB | 25 | 25 | 32 | 16 | 25 | 25 | 32 | 16 | 52 | 51 | 66 | 32 |
| NOTE 1: RBa is the lowest PRB index to guarantee the BWP including SSB PRB index (RBJ, RBJ+1,.…, RBJ+19) which is defined in Clause A.3. NOTE 2: RBb is the lowest PRB index to guarantee the BWP not fully overlapped with SSB PRB index (RBJ, RBJ+1,.…, RBJ+19) which is defined in Clause A.3. NOTE 3: RBc is the lowest PRB index to guarantee the BWP including SSB PRB index (RBJ, RBJ+1,.…, RBJ+19) which is defined in Clause A.3A. |  |  |  |  |  |  |  |  |  |  |  |  |  |

A.8A.2	Uplink BWP configurations

Table A.8A.2-1: Uplink BWP patterns for dedicated BWP configurations

| BWP Parameters | Unit | Values |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL BWP |  | ULBWP.1.1 RedCap |  |  |  | ULBWP.1.2 RedCap |  |  |  | ULBWP.1.3 RedCap |  |  |  |
| Starting PRB index |  | RBa Note 1 |  |  |  | RBb Note 2 |  |  |  | RBc Note 3 |  |  |  |
| SSB SCS | kHz | 15 | 30 | 120 | 240 | 15 | 30 | 120 | 240 | 15 | 30 | 120 | 240 |
| Bandwidth | RB | 25 | 25 | 32 | 16 | 25 | 25 | 32 | 16 | 52 | 51 | 66 | 32 |
| NOTE 1: RBa is the same as RBa for DLBWP.1.1 RedCap. NOTE 2: RBb is the same as RBb for DLBWP.1.2 RedCap. NOTE 3: RBc is the same as RBc for DLBWP.1.3 RedCap. |  |  |  |  |  |  |  |  |  |  |  |  |  |

<End of modified section 2>