3GPP TSG WG RAN5 Meeting #99	R5-232527

# Incheon, South Korea, 22 – 26 May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Text Proposal |  |  |  |  |  |  |  |  |
|  | 36.521-4 | CR | N/A | rev | - | Current version: | 0.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding test case 6.3A.3.2.2 for SRS time mask |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Beijing Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN_req-UEConTest |  |  |  |  |  | Date: |  |  | 2023-05-11 |
| Category: | N/A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Adding new test case 6.3A.3.2.2 for SRS time mask for category M1 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Complete test case for 6.3A.3.2.2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case is incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3A.3.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Start of changes >>

6.3A.3.2.2	SRS time mask for UE category M1

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

- Initial condition and call setup procedure to support satellite access are TBD

- Message exceptions specific to satellite access is TBD

- Test Points analysis is TBD

- Test configuration is TBD

- Annex F MU/TT is TBD

6.3A.3.2.2.1	Test purpose

Same test purpose as in TS 36.521-1[14] clause 6.3.4.2.2.1.

6.3A.3.2.2.2	Test applicability

This test case applies to all types of E-UTRA UE release 17 and forward of UE category M1 that support satellite access operation.

6.3A.3.2.2.3	Minimum conformance requirements

Same minimum conformance requirements as in TS 36.521-1[14] clause 6.3.4.2.2.3.

6.3A.3.2.2.4	Test description

6.3A.3.2.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA operating bands specified in clause 5.2A. All these configurations shall be tested with applicable test parameters for each channel bandwidth and are shown in table 6.3A.3.2.2.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in TS 36.521-1 Annexes A.2 and A.3

Table 6.3A.3.2.2.4.1-1: Test Configuration Table

| Initial Conditions |  |
| --- | --- |
| Test Environment (as specified in TS 36.508 [12] subclause 4.1) | Normal, TL/VL, TL/VH, TH/VL, TH/VH |
| Test Frequencies (as specified in TS 36.508 [12] subclause 4.3.1) | Mid range |
| Test Channel Bandwidths (as specified in TS 36.508 [12] subclause 4.3.1) | 1.4MHz |
| SRS configuration |  |
|  | FDD |
| srs-BandwidthConfig | bw7 |
| srs-SubframeConfig | sc3 |
| ackNackSRS-SimultaneousTransmission | FALSE |
| srsMaxUpPts | N/A |
| srs-Bandwidth | bw3 |
| srs-HoppingBandwidth | hbw3 |
| freqDomainPosition | 0 |
| Duration | TRUE |
| srs-ConfigIndex | 7 |
| transmissionComb | 0 |
| cyclicShift | cs0 |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [12] Annex A Figure A.3

2.	The parameter settings for the cell are set up according to TS 36.508 [12] subclause 4.4.3.

3.	Downlink signals are initially set up according to TS 36.521-1[14] Annex C.0, C.1, and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	Propagation conditions are set according to TS 36.521-1[14] Annex B.0.

5.	Ensure the UE is in State 3A-RF-CE according to TS 36.508 [12] clause 5.2A.2AA. Message contents are defined in clause 6.3A.3.2.2.4.3. Note that MPDCCH DCI format 6-0A sent after resetting uplink power with RRC Connection Reconfiguration, should have TPC command 0dB.

6.3A.3.2.2.4.2	Test procedure

1.	For FDD UE, the SS measure the UE transmission OFF power during the 13 symbols preceding the SRS symbol excluding a transient period of 20 µs according to TS 36.521-1[14] Figure 6.3.4.2.2.3-1

2.	Measure the output power of the transmitted SRS according to TS 36.521-1[14] Figure 6.3.4.2.2.3-1 for FDD and HD-FDD UE and the transient periods are excluded from measurement accordingly.

3.	Measure the UE transmission OFF power during the sub-frame following the SRS under test, excluding a transient period of 20 µs according to TS 36.521-1[14] Figure 6.3A.3.2.2.3-1 for FDD and HD-FDD UE

6.3A.3.2.2.4.3	Message contents

Same message contents as in TS 36.521-1[14] clause 6.3.4.2.2.4.3 with the following exception.

Table 6.3A.3.2.2.4.3-1: SchedulingRequest-Config

| Derivation Path: 36.331[6] clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SchedulingRequest-Config-DEFAULT ::= CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| sr-ConfigIndex | 33 |  | HD-FDD |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| HD-FDD | Half Duplex FDD cell environment |

6.3A.3.2.2.5	Test requirement

The requirement for the power measured in steps (1), (2) and (3) of the test procedure shall not exceed the values specified in Table 6.3A.3.2.2.5-1.

Table 6.3A.3.2.2.5-1: SRS time mask

|  | Channel bandwidth / Output Power [dBm] / measurement bandwidth |
| --- | --- |
|  | 1.4 MHz |
| Transmit OFF power | For carrier frequency f ≤ 3.0GHz: -48.5 dBm For carrier frequency 3.0GHz < f ≤ 4.2GHz: ≤ -48.2 dBm |
| Transmission OFF Measurement bandwidth | 1.08 MHz |
| Expected SRS Transmission ON Measured power | -2.6 dBm |
| ON power tolerance f ≤ 3.0GHz 3.0GHz < f ≤ 4.2GHz | ± 7.5dB ± 7.8dB |

<< End of changes >>