3GPP TSG WG RAN5 Meeting #99	R5-232555

# Incheon, South Korea, 22 – 26 May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Text Proposal |  |  |  |  |  |  |  |  |
|  | 36.521-4 | CR | N/A | rev | - | Current version: | 0.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding test case 6.3B.3.2 NPRACH time mask |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Beijing Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN_req-UEConTest |  |  |  |  |  | Date: |  |  | 2023-05-11 |
| Category: | N/A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Adding test case 6.3B.3.2 NPRACH time mask for category NB1 and NB2 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Complete test case for 6.3B.3.2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case is incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3B.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Start of changes >>

6.3B.3.2	NPRACH time mask for category NB1 and NB2

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

- Initial condition and call setup procedure to support satellite access are TBD

- Message exceptions specific to satellite access is TBD

- Test Points analysis is TBD

- Test configuration is TBD

- Annex F MU/TT is TBD

6.3B.3.2.1	Test purpose

To verify that the NPRACH time mask meets the requirements given in 6.3B.3.2.5.

The time mask for NPRACH time mask defines the ramping time allowed for the UE between transmit OFF power and transmit ON power when transmitting the NPRACH.

Transmission of the wrong power increases interference to other channels or increases transmission errors in the uplink channel.

6.3B.3.2.2	Test applicability

This test case applies to all types of NB-IoT FDD UE release 17 and forward of category NB1 and NB2 that support satellite access operation.

6.3B.3.2.3	Minimum conformance requirements

The NPRACH ON power is specified as the mean power over the NPRACH measurement period excluding any transient periods as shown in Figure 6.3B.3.2.3-1. The measurement period for different NPRACH preamble format is specified in Table 6.3B.2.3-1.

There are no additional requirements on UE transmit power beyond that which is required in subclause 6.2B and 6.5B

Table 6.3B.3.2.3-1: NPRACH ON power measurement period

| NPRACH preamble format | Measurement period (ms) |
| --- | --- |
| 0 | 5.6 |
| 1 | 6.4 |

Figure 6.3B.3.2.3-1: NPRACH ON/OFF time mask

The normative reference for this requirement is TS 36.102 [11] clause 6.3B.3.3.

6.3B.3.2.4	Test description

6.3B.3.2.4.1	Initial conditions

Initial conditions are set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions and test frequencies based on the subset of E-UTRA operating bands defined for NB-IoT in clause 5.2B. All of these configurations shall be tested with applicable test parameters for each channel bandwidth and are shown in table 6.3B.3.2.4.1-1. The details of the uplink reference measurement channel (RMCs) are specified in TS 36.521-1[14] Annex A.2.4.

Table 6.3B.3.2.4.1-1: Test Configuration Table

| Initial Conditions |  |
| --- | --- |
| Test Environment as specified in TS 36.508 [12] subclause 8.1.1 | Normal |
| Test Frequencies as specified in TS 36.508 [12] subclause 8.1.3.1 | Frequency ranges defined in Annex K.1.1 |
| NPRACH preamble format | 0 |
|  | 1 |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [12] Annex A Figure A.3 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [12] subclause 8.1.4.3.

3.	Downlink signals are initially set up according to TS 36.521-1[14] Annex C.0, C.1 and C.3.0, and uplink signals according to TS 36.521-1 [14] Annex H.1.1 and H.4.0.

4.	Propagation conditions are set according to TS 36.521-1[14] Annex B.0.

5.	Ensure the UE is in State 3A-NB with CP CIoT Optimisation according to TS 36.508 [12] clause 8.1.5. Message contents are defined in clause 6.3B.3.2.4.3.

6.3B.3.2.4.2	Test procedure

1.	The SS shall set RS EPRE according to Table 6.3B.3.2.4.1-1.

2.	The SS send the paging and UE shall send a preamble to the SS.

3.	The SS measure the UE transmission OFF power during the sub-frame preceding the NPRACH preamble excluding a transient period of 20 µs according to Figure 6.3B.3.2.3-1.

4.	Measure the output power of the transmitted NPRACH preamble according to Figure 6.3B.3.2.3-1.

5.	Measure the UE transmission OFF power, starting 20 µs after the NPRACH preamble ends for a measurement period of 980 µs.

6.	Switches off and on the UE and ensures the UE is in State 3A-NB with CP CIoT Optimisation according to TS 36.508 [12] clause 8.1.5. Message contents are defined in clause 6.3B.3.2.4.3 with NPRACH Format 1.

7.	Repeat test with step 1-5.

6.3B.3.2.4.3	Message contents

Message contents are according to TS 36.508 [12] subclause 8.1.6 with the following exceptions.

Table 6.3B.3.2.4.3-1: RACH-ConfigCommon-NB-DEFAULT

| Derivation Path: TS 36.508 [12] clause 8.1.6.3, Table 8.1.6.3-8 RACH-ConfigCommon-NB-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigCommon-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| powerRampingParameters-r13 SEQUENCE { |  |  |  |
| powerRampingStep | dB0 | 0 dB |  |
| preambleInitialReceivedTargetPower | -dBm-120 | -120 dBm | NPRACH Format 0 |
|  | dBm-120 | -120 dBm | NPRACH Format 1 |
| } |  |  |  |
| ) |  |  |  |

Table 6.3B.3.2.4.3-2: NPDSCH-ConfigCommon-NB-DEFAULT

| Derivation Path: TS 36.508 [12] clause 8.1.6.3, Table 8.1.6.3-4 NPDSCH-ConfigCommon-NB-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NPDSCH-ConfigCommon-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| nrs-Power-r13 | 24 (dBm) |  |  |
| } |  |  |  |

Table 6.3B.3.2.4.3-3: NPRACH-ConfigSIB-NB-DEFAULT

| Derivation Path: TS 36.508 [12] clause 8.1.6.3, Table 8.1.6.3-5 NPRACH-ConfigSIB-NB-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NPRACH-ConfigSIB-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| nprach-CP-Length-r13 | us66dot7 | 2048*Ts | NPRACH Format 0 |
|  | us266dot7 | 8192*Ts | NPRACH Format 1 |
| } |  |  |  |

6.3B.3.2.5	Test requirement

The requirement for the power measured in steps (2), (3) and (4) of the test procedure shall not exceed the values specified in Table 6.3B.1.5-1.

Table 6.3B.3.2.5-1: General ON/OFF time mask for category NB1 and NB2

|  | Channel bandwidth / minimum output power / measurement bandwidth |
| --- | --- |
|  | 200 kHz |
| Transmit OFF power | For carrier frequency f ≤ 3.0GHz: ≤ -48.5 dBm |
| Transmission OFF Measurement bandwidth | 180kHz |
| Expected Transmission ON Measured power | -11 dBm |
| ON power tolerance f ≤ 3.0GHz | ± 7.5 dB |

<< End of changes >>