3GPP TSG WG RAN5 Meeting #99	R5-232556

# Incheon, South Korea, 22 – 26 May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Text Proposal |  |  |  |  |  |  |  |  |
|  | 36.521-4 | CR | N/A | rev | - | Current version: | 0.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding test case 6.3B.4.1 Power Control Absolute power tolerance |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Beijing Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN_req-UEConTest |  |  |  |  |  | Date: |  |  | 2023-05-11 |
| Category: | N/A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Adding test case 6.3B.4.1 Power Control Absolute power tolerance for category NB1 and NB2 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Complete test case for 6.3B.4.1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case is incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3B.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Start of changes >>

6.3B.4	Power Control for category NB1 and NB2

6.3B.4.1	Power Control Absolute power tolerance for category NB1 and NB2

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

- Initial condition and call setup procedure to support satellite access are TBD

- Message exceptions specific to satellite access is TBD

- Test Points analysis is TBD

- Test configuration is TBD

- Annex F MU/TT is TBD

6.3B.4.1.1	Test purpose

To verify the ability of the UE transmitter to set its initial output power to a specific value at the start of a contiguous transmission or non-contiguous transmission with a long transmission gap, i.e., transmission gap is larger than 20 ms.

6.3B.4.1.2	Test applicability

This test case applies to all types of NB-IoT FDD UE release 17 and forward of category NB1 and NB2 that support satellite access operation.

6.3B.4.1.3	Minimum conformance requirements

The minimum requirement for absolute power tolerance is given in Table 6.3B.4.1.3-1 over the power range bounded by the Maximum output power as defined in subclause 6.2B and the Minimum output power as defined in subclause 6.3B

Table 6.3B.4.1.3-1: Absolute power tolerance

| Conditions | Tolerance |
| --- | --- |
| Normal | ± 9.0 dB |
| Extreme | ± 12.0 dB |

The normative reference for this requirement is TS 36.102 [11] clause 6.3B.3.4.

6.3B.4.1.4	Test description

6.3B.4.1.4.1	Initial conditions

Initial conditions are set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions and test frequencies based on the subset of E-UTRA operating bands defined for NB-IoT in clause 5.2B. All of these configurations shall be tested with applicable test parameters for each channel bandwidth and are shown in Table 6.3B.4.1.4.1-1. The details of the uplink reference measurement channel (RMCs) are specified in TS 36.521-1[14] Annex A.2. Configurations of NPDSCH and NPDCCH before measurement are specified in TS 36.521-1[14] Annex C.2.

Table 6.3B.4.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [12] clause 8.1.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS36.508 [12] clause 8.1.3.1 |  |  | Frequency ranges defined in Annex K.1.1 |  |  |
| Configuration ID | Downlink Configuration |  | Uplink Configuration |  |  |
|  | Modulation | Subcarriers | Modulation | Ntones | Subcarrier spacing |
| 1 | N/A |  | QPSK | 1@0 | 3.75 kHz |
| 2 |  |  | QPSK | 1@0 | 15 kHz |
| 3 (NOTE 1) |  |  | QPSK | 12@0 | 15 kHz |
| Note 1: Applicable to UE supporting UL multi-tone transmissions |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [12] Annex A Figure A.3 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [12] subclause 8.1.4.3.

3.	Downlink signals are initially set up according to TS 36.521-1[14] Annex C.0, C.1 and C.3.0, and uplink signals according to TS 36.521-1 [14] Annex H.1.1 and H.4.0.

4.	The UL Reference Measurement channel is set according to Table 6.3B.4.1.4.1-1.

5.	Ensure the UE is in State 2A-NB with CP CIoT Optimisation according to TS 36.508 [12] clause 8.1.5. Message contents are defined in clause 6.3B.4.1.4.3.

6.3B.4.1.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via NPDCCH DCI format N0 for C_RNTI to schedule the UL RMC according to Table 6.3B.4.1.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	The SS shall configure the UE to transmit according to Table 6.3B.4.1.4.3-1 and Table 6.3B.4.1.4.3-2.

3.	Measure the initial output power of the first resource unit of UE NPUSCH first transmission. The transient periods of 20us are excluded. The period of measurement shall be at least the continuous duration of one sub-frame (1ms) for 15 kHz sub-carrier spacing or one slot (2ms) excluding the 2304Ts gap when UE is not transmitting for 3.75 kHz sub-carrier spacing. Half-Duplex guard subframes are not under test.

4.	Release the connection through State 3A-NB.

5.	Modify system information elements according to Table 6.3B.4.1.4.3-3 and Table 6.3B.4.1.4.3-4 and notify the UE via paging message with SystemInformationModification included.

6.	Ensure the UE is in State 2A-NB with CP CIoT Optimisation according to TS 36.508 [12] clause 8.1.5 using the new UL power control setting.

7.	SS sends uplink scheduling information for each UL HARQ process via NPDCCH DCI format N0 for C_RNTI to schedule the UL RMC according to Table 6.3B.4.1.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

8.	Measure the initial output power of the first resource unit of UE NPUSCH first transmission. The transient periods of 20us are excluded. The period of measurement shall be at least the continuous duration of one sub-frame (1ms) for 15 kHz sub-carrier spacing or one slot (2ms) excluding the 2304Ts gap when UE is not transmitting for 3.75 kHz sub-carrier spacing. Half-Duplex guard subframes are not under test. For TDD slots with transient periods are not under test.

NOTE 1:	For configuration IDs applicable to UE depending on UE capability in Test Configuration Table with different UL sub-carrier spacing, the SS shall release the connection through State 3A-NB and finally ensure the UE is in State 2A-NB with CP CIoT Optimisation according to TS 36.508 [12] clause 8.1.5 using the appropriate UL subcarrier spacing in Random Access Response message.

6.3B.4.1.4.3	Message contents

Message contents are according to TS 36.508 [12] subclause 8.1.6 with the following exceptions.

Table 6.3B.4.1.4.3-1: UplinkPowerControlCommon: Test point 1

| Derivation Path: TS 36.508 [12] clause 8.1.6.3, Table 8.1.6.3-14 UplinkPowerControlCommon-NB-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UplinkPowerControlCommon-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| p0-NominalNPUSCH-r13 | -124 (-124 dBm) | Test point 1 to verify a UE relative low initial power transmission |  |

Table 6.3B.4.1.4.3-2: NPDSCH-ConfigCommon-NB-DEFAULT: Test Point 1

| Derivation Path: 36.331 clause 6.7.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NPDSCH-ConfigCommon-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| nrs-Power-r13 | 20 (dBm) |  |  |
| } |  |  |  |

Table 6.3B.4.1.4.3-3: UplinkPowerControlCommon: Test point 2

| Derivation Path: TS 36.508 [12] clause 8.1.6.3, Table 8.1.6.3-14 UplinkPowerControlCommon-NB-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UplinkPowerControlCommon-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| p0-NominalNPUSCH-r13 | -94 (-94 dBm) | Test point 2 to verify a UE relative high initial power transmission |  |

Table 6.3B.4.1.4.3-4: NPDSCH-ConfigCommon-NB-DEFAULT: Test Point 2

| Derivation Path: 36.331 clause 6.7.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NPDSCH-ConfigCommon-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| nrs-Power-r13 | 3 (dBm) |  |  |
| } |  |  |  |

Table 6.3B.4.1.4.3-5: PhysicalConfigDedicated

| Derivation Path: TS 36.508 [12] clause 8.1.8.2, Table 8.1.8.2.1.6-1 PhysicalConfigDedicated-NB-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PhysicalConfigDedicated-NB-DEFAULT ::= SEQUENCE { |  |  |  |  |
| uplinkPowerControlDedicated-r13 |  | UplinkPowerControlDedicated-NB-DEFAULT | See subclause 8.1.6.3 |  |

Table 6.3B.4.1.4.3-6: UplinkPowerControlDedicated-NB-DEFAULT

| Derivation Path: TS 36.508 [12] clause 8.1.6.3, Table 8.1.6.3-15 UplinkPowerControlDedicated-NB-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UplinkPowerControlDedicated-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| p0-UE-NPUSCH-r13 | 0 | Default |  |
| } |  |  |  |

6.3B.4.1.5	Test requirement

The requirement for the power measured in steps (2), (3) and (4) of the test procedure shall not exceed the values specified in Table 6.3B.4.1.5-1 and 6.3B.4.1.5-2.

Table 6.3B.4.1.5-1: Absolute power tolerance: test point 1

|  | Channel bandwidth / expected output power (dBm) |  |  |
| --- | --- | --- | --- |
|  | Configuration ID 1 3.75 kHz (1 tone) | Configuration ID 2 15 kHz (1 tone) | Configuration ID 3 15 kHz (12 tones) |
| Expected Measured power Normal conditions | -25 dBm | -19 dBm | -8.2 dBm |
| Power tolerance f ≤ 3.0GHz | ± 10.0dB | ± 10.0dB | ± 10.0dB |
| Expected Measured power Extreme conditions | -25 dBm | -19 dBm | -8.2 dBm |
| Power tolerance f ≤ 3.0GHz | ± 13.0dB | ± 13.0dB | ± 13.0dB |
| Note 1: The lower power limit shall not exceed the minimum output power requirements defined in sub-clause 6.3B.1 |  |  |  |

Table 6.3B.4.1.5-2: Absolute power tolerance: test point 2

|  | Channel bandwidth / expected output power (dBm) |  |  |
| --- | --- | --- | --- |
|  | Configuration ID 1 3.75 kHz (1 tone) | Configuration ID 2 15 kHz (1 tone) | Configuration ID 3 15 kHz (12 tones) |
| Expected Measured power Normal conditions | -12 dBm | -6 dBm | 4.8 dBm |
| Power tolerance f ≤ 3.0GHz | ± 10.0dB | ± 10.0dB | ± 10.0dB |
| Expected Measured power Extreme conditions | -12 dBm | -6 dBm | 4.8 dBm |
| Power tolerance f ≤ 3.0GHz | ± 13.0dB | ± 13.0dB | ± 13.0dB |
| Note 1: The upper power limit shall not exceed the maximum output power requirements defined in sub-clause 6.2B.1 |  |  |  |

<< End of changes >>