3GPP TSG RAN WG5#99	R5-232575

# Incheon, Korea, May 22 – 26, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-4 | CR | 0664 | rev | - | Current version: | 17.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition on MU and TT for newly introduced RedCap CQI test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2023-05-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Newly introduced RedCap CQI test cases need MU and TT values. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding MU and TT for test case 6.2.1.1.1.1., 6.2.1.2.1.1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The RedCap WI cannot be finalized |  |  |  |  |  |  |  |  |
| Clauses affected: |  | F.1.1.3, F.1.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### << Start of changes >>

### F.1.1.3	Measurement of Channel State Information reporting

This clause defines the maximum test system uncertainty for channel state information reporting requirements. The maximum test system uncertainty allowed for the measurement uncertainty contributors are defined in Table F.1.1.3-1.

Table F.1.1.3-1: Maximum measurement uncertainty values for the test system for FR1 (up to 6 GHz) and Channel BW ≤ 40 MHz

| MU contributor | Unit | Value | Comment |
| --- | --- | --- | --- |
| AWGN flatness and signal flatness, max deviation for any Resource Block, relative to average over BWconfig | dB | Same as in table F.1.1.2-1 |  |
| Signal to noise ratio uncertainty | dB | Same as in table F.1.1.2-1 |  |
| Signal to noise ratio variation | dB | Same as in table F.1.1.2-1 |  |
| Fading profile power uncertainty for 1Tx | dB | Same as in table F.1.1.2-1 |  |
| Fading profile power uncertainty for 2Tx | dB | Same as in table F.1.1.2-1 |  |

The maximum test system uncertainty for test cases defined in section 6 is defined in Table F.1.1.3-2.

Table F.1.1.3-2: Maximum test system uncertainty for FR1 channel state information reporting test cases

| Subclause |  |  | Maximum Test System Uncertainty |  |  | Derivation of Test System Uncertainty |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6.2.1.1.1.1 1Rx FDD FR1 periodic CQI reporting under AWGN conditions for RedCap |  |  | Same as 6.2.2.1.1.1 |  |  | Same as 6.2.2.1.1.1 |  |  |
| 6.2.1.2.1.1 1Rx TDD FR1 periodic CQI reporting under AWGN conditions for RedCap |  |  | Same as 6.2.2.1.1.1 |  |  | Same as 6.2.2.1.1.1 |  |  |
| 6.2.2.1.1.1 2Rx FDD FR1 periodic CQI reporting under AWGN conditions for both SA and NSA |  |  | +/- 0.3 dB |  |  | Overall system uncertainty for AWGN conditions comprises: Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±2.0 dB not expected to have any significant effect |  |  |
| 6.2.2.1.1.2 2Rx FDD FR1 periodic CQI reporting with Table 3 under AWGN conditions for both SA and NSA |  |  | Same as 6.2.2.1.1.1 |  |  | Same as 6.2.2.1.1.1 |  |  |
| 6.2.2.1.1.4 2Rx FDD FR1 periodic CQI reporting under AWGN conditions for RedCap |  |  | Same as 6.2.2.1.1.1 |  |  | Same as 6.2.2.1.1.1 |  |  |
| 6.2.2.1.2.1 2Rx FDD FR1 periodic wideband CQI reporting under fading conditions for both SA and NSA |  |  | +/- 0.8 dB |  |  | Overall system uncertainty for fading conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty ±0.3 dB 2. Fading profile power uncertainty for 2Tx ±0.7 dB Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) AWGN flatness and signal flatness ±2.0 dB not expected to have any significant effect |  |  |
| 6.2.2.1.2.2 2Rx FDD FR1 aperiodic subband CQI reporting under fading conditions for both SA and NSA |  |  | Same as 6.2.2.1.2.1 |  |  | Same as 6.2.2.1.2.1 |  |  |
| 6.2.2.1.2.4 2Rx FDD FR1 periodic wideband CQI reporting under fading conditions for RedCap |  |  | Same as 6.2.2.1.2.1 |  |  | Same as 6.2.2.1.2.1 |  |  |
| 6.2.2.2.1.1 2Rx TDD FR1 periodic CQI reporting under AWGN conditions for both SA and NSA |  |  | Same as 6.2.2.1.1.1 |  |  | Same as 6.2.2.1.1.1 |  |  |
| 6.2.2.2.1.2 2Rx TDD FR1 periodic CQI reporting with Table 3 under AWGN conditions for both SA and NSA |  |  | Same as 6.2.2.1.1.1 |  |  | Same as 6.2.2.1.1.1 |  |  |
| 6.2.2.2.2.1 2Rx TDD FR1 periodic wideband CQI reporting under fading conditions for both SA and NSA |  |  | Same as 6.2.2.1.2.1 |  |  | Same as 6.2.2.1.2.1 |  |  |
| 6.2.2.2.2.2 2Rx TDD FR1 aperiodic subband CQI reporting under fading conditions for both SA and NSA |  |  | Same as 6.2.2.1.2.1 |  |  | Same as 6.2.2.1.2.1 |  |  |
| 6.2.3.1.1.1 4Rx FDD FR1 periodic CQI reporting under AWGN conditions for both SA and NSA |  |  | Same as 6.2.2.1.1.1 |  |  | Same as 6.2.2.1.1.1 |  |  |
| 6.2.3.1.1.2 4Rx FDD FR1 periodic CQI reporting with Table 3 under AWGN conditions for both SA and NSA |  |  | Same as 6.2.2.1.1.1 |  |  | Same as 6.2.2.1.1.1 |  |  |
| 6.2.3.1.2.1 4Rx FDD FR1 periodic wideband CQI reporting under fading conditions for both SA and NSA |  |  | Same as 6.2.2.1.2.1 |  |  | Same as 6.2.2.1.2.1 |  |  |
| 6.2.3.1.2.2 4Rx FDD FR1 aperiodic subband CQI reporting under fading conditions for both SA and NSA |  |  | Same as 6.2.2.1.2.1 |  |  | Same as 6.2.2.1.2.1 |  |  |
| 6.2.3.2.1.1 4Rx TDD FR1 periodic CQI reporting under AWGN conditions for both SA and NSA |  |  | Same as 6.2.2.1.1.1 |  |  | Same as 6.2.2.1.1.1 |  |  |
| 6.2.3.2.2.1 4Rx TDD FR1 periodic wideband CQI reporting under fading conditions for both SA and NSA |  |  | Same as 6.2.2.1.2.1 |  |  | Same as 6.2.2.1.2.1 |  |  |
| 6.2.3.2.2.2 4Rx TDD FR1 aperiodic subband CQI reporting under fading conditions for both SA and NSA |  |  | Same as 6.2.2.1.2.1 |  |  | Same as 6.2.2.1.2.1 |  |  |
| 6.2A.3.1.1 2Rx CQI reporting accuracy under AWGN conditions for CA (2DL CA) |  |  | Same as 6.2.2.1.1.1 for each CC |  |  | Same as 6.2.2.1.1.1 |  |  |
| 6.2A.3.1.2 2Rx CQI reporting accuracy under AWGN conditions for CA (3DL CA) |  |  | Same as 6.2.2.1.1.1 for each CC |  |  | Same as 6.2.2.1.1.1 |  |  |
| 6.2A.3.1.3 2Rx CQI reporting accuracy under AWGN conditions for CA (4DL CA) |  |  | Same as 6.2.2.1.1.1 for each CC |  |  | Same as 6.2.2.1.1.1 |  |  |
| 6.3.2.1.1 2Rx FDD FR1 Single PMI with 4Tx Type I- SinglePanel codebook for both SA and NSA |  |  | Same as 6.2.2.1.2.1 |  |  | Same as 6.2.2.1.2.1 |  |  |
| 6.3.2.1.2 2Rx FDD FR1 Single PMI with 8Tx Type I – SinglePanel codebook for both SA and NSA |  |  | Same as 6.2.2.1.2.1 |  |  | Same as 6.2.2.1.2.1 |  |  |
| 6.3.2.1.3 2Rx FDD FR1 Multiple PMI with 16Tx Type I – SinglePanel Codebook for both SA and NSA |  |  | Same as 6.2.2.1.2.1 |  |  | Same as 6.2.2.1.2.1 |  |  |
| 6.3.2.1.4 2Rx FDD FR1 Single PMI with 32Tx Type1 - SinglePanel codebook for both SA and NSA |  |  | Same as 6.2.2.1.2.1 |  |  | Same as 6.2.2.1.2.1 |  |  |
| 6.3.2.1.5 2Rx FDD FR1 Multiple PMI with 16Tx TypeII codebook for both SA and NSA |  |  | Same as 6.2.2.1.2.1 |  |  | Same as 6.2.2.1.2.1 |  |  |
| 6.3.2.1.6 2Rx FDD FR1 Multiple PMI with 16Tx Enhanced TypeII codebook for both SA and NSA |  |  | Same as 6.2.2.1.2.1 |  |  | Same as 6.2.2.1.2.1 |  |  |
| 6.2.3.2.1.2 4Rx TDD FR1 periodic CQI reporting with Table 3 under AWGN conditions for both SA and NSA |  |  | Same as 6.2.2.1.1.1 |  |  | Same as 6.2.2.1.1.1 |  |  |
| 6.3.2.2.1 2Rx TDD FR1 Single PMI with 4Tx TypeI – SinglePanel codebook for both SA and NSA |  |  | Same as 6.2.2.1.2.1 |  |  | Same as 6.2.2.1.2.1 |  |  |
| 6.3.2.2.2 2Rx TDD FR1 Single PMI with 8Tx TypeI – SinglePanel codebook for both SA and NSA |  |  | Same as 6.2.2.1.2.1 |  |  | Same as 6.2.2.1.2.1 |  |  |
| 6.3.2.2.3 2Rx TDD FR1 Single PMI with 16Tx Type1 – SinglePanel codebook for both SA and NSA |  |  | Same as 6.2.2.1.2.1 |  |  | Same as 6.2.2.1.2.1 |  |  |
| 6.3.2.2.4 2Rx TDD FR1 Single PMI with 32Tx Type1 – SinglePanel codebook for both SA and NSA |  |  | Same as 6.2.2.1.2.1 |  |  | Same as 6.2.2.1.2.1 |  |  |
| 6.3.2.2.6 2Rx TDD FR1 Multiple PMI with 16Tx Enhanced TypeII codebook for both SA and NSA |  |  | Same as 6.2.2.1.2.1 |  |  | Same as 6.2.2.1.2.1 |  |  |
| 6.3.3.1.1 Single PMI with 4TX TypeI-SinglePanel Codebook– SinglePanel codebook for both SA and NSA |  |  | Same as 6.2.2.1.2.1 |  |  | Same as 6.2.2.1.2.1 |  |  |
| 6.3.3.1.2 Single PMI with 8TX TypeI-SinglePanel Codebook– SinglePanel codebook for both SA and NSA |  |  | Same as 6.2.2.1.2.1 |  |  | Same as 6.2.2.1.2.1 |  |  |
| 6.3.3.1.3 4Rx FDD FR1 Multiple PMI with 16Tx Type I – SinglePanel Codebook for both SA and NSA |  |  | Same as 6.2.2.1.2.1 |  |  | Same as 6.2.2.1.2.1 |  |  |
| 6.3.3.1.4 4Rx FDD FR1 Single PMI with 32Tx Type1 - SinglePanel codebook for both SA and NSA |  |  | Same as 6.2.2.1.2.1 |  |  | Same as 6.2.2.1.2.1 |  |  |
| 6.3.3.1.5 4Rx FDD FR1 Multiple PMI with 16Tx TypeII codebook for both SA and NSA |  |  | Same as 6.2.2.1.2.1 |  |  | Same as 6.2.2.1.2.1 |  |  |
| 6.3.3.1.6 4Rx FDD FR1 Multiple PMI with 16Tx Enhanced TypeII codebook for both SA and NSA |  |  | Same as 6.2.2.1.2.1 |  |  | Same as 6.2.2.1.2.1 |  |  |
| 6.3.3.2.1 4Rx TDD FR1 Single PMI with 4Tx Type1 - SinglePanel codebook for both SA and NSA |  |  | Same as 6.2.2.1.2.1 |  |  | Same as 6.2.2.1.2.1 |  |  |
| 6.3.2.2.5 2Rx TDD FR1 Multiple PMI with 16Tx TypeII codebook for both SA and NSA |  |  | Same as 6.2.2.1.2.1 |  |  | Same as 6.2.2.1.2.1 |  |  |
| 6.3.3.2.2 4Rx TDD FR1 Single PMI with 8Tx Type1 - SinglePanel codebook for both SA and NSA |  |  | Same as 6.2.2.1.2.1 |  |  | Same as 6.2.2.1.2.1 |  |  |
| 6.3.3.2.3 4Rx TDD FR1 Single PMI with 16Tx Type1 - SinglePanel codebook for both SA and NSA |  |  | Same as 6.2.2.1.2.1 |  |  | Same as 6.2.2.1.2.1 |  |  |
| 6.3.3.2.4 4Rx TDD FR1 Single PMI with 32Tx Type1 - SinglePanel codebook for both SA and NSA |  |  | Same as 6.2.2.1.2.1 |  |  | Same as 6.2.2.1.2.1 |  |  |
| 6.3.3.2.5 4Rx TDD FR1 Multiple PMI with 16Tx TypeII codebook for both SA and NSA |  |  | Same as 6.2.2.1.2.1 |  |  | Same as 6.2.2.1.2.1 |  |  |
| 6.3.3.2.6 4Rx TDD FR1 Multiple PMI with 16Tx Enhanced TypeII codebook for both SA and NSA |  |  | Same as 6.2.2.1.2.1 |  |  | Same as 6.2.2.1.2.1 |  |  |
| 6.4.2.1_1 2Rx FDD FR1 RI reporting for both SA and NSA |  |  | Same as 6.2.2.1.2.1 |  |  | Same as 6.2.2.1.2.1 |  |  |
| 6.4.2.2_1 2Rx TDD FR1 RI reporting for both SA and NSA |  |  | Same as 6.2.2.1.2.1 |  |  | Same as 6.2.2.1.2.1 |  |  |
| 6.4.3.1_1 4Rx FDD FR1 RI reporting for both SA and NSA |  |  | Same as 6.2.2.1.2.1 |  |  | Same as 6.2.2.1.2.1 |  |  |
| 6.4.3.2_1 4Rx TDD FR1 RI reporting for both SA and NSA |  |  | Same as 6.2.2.1.2.1 |  |  | Same as 6.2.2.1.2.1 |  |  |

#### << Clauses skipped here >>

### F.1.3.3	Measurement of Channel State Information reporting

The derivation of the test requirements for the test cases in section 6 is defined in Table F.1.3.3-1.

Table F.1.3.3-1: Derivation of Test Requirements (FR1 channel state information reporting tests)

| Test | Minimum Requirement in TS 38.101-4 | Test Tolerance (TT) | Test Requirement in TS 38.521-4 |
| --- | --- | --- | --- |
| 6.2.1.1.1.1 1Rx FDD FR1 periodic CQI reporting under AWGN conditions for RedCap | SNRs as specified Limits as in the Test Procedure | No test tolerances applied | SNR unchanged |
| 6.2.1.2.1.1 1Rx TDD FR1 periodic CQI reporting under AWGN conditions for RedCap | SNRs as specified Limits as in the Test Procedure | No test tolerances applied | SNR unchanged |
| 6.2.2.1.1.1 2Rx FDD FR1 periodic CQI reporting under AWGN conditions for both SA and NSA | SNRs as specified Limits as in the Test Procedure | No test tolerances applied | SNR unchanged |
| 6.2.2.1.1.2 2Rx FDD FR1 periodic CQI reporting with Table 3 under AWGN conditions for both SA and NSA | SNRs as specified Limits as in the Test Procedure | No test tolerances applied | SNR unchanged |
| 6.2.2.1.1.4 2Rx FDD FR1 periodic CQI reporting under AWGN conditions for RedCap | SNRs as specified Limits as in the Test Procedure | No test tolerances applied | SNR unchanged |
| 6.2.2.1.2.1 2Rx FDD FR1 periodic wideband CQI reporting under fading conditions for both SA and NSA | SNRs as specified  20% 1.05 BLER 0.02 | SNR 0 dB  0% 0.01 BLER 0 | SNR unchanged  unchanged  1.04 BLER limit unchanged |
| 6.2.2.1.2.2 2Rx FDD FR1 periodic subband CQI reporting under fading conditions for both SA and NSA | SNRs as specified  2% 55% 1.05 BLER 0.02 | SNR 0 dB  0% 0% 0.01 BLER 0 | SNR unchanged  limit unchanged limit unchanged  1.04 BLER limit unchanged |
| 6.2.2.1.2.4 2Rx FDD FR1 periodic wideband CQI reporting under fading conditions for RedCap | SNRs as specified  20% 1.05 BLER 0.02 | SNR 0 dB  0% 0.01 BLER 0 | SNR unchanged  unchanged  1.04 BLER limit unchanged |
| 6.2.2.2.1.1 2Rx TDD FR1 periodic CQI reporting under AWGN conditions for both SA and NSA | SNRs as specified Limits as in the Test Procedure | No test tolerances applied | SNR unchanged |
| 6.2.2.2.1.2 2Rx TDD FR1 periodic CQI reporting with Table 3 under AWGN conditions for both SA and NSA | SNRs as specified Limits as in the Test Procedure | No test tolerances applied | SNR unchanged |
| 6.2.2.2.2.1 2Rx TDD FR1 periodic wideband CQI reporting under fading conditions for both SA and NSA | SNRs as specified  20% 1.05 BLER 0.02 | SNR 0 dB  0% 0.01 BLER 0 | SNR unchanged  unchanged  1.04 BLER limit unchanged |
| 6.2.2.2.2.2 2Rx TDD FR1 periodic subband CQI reporting under fading conditions for both SA and NSA | SNRs as specified  2% 55% 1.05 BLER 0.02 | SNR 0 dB  0% 0% 0.01 BLER 0 | SNR unchanged  limit unchanged limit unchanged  1.04 BLER limit unchanged |
| 6.2.3.1.1.1 4Rx FDD FR1 periodic CQI reporting under AWGN conditions for both SA and NSA | SNRs as specified Limits as in the Test Procedure | No test tolerances applied | SNR unchanged |
| 6.2.3.1.1.2 4Rx FDD FR1 periodic CQI reporting with Table 3 under AWGN conditions for both SA and NSA | SNRs as specified Limits as in the Test Procedure | No test tolerances applied | SNR unchanged |
| 6.2.3.1.2.1 4Rx FDD FR1 periodic wideband CQI reporting under fading conditions for both SA and NSA | SNRs as specified  5% 1.05 BLER 0.02 | SNR 0 dB  0% 0.01 BLER 0 | SNR unchanged  unchanged  1.04 BLER limit unchanged |
| 6.2.3.1.2.2 4Rx FDD FR1 aperiodic subband CQI reporting under fading conditions for both SA and NSA | SNRs as specified  2% 55% 1.05 BLER 0.02 | SNR 0 dB  0% 0% 0.01 BLER 0 | SNR unchanged  limit unchanged limit unchanged  1.04 BLER limit unchanged |
| 6.2.3.2.1.1 4Rx TDD FR1 periodic CQI reporting under AWGN conditions for both SA and NSA | SNRs as specified Limits as in the Test Procedure | No test tolerances applied | SNR unchanged |
| 6.2.3.2.1.2 4Rx TDD FR1 periodic CQI reporting with Table 3 under AWGN conditions for both SA and NSA | SNRs as specified Limits as in the Test Procedure | No test tolerances applied | SNR unchanged |
| 6.2.3.2.2.1 4Rx TDD FR1 periodic wideband CQI reporting under fading conditions for both SA and NSA | SNRs as specified  5% 1.05 BLER 0.02 | SNR 0 dB  0% 0.01 BLER 0 | SNR unchanged  unchanged  1.04 BLER limit unchanged |
| 6.2.3.2.2.2 4Rx TDD FR1 aperiodic subband CQI reporting under fading conditions for both SA and NSA | SNRs as specified  2% 55% 1.05 BLER 0.02 | SNR 0 dB  0% 0% 0.01 BLER 0 | SNR unchanged  limit unchanged limit unchanged  1.04 BLER limit unchanged |
| 6.2A.3.1.1 2Rx CQI reporting accuracy under AWGN conditions for CA (2DL CA) | SNRs as specified Limits as in the Test Procedure | No test tolerances applied | Test requirement unchanged |
| 6.2A.3.1.2 2Rx CQI reporting accuracy under AWGN conditions for CA (3DL CA) | SNRs as specified Limits as in the Test Procedure | No test tolerances applied | Test requirement unchanged |
| 6.2A.3.1.3 2Rx CQI reporting accuracy under AWGN conditions for CA (4DL CA) | SNRs as specified Limits as in the Test Procedure | No test tolerances applied | Test requirement unchanged |
| 6.3.2.1.1 2Rx FDD FR1 Single PMI with 4Tx Type I- SinglePanel codebook for both SA and NSA | SNRs as specified 1.30 | SNR 0 dB 0.01 | SNR unchanged  1.29 |
| 6.3.2.1.2 2Rx FDD FR1 Single PMI with 8Tx Type I – SinglePanel codebook for both SA and NSA | SNRs as specified 1.50 | SNR 0 dB 0.01 | SNR unchanged  1.49 |
| 6.3.2.1.3 2Rx FDD FR1 Multiple PMI with 16Tx Type I – SinglePanel Codebook for both SA and NSA | SNRs as specified 2.50 | SNR 0 dB 0.01 | SNR unchanged  2.49 |
| 6.3.2.1.4 2Rx FDD FR1 Single PMI with 32Tx Type1 - SinglePanel codebook for both SA and NSA | SNRs as specified g 5.0 | SNR 0 dB g 0.01 | SNR unchanged g 4.99 |
| 6.3.2.1.5 2Rx FDD FR1 Multiple PMI with 16Tx TypeII codebook for both SA and NSA | SNRs as specified g 1.9 | SNR 0 dB g 0.01 | SNR unchanged g 1.89 |
| 6.3.2.1.6 2Rx FDD FR1 Multiple PMI with 16Tx Enhanced TypeII codebook for both SA and NSA | SNRs as specified 2.2 | SNR 0 dB 0.01 | SNR unchanged  2.19 |
| 6.3.2.2.1 2Rx TDD FR1 Single PMI with 4Tx TypeI - SinglePanel codebook for both SA and NSA | SNRs as specified 1.30 | SNR 0 dB 0.01 | SNR unchanged  1.29 |
| 6.3.2.2.2 2Rx TDD FR1 Single PMI with 8Tx TypeI - SinglePanel codebook for both SA and NSA | SNRs as specified 1.50 | SNR 0 dB 0.01 | SNR unchanged  1.49 |
| 6.3.2.2.3 2Rx TDD FR1 Single PMI with 16Tx Type1 - SinglePanel codebook for both SA and NSA | SNRs as specified 2.50 | SNR 0 dB 0.01 | SNR unchanged  2.49 |
| 6.3.2.2.4 2Rx TDD FR1 Single PMI with 32Tx Type1 - SinglePanel codebook for both SA and NSA | SNRs as specified 5.0 | SNR 0 dB 0.01 | SNR unchanged  4.99 |
| 6.3.2.2.5 2Rx TDD FR1 Multiple PMI with 16Tx TypeII codebook for both SA and NSA | SNRs as specified g 1.9 | SNR 0 dB g 0.01 | SNR unchanged g 1.89 |
| 6.3.2.2.6 2Rx TDD FR1 Multiple PMI with 16Tx Enhanced TypeII codebook for both SA and NSA | SNRs as specified 2.2 | SNR 0 dB 0.01 | SNR unchanged  2.19 |
| 6.3.3.1.1 Single PMI with 4TX TypeI-SinglePanel Codebook– SinglePanel codebook for both SA and NSA | SNRs as specified 1.30 | SNR 0 dB 0.01 | SNR unchanged  1.29 |
| 6.3.3.1.2 Single PMI with 8TX TypeI-SinglePanel Codebook– SinglePanel codebook for both SA and NSA | SNRs as specified 1.50 | SNR 0 dB 0.01 | SNR unchanged  1.49 |
| 6.3.3.1.3 4Rx FDD FR1 Multiple PMI with 16Tx Type I – SinglePanel Codebook for both SA and NSA | SNRs as specified 3.00 | SNR 0 dB 0.01 | SNR unchanged  2.99 |
| 6.3.3.1.4 4Rx FDD FR1 Single PMI with 32Tx Type1 - SinglePanel codebook for both SA and NSA | SNRs as specified g 7.0 | SNR 0 dB g 0.01 | SNR unchanged g 6.99 |
| 6.3.3.1.5 4Rx FDD FR1 Multiple PMI with 16Tx TypeII codebook for both SA and NSA | SNRs as specified g 1.9 | SNR 0 dB g 0.01 | SNR unchanged g 1.89 |
| 6.3.3.1.6 4Rx FDD FR1 Multiple PMI with 16Tx Enhanced TypeII codebook for both SA and NSA | SNRs as specified 2.2 | SNR 0 dB 0.01 | SNR unchanged  2.19 |
| 6.3.3.2.1 4Rx TDD FR1 Single PMI with 4Tx Type1 - SinglePanel codebook for both SA and NSA | SNRs as specified 1.30 | SNR 0 dB 0.01 | SNR unchanged  1.29 |
| 6.3.3.2.2 4Rx TDD FR1 Single PMI with 8Tx Type1 - SinglePanel codebook for both SA and NSA | SNRs as specified 1.50 | SNR 0 dB 0.01 | SNR unchanged  1.49 |
| 6.3.3.2.3 4Rx TDD FR1 Single PMI with 16Tx Type1 - SinglePanel codebook for both SA and NSA | SNRs as specified 3.0 | SNR 0 dB 0.01 | SNR unchanged  2.99 |
| 6.3.3.2.4 4Rx TDD FR1 Single PMI with 32Tx Type1 - SinglePanel codebook for both SA and NSA | SNRs as specified g 7.0 | SNR 0 dB g 0.01 | SNR unchanged g 6.99 |
| 6.3.3.2.5 4Rx TDD FR1 Multiple PMI with 16Tx TypeII codebook for both SA and NSA | SNRs as specified g 1.8 | SNR 0 dB g 0.01 | SNR unchanged g 1.79 |
| 6.3.3.2.6 4Rx TDD FR1 Multiple PMI with 16Tx Enhanced TypeII codebook for both SA and NSA | SNRs as specified 2.2 | SNR 0 dB 0.01 | SNR unchanged  2.19 |
| 6.4.2.1_1 2Rx FDD FR1 RI reporting for both SA and NSA | SNRs as specified 1.00 for Test 1 1.05 for Test 2 0.90 for Test 3 | SNR 0 dB 0.01 for Test 1 0.01 for Test 2 0.01 for Test 3 | SNR unchanged  0.99 for Test 1  1.04 for Test 2  0.89 for Test 3 |
| 6.4.2.2_1 2Rx TDD FR1 RI reporting for both SA and NSA | SNRs as specified 1.00 for Test 1 1.05 for Test 2 0.90 for Test 3 | SNR 0 dB 0.01 for Test 1 0.01 for Test 2 0.01 for Test 3 | SNR unchanged  0.99 for Test 1  1.04 for Test 2  0.89 for Test 3 |
| 6.4.3.1_1 4Rx FDD FR1 RI reporting for both SA and NSA | SNRs as specified 0.90 for Test 1 1.05 for Test 2 0.90 for Test 3 0.90 for Test 4 | SNR 0 dB 0.01 for Test 1 0.01 for Test 2 0.01 for Test 3 0.01 for Test 4 | SNR unchanged  0.89 for Test 1  1.04 for Test 2  0.89 for Test 3  0.89 for Test 4 |
| 6.4.3.2_1 4Rx TDD FR1 RI reporting for both SA and NSA | SNRs as specified 0.90 for Test 1 1.05 for Test 2 0.90 for Test 3 0.90 for Test 4 | SNR 0 dB 0.01 for Test 1 0.01 for Test 2 0.01 for Test 3 0.01 for Test 4 | SNR unchanged  0.89 for Test 1 as per Table G.3.4  1.04 for Test 2 as per Table G.3.4  0.89 for Test 3 as per Table G.3.4  0.89 for Test 4 as per Table G.3.4 |

#### << End of changes >>