3GPP TSG-RAN5 Meeting #99	R5-232597

# Incheon, Korea (Republic Of), 22nd May 2023 - 26th May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-1 | CR | 0306 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to MCData UE Configuration and User Profile |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NIST |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCProtoc16_enh2MCPTT_eMCData2-ConTest |  |  |  |  |  | Date: |  |  | 2023-05-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The MCData User Profile document and the MCData Service Configuration document needed to be updated for functional alias |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added parameters for functional alias to the MCData User Profile document and the MCData Service Configuration document |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MCData functional alias testing may not work properly and may produce false results |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.8.11, 5.5.8.12 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 5.5.8	Default MCS configuration management messages and other information elements

#### 5.5.8.1	MCX Initial UE Configuration

…

<START OF CHANGES>

#### 5.5.8.11	MCData User Profile

The structure of a user profile document is specified in TS 24.484 [14] clause 10.3.2.1. Single MCData configuration parameters are defined in TS 24.483 [13] clause 10.2.

Table 5.5.8.11-1: MCData User Profile Defaults

| Derivation Path: TS 24.484 [14], clause 10.3.2.1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcdata-user-profile |  |  |  |  |
| XUI-URI attribute | "sip:" & px_MCData_ID_User_A | same as the XUI value of the Document URI |  |  |
| user-profile-index attribute | "49" | value arbitrarily selected |  |  |
| Status | “true” | MCData user profile is enabled |  |  |
| ProfileName | "mcdata-user-profile-" & user-profile-index & ".xml" | name of the user profile document; user-profile-index is the value of the user-profile-index attribute | TS 24.483 [13] clause 5.2.7B |  |
| Common |  |  |  |  |
| index attribute | "0" | Index for the particular MCData user profile | TS 24.483 [13] clause 10.2.6 |  |
| UserAlias |  |  |  |  |
| alias-entry | px_MCData_User_A_Alias | Alphanumeric aliases of MCData user | TS 24.483 [13] clause 10.2.11 |  |
| MCDATAUserID |  |  |  |  |
| entry | px_MCData_ID_User_A |  |  |  |
| MissionCriticalOrganization | px_MCX_DomainName_Organization_A | Indicates the organization an MCData user belongs to | TS 24.483 [13] clause 10.2.16 |  |
| FileDistribution |  |  |  |  |
| FD-cancel-List-Entry |  |  |  |  |
| MCData-ID | px_MCData_ID_User_A | Contains the MCData user identity (MCData ID) of an MCData user that the configured MCData user is authorised to initiate a one-to-one communication, and corresponds to the "MCDataID" element of clause 10.2.16E in 3GPP TS 24.483 [4]; | TS 24.483 clause 10.2.21A |  |
| MCData_ID_KMSURI | tsc_MCX_KMS_Hostname | Contains the KMS URI for the security domain of the MCData user identity (MCData ID) of the MCData user and corresponds to the "MCDataUserIDKMSURI" element of clause 10.2.9A in 3GPP TS 24.483 [4]. If this parameter is absent, the KMS URI is identified by the <kms-sec> element of the <App-Server-Info> of the MCS UE initial configuration document as specified in clause 7.2.2.1 | TS 24.483 [13] clause 10.2.21A |  |
| TxRxControl |  |  |  |  |
| MaxData1To1 | “65535” | Indicates the maximum amount of data (in megabytes) that an MCData user can transmit in a single request during one-to-one communication. | TS 24.483 [13] clause 10.2.25 |  |
| MaxTime1to1 | “65535” | Indicates the maximum amount of time that an MCData user can transmit for in a single request during one-to-one communication. | TS 24.483 [13] clause 10.2.26 |  |
| TxReleaseList | px_MCData_ID_User_A | Indicates an MCData ID of an MCData user that this MCData user is allowed to request release of an ongoing transmission | TS 24.483 [13] clause 10.2.30 |  |
| GroupEmergencyAlert |  | Indicates the MCData group recipient for an MCData emergency Alert | TS 24.483 [13] clause 10.2.38 |  |
| entry | px_MCData_ID_User_A |  |  |  |
| OnNetwork |  |  |  |  |
| index attribute | "0" | Is of type "token" and is included within some elements for uniqueness purposes, and does not appear in the user profile configuration managed object specified in 3GPP TS 24.483 [4]. |  |  |
| MCDataGroupInfo |  |  |  |  |
| MCData-Group-ID | px_MCData_Group_A_ID | Indicates the MCData group ID for the on-network MCData group that the MCData user is allowed to use. | TS 24.483 [13] clause 10.2.47 |  |
| GMS-App-Serv-ID | tsc_MCX_GMS_Hostname | URI of the group management server hosting the on-network MCData group identified by the <MCData-Group-ID> element | TS 24.483 [13] clause 10.2.51 |  |
| IdMS-Token-Endpoint | "https://" & px_MCX_IdMS_token_IPAddress & ":" & px_MCX_IdMS_token_Port & tsc_MCX_IdMS_token_UriPath | Identity management server token endpoint identity information | TS 23.003 [69] TS 24.483 [13] clause 8.2.41A | IPv4 |
|  | "https://[" & px_MCX_IdMS_token_IPAddress & "]:" & px_MCX_IdMS_token_Port & tsc_MCX_IdMS_token_UriPath | Identity management server token endpoint identity information | TS 23.003 [69] TS 24.483 [13] clause 8.2.41A | IPv6 |
| GroupKMSURI | tsc_MCX_KMS_Hostname |  | TS 24.483 [13] clause 10.2.54A |  |
| Relativepresentation Priority | "7" |  |  |  |
| MaxAffiliations | “10” | contains an integer value between 0 and 255 indicating the presentation priority of the off-network group relative to other off-network groups and off-network users | TS 24.483 clause 10.2.71 |  |
| One-To-One-EmergencyAlert |  | Indicates the MCData user recipient for an on-network MCData emergency one-to-one alert | TS 24.483 clause 10.2.91 |  |
| entry | px_MCData_ID_User_A | Indicates the name of the MCData user recipient for an on-network MCData emergency one-to-one alert | TS 24.483 clause 10.2.92 |  |
| anyExt |  |  |  |  |
| MCDataContentServerURI | "http://" & tsc_MCData_MSF_Hostname & "/userA/files" | absolute URI associated with media storage function of MCData content server | TS 24.483 clause 10.2.97A |  |
| FunctionalAliasList |  |  | TS 24.483 clause 10.2.97B |  |
| entry[1] |  |  |  |  |
| uri-entry[1] | px_MCData_ID_User_A |  |  |  |
| anyExt |  |  |  |  |
| LocationCriteriaForActivation |  |  |  |  |
| EnterSpecificArea |  |  |  |  |
| EllipsoidArcArea |  |  |  |  |
| Center |  |  |  |  |
| Latitude | "35.74428" |  |  |  |
| longitude | "139.69806" |  |  |  |
| Radius | "50" | 50 meters |  |  |
| OffsetAngle | "0" |  |  |  |
| IncludedAngle | "360" | Full circle |  |  |
| ExitSpecificArea |  |  |  |  |
| EllipsoidArcArea |  |  |  |  |
| Center |  |  |  |  |
| latitude | "35.74428" |  |  |  |
| longitude | "139.69695" |  |  |  |
| Radius | "50" | 50 meters |  |  |
| OffsetAngle | "0" |  |  |  |
| IncludedAngle | "360" | Full circle |  |  |
| LocationCriteriaForDeactivation |  |  |  |  |
| EnterSpecificArea |  |  |  |  |
| EllipsoidArcArea |  |  |  |  |
| Center |  |  |  |  |
| latitude | "35.74428" |  |  |  |
| longitude | "139.69695" |  |  |  |
| Radius | "50" | 50 meters |  |  |
| OffsetAngle | "0" |  |  |  |
| IncludedAngle | "360" | Full circle |  |  |
| ExitSpecificArea |  |  |  |  |
| EllipsoidArcArea |  |  |  |  |
| Center |  |  |  |  |
| latitude | "35.74428" |  |  |  |
| longitude | "139.69806" |  |  |  |
| Radius | "50" | 50 meters |  |  |
| OffsetAngle | "0" |  |  |  |
| IncludedAngle | "360" | Full circle |  |  |
| manual-deactivation-not-allowed-if-location-criteria-met | "false" |  | TS 24.483 [13] clause 10.2.97B3D |  |
| MessageStoreHostname | tsc_MCData_MSF_Hostname | hostname identifying the message store function | TS 24.483 clause 10.2.97E |  |
| OffNetwork |  |  |  |  |
| index attribute | "0" |  |  |  |
| MCDataGroupInfo |  |  |  |  |
| MCData-Group-ID | px_MCData_Group_A_ID | Indicates the MCData group ID for the off-network MCData group that the MCData user is allowed to use. | TS 24.483 [13] clause 10.2.103 |  |
| GMS-App-Serv-Id | tsc_MCX_GMS_Hostname |  |  |  |
| IdMS-Token-Endpoint | "https://" & px_MCX_IdMS_token_IPAddress & ":" & px_MCX_IdMS_token_Port & tsc_MCX_IdMS_token_UriPath | Identity management server token endpoint identity information | TS 23.003 [69] TS 24.483 [13] clause 8.2.41A | IPv4 |
|  | "https://[" & px_MCX_IdMS_token_IPAddress & "]:" & px_MCX_IdMS_token_Port & tsc_MCX_IdMS_token_UriPath | Identity management server token endpoint identity information | TS 23.003 [69] TS 24.483 [13] clause 8.2.41A | IPv6 |
| Group-KMSURI | tsc_MCX_KMS_Hostname |  | TS 24.483 [13] clause 10.2.110A |  |
| RelativePresentationPriority | "7" | When it appears in: the <MCDataGroupInfo> element of the <OnNetwork> element, contains an integer value between 0 and 255 indicating the presentation priority of the on-network group relative to other on-network groups and on-network users, and corresponds to the "PresentationPriority" element of clause 10.2.55 in 3GPP TS 24.483 [4]; and the <MCDataGroupInfo> element of the <OffNetwork> element, contains an integer value between 0 and 255 indicating the presentation priority of the off-network group relative to other off-network groups and off-network users, and corresponds to the "PresentationPriority" element of clause 10.2.111 in 3GPP TS 24.483 [4]; |  |  |
| User-Info-Id | '555555555555'O |  |  |  |
| ruleset |  |  |  |  |
| rule |  |  |  |  |
| actions |  |  |  |  |
| allow-create-delete-user-alias | "true" |  |  |  |
| allow-create-group-broadcast- group | "true" |  |  |  |
| allow-create-user-broadcast-group | "true" |  |  |  |
| allow-transmit-data | "true" |  |  |  |
| allow-request-affiliated-groups | "true" |  |  |  |
| allow-request-to-affiliate-other-users | "true" |  |  |  |
| allow-recommend-to-affiliate-other-users | "true" |  |  |  |
| allow-regroup | "true" |  |  |  |
| allow-presence-status | "true" |  |  |  |
| allow-request-presence | "true" |  |  |  |
| allow-activate-emergency-alert | "true" |  |  |  |
| allow-cancel-emergency-alert | "true" |  |  |  |
| allow-cancel-emergency-alert-any-user | "true" |  |  |  |
| allow-enable-disable-user | "true" |  |  |  |
| allow-enable-disable-UE | "true" |  |  |  |
| allow-off-network-manual-switch | "true" |  |  |  |
| allow-off-network | "true" |  |  |  |
| anyExt |  |  |  |  |
| allow-query-functional-alias-other-user | "true" |  |  |  |
| allow-takeover-functional-alias-other-user | "true" |  |  |  |
| allow-one-to-one-communication-from-any-user | "true" |  |  |  |

| Condition | Explanation |
| --- | --- |
| IPv4 | IP address is IPv4 address |
| IPv6 | IP address is IPv6 address |

#### 5.5.8.12	MCData Service Configuration

The structure of a service configuration document is specified in TS 24.484 [14] clause 10.4.2.1. Single MCData group configuration parameters are defined in TS 24.483 [13] clause 11.2.

Table 5.5.8.12-1: MCData Service Configuration Defaults

| Derivation Path: TS 24.484 [14], clause 10.4 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| service configuration |  |  |  |  |
| domain attribute | px_MCData_User_A_Organization | Mandatory attribute: domain name of the mission critical organization |  |  |
| on-network |  |  |  |  |
| tx-and-rx-control |  |  |  |  |
| max-data-size-sds-bytes | "10000000" | The maximum data that the originating client can send in an SDS message |  |  |
| max-payload-size-sds-cplane-bytes | "1000" | The maximum payload data that the originating client can send in an SDS message over C-plane |  |  |
| max-data-size-fd-bytes | "100000000" | The maximum data that the originating client can send in an FD message |  |  |
| max-data-size-auto-recv-bytes | "10000000" | The maximum data that the server can send to the terminating client without requesting the user to indicate a present need for the data |  |  |
| signalling-protection |  |  |  |  |
| confidentiality-protection | "true" | Indicating whether confidentiality protection of MCData signalling is enabled or disabled between the MCData client and MCData server |  |  |
| integrity-protection | "true" | Indicating whether integrity protection of MCData signalling is enabled or disabled between the MCData client and MCData server |  |  |
| protection-between-mcdata-servers |  |  |  |  |
| allow-signalling-protection | :true" | Indicating whether protection of MCData signalling is enabled between MCData servers |  |  |
| file-availability |  |  |  |  |
| default-file-availability | "10000000" | The default time for which a file is available on the server for download, if a explicit time period is not requested by the originating client |  |  |
| max-file-availability | "10000000" | The maximum time for which a file can be made available on the server for download |  |  |
| anyExt |  |  |  |  |
| functional-alias-list |  |  |  |  |
| functional-alias-entry[1] |  |  |  |  |
| functional-alias | px_MCData_ID_FA_A |  |  |  |
| max-simultaneous-activations | "1" |  |  |  |
| allow-takeover | "true" |  |  |  |
| mcptt-user-list |  |  |  |  |
| entry[1] |  |  |  |  |
| uri-entry | px_MCData_ID_User_A |  |  |  |
| off-network |  |  |  |  |
| default-prose-per-packet-priority |  |  |  |  |
| mcdata-one-to-one-call-signalling | "1" |  | TS 24.483 [13] clause 11.2.11 |  |
| mcdata-one-to-one-call-media | "1" |  | TS 24.483 [13] clause 11.2.12 |  |

<END OF CHANGES>