# Title:	Test Tolerances for FR1 SRS-RSRP measurement in non-DRX

# Source:	Qualcomm

# 1.	Introduction

The purpose of this document is to present the methodology and principles used for the development of Test Tolerances for FR1 SRS-RSRP measurement in non-DRX.

The test cases in agenda for this document from 38.533, are:

- 4.6.5.1	EN-DC FR1 SRS-RSRP measurement in non-DRX

- 6.6.6.1	NR SA FR1 SRS-RSRP measurement in non-DRX

This document describes the process to derive the Test Tolerances. The calculations are provided in the accompanying spreadsheet.

# 2. 	Prototype test case in Annex A of TS 38.133

The test conditions are defined in the following extract from 3GPP TS 38.133 V18.1.0. The SA version of the test, A.6.6.6.1, is chosen to show only the NR related parameters.

<<Extract from TS 38.133>>

#### A.6.6.6.1	SRS-RSRP measurement with DRX

##### A.6.6.6.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of SRS-RSRP measurement. This test will verify the SRS-RSRP measurement requirements in clause 9.7.2.5 with the testing configurations for NR cells in Table A.6.6.6.1.1-1.

Table A.6.6.6.1.1-1: Applicable NR configurations for FR1 SRS-RSRP test

| Configuration | Description |
| --- | --- |
| 1 | NR 15 kHz SRS SCS, 10 MHz bandwidth, TDD duplex mode |
| 2 | NR 30 kHz SRS SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

##### A.6.6.6.1.2	Test Parameters

One cell is deployed in the test, which is FR1 PCell (Cell 1). The test parameters for PCell is given in Table A.6.6.6.1.2-1 and A.6.6.6.1.2-2 below. In the measurement control information, a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event I1 is used. The test consists of two successive time periods, with time duration of T1 and T2, respectively.

During the test, the test system transmits SRS resource for measurement in the DL slot according to the SRS configuration in Table A.6.6.6.1.2-4 and the test parameters for the (virtual) neighbour cell UE in Table A. 6.6.6.1.2-3. During the test, the test system does not transmit PDCCH/PDSCH/OCNG on SRS symbol to be transmitted and on 1 data symbol before SRS to be transmitted.

Table A.6.6.6.1.2-1: General test parameters for SRS-RSRP event triggered reporting for PCell in FR1

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1, 2 | Cell 1 |  |
| RF Channel Number |  | 1, 2 | 1: Cell 1 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.1 |  |
|  |  | 2 | SMTC.1 |  |
| SRS configuration |  | 1 | SRSConf.1 | Table A.6.6.6.1.2-3 |
|  |  | 2 | SRSConf.2 |  |
| CP length |  | 1, 2 | Normal |  |
| i1-Threshold | dBm | 1 | -97 |  |
|  |  | 2 | -95 |  |
| Hysteresis | dB | 1, 2 | 0 |  |
| Time To Trigger | s | 1, 2 | 0 |  |
| Filter coefficient |  | 1, 2 | 0 | L3 filtering is not used |
| DRX |  | 1, 2 | DRX.7 |  |
| Time offset between DL from serving cell and SRS from test system | s | 1,2 | 17.67 |  |
| T1 | s | 1, 2 | 5 |  |
| T2 | s | 1, 2 | 5 |  |

Table A.6.6.6.1.2-2: NR Cell specific test parameters for SRS-RSRP event triggered reporting for PCell in FR1

| Parameter | Unit | Test configuration | Cell 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| TDD configuration |  | 1 | TDDConf.1.1 |  |
|  |  | 2 | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 TDD |  |
|  |  | 2 | SR.2.1 TDD |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 TDD |  |
|  |  | 2 | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 TDD |  |
|  |  | 2 | CCR.2.1 TDD |  |
| OCNG Patterns |  | 1, 2 | OP.1 |  |
| TRS Configuration |  | 1 | TRS.1.1 TDD |  |
|  |  | 2 | TRS.1.2 TDD |  |
| Initial BWP configuration |  | 1, 2 | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2 | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2 | ULBWP.1.1 |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |
|  |  | 2 |  |  |
| Note 2 | dBm/SCS | 1 | -98 |  |
|  |  | 2 | -95 |  |
| Propagation Condition |  | 1, 2 | AWGN |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. |  |  |  |  |

Table A.6.6.6.1.2-3: NR Cell specific test parameters for SRS-RSRP event triggered reporting for neighbour cell UE

| Parameter | Unit | Test configuration | Neighbour cell UE |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| Note 2 | dBm/15 kHz | 1 | -98 |  |
|  |  | 2 |  |  |
| Note 2 | dBm/SCS | 1 | -98 |  |
|  |  | 2 | -95 |  |
|  | dB | 1 | -infinity | 4 |
|  |  | 2 |  |  |
|  | dB | 1 | -infinity | 4 |
|  |  | 2 |  |  |
| SRS-RSRP Note 3 | dBm/SCS kHz | 1 | -infinity | -94 |
|  |  | 2 | -infinity | -91 |
| Io | dBm/9.36 MHz | 1 | -70.05 | -64.59 |
|  | dBm/38.16 MHz | 2 | -63.96 | -58.50 |
| Propagation Condition |  | 1, 2 | AWGN |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SRS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

Table A.6.6.6.1.2-4: SRS configuration for measurement reporting

|  | Field | SRSConf.1 | SRSConf.2 | Comments |
| --- | --- | --- | --- | --- |
| SRS-ResourceSet | srs-ResourceSetId | 0 | 0 |  |
|  | srs-ResourceIdList | 0 | 0 |  |
|  | resourceType | Periodic | Periodic |  |
|  | Usage | Codebook | Codebook |  |
| SRS-Resource | SRS-ResourceId | 0 | 0 |  |
|  | nrofSRS-Ports | Port1 | Port1 |  |
|  | transmissionComb | n2 | n2 |  |
|  | combOffset-n2 | 0 | 0 |  |
|  | cyclicShift-n2 | 0 | 0 |  |
|  | resourceMapping startPosition | 0 | 0 |  |
|  | resourceMapping nrofSymbols | n1 | n1 |  |
|  | resourceMapping repetitionFactor | n1 | n1 |  |
|  | freqDomainPosition | 0 | 0 |  |
|  | freqDomainShift | 0 | 0 |  |
|  | freqHopping c-SRS | 12 | 12 |  |
|  | freqHopping b-SRS | 0 | 0 |  |
|  | freqHopping b-hop | 0 | 0 |  |
|  | groupOrSequenceHopping | Neither | Neither |  |
|  | resourceType | Periodic | Periodic |  |
|  | periodicityAndOffset | sl640, 4 | sl640, 9 |  |
|  | sequenceId | 0 | 0 | Any 10 bit number |

##### A.6.6.6.1.3	Test Requirements

The UE shall send one Event I1 triggered measurement report, with a measurement reporting delay less than 1920 ms from the beginning of time period T2.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

# 3. 	Discussion

The test cases consist of 2 NR test configurations, with 15kHz and 30kHz SCS configurations, accordingly.

The test scenario comprises of one serving NR cell and one virtual intra-frequency UE transmitting SRS periodically, which is the target of the CLI measurement report evaluated in the test. The test description specifies that “During the test, the test system does not transmit PDCCH/PDSCH/OCNG on SRS symbol to be transmitted and on 1 data symbol before SRS to be transmitted”. Therefore, neither the original test parameters or this analysis will consider any interference between the NR cell and the virtual UE.

Two subsequent time periods are considered, T1 and T2. During T1 only the serving NR cell is powered on, but at the start of T2 the virtual UE starts transmitting in addition to the serving NR cell, which is kept powered on. The purpose of the test is to evaluate the event i1 reporting delay upon a newly powered up virtual UE, with a minimum requirement of 1920ms from the start of T2.

The diagram below shows the evolution of the virtual UE SRS-RSRP over time, along with the configured event i1 Threshold and the absolute accuracy applicable to this test configuration (based on 38.133 Table 10.1.22.1.1-1):

Thanks to the diagram, it can be easily noted that for both test configurations, the i1-Threshold is above the lower end of the allowed SRS-RSRP variation. In addition, the effect of uncertainties will need to be evaluated. Therefore, it will be necessary to take both observations into consideration and adjust the corresponding test parameters to overcome this situation.

A detailed analysis of the test case is given in section 4 of this document.

# 4. 	Calculation of Test Tolerances

General approach

The general approach is given in the steps below:

- Copy the originally specified key parameters from the core requirements.

- Where relevant, calculate derived parameters from the core requirements.

- Define uncertainties for a minimum set of parameters.

- Define controlled parameters (critical to the test verdict), calculate sensitivity factors and uncertainty.

- Determine which original or derived parameters to offset (apply Test Tolerances to) and by how much.

- Recalculate original or derived parameters including Test Tolerances.

- Check that the controlled parameters meet requirements to get the correct test verdict.

Each step is explained below, and the calculations are given in the accompanying spreadsheet.

a) Original specified key parameters

The key parameters are copied from Tables A.6.6.6.1.1-1, A.6.6.6.1.2-1, A.6.6.6.1.2-2, A.6.6.6.1.2-3 and A.6.6.6.1.2-4 in TS 38.133. The key parameters are selected as the minimum set to define the power levels, and which are subject to a test system uncertainty which may affect the verdict of the test.

A number of UE core requirement values and their associated parameters are also entered in this section. The requirements are entered because they affect how the UE sees the received signals at baseband.

It is important to notice that the SRS resources configured for both configurations 1 and 2, SRSConf.1 and SRSConf.2 correspondingly, have been defined to span 48 RBs in both cases. This is the result of core requirement 10.1.22.1.1 where SRS-RSRP accuracy requirements have been defined for bandwidth of SRS resource of 48 RBs (quoted and highlighted in section c) below). This narrower resource configuration will need to be considered when calculating the SRS contribution to Io in sections b) and f).

The key parameters appear in section a) of the accompanying spreadsheet.

b) Derived parameters

A number of derived parameters are calculated, using the base information in a). The reason for deriving each additional parameter is given in the “Comment” column of section b) in the accompanying spreadsheet.

Some UE core requirements in section a) are converted into UE-related derived parameters, in a form that allows them to be used in later sections of the spreadsheet. The rationale and method for UE gain G midpoint is given in TR 38.903 clause A.2.6, and for UE Spherical coverage gain midpoint in TR 38.903 clause A.2.1.

In this test case at the start of T2, the virtual UE becomes detectable, and the UE is expected to detect and measure the SRS transmissions, and to trigger event i1-based measurement report. The criterion for event i1 is defined as: interference becomes higher than absolute threshold. Therefore, we have defined derived parameter (SRS-RSRP – i1-Threshold) to control whether the event is triggered.

c) Uncertainties

The Test system provides 1 cell and 1 virtual UE, on the same frequencies. We propose to control the following parameters:

- AWGN Absolute power, Noc averaged over BWConfig uncertainty, ±1.5dB

- Ratio of virtual UE signal / AWGN, Ês / Noc averaged over BWConfig uncertainty, ±0.3dB

In this test the UE is requested to measure the SRS-RSRP over the BW of the target SRS resource configured for CLI measurements. In addition, this test has separate constraints on the overall power in the configured bandwidth Io.

This choice forms a minimum set, so the superposition principle can be applied if necessary.

UE-related uncertainties are considered, together with selected UE core requirements that have an allowed range, and are therefore handled as a UE uncertainty:

- UE SRS-RSRP absolute accuracy: Value taken from TS 38.133, section 10.1.22.1

Based on the Io calculation in b) we observe that Io nominal is > -70dBm/CBW, therefore the allowed SRS-RSRP accuracy in this scenario is ±6.5dB for SCS 15kHz and ±7dB for SCS 30kHz, as highlighted in the below extract from 38.133:

<< Extracts from TS 38.133 >>

##### 10.1.22.1.1	SRS-RSRP Accuracy

The SRS-RSRP measurement reported by the UE shall fulfil the accuracy requirements defined in Table 10.1.22.1.1-1 for FR1 and Table 10.1.22.1.1-2 for FR2, provided that the following conditions are met. The accuracy requirements in this clause are derived based on AWGN radio propagation conditions.

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for SRS-RSRP measurements are fulfilled according to Annex B.2.z for a corresponding Band for each relevant SRS resource configured for measurement.

-	The time difference between UE’s DL reference timing in the serving cell and SRS arrival time is no larger than Terror_SRS_RSRP, where

-	Terror_SRS_RSRP = TC × NTA_offset + 4.67us for FR1

-	Terror_SRS_RSRP = TC × NTA_offset + 3.67us for FR2

-	NTA_offset is defined in Table 7.1.2-2

-	TC is 0.509ns

-	The number of SRS ports in the SRS resource configured for measurement is 1,

-	The number of symbols in the SRS resource configured for measurement is 1,

-	The number of repetitions in the SRS resource configured for measurement is 1,

-	Frequency hopping, sequence group hopping or sequence hopping is disabled in the SRS resource configured for measurement,

-	The bandwidth of the SRS resource is 48 PRBs.

-	One of the following conditions is met

-	There is no other SRS resource with the same root sequence and on the same symbol and with same comb as the relevant SRS resource.

-	If multiple SRS resources are on the same symbol and with same comb, the distance between cyclic shifts of any two resources is no less than 6 if transmissionComb = n4, and no less than 4 if transmissionComb = n2.

Table 10.1.22.1.1-1:  SRS-RSRP absolute accuracy in FR1

| Accuracy |  |  |  |  |  | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition |  |  | Extreme condition |  |  | SRS Ês/Iot | Io Note 1 range |  |  |  |  |  |
|  |  |  |  |  |  |  | NR operating band groups Note 2 | Minimum Io |  |  |  | Maximum Io |
| dB |  |  |  |  |  | dB |  | dBm / SCSSRS |  |  | dBm/BW Channel | dBm/BW Channel |
| SCSSRS (kHz) |  |  | SCSSRS (kHz) |  |  |  |  |  |  |  |  |  |
| 15 | 30 | 60 | 15 | 30 | 60 |  |  | SCSSRS = 15 kHz | SCSSRS = 30 kHz | SCSSRS = 60 kHz |  |  |
| 3 | 3.5 | 5 | 7.5 | 8 | 9.5 | 1 | NR_TDD_FR1_A, | -120 | -117 | -114 | N/A | -70 |
|  |  |  |  |  |  |  | NR_TDD_FR1_C | -119 | -116 | -113 | N/A | -70 |
|  |  |  |  |  |  |  | NR_TDD_FR1_D | -118.5 | -115.5 | -112.5 | N/A | -70 |
|  |  |  |  |  |  |  | NR_TDD_FR1_E | -118 | -115 | -112 | N/A | -70 |
| 6.5 | 7 | 8.5 | 9.5 | 10 | 11.5 | 1 | NR_TDD_FR1_A, NR_TDD_FR1_C, NR_TDD_FR1_D, NR_TDD_FR1_E | N/A | N/A | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 10.1.22.1.1-2: SRS-RSRP absolute accuracy in FR2

| Accuracy |  |  |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition |  | Extreme condition |  | SRS Ês/Iot | Io Note 1 range |  |  |  |
|  |  |  |  |  | Minimum Io |  |  | Maximum Io |
| dB |  |  |  | dB | dBm / SCSSRS Note 2 |  | dBm/BWChannel | dBm/BWChannel |
| SCSSRS (kHz) |  | SCSSRS (kHz) |  |  | SCSSRS = | SCSSRS = |  |  |
| 60 | 120 | 60 | 120 |  | 60kHz | 120kHz |  |  |
| 6 | 8.5 | 9 | 11.5 | ≥1 | Same value as SRS_RP in Table B.2.7-2, according to UE Power class, operating band and angle of arrival |  | N/A | -70 |
| 9 | 11.5 | 11 | 13.5 | ≥1 | N/A |  | -70 | -50 |
| NOTE 1: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. NOTE 2: Values based on Refsens and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. NOTE 3: In the test cases, the SSB Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. |  |  |  |  |  |  |  |  |

We observe in the highlighted snippets of 38.133 section 10.1.22.1.1, that the highest minimum Io requirement is -118dBm for SCS 15kHz and -115dBm for SCS 30 kHz. This needs to be considered as well in the analysis.

In addition, Annex B.2.z (namely, B.2.7) with the minimum SRS-RSRP for the corresponding band needs to be satisfied. The highest minimum SRS-RSRP is -117dBm for SCS 15kHz and -114dBm for SCS 30kHz. This needs to be considered as well in the analysis.

We notice that the minimum SRS Es/Iot is same to that in Table 10.1.22.1.1-1, so no new requirement to be satisfied.

<< Extracts from TS 38.133 >>

## B.2.7	Conditions for SRS-RSRP measurements

This clause defines the following conditions for SRS-RSRP measurement and corresponding procedures performed based on SRSs: SRS_RP and SRS Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.2.7-1 for FR1 NR cells.

The conditions are defined in Table B.2.7-2 for FR2 NR cells.

Table B.2.7-1: Conditions for SRS-RSRP measurements in FR1

| Parameter | NR operating band groups Note1 | Minimum SRS_RP |  |  | SRS Ês/Iot |
| --- | --- | --- | --- | --- | --- |
|  |  | dBm / SCSSRS |  |  | dB |
|  |  | SCSSRS = 15 kHz | SCSSRS = 30 kHz | SCSSRS = 60 kHz |  |
|  | NR_TDD_FR1_A | -120 | -117 | -114 |  |
|  | NR_TDD_FR1_C | -119 | -116 | -113 |  |
| Conditions | NR_TDD_FR1_D | -118.5 | -115.5 | -112.5 | 1 |
|  | NR_TDD_FR1_E | -118 | -115 | -112 |  |
|  | NR_TDD_FR1_F | -117.5 | -114.5 | -111.5 |  |
|  | NR_TDD_FR1_G | -117 | -114 | -111 |  |
| NOTE 1: NR operating band groups are defined in clause 3.5.2. |  |  |  |  |  |

Table B.2.7-2: Conditions for SRS-RSRP measurements in FR2

| Parameter | Angle of arrival | NR operating bands | Minimum SRS_RP Note 2, Note 3 |  |  |  |  |  | SRS Ês/Iot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | dBm / SCSSRS |  |  |  |  |  | dB |
|  |  |  | SCSSRS = 60 kHz |  |  |  |  | SCSSRS = 120 kHz |  |
|  |  |  | UE Power class |  |  |  |  | UE Power class |  |
|  |  |  | 1 | 2 | 3 | 4 | 5 | 1, 2, 3, 4, 5 |  |
| Conditions | Rx Beam Peak | n257 | -124.5 | -119.0 | -115.3 | -124.0 | -119.6 | (Value for SCSSRS = 60 kHz) +3dB | ≥1 |
|  |  | n258 | -124.5 | -119.0 | -115.3 | -124.0 | -119.8 |  |  |
|  |  | n260 | -121.5 |  | -112.7 | -122.0 |  |  |  |
|  |  | n261 | -124.5 | -119.0 | -115.3 | -124.0 |  |  |  |
|  | Spherical coverage Note 1 | n257 | -116.5 | -108.0 | -104.4 | -115.0 | -111.6 | (Value for SCSSRS = 60 kHz) +3dB | ≥1 |
|  |  | n258 | -116.5 | -108.0 | -104.4 | -115.0 | -111.8 |  |  |
|  |  | n260 | -113.5 |  | -100.1 | -110.0 |  |  |  |
|  |  | n261 | -116.5 | -108.0 | -104.4 | -115.0 |  |  |  |
| NOTE 1: Values based on EIS spherical coverage as defined in clause 7.3.4 of TS 38.101-2 [19]. Side condition applies for directions in which EIS spherical coverage requirement is met. NOTE 2: Values specified at the Reference point to give minimum SRS Ês/Iot, with no applied noise. NOTE 3: For UEs that support multiple FR2 bands, Rx Beam Peak values are increased by ∆MBP,n and Spherical coverage values are increased by ∆MBS,n, the UE multi-band relaxation factor in dB specified in clause 6.2.1 of TS 38.101-2 [19]. |  |  |  |  |  |  |  |  |  |

d) Controlled parameters critical to verdict

In many RRM test cases there is not a simple one-to-one relationship between the parameters that can be set by the test equipment, and their effect on parameters determining the test verdict.

It is therefore essential to identify those parameters determining the test verdict, and the side conditions that should be met for the UE to be tested correctly. The controlled parameters listed in the accompanying spreadsheet for T1 and T2, have been derived by study of the test case and by careful reading of the relevant clauses in TS 38.133. The reason for each parameter being critical to the test verdict is given briefly in the “Comment” column of section d) in the accompanying spreadsheet. Information about the value to be achieved is given in the “Requirement” column of section g) in the spreadsheet.

Having identified the parameters critical to the test verdict which need to be controlled, we now need to consider how they are affected by the parameters which can be set by the test equipment. This is done by working out “sensitivity factors”. A sensitivity factor is just the ratio (effect on a critical parameter y / a test equipment uncertainty x) and is usually in dB/dB.

Where there is a simple one-to-one relationship between the parameters that can be set by the test equipment, and their effect on parameters determining the test verdict, the sensitivity factors are derived by inspection as one, or if no effect, as zero.

Where there is not a simple one-to-one relationship between the parameters that can be set by the test equipment, and their effect on parameters determining the test verdict, the calculation of sensitivity factors is done using a method selected from TR 38.903. Es/Iot at UE baseband is calculated according to TR 38.903 A.2.4, and applied Io calculated according to TR 38.903 A.2.5.

Sensitivity factors for UE-reported SRS-RSRP also follow methods defined in TR 38.903. The UE-related factors are from clause A.2.6, depending on the type of measurement and the the test setup. UE-reported SRS-RSRP is affected by a combination of Test system uncertainties and UE-related uncertainties.

The method of combining uncertainties is different for T1 and T2 because T1 doesn’t have applied interference, while T2 has virtual UE. More detailed explanation is given in the accompanying spreadsheet, and in TR 38.903 A.2.7.

Having filled in the matrix of sensitivity factors, the accompanying spreadsheet calculates the overall uncertainty for each controlled parameter, considering the uncertainties and sensitivity factors for each parameter that can be set by the test equipment, and the UE-related uncertainties where relevant. This process follows the superposition principle. More details and explanation can be found in section 4 of TR 38.903.

e) Determine parameters to offset

As shown in the diagram presented above, the SRS-RSRP level originally proposed in the test case description is very close to the i1-Threshold. This causes the lowest of the actual measurement variation, considering all factors (accuracy and uncertainties) to be below this threshold, and to therefore have impact to the test case result.

We propose to overcome this situation by sufficiently increasing the SRS-RSRP so the lowest possible measurement, considering all factors, is still above the i1-Threshold, and therefore the event can still be satisfied. In this line, the SRS-RSRP is increased by +5.25dB for configuration 1 and by +4.75dB for configuration 2. No other test parameter needs to be adjusted.

f) Parameters modified by Test Tolerances

Based on the decision in e), the set of parameters in a) and b) is reproduced in section f) of the accompanying spreadsheet, modified by the Test Tolerances (applied offsets).

Re-derived parameters are calculated using the same methods as were used in step b).

g) Check controlled parameters Min/Max

Using a format similar to that in step d), the nominal value of each controlled parameter is recalculated, as some will have changed from the original due to the application of the Test Tolerances in step f).

The minimum and maximum values, due to variability from uncertainties, of controlled parameters is then calculated and compared against the different requirements (Es/Iot, SRS-RSRP Io and SRS-RSRP).

In addition, the derived parameter (SRS-RSRP – i1-Threshold) is also checked. According to 38.331 definition of event i1:

<< Extracts from TS 38.331 >>

#### 5.5.4.10	Event I1 (Interference becomes higher than threshold)

The UE shall:

1>	consider the entering condition for this event to be satisfied when condition I1-1, as specified below, is fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition I1-2, as specified below, is fulfilled.

Inequality I1-1 (Entering condition)

Mi – Hys > Thresh

Inequality I1-2 (Leaving condition)

Mi+ Hys < Thresh

The variables in the formula are defined as follows:

Mi is the measurement result of the interference, not taking into account any offsets.

Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within reportConfigNR for this event).

Thresh is the threshold parameter for this event (i.e. i1-Threshold as defined within reportConfigNR for this event).

Mi, Thresh are expressed in dBm.

Hys is expressed in dB.

We note that:

- Mi = evaluated measured SRS-RSRP

- Hys = 0dB

And therefore, the inequality can be reduced to:

- SRS-RSRP – i1-Threshold < 0 during T1

- SRS-RSRP – i1-Threshold > 0 during T2

Where only T2 is of interest to this test, as the virtual UE is powered off during T1 which causes its SRS to be undetectable during this time interval.

The critical requirement for each parameter is given in the “Requirement” column of section g) in the accompanying spreadsheet. The cases closest to limit are identified by turquoise cells in the spreadsheet. If all the requirements are met, then the chosen offsets are acceptable. It can be seen that with the uncertainty values and Test Tolerances proposed, the requirements are met.

# 5. Treatment of other configurations within the same test case

TS 38.533 test case 6.6.6.1 has two configurations. Both test cases have been addressed in the attached excel sheet.

# 6. Treatment of related test cases

TS 38.533 also contains a similar test case in clause 4.6.5.1, which has the same NR Cell signal levels, so we propose to apply the same level uncertainties and Test Tolerances. There is no need to provide a tab on the spreadsheet as all the numeric values are identical.

# 7. Treatment of extreme conditions

SRS-RSRP values and other analysis considerations have been calculated are for normal conditions.